Romania General Transport Master Plan

Revised Final Report on the Master Plan Short, Medium and Long Term

Prepared by:			Checked by:

Iain Mobbs, Frank Mohan, Geoff Clarke	Craig Bell

Johan Els	Regional Director

Approved by:

Martin Bright

Director

Romania General Transport Master Plan

Revised Final Report on the Master Plan Short, Medium and Long Term

| Rev No | Comments | Checked by | Approved by | Date |
| --- | --- | --- | --- | --- |
| 1 | First Draft for Client Comment | CB | MJB | 19/08/2013 |
| 2 | Revised following Client Comments | CB | MJB | 29/08/2014 |
| 3 | Revised following Client Comments | CB | MJB | 30/09/2014 |
| 4 | Revised following Client Comments | CB | MJB | 13/02/2015 |

Strada Polona, Nr. 68-72, Sector 1, Bucuresti, Romania

Telephone: +4 021 316 1163   Website: http://www.aecom.com

Job No: 60268467	Reference: Revised Final Master Plan	Date Created: February 2015

This document has been prepared by AECOM Ingenieria SRL for the sole use of our client (the “Client”) and in accordance with generally accepted consultancy principles, the budget for fees and the terms of reference agreed between AECOM Ingenieria SRL and the Client. Any information provided by third parties and referred to herein has not been checked or verified by AECOM Ingenieria SRL, unless otherwise expressly stated in the document. No third party may rely upon this document without the prior and express written agreement of AECOM Ingenieria SRL.

IMPORTANT NOTE

The current draft document should be read strictly from the perspective of the following:

CAVEATS

Status of current document

- The current document is a working draft of the Master Plan Report. This version is not final and will be updated with the results of more detailed analysis which is being carried out in particular for the rail, ports, airports and intermodal transport sectors. The updating process will be done in parallel with the incoroporation of the feed-back collected from the public consultation process.

- The Strategic Environmental Assessment (SEA) procedure (including the Appropriate Assessment) is ongoing. Upon completion its conclusions will be incorporated in the final version of the Master Plan report.

Coverage of the Master Plan

- The Master Plan refers to the major objectives of the national transport system. Therefore it is a high-level planning instrument relevant for major interventions (projects and other actions) with significance for the objectives of the national transport system – which are suitable for modelling, apprasial and prioritisation at Master Plan level. This means a range of small scale interventions are not within the scope of the Master Plan, which does not mean they shouldn’t be financed by the Ministry of Transport, but that the Master Plan is not a proper instrument for planning it and therefore that there should be a different process of defining, planning and prioritising such interventions. Typical interventions outside the scope of the Master Plan are:

- Actions required for the day-to-day running of business (regular operations and maintenance activities of the various entities under MoT), e.g. administrative buildings,office  equipment, consumables, operating costs, regular maintenance, etc.

- Investments of nature of such small size and detailed nature that goes well below the Master Plan level, e.g. small scale renewals/rehabilitations of small rail stations, small scale safety interventions, small scale intra-zone interventions which cannot be modelled, etc.

- In addition, there is another group of projects not evaluated within the Master Planning exercise. These are the “Reference Case” projects i.e. those already committed investments which had finacing already decided (and often were already under construction) at the time of defining the “Reference Case” for the purpose of the National Transport Model (year 2013). The list and map of the committed “Reference Case” projects assumed to continue to be implemented “by default” is provided in Annex …

Time horizon of the Master Plan

- The Master Plan planning horizon is the year 2030. Given the level of uncertainty associated with long-term forecasting, any action beyond the year 2030 should be reconfirmed on the basis of an updated plan (e.g. carried out within 10 years time i.e. in 2025).

- However, for more volatile sectors, especially airports – where the level of uncertainty in the traffic forecast is much higher than e.g. for roads sector, the Master Plan horizon to be taken up for implementation should not exceed the year 2020, whilst any projects beyond 2020 should be re-confirmed on the basis of the actual market (demand/traffic) developments.

Level of analysis and relation with Feasibility Studies

- The level of the Master Plan analysis is high by its very nature. Consequently and in order to ensure a fair comparison basis across projects and thus a relevant comparison of their economic performance indicators, high-level uniform assumptions have been made on the project costs (based on average values per km/type of infrastructure/type of terrain). This means the cost estimates used in the Master Plan do not necessarily match precisely more detailed estimates from e.g. existing Feasibility Studies (FS), which is not an error but a methodological choice. However, normally the difference between the Master Plan estimates and FS values should not exceed <25-30%>, which is the usual approximation margin for this level of analysis.

- Inherent from the high level of analysis is the approximation of the economic performance, which because of (i) the margin in the cost estimation and (ii) the scale of the economic benefits (i.e. some local benefits –e.g. intra-zone- might not be captured by the National Transport Model) should be considered preliminary until a more detailed analysis at the level of each project is carried out further at FS level. A methodological consequence was the choice of not excluding projects at the usual threshold of the economic discount rate (5%) but lowering the pass/fail bar to 3% EIRR at this level, whilst the final decision on the economic acceptance of any project would be (re-)confirmed on the basis of the more detailed analysis of both the costs and benefits at FS level.

- The same logic applies to the technical solutions. The Master Plan defined generic measures (interventions) to serve the specific operational objectives defined in response to the problems identified – e.g. “Improve travel speeds on Bucharest – West Road Corridor”. To enable model testing, costing and economic appraisal, certain preliminary technical solutions have been defined e.g. 2x2 motorway/express-way, rehabilitation of a rail section to a specific speed, extension of a passenger terminal to a certain capacity, etc. Such technical solutions will need to be however reconfirmed at the level of each particular project within the FS on the basis of a detailed options analysis including more detailed cost, capacity, economic and environmental impact analyses. In this respect it is imperative that the ToR for the new Feasibility Studies launched for the Master Plan projects includes clear and explicit provisions for such capacity and options analysis.

- Similarly, the recommended approach - which will have to be reconfirmed for each case within Feasibility Studies - is that the infrastructure should be designed in such a way as to allow future development (for example from an expressway profile to a motorway profile, from 2 lanes to 3 lanes per direction, etc) if and when such developments are justified by the demand and covered by funding sources.

Conclusions

- In summary, this Master Plan has been produced at a point in time based upon the best information available at that time and underpinned by a robust process which has been used to appraise the various schemes being proposed. There will be changes which occur over time which will impact upon the costs, standards and performance of each individual infrastructure scheme proposed. These include, inter alia:

- Changes in the timetable for implementation of other schemes, across all modes, which impact upon the scheme in question;

- Improved information as a consequence of feasibility, or other, studies in to the particular scheme; and

- External factors, such as changes in the macro level economic performance of the country, which impact upon forecast demand for a particular scheme.

- Each of these will potentially affect the required standard of the improvement, the costs of implementation and the economic merit of an individual scheme. It is important therefore that the Master Plan is seen as a live document which provides a robust platform for moving the transport sector forward, but which also needs to be maintained and reviewed over time in order to ensure that the objectives for improving the transport sector are met.

- This Master Plan Report will be supported by a separate Implementation Plan which will outline the schedule for when projects will be delivered. The Implementation Plan will take into account project maturity and eligibility for alternative funding streams, criteria which have not been included in the multi criteria assessment which has been used to develop the Master Plan list of projects.  It is therefore possible that in the Implementation Plan projects will be delivered in a different order to that indicated in the prioritised list of projects in this Master Plan Report.

The way the Master Plan fulfills the ex-ante conditionalities defines in the reference document

EUROPEAN COMMISSION, Directorate-General- Regional and Urban Policy, DRAFT -“Guidance on Ex ante Conditionality for the European Structural and Investment Funds”, PART II, “Criteria for fulfilment

| Criteria for fulfilment | Criteria fulfilled? |  |
| --- | --- | --- |
|  | YES / NO | Elements of non-fulfilment |
| The existence of a comprehensive transport plan or plans or framework or frameworks for transport investment which: |  |  |
| - The relevant operational programme and where appropriate, the Partnership Agreement contains a reference to the name of the plan or framework and provides a hyperlink to the documents(s). | NO | Master Plan not yet approved |
| – complies with legal requirements for strategic environmental assessment : |  |  |
| - An environmental report has been prepared in which the likely significant effects on the environement of the implementation of the plan or framework and reasonable alternatives taking into account the objectives and the geographical scope of the comprehensive transport plan or framework, are identified, described and evaluated. | YES |  |
| - The draft plan or framework and the environmental report have been made available to the public and the authorities with specific environmental responsibilities designated by the Member States who are likely to be concerned by the environemental effects of the implementing plans. | NO | The draft plan has been made available to the public and the authorities on October 2012 and October 2013 . Each chapter from Environmental Report have been presented in the working group. The final Environmental Report will be make available after Appropriate Assessment will be approved by MECC. The final Environmental Report must include the conclusion of Appropriate Assessment. |
| - In case of possible significant transboundary effects, the draft plan or framework and the environmental report have been forwarded to the relevant/affected Member States. | NO | In case of possible significant transboundary effects the Final Environmental Report and Master Plan will be forwarded to the relevant/affected Member States. Responsibility to inform the relevant/affected Member States lies the central public authority promoting the plan (in this case the Ministry of Transport), on the recommendation of the Ministry of Environment and Climate Change. It is recommended that the establishment of States which are notified to be performed by the central public authority promotes plan with the Ministry of Environment and Climate Change. In according with GD 1076/2004 Art. 22 (2) din HG 1076/2004 menţionează că „În cazul în care implementarea planului sau programului poate avea efecte semnificative transfrontieră, titularul, prin intermediul autorităţii publice centrale care promovează planul sau programul, este obligat să transmită proiectul de plan sau de program şi raportul de mediu elaborat pentru acesta, în limba engleză, autorităţilor centrale de mediu din statele posibil afectate, în termen de maximum 20 de zile calendaristice de la finalizarea raportului de mediu, conform art. 21 alin. (3)”. |
| - The environmental report and the opinions expressed in the relevant consultations (including as appropriate transboundary ones) have been duly taken into account during the preparation of the comprehensive transport plan or framework. | NO | See previous. |
| - When the plan or framework has been adopted, the authorities with environmental responsibilities, the public and any Member State consulted, are informed and the following items have been made available to them: the plan or framework as adopted, the statement referred to in Article 9(1) of the SEA Directive and the measures concerning monitoring referred to in Article 10 of the SEA Directive. | NO | See previous. |
| ‒ sets out the contribution to the single European Transport Area consistent with Article 10 of Regulation (EU) No1315/2013 of the European Parliament and of the Council, including priorities for investments in the core TEN-T network and the comprehensive network where investment from the ERDF and CF is envisaged; and secondary connectivity. | YES |  |
| - The investment priorities included in the comprehensive transport plan or framework connect the identified main nodes (see the list in annex) and provide for connections with neighbouring countries' transport infrastructure networks. The Member State shall also demonstrate how its investments in secondary connectivity will contribute to the single European Transport Area57. | YES |  |
| - The comprehensive transport plan or framework includes measures that are necessary for: |  |  |
| ensuring enhanced accessibility and connectivity for all regions of the Union while taking into consideration the specific case of islands, isolated networks and sparsely populated, remote and outermost regions; | YES |  |
| ensuring optimal integration of the transport modes and interoperability within transport modes; | YES |  |
| bridging missing links and removing bottlenecks, in particular in cross-border sections; | YES |  |
| promoting the efficient and sustainable use of the infrastructure and, where necessary, increase the capacity; | YES |  |
| improving or maintaining the quality of infrastructure in terms of safety, security, efficiency, climate and where appropriate disaster resilience, environmental performances, social conditions, accessibility for all users, including elderly people, persons with reduced mobility and disabled passengers, as well as the quality of services and continuity of traffic flows; | YES |  |
| implementing and deploying telematic applications as well as promoting innovative technological development; | YES |  |
| - Particular consideration shall also be given in the comprehensive transport plan or framework to measures that are necessary for: |  |  |
| ensuring fuel security through increased energy efficiency and promoting the use of alternative and in particular low or zero carbon energy sources and propulsion systems; | YES |  |
| mitigating exposure of urban areas to negative effects of transiting rail and road transport; | YES |  |
| removing administrative and technical barriers, in particular to the interoperability of the trans-European transport network and to competition. | YES |  |
| - sets out a realistic and mature pipeline for projects envisaged for support from the ERDF and CF |  |  |
| - The plan or framework for transport investments includes a table containing : |  |  |
| a list of prioritised projects (studies, upgrading or works) that the Member State envisages launching over the period and asking for support from the ERDF and CF. | NO | To be included in separate Strategy Report containing Implementation Plan |
| the name of the authorities and stakeholders involved in the lead of these projects, the foreseen expenditures and a financing plan, | NO | To be included in separate Strategy Report containing Implementation Plan |
| a realistic timetable for delivery of the projects identified indicating dates for feasibility studies, a Cost Benefit Analysis, EIA procedure58, an implementation timetable including procurement and permission procedures, and for potential state aid notification (per phase for bigger projects). | NO | To be included in separate Strategy Report containing Implementation Plan |
| - Measures to ensure the capacity of intermediary bodies and beneficiaries to deliver the project pipeline. |  |  |
| - The Member State has provided an adequate description of the measures already in place to ensure the capacity of intermediary bodies and beneficiaries to deliver the project pipeline: |  |  |
| These measures are based on the analysis of both the bottlenecks and of the weaknesses of intermediary bodies and beneficiaries to deliver timely the project pipeline, as regards: |  |  |
| tendering (including tenders without competition, irregularities) | NO | To be included in the Final version of the Master Plan |
| implementing environmental requirements, | NO | To be included in the Final version of the Master Plan |
| developing and prioritising a mature project pipeline, | YES |  |
| financial project management, | NO | To be included in the Final version of the Master Plan |
| funding for maintenance and operations, | YES |  |
| administrative burden and red tape, | NO | To be included in the Final version of the Master Plan |
| managing complex systems (ITS such as ETCS-ERTMS, VTMIS, RIS, emaritime services and air traffic management system). | YES |  |
| They include training and appropriate internal procedures to monitor and identifiy potential delays and to ensure a smooth and effective procurement; | NO | MT are still in the process of establishing new organizational structures |
| A early warning system is in place to identify and solve any difficulties rising from intermediary bodies and beneficiairies when delivering the project pipeline; | NO | MT are still in the process of establishing new organizational structures |
| Adequate assistance schemes are in place to help beneficiaries during procedure and implementation to be able to replace projects quickly when implementation is blocked. | NO | MT are still in the process of establishing new organizational structures |

Table of Contents

Glossary and Terminology	11

1	Introduction	13

2	Methodology	16

2.1	Overall Methodology	16

2.2	Objective setting	17

2.3	Identifying the Problems and Defining the Interventions	23

2.4	The National Transport Model	26

2.5	Cost-Benefit Analysis	49

2.6	Prioritisation of Projects	54

2.7	Ex-Ante Conditionalities	60

3	Existing Situation and Recent Transport Trends in Romania and Europe	66

3.1	GTMP Relation with the Relevant EU Transport Policy Documents	66

3.2	Road Transport Trends in European Context	74

3.3	Rail Transport Trends in European Context	79

3.4	Ports and Waterways Trends in European Context	85

3.5	Aviation Trends in European Context	88

3.6	Transport and Economic Development	90

4	Road Transport	92

4.1	Structure of this Chapter	92

4.2	Current Situation	92

4.3	Strategic Objectives	108

4.4	Operational Objectives	113

4.5	Cost-Benefit Analysis	136

4.6	MCA Analysis	180

4.7	Summary of Interventions	191

5	Rail Transport	195

5.1	Existing Situation and Historic Trends	195

5.2	Road Competition	218

5.3	Strategic objectives	221

5.4	Proposed Improvements	231

5.5	Description of the Proposals	232

5.6	Testing Rail Service Options	248

5.7	Summary of Interventions	348

6	Ports and Waterways	351

6.1	Existing Conditions	351

6.2	Strategic Objectives	387

6.3	Operational Objectives	388

6.4	Testing Interventions	388

6.5	Institutional Reform	435

6.6	Summary of Interventions	439

7	Aviation	446

7.1	Introduction	446

7.2	Existing Conditions	446

7.3	Strategic Objectives	464

7.4	Operational Objectives	464

7.5	Interventions	465

7.6	Results of Testing	491

8	Intermodal Transport	494

8.1	Existing Conditions	494

8.2	Operational Objectives	515

8.3	Interventions	516

8.4	Testing interventions	523

8.5	Summary of Interventions for Intermodal	544

9	Funding Analysis	549

9.1	Basic Assumptions	549

9.2	Mandated Expenditure for Maintenance and Renewals	552

9.3	Available Expenditure for Investment	553

10	Overall Strategy, 2020 and 2030	557

10.1	Introduction	557

10.2	Existing Situation	559

10.3	Overall Objectives	565

10.4	Interventions	567

10.5	Testing Projects	567

10.6	Project Appraisal – Role of CBA and MCA	569

10.7	ES and EES Scenarios	572

10.8	Funding Analysis	583

10.9	Public Consultation	586

10.10	Cross-Sectoral Issues	586

11	Appendices	598

11.1	Appendix A. GTMP Financial Plan	599

11.2	Appendix B. TEN-T Maps for Romania	600

11.3	Appendix D. Potential Electrification Schemes	602

11.4	Appendix E. List of Reference Case Projects	603

Do not delete the line below as you will remove the Bookmark TOA (Table of Appendices)

Do not delete the line below as you will remove the Bookmark TOT (Table of Tables)

Do not delete the line below as you will remove the Bookmark TOF (Table of Figures)

Do not delete the line below as you will remove the Bookmark TOP (Table of Photographs)

- Glossary and Terminology

| ACN | The Administration of Navigable Channels in Romania |
| --- | --- |
| AIS AFDJ | Automatic Information System to track ships The River Administration of the Lower Danube, located in Galati |
| APDF APDM | The Administration of the River Ports in Romania, located in Giurgiu The Administration of Maritime Danube Ports, located in Galati |
| Base Year | 2011, the year for which the National Transport Model was calibrated. |
| CESTRIN | The Centre of Research and Road Technical Studies in Romania, part of CNADNR |
| CFR Calatori | The publicly-owned passenger train operating company |
| CFR Infrastructure (also CFR SA) | The publicly-owned rail infrastructure company |
| CFR Marfa | The publicly-owned freight train operating company |
| CNADNR | The Administration of National Roads and Motorways in Romania |
| Desiro Train | A modern diesel multiple unit train |
| DMU | Diesel multiple unit: typically a 2 or 4 car set that can be coupled together and can be driven from either end. |
| ECR | AECOM Existing Conditions Report |
| EMU | Electric multiple unit |
| Euro RAP | European Road Assessment Programme, an international organisation dealing with road safety |
| Fairway | The navigable part of the River Danube |
| GDP | Gross Domestic Product |
| GJT | Generalised Journey Time. It includes waiting, access time and fares (converted to time equivalents) |
| GTMP | General Transport Master Plan |
| ILS | Instrument Landing System |
| Intermodal Transport | Transport that uses two or more modes, for example road and rail, or water and road. |
| Inter-Regio | Limited stop passenger rail services |
| NAPA | North Adriatic Ports Association |
| NTM | National Transport Model |
| Passenger kms | The aggregate distance travelled by passengers |
| PCN | Pavement Classification a measure of the loading capacity of runways and taxiways |
| PSC | Public Service Contract, the agreement between the government and rail operators to provide socially necessary rail services |
| Push-pull | A type of loco-hauled train operation where the train can be driven from either end |
| Push Tug | The type of tug used on the River Danube to push barges |
| Reference Case | The transport networks that would exist if existing committed projects were completed. It forms a reference against which “new” projects can be assessed. |
| Regio | Stopping passenger rail services |
| Tonne kms | The aggregate distance freight is carried |
| UNTRR | National Union of Road Hauliers from Romania |
| UTI | Unité du Transport Intermodal, a container or unit load |

Introduction

Introduction

The Ministry of Transport (MT) appointed AECOM in April 2012 to produce a General Transport Master Plan (GTMP) for Romania.

The General Transport Master Plan will provide a clear strategy for the development of Romania’s transport sector for the next 20 years.  To be of value it needs to provide implementable solutions to Romania’s transport problems and challenges.

The Master Plan identifies the projects and policies which best meet Romania's National transport needs over the next 5-20 years, for all modes of transport, and providing a sound, analytical basis for the choice of those policies and projects.

The completion of the Master Plan is conditionality for European Commission approval of the Strategic Operational Programme for Transport (SOPT) for the 2014-2020 programming period and will support other decisions required for the optimal planning of transport infrastructure investment.

The Master Plan has been developed following the advice of the European Commission, and in co-operation with the JASPERS unit in Bucharest.

A Transport Master Plan is not an end in itself.  The Master Plan must contribute to Romania’s economic development in a sustainable manner.  The high level outcomes that the Master Plan will produce are:

Outcome 1: A long term plan which will contribute to Romania’s national economy in a sustainable way.

The Plan’s duration will be 15 years, and the whole programme of projects will take longer than that to implement.  This is logical since large transport infrastructure projects typically take 5-10 years from inception to implementation, and their impacts last for 50+ years, although convention assumes that the economic life of transport projects is 30 years.  This approach also implies a consistent approach to transport policy over a long period of time, which transcends political expediencies.

Secondly, the primary purpose of the Plan is to define the projects and policies that will have an impact at a National level, and on the European TEN-T corridors.

Outcome 2: More efficient spending of financial resources on transport.

The key word here is “efficient”. Every country in the EU has a greater perceived need for improved transport investment than the financial resources available to meet that need, and this will not change in the next 15-20 years. Therefore, given the limited financial resources available, the emphasis must be on projects and policies that give a good economic return, and which perform a useful function.

Outcome 3: Improved connections and therefore improved trade with neighbouring countries.

The Plan recognises not only that Romania is part of the European Union, which at its heart is an Economic Union with free trade and fair competition between its members, but that it also has important markets (relatively undeveloped at the moment) to the Ukraine, and Moldova.

Outcome 4: Higher productivity for Romanian industry and services, and therefore higher economic growth and improved standards of living.

Efficient transport systems reduce costs for industry and individuals. For industry, this means lower costs and increased productivity, less resource tied up in inventories, and more competitive products and larger markets for those products.  For transport operators, better transport means lower costs and higher utilisation of vehicles and staff. For individuals, better transport saves time, and provides wider choice of work, consumer goods, and leisure opportunities.

The Cost-Benefit Analysis captures the majority of these productivity benefits.

Outcome 5: A sustainable transport system.

The word sustainable embraces more than environmental sustainability, although this is the context in which the word is often used.  It includes the concepts of economic, financial and operational sustainability as well as environmental sustainability.  The issue of financially sustainability is particularly relevant to the financing of the Romanian railways.

In summary, the Master Plan will identify the projects and policies which will best meet Romania's National transport needs over the next 5-15 years, for all modes of transport, and providing a sound, analytical basis for the choice of those policies and projects.

Methodology

Methodology

Overall Methodology

The overall process for developing the Master Plan is set out in Figure 2.1 below:

Figure 2.1  Overall Process for Developing the Romanian Transport Master Plan

Step 1: the Strategic Objectives are those which are defined at a Government, or Ministerial Level, and apply at a high level, as overall goals of the Government, and the Ministry of Transport. For the Master Plan, these were defined using the objectives from the Terms of Reference, various statements from the Ministry of Transport, and the European Commission’s White Paper on Transport.

Step 2: Problem Definition is the outcome of a diagnostic of the Transport System.  We have identified the underlying causes which are responsible for the manifestation of problems, as well as identifying the problems at a spatial level so that specific objectives and interventions can be identified.

Step 3: Operational Objectives: these are objectives that relate to the specific problems which have been identified, and are a subset of the Strategic Objectives.

Step 4: Project Generation: these are the specific interventions which will address the operational objectives, and the problems.

Step 5: Project Appraisal and Prioritisation: A systemised project appraisal process is required for two main reasons.  First, there may be more than one project which addresses an operational objective, so selection is required.  Secondly, a project may address the problem but may offer poor value for money.  In a situation like Romania’s, where the funds available for transport are much less than the needs, financial resources must be allocated in an economically efficient way.  A fair, independent way of appraising projects must be used for this purpose.  A multi-criteriate analysis (MCA) has been undertaken for this purpose.

Step 6: Develop Master Plan Scenarios: the Terms of Reference require that two scenarios are developed, an “Economically Sustainable” Scenario, and an Economically and Environmentally Sustainable” Scenario.  Each project was scored in the MCA according to how well it met the defined appraisal criteria. Using different weights for the scores, each projects was given two scores, applying to each scenario, which gave a different set of priority projects for each scenario

The Romanian National Transport Master Plan is, as its name implies, a National Plan. There is therefore an issue of scale in the projects, policies and programmes that the Master Plan will contain.  The high-level objectives will therefore be met by policies, programmes and projects of sufficient scale to which will make a difference at a National Level.  These include interventions such as:

Large infras``tructure projects

National Maintenance Programmes

New Rolling Stock and Locomotives

Large Scale Rehabilitation projects

National Policies such as Rail Reform

Objective setting

Establishing objectives is fundamental to the development of any strategy or project.  The objectives focus the appraisal and the outcome of the study. Furthermore, the objectives are central to the monitoring and evaluation required during the implementation stage.

The “strategic” objectives will provide clear and concise goals that the strategy will aim to deliver.  They encapsulate the underlying purpose of Transport Policy, Projects and Interventions, and represent the overall aims and objectives of the Ministry of Transport, and indeed the Romanian Government, as far as transport is concerned.

It is important to stress that the Master Plan is a long term Strategy for all of Romania, not just the areas of the country that are located on European corridors.  The determining factor for projects and policies in the Master Plan will be National need; clearly the availability of funding will be an important determinant of prioritisation and programming.

The concept of high-level and operational objectives, which are defined following the thorough assessment of problems, provides a hierarchy of objectives.  This structure clarifies the logic of the intervention and provides a framework for future appraisal and evaluation.  The appraisal process for the Master Plan contains a two level hierarchy which consists of:

High level or strategic objectives – For a strategy, this may be to aid economic development of the country or, at project level, to aid the development of the Trans-European Transport Network.  These are generally objectives to which transport contributes, but not always in a direct manner. Furthermore these objectives may already be predefined, for example in EU or national policy documents; and

Operational objectives – These are derived from the detailed examination of problems, and the underlying causes of these problems.  They are therefore specific to a corridor, route, or transport node (such as port or airport), and allow the interventions to be designed in a precise way to meet the objectives.

It is also important to note that setting objectives implies a commitment to follow them through in actions and projects.  There may be legitimate reasons for slower than desired or planned progress in implementation, but the underlying driver of transport projects in the Master Plan must be the achievement of the objectives.

Lastly, the Master Plan strategic objectives are not mode-specific: the actions and projects, for which we use the term “interventions” which follow from the objectives should, of course, relate to specific modes which are best-suited to achieve the objectives.

The High-Level Objectives

The documents relevant to setting the strategic objectives are as follows:

The objectives in the Terms of Reference (ToR)

Mission: Minister’s Statement in the Forward to the Strategic Plan of the Ministry of Transport and Infrastructure

EU White Paper on Transport 2011

Romanian Government Statement on Transport Policy (Program de Guvernare 2012-2016)

Partnership Agreement 2014 – 2020 (see pages 176-177)

AECOM Existing Conditions Report

National Spatial Plan Section 1 Transport Networks

EU Core Networks for Road and Rail

The key objectives for the Transport Master Plan, as set out in the Terms of Reference, relate to

Economic Efficiency: that is the transport sector must contribute to the national economy and the economic benefits it generates must exceed its costs;

Sustainability: the transport system should be energy efficient, and leave a legacy for future generations;

Safety: the transport system should be safe;

Economic Development: the transport system should facilitate national economic development;

Financial: the Master Plan should enable increased absorption of EU funds.

The Ministry of Transport’s Mission as set out in the Minister’s Forward to the Draft Strategic Plan for the Ministry emphasises the following points:

Economic Efficiency: a transport system that generates benefits that are greater than its costs;

Equity: the costs and benefits of the transport system should be distributed fairly among citizens, industries and geographic areas;

Safety: the transport infrastructure and services should be provided in a manner that protects people from death and injury;

Integration: the transport system should enable people to travel conveniently and reliably using a combination of different modes of transport, and to minimise the costs of transporting goods;

Environment: the transport system should protect the environment and by so doing should support social and economic development for the benefit of today’s and future generations.

Romania relies on the EU Cohesion Funds and Economic Regional Development Funds for the majority of its funding for new transport infrastructure, and the Terms of Reference recognise this.  Therefore, the Romanian transport objectives must pay due regard for to the current policies of the EU.

The Transport White Paper published in 2011 sets out in Annex I a list of initiatives, which are really actions, under several headings which may be regarded as objectives.  These are:

1.	An efficient and integrated Mobility System

  - A single European Transport Area

  - Promoting quality jobs and working conditions

  - Secure transport

  - Service quality and reliability

2.	Innovating for the Future: Technology and Behaviour

  - A European Transport and Research and Innovation Policy

  - Promoting more sustainable behaviour

  - Integrated urban mobility

3.	Modern Infrastructure and Smart Funding

- Transport Infrastructure: territorial cohesion and economic growth

- A coherent funding framework

- Getting prices right and avoiding distortions

4.	The External Dimension: this area refers predominantly to actions at an EU rather than a National level.

The 2014-2020 Partnership Agreement has the following Thematic Objectives (TO 7) with regard to transport, which are relevant to the Master Plan:

Improving the accessibility of Romania and its Regions and their connectivity with markets, thereby significantly reducing the obstacles to their development and diversification in the context of the GTMP; improving the governance of the transport sector; and

Improving the sustainability of Romania’s transport mix and the attractiveness of alternatives to road-based transport.

The AECOM Existing Conditions Report analyses the transport system by mode, but certain common themes emerge:

Economic: The level of service offered by all modes of transport is generally poor, and this means both freight and passenger transport is slow and inefficient.  The topography of Romania means that many main routes cross the Carpathian Mountains, but journey times by road and rail are slow, high capacity roads are few in number, and the rail network has provided progressively slower speeds and greater unreliability on unimproved routes.

Sustainable Modes: intermodal freight transport is very poorly developed, and road-based passenger transport competes rather than complements rail transport.  Rail freight and passenger travel has declined in recent years so reversing this trend will require a variety of interventions, some involving policy as well infrastructural.  The River Danube is a valuable resource for low-energy transport, but the river through Romanian section is not managed and there are many points where the depth frequently fall below the minimum desirable (2.5m) and the fairway is below the desirable width (180m.).

Environment: road transport of both passengers and freight has been increasing in recent years.  There is a conflict between desirable transport improvements from an emissions point of view but which have serious environmental impacts.  Thus there is often a conflict in Romania between the environmental objective, and the economic objective.  A classic example of this conflict is the River Danube, where the river banks are part of Natura 2000 sites, and dredging operations disturb the hydrology of the river and some fish and other river mammals.

Funding: all transport modes are under-funded, in terms of the infrastructure that the vehicles and services use, the maintenance of that infrastructure, and the services and vehicles themselves.  This leads to unnecessarily costly operations and a low-level of service to the end-users.

There are some common themes which emerge from these various sources.  These are:

Economic: the transport system should be economically efficient as far as transport operations and users themselves are concerned.  Specifically, the transport system benefits should exceed its costs.  In addition, the transport system should be configured to enable economic development both nationally and regionally.  The investment should also favour equity as far as Romanian citizens are concerned.

Environment: the transport system should not impact negatively on the environment.  Transport investment should minimise negative impact on the physical environment.

Sustainability: the so-called sustainable modes of transport which are more energy efficient and have lower emissions should be developed as a priority;

Safety: investment in transport should produce a safer transport system; and

Funding: there is a substantial shortfall in funding for transport in Romania.

Bearing in mind the common themes in the above documents, the following high-level objectives were established for the Master Plan strategy.

Economic Efficiency: the transport system should be economically efficient as far as transport operations and users themselves are concerned.  Specifically, the benefits of investments in transport should exceed the cost of that investment.  This objective measures the benefit to the users and providers of the transport system, and its quantitative measures are the Benefit:Cost Ratio (BCR), Net Present Value (NPV) and Economic Internal Rate of Return (EIRR).

Sustainability: this concept includes financial, economic, as well as  environmental sustainability.  The so-called sustainable modes of transport – rail, bus and waterways - which are more energy efficient and have lower emissions should be developed as a priority.  Monetary values are assigned to operating costs and emissions in the economic evaluation, but making Sustainability a separate objective respects not only the intentions of the Romanian Government and the EU, but the concerns of future generations.

Safety: investment in transport should produce a safer transport system.  The economic cost of accidents is monetised in the economic evaluation, but since the goals of the Government, the EU and the ToR are clearly a reduction in transport-related accidents, safety must remain as a separate objective.

Environmental Impact: Transport investment should minimise negative impact on the physical environment.  Agreed methods for monetising physical impacts are not yet available, and therefore the impact on the physical environment is not included in the economic evaluation.  Most infrastructure projects by their nature impact negatively on the natural environment, and therefore the objective is to select projects and options which minimise the extent of the negative impact.

Economic Development.  The transport system should be configured to enable economic development both nationally and regionally.  The investment should also favour equity as far as Romanian citizens are concerned.  Efficient transport is not an end in itself, but one important way of contributing to overall economic development.  A “pure” economic objective will tend to favour projects where demand is highest and where the existing economy is already strong (generally around Bucharest and the major cities).  This objective recognises first, that transport investment has a role to play in the development of less-advanced regions in economic terms, and secondly, that transport should provide services which are available to all citizens, regardless of social status, income or where they live.

Funding: there is a substantial shortfall in funding for transport in Romania.  Policies which produce more efficient pricing for transport, such as road-user charging, particularly for HGV, should be examined.  At a project level, availability of EC funding from the Structural Funds (CF and ERDF, Connecting Europe Facility (CEF)) and PPP will affect “buildability” and therefore the prioritisation of projects.  The overall programme will have to be within a realistic estimate of national and other funds over the plan period.

The High-Level Objectives for the Master Plan are summarised below:

Master Plan High-Level Objectives

Economic Efficiency: the transport system should be economically efficient as far as transport operations and users themselves are concerned.  Specifically, the benefits of investments in transport should exceed the cost of that investment.

Sustainability: the transport system must economically, financially and environmentally sustainable.  The so-called sustainable modes of transport – rail, bus and waterways - which are more energy efficient and have lower emissions should be developed as a priority.

Safety: investment in transport should produce a safer transport system.  The economic cost of accidents is monetised in the economic evaluation, but since the goals of the Government, the EU and the ToR are clearly a reduction in transport-related accidents, safety must remain as a separate objective.

Environmental Impact: Transport investment should minimise negative impact on the physical environment.

Balanced Economic Development.  The transport system should be configured to enable economic development both nationally and regionally.  The investment should also favour equity as far as Romanian citizens are concerned.

Funding:  Availability of EC funding from the Structural Funds (CF and ERDF, Connecting Europe Facility (CEF)) and PPP will affect “buildability” and therefore the prioritisation of projects.  The overall programme will have to be within a realistic estimate of national and other funds over the plan period.

Operational Objectives

Operational objectives are a subset of the high-level objectives.  They are derived from the problem analysis and, unlike the high-level objectives are specific to each mode, and, in many cases, are also location-specific.  They enable the interventions to be designed in such a way to meet these objectives.  This is best illustrated by specific examples.

For the rail sector, a particular issue is the lack of a regular interval train service on the main routes, which, combined with slow travel times, creates inconvenience for passengers and contributes to the decline in passenger numbers.  An operational objective which corresponds to this problem would be:

Improve the competitiveness of passengers rail services on the route between Bucharest and Hungary via Brasov, Teius / Cluj.

The specific problem which was identified on this corridor was:

Infrequent services: headway 180 minutes

Slow average speed

Slow rail travel times compared to road.

The specific interventions for this route are:

Introduction of regular interval services with trains every 2 hours

Infrastructure upgrade, particularly between Predeal and Brasov. Review the potential options to upgrade different proportions of the line to 160kph. Other infrastructure schemes include Dej to Coslariu via Apahide and F039 Brasov to Simeria (assumes Brasov to Sigishoara is upgraded to 160kph)

Introduction of tilting trains to help increase running speeds

The problem, operational objective and interventions relate directly to the Economic Efficiency, and Sustainability high-level objectives, and indirectly to the safety objective since some passengers would transfer to rail, which is a safer mode.

For the roads sector, road safety is a major issue - Romania has 259 fatalities per 10 billion pkm (against an EU average of 61) and 466 fatalities per million passenger cars (against an EU average of 126) – and it is the worst performing EU country for both indices.  The corresponding operational objective is:

Decrease by half the accident rates by 2020 and down to EU average by 2030.

In fact, in this case there are seven further operational objectives designed to meet this target, each with an appropriate intervention.  These come under the “safety” high-level objective.

The above outputs from the Problem Definition Report provided the essential "operational objectives" which guide Project Identification Phase.  These were brought together in working documents (the Problems/Objectives/Interventions (POI) reports), which set out in detail the problems, objectives and interventions in a logical and consistent manner.  These were agreed with the MT and JASPERS.

Environmental Objectives

The general environmental objective of the GTMP

OM1: The development of a modern transport infrastructure, considering the environmental impact.

Specific environmental objectives of the GTMP

OM1-1 Promoting projects with investments in transport that contribute in achieving a sustainable transport system, with measures to avoid and reduce adverse effects, such as emissions of pollutants into the atmosphere, noise pollution in urban areas and on roads with heavy traffic, water and soil pollution due to diffuse sources, the impact on landscape and cultural heritage;

OM1-2 Reduction of greenhouse gas emissions from the transport sector;

OM1-3 Protection of public health by improving environmental and safety conditions of transport;

OM1-4 Reducing the impact on biodiversity by providing measures to protect and conserve biodiversity and ensure consistency of the national network of natural protected areas.

Identifying the Problems and Defining the Interventions

The Problem Identification stage in the development of any plan or strategy is a key procedure, as it identifies and confirms the underlying problems of the transport system rather than merely describing the symptoms. Problem identification also provides a basis for developing operational objectives which in turn form a framework for the appraisal of measures for improving the current transport system.

This step in the process is designed to provide an understanding of the need for a transport intervention and to provide strong input into the setting of objectives through identifying existing and potential transport problems, opportunities and constraints.  AECOM’s Existing Conditions Report describes the current challenges on a modal basis.  There is an existing identified need for improvements to transport infrastructure and services, and there is a large “backlog” of projects already identified by project sponsors.  This particularly applies to maintenance of the road and rail networks.

It is crucial that the causes of the problems are investigated before solutions are generated.  Focusing on problems (rather than underlying causes) as the stimulus for option development may result in solutions which address the symptoms without solving the real underlying problems.

Figure 2.1 The Approach used for the Problem Identification

Several sources of information have been used to support problem analysis, including:

Statistical data on current network operations;

Modelling of current transport network performance;

Forecasting of future year transport demand and network performance; and

Consultations with key stakeholders.

The National Transport Model (NTM) provided a core component of current problem analysis, and provides forecasts for the future year “Reference Case” scenario, enabling analysis of the future year transport networks to be undertaken, identifying which problems may be exacerbated in the future.

The NTM contains a representation of the transport system, in the supply side in the form of the networks, capacities and services, and the demand side, in terms of travel between origins and destinations for each mode. The outputs are flows on each link in the network, together with statistics such as passenger and vehicle kms, freight tonnes kms, and travel times and costs by mode.

Consultations with key stakeholders have provided key information covering current transport policies and operational issues. The key stakeholders consulted include:

CFR Infrastructure

CFR Calatori

CFR Marfa

MT Railways Directorate

CNADNR

MT Naval Directorate,

the Civil Aviation Authority,

the Ports and River Authorities, including APDM Galați, APDF Giurgiu, ACN Constanța și APM Constanța,

TAROM,

Henri Coanda Airport management,

private companies such as DHL, and

representative bodies such as UNTRR.

Full details of the review of existing conditions are provided in the Romania GTMP Existing Conditions Report (ECR) and problem identification process in the Problem Definition Report (PDR).

The appraisal of a transport intervention involves the comparison of the ‘with intervention’ situation against the situation which would be obtained without the intervention in place.  The ‘without intervention’ scenario needs careful consideration and will involve specifying a Reference Scenario which has a very high probability of occurring.  This is very important as it will affect both the identification of the need for the intervention and the assessment of the costs and benefits of the proposal.

The Reference Scenario provides a realistic view of what is likely to happen in the absence of the intervention proposals.  It is based on the continuation of existing maintenance regimes plus any transport improvement commitments that have policy and funding approval and from which it would be difficult to withdraw.  It corresponds to maintaining present transport facilities and implementing those aspects of national and county transport strategies that are certain.  It takes into account forecast changes in demographics (population, employment and households) and car ownership factors, from European and national datasets, together with changes in land use.

Problems and Opportunities

The identification of transport problems, constraints and opportunities which affect an area and its aspirations for the future, ensures transport interventions are forward-facing and not simply reacting to current issues.  Thus, both the transport problems affecting an area and the aspirations for the future – which are often broader than transport – must be the drivers of the proposals for a transport intervention.

The Master Plan is intended for the development and appraisal of proposals which either contribute to objectives relating to transport, or where the underlying opportunities are transport opportunities.  This is because if transport proposals are being considered to help meet an objective that could be met by other means (rather than transport), poor decisions could easily result.

Current and Future Transport Related Problems

Problems were identified in a number of ways, including:

Perceptions of the problems from users, both those that they encounter when travelling and those which result from other people travelling;

Through discussions with representatives of stakeholders to gain an understanding of the transport and planning professional’s perceptions of problems with the transport system;

Conducting audits of specific elements of the transport system in order to gain a deeper understanding of the roles performed and to analyse the extent to which the expected aims are not met;

Analysing outputs from the National Transport Model, or analysing existing data sets, to determine the extent to which local, county and national transport and wider policy objectives are being met; and

Benchmarking the local performance against similar situations elsewhere.

Future problems were analysed from the future travel demands and changes in the transport supply in the Reference Scenario.

Problem Definition Report

The formal Problem Definition Report (PDR) identified the problems for each sector of the transport system, supported by an analysis of the performance of each sector and taking into account consultation with operators and providers, and users of the transport system.

The PDR is built up from the following processes:

Analysis of Existing Transport System containing a review of available data, new data collected, existing analyses;

Use of the National Transport Model to provide data for base year and future year Reference scenario to provide an evidence base to support the problem identification process;

The results of consultation with key stakeholders in order to identify the root cause of problems; and

Concise statements of the problems identified, following consolidation of all sources of evidence, which will provide an evidence-based identification of the real problems and challenges facing the Romanian Transport Sector.

The above outputs from the PDR provided the essential  "operational objectives" which guide Project Identification Phase.  These were brought together in working documents (the Problems/Objectives/Interventions (POI) reports) which were agreed with the MT and JASPERS.

The National Transport Model

Overview

A transport model is a computer-based representation of the movement of people, goods (trips), and vehicles around the transport system. It is intended to provide an indication of how travel behaviour, travel patterns and demands will respond, over time, to changes in policy, infrastructure or services.

A major benefit of using a transport model is that it ensures that schemes, or scheme options, are considered on a consistent basis both within and between studies. An objective of these guidelines is to ensure that all scheme evaluations follow the principles discussed herein and therefore enable the assessing organisation to compare schemes on a like for like basis.

There are certain terms and concepts which have specific meanings when used in transport modelling.  These are:

Base Year: this is the year for which a Model is calibrated (or fitted to the data).  It is the most recent year for which good, accurate travel and demographic data (relevant to the project) is available.  The Base Year of the National Model is 2011.

Forecast years: these are the years for which travel forecasts are made by the model.  At least two forecast years should be modelled: the assumed opening year of the project which is being analysed, and 15 years after that.  This is because first, typically projects design years are normally 15 years after opening, and secondly, the economic evaluation of a project assumes a 30 year “life” for the project.  Forecasting for 15 years after the opening year is an accepted compromise between the uncertainty of long term forecasts, and excessive extrapolation of the economic evaluation.  The forecast years for the National Model are 2015, 2020, 2030 and 2040.

Zones: in transport models, the area in which the project is located is divided into geographical areas called zones.  Within the model, traffic is assumed to enter and leave the transport networks at these zones.  Of course, in reality traffic enters and leaves the transport system at individual addresses, and businesses, but it is impossible to obtain the necessary detail of each individual journey, so trips are modelled as zone to zone movements.  In other words, zones represent the origins and destinations of trips.  In the National Model there are 1160 zones within Romania, and 150 zones outside of Romania.

Modes: in transport modelling, “modes” is shorthand for the modes of transport people and goods use to travel between zones.  These can include “non-mechanised” modes of transport such as walking and cycling, and “mechanised” modes such as private car, bus, tram, train, metro, air and waterways and sea transport.  “Single mode” models represent travel by one mode only, and “multi-modal” models represent travel by more than one mode.  The National Model is a multi-modal model, with all the mechanised modes of travel included, and intermodal freight transport as well.

Networks: In transport models, networks are a computerised representation of the transportation networks.  In most models these include the road network, and the public transport networks: bus, tram, metro and heavy rail networks.  The public transport networks also include details of routes, frequencies and fares.

Trip Matrices: these are tables of trips between zones.  They may contain numbers of vehicles, or the number of persons, for a particular journey purpose, or for a particular time of day, or for a particular mode.  These are commonly referred to as Origin – Destination (“O-D”) matrices.

Assignment: During Assignment, the model calculates the least cost route through the network for each O-D pair in the trip matrices, and accumulates the flows on each link in the netowork.  Between most origins and destinations there is more than one possible route, so the model repeats this procedure many times until the network is in equilibrium.  Each calculation of the paths is called an iteration.  The National Model has a total of 1310 zones so the model calculates 1.7m different paths in every iteration.

Calibration and Validation: Calibration is the process of fitting the model and its relationship to the data, while validation is the process of comparing model outputs with independent data.  There are internationally-recognised standards for assessing the “goodness-of-fit” for models, depending on the size and scope of the model in question.

Modelling Software.  Simple transport models can be built using EXCEL, but there are limitations using this approach.  For most modelling applications, there are bespoke packages available and we recommend that these should be used.  The National Model is built using EMME, which is a well-known and widely-used suite, which has the necessary flexibility to meet the required scope.

The basic structure of the model is shown on Figure 2.3 below:

Figure 2.3 Basic Structure of National Transport Model

Further details of these processes are given in subsequent sections.

Specific interventions that the National Transport Model (NTM) is capable of modelling include:

The impact of economic (GDP, income, car ownership) and social (population levels and distribution) changes on travel demand;

Infrastructure changes;

New PT services;

Policy measures including:

Differential pricing for rail and air;

Internalisation of external transport costs; and

Climate change policies (subsidy of low emission modes)

Implementation of road tax; and

Car ownership and its linkage to level of taxation

NTM modelled demands are responsive to changes in costs and time for all aspects of travel.  The most common interventions in Romania for the Master Plan are improvements to infrastructure, such as motorways and expressways, re-habilitation of railways, more frequent train services, and improved freight terminals at ports and intermodal centres.  But the model is also designed to evaluate “policy” interventions, including road user charges, fuel duty increases, or the additional costs of car ownership through registration/tax measures. For example road user charges for heavy vehicles will lead to higher operating costs and this will result in shorter distance HGV trips, transfer to other modes, particularly rail, or suppression of some trips altogether. The developed model structure includes all of these responses. Increasing registration or tax fees on car ownership will moderate car ownership growth through the responses in the car ownership model.

Study Area

The NTM includes:

Journeys made wholly within Romania, mainly inter-urban movements. The NTM does not examine urban travel patterns and demands in detail and the data collection and model development was structured accordingly ;

International journeys with either an origin or destination in Romania; and

International journeys with both their origin and destination outside Romania

A key requirement was to understand the inter-urban and international travel patterns, recognising at the same time that local travel congestion affects parts of the strategic network.  Therefore, the study needed good quality data for inter-urban travel, with local urban area travel data deemed less important. These requirements applied equally to the passenger transport and freight markets. However, it is important to recognise that there are important differences between them, particularly in relation to the choice of transport modes.

The NTM, therefore, provides a detailed representation of all transport modes in Romania and their connectivity to the wider European TEN-T transport corridors by mode. The main focus of the model is on inter-urban travel and as such the treatment of urban travel is at a simplified level of detail.

Zoning System

The zoning structure is at a level of spatial detail that meets the objectives of the NTM.  The principles adopted in developing the zone system were to ensure that:

adequate spatial detail was provided in terms of access to the extensive rail system;

zones contained no more than one main town as far as practical;

zones outside of the main cities and towns were limited in area so that very large zones were avoided, except in the mountains, where the average population per zone was kept below 15,000;

zones followed natural boundaries;

zonal boundaries were aggregations of administrative boundaries so that compatibility with planning inputs, and socio-economic datasets could be guaranteed;

access to the highway network as properly represented in terms of loading points on key roads;

account was taken of special land uses such as Constanta Port; and

the zonal structure took account of future development proposals.

Figure 2.4 shows the internal zone system and the model is populated with zonal data on population, jobs, active population, GDP by sector, and car ownership all of which are used in the model forecasting routines.

Figure 2.4 NTM Internal Zone System

County

Zones

County

Zones

This is supplemented by the external zones being specified at the relevant NUTS level as indicated in the study brief. The external zone detail is compatible with Trans-Tool zoning through either aggregation, or disaggregation, of the Trans-Tool zones. Figure 2.5 shows the external zone system.

Figure 2.5 NTM External Zone system

NTM Model Segmentation and Transport Networks

The NTM considers demand at the following levels of segmentation:

Passenger Demand (Domestic and International) by:

Car availability (CA/NCA);

Trip purpose: Business, Commuting, Personal and Vacation/Leisure; and

Mode of travel: Car, Rail, Bus, Air and Ferry where available

Freight demand by:

Commodity (16 separate commodities);

Container and general freight; and

Mode: Road, Rail, Water and air for international movements

The NTM includes representations of the following transport networks for carriage of passengers and freight:

Highway network – passenger cars, passenger bus services, light and heavy goods vehicles;

Rail network – rail passenger services (region, interregio and intercity) and rail freight services;

Air network – air passenger services and air freight services;

Waterway network – freight; and

Intermodal facilities.

The NTM highway network, Figure 2.6, contains all motorways and National Roads in Romania, the more heavily used county roads and some local roads to ensure connectivity from zones to the highway network. The rail network, Figure 2.7, includes all stations and rail links on which passenger rail services operate. Finally the waterway network, Figure 2.8, is comprised of the Danube and the channels between Constanta and the Danube, and all the ports along the Danube both in Romania and other countries through which it travels.

Figure 2.6 NTM Internal Highway Network

Figure 2.7 NTM Internal Rail Network

Legend

Stations

Rail lines

Legend

Stations

Rail lines

Figure 2.8 NTM Waterway Network

Legend

Waterways

Legend

Waterways

The NTM also includes a representation of air travel through a network comprised of all airports in Romania and links representing all flights from each airport in Romania to all destinations served by direct flights.

NTM Structure

Figures 2.9 and 2.10 show the main components of the model. These are the domestic and international passenger demand model, and the freight demand model (domestic/international). The domestic and international passenger models use the same structure but the coefficients for the demand models are different and reflect the differing characteristics of the travellers making each type of trip.

Figure 2.9 Passenger Model Structure

Generation

Mode Choice

Car / Rail / Air / Bus / Ferry

Domestic Passenger Demands

Car person trips converted Vehicle Trips

Highway Network

PT Network & Lines

Initial Highway

&  PT Cost  Skims

Passenger trips

PT Assignments

Time Period Factors

24 hr – Period

Highway Assignments

Highway Costs

PT C Costs

Distribution

Generation

Mode Choice

Car / Rail / Air / Bus / Ferry

Domestic Passenger Demands

Car person trips converted Vehicle Trips

Highway Network

PT Network & Lines

Initial Highway

&  PT Cost  Skims

Passenger trips

PT Assignments

Time Period Factors

24 hr – Period

Highway Assignments

Highway Costs

PT C Costs

Distribution

The main modules contained in the above passenger model structure are:

Highway and public transport network definitions:

Road(car, bus, and freight);

Interurban bus and minibus services;

Rail network, intermodal terminals, and rail services;

Waterways; and

Air

Domestic passenger demand model

Car ownership model as a function of income growth and car ownership costs;

Trip generation/attraction growth based on demographic (population and employment) and socio-economic changes (GDP);

Distribution – calibrated functions by trip purpose including responsiveness to cost changes ; and

Mode choice

International passenger demand model

Direct demand model based on GDP, population and car ownership changes;

Distribution based on changes in trip ends and generalised costs; and

Mode choice

Figure 2.10 Domestic and International Freight Model

The main modules contained the above freight model structure are:

Highway and public transport network definitions:

Road(freight);

Rail network, intermodal terminals, and rail services;

Waterways; and

Air

Freight model for international and domestic freight demand by commodity in tonnes and vehicle movements

Growth factors based on GDP for international and for domestic combination of GDP and population changes;

Distribution;

Mode choice and intermodal modelling; and

Conversion from tonne to vehicles for highway assignment (LGV/HGV)

Network assignment models and generalised cost derivation

Demand split into four time periods (AM peak, Off-peak, PM Peak, night time); and

Generalised costs combined across time periods, modes and destinations for use in demand models

Trip Generation

The NTM contains a set of calibrated base year demands that are used as the basis from which to derive future year forecasts of travel demand by mode based on changes in the main drivers of travel demand. The main drivers of demand used in the model are population, car ownership, employment, and GDP by sector. Separate approaches are taken to the development of future trip ends depending on the nature of the movement being modelled.

Changes in travel demand are typically driven by changes in the socio-economic indicators of the population making the trips. These include indicators related to the size of the potential trip-making group, for example changes in active population dictates the number of commuting trips, and changes in the  level of economic activity, given by GDP, impact on the number of freight trips made. Indicators related to the wealth of the trip makers, such as GDP/head, increase rates of trip making as people have greater disposal income and increase their rates of car ownership.

A series of economic factors for Romania and other key countries and regions are used in the model to determine the growth in traffic demand between base and future years. The table below provides a summary on the economic factors used by the model.

Table 2.1 Economic Factors Required by NTM Growth Model

| Factor / Geography | Romania (at regional level) | Other key countries & regions (at national level) |
| --- | --- | --- |
| GDP growth |  |  |
| GDP by Economic Activity growth |  |  |
| Total Population growth |  |  |
| Economically Active Population growth |  |  |
| Employment growth |  |  |
| Car Ownership growth |  |  |

Future year forecasts for each of the factors identified in the table above have been derived with the exception of car ownership, which is derived from changes in GDP through a car ownership model calibrated to fit historic ownership trends.

Trip Distribution

The NTM trip distribution model links the estimated trips produced from each zone with trips attracted to other zones. The model considers the balance between the location of these trip ends and the cost of travel (generalised in terms of time, distance, parking charges, and fares) between them, for all locations within Romania, to/from Romania and through Romania.

The number of trips between two zones is estimated to be directly proportional to the number of trip productions in the production zone and attractions in the attraction zone, and inversely proportional to the cost of travel between the zones. The output of the distribution model is a set of travel demand matrices that detail the number of trips from each zone to all other zones for various trip purposes for an average weekday.

The trip distribution models in NTM have the following form:

Tij = Ai * Pi * Bj * Dj * IMPij

Where:

Tij = predicted trips;

Pi = production total;

Dj = attraction total;

IMPij is impedance between production and attraction zone pair; and

Ai and Bj are adjustment factors.

The impedance functions calibrated in NTM are specific to each purpose and car availability type combination. Therefore a different set of x0, x1 and x2 parameters has been derived for each demand segment within NTM.

Mode Choice Models

Each of the demand models in the study adopts a hierarchical model formulation for the mode choice but in each case, there are specific differences that reflect the domestic and international passenger and freight markets. Figure 1.8 shows the structure of the car available mode choice model as an example of the hierarchical form.

Figure 2.11 NTM Mode Choice Model Hierarchy

Car Available  Mode Choice

Total CA

Car

PT

Air

Rail

Ferry

Bus

Regional

Inter City

Inter Region

Non-Tolled

Tolled

Car Available  Mode Choice

Total CA

Car

PT

Air

Rail

Ferry

Bus

Regional

Inter City

Inter Region

Non-Tolled

Tolled

Highway Assignment

The highway assignments in NTM are undertaken using a link based capacity restraint process with the speed/flow relationships reflecting the effect of volume, HGV composition, road type, lanes, frontage type, gradient, and road condition on achievable travel speeds. Three highway assignments are undertaken to represent peak, inter-peak and night time travel so that appropriate costs by time of day are generated for use in the model. The three time period assignments are aggregated to create 24 hour flows on the highway network.

The key difference between the assignment periods is the magnitude and pattern of travel demand and trip purpose composition. However, there are also variations in network restrictions, e.g. truck bans for certain times of the day. Public transport service patterns and frequency may also differ by time of day. These differences are applicable both in the base year and in future years.

The transport networks represented in the NTM include:

Road – including all expressways and National  roads, plus the main county roads in traffic terms along with other county roads required for zone connectivity;

All rail routes and services;

Long distance bus services;

Airports and internal domestic air services; and

Waterways including River Danube and the existing, and proposed linkages to Constanta Port.

Public Transport Assignment

Public transport assignments are undertaken separately by mode of travel. The rail assignment is done in three parts to represent the three main train service types in Romania, each of which has a different fare scale. Separate assignments are then undertaken for inter urban bus passengers and air passengers.

The public transport system is described in terms of the following components:

The physical network:

Nodes;

Links; and

Interchanges.

The public transport services:

Public transport modes;

Representation of public transport routes (time table information);

Routeing;

Service frequency;

Dwell time; and

Stopping pattern

Public transport fares.

Bus stops, and rail stations, are represented as nodes in the transport network. In the rail network the station nodes are coded with a boarding time factor in minutes which in the base case is taken to represent an element of rail reliability (average train delay), preference for using the station (ease of interchange), and station quality and range of services. In testing the effect of rail proposals the base year boarding penalty can be adjusted to reflect:

Improved reliability which is modelled by reducing the element of the boarding penalty that relates to average lateness; and/or

Improvements to a station which result in rail becoming a more attractive option and which can be modelled by reducing the element of the boarding penalty that reflects station attractiveness.

Rail stations are separately identified and are connected to the adjacent road network by a walk link. All rail stations in Romania have been included in the rail network model definition.

Airports are defined as separate nodes that link to the network of air routes and also to the highway network to enable representation of access by car/taxi or bus, and the rail network where a rail exists to the airport.

Ports on the River Danube are individually represented and have links to the adjacent highway network as well as the waterway network.

NTM Capabilities

This section summarises the type of transport operations and specific interventions that the NTM is capable of representing, which are set out in Table 2.2. Where there are areas that cannot be fully modelled in the NTM, such as new/modernised airports, and passenger ferry services separate bespoke demand models have been constructed.

- Table 2.2 NTM Capabilities

| #### Intervention |  | #### Treatment in Model |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Modelled | Frequency | Distribution | Mode Choice | Assignment |
| Road Improvements | New Infrastructure | Yes | * | ** | ** | **** |
|  | Change in speed limits | Yes | * | * | * | *** |
|  | Heavy vehicle restrictions | Yes |  |  | * | **** |
|  | Road User Charges | Yes | * | ** | ** | **** |
|  | Road Closures | Yes |  |  | * | **** |
|  | Rehabilitation | Yes | * | * | * | ** |
|  | Driver Information Systems | No |  |  |  |  |
| Railway Projects | New Lines, conventional and high speed. | Yes | * | ** | *** | ** |
|  | Electrification | Yes | * | * | *** | ** |
|  | Improvements to Existing Lines, including reconstruction | Yes |  | * | *** | ** |
|  | New services, higher frequencies and new connections | Yes |  | * | ** | * |
|  | Changes in Fares | Yes | ** | ** | **** | ** |
|  | New Rolling Stock: carriages | Yes |  |  | ** |  |
|  | New Rolling Stock: Locomotives | Yes |  |  | ** |  |
|  | Rehabilitation of Bridges and Tunnels | Yes |  | * | ** | ** |
|  | Signalling systems / Advanced Telematics | No |  |  |  |  |
|  | Modernisation of stations | Yes | * |  | *** |  |
| Improvements to Inter-City Bus Services | Improvements to roads which will affect existing Services | Yes | * | ** | *** | * |
|  | New services, higher frequencies and new connections | Yes | * | ** | *** | * |
|  | Changes in Fares | Yes | * | ** | *** |  |
|  | New buses | Yes |  |  | ** |  |
|  | New bus stations | Yes |  |  | ** |  |
|  | Modernisation of existing bus stations | Yes |  |  | ** |  |
|  | Integration of bus and rail services | Yes |  | * | *** | ** |
| Air Transport | New services | Yes | ** | ** | *** |  |
|  | Changes in Fares | Yes | ** | ** | *** |  |
|  | New aeroplanes / Airport Modernisation / Air Traffic Control Systems | No |  |  |  |  |
|  | Integration of internal air and rail services | Yes |  | * | *** | * |
| Freight Transport | Improvements to existing roads, and new roads. | Yes |  |  | ** | **** |
|  | Changes in Rates, including infrastructure charges on CFR | Yes |  |  | *** |  |
|  | Road User Charges for HGV | Yes |  |  | *** | **** |
|  | Restrictions on HGV movements, e.g. at weekends, or weight limits | Yes |  |  | * | **** |
|  | Improvements to the River Danube and Channels | Yes |  |  | *** |  |
|  | Modernisation of Danube ports | Yes |  |  | ** |  |
| Intermodal Transport | New or improved intermodal terminals, including logistics centres | Yes |  |  | *** | ** |
|  | New container services by rail | Yes |  |  | *** |  |
|  | New or improved rail stations | Yes |  |  | *** | * |
|  | New or improved bus stations | Yes |  |  | *** | * |
|  | Integration of bus and rail | Yes | * | * | *** | * |

Dimensions of the NTM

The main physical dimensions of the NTM transport system base year representations are shown in Table 2.3. The NTM contains a detailed representation of the inter-urban transport networks and the services that operate on it.

Table 2.3 NTM Base Year Model Dimensions (Internal Network)

| Variable | Mode | Total |  |
| --- | --- | --- | --- |
| Zones | All | 1,169 (134 external) |  |
|  |  | Nodes | Links |
| Transport Networks | Road | 9,430 | 20,882 |
|  | Rail | 2,466 | 5,028 |
|  | Water | 66 | 132 |
|  | Air | 15 | 30 |
| Transport Services (All) | Rail | 2,240 |  |
|  | Bus | 4,155 |  |
|  | Air | 469 |  |

- Source: AECOM NTM

Base Year – Overall Indicators

The base year passenger travel demands in the NTM are derived from comprehensive surveys undertaken with car, bus, rail and air passengers. This data was combined with ticket sales data for rail and air movements to produce passenger travel demands by mode as shown in Figure 3.12. Over  three million inter-urban passenger trips were made on an average day in the  base year.

- Source: AECOM NTM

Figure 2.12 NTM Base Passenger Trips

The passenger mode shares in the base year are shown in Figure 3.13 for car available and non-car available travellers respectively. As expected only a relatively small proportion of car available travellers opt to use public transport, 10%. In the non-car available market the main mode is bus, 77% , followed by rail, 20%. Overall 77% of passenger travel is by car, 18% by bus, and only 5% by rail and air.

- Source: AECOM NTM

Figure 2.13 NTM Base Year Passenger Mode Shares

The base year freight demands in the NTM are derived from comprehensive surveys undertaken with road hauliers, individual consignment data for rail freight, and port and airport details of freight handled by commodity. The split of freight traffic between the main modes is as follows:

Road freight tonne kms		53.3%

Rail freight tonne kms			24.2%

Waterways tonne kms			22.5%

Source: INS

NTM Reference Case Scenario (2020)

Travel Demand Growth

The NTM uses economic and demographic factors for Romania and neighbouring countries, together with the transport network structure and conditions as inputs to its forecasts. The main drivers of forecast demand are GDP, population, employment, economically active population, and car ownership.

Between 2011 and 2020 the current predicted change in the drivers of travel demand is:

GDP increases by 26.4%;

Population decreases by 1.8%;

Employment increases by 3.6%;

Economically active population decreases by -3.4%; and

Car ownership increases by 29%.

Source: National Committee of Prognosis, EIU, IMF, OECD

The NTM model outputs are the absolute travel demands, and changes between the base situation and future years. Figure 3.14 shows the forecast changes in passenger and freight demands by mode between the model base year 2011 and the reference case in 2020, both in terms of trips made and kilometres travelled/transported.

- Source: AECOM NTM

Figure 2.14 Change in Passenger and Freight Demand 2011 – 2020 Reference Case

The total increase in daily passenger trips in the 2020 Reference Case is forecast to be 10.7% with a 26.6% increase in total passenger km. Freight demands experience similar patterns of growth with tonnes up by 26% and tonne km by 34%.

Car ownership levels in Romania are still at relatively low levels which means that car ownership is forecast to grow strongly, 19.3% of population have a car available in 2011 rising to 24.9% by 2020. As car ownership rises, the proportion of the population that is “captive” to public transport declines. This reduction in the captive market, changes the dynamics of the public transport sector, with an increasing need to compete with road for people who now have a car available.

- Source: AECOM NTM

Figure 2.15  Car Ownership Growth, 2011 - 2020

The 2020 Reference Scenario contains a number of committed highway schemes, but in comparison relatively little investment in rail. The 20.9% reduction in rail trips is mainly driven by the growth in car ownership levels, and committed investment in the highway network, which makes car travel more attractive. Bus travel is forecast to increase slightly by 1.2%. Like rail, the bus sector will be adversely affected by the growth in car ownership, but will benefit from the highway investment projects which provide faster journey times. Car trips are forecast to increase by 15.1%, reflecting of the increase in car ownership, and the committed investments in the highway network.

The forecast increase in total passenger km (of 26.6%) is due to a number of factors, including:

Significantly increased speeds on a number of corridors due to the almost doubling of the motorway network by 2020 (from 550kms to 993kms), which encourages greater interaction between major cities and leads to an increase in journey distances; and

Growth in household incomes of 29% and therefore lower cost of operating a car, which leads to a propensity for individuals to make longer distance trips.

Passenger and Freight Flows

The following sections provide an overview of the situation in 2011 and 2020 with respect to the passenger and freight flows on the road, rail, and waterways networks.

The current dominant position of road transport for passenger trips is clear to see in 2011 with significant rail shares only on the radial routes from Bucharest.

- Source: AECOM NTM

Figure 2.16 Daily Passenger Trip Volumes in Romania, by Mode, 2011

By 2020 the contribution made by rail to passenger trip volumes has declined further and passenger trip volumes are dominated by road trips.  This is the forecast situation without the interventions proposed by the Master Plan, with only the current committed projects in place.  Essentially it rerpresents a “business-as-usual” scenario.

- Source: AECOM NTM

Figure 2.17 Daily Passenger Trip Volumes in Romania, by Mode, 2020

There is a much larger share of non-road freight than non-road passenger movements. The main freight movements by road are into Bucharest, and by rail into Constanta port. The role of the River Danube as a freight artery is clearly illustrated. Corridor IV (N) is an important road corridor, as well as flows between industrial centres as Craiova and Pitesti. There is also a strong road freight movement in the Pitesti-Ploiesti-Buzau-Bacau corridor.

The major cross-border flows are to Bulgaria at Giurgiu and the Hungarian border. For rail, the border crossing with the Ukraine at Siret and Moldova at Ungheni are also of some importance. The largest cross border flows in terms of tonnes is on the Danube at Iron Gates.

- Source: AECOM NTM

Figure 2.18 Daily Freight Volumes (Tonnes) in Romania by Mode, 2011

By 2020, the increase in freight tonne km is more closely aligned with the change in tonnes carried than was observed for passenger growth. This is because reducing the journey time and cost of freight transport will not, in itself, necessarily lead to longer journeys. For freight the average distance moved is also dependant on changes in the distribution of economic activities, that is where goods are produced, or imported, and consumed or exported.

- Source: AECOM NTM

Figure 2.19 Daily Freight Volume (Tonnes) in Romania by Mode, 2020

This section has presented an overview of the National Model with some key statistics.  The Model, in conjunction with the CBA tool, was subsequently used to test, and evaluate, a large number of potential interventions.  These tests and the results are described in Chapters 5-10 of this Report.

Cost-Benefit Analysis

Cost Benefit Analysis (CBA) is a standard method used to assess the performance of a project or intervention in terms of measuring the return on investment.

The approach that should be adopted in undertaking CBA is set out in the National Assessment Guidelines for Transport Projects, Vol 2 Part C developed as part of the Masterplan study.  The guidance has  been developed to meet the requirements set out in the ‘General Guidelines for Cost Benefit Analysis of Projects to be supported by the Structural Instruments’ published by the Ministry of Economy and Finance, Authority for Coordination of Structural Instruments.

The following documents have also informed the advice contained in this guidance:

- Developing Harmonised European Approaches for Transport Costing and Project Assessment (HEATCO), ‘HEATCO Deliverable 5. Proposal for Harmonised Guidelines’, 2006;

- European Commission (EC), ‘Guidance on the Methodology for Carrying out Cost-Benefit Analysis: Working Document 4’, 2006;

- Internalisation Measures and Policies for All external Cost of Transport (IMPACT), ‘Handbook on estimation of external costs in the transport sector’, 2008;

- Ministry of Economy and Finance – Authority for the Coordination of Structural Instruments, ‘Cost Benefit Analysis of Transport Projects to be supported by the Cohesion Fund and the European Regional Development Fund in 2007-2013’, 2008

- New Energy Externalities Developments for Sustainability (NEEDS), ‘NEEDS Deliverable 2.1. Value Transfer Techniques and Expected Uncertainties’, 2009;

- Unification of accounts for and marginal costs for Transport Efficiency (UNITE), ‘Valuation Conversions for UNITE’, 2001.

- UK Department for Transport (UK DfT), ‘Web-based Transport Appraisal Guidance (WebTAG)’, 2002, 2010.

- Government of Romania, ‘Government Decision No28 on the approval of general framework contents of the technical-economic documentations for public investments and the methodology for developing general cost estimates for investment objectives and works’, 2008

There are three stages to Cost Benefit Analysis, economic analysis, financial analysis and risk analysis.  As outlined in the National Assessment Guidelines for Transport Projects at a strategy stage only economic analysis is required as this indicates which projects provide the greatest total benefit to society for the cost invested.  Financial and risk analysis follows at the more detailed scheme appraisal stage.

The flow chart for the CBA process is shown below:

Purpose of Economic Cost Benefit Analysis

The main purpose of the economic analysis is to assess whether the project’s benefits exceed its costs and whether it is therefore worthwhile to progress in a strict economic sense.  Other factors are considered using Multi-Criteriate Analysis (MCA), see the following paragraph.  The analysis is conducted from the view point of the whole of society, not just the project owners.

To capture the range of economic impacts the analysis includes both elements with direct monetary value, such as construction and maintenance costs and vehicle operating cost savings; as well as elements without direct market value such as time savings, accident reduction and environmental impacts.  In order to allow consistent comparison of costs and benefits across a project all impacts are monetised (i.e. attached a monetary value) and then aggregated to determine the net benefits of the project. From this it can be determined whether the project is desirable and worth implementing.  However it is important to recognise that not all project impacts can be monetised and it is therefore important to consider the results of the CBA in conjunction with the wider Multi-Criteria Appraisal (MCA), which considers these non-monetised impacts.

Purpose of Financial Cost Benefit Analysis

The main purpose of the financial analysis is to assess the financial profitability and sustainability of the project from the viewpoint of the project owners.  This is done by considering the financial cash flow for the project; this includes both outflows in terms of investment, maintenance and operating costs; and inflows in terms of funding sources and user revenues/charges.  Simplistically this analysis shows whether the project will generate a positive net cash flow over the appraisal period (profitability) and whether the cumulative net cash flow since project inception is never less than zero (sustainability).

The analysis initially considers the project’s financial profitability without EU funding to assess whether EU funding is needed.  For projects seeking EU funding this is required to demonstrate that the project is eligible for EU funding.

For a project to be viable the financial analysis needs to demonstrate that the funding sources (including, if relevant an EU grant) and revenue generated by the project are sufficient to offset the project costs, and that the funding and revenue are appropriately profiled across the appraisal period to ensure that in any year the project will not require any additional external bridging funding.

Purpose of Risk Assessment

Project appraisal is a forecasting process and as such has inherent uncertainties.  These uncertainties come from both data limitations in the existing situation, and uncertainties as to how aspects, such as demand for travel, costs for infrastructure etc will change over time.  The risk assessment considers these uncertainties and their impact on the outcomes of both the economic and financial appraisal.

Cost Benefit Analysis within Masterplan study

Economic Cost Benefit Appraisal has been undertaken for schemes as part of the Masterplan selection and prioritisation process.  To assist in this a CBA Tool has been developed by AECOM that interfaces with the National Transport Model to undertake multi-modal CBA analysis of projects.

The CBA Tool is an Excel based program for estimating the economic impact of a project or a scenario (consisting of more than one projects), building on the theoretical background of welfare economics.  The CBA Tool uses data automatically extracted from the National Transport model, along with scheme cost estimates to calculate the economic impact of a project in a manner consistent with requirements of the National Assessment Guidelines for Transport Projects.

The main functions of the CBA Tool are:

to import the outputs of the National Transport Model;

to use appropriate values to express anticipated benefits in monetary terms;

to analyse the value of the anticipated benefits, by discounting and applying conversion factors where applicable;

to produce key economic indicators, by comparing anticipated benefits to estimated costs.

Costs elements considered in the multi-modal analysis include:

Investment costs,

Changes in maintenance cost,

Changes in operating costs (for operating new infrastructure/services).

Benefits considered in the multi-modal analysis include:

Vehicle operating cost changes for users (Freight and road passengers)

Time savings for users,

Variations in external costs:

- Emissions (greenhouse gas and local pollution),

- Noise,

- Accidents,

- Congestion costs.

Costs and benefits can be positive or negative depending upon the nature of the project being assessed.

Key indicators from the analysis are:

Present Value of Benefits (PVB) – total discounted benefits during the appraisal period

Present Value of Costs (PVC) – total discounted economic costs incurred during the appraisal period

Economic Net Present value (ENPV) –the absolute size of the project net benefits.

Benefit to Cost Ratio (BCR) – the ratio between total benefit and costs.  This gives the relative size of the project net benefits but is independent of project size

Economic Internal Rate of Return (EIRR) – Similar to BCR it is independent of project size and gives an indication of the scale of benefits relative to the investment cost.

The tool is fully described in Volume 2, Appendix A of the Appraisal Guidance.  It produces a wide range of outputs, describing all the various inputs and outputs for all modes.  Some of the screens are reproduced below:

Prioritisation of Projects

Overview

Projects prioritisation forms a critical step in producing the Master Plan programme of investments as the investment needs identified are far greater than the available financial allocations. This implies the necessity of ranking projects considering a set of predefined evaluation criteria, which will assure a fair and neutral project prioritisation. Adding the funding restrictions to the list of ranked projects leads to obtaining of the implementation calendar.

The first phase appraisal for a strategy is aimed at discarding the projects having a low economic performance, having in view one of the high-level objectives of the Master Plan, which is “Economic Efficiency”. The projects which emerge from this will be strong candidates for inclusion in one or both of the development scenarios based on “Economic Sustainability” – termed “ES” - or “Economic and Environmental Sustainability” – termed “EES”.

This requires a broad, but robust, appraisal of projects.  The appraisal was undertaken using a Cost Benefit Analysis (CBA) and a summary Multi Criteria Analysis (MCA).

The overall process of project appraisal is illustrated in Figure 2.20 below.

Figure 2.20 Project and Scenario Appraisal Process

The process of generating projects based on the problem analysis and identification of the interventions that best address the specific problems has been described in Sections 2.2-2.4. Sections 2.5 and 2.6 described the main evaluation tools used in the project appraisal stage, which are the National Transport Model and the Cost-Benefit Analysis.

The following sections describe how projects were appraised individually and then incorporated into the two “Do Something” Scenarios, the Economic Sustainability Scenario (“ES”) and the Economic and Environmental Sustainability Scenario (“EES”).

Individual projects have been tested before assembly into the “ES” and “EES” scenarios.  This is because it is a firm requirement of the EU that each of projects in the Master Plan must be justified in economic terms, before consolidation into scenarios.  If the scenarios were assembled first there is danger that a form of “cross-subsidization” would occur, that is, while the scenario as a whole might produce economic benefits, which might disguise the fact that the economic benefits from some projects might offset disbenefits from others.

Therefore, the initial list of projects has been processed using the economic value as the sole criterion.  All projects for consideration in the Master Plan will have to meet this criterion (EIRR > 3%) and this is consistent with EU requirements. This minimum value for EIRR was defined considering the uncertainty margin inherent to the high level of analysis, having in mind that the economic discount rate is 5% for the next EU programming period.

The projects which pass the economic criterion have been combined into the ES and EES Scenario.

Selection of Projects for the ES and EES Scenarios

The output from the project appraisal is a list of projects with scores out of 100 for each Scenario. A critical step was then to determine which projects should then have priority and for this the approach is to assign weights to each score for each criterion to give an overall project score.

The weighting of the projects and their score depends on the characteristics of the two scenarios.  For the ES Scenario, projects have a higher weighting for the economic criteria than for the EES Scenario. The weighting system, based on past consultations with MT and JASPERS, are presented in Table 2.21.

Table 2.21 Criteria and Weights for ES and EES Scenarios

| Criteria | ES | EES |
| --- | --- | --- |
| Economic Efficiency | 70% | 50% |
| Trans-European Integration/TEN-T Policy | 30% | 20% |
| Environmental Impact | - | 20% |
| Sustainability | Not scored but dealt with the distribution of funds by mode |  |
| Balanced Economic Development | - | 10% |

- Source: AECOM / MT / Jaspers

Each project was scored for each scenario using the above weights.  A ranking of projects within each scenario was then be made.

Based on the results of the project appraisal, there are identified projects which are strong candidates for inclusion in the two development scenarios “economic sustainability” and “economic and environmental sustainability”. Typically:

Projects that have limited economic benefits and significant environmental disbenefits were dropped;

Projects which have high economic benefits and significant environmental disbenefits were included in the “economic sustainability” scenario;

Projects that have limited economic benefits but positive environmental benefits were included in the “economic and environmental sustainability” scenario; and

Projects which have high economic benefits and are neutral or positive in environmental terms were included in both scenarios.

Appraisal of the ES and EES Scenarios: Application of Multi Criteria Analysis

The Master Plan represents the direction of the country’s transport for the next 15-20 years.  It is therefore important that a broad appraisal is made at the strategic level so that the overall impact of the Master Plan is known and assessed.  This assessment will be made using the following criteria.

Economic Impacts

- EIRR

Transport Policy

- On TEN-T Core/Comprehensive

Environmental Impacts

- Impact on Natura 2000 sites

Sustainability

- Transfer of Traffic to Sustainable Modes

Balanced Economic Development

- Accessibility of less accessible regions

The weighting and scoring systems for the ES and EES scenarios are presented in Tables 2.22 and 2.23. These have been agreed with the Ministry of Transport and reflect the high-level objectives of the Master Plan as defined in the Terms of Reference.

Table 2.22 Economic Sustainability Scenario

| No. | Master Plan High-Level Objective | Criteria | Indicator/ Reference | Weight | Scoring criteria | Scoring (points) |
| --- | --- | --- | --- | --- | --- | --- |
| A | Economic Efficiency | Economic Performance | EIRR | 70% | 5% | 0 |
|  |  |  |  |  | Maximum EIRR | 100 |
|  |  |  |  |  | > 5% < highest EIRR% | Proportionally from 100 = highest EIRR |
| B | Trans-European Integration | Relation with TEN-T network | TEN-T Regulation | 30% | Core TEN-T link | 100 |
|  |  |  |  |  | Comprehensive TEN-T link | 30 |
|  |  |  |  |  | Other links | 0 |
| C | Sustainability | Contribution to cleaner transport modes policy | White Paper | Not scored under MCA but dealt with via pre-allocation of funding per sectors: this scenario assumes 51% roads, 44% for rail and 5% for Ports, IWT, Intermodal and Aviation |  |  |

- Source: AECOM / MT / Jaspers

Table 2.23  Economic and Environmental Sustainability Scenario

| No. | Master Plan High-Level Objective | Criteria | Indicator/ Reference | Weight | Scoring criteria | Scoring (points) |
| --- | --- | --- | --- | --- | --- | --- |
| A | Economic Efficiency | Economic Performance | EIRR | 50% | 5% | 0 |
|  |  |  |  |  | Maximum EIRR | 100 |
|  |  |  |  |  | > 5% < highest EIRR% | Proportionally from 100 = highest EIRR |
| B | Trans-European Integration | Relation with TEN-T network | TEN-T Regulation | 20% | Core TEN-T link | 100 |
|  |  |  |  |  | Comprehensive TEN-T link | 50 |
|  |  |  |  |  | Secondary connectivity with TEN-T | 10 |
|  |  |  |  |  | Other links | 0 |
| C | Environmental Impact | Potential environmental impact (in particular on NATURA 2000 sites) | SEA | 20% | Very high | -100 |
|  |  |  |  |  | High | -80 |
|  |  |  |  |  | Moderate | -50 |
|  |  |  |  |  | Low | -30 |
|  |  |  |  |  | None | 0 |
| D | Sustainability | Contribution to cleaner transport modes policy | White Paper | Not scored under MCA but dealt with via pre-allocation of funding per sectors: this scenario assumes 51% roads, 44% for rail and 5% for Ports, IWT, Intermodal and Aviation |  |  |
| E | Balanced Economic Development | Improving the accessibility of less accessible regions | GTMP accessibility maps | 10% | Improving link to an area with low accessibility both to foreign and domestic markets | 100 |
|  |  |  |  |  | Improving link to an area with low accessibility to foreign markets | 70 |
|  |  |  |  |  | Improving link to an area with low accessibility to domestic markets | 50 |
|  |  |  |  |  | Links to areas with good accessibility | 0 |

- Source: AECOM / MT / Jaspers

Description of the Evaluation Criteria

- Economic Efficiency

The Transport Economic Impacts criterion relates to the direct economic impacts of the project on the efficiency of the transport system, evaluated by the EIRR (Economic Internal Rate of Return).

- Trans-European Integration

On TEN-T Core or Comprehensive: this sub criterion reflects the fact that it is both Romania’s and the EU’s policy to improve the quality of the most important routes within and across the country.  In addition, the routes selected for the core TEN-T have already been the subject of careful analysis and evaluation, so it is logical that the Master Plan should favour projects which improve these routes.  The inclusion of a National Network criterion acknowledges the fact that the TEN-T network is not comprehensive geographically and there are many large and medium sized cities which are connected only by national roads, and lines designated “core” on the railway network.

Maps of the TEN-T network in Romania are provided in Appendix B.

- Environmental Impact

Natura 2000 relates to the Natura 2000 network of sites that contain the most important habitats across Europe.  Natura 2000 sites have EU legislation to protect them.  This sub-objective should appraise the extent, if any, to which the strategy, or the major projects within a strategy, is likely to impact on these sites in terms of numbers and magnitude.  The habitat and species in these locations are protected, so any impact from a project is likely to be seen as significant.

The assessment is also considering the extent, if any, the project is likely to impact on biodiversity features outside Natura 2000 sites, rating the importance of these features and any inter-relationships, as well as providing a description of the impact on biodiversity, including the effects on its distinctive quality and local diversity.

- Transfer of Traffic to Sustainable Modes

Sustainability includes the transfer of traffic to sustainable modes of transport and reflects both national and EU policy towards “sustainable” modes of transport.  These are those modes, which, in the long term, have lower emissions and energy consumption per passenger km, and which have better safety records.  In practice this means rail and water transport, and to some extent long distance bus.  The National Model provides a means of quantifying the transfer of passengers and freight to these sustainable modes of transport, as a basis for assessing the impact of each scenario.

- Balanced Economic Development

It comprised the improvements towards areas with low accessibility both to foreign and domestic markets. A comprehensive analysis on the accessibility was produced using the National Transport Model and its results were then used to quantify this criterion.

Selection of the Preferred Scenario

At the end of appraisal stage, the optimised strategies for the development of Romania’s transport system have been developed based on “economic sustainability” and “economic and environmental sustainability” respectively. The final stage of the process was to determine an overall recommended strategy.

The recommended strategy seeks to synthesise the two scenario strategies by combining the strongest elements of both scenarios, within the likely funding available.  Given that both scenarios refer to economic sustainability it is likely that there will be substantial overlap between them.  Projects that are advocated under both scenarios are likely to be included in the final recommended strategy, plus some projects which are included in one but not the other.

Outputs from the CBA and MCA provide a succinct and objective assessment of the main impacts of each scenario.  This allows the appraisers to consider the benefits and disbenefits of each scenario, based on a consistent, transparent and auditable approach.

Chapter 12 includes the results of the prioritisation process and the recommendations of the preferred scenario.

Ex-Ante Conditionalities

The document “Guidance on Ex ante Conditionalities for the European Structural and Investment Funds PART II”  states that the Thematic Objective transport objective for transport is:

Promoting sustainable transport and removing bottlenecks in key network infrastructures (referred to in Article 9(7))

The corresponding ex-ante conditionality is:

The existence of a comprehensive plan or plans or framework or frameworks for transport investment in accordance with the Member States’ institutional set-up (including public transport at regional and local level) which supports infrastructure development and improves connectivity to the TEN-T comprehensive and core networks.

The Master Plan is a comprehensive plan for all modes of transport, developed using quantified measures for the economic contribution of projects, their contribution to the Core TEN-T, and their contribution to national and regional development.  The key assessment tools are the National Transport Model, which assesses the demand for travel based on observed data and mode choice relationships, and takes into account the level of service offered and travel times and costs for the different modes of transport.  It provides an integrated, neutral, and holistic for assessing transport interventions.  The CBA procedure, developed especially for the Master Plan, uses EC-recommended procedures to evaluate the economic benefits of an intervention taking into account the benefits to existing travellers and freight, diverted and traffic from other routes and modes, and newly-generated traffic.  It values travellers time, costs, vehicle operating costs, and environmental impacts such as vehicle emissions and noise, using accepted values, and compares these with the investment and operating costs.  The outputs are produced in the EC-approved format for Funding Applications to the Structural Funds.

Table 2.24 shows the ex-ante conditionalities and the way the Master Plan follows them.

Table 2.24 Ex-ante conditionalities

| Criteria for fulfilment | Criteria fulfilled? |  |
| --- | --- | --- |
|  | YES / NO | Elements of non-fulfilment |
| The existence of a comprehensive transport plan or plans or framework or frameworks for transport investment which: |  |  |
| - The relevant operational programme and where appropriate, the Partnership Agreement contains a reference to the name of the plan or framework and provides a hyperlink to the documents(s). | NO | Master Plan not yet approved |
| – complies with legal requirements for strategic environmental assessment : |  |  |
| - An environmental report has been prepared in which the likely significant effects on the environement of the implementation of the plan or framework and reasonable alternatives taking into account the objectives and the geographical scope of the comprehensive transport plan or framework, are identified, described and evaluated. | YES |  |
| - The draft plan or framework and the environmental report have been made available to the public and the authorities with specific environmental responsibilities designated by the Member States who are likely to be concerned by the environemental effects of the implementing plans. | NO | The draft plan has been made available to the public and the authorities on October 2012 and October 2013 . Each chapter from Environmental Report have been presented in the working group. The final Environmental Report will be make available after Appropriate Assessment will be approved by MECC. The final Environmental Report must include the conclusion of Appropriate Assessment. |
| - In case of possible significant transboundary effects, the draft plan or framework and the environmental report have been forwarded to the relevant/affected Member States. | NO | In case of possible significant transboundary effects the Final Environmental Report and Master Plan will be forwarded to the relevant/affected Member States. Responsibility to inform the relevant/affected Member States lies the central public authority promoting the plan (in this case the Ministry of Transport), on the recommendation of the Ministry of Environment and Climate Change. It is recommended that the establishment of States which are notified to be performed by the central public authority promotes plan with the Ministry of Environment and Climate Change. In according with GD 1076/2004 Art. 22 (2) din HG 1076/2004 menţionează că „În cazul în care implementarea planului sau programului poate avea efecte semnificative transfrontieră, titularul, prin intermediul autorităţii publice centrale care promovează planul sau programul, este obligat să transmită proiectul de plan sau de program şi raportul de mediu elaborat pentru acesta, în limba engleză, autorităţilor centrale de mediu din statele posibil afectate, în termen de maximum 20 de zile calendaristice de la finalizarea raportului de mediu, conform art. 21 alin. (3)”. |
| - The environmental report and the opinions expressed in the relevant consultations (including as appropriate transboundary ones) have been duly taken into account during the preparation of the comprehensive transport plan or framework. | NO | See previous. |
| - When the plan or framework has been adopted, the authorities with environmental responsibilities, the public and any Member State consulted, are informed and the following items have been made available to them: the plan or framework as adopted, the statement referred to in Article 9(1) of the SEA Directive and the measures concerning monitoring referred to in Article 10 of the SEA Directive. | NO | See previous. |
| ‒ sets out the contribution to the single European Transport Area consistent with Article 10 of Regulation (EU) No1315/2013 of the European Parliament and of the Council, including priorities for investments in the core TEN-T network and the comprehensive network where investment from the ERDF and CF is envisaged; and secondary connectivity. | YES |  |
| - The investment priorities included in the comprehensive transport plan or framework connect the identified main nodes (see the list in annex) and provide for connections with neighbouring countries' transport infrastructure networks. The Member State shall also demonstrate how its investments in secondary connectivity will contribute to the single European Transport Area57. | YES |  |
| - The comprehensive transport plan or framework includes measures that are necessary for: |  |  |
| ensuring enhanced accessibility and connectivity for all regions of the Union while taking into consideration the specific case of islands, isolated networks and sparsely populated, remote and outermost regions; | YES |  |
| ensuring optimal integration of the transport modes and interoperability within transport modes; | YES |  |
| bridging missing links and removing bottlenecks, in particular in cross-border sections; | YES |  |
| promoting the efficient and sustainable use of the infrastructure and, where necessary, increase the capacity; | YES |  |
| improving or maintaining the quality of infrastructure in terms of safety, security, efficiency, climate and where appropriate disaster resilience, environmental performances, social conditions, accessibility for all users, including elderly people, persons with reduced mobility and disabled passengers, as well as the quality of services and continuity of traffic flows; | YES |  |
| implementing and deploying telematic applications as well as promoting innovative technological development; | YES |  |
| - Particular consideration shall also be given in the comprehensive transport plan or framework to measures that are necessary for: |  |  |
| ensuring fuel security through increased energy efficiency and promoting the use of alternative and in particular low or zero carbon energy sources and propulsion systems; | YES |  |
| mitigating exposure of urban areas to negative effects of transiting rail and road transport; | YES |  |
| removing administrative and technical barriers, in particular to the interoperability of the trans-European transport network and to competition. | YES |  |
| - sets out a realistic and mature pipeline for projects envisaged for support from the ERDF and CF |  |  |
| - The plan or framework for transport investments includes a table containing : |  |  |
| a list of prioritised projects (studies, upgrading or works) that the Member State envisages launching over the period and asking for support from the ERDF and CF. | NO | To be included in separate Strategy Report containing Implementation Plan |
| the name of the authorities and stakeholders involved in the lead of these projects, the foreseen expenditures and a financing plan, | NO | To be included in separate Strategy Report containing Implementation Plan |
| a realistic timetable for delivery of the projects identified indicating dates for feasibility studies, a Cost Benefit Analysis, EIA procedure58, an implementation timetable including procurement and permission procedures, and for potential state aid notification (per phase for bigger projects). | NO | To be included in separate Strategy Report containing Implementation Plan |
| - Measures to ensure the capacity of intermediary bodies and beneficiaries to deliver the project pipeline. |  |  |
| - The Member State has provided an adequate description of the measures already in place to ensure the capacity of intermediary bodies and beneficiaries to deliver the project pipeline: |  |  |
| These measures are based on the analysis of both the bottlenecks and of the weaknesses of intermediary bodies and beneficiaries to deliver timely the project pipeline, as regards: |  |  |
| o tendering (including tenders without competition, irregularities) | NO | To be included in the Final version of the Master Plan |
| o implementing environmental requirements, | NO | To be included in the Final version of the Master Plan |
| o developing and prioritising a mature project pipeline, | YES |  |
| o financial project management, | NO | To be included in the Final version of the Master Plan |
| o funding for maintenance and operations, | YES |  |
| o administrative burden and red tape, | NO | To be included in the Final version of the Master Plan |
| o managing complex systems (ITS such as ETCS-ERTMS, VTMIS, RIS, emaritime services and air traffic management system). | YES |  |
| They include training and appropriate internal procedures to monitor and identifiy potential delays and to ensure a smooth and effective procurement; | NO | MT are still in the process of establishing new organizational structures |
| A early warning system is in place to identify and solve any difficulties rising from intermediary bodies and beneficiairies when delivering the project pipeline; | NO | MT are still in the process of establishing new organizational structures |
| Adequate assistance schemes are in place to help beneficiaries during procedure and implementation to be able to replace projects quickly when implementation is blocked. | NO | MT are still in the process of establishing new organizational structures |

Existing Situation and Recent Transport Trends in Romania and Europe

Existing Situation and Recent Transport Trends in Romania and Europe

GTMP Relation with the Relevant EU Transport Policy Documents

Any national or regional economic growth is strengthened by a competitive and progressive transportation system, that is geared to the needs of customers and operated as a sustainable network offering high quality and affordable services.

To achieve such a system, policies of all levels have to be coordinated and harmonised. This harmonisation supports not only the economic development and trade but also avoids extra costs to the transport system, and improves the capital and labour productivity within the European Union.

The EU’s responsibility for policy-making varies enormously across its range of  policy interests. In some spheres arrangements are well established, and effective policy instruments – legal and financial – are usually available. In other spheres, EU involvement is marginal, policy processes may be confined to little more than occasional exchanges of ideas and information.

The main aim of all EU policies is to create an integrated market, not only to dismantle internal barriers and provide conditions for fair trade, but also to give it  many of the characteristics of an economic and monetary union.

The EU's main policy responsibilities could be divided into five major groups:

establishing the Single European Market;

macroeconomic and financial policies;

functional policies;

sectorial policies; and

external policies.

Functional policies have a well-defined functional purpose and more specific nature than the macroeconomic policies. The best known functional policies are the justice and home affairs, cohesion, and research and technological development. Other policies are directed towards specific economic sectors – covering coal and steel, atomic energy, agriculture and transport. These sectoral policies were explicitly provided for in the Founding Treaties.

The EU transport policy is a component of this integration process. It does have a regulatory emphasis in the areas of working conditions and the environmental  protection because of market implications. It also develops a framework for an integrated transport market not just finding solutions to internal barriers. rriers. In the transport sector, the extent of the EU policy involvement can be seen as responsibility shared between the EU and the member states

From the multitude of plans, strategies European programs related to the transport sector there have been considered the major plans programs, strategies, policies and existing conventions order to identify problems, issues and aspects that may influence the General Transport Master Plan.

As a result of this analysis the following conclusions were drawn:

The main national transport objectives are common with those of GTMP and follow these:

Ensuring economic development: transport sector should contribute to the development of the national economy and the economic benefits should exceed its costs;

Sustainable development: transport system must be efficient in terms of energy consumption, providing reserves for future generations;

Safety: transportation system must provide security;

Providing funding: Master Plan should be able to absorb EU funds.

GTMP takes account of necessary measures to adapt to climate change;

Based on the strategies / plans aimed particularly to environmental protection and on the existing environmental issues at national level related to the transport sector, for GTMP there have been proposed a number of relevant environmental objectives which take into account the targets set at national and European level for the environment protection (see also Section 2.3 – Objective Setting).

A series of national and European policty documents were analyzed, with respect to the relation with the Master Plan, among wich the most relevant ones were:

European and National Policies

TEN-T Regulations

EU White Paper on Transport 2011)

Government programs 2013-2016 - transport policy proposed by the Romanian Government

Strategic Plan Integrated for Transport and Infrastructure - June 2009 (Ministry of Transport Policy)

Partnership Agreements

Partnership Agreement proposed by Romania for the programming period 2014-2020 (Ministry of European Funds)

European and National Programs

Integrated European action program for inland owaterway - NAIADES I and II

Sectoral Operational Programme Transport (POST) 2007-2013, Revision 2

Strategic development program of airport infrastructure Bucharest Otopeni International Airport (1999-2015) – Law220/2002 (Gazette no 288/29.04.2002) approving the government ordinance no. 64/1999 (Gazette no. 405/26.08.1999)

Strategic development program of airport infrastructure at SN International Airport Constanta SA between 2002-2015 - GD. 623/2002 (Gazette. No. 458 / 27.06.2002)

Strategic development program of airport infrastructure at SN International Airport SA between 2002-2015 - GD. 60/2003 (Gazette. No. 76 / 06.02.2003) amending GD 615/2002 (Gazette. 488 / 08.07.2002)

National Plans

National Action Plan for Energy Efficiency (NAPEF) - developed in 2007, under review

National Spatial Plan (NSP) - Section 1 transport network. Key rail and road networks (Law 363/2006)

LAW. 203 of 16 May 2003 on the implementation, development and modernization of the national and European transport

Management Plan for the National portion of the Danube River International Basin -Synthesis of the management plans at basin / catchment areas level.

European and National Strategies

Europe 2020 Strategy

EU Strategy for the Danube Region

EU strategy on climate change adaptation (2013)

Marine environment Strategy (Directive 2008/56 / EC of the European Parliament and of the Council of 17 June 2008 establishing a community action framework in the field of marine environmental policy)

Strategy for the sustainable transport for the period 2007-2013 and 2020, 2030

Romanian intermodal transport strategy – 2020

Romanian National Strategy on climate changes 2013 – 2020

National Strategy for the sustainable development of Romania -Horizons 2013 – 2020 – 2030

National Strategy and Action Plan for the biodiversity conservation 2010 -2020

National Road Safety Strategy 2011-2020 - consultative version

National Waste Management Strategy 2014-2020, approved by Government Decision no. 870/2013

Conventions

Law 98/1992 ratifying the Convention on the Protection of the Black Sea against pollution, signed in Bucharest on 21 April 1992 (known as the "Bucharest Convention”)

Convention regarding the regime of navigation on the Danube (Belgrade, 1948) ratified by Decree no. 298 of 30 October 1948 Additional Protocol of 26 March 1998 to the Convention of 18 August 1948 regarding the regime of navigation on the Danube*)

Law 14/1995 for the ratification of the Convention for the Protection of the Danube River

Other Documents

Preliminary report on the Transportation developed within the project: The operationalization of the national strategy and development component of the Operational Programs 2014-2020 Climate "project of the MECC (Ministry of Environment and Climate Changes)

Working Document of the European Commission - "Adapting Infrastructure to Climate Change”

Joint Declaration on the principles guiding the development of navigation and environmental protection in the Danube River Basin (Joint Statement on Guiding Principles for the Development of Inland Navigation and Environmental Protection in the Danube River Basin)”.

Table 3.1 includes a brief description of the contents of the most relevant EU Policy documents, the reference period for the actions to be undertaken and also the relation with the Master Plan.

Table 3.1 Relationship of General Transport Master Plan with the most relevant EU Transport Policy documents

| Strategy/Plan/ Program relevant to GTMP | Summary description of document | Reference period | Relation to General Transport Master Plan |
| --- | --- | --- | --- |
| TEN-T Regulations | The TEN-T network consists of two layers: a core network intended to be completed by 2030 and a comprehensive network feeding into this, intended to be completed by 2050. The comprehensive network will ensure full coverage of the EU and accessibility of all regions. The core network will prioritize the most important links and nodes of the TEN-T, to be fully functional until 2030. Both layers include all transport modes: road, rail, air, inland waterways and maritime transport, as well as intermodal platforms. The TEN-T guidelines set common requirements for the TEN-T infrastructure – with tougher requirements for the core network. This will ensure fluent transport operations throughout the network. The policy also fosters the implementation of traffic management systems which will allow optimising the use of infrastructure and by increasing efficiency, to reduce CO2 emissions. Intelligent transport systems include the traffic management systems for road, rail, air and waterborne transport as well as the positioning and navigation systems. | 2030 | The relation with the EU TEN-T policy consists of one the evaluation criteria in the project prioritisation process. This reflects the fact that it is both Romania’s and the EU’s policy to improve the quality of the most important routes within and across the country. In addition, the routes selected for the core TEN-T have already been the subject of careful analysis and evaluation, so it is logical that the Master Plan should favour projects which improve these routes. The TEN-T concept includes the connection of major nodes (cities with a population of 1 million or more) via multi-modal corridors. The assessment of network improvements used in the Master Plan implicitly takes into account the travel demand from these and other major centres. Maps of TEN-T Network in Romania are provided in Appendix B. |
| White Paper on Community Transport Policy (2011) | The White Paper of Transport is a Roadmap to a single European space of transport to a competitive and efficient transportation system. Examines developments in the transport sector, future challenges and policy initiatives that should be considered globally. The document presents both the European Commission vision on transport as well as the key measures to allow the vision implementation. The vision for a sustainable and competitive transport system refers to: - Increasing transport and supporting mobility while reaching the goal of reducing emissions of greenhouse gases by 60%. This can be achieved by: - Developing and implementing sustainable fuels and propulsion systems - Optimizing performance of multimodal logistic chains, including wider use of more efficient modes of transport in terms of energy - Increasing the efficiency of transport and infrastructure use with information systems and market-based incentives - An efficient core network for transport and long-distance journeys - Multimodal. - Global fair transport conditions for long distance journeys and for intercontinental freight transport. - Clean urban and commuting transport Achieving this vision involves the following: - Creating a single European transport space: by developing rail services, including the quality of air service development and airport capacity, further transformation of European maritime transport space, optimizing the internal market for inland waterway transport, improvement of road freight creating the necessary framework for multimodal transport of goods - Promoting employment and quality working conditions - Transport safety - Quality and reliability of services - Innovation and implementation strategy - Promoting a more sustainable behaviour - Modern infrastructures and smart financing | 2020-2050 | The General Transport Master Plan objectives are based on the provisions of the White Paper and are focused on ensuring sustainable transport, ensuring funding sources, transport safety, economic development and environmental protection. GTMP will help create an efficient transport system in terms of supporting economic competitiveness but also encourage the most efficient use of resources, reduce negative impacts that the transportation system has on the environment respectively on water, soil, air populations and natural ecosystems GTMP proposes to develop transportation infrastructure by creating new transport corridors (development of highways, bypasses, road), rehabilitation of railway lines to achieve high speeds, improving navigation on the Danube and modernization of port infrastructure, modernize airports. |
| Europe 2020 Strategy – An European strategy for a smart, sustainable and inclusive development | Europe 2020 Strategy proposes three mutually reinforcing priorities: – smart growth: developing an economy based on knowledge and innovation; –sustainable growth: promoting a more efficient economy in terms of resource use, greener and more competitive; –inclusive growth: promoting an economy with a high rate of employment, ensuring social and territorial cohesion. To ensure that each Member State tailors the Europe 2020 strategy to its particular situation, the Commission proposes that EU goals are translated into targets and trajectories. Romania proposes for the Energy Sector and transport the following: - Based on the current situation, namely that Romania has a low degree of competitiveness and energy efficiency in the transport is considered necessary to ensure the liberalization of gas and electricity prices, strengthen corporate governance and state regulatory bodies and to complete cross-border connections. In terms of infrastructure for broadband connections, now it is the least developed in the EU, which should be corrected. In the transport sector is necessary a long-term comprehensive plan. | 2020 | GTMP includes Romanian strategy for the transport sector in the short, medium and long term and include integrated transport development projects that may be proposed for funding in the next phase of funding by 2014-2020 Through its proposals in the transport sector, GTMP will contribute to effective and sustainable economic development of Romania, the creation of environmentally friendly transport systems and low greenhouse gas emissions, ensuring mobility and improve connections between different modes of transport. |
| EU Strategy for the Danube region | This strategy has been elaborated by the European Commission (EC), through the General Directorate for Regional Policies (REGIO). The strategy focuses on these: Danube region interconnectivity (improving transport infrastructure, encouraging sustainable energies, culture and tourism promotion), environment protection (achieving the environment objectives included in the Danube management plan, nutrient pollution reduction, implementation of protection plans against floods, protection against floods risks, reduction of the areas affected by erosion and biodiversity protection), consolidation of the Danube region (institutional cooperation, security), increasing prosperity in the Danube region (socio-economic development, research, employment development, improving education). The objectives of this strategy in terms of the transport sector are the following: - 20% increase compared to 2010 of the freight transport on the river, 2020; - removing bottlenecks to navigation on the river, until 2052, to use the VIb type vessels throughout the year; - improved travel times for competitive rail connections between major cities; - implementing the 4 rail freight corridors crossing the Danube, as planned, within 3 or 5 years; - Development, by 2020, of efficient multimodal terminals at Danube River ports to connect inland waterways with rail and road transport. | 2010-2020 | General Transport Master Plan proposes a series of investments to improve waterways in the Romanian Danube, modernization of port infrastructure, development of intermodal freight transport. |
| European Union (EU) Strategy on adaptation to climate changes (2013) | The overall aim of the EU strategy on adaptation to climate change is to contribute to a more resilient Europe to climate change, thereby increasing the preparedness and capacity to respond to climate change impacts at local, regional and national levels as well as EU level, developing a consistent approach and improving coordination. The recommended tool globally in the context of the UN Framework Convention on Climate Change is the national adaptation strategies. | 2014-2020 | It is necessary to apply climate change adaptation measures in key vulnerable sectors such as the transport sector is. Thus, planning and prioritizing investments included in the Master Plan of Transportation has considered the climate component. It is expected that the projects proposed in the Master Plan of Transportation to lead to the reduction of greenhouse gas. |
| Sustainable transport strategy | Sets the priority projects in transport identified by Romania together with EU which should be achieved by 2015. The overall objective is to develop balanced national transportation system to provide infrastructure and transport services, sustainable development of economy and quality of life. The specific objectives envisaged for the overall objective are the following: - modernization and development of the transport network of European and national interest; - increase the safety and quality of services; - liberalization of internal transport; - stimulate economic development and competitiveness; - strengthening social and territorial cohesion at regional and national; - Compatibility with the environment. | 2007-2013, 2020, 2030 | General Transport Master Plan will propose a list of priority investments selected by specific selection criteria taking into account the provisions of sustainable transport strategy. Proposed investments by GTMP consider the following: - development of transportation infrastructure - improvement and efficient rail infrastructure - development of intermodal transport - improvement of air transport infrastructure |
| Convention regarding the regime of navigation on the Danube (Belgrade, 1948), ratified by Decree no. 298 of October 30, 1948 Additional Protocol of 26 March 1998 to the Convention of 18 August 1948 regarding the regime of navigation of the Danube*) | It is an international legal instrument governing navigation on the Danube. - Navigation on the Danube shall be free and open to trade and cargo ships of all states on an equal footing in terms of port rights and duties of navigation, as well as the conditions which is subject to Merchant Shipping - All countries of the Danube will provide the necessary work to improve navigation conditions without hindering or impeding navigation on the Danube waterways Convention is coordinated by a "commission" which includes representatives of the Member States. | Starting with the date the convention was ratified | Romania must comply with the Convention regarding the regime of navigation on the Danube. The Danube Fairway must meet the internationally accepted standards as set by the Danube Commission GTMP implementation will contribute to the improvement of navigation and refurbished port infrastructure. |

Sections 3.2.-3.7 describe the headline figures and historical trends for transport in Romania, on all modes and at an integrated European level.

Road Transport Trends in European Context

An efficient, safe and reliable road network is of fundamental importance to the success of the Romanian economy and, ultimately, to recognising the potential for growth.

The highway network in Romania is classified into five categories:

Motorways (Autostrăzi) - A;

National and European Roads (Dumuri Naționale si Europene) – DN/E;

National Roads (Dumuri Naționale) – DN;

County Roads (Dumuri Județene) – DJ; and

Communal Roads (Dumuri Comunale) – DC.

The motorway and national road network accounts for just 20% of the entire network as summarised in Table 3.2.

Table 3.2 Length of Road Network by Category

| Road Type | Kilometres | Proportion |
| --- | --- | --- |
| Motorway | 362.6 | 0.5% |
| National European | 5,697.7 | 7.1% |
| Other National | 9,930.9 | 12.4% |
| County | 36,009.8 | 45.1% |
| Commune | 27,780.8 | 34.8% |
| Total | 79,781.7 | - |

Source: AECOM analysis of individual DRDP data

Standard of Network Provision and Function: Almost 90% of the national network is single carriageway standard, this impacts upon both travel time and safety. Some national roads utilise a single carriageway with a hard shoulder but this is typically of sub-standard width. This is not sufficient to accommodate overtaking of local agricultural vehicles, which commonly make use of the network, nor heavy goods vehicles which are prevalent on any national network and need to be accommodated safely.

Compared to the rest of Europe, the level of motorway provision is very low. Recent European statistics comparing the level of motorway provision to that of all national / main roads, places Romania 30th out of 31 (for those countries with relevant data available). This has an impact upon the reliability of the road network for business and the economy.

It is also worth noting that the same data source identifies that Romania is 7th out of 33 countries in terms of the proportion of the total network that is classified as “national”. This shows that the proportional extent of the national network is higher than most EU countries. Figure 3.1 shows the percentage of the overall road network which is categorised as being part of the National Network and the percentage of that network which is Motorway standard for countries with comparable total road network lengths as Romania.

Source: EUROSTAT

Figure 3.1 Percentage of Road Networks Classified as “National”

CESTRIN have provided data to AECOM on the state of the national network as at 1st January 2012. The data provided includes information on the type of surfacing, an assessment of the quality of that surfacing (based on IRR surface roughness values) and also the type of topography through which the road section runs. We have not been able to obtain equivalent data for the non-national network.

Table 3.3 shows the proportion of the network that is considered good, average or poor broken down by surface type and topography.

Table 3.3 National Road Network - Surface Condition

|  |  | Asphalt | Concrete | Paved | Light Bituminous Pavement | Stone | Earth | Total |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hill | Good | 19.2% | 0.8% | 0.0% | 0.3% | 0.0% | 0.0% | 20.4% | 40.6% |
|  | Average | 11.5% | 1.4% | 0.1% | 1.4% | 0.3% | 0.0% | 14.6% |  |
|  | Poor | 3.8% | 0.9% | 0.0% | 0.8% | 0.1% | 0.0% | 5.6% |  |
| Mountain | Good | 10.3% | 0.4% | 0.0% | 0.1% | 0.0% | 0.0% | 10.8% | 18.3% |
|  | Average | 4.0% | 0.2% | 0.0% | 0.2% | 0.1% | 0.0% | 4.5% |  |
|  | Poor | 1.2% | 0.3% | 0.0% | 0.5% | 0.9% | 0.0% | 2.9% |  |
| Plains | Good | 19.9% | 1.0% | 0.1% | 0.7% | 0.0% | 0.0% | 21.7% | 41.2% |
|  | Average | 10.4% | 0.5% | 0.0% | 0.7% | 0.1% | 0.0% | 11.7% |  |
|  | Poor | 5.5% | 0.6% | 0.1% | 1.4% | 0.1% | 0.1% | 7.7% |  |
| Total |  | 85.7% | 6.3% | 0.2% | 6.0% | 1.6% | 0.1% | 100.0% | 100.0% |

Source: AECOM analysis of CESTRIN Viabilitate data

The notable point from this is that only just over 50% of the national network is classified as good with a further 30% average and 20% poor. The expectation is that the national network should be at the top end of the standard for any country.

Safety: Romania has a significant road accident problem as evidenced by comparisons with the EU. The EC use three separate indicators as follows:

Fatalities per million inhabitants;

Fatalities per 10 billion pkm; and

Fatalities per million passenger cars.

In this order, Romania’s score and ranking is as follows:

24th out of 28 – 94 versus EU average of 60;

28th out of 28 – 259 versus EU average of 61; and

28th out of 28 – 466 versus EU average of 126.

From these data, it is fair to conclude that Romania has the worst road fatality accident rate in Europe.

It is also important to recognise that whilst the national network accounts for around 30% of all accidents in Romania, it accounts for well over 50% of all fatal accidents.

The overall pedestrian accident rate for Romania is the worst in Europe and significantly above the EU average – see Table 3.4.

Table 3.4	EU Comparative Statistics on Pedestrian Fatality Rates

| Country |  | No. of Fatalities | Population [million] | Pedestrian fatalities by million inhabitants |
| --- | --- | --- | --- | --- |
| BE | 106 |  | 10.8 | 9.8 |
| CZ | 168 |  | 10.5 | 16.0 |
| DK | 44 |  | 5.5 | 8.0 |
| DE | 476 |  | 82.0 | 5.8 |
| EE | 23 |  | 1.3 | 17.7 |
| IE | 44 |  | 4.4 | 10.0 |
| EL | 179 |  | 11.3 | 15.8 |
| ES | 471 |  | 45.8 | 10.3 |
| FR | 485 |  | 64.4 | 7.5 |
| IT | 614 |  | 60.0 | 10.2 |
| LV | 79 |  | 2.3 | 34.3 |
| LU | 1 |  | 0.5 | 2.0 |
| HU | 192 |  | 10.0 | 19.2 |
| MT | 2 |  | 0.4 | 5.0 |
| NL | 63 |  | 16.5 | 3.8 |
| AT | 98 |  | 8.4 | 11.7 |
| PL | 1,236 |  | 38.1 | 32.4 |
| PT | 195 |  | 10.6 | 18.4 |
| RO | 868 |  | 21.5 | 40.4 |
| SI | 26 |  | 2.0 | 13.0 |
| SK | 126 |  | 5.4 | 23.3 |
| FI | 35 |  | 5.3 | 6.6 |
| SE | 44 |  | 9.2 | 4.8 |
| UK | 429 |  | 61.6 | 7.0 |
| EU-24 | 6,004 |  | 487.8 | 12.3 |

Source: EU DaCoTA - Pedestrian fatalities per million inhabitants by country, EU-24*, 2010 Pedestrian fatalities

Car Ownership Levels: There has been a steady growth in car ownership levels in Romania with the long term trend averaging around 5% per annum.

This needs to be seen in context however with car ownership levels in the rest of Europe. The comparison is provided in Figure 3.2 with the values for Romania highlighted in red.

Source: Eurostat

Figure 3.2 Comparison of EU Car Ownership Levels (2010)

Clearly car ownership levels are lagging some way behind the rest of Europe. Car ownership is forecast to grow strongly in Romania, as shown in Figure 3.3. 19.3% of population have a car available in 2011 rising to 24.9% by 2020. This will have a direct impact on the likelihood of road becoming the mode of choice for even more passenger journeys than presently is the case.

Source: INS, AECOM Model Development Report

- Figure 3.3 Forecast of Car Ownership in Romania

By 2020 however car ownership will still be well below the current national average across the rest of Europe.

Rail Transport Trends in European Context

Rail Network Characteristics: Total Romanian route network length stands at 10,818 km. Table 3.4 sets out some of the key statistics associated with the network.

Table 3.5 Rail Infrastructure Statistics

| Length of rail network | 10,818 km | Number of level crossings (number of which are automatic) | 5,119 (1,082) |
| --- | --- | --- | --- |
| Double track | 2,909 (27%) | Number of points/switches | 20,868 |
| Single track | 7,771 (72%) | Train control (signalling) infrastructure: Electronic interlocking systems Relay interlocking systems Installations without interlocking Automatic line block: Number of installations | 28 618 354 577 |
| Electrified (overhead 25 Kv) | 4,002 (37%) |  |  |
| Non-electrified | 6,816 (63%) |  |  |
| Number of stations | 965 |  |  |
| Number of tunnels | 177 |  |  |
| Length of tunnels | 6,809 |  |  |
| Number of bridges | 4,216 |  |  |
| Number of culverts | 13,961 |  |  |

Source: CFR SA: CFR Network Statement

A significant proportion (72%) of the rail network is single track – the EU27 average is 59%. The 37% of the network that is electrified compares to the EU27 average of 52%.

In terms of the density of the network, Table 3.6 compares Romania to some other European and neighbouring countries.

Table 3.6 Rail Network Density

Romania is above the average in terms of density per population (but not as high as in neighbouring countries), but lower than average in terms of spatial density.

Rail Passenger Demand: Figure 3.4 provides a comparison between Romanian rail passenger kilometres with those of other EU countries.

Source: http://ec.europa.eu/transport/facts-fundings/statistics/pocketbook-2013_en.htm

Figure 3.4 Comparison of Rail Passenger Kilometres – Romania versus other EU Countries

Compared with those countries with a similar level of rail network provision per head of population, the rate of railway use in Romania is low. Rail demand in Romania, as measured by kms travelled per person is two to three times lower than in these countries, and this failure to capture the potential rail market results in reduced revenue. The EU27 average is 650km per passenger per year, as shown in Figure 3.4, while the equivalent figure for Romania is 66% lower.

Figure 3.5 illustrates the number of journeys declined between 2004 and 2012 from about 100m per annum to 58m. The 28% decline in patronage experienced in Romania between 2004 and 2009 is the largest decrease across all the EU Member states.

Source: AECOM analysis of CFR Calatori data

Figure 3.5 Number of Rail Passenger Journeys per annum in Romania (2004-2012)

The relationship between passenger kilometres per person has also been benchmarked for Romania against other EU countries. The ratio for Romania (239 passenger kilometres / person) is lower than other EU countries except Estonia (172), Greece (118), Turkey (74) and Lithuania (34). The results for Romania are lower than Bulgaria (270 passenger kilometres / person) or Hungary (731). These statistics indicate rail is capturing a good percentage of the total trips, albeit recognising the actual trip rate per person is significantly lower. The comparison of passenger kilometres per person for Romania against other EU countries indicates there is significant scope to grow the market if service improvements were delivered.

Freight Usage: With about 28% of goods moved by rail, Romania is ranked 6th compared with other countries. This proportion was about 10% higher compared with the EU27 average of 18.4%.

- Table 3.7 Proportion of Goods Transported by Rail

| Country | % of goods transported by rail (tonne km) | Rank |
| --- | --- | --- |
| Latvia | 63.8 | 1 |
| Estonia | 51.0 | 2 |
| Lithuania | 41.2 | 3 |
| Austria | 39.9 | 4 |
| Sweden | 38.2 | 5 |
| Romania | 28.0 | 6 |
| EU27 Average | 18.4 |  |

Proportion of goods transported by rail in the Transport model year 2011 by tonne km (Source Eurostat)

Similar to the passenger market, the rail freight market has also contracted, with many traditional heavy industries such as coal mining and metal smelting, which provided the core rail freight demand, having declined. Coal fired power stations for electricity generation are gradually being replaced by renewable energy sources such as wind farms where Romania is the 5th largest producer in the EU. Tonnage moved by rail has declined steadily since 2004 (from 72m tonnes in 2008 to 56m tonnes in 2012). This is against a background of growth in road freight - in 2012, road freight grew by 4% and rail freight declined by 4% and 2012 saw a significant downward trend in market share in tonne kms from 28% to 21% in just one year.

Passenger Ticket Prices: A benchmarking exercise indicates that Romanian rail fares are relatively high compared with other countries when they are normalised to take account of comparative spending power of other European economies. This is particularly relevant for advance purchase tickets. The high fares restrict the opportunities to attract new passengers and reduces the net economic benefit of the railway system, as shown in Figure 3.6.

Source: AECOM analysis of a selection of fares for a 100km journey. Fares are adjusted to take account of differences in GDP

Figure 3.6: Comparison of Romanian Fares with Selected EU Examples

Track Access Charges: In line with EU legislation, the MTI administers a non-discriminatory track access charge for passenger and freight operations. The basis for the charge, whether passenger or freight, is a combination of train tonnage, line categorisation (essentially line speed) and whether the line is electrified or not. Figure 3.7 below demonstrates that Romania has some of the highest track access charges in the EU.

- Figure 3.7 Access Charges for Typical Frieght Train

Passenger Rail Journey Speeds: Selected rail journey speeds in Romania have been compared with a sample of European services as shown in Figure 3.8. For example, the average speeds in Romania are about 65-70km/h, yet a comparison with the UK and Germany indicates rail journey speeds in Romania are about twice as slow as the examples. Furthermore, timings in Romania are 40% slower compared with Hungary and Poland. The topography in Romania is a partial factor for some journeys, for example, the hilly terrain between Bucharest and Cluj, but there are other contributory factors including the numerous intermediate stops, lengthy dwell times and various technical factors

Source: AECOM analysis of CFR Calatori timetables and DB website

Figure 3.8 Comparison of Rail Journey Times – Romanian versus European Examples (km/h)

Rail Safety: The the number of fatalities involving passengers or railway employees is relatively low when expressed in terms of numbers per billion passenger kilometres, but there are still a relatively high number of deaths due to suicides and other incidents at level crossings.  There were 79 rail-related fatalities (all deaths occurring on railway property) per billion train kilometres between 2007 and 2011.  This is almost four times higher than the EU average, and the 4th highest rate overall, as demonstrated in Figure 3.9.

- Source: European Union information, printed in the Guardian 13 May 2013

Figure 3.9: Comparative Number of Fatalities (per billion train kilometres)

Ports and Waterways Trends in European Context

The proportion of total inland freight in Romania moved by water is the second highest in the EU. This is due to Romania’s position relative to the Danube. However Romania compares unfavourably to Holland, which has a similar position relative to the Rhine and the key difference between the countries is the lack of modern facilities in Romania. The vast majority of the goods transported by water in Romania are bulk goods. The Danube carries 3 types of traffic, domestic, transit, and import/export.

Table 3.8 Percentage in total inland freight tonne-km 2011

| Country | % | Rank |
| --- | --- | --- |
| Holland | 36.7 | 1 |
| Romania | 21.7 | 2 |
| Belgium | 18.5 | 3 |
| Bulgaria | 15 | 4 |
| Germany | 11.2 | 5 |
| Croatia | 5.7 | 6 |
| EU27 | 6.2 |  |

Source: Eurostat

Compliance with International Standards for Waterway Draught: The UN defines the Danube as an international waterway and the draught required for such status is 2.5m, although 2.8m is preferable. Several sections of the Danube regularly fall below 2.5m draught and navigation was very difficult on the lower river for more than 38 days in September and October 2011 because of insufficient water levels.

A convoy comprises either one motor cargo vessel (a ship with its own cargo hold) or a pusher and one or more barges rigidly coupled to the freighter or pusher.

Table 3.9 Barge operation comparison between the Danube and Rhine

| Pushed Convoy with four barges |  |  |  |
| --- | --- | --- | --- |
|  | Danube | Rhine |  |
| Length | 193 m | 193 m |  |
| Width | 22 m | 22.80 m |  |
| Draught | 2.70 m | 3.70 m |  |
| Total deadweight (all told) | 7,000t | 11,000t |  |

Source VNF: Voies navigable de France

The table above provides a basic comparison between the Danube and the Rhine that shows whilst a Danube convoy can be the equivalent of 280 HGVs, a Rhine convoy can be the equivalent of 440 HGVs. The reason a Rhine convoy can handle 57% more than a Danube convoy is because of the additional 0.80m in width and the extra 1m in draught. This allows the barges to hold more deadweight. The increased dimensions are incremental (extra 4% width and 37% draught), but add up to a significant increase in capacity (57%), the sum being greater than the parts.

The operational speed upstream on the Romanian Danube is 10-12kph for a convoy and the operational speed downstream is 16-18kph. These figures are similar on the Danube-Black Sea canal. From Braila out to sea there is no speed limit. However on the Sulina navigation the speed is limited by regulation based on type and size of vessel and this typically is 15-20kph.

Romania currently spends €17m on maintaining the Danube. This compares to €1m that Bulgaria spends and €80m that Austria spends. Taking into account the length of the Danube these countries must maintain gives the following maintenance budgets.

- Table 3.10 – Maintenance budgets on the River Danube for selected countries

| Country | Danube Maintenance Budget | Length of Danube Responsible for | Budget per km |
| --- | --- | --- | --- |
| Romania | €17m | 1,500km | €11,333/km |
| Austria | €80m | 320km | €250,000/km |
| Bulgaria | €1m | 470km | €2,128/km |

Source: AFDJ Giurgiu

Austria spends significantly more than Romania or Bulgaria, this is despite it having by far the shorter stretch to maintain. Bulgaria’s Danube maintenance budget is minimal, especially when considered alongside Romania’s Danube maintenance budget. What is unclear from the figures is why the countries have allocated the budget that they have, particularly with regards to the maintenance issues that they must contend with.

Romania’s major sea port is at Constanta, located on Romania’s Black Sea coast. Constanta is now home to the largest container port in the Black Sea and is strategically situated to feed freight into the heartland of Central and Eastern Europe.

It is a major regional container port. Constanta is not amongst the top 20 container ports (by volume) in Europe as it handled 684,000 TEU (Twenty foot equivalent units) in 2012, but it was back in 2007 when the port handled 1.41 million TEU. Constanta is used as a hub by the shipping lines with a number of containers being transited.

The modal share of containers leaving Constanta (excluding transiting) has been estimated as currently:

Road – 56%

Rail – 41%

Barge – 3%

It is possible to compare Constanta’s modal split of containers with other major European container ports:

Table 3.11 Modal Split of Containers for some Major European Ports

| Modal Split | Road | Rail | Barge |
| --- | --- | --- | --- |
| Constanta | 56% | 41% | 3% |
| Antwerp | 56% | 44% (combined) |  |
| Amsterdam | 55% | 45% (combined) |  |
| Rotterdam | 54% | 11% | 35% |
| Hamburg | 62% | 36% | 2% |
| Bremen | 50% | 46% | 4% |
| Felixstowe | 69% | 28% | 3% (coastal) |

Many of these ports have their own future plans about modal split in an attempt to move containers from road to more sustainable modes of transport. In some cases this is for environmental reasons, in others it is because it allows the port to make better use of limited space and have a greater hinterland (some of these ports cannot develop much more at their current site) to react to increased demand.

Table 3.13 Planned Future Modal Split of Containers for some Major European Ports by 2020

| Planned Future Modal Split | Road | Rail | Barge |
| --- | --- | --- | --- |
| Antwerp | 42% | 15% | 43% |
| Amsterdam | 40% | 60% (combined) |  |
| Rotterdam | 40% | 15% | 45% |
| Hamburg | 45% | 53% | 1% |

Some of these ports intend to increase the modal split of containers leaving the port by rail and barge. Constanta is well placed to do the same and could learn from the practices and operations at some of these ports. It is considered that Constanta could expand the use of both modes of transport and particular barge traffic when looking at planned percentages at Antwerp and Rotterdam.

Aviation Trends in European Context

In 2011, a total of 10.8 million passengers travelled through all Romanian airports.  Of this total, over 50% of passengers travelled through Bucharest’s Henri Coanda International Airport – showing the strong capital-centric nature of the country’s air transport system.

A total of 21 airfields are listed in Romania’s Aeronautical Information Publication (AIP), and seven of the country’s airports are not currently served by scheduled flights.  The development of the Romania air transport infrastructure aims to provide regional centres with a means of fast transport to Bucharest, the capital, along with other regional centres.  Air transport is also relied upon to provide international connectivity.  As the country’s highway network remains in development, and even the fastest section of the rail network (currently between Bucharest and Constanta) are subject to speed restrictions, air travel is a preferred choice for domestic transport where fast journey times are a critical factor.

The construction of the new Brasov-Ghimbav International Airport has also commenced. The new runway, destined to receive medium-haul aircraft, will be 2,820 metres long and 45 metres wide.  The opening of the airport is likely to increase the investment potential of the area, mainly in the tourism industry, as well as new development opportunities for small and medium-sized companies. The airport is scheduled to open at the end of 2014/beginning of 2015.

For the purposes of benchmarking, the air transport market of Romania has been compared to the market in the Czech Republic which joined the EU only three years earlier. Table 3.14 below compares the two countries in terms of a small number of key socio-economic and political indicators.

Table 3.14 Socio-economic and political comparison between Romania and the Czech Republic

| 2011 statistic | Romania | Czech Republic |
| --- | --- | --- |
| GDP per capita | €6,100 | €14,800 |
| Area | 238,391km2 | 78,866km2 |
| Population | 21,413,815 | 10,486,731 |
| Population density | 93/km2 | 135.5/km2 |
| Post-Communist revolution | 1989 | 1989 |
| EU accession | 2007 | 2004 |
| Key industry sectors | Construction, automotive, machinery, chemicals | Machinery, chemicals, foods, telecommunications |

Source:Eurostat

While Romania is a larger and more populous country than the Czech Republic, the latter state is significantly wealthier, despite having entered the EU only three years earlier. As a smaller country, it would be expected that the domestic air market in the Czech Republic was smaller, and Table 3.* shows this to be correct. Nevertheless, in 2011 the Czech Republic saw 19% more air passengers in total than Romania, although the domestic market was significantly smaller as expected. The larger international market could be explained by the country’s higher wealth, giving Czechs a higher propensity and ability to travel abroad, and a higher level of international business activity is likely to encourage business from abroad.

The Czech air freight market in 2011 was more than 2.6 times larger than that in Romania; perhaps reflecting a larger manufacturing base in the country or different industry sectors may trigger greater levels of air freight activity, such as foods or telecommunications.

In terms of the penetration of low-cost carriers, the Romanian market ranks more favourably than the Czech Republic. This factor may in part be explained by the lower wealth of Romania; those travelling from Romania may be attracted to the lower fares of low-cost carriers, rather than ‘full-service’ airlines. The impact of low-cost carriers can be felt mainly on international routes; the domestic market in Romania sees little penetration by low-cost carriers.

Table 3.15 Comparison of air transport markets in Romania and the Czech Republic

| 2011 statistic | Romania | Czech Republic |
| --- | --- | --- |
| Total air pax | 10,810,570 | 12,824,895 |
| Domestic air pax | 1,129,310 | 229,762 |
| International pax | 9,681,260 | 12,595,133 |
| Air freight | 26,156t | 69,106t |
| LCC penetration | 37% | 29% |
| Total flights | 95,836 | 152,215 |
| Total domestic flights | 24,492 | 7,075 |
| Total international flights | 71,344 | 145,140 |

Source :Eurostat

Intermodal Transport Trends

The table below demonstrates that Romania transports far fewer containers by rail than many other European countries (although in excess of neighbours Bulgaria). The table shows that containerisation is particularly advanced in Italy, Turkey, Austria and Germany but Bulgaria and Romania are lagging behind. The volume and percentage in countries like Austria are high due to the Alpine effect where on certain routes it is compulsory to use rail services and there is a considerable volume of transit traffic. Nevertheless Romania could potentially also attract significant volumes of transit traffic.

Table 3.16 Annual railway transport of goods (containers and swap bodies) in intermodal transport units (in 000s tonnes) and the % of total rail freight containers represent

| Country | Total volume of containers and swap bodies ('000 tonnes) |  | % in containers of total goods transported by rail |  |
| --- | --- | --- | --- | --- |
|  | 2011 | 2012 | 2011 | 2012 |
| Austria | 16,312 | 15,806 | 18.2% | 19.0% |
| Bulgaria | 789 | 664 | 5.8% | 5.6% |
| Czech Republic | 7,321 | 7,852 | 8.4% | 9.5% |
| Germany | 64,301 | 66,230 | 17.2% | 18.1% |
| Italy | 34,275 | 33,985 | 43.4% | 45.1% |
| Romania | 2,611 | 2,372 | 4.6% | 4.7% |
| Turkey | 7,601 | 8,264 | 30.7% | 33.2% |
| United Kingdom | 11,098 | 11,742 | 11.1% | 10.2% |

(Source Eurostat)

Transport and Economic Development

Efficient transport is a critical component of economic development, globally and nationally. Transport availability affects global development patterns and can be a boost or a barrier to economic growth within individual nations. Transportation investments link factors of production together in a web of relationships between producers and consumers to create a more efficient division of production, leverage geographical comparative advantage, and provide the means to expand economies of scale and scope. Transport’s contribution to economic development operates through the following mechanisms:

Increasing business efficiency, through time savings and improved reliability for business travellers, freight and logistics operations.

Increasing business investment and innovation by supporting economies of scale or new ways of working.

Supporting clusters and agglomerations of economic activity. Transport improvements can expand labour market catchments, improve job matching, and facilitate business to business interactions. Such productivity effects extend across commuter and business travel catchment areas.

Improving the efficient functioning of labour markets, increasing labour market flexibility and the accessibility of jobs. Transport can facilitate geographic and employment mobility in response to shifting economic activity e.g. in response to the forces of globalisation, new technological opportunities, and rising part-time and female participation in the labour market.

Increasing competition by opening up access to new markets. Transport improvements can allow businesses to trade over a wider area, increasing competitive pressure and providing consumers with more choice.  Romania will benefit from both improvements in internal connectivity and international connectivity.

Increasing domestic and international trade by reducing the costs of trading.  Domestic trade links are particularly important to the economic success of some urban areas.

Attracting globally mobile activity to Romania by providing an attractive business environment and good quality of life. Such effects are of increasing importance but extremely difficult to quantify. However, the strategic focus of transport policy can be guided by the survey evidence which suggests that both domestic and international transport links can be important to attracting, retaining and expanding such activity, and that there is much commonality

The fundamental requirement from the Master Plan is that the identified investment plan, however funded and by whatever agency, must make a positive contribution to Romania’s economic development. The economic benefits from the overall strategy must be greater than the investment required to produce them.

Road Transport

Road Transport

Structure of this Chapter

This chapter sets out the way in which the recommended improvements to the road transport sector were identified. The structure of this chapter is as follows:

Current Situation;

Strategic Objectives;

Operational Objectives;

Cost Benefit Analysis (CBA);

Multi Criteria Analysis (MCA);

Maintenance Needs;

Institutional Reform; and

Summary of Interventions.

Extensive use is made of the National Transport Model in providing analysis and outputs to support the conclusions reached, particularly with regard to objectives and the cost benefit and multi criteria analyses. The overall approach was to identify potential individual projects, based on specific identified issues, and then test these individually to see how they performed in terms of CBA and MCA.

Projects were then grouped into scenarios, which often involved some refinement of individual projects, in order to provide a cohesive and logical network at each stage.

Current Situation

Context - Mode Share for Road Text.

According to the National Institute of Statistics, road accounts for almost 75% of all passenger kilometres, and just under 50% of all goods kilometres, travelled in Romania (INSSE 2010 data). This makes it the most significant mode in both cases as clearly illustrated in Figure 4.1. The goods trip kilometres are based on tonne kilometres by mode.

Figure 4.1 Proportion of Trip Kilometres by Mode (2010)

Source: National Institute of Statistics (INSSE, 2010 Data)

The highway network in Romania is classified into five categories:

Motorways (Autostrăzi) - A;

National and European Roads (Dumuri Naționale si Europene) – DN/E;

National Roads (Dumuri Naționale) – DN;

County Roads (Dumuri Județene) – DJ; and

Communal Roads (Dumuri Comunale) – DC.

The motorway and national road network accounts for just 20% of the entire network as summarised in Table 4.1.

Table 4.1 Length of Road Network by Category

| Road Type | Kilometres | Proportion |
| --- | --- | --- |
| Motorway | 362.6 | 0.5% |
| National European | 5,697.7 | 7.1% |
| Other National | 9,930.9 | 12.4% |
| County | 36,009.8 | 45.1% |
| Commune | 27,780.8 | 34.8% |
| Total | 79,781.7 | - |

Source: AECOM analysis of individual DRDP data

Safety

Romania has a significant road accident problem as clearly evidenced by comparisons with other countries in the European Union (EU). The European Commission use three separate indicators to measure road safety as follows:

Fatalities per million inhabitants;

Fatalities per 10 billion passenger kilometres; and

Fatalities per million passenger cars.

In this order, Romania’s score and ranking is as follows:

24th out of 28 – 94 versus an EU average of 60;

28th out of 28 – 259 versus an EU average of 61; and

28th out of 28 – 466 versus an EU average of 126.

From this data, it is fair to conclude that Romania has the worst road fatality accident rate in Europe. During the period 2007 to 2012, there were a total of 8,401 fatalities on the National Network alone. This equates to an average of 1,400 fatalities per annum purely on the National Network which, as highlighted later, only carries some 20% of all network kilometres.

The following table provides a breakdown of all accidents within the database in terms of the road classification on which they occur. This is to highlight the scale of contribution of accidents on the national network to the overall total.

Table 4.2 Road Accident Numbers by Type of Road

| Type | 2007 | 2008 | 2009 | 2010 | 2011 | 2012 | 2007-12 Average |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Motorway | 120 | 139 | 101 | 115 | 107 | 131 | 119 | 0.44% |
| National | 7,092 | 8,628 | 8,195 | 7,483 | 7,119 | 7,192 | 7,618 | 28.09% |
| County | 3,262 | 4,318 | 4,295 | 3,841 | 3,924 | 3,929 | 3,928 | 14.48% |
| Other | 14,188 | 16,776 | 16,021 | 14,557 | 15,498 | 15,676 | 15,453 | 56.98% |
| Total | 24,662 | 29,861 | 28,612 | 25,996 | 26,648 | 26,928 | 27,118 | - |

Source: AECOM analysis of police records

The contribution of around 30% of all accidents needs to be seen in the context of the motorway and national road accounting for just under 20% of all road network kilometres. The economic impact of these accidents on the national network is €1.2billion per annum.

Single carriageway roads are recognised as the most dangerous with recent Euro RAP studies indicating that, across Europe, the average risk of single carriageway road is four times that of a motorway. This is also reflected in the Romanian statistics which show a significantly greater risk on single carriageway roads: over six times worse than motorways for all national roads and over three times if only rural national roads are considered. Currently almost 90% of the national network is single carriageway standard and this undoubtedly contributes to the poor accident record and the consequent significant economic cost.

Level of Service

It is common practice to consider level of service in terms of volume to capacity ratios but this is not appropriate in the Romanian context. Whilst flow levels for the majority of the national network are not high, as a proportion of the theoretical capacity, the effect of the high proportion of single carriageways is evident. Even at low flow levels, single carriageways offer limited safe overtaking opportunities particularly when the proportion of goods vehicles is high which is the case on a national network.

Setting aside the small proportion of the national network that is of motorway standard (less than 3%), the average speed for the inter-urban element of the national network is around 66 kph. This is not considered to be adequate for a national network where, international comparisons suggest, speeds of between 90 and 100 kph would be more reasonable. The requirements of TEN-T, which are for high quality motorway or expressway standard routes, suggest speeds of at least 100kph should be achieved or exceeded.

Figure 4.2 National Network – Actual versus Target Speeds

Source: AECOM National Transport Model

Only those roads highlighted green are close to achieving what is considered a reasonable speed for a national network. Table 4.3 shows the length of road within each speed band. Over half of the national network has a speed less than 70% below the target.

Table 4.3 Assigned Speed vs. Target Speed of Road in Each Band

| Ratio Interval | Percentage |
| --- | --- |
| > 0.9 | 9.6% |
| 0.8 to 0.9 | 17.1% |
| 0.7 to 0.8 | 22.4% |
| < 0.7 | 50.9% |

Source: AECOM National Transport Model

Slow road journey speeds represent inefficient use of both work and non work time and have a significant adverse economic impact, reducing the opportunities for travel for individuals and businesses. In order to compete at an EU level the national network needs to improve journey times both in absolute terms and with regard to reliability.

The National Model has also been used to quantify, in both scale and locality, the amount of lost time on the network. The analysis took the following form:

Lost Time= i=0nFlowiTarget Time-Actual Time×VoTiLength

Where each model link was evaluated and:

Flow 	=	AADT;

i 		=	vehicle purpose (e.g. commuting, business etc.); and

VoT		=	Value of Time.

The results of that analysis for the base year are shown in Figure 4.3.

In monetary terms, this equates to a total of €1,420 million per annum.

Figure 4.3 Analysis of Lost Time – Base Year

Source: AECOM National Transport Model

There are a number of significant corridors highlighted by this analysis including, but not limited to:

Bucharest – Ploiesti – Brasov;

Bucharest – Targoviste – Gaesti;

Bucharest – Giurgui & Alexandria;

Pitesti – Sibiu;

Pitesti – Craiova;

‘Corridor IX’ i.e. Bucharest – Buzau – Bacau – Botosani;

Routes northeast of Braila / Galati; and

Oradea – Cluj – Targu Mures – Iasi.

The delay is widespread with poor speeds commonplace and consequent poor connectivity / accessibility for many parts of the country.

Road Network Condition

CESTRIN have provided data to AECOM on the state of the national network as at 1st January 2012. The data provided includes information on the type of surfacing, an assessment of the quality of that surfacing (based on IRR surface roughness values) and also the type of topography through which the road section runs.

Table 4.4 considers the proportion of the network that is considered good, average or poor broken down by surface type and topography.

Table 4.4 National Road Network – Surface Condition

|  |  | Asphalt |  | Concrete | Paved | Light Bituminous Pavement | Stone | Earth | Total |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hill | Good |  | 19.2% | 0.8% | 0.0% | 0.3% | 0.0% | 0.0% | 20.4% | 40.6% |
|  | Average |  | 11.5% | 1.4% | 0.1% | 1.4% | 0.3% | 0.0% | 14.6% |  |
|  | Poor |  | 3.8% | 0.9% | 0.0% | 0.8% | 0.1% | 0.0% | 5.6% |  |
| Mountain | Good |  | 10.3% | 0.4% | 0.0% | 0.1% | 0.0% | 0.0% | 10.8% | 18.3% |
|  | Average |  | 4.0% | 0.2% | 0.0% | 0.2% | 0.1% | 0.0% | 4.5% |  |
|  | Poor |  | 1.2% | 0.3% | 0.0% | 0.5% | 0.9% | 0.0% | 2.9% |  |
| Plains | Good |  | 19.9% | 1.0% | 0.1% | 0.7% | 0.0% | 0.0% | 21.7% | 41.2% |
|  | Average |  | 10.4% | 0.5% | 0.0% | 0.7% | 0.1% | 0.0% | 11.7% |  |
|  | Poor |  | 5.5% | 0.6% | 0.1% | 1.4% | 0.1% | 0.1% | 7.7% |  |
| Total |  | 85.7% |  | 6.3% | 0.2% | 6.0% | 1.6% | 0.1% | 100.0% | 100.0% |

Source: AECOM analysis of CESTRIN Data

The notable point from this is that only just over 50% of the national network is classified as good with a further 30% average and 20% poor. The expectation should be that the national network should be at the top end of the standard for any country. Further information has been obtained on the proportion of national road network for which the design life has expired; this is shown in Figure 4.4. 48% of all routes have some element classified poor. Of these the average proportion, by length, is 24%. There is no distinction by topography, thereby suggesting a network wide issue.

Whilst there are anecdotal references to the relative quality of the Romanian network and some subjective surveys by, for example, the World Economic Forum, in their report on “Global Competitiveness (2011–2012)”, places Romania 137th out of 142 countries considered with regard to the quality of the road infrastructure, we have not been able to obtain statistically robust comparative data. As a proxy, we have looked at the level of motorway provision across Europe as motorway implies a level of quality and standard. The results are presented in the Figure 4.5 and Figure 4.64.6.

From both indicators used, it is clear that the relative proportion of motorway standard routes is low compared to the rest of Europe.

Figure 4.4 Proportion of National Network Beyond Service Life

Source: CNADNR

Figure 4.5 Length of Motorway per 100,000 Population (2005)

Source: AECOM analysis of Eurostat Data

Figure 4.6 Proportion of Network Classified as Motorway (2005)

Source: AECOM analysis of Eurostat Data

Road Maintenance

There are currently in the order of 50 framework agreements in place covering maintenance activities on the national network which are subject to renewal every two years. In the UK, which has around ten times as much motorway network, there are 13 area based framework arrangements. Typical contract durations of between 5 and 10 years are prevalent in the rest of Europe.

The case study of the Rijkswaterstaat in the Netherlands indicated savings of 20% in road maintenance costs through the extension of contract terms from 1–2 years to 5–7 years and by bundling more maintenance activities together in the same contract. In addition to these contract savings, longer contract periods incentivise the suppliers to invest in more efficient plant as the ‘payback’ will be more transparent.

Consultation with suppliers in the UK has also revealed that bulk-buying of materials across a large programme can provide greater efficiency and savings are likely to be significant. Contractors could use their fixed and mobile plant with significantly greater efficiency if contracts included both local and national roads.

The level of maintenance spend required has been determined with reference to the World Bank report “Road and Railway Cost Recovery in Romania”, prepared by AECOM. The model used in that project derived Base Case Lifecycle Expenditure (CapEx), Operations and Maintenance Expenditure (OpEx) and Revenues forecasts for the road and railways networks in Romania.

The resulting Base Case OpEx and CapEx forecasts were governed by the following key assumptions:

Asset physical characteristics

Key socio-economic factors such as GDP growth and other specific indicators (for example car ownership growth in the case of the road model);

Key project environmental factors such as initial and forecast traffic volumes;

Local standards and practices applicable to O&M and whole life costs;

Assumed unit rates (derived from benchmarks and local knowledge);

Proposed maintenance strategy (for example pavement in the case of the road model); and

Proposed rehabilitation programs.

The resulting revenues forecasts were governed by the following key assumptions:

Asset physical characteristics;

Key socio-economic factors such as GDP growth and other specific indicators (for example car ownership growth in the case of the road model);

Key project environmental factors such as initial and forecast traffic volumes;

User charges; and

Other sources of revenues.

The work for the National Transport Strategy was able to refine further this model based upon our improved knowledge of the condition of the road network as developed with assistance from CESTRIN. The outputs of that process are provided in Table 4.5 below.

Table 4.5 Required Road Maintenance and Lifecycle Costs (million Lei)

| Period | Total Cost in period |  |  | Average Annual |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Maintenance | Lifecycle | Total | Maintenance | Lifecycle | Total |
| 2014-2020 | 6,033.3 | 36,924.6 | 42,957.9 | 861.9 | 5,274.9 | 6,136.8 |
| 2021-2030 | 8,260.4 | 59,113.5 | 67,373.9 | 826.0 | 5,911.3 | 6,737.4 |

Source: AECOM

Future Trends

Whilst road is already a highly significant mode for passenger transport in Romania, it is also worth reflecting on the level of car ownership currently, and how that may change in the future, as that directly influences mode choice.

There has been a steady growth in car ownership levels in Romania with the long term trend averaging around 5% per annum. This needs to be seen in context however with car ownership levels in the rest of Europe. The comparison is provided in Figure 4.7 with the values for Romania highlighted in red.

Figure 4.7 Comparison of EU Car Ownership Levels (2010)

Source: AECOM analysis of Eurostat Data

Clearly car ownership levels are currently lagging some way behind the rest of Europe. It is reasonable to expect that they are going to increase significantly over time which will have a direct impact on the likelihood of road becoming the mode of choice for even more passenger journeys than presently is the case.

The road sector is the most dominant element of the Romanian transportation system, in terms of the transportation of both passengers and goods. The road network carries almost 75% of all passenger kilometres and just under 50% of all goods kilometres. As the Reference Case model relates to a forecast year of 2020, growth has been applied to the various trip demand matrices which feed into the modelling process. The following Table 4.6 summarises the trip matrix totals for the 2011 Base case and the 2020 Reference Case.

The Reference Case is the modelled scenario which includes all schemes which have sufficient levels of commitment for it to be reasonably assumed that they will be constructed by 2020. Information on the schemes included within the Reference case are provided later in Chapter 10.

This shows that the overall demand for road travel is forecast to increase by just under 20%. In absolute terms, the growth in car trips is more significant but there is a proportionately higher growth in road freight.

Table 4.6 Base 2011 and Reference Case 2020 Trip Totals

| Vehicle Type | Trip Purpose | Trip Matrix Totals |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Base 2011 |  | Reference 2020 |  | Absolute Change | Percentage Change |
| Car | Business | 282,410 | 12.0% | 350,072 | 12.9% | +67,662 | +24.0% |
| Car | Commuting | 780,498 | 33.1% | 757,071 | 27.9% | -23,427 | -3.0% |
| Car | Private Business | 1,056,692 | 44.9% | 1,309,886 | 48.3% | +253,194 | +24.0% |
| Car | Vacation | 236,231 | 10.0% | 295,534 | 10.9% | +59,303 | +25.1% |
| Car | Total | 2,355,831 |  | 2,712,563 |  | +356,732 | +15.1% |
| Goods | Total | 582,982 |  | 762,913 |  | +179,931 | +30.9% |
| All | Total | 2,938,813 |  | 3,475,476 |  | +536,663 | +18.3% |

Source: AECOM National Transport Model

Within the following sections, we present the results of the Reference Case model run with reference to the 2011 Base Year equivalent.

Tables 4.7 to 4.9 show the changes forecast by the model in terms of vehicle kilometres, vehicle hours and average speed for car based trips.

Table 4.7 Vehicle Kilometres (Cars) – Base 2011 and Reference Case 2020

| Road Type | Vehicle Kilometres - Cars |  |  |  |
| --- | --- | --- | --- | --- |
|  | Base 2011 | Reference 2020 | Absolute Change | Percentage Change |
| Motorway | 5,221,090 | 16,547,260 | +11,326,170 | +216.9% |
| National | 50,464,502 | 66,597,842 | +16,133,340 | +32.0% |
| County | 8,235,621 | 9,816,889 | +1,581,269 | +19.2% |
| Local | 2,399,483 | 2,764,667 | +365,184 | +15.2% |
| All journeys | 66,320,696 | 95,726,658 | +29,405,962 | +44.3% |

Source: AECOM National Transport Model

Table 4.8 Vehicle Hours (Cars) – Base 2011 and Reference Case 2020

| Road Type | Vehicle Hours - Cars |  |  |  |
| --- | --- | --- | --- | --- |
|  | Base 2011 | Reference 2020 | Absolute Change | Percentage Change |
| Motorway | 42,449 | 130,728 | +88,279 | +208.0% |
| National | 776,878 | 1,045,929 | +269,050 | +34.6% |
| County | 181,060 | 215,950 | +34,890 | +19.3% |
| Local | 74,264 | 86,166 | +11,902 | +16.0% |
| All journeys | 1,074,651 | 1,478,773 | +404,122 | +37.6% |

Source: AECOM National Transport Model

Table 4.9 Vehicle Speeds (Cars) – Base 2011 and Reference Case 2020

| Road Type | Average Speed - Cars |  |  |  |
| --- | --- | --- | --- | --- |
|  | Base 2011 | Reference 2020 | Absolute Change | Percentage Change |
| Motorway | 123.0 | 126.6 | +3.6 | +2.9% |
| National | 65.0 | 63.7 | -1.3 | -2.0% |
| County | 45.5 | 45.5 | -0.0 | -0.1% |
| Local | 32.3 | 32.1 | -0.2 | -0.7% |

Source: AECOM National Transport Model

The increase, for car trips, of 44% for vehicle kilometres against an increase of 15% in car trips highlights the trip lengthening which occurs as a result of increased accessibility and improved inter-urban road networks. The lesser increase in vehicle hours, compared to vehicle kilometres, also points to improved speeds as a consequence of improvements in the Reference Case Scenario.

Figure 4.8 illustrates the impact of the reference case, as forecast by the transport model, relative to the base year for cars across the indicators discussed earlier.

Figure 4.8 Base Year to Reference Case (2020) – Cars

Source: AECOM National Transport Model

Tables 4.10 to 4.12 show the changes forecast by the model in terms of vehicle kilometres, vehicle hours and average speed for all goods vehicle trips.

Table 4.10 Vehicle Kilometres (Goods) – Base 2011 and Reference Case 2020

| Road Type | Vehicle Kilometres - Goods |  |  |  |
| --- | --- | --- | --- | --- |
|  | Base 2011 | Reference 2020 | Absolute Change | Percentage Change |
| Motorway | 4,122,421 | 12,645,813 | +8,523,392 | +206.8% |
| National | 33,530,573 | 38,909,327 | +5,378,754 | +16.0% |
| County | 3,329,595 | 3,856,655 | +527,060 | +15.8% |
| Local | 556,033 | 649,580 | +93,547 | +16.8% |
| All journeys | 41,538,622 | 56,061,375 | +14,522,753 | +35.0% |

Source: AECOM National Transport Model

Table 4.11 Vehicle Hours (Goods) – Base 2011 and Reference Case 2020

| Road Type | Vehicle Hours - Goods |  |  |  |
| --- | --- | --- | --- | --- |
|  | Base 2011 | Reference 2020 | Absolute Change | Percentage Change |
| Motorway | 68,744 | 199,828 | +131,084 | +190.7% |
| National | 744,698 | 899,495 | +154,797 | +20.8% |
| County | 105,872 | 124,002 | +18,129 | +17.1% |
| Local | 26,163 | 30,786 | +4,623 | +17.7% |
| All journeys | 945,477 | 1,254,110 | +308,633 | +32.6% |

Source: AECOM National Transport Model

Table 4.12 Vehicle Speeds (Goods) – Base 2011 and Reference Case 2020

| Road Type | Average Speed - Goods |  |  |  |
| --- | --- | --- | --- | --- |
|  | Base 2011 | Reference 2020 | Absolute Change | Percentage Change |
| Motorway | 60.0 | 63.3 | +3.3 | +5.5% |
| National | 45.0 | 43.3 | -1.8 | -3.9% |
| County | 31.4 | 31.1 | -0.3 | -1.1% |
| Local | 21.3 | 21.1 | -0.2 | -0.7% |

Source: AECOM National Transport Model

Figure 4.9 illustrates the impact of the reference case, relative to the base year for goods vehicles across the same indicators used for cars. The growth in goods traffic is greater than that for cars, (31% compared to 15%) but the overall pattern of impacts is similar to that reported on for cars.

There is a significant growth in vehicle kilometres on motorways (207%) and a more modest growth on the National network (16%). As with cars, the road network, and motorways in particular, are becoming a much more attractive mode as a consequence of the Reference Case schemes.

Figure 4.9 Base Year to Reference Case (2020) – Goods

Source: AECOM National Transport Model

The Reference Case model runs, forecast significant increases on many of the main corridors for car traffic, reflecting a number of factors raised earlier in this report, notably:

Increasing levels of car ownership;

Improvements to the national highway network; and

Reductions in the performance of rail services, as discussed in Chapter 6.

Without improvements in the public transport offer, the result is a mode shift towards car with an increase in car trips of over 15% against a total increase in passenger trips across all modes of just under 11%. Longer trip lengths are also forecast with passenger kilometres increasing for car by 44% against an all modes figure of 27%.

The most significant increases are on the East-West corridor of Constanta-Bucharest-Sibiu-Arad but these are also increases on the North-South movements on Bucharest-Buzau-Bacau and Sibiu-Cluj.

Road freight experiences similar patterns of growth with volumes up by 31% against a total figure of around 27%. Again the trip length is increasing with road freight tonne kilometres increasing by 36% against an all modes figure of 32%.

In summary, the transport model forecasts significant growth in trips on the road network, both cars and freight, which will result in capacity and level of service issues if there are not improvements to the network. The main problems which have been identified may be summarised as follows:

Air pollution and noise resulting from poor accessibility;

Poor accessibility to international trade;

High travel times leading to uncompetitive service;

High administration costs on local and national hauliers;

Delays at border crossings;

High fatal accident rates compared to rest of EU;

Lack of secure and safe goods vehicle parking;

Inadequate asset maintenance;

Institutional and organisation stability & capacity issues; and

Excessive axle loads causing significant road surface damage.

Strategic Objectives

The strategic objectives for the Road Sector have been identified as:

Improving the population mobility and freight traffic along the TEN – T Core and Comprehensive network through the execution of a motorway and expressway network;

Ensuring the accessibility of population and business environment to the TEN – T Core and Comprehensive network through the execution of  the national connectivity corridors;

Ensuring a safe and operational road transport network which contributes to the reduction in the number of road accidents, in line with Clause 33 of EU Regulation No. 1315/2013 which seeks to ensure the security and safety of passengers and freight movements, as well as the reduction of journey times;

Ensuring international accessibility through the connection with the neighbour countries; and

Ensuring a more environmentally friendly transport network through the implementation of by-pass schemes.

In the context of limited funding, there is a need to focus national policy and investment on those parts of the network which, in terms of role and function, form the primary sections of the national network.  We refer to this as the Primary Economic Network (PEN).

The definition of the Primary Economic Network for roads is a function of the following factors:

Connectivity between significant population centres with significant centres being defined by population levels in excess of 125,000;

Connectivity with international gateways such as border crossings, airports and ports; and

Economically significant corridors, defined as those which carry high volumes of goods traffic.

The population centres and border crossing points that have been used to define the primary network are set out in Table 4.13 and Table 4.14.

Table 4.13 Top 20 Cities by Population

| Order | City | Population | Order | City | Population |
| --- | --- | --- | --- | --- | --- |
| 1 | Bucharest | 1,883,425 | 11 | Brăila | 180,302 |
| 2 | Cluj-Napoca | 324,576 | 12 | Arad | 159,704 |
| 3 | Timișoara | 319,279 | 13 | Pitești | 155,383 |
| 4 | Iași | 290,422 | 14 | Sibiu | 147,245 |
| 5 | Constanța | 283,872 | 15 | Bacău | 144,307 |
| 6 | Craiova | 269,506 | 16 | Târgu Mureș | 134,290 |
| 7 | Brașov | 253,200 | 17 | Baia Mare | 123,738 |
| 8 | Galați | 249,342 | 18 | Buzău | 115,494 |
| 9 | Ploiești | 209,945 | 19 | Botoșani | 106,847 |
| 10 | Oradea | 196,367 | 20 | Satu Mare | 102,441 |

Source: Romanian National Census

Table 4.14 Top 10 Border Crossings (Goods Vehicle Flows – 2011 AADT)

| Order | Border Crossing | HGV Volumes | Proportion of Countrywide Border Flows |  |
| --- | --- | --- | --- | --- |
|  |  |  | Individual | Cumulative |
| 1 | Nadlac | 1,778 | 22.7% | 22.7% |
| 2 | Bors | 1,498 | 19.2% | 41.9% |
| 3 | Giurgiu | 1,126 | 14.4% | 56.3% |
| 4 | Varsad | 680 | 8.7% | 65.0% |
| 5 | Stanca-Costesti | 501 | 6.4% | 71.4% |
| 6 | Petea | 298 | 3.8% | 75.2% |
| 7 | Bechet | 290 | 3.7% | 78.9% |
| 8 | Siret | 234 | 3.0% | 81.9% |
| 9 | Calafat | 155 | 2.0% | 83.9% |
| 10 | Negru Voda | 153 | 2.0% | 85.8% |

Source: Romanian Customs

The corridors with significant goods vehicle traffic flows, as derived from the National Transport Model, are shown in Figure 4.10 together with the top twenty population centres and top 10 border crossing points. We also provide a similar plot but taking account of total vehicle flows – this is provided in Figure 4.11.

The combination of population centres, vehicle flows and primary border crossings leads to a definition of the Primary Economic Network as shown in Figure 4.12. This takes the results from Figure 4.10 and Figure 4.11 and highlights the relevant links in red. There is close correlation between the PEN and the TEN-T corridors, as shown in Figure 4.13.

We have also analysed the major movements at an aggregate level to identify the most important international, regional and county level movements. Figures 4.14 to 4.16 present the results of that analysis.

Figure 4.10 Goods Vehicle Flows, Population Centres and Major Border Road Crossings

Source: AECOM National Transport Model

Figure 4.11 Total Vehicle Flows, Population Centres and Major Border Road Crossings

Source: AECOM National Transport Model

Figure 4.12 Proposed Primary Economic Network (Overlaid on National Model Analysis)

Source: AECOM National Transport Model

Figure 4.13 Correlation Between PEN (Red) and TEN-T (Purple)

Source: AECOM National Transport Model

Operational Objectives

Introduction

The proposed interventions will support the following operational objectives as indicated in Figure 4.14 which maps them in relation to the high level objectives and problems.

Figure 4.14 Links Between Problems, Interventions and Operational Objectives

Source: AECOM analysis

Table 4.15 summarises the main problems using supporting evidence, with the operational objectives described above ‘mapped’ to the issues. The proposed interventions and their suitability for testing using the National Transport Model are also shown.

Table 4.15: High Level Objectives and Supporting Evidence for Passenger Rail and Freight

| High level objective | Problem |  |  | Summary of supporting evidence / metrics | Operational objectives |  | Intervention(s) | Testing plan |  |  |  | Project/ Intervention number / reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | No. | Category | Description |  | Description | Ref. | Description / Alternatives | Suitable for testing | AADT Ref. | Road Improvment to be tested | Level of Aggregation to be Tested |  |
| Sustainability | 01 | Inadequate Maintenance | Inadequately maintained existing and future assets on the national network jeopardise its long-term sustainability | Only just over 50% of the national network is classified as good condition with a further 30% average and 20% poor. Over 60% of the national network is categorised as being beyond its service life. - See Appendix E: Maintenance | Improve maintenance as to ensure ‘good’ standard surface conditions (as defined by CESTRIN viabilitate survey) at least on the primary economic network and good/average for the rest of the national network. | OR01 | Adopt best practice PMS approach to prioritisation of roads maintenance. In particular ensure by regulation that the PMS is the basis for planning maintenance and is regularly updated and operated to that end. | N | N.A. | N.A. | N.A. | H60 |
|  |  |  |  |  |  | OR02 | Carry out a detailed study review of the existing road assets data and systems in order to establish accuracy, completeness and appropriateness of the currently available inventory, inspection and maintenance records data and functionality of the associated asset management systems such as PMS, BMS etc. together with appropriate improvements measures. | N | N.A. | N.A. | N.A. | H61 |
|  |  |  |  |  |  | OR03 | Carry out a detailed analysis of O&M funding requirements (both for backlog removal and on-going routine & planned maintenance activities for all current and planned assets) and historic allocations to identify appropriate annual O&M funding needs and their earmarking. As a stop-gap measure the annual roads O&M budget for 2015 should be increased from the historic (last 10 years) average of €158M to minimum €1,363M. [These figures are subject to confirmation pending further discussions with CNADNR]. This should be ensured in long-term by government commitments supported by appropriate additional revenue sources e.g. increased vignette levels. A separate feasibility study into options for distance based user-charging should also be undertaken. A preliminary test will be undertaken using the National Model to assess the impacts of distance based charging. | Yes (high level test for charging option) | N.A | N.A | Network level | H62 |
|  |  |  |  |  |  | OR04 | Introduce an efficient and effective system for prioritisation and allocation of O&M expenditures on a multi-year basis and develop dedicated backlog removal plans for all major assets (roads and bridges). | N | N.A. | N.A. | N.A. | H63 |
|  |  |  |  |  |  | OR05 | Review the existing O&M relevant standards and specifications in terms of their appropriateness and affordability and define appropriate ‘Quality of Service’ parameters the network should provide. | N | N.A. | N.A. | N.A. | H64 |
|  |  |  |  |  |  | OR06 | Reform the current maintenance practice (>100 contracts of max 2 years) by proceeding systematically with longer term (5-7 years) performance based contracts at regional scale. | N.A. | N.A. | N.A. | N.A. | H65 |
|  | 02 | Management & Operations | Institutional capacity issues Significant damage to roads assets (pavements and structures predominantly) by excessive axel loads | At all levels - lack of ownership, frequent organisational changes, high staff turnover, adequacy of human resources and their experience, adequate training options etc. | Improved institutional & organisation stability and capacity to ensure efficient and effective management of activities in the road sector during all project phases (planning & preparation, implementation, operation & maintenance) | OR07 | Audit and review of the existing situation followed by identification of relevant measures and recommendations in terms organisation of the departments to be in charge of all stages of project cycle, completed by a Training Needs Assessment for all key staff. Appoint and retain management based on longer term performance-based contracts with the Ministry. | N.A. | N.A. | N.A. | N.A. | H66 |
|  |  |  |  | Effect of excessive axle loading on the expected (designed) life of assets is considerable and well documented (e.g. Liddle, 1963). Data from CESTRIN’s WIM database suggests 20% of 4-axle trucks overweight which (as they can weigh up to 50 tonnes) is a major concern. (see also Appendix E) | Limit damage to roads caused by excessive overloading of trucks | OR08 | Review appropriateness of axle load legislation and ensure its effective enforcement via regular spot-checks and fines for non-compliance. Ensure sufficient monitoring and checking procedures / facilities in place. | N.A. | N.A. | N.A. | N.A. | H67 |
| Economic Efficiency | 03 | Infrastructure Deficit | High travel times leading to uncompetitive service on the key national connectivity corridors. | Less than 3% of network is motorway standard. Average speed for non-motorway links is 66 kph. See detailed national model analysis in Appendix B: Economic Impact of Delays (2020 Reference Case) and Figure A1. | Improve travel speeds on Bucharest–Center Corridor (Bucharest-Brasov) | OR1 | Ploiesti-Comarnic-Brasov | Y | 65,000 | Motorway 2x2 | Tested against Reference Case | H1 H8 |
|  |  |  |  |  | Improve travel speeds on Bucharest – West Corridor (Bucharest-Sibiu) | OR2 | Option 1: Pitesti – Sibiu | Y | 57,000 | Motorway 2x2 | Each option to be tested against Reference Case and also Reference Case plus alternative. | H2 |
|  |  |  |  |  |  |  | Option 2: Ploiesti–Comarnic-Brasov-Sibiu | Y | 65,000 | Motorway 2x2 |  | H1, H7, H8 |
|  |  |  |  |  | Improve travel speeds on Bucharest - North East Corridor (Bucharest - Bacau-Iasi) | OR3 | Option 1: Ploiesti-Buzau- Bacau-Iasi | Y | 47,000 | Motorway 2x2 | Each option to be tested against Reference Case and also Reference Case plus alternative | H38 H13 |
|  |  |  |  |  |  |  | Option 2: Ploiesti-Brasov-Bacau-Iasi | Y | 65,000 | Motorway 2x2 |  | H1, H8, H12, H13 |
|  |  |  |  |  |  |  | Bucharest-Bacau-Iasi via Pascani (using preferred choice from option 1 and 2 as far as Bacau) | Y |  | Motorway 2x2 | Reference Case plus best performing of Option 1 and 2 | TBC |
|  |  |  |  |  | Improve travel speeds on Bucharest - South West Corridor (Bucharest – Craiova) | OR4 | Option 1: Pitesti-Craiova | Y | 37,000 | Motorway 2x2 | Each option to be tested against Reference Case and also Reference Case plus alternative | H6 |
|  |  |  |  |  |  |  | Option 2: Bucharest-Alexandria-Craiova | Y | 42,000 | Motorway 2x2 |  | H9 |
|  |  |  |  |  | Improve travel speeds on North-East – Center - West Corridor (Iasi – Brasov/Cluj – HU Border) | OR5 | Option 1: Iasi - Turda + Nadaselu - Bors | Y | 35,000 | Motorway 2x2 | Each option to be tested against Reference Case and also Reference Case plus alternative | H14 H15 |
|  |  |  |  |  |  |  | Option 2: Iasi – Bacau – Brasov – Sibiu | Y | 44,000 | Motorway 2x2 |  | H7, H12, H13 |
|  |  |  |  |  | Aggregate all national corridor objectives | OR1+OR2+OR3+OR4+OR5 | Option A: sum of best option of each of OR1 to OR5 Option B: Variant combination based on Core TEN-T networks | Y | TBC | TBC | Aggregate tests OR1+Or2+OR3+OR4+OR5 | TBC |
| Economic Efficiency | 04 | Infrastructure Deficit | High travel times leading to uncompetitive service on other significant regional connectivity links | Less than 3% of network is motorway standard. Average speed for non-motorway links is 66 kph. See detailed national model analysis in Appendix A: Economic Impact of Delays (2020 Reference Case) Schemes identified by analysis of completed level 1 networks using lost time, target speed and level of service analysis to inform Level 2 schemes. The tests herein are those considered most likely to emerge based on current knowledge but may be refined following completion of the Level 1 tests. | Improve travel speeds on Braila – Galati - Focsani link | OR6 | Focsani-Galati-Braila | Y | TBC | Expressway | Scheme versus Ref. Case plus L1 | H39 |
|  |  |  |  |  | Improve travel speeds in vicinity of Suceava, Botosani and Siret | OR7 | Bacau-Suceava | Y | TBC | Expressway | Scheme versus Ref. Case plus L1 | H40 |
|  |  |  |  |  |  |  | Suceava-Siret | Y | TBC | Expressway | Scheme versus Ref. Case plus L1 | H41 |
|  |  |  |  |  |  |  | Suceava-Botosani | Y | TBC | Expressway | Scheme versus Ref. Case plus L1 | H42 |
|  |  |  |  |  | Improve travel speeds on Bacau – Piatra Neamt link | OR8 | Bacau-Piatra Neamt | Y | TBC | Expressway | Scheme versus Ref. Case plus L1 | H43 |
|  |  |  |  |  | Improve travel speeds on Cluj - Dej link | OR9 | Cluj / Targu Mures - Halmeu | Y | TBC | Expressway | Scheme versus Ref. Case plus L1 | H30 H31 |
|  |  |  |  |  | Improve travel speeds between Craiova and Lugoj | OR10 | Craiova-Lugoj | Y | TBC | Expressway | Scheme versus Ref. Case plus L1 | H28 |
|  |  |  |  |  | Constanta-Tulcea-Braila/Galati | OR11 | Constanta-Tulcea-Braila/Galati | Y | TBC | Expressway | Scheme versus Ref. Case plus L1 | H36 |
|  |  |  |  |  | To be confirmed – remaining routes identified post L1 & L2 testing | OR12 | TBC | Y | TBC | TBC | TBC | TBC |
|  |  |  | High travel times leading to uncompetitive service on remaining National Network links | Schemes / locations identified by model analysis post Level 1, Level 2 and Bypass interventions. Consider 2+1 options(solid barrier) | To be confirmed – remaining routes identified post L1, L2 and Bypass testing | OR13 |  |  |  |  |  |  |
|  | 05 | Management & Operations | High administrative costs on local and national hauliers from complex and uncoordinated charging regimes | Lack of co-ordinated charging regime with county and municipal authorities charging additional fees over and above national vignette. These vignettes are not co-ordinated or easily payable by international or regional hauliers. | Minimise administrative burden on all aspects of government and haulage industry related to goods vehicle charging. | OR14 | Introduce single co-ordinated system for charging covering the entire road network. The system will be accessible to all potential users (national and international via an internet based system. This requires a legislative act. The Ministry to be responsible for dissemination of revenues. | N.A | N.A | N.A | NA | H63 |
|  | 06 | Management & Operations | Excessive delays at certain Border Crossings | Advice from Hungarian government to avoid certain routes backed up by anecdotal evidence from hauliers in respect of Nadlac: 7 kms queues and delays of up to 12 hours. See also Appendix F for more detailed responses from haulier representatives. | Reduce delays at border crossing points such that the maximum waiting time for any vehicle is 40 minutes as set out in European Standards by the Trade and Transport Facilitation in Southeast Europe Program (TTFSE). | OR15 | Use of ITS to provide alternative route information coupled with improved management of border crossing arrangements. |  |  |  |  | H64 |
| Economic Development (Accesibility) | 07 | Infrastructure Deficit | Poor accessibility to international trade of certain areas potentially hindering their economic development | International accessibility analysis – Figure A6 | Improve the accessibility to the Western markets of the North-East region (Iasi/Suceava/Bacau) | OR16 | Covered under the interventions for OR3 | Covered under OR3 | N.A |  |  |  |
|  |  |  |  |  | Improve the accessibility to the Western markets of area Braila – Galati | OR17 | Covered under the interventions for OR6 | Covered under OR6 | N.A |  |  |  |
|  |  |  |  |  | Improve the accessibility to the Western markets of the South-East Region (Constanta) | OR18 | Covered under the interventions for OR2 | Covered under OR2 | N.A |  |  |  |
|  |  |  |  |  | Improve the accessibility to the Western markets of the South Region (Craiova) | OR19 | Covered under the interventions for OR10 | Covered under OR10 | N.A |  |  |  |
| Environmental Sustainability | 08 | Infrastructure Deficit | Heavy traffic transiting localities leading to air pollution and noise affecting the communities | Analysis of the highest volumes of transit traffic (after the implementation of Reference Case, Level 1 and Level 2 schemes to determine if potential by-pass projects remain. | Minimize the local environmental impacts (noise, air quality) associated with the national transport infrastructure. | OR20 | Focus on villages with significant volumes of through traffic post Level 1 testing and identify and test these though model. | Y | N.A. | TBC | Individual | H90 |
| Safety & Security | 09 | Safety | Very high fatal accident rates on the road network compared to rest of the EU. | Romania has 259 fatalities per 10 billion pkm (against an EU average of 61) and 466 fatalities per million passenger cars (against an EU average of 126). It is the worst performing EU country for both indices. The national network accounts for 50% of all fatal accidents. 28% of all fatal accidents on the national network cited “pedestrians in the road” as a causation factor. High proportion of accidents towards end of day suggest fatigue is a contributory factor in many cases. See further analysis in Appendix C: Pedestrian Accidents and Appendix D: Road Accidents including blackspot identification in Figure C1. Road Safety Audits (RSA) procedures not properly implemented and ineffective. The EU Directive 2008/96E/EC was transposed into RO legislation but its accompanying Methodological Norms were not issued. | Decrease by half the accident rates by 2020 and down to EU average by 2030 | OR21 | Infrastructure – linear villages (pack of e.g. constructing sidewalks, speed reduction e.g. entry mini roundabouts, lighting at pedestrian crossings) – to be provided for all linear villages with AADT > 15,000 in 2020 Reference scenario. | N | n/a | n/a | n/a | H91 |
|  |  |  |  |  |  | OR22 | Infrastructure – building safety barriers (e.g. New Jersey) on all 2x2 roads – undertake separate feasibility study to identify + cost all the respective sections. | N | n/a | n/a | n/a | H92 |
|  |  |  |  |  |  | OR23 | Infrastructure – punctual specific problems: to define a blackspots action to study and design safety improvement measures for treating the blackspots identified in Figure C1 (excluding the spots expected to be solved with Level 1 or Level 2 schemes). | N | n/a | n/a | n/a | H93 |
|  |  |  |  |  |  | OR24 | Infrastructure: Provide adequate rest areas at appropriate points on national network based on maximum 2-hour drive time between locations. | N | n/a | n/a | n/a | H94 |
|  |  |  |  |  |  | OR25 | Increase safety and security of HGV operations to levels consistent with the recommendations of Directive 2008/96/EC and Directive 2010/40/EU (Article 3) requiring the provision of information and reservation services for safe and secure parking places for trucks and commercial vehicles. | N | n/a | n/a | n/a | H95 |
|  |  |  |  |  |  | OR26 | Ensure effective and efficient implementation of the RSA procedures by introducing accompanying Methodological Norms and their enforcement at all stages of project lifecycle. | N | n/a | n/a | n/a | H96 |
|  |  |  |  |  |  | OR27 | Revise the institutional structure of the Inter-ministerial Council for Traffic Safety - which is currently operationally managed (by the executive Secretariat) at a too low level (a unit of ARR) and as such unable to effectively act on either policy making, co-ordination between institutions (Road Police, NCMNR, RAR) or enforcement/ implementation level. To be effective the Secretariat should be given higher authority and visibility e.g. under the PM Office. At operational level the Secretariat should be supported with specialised TA for the diagnosis of the situation and defining a sound and comprehensive Road Safety Action Plan. | N | n/a | n/a | n/a | H97 |
|  | 10 | Security | Lack of secure and safe parking for goods vehicles. | There is currently no secure truck parking provision at all, even on new highway schemes. This presents a security and safety problem for hauliers and other road users and imposes unnecessary costs on the system. Directive 2008/96/EC and Directive 2010/40/EU (Article 3) requiring the provision of information and reservation services for safe and secure parking places for trucks and commercial vehicles. | Increase safety and security of HGV operations to levels consistent with the EU requirement and practice. | OR28 | E.g. Regulation 1315/2013 requires for TEN-T appropriate parking areas every 100 km –As the entire TEN-T network (core and comprehensive) comprises approximately 4,900 km, this implies 49 such facilities or 27 for the core only. Unit costs for a service area are approximately €20M so the total cost would be €540M for the core and €980M for the core and comprehensive. | N | n/a | n/a | n/a | H98 |

Source: AECOM analysis

Safety Measures

As highlighted previously, the safety record on the Romanian road network is extremely poor. Whilst the proposed major investment in the network, as set out later in this Chapter for Level 1 and Level 2, will bring significant road safety benefits, these will not be realised for some time due to the scale of investment required.

We have therefore adopted a process to identify particularly poor accident record locations, and identify measures that can be undertaken in the short to medium term. These ‘quick wins’ can then be undertaken with lower levels of investment required but with high returns in terms of road safety benefits. This is in keeping with the priority accorded to the improvement and promotion of road safety within Article 19 of EU Regulation No. 1315/2013.

The identification of safety interventions for the blackspot sites has been undertaken through the following process:

Collate the accident records for the fatal, serious and slight accidents by blackspot site;

Identify settlement type where the blackspot is - i.e. open country, city, town, village etc.;

Allocate each site to an urban or rural category and identify the number of lanes per direction;

Examine the detailed accident records and group the accident descriptions and accident causes into categories that can be linked to specific interventions at each site;

Based on the interventions to be implemented at each site derive the reduction in fatal and serious  accidents that would take place at each site; and

Determine the total costs of the intervention measures by site based on unit rates for the interventions identified.

The allocation of interventions by site was based on a set of criteria linked to accident description, accident cause, location, and carriageway type. This is summarised in Table 4.16. A separate paper has been prepared providing further details on this process.

Table 4.16 Identification of Safety Interventions for Blackspot Sites

| Intervention Type | Suitable Locations for Implementation |
| --- | --- |
| Village entry treatment (Splitter island, signs, lines) | Village or commune |
| 4 lanes into 2 at village entrance (build-outs, lining) | Village or commune |
| Rumble strips | Village or commune |
| Continuous kerbed median strip | Urban areas and in rural areas where frontal impact accidents are prevalent |
| Plastic median separation | Locations where neither New Jersey style barriers or continuous kerbed medians are installed. |
| Pedestrian refuge | Where there are high levels of pedestrian accidents. |
| Plastic road humps | Villages and communes on single carriageways |
| Footpaths (2m wide) | Villages and communes where not already in place. |
| Signal controlled pedestrian crossing | Where there are high volumes of pedestrian accidents on single carriageways |
| Dedicated left turn lane (lines + island) | Where there are high numbers of lateral collisions |
| Lay-by for buses / local parking (for stalls, etc.) | Villages and communes |
| Street lighting | Where there are high volumes of pedestrian accidents |
| Speed cameras | Where there are high numbers of accidents caused by speed/reckless driving, and/or accidents involving shunts |
| Interurban 2*2 Undivided Carriageway New Jersey Style Barriers | In rural areas where road is four lanes wide. |
| Pedestrian footbridge/underpass | Where there are high numbers of pedestrian accidents and the road is 4 lanes wide, or dual carriageway |
| Safety Barriers (Verge) | Where there are high numbers of accidents involving vehicles leaving the carriageway |

Source: AECOM analysis

Telematic Applications

Articles 19 and 31 of EU Regulation No. 1315/2013 recognise the important contribution that telematic applications can make to safety, security and environmental performance, as well as simplifying administrative procedures.

Within the Roads sector, the Master Plan has identified a number of ITS, or telematics, improvements which should be made. These are summarised here.

There is the basis of an information centre for national highways, albeit with limited ITS coverage at present, but no interaction or information sharing with similar systems operated by the police. Incomplete or uncoordinated intelligence gathering on road network operating conditions limits the effectiveness of information provision to users. This results in a lack of accurate information to drivers regarding delays, so that drivers are unable to choose alternative routes when there are accidents for example. A National Information / Control Centre should be developed, preferably incorporating shared resourcing by Ministry of Transport and Police staff, to collate data from across the road network and share this information in an intelligent manner with road users and other modes.

All new road schemes should incorporate the use of ITS, in particular multimodal information and traffic management systems, and integrated communication and payment systems where appropriate in line with Article 19. In particular the use of Variable Message Signs (VMS), to help disseminate timely information to road users should be encouraged. Weigh In Motion (WIM) installations should also be included to meet the operational objective of limiting damage to the road surface due to excessive overloading of trucks.

Smart telematic applications should be used to support the introduction of a single co-ordinated national charging system for goods vehicles, to reduce the administrative burden on local authorities as well as national government and to provide an improved service offering to national and international hauliers considering using the Romanian network.

The use of ITS will also help to provide alternative route information and thereby imporve the management of border crossing facilities with consequent reduction in delays. The goal would be to meet the European Standard, as set out in the Trade and Transportation Facilitation in Southeast Europe Programme (TTFSE), of a maximum waiting time for any vehicel of 40 minutes at border crossing points.

Defining the Motorway Network

As previously identified, the proportion of motorway standard network in Romania is low. There are a number of Reference Case schemes which will improve the situation but there is forecast to remain a number of corridors where travel times will be poor even with this investment.

The National transport model has been used to investigate the economic impact of the remaining delays on the road network in 2020 assuming all the Reference Case Schemes are constructed.

The analysis took the following form and the plot below shows the results of that computation:

Lost Time= i=0nFlowiTarget Time-Actual Time×VoTiLength

Where each model link was evaluated and:

Flow	= AADT;

i		= vehicle purpose (e.g. commuting, business etc.); and

VoT	= Value of Time.

Figure 4.15	Analysis of Lost Time by Corridor

Source: AECOM National Transport Model

The plot highlights five main corridors or areas of movement which still require improvement. The image in Figure 4.16 translates these into key corridors of movement and relates them to the main border crossings and the TEN-T Core and Comprehensive networks.

Figure 4.16	Key Corridors

Source: AECOM Analysis

Analysis was also undertaken of the main county and regional level movements. These are illustrated in the following figures.

Figure 4.17 County Level Analysis of Road Demand – 2020 Reference Case (Vehicles)

Note:		Excludes international trips

Source: AECOM National Transport Model

Figure 4.18 Regional Level Analysis of Road Demand – 2020 Reference Case (Vehicles)

Note:		Excludes international trips

Source: AECOM National Transport Model

Figure 4.19 Regional Level Analysis of Road Demand – 2020 Reference Case (Vehicles)

Note:		Includes international trips over 500 vehicles

Source: AECOM National Transport Model

Within each corridor or area, a number of options were identified which would meet the strategic and operational objectives and address the particular lost time issues.

Each option was coded into the National Model and then subjected to an appraisal process which included a full run of the NTM, economic and multi-criteria analysis in accordance with the procedures described in the Masterplan Guidance Document, in order to objectively determine the most appropriate scheme to take forward in each area.

The results of these tests, referred to as the Level 1 tests, are summarised in the following table.

Table 4.17 Summary of Level 1 Testing Results (CBA)

| Ref. | Project | Investment cost (undiscounted) €M | NPV €M | BCR | EIRR % |
| --- | --- | --- | --- | --- | --- |
| OR1 | Ploiesti-Brasov Motorway | 1,427 | 2,495 | 3.55 | 13.7% |
| OR2, Option A | Pitesti-Sibiu | 2,471 | 3,301 | 2.96 | 12.1% |
| OR2, Option B | Ploiesti-Brasov-Sibiu | 2,117 | 6,895 | 5.70 | 18.1% |
| OR3, Option A | Ploiesti-Buzau-Bacau-Iasi | 2,524 | 3,232 | 2.81 | 11.5% |
| OR3, Option B | Ploiesti-Brasov-Bacau-Iasi | 4,428 | 3,440 | 2.13 | 9.9% |
| OR3, Option C | Ploiesti-Brasov-Bacau-Iasi (via Pacsani spur) | 4,561 | 943 | 1.30 | 6.8% |
| OR4, Option A | Pitesti-Craiova | 870 | 1,242 | 3.03 | 12.2% |
| OR4, Option B | Bucharest-Alexandria-Craiova | 1,190 | 329 | 1.43 | 7.2% |
| OR5, Option A | Iasi-Turda-Nadaselu-Bors | 5,026 | 5,625 | 2.62 | 11.4% |
| OR5, Option B | Iasi-Bacau-Brasov-Sibiu | 3,691 | 3,851 | 2.50 | 11.0% |
| Preferred L1 Highway Network | Comprising OR1,OR2B, OR3B, OR4A and OR5B | 5,988 | 9,835 | 3.36 | 13.2% |
| Alternative L1 Highway Network | Comprising OR1, OR2A, OR3A, OR4B and OR5A | 17,080 | 9,470 | 2.06 | 8.6% |

Source: AECOM National Transport Model

The composition of the Preferred Level 1 network was done primarily based on the performance of the individual component schemes but also taking into consideration the connectivity of the combined network and ensuring that it provided a logical structure. It is important to recognise that this is not purely a mechanistic process as the resultant network needs to be cohesive and logical in its extent and the connections which are made. It is not possible to achieve this purely by selecting the most economically advantageous schemes and combining them as they do not necessarily form a cohesive network.

Figure 4.20 shows the comparison of the two alternative strategies in terms of investment cost and Net Present Value (i.e. total benefits minus total costs over the full appraisal period). The indicators all combine to provide a very strong argument in favour of the preferred ‘Level 1’ network. In summary:

The preferred Level 1 network provides much better value for money as indicated by the higher BCR (3.36 compared to 2.06);

The preferred Level 1 network has a much lower investment cost (€5.5M compared to €13.8M); and

The preferred Level 1 network provides a higher Net Present Value (i.e. sum of all benefits minus sum of all costs).

Figure 4.20 Comparison of Preferred and Alternative Level 1 Strategies

Source: AECOM analysis of Romanian National Transport Model

The Preferred Level 1 Network and the Alternative Level 1 Network are illustrated in Figure 4.21 and Figure 4.22. Visual inspection of these Figures also highlights that the Preferred Network provides a more cohesive and efficient means of connecting the country.

The Preferred L1 network represents the best strategy for connecting the regions of Romania with a high-quality motorway network in an economically efficient way.  Following the selection of this Preferred Strategy, some modifications were made to individual projects.  This included the direct connection between Bacau and Iasi.  Since there would be a motorway connection between Bacau and Iasi as part of the Bacau – Suceava project, and Pascani and Iasi are connected as part of the Tg Mures – Iasi project, thereby providing a high quality connection between Bacau and Iasi via Pascani.  Together with the difficult geological conditions on the direct route, the Bacau – Iasi route via Pascani is the best option for connecting these two important cities. This is reflected in subsequent illustrations of the Preferred Networks.

Figure 4.21 Level 1 Preferred Network

Source: AECOM analysis of Romanian National Transport Model

Figure 4.22 	Level 1 Alternative Network

Source: AECOM analysis of Romanian National Transport Model

The impact of the preferred Level 1 Strategy is shown in Figure 4.23 in terms of the change in total flow compared to the Reference Case Situation. Increases are shown in red and reductions in green. Flows are in AADT. For comparative purposes, the equivalent changes in flow were the Alternative Level 1 network to be adopted, are shown in Figure 4.24.

Note that the Preferred Level 1 Network provides considerable relief to the Sibiu – Pitesti corridor. The converse effect is not true with the Alternative Level 1 network where the Ploiesti – Brasov corridor continues to experience an incarese in traffic even with improvements to the Sibiu – Pitesti corridor. This indicates that this latter improvement is not the most efficient way of dealing with demand in this NW – SE corridor.

Figure 4.23	Change in Total Flow for Level 1 Preferred Relative to Reference Case

Source: AECOM analysis of Romanian National Transport Model

Figure 4.24 Change in Total Flow for Level 1 Alternative Relative to Reference Case

Source: AECOM analysis of Romanian National Transport Model

Defining the Expressway Network

The Level 1 testing determined those parts of the network which were most in need of significant improvement, and for which a motorway standard project was appropriate. However there remain a number of corridors which still require enhancement in order to ensure the road network meets the strategic objectives as previously defined.

The lost time analysis was therefore repeated using the National Model but in this scenario all of the Level 1 Motorway schemes, as well as the agreed Reference Case schemes, were assumed to be complete.

The output from the analysis is shown in Figure 4.25 overleaf.

Figure 4.25 Analysis of Lost Time by Corridor Post Level 1

Source: AECOM analysis of Romanian National Transport Model

The candidate projects were then identified and each option coded into the National Model. All schemes were then subjected to a full model run, economic and multi-criteria analysis in order to objectively determine the most appropriate scheme to take forward in each area.

The results of these tests, referred to as the Level 2 tests, are summarised in the Table 4.18 overleaf.

The selected Level 2 schemes are illustrated in Figure 4.26 overleaf, in order to show the linkage and connectivity with the Reference Case and Level 1 schemes.

Table 4.18 Summary of Level 2 Testing Results (CBA)

| Ref. | Project | Investment cost (undiscounted) €M | NPV €M | BCR | EIRR % |
| --- | --- | --- | --- | --- | --- |
| OR19 | Bucharest-Alexandria Expressway | 370 | 486 | 2.35 | 11.8% |
| OR6B | Bacau-Focsani-Braila-Galati Expressway | 1,024 | 1,268 | 2.63 | 11.2% |
| OR7A | Bacau-Suceava Expressway | 645 | 1,107 | 3.23 | 12.6% |
| OR7B | Suceava-Siret Expressway | 186 | 1,172 | 2.84 | 11.7% |
| OR7C | Suceava-Botosani Expressway | 346 | 623 | 1.83 | 8.9% |
| OR8 | Bacau-Piatra Neamt Expressway | 335 | 350 | 2.45 | 10.6% |
| OR9 | Fagaras - Tg Mures Expressway | 1,158 | 1,461 | 2.73 | 11.5% |
| OR9B | Turda-Halmeu Expressway | 975 | 1,766 | 3.29 | 13.5% |
| OR10 | Lugoj- Craiova Expressway | 1,811 | 1,546 | 2.23 | 10.0% |
| OR11 | Constanta-Tulcea-Braila Expressway (including Braila Bridge) | 1,369 | 66 | 1.07 | 5.4% |
| OR12 | Nadaselu - Suplacu de Barcau Motorway | 550 | 1,219 | 1.95 | 9.2% |
| OR13 | Targu Mures-Targu Neamt Expressway | 3,400 | 2,213 | 1.74 | 8.4% |
| OR13C | Buzau-Focsani Expressway | 282 | 1,350 | 2.80 | 12.0% |
| OR14 | Brasov-Pitesti Expressway | 1,843 | 1,222 | 1.97 | 9.1% |
| OR15 | Sibiu-Pitesti Motorway | 2,471 | 3,301 | 2.96 | 12.1% |
| OR13D | Targu Neamt-Iasi-Ungheni Motorway | 700 | 490 | 1.60 | 10.8% |
| OR17 | Gaesti-Ploiesti-Buzau-Braila Expressway | 1,280 | 1,754 | 2.87 | 11.9% |
| OR18A | Bucharest Ring Road Motorway | 1,684 | 133 | 1.14 | 5.7% |
| OR19A | Henri Coanda Airport connection to A3 | 126 | 86 | 1.99 | 9.3% |
| OR21 | A1 Widening Bucharest-Pitesti | 442 | 181 | 1.65 | 7.6% |
| OR18B | Bucharest Southern Ring Road Upgrade | 176 | 380 | 4.42 | 14.5% |

Source: AECOM analysis

Figure 4.26 Level 2 Network in Relation to Level 1 & Reference Case Schemes

Source: AECOM analysis of Romanian National Transport Model

National Road Network

Following the selection of the Level 1 and Level 2 projects, there remain some sections of the National Network which, although not meriting an improvement scheme, perform an important function as they provide linkages to county towns and/or the Core TEN-T network.

An approach was developed to prioritise these sections of the network, considering the level of demand and road condition.

The results of the methodology are presented in Table 4.19.

Table 4.19 National Network Rehabilitation Schemes (Ranked)

| Nr. | Project Description | Length (km) | Estimated Cost (2014 prices, mill EUR, VAT excluded) | Route classification | Demand AADT+ 2xHGV | Condition |
| --- | --- | --- | --- | --- | --- | --- |
| ROAD SECTOR - Transregio Roads |  |  |  |  |  |  |
| 1 | Brasov - Sighisoara - Tg Mures (DX 3) | 161 | 102.1 | TEN-T Comprehensive | 16,391 | 2.64 |
| 2 | Braila - Slobozia - Dranjna (A2) - Calarasi - Chiciu (BG) | 142 | 71.0 | Other | 16,032 | 2.33 |
| 3 | Constanta - Vama Veche (BG) | 49 | 36.8 | TEN-T Comprehensive | 15,727 | 1.04 |
| 4 | A1 - Arad - Salonta - Oradea | 121 | 60.5 | TEN-T Comprehensive | 14,303 | 1.15 |
| 5 | Dr.Tr.Severin – Tg. Jiu – Rm.Valcea – Pitesti | 246 | 134.8 | Other | 13,553 | 1.35 |
| 6 | Vaslui - Barlad - Tecuci - Galati | 179 | 89.5 | Partial TEN-T Comprehensive | 13,397 | 2.36 |
| 7 | Filiasi - Tg. Jiu - Petrosani - Hateg - Deva - A1 | 226 | 136.5 | TEN-T Comprehensive | 13,227 | 1.08 |
| 8 | Botosani - Targul Frumos | 73 | 36.5 | Other | 12,540 | 2.78 |
| 9 | Suceava - Vatra Dornei - Bistrita - Saratel -Dej | 178 | 124.9 | Other | 11,554 | 2.03 |
| 10 | Saratel - Reghin - Tg Mures | 78 | 44.0 | Other | 10,670 | 1.00 |
| 11 | A3 (Oradea) - Carei - Satu Mare - DX 4 | 137 | 68.5 | Other | 10,600 | 2.88 |
| 12 | A1 (Deva) - Brad - Stei - Oradea - A3 | 197 | 124.3 | Other | 8,366 | 1.00 |
| 13 | Corabia - Caracal - Dragasani - Rm. Valcea - DX 2 | 199 | 112.4 | Other | 7,708 | 2.61 |
| 14 | Zalau - Satu Mare | 95 | 49.3 | TEN-T Comprehensive | 7,571 | 1.04 |
| 15 | A5 - Sf. Gheorghe - B. Tusnad - Miercurea Ciuc - Ditrau (DX 3) | 147 | 79.1 | Other | 6,708 | 3.32 |
| 16 | Iasi - Vaslui - Bacau | 151 | 99.5 | Other | 6,675 | 3.38 |
| 17 | Targu Neamt - Piatra Neamt | 35 | 19.1 | Other | 6,063 | 2.36 |
| 18 | Iacobeni - Borsa - S.Marmatiei - Negresti Oas - DX 4 | 235 | 159.1 | Other | 4,461 | 2.03 |
| 19 | Caransebes - Resita - Bocsa - Voiteg | 104 | 62.6 | Other | 3,668 | 1.83 |
| 20 | Focsani - A5 - Tg. Secuiesc | 114 | 80.3 | Other | 3,529 | 2.37 |
|  | Subtotal Drumuri Transregio | 2,867 | 1,690.8 |  |  |  |
| Nr. | Project Description | Length (km) | Estimated Cost (2014 prices, mill EUR, VAT excluded) | Route classification | Demand AADT+ 2xHGV | Condition |
| ROAD SECTOR - Eurotrans Roads |  |  |  |  |  |  |
| 1 | Bucuresti - Giurgiu (BG) | 49 | 41.3 | TEN-T Core | 21,148 | 2.02 |
| 2 | A1 - Timisoara - Moravita (SRB) | 59 | 29.5 | TEN-T Core | 10,990 | 3.90 |
| 3 | Craiova - Calafat (BG) | 83 | 41.5 | TEN-T Core | 6,513 | 1.00 |
| 4 | Drobeta Tr. Severin - Calafat (BG) | 82 | 50.8 | TEN-T Core | 5,721 | 2.22 |
|  | Subtotal Drumuri Eurotrans | 273 | 163.1 |  |  |  |

Source: AECOM analysis

Priorities for Bypasses

The National Transport Model was developed primarily for assessing strategic issues on the transport network. This means that the representation and forecasts of local traffic around smaller settlements may be less robust than that for major inter-urban movements.

The model assignment proess would also be less likely to reflect the relatively small scale journey time improvements which may arise from localised small scale bypass schemes.

In order to ensure that these valuable, albeit smaller scale schemes, were not overlooked, an additional analysis was undertaken. The starting point for this was a road network containing all of the Reference Case, Level 1 and Level 2 schemes.

As a first stage, all towns/cities that the National road network passes through, and which are not already to be by-passed by Level 1 and Level 2, projects were identified. For each of these centres, those with a population of greater than 10,000 were selected. It is unlikely that a population smaller than this would justify a bypass.

Based on analysis undertaken when developing the National Model, the volume of ‘through’ traffic (i.e. trips not starting or finishing in the area in question) was then derived. Thresholds were then set separately for cars and goods traffic (25% and 50%) respectively. Locations where these thresholds were exceeded were then taken forward for full testing in through the modelling and appraisal process.

The Bypasses which were taken forward for testing, and their performance, is summarised in Table 4.20.

Table 4.20 Summary of Bypass Testing Results (CBA)

| Ref. | Project | Investment cost (undiscounted) €M | NPV €M | BCR | EIRR % |
| --- | --- | --- | --- | --- | --- |
| BP1 | Oradea | 108 | 20 | 1.28 | 6.5% |
| BP2 | Dej | 30 | 107 | 5.91 | 19.7% |
| BP3 | Sighisoara | 48 | 92 | 3.84 | 15.8% |
| BP4 | Roman | 62 | 129 | 4.11 | 15.9% |
| BP5 | Focsani | 76 | 130 | 3.44 | 13.1% |
| BP6 | Ramnicu Sarat | 37 | 8 | 1.31 | 6.8% |
| BP7 | Buzau | 105 | 37 | 1.51 | 7.5% |
| BP8 | Alexandria | 55 | -18 | 0.53 | 1.5% |
| BP9 | Ramnicu Valcea | 195 | 31 | 1.23 | 6.2% |
| BP10 | Targoviste | 78 | 358 | 7.83 | 22.7% |
| BP11 | Filiasi | 28 | 68 | 4.40 | 15.6% |
| BP12 | Adjud | 46 | 170 | 5.96 | 19.0% |
| BP13 | Mizil | 36 | -11 | 0.57 | 1.9% |
| BP14 | Ludus | 102 | 113 | 2.53 | 11.2% |
| BP15 | Falticeni | 41 | 70 | 3.34 | 13.2% |
| BP16 | Caransebes | 81 | 7 | 1.13 | 5.7% |
| BP17 | Beclean | 42 | 24 | 1.81 | 8.4% |
| BP18 | Bistriţa | 157 | 32 | 1.31 | 6.5% |
| BP19 | Miercurea Ciuc | 111 | -3 | 0.95 | 4.8% |
| BP20 | Sfântu Gheorghe | 34 | 41 | 2.84 | 11.4% |
| BP21 | Giurgiu | 72 | 24 | 1.50 | 7.4% |
| BP22 | Vaslui | 73 | 31 | 1.64 | 8.6% |
| BP23 | Bârlad | 51 | 54 | 2.57 | 11.1% |
| BP24 | Slobozia | 23 | 16 | 2.06 | 10.6% |
| BP25 | Vatra Dornei | 18 | 57 | 5.65 | 17.9% |
| BP26 | Câmpulung Moldovenesc | 99 | -23 | 0.65 | 2.5% |
| BP27 | Mangalia | 44 | 14 | 1.49 | 7.5% |
| BP28 | Timisoara South | 127 | 104 | 2.21 | 9.7% |

Source: AECOM analysis

The location of the selected bypass schemes, with reference to the Reference Case, Level 1 and Level 2 schemes, is shown in Figure 4.27.

Figure 4.27	Bypass Schemes in Relation to Level 1, Level 2 & Reference Case Schemes

Source: AECOM analysis of Romanian National Transport Model

Cost-Benefit Analysis

Within this section we provide details of all of the road infrastructure schemes taken through the model testing and appraisal process and which have been selected to form part of the overall strategy.

For each scheme, we provide information as follows:

Brief description of proposal;

Brief Description of Problem(s) Addressed;

Undiscounted Costs;

Outcome(s);

Implementing Organisation	; and

Implementation Year(s).

We have considered two implementation scenarios, ‘ES’ and ‘EES’ and implementation years are provided for both cases. We present the information in the order of Level 1 schemes first, then Level 2 and finally Bypass schemes.

The Level 1 and Level 2 schemes would be developed in accordance with Article 17 of EU Regulation No. 1315/2013 in that they would incorporate:

Suitable parking and rest areas; and

Relevant telematic applications such as VMS.

Also, as per Article 17, they must be adequately maintained. The regulations do not stipulate whether infrastructure on the comprehensive network should be motorway, express road or conventional strategic road. As set out in the notes at the beginning of this document, the technical solutions for each intervention will need to be reconfirmed at the level of each particular project within the FS on the basis of a detailed options analysis including more detailed cost, capacity, economic and environmental impact analyses.

Finally it is important to note that, for the purposes of this element of the project, each of the schemes that are reported on in the following sections have been tested in isolation. The changes in flow forecast by the model need to be seen in that context. It is inevitable that schemes which are relatively close to one another or which cater for similar traffic patterns, e.g. longer distance East-West movements, will produce different effects if considered in combination.

This is particularly relevant when considering scenarios which are comprised of multiple schemes; it is not possible to simply sum the constituent model results in order to forecast the combined effect. The flow difference plots which are used to illustrate the impacts of each scheme are for an opening year of 2020.

Level 1 Schemes - OR1 - Ploiesti-Brasov Motorway

Brief Description of Proposal:

The proposal will provide a new motorway link between Ploiesti and Brasov to link with the existing Bucharest to Ploiesti motorway. The combination will provide a motorway standard route between Bucharest and Brasov. It fulfils operational objectives set out in Table 4.15 for OR1 (Bucharest- Brasov) and contributes to improved regional and national connectivity for East-West traffic through the country. It therefore also has European added value as it is a key link along the corridor that ultimately provides access from Romania’s Western borders to the Black Sea.

The alignment of the route and its linkage to the existing network is shown in Figure 4.28 below.

Figure 4.28	Ploiesti-Brasov Motorway Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

The route forms part of the corridor that will provide a part of the link between the two TEN-T core nodes of Bucharest and Timisoara. It also directly links Brasov, which is a TEN-T node to Bucharest.

Brief Description of Problem(s) Addressed

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds in the Bucharest to Brasov corridor.

There are poor average travel times between Bucharest and Brasov; the transport model forecasts typical average peak speeds for this route of less than 70 kph for 2020. Bucharest and Brasov are the first and seventh most populace cities in Romania, and this is a key transport corridor as it also provides a link, via Sibiu, to the committed A1 motorway and out to the Hungarian border.

This corridor was identified as part of an analysis of the economic impact of delays in addition to the presence of multiple accident blackspot locations in the corridor.

Domestically, accessibility to Brasov is poorer than cities such as Pitesti, which do have a high quality link to Bucharest.

Whilst there is a good quality route from Bucharest to south of Ploiesti via the A3, the remainder of the infrastructure provision in the corridor is poor – 44 % of the remainder of the route is only single carriageway standard.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€1,427M

Outcome(s):

This project returns a very positive BCR value of 3.55 and is forecast to carry around 50,000 vehicles two-way (AADT) in 2020. The impact on the other modes is small, with rail experiencing some reduction in passengers and freight.

Almost all of the existing traffic in the immediate corridor switches to the new route. A significant amount of traffic is also attracted from the route between Bucharest and Sibiu (via Pitesti) and Buzau - Brasov. The flow changes are illustrated in Figure 4.29.

Figure 4.29	Flow Changes Due to Ploiesti-Brasov Motorway Project

Source: AECOM analysis of Romanian National Transport Model

Average speeds between Bucharest and Brasov increase to 88 kph as a consequence of the scheme. Peak time journey times are reduced by 25%.

The primary economic indicators for the project are as follows:

- NPV	=	€2,495M;

- BCR	=	3.55; and

- EIRR	=	13.7%.

Level 1 Schemes – OR2 - Brasov-Sibiu Motorway

Brief Description of Proposal

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds in the west corridor from Bucharest to Sibiu.

The proposal will provide a new motorway link between Brasov and Sibiu. In combination with OR1, this will provide a motorway standard route between Bucharest and Sibiu and thereafter, via the committed A1 improvements, to the Hungarian border. It therefore also fulfils operational objectives set out in Table 4.15 for OR1 (Bucharest- Brasov) and contributes to improved regional and national connectivity for East-West traffic through the country. Similarly it has European added value as it is a key link along the corridor that ultimately provides access from Romania’s Western borders to the Black Sea.

The alignment of the route and its linkage to the existing network is shown in Figure 4.30 below.

Figure 4.30	Brasov-Sibiu Motorway Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

The route forms part of the corridor that will provide a link from the TEN-T core node of Bucharest. It also directly links Brasov and Sibiu; both TEN-T nodes.

Brief Description of Problem(s) Addressed:

The corridor was identified as part of an analysis of the economic impact of delays and there are multiple accident blackspot locations in the corridor.

Domestically, accessibility is poorer than in cities which do have a high quality link to Bucharest. Presently, the bulk of the infrastructure provision in the corridor is poor as nearly 72% of the route is only single carriageway standard.

Analysis of the base year model shows poor average travel times between Brasov and Sibiu; with typical average speeds for this route around 68kph.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€2,117M

Outcome(s)

This project returns excellent value for money (BCR of 5.70), and carries around 50,000 vehicles (AADT). Almost all of the existing traffic in the immediate corridor switches to the new route. The flow changes are illustrated in Figure 4.31 below. This shows there is also a significant transfer of traffic from the Bucharest-Pitesti-Sibiu corridor.

Figure 4.31 Flow Changes Due to Brasov-Sibiu Project

Source: AECOM analysis of Romanian National Transport Model

Average speeds between Bucharest and Sibiu are forecast to increase to nearly 90 kph as a consequence of the scheme implementation. Meanwhile, peak time journey times are reduced by 29%.

The primary economic indicators for the project are as follows:

- NPV	= €6,895M;

- BCR	= 5.70; and

- EIRR	= 18.1%.

Level 1 Schemes – OR3 - Brasov-Bacau-Iasi Motorway

Brief Description of Proposal:

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds in the corridor from Bucharest to the North East as set out in Table 4.15.

The proposal will provide a new motorway link between Brasov and Iasi near the Moldovan Border. The alignment of the route and its linkage to the existing network is shown in Figure 4.32 below. This illustrates how the route connects with the route north from Bucharest via Ploiesti to Brasov to provide a strong regional link for the North East to the capital. In combination with other routes, it also enhances links across Romania to Moldova thereby adding European value.

Figure 4.32	Brasov-Bacau-Iasi Motorway Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

The route forms part of the corridor that will provide a link from the TEN-T core node of Bucharest. It also directly links Brasov and Bacau; both TEN-T nodes.

Brief Description of Problem(s) Addressed:

The corridor was identified as part of an analysis of the economic impact of delays and there are multiple accident blackspot locations in the corridor.

Domestically, accessibility is poorer from cities which do not have a high quality link to Bucharest.

There are poor average travel times between Brasov and Iasi; with the base year model estimating average speeds for this route to be around 64 kph.

Currently, the bulk of the infrastructure provision in the corridor is poor with nearly 82% of the route classified as only single carriageway standard.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€4,561M

Outcome(s):

This project returns a value for money figure of 1.3, and is forecast to carry around 35,000 vehicles (AADT) between Brasov and Bacau and as far north as Roman. The eastbound link towards Iasi is forecasted to carry on average around 20,000 vehicles (AADT). Following the implementation of OR 3, almost all of the existing traffic in the immediate corridor switches to the new route. It also provides relief to the Bucharest-Buzau-Bacau corridor and Figure 4.33 below provides an illustration of the flow changes described.

Figure 4.33 Flow Changes Due to Brasov-Bacau-Iasi Motorway Project

Source: AECOM analysis of Romanian National Transport Model

The forecast model estimates an increase in average speeds between Bucharest and Brasov to 95 kph as a consequence of the scheme implementation. Meanwhile, journey times during the peak period are reduced by 34%.

The primary economic indicators for the project are as follows:

- NPV	= €943M;

- BCR	= 1.3; and

- EIRR	= 6.8%

Level 1 Schemes – OR4 - Pitesti-Craiova Motorway

Brief Description of Proposal

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds in the corridor from Bucharest to the South West as set out in Table 4.15. The corridor is significant as shown in Figure 4.16 which highlights the TEN-T corridor in the vicinity.

The proposal will provide a new motorway link between Pitesti and Craiova.  In combination with the existing A1 Motorway, this will provide a motorway standard route between Bucharest and Craiova, meeting regional connectivity objectives. In conjunction with OR3, the proposal connects the industrial city of Craiova with Brasov, Bacau and Iasi providing wider economic benefits as access to wider markets is enhanced.

The alignment of the route and its linkage to the existing network is shown in Figure 4.34 below.

Figure 4.34	Pitesti-Craiova Motorway Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

The route forms part of the corridor that will provide a link from the TEN-T core node of Bucharest. It also directly links to Craiova, a TEN-T node.

Brief Description of Problem(s) Addressed:

The corridor was identified as part of an analysis of the economic impact of delays and there are multiple accident blackspot locations in the corridor.

Domestically, accessibility is poorer from cities, which do not have a high quality link to Bucharest. Currently, the bulk of the infrastructure provision in the corridor is poor – 92% of the route is only single carriageway standard.

There are poor average travel times between Pitesti-Craiova; the base year model estimates typical average speeds for this route to be around 82 kph.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€ 870M

Outcome(s):

This project returns excellent value for money (BCR of 3.03), and is forecasted to carry around 30,000 vehicles (AADT). Following the implementation of the scheme, the model forecasts that almost all of the existing traffic in the immediate corridor switches to the new route. There are limited wider impacts from the scheme. The flow changes are illustrated in Figure 4.35 below.

Figure 4.35	Flow Changes Due to Pitesti-Craiova Motorway Project

Source: AECOM analysis of Romanian National Transport Model

Average speeds between Bucharest and Craiova increase to 110 kph as a consequence of the scheme implementation. Meanwhile, peak time journey times are reduced by 27%.

The primary economic indicators for the project are as follows:

- NPV	= €1,242M;

- BCR	= 3.03; and

- EIRR	= 12.2%.

Level 1 Schemes – OR5 – Iasi-Bacau-Brasov-Sibiu Motorway

Brief Description of Proposal:

The proposal will provide a new motorway link between Sibiu and Iasi.

The alignment of the route and its linkage to the existing network is shown in Figure 4.36 below.

Figure 4.36	Iasi-Bacau-Brasov-Sibiu Motorway Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

The route forms a corridor that will provide a link between the four TEN-T nodes. This is a key transport corridor as it also provides a link, via Sibiu, to the committed A1 motorway and out to the Hungarian border.

Brief Description of Problem(s) Addressed

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds on North – East – Center – West corridors.

There are poor average travel times between Iasi and Sibiu; with the base year model estimating typical average peak speeds for this route to be around 67 kph. The corridor was identified as part of an analysis of the economic impact of delays and there are multiple accident blackspot locations in the corridor.

Domestically, accessibility is poorer compared to cities which do have a high quality link to Bucharest. The infrastructure provision in the corridor presently, is poor – 91 % of the route is only single carriageway standard.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€3,392M

Outcome(s):

This project returns a BCR value of 2.50 and carries around 50,000 vehicles (AADT) throughout the links connecting Sibiu to Bacau, and around 10,000-15,000 vehicles (AADT) on the links connecting Bacau to Iasi and the Moldovan border eastwards.

Almost all of the existing traffic in the immediate corridor switches to the new route. The flow changes are illustrated in Figure 4.37.

Figure 4.37 Flow Changes Due to Iasi-Bacau-Brasov-Sibiu Motorway Project

Source: AECOM analysis of Romanian National Transport Model

The model forecasts average speeds between Iasi and Sibiu increasing to 104 kph as a consequence of the scheme implementation. Peak time journey times are reduced by 41%.

The primary economic indicators for the project are as follows:

- NPV	=	€3,851M;

- BCR	=	2.50; and

- EIRR	=	11.0%.

Level 2 Schemes – OR6B – Bacau-Focsani-Braila-Galati Expressway

Brief Description of Proposal:

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds between Bacau and Galati, thus improving regional connectivity. The lost time analysis in Figure 4.25 shows residual congestion and poor LoS in this corridor even after the implementation of the Preferred Level 1network.  It provides a high quality connection between the ports of Galati and Braila (Romania’s second and third largest ports) and the North East of Romania via Bacau.  Flow analysis from the National Model shows an expressway is the appropriate standard.

Figure 4.38 shows the expressway alignment.

Figure 4.38	Bacau-Focsani-Braila-Galati Expressway Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

The route links the nodes of Bacau, Braila and Galati on the TEN-T network.

Brief Description of Problem(s) Addressed:

There are poor average travel times; typical average speeds for this route in the base year model are estimated to be around 65 kph. Currently, the bulk of the infrastructure provision in the corridor is poor – 98% of the route is only single carriageway standard.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€1,024M

Outcome(s):

This project returns excellent value for money (BCR of 2.63), and carries around 30,000 vehicles (AADT). Almost all of the existing traffic in the immediate corridor switches to the new route; the aforementioned flow changes are illustrated in Figure 4.39, overleaf.

Figure 4.39 Flow Changes Due to Bacau-Focsani-Braila-Galati Expressway Project

Source: AECOM analysis of Romanian National Transport Model

Average speeds increase to 106 kph as a consequence of the scheme. Peak time journey times are reduced by 40%.

The primary economic indicators for the project are as follows:

- NPV	= €1,268M;

- BCR	= 2.63; and

- EIRR	= 11.2%.

Level 2 Schemes – OR7A - Bacau-Suceava Expressway

Brief Description of Proposal

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds between Bacau and Suceava, thus improving regional connectivity. In conjunction with OR3 the project connects the North East of the country to the Centre and South and, with OR6B, to Galati/Braila. Analysis of the National Model flows show expressway is the appropriate standard for the proposal.

The alignment of the route is shown in Figure 4.40.

Figure 4.40 Bacau-Suceava Expressway Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

The route links the nodes of Bacau and Suceava, both on the TEN-T network.

Brief Description of Problem(s) Addressed:

There are poor average travel times; typical average speeds for this route are around 69 kph.

The bulk of the infrastructure provision in the corridor is poor – 97% of the route is only single carriageway standard.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€645M

Outcome(s):

This project returns excellent value for money (BCR of 3.23), and carries around 20,000 vehicles (AADT). Almost all of the existing traffic in the immediate corridor switches to the new route, the flow changes described earlier are illustrated in Figure 4.41 overleaf.

Figure 4.41 Flow Changes Due to Bacau-Suceava Expressway Project

Source: AECOM analysis of Romanian National Transport Model

Average speeds increase to 96 kph as a consequence of the scheme. Peak time journey times are reduced by 28%.

The primary economic indicators for the project are as follows:

- NPV	= €1,107M;

- BCR	= 3.23; and

- EIRR	= 12.6%.

Level 2 Schemes – OR7C - Suceava-Botosani Expressway

Brief Description of Proposal:

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds between the county capital cities of Suceava and Botosani, thus improving regional connectivity.  Analysis of the National Model flows show expressway is the appropriate standard.

The expressway alignment is shown in Figure 4.42.

Figure 4.42 Suceava-Botosani Expressway Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

The route connects to the node of Suceava on the TEN-T network.

Brief Description of Problem(s) Addressed:

There are poor average travel times; typical average speeds for this route are around 69 kph.

The bulk of the infrastructure provision in the corridor is poor – 97% of the route is only single carriageway standard.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€346M

Outcome(s):

This project returns a value for money figure (BCR) of 1.83, and carries around 20,000 vehicles (AADT). Almost all of the existing traffic in the immediate corridor switches to the new route; the aforementioned flow changes are illustrated in Figure 4.43 overleaf.

Figure 4.43 Flow Changes Due to Suceava-Botosani Expressway Project

Source: AECOM analysis of Romanian National Transport Model

Average speeds increase to 98 kph as a consequence of the scheme. Peak time journey times are reduced by 29%.

The primary economic indicators for the project are as follows:

- NPV	= €623M;

- BCR	= 1.83; and

- EIRR	= 8.9%.

Level 2 Schemes – OR9B - Turda-Halmeu Expressway

Brief Description of Proposal:

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds between Turda and Halmeu, thus improving regional connectivity.  The route connects the County Capital Cities of Cluj, Satu Mare and Baia Mare.  With OR2, it provides a connection between the South, Central and North West areas, and on to the Ukraine.  It incorporates links to the border crossing points of Halmeu and Petea and therefore also provides European added value.

Figure 4.44 shows the alignment of the expressway which analysis of the National Model flows shows is the appropriate standard for this proposal.

Figure 4.44 Turda-Halmeu Expressway Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

The route connects to the node of Turda on the TEN-T network.

Brief Description of Problem(s) Addressed:

There are poor average travel times; typical average speeds estimates of approximately 70 kph in the base year model for this route. The bulk of the current infrastructure provision in the corridor is poor – 89% of the route is only single carriageway standard.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€975M

Outcome(s):

This project returns excellent value for money (BCR of 3.29), and carries around 30,000 vehicles (AADT). The model forecasts almost all of the existing traffic in the immediate corridor switching to the new route; meanwhile the flow changes are illustrated in Figure 4.44 overleaf.

Figure 4.45 Flow Changes Due to Turda-Halmeu Expressway Project

Source: AECOM analysis of Romanian National Transport Model

Average speeds increase to 109 kph as a consequence of the scheme. Meanwhile, peak time journey times are reduced by 45%.

The primary economic indicators for the project are as follows:

- NPV	= €1,766M;

- BCR	= 3.29; and

- EIRR	= 13.5%.

Level 2 Schemes – OR10 - Lugoj- Craiova Expressway

Brief Description of Proposal

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds between Lugoj and Craiova, thus improving regional connectivity.  The route connects the industrial centre of Craiova with the west of Romania and Hungary and the remainder of Europe.  It also connects with the Vidin – Calafat Danube Crossing thus providing European added value. In conjunction with OR4 (Pitesti – Craiova), the route provides a connection between the remainder of Europe with the industrial and distribution centre of Pitesti.  Analysis of the National Model flows show expressway is the appropriate standard for the proposal.

Figure 4.46 shows the indicative alignment.

Figure 4.46 Lugoj – Craiova Expressway Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

The route connects to the node of Craiova on the TEN-T network.

Brief Description of Problem(s) Addressed:

There are poor average travel times; typical average speeds for this route are around 71 kph. The bulk of the infrastructure provision in the corridor is poor – 84% of the route is only single carriageway standard.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€1,811M

Outcome(s):

This project returns excellent value for money (BCR of 2.23), and carries around 30,000 vehicles (AADT). Almost all of the existing traffic in the immediate corridor switches to the new route; the aforementioned flow changes are illustrated in Figure 4.47.

Figure 4.47 Flow Changes Due to Lugoj – Craiova Expressway Project

Source: AECOM analysis of Romanian National Transport Model

Average speeds increase to 98 kph as a consequence of the scheme. Peak time journey times are reduced by 32%.

The primary economic indicators for the project are as follows:

- NPV	= €1,546M;

- BCR	= 2.23; and

- EIRR	= 10.0%.

Level 2 Schemes – OR13 – Campia Turzii -Targa Mures- Iasi Expressway

Brief Description of Proposal

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds in the corridor thus improving regional connectivity. The project provides a direct link between the North East of Romania, the Cluj area and Hungary/rest of Europe via Sebes/Timisoara (Reference case project), or Oradea via OR12 (Gilau – Bors).

The project will be implemented in two parts, Tg Mures – Tg Neamt as an Expressway, and Tg Neamt – Pascani – Iasi as a Motorway based on an assessment of flows from the National Model.

Figure 4.48 shows the route alignment.

Figure 4.48 Campia Turzii -Targa Mures- Iasi Expressway Indicative Alignment

Reference Case

Expressway

Motorway

Reference Case

Expressway

Motorway

Source: AECOM analysis of Romanian National Transport Model

The route connects to the node of Turda on the TEN-T network.

Brief Description of Problem(s) Addressed:

There are poor average travel times; typical average speeds for this route are around 61 kph. The bulk of the infrastructure provision in the corridor is poor – 100% of the route is only single carriageway standard.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€3,400M

Outcome(s):

This project returns a BCR of 1.74, and carries around 31,000 vehicles (AADT). Over half of all of the existing traffic in the immediate corridor switches to the new route. The flow changes described previously are illustrated in Figure 4.49 overleaf.

Figure 4.49 Flow Changes Due to Campia Turzii -Targa Mures- Iasi Expressway Project

Source: AECOM analysis of Romanian National Transport Model

The model forecasts average speeds increasing to 100 kph as a consequence of the scheme implementation. While peak time journey times are reduced by 42%.

The primary economic indicators for the whole route  (i.e. including Câmpia Turzii to Targu Mures) are as follows:

- NPV	= €2,213M;

- BCR	= 1.74; and

- EIRR	= 8.4%.

Level 2 Schemes – OR17 - Gaesti-Ploiesti-Buzau-Braila Expressway

Brief Description of Proposal

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds between Gaesti and Braila, thus improving regional connectivity. The project also provides a high quality route for goods vehicles between the industrial centres of Craiova, Pitesti and Ploiesti with Galati and Braila.  The National Model flows show an expressway is the appropriate standard.

Figure 4.50 shows the alignment of the route.

Figure 4.50 Gaesti-Ploiesti-Buzau-Braila Expressway Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

Brief Description of Problem(s) Addressed:

There are poor average travel times; typical average speeds for this route are around 87 kph.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€1,280M

Outcome(s):

This project returns excellent value for money (BCR of 2.87), and is forecasted to carry a maximum of  40,000 vehicles (AADT) between Ploiesti and Buzau, around 20,000-30,000 vehicles (AADT) for the sections between Gaesti and Ploiesti and Buzau to Braila. Almost all of the existing traffic in the immediate corridor switches to the new route (15,000 AADT). The flow changes described earlier are illustrated in Figure 4.51 overleaf.

Figure 4.51 Flow Changes Due to Gaesti-Ploiesti-Buzau-Braila Expressway Project

Source: AECOM analysis of Romanian National Transport Model

Average speeds increase to 101 kph as a consequence of the scheme. Peak time journey times are reduced by 17%.

The primary economic indicators for the project are as follows:

- NPV	= €1,754M;

- BCR	= 2.87; and

- EIRR	= 11.9%.

Level 2 Schemes – OR7B – Suceava-Siret Expressway

Brief Description of Proposal:

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds between Suceava and Siret, thus improving regional connectivity. In conjunction with OR7A (Bacau – Suceava), OR 6B (Bacau – Braila/Galati, OR11 (Braila – Constanta) and OR3 (Ploesti – Brasov – Bacau) the scheme provides high quality routes from the Ukraine to the largest ports in Romania, and to the central and southern regions of Romania. It therefore also adds European value.

The indicative alignment is shown in Figure 4.52.

Figure 4.52 Suceava-Siret Expressway Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

The route connects to the node of Suceava on the TEN-T network.

Brief Description of Problem(s) Addressed:

There are poor average travel times; typical average speeds for this route are around 69 kph

The bulk of the infrastructure provision in the corridor is poor – 97% of the route is only single carriageway standard.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€186M

Outcome(s):

This project returns excellent value for money (BCR of 2.84), and carries around 10,000 - 15,000 vehicles (AADT). Almost all of the existing traffic in the immediate corridor switches to the new route.

The flow changes are illustrated in Figure 4.53.

Figure 4.53 Flow Changes Due to Suceava-Siret Expressway Project

Source: AECOM analysis of Romanian National Transport Model

Average speeds increase to 114 kph as a consequence of the scheme. Peak time journey times are reduced by 38%.

The primary economic indicators for the project are as follows:

- NPV	= €1,172M;

- BCR	= 2.84; and

- EIRR	= 11.7%

Level 2 Schemes – OR8 – Bacau-Piatra Neamt Expressway

Brief Description of Proposal

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds between Bacau and Piatra Neamt, thus improving regional connectivity.  The route provides a connection between the County Capitals of Piatra Neamt and Bacau, and via OR 3, and OR11 to the south/centre and major ports of Romania. Analysis of the National Model flows show expressway is the appropriate standard.

The expressway alignment is shown in Figure 4.54.

Figure 4.54 Bacau-Piatra Neamt Expressway Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

The route connects to the node of Bacau on the TEN-T network.

Brief Description of Problem(s) Addressed:

There are poor average travel times; typical average speeds for this route are around 52 kph

The bulk of the infrastructure provision in the corridor is poor – 92% of the route is only single carriageway standard.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€335M

Outcome(s):

This project returns excellent value for money (BCR of 2.45), and carries around 18,000 vehicles (AADT). Almost all of the existing traffic in the immediate corridor switches to the new route; the flow changes are illustrated in Figure 4.55 overleaf.

Figure 4.55	Flow Changes Due to Bacau-Piatra Neamt Expressway Project

Source: AECOM analysis of Romanian National Transport Model

Average speeds increase to 83 kph as a consequence of the scheme. Peak time journey times are reduced by 32%.

The primary economic indicators for the project are as follows:

- NPV	= €350M;

- BCR	= 2.45; and

- EIRR	= 10.6%.

Level 2 Schemes – OR11 – Constanta-Tulcea-Braila Expressway (including Braila Bridge)

Brief Description of Proposal

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds between Constanta and Tulcea, thus improving regional connectivity. This project also satisfies a Regional Development objective, as Tulcea is the gateway to the Danube delta with its unique ecological resources and tourist potential.  The project forms part of the route between Constanta and the North East region of Romania in conjunction with OR7A and OR6B.  Analysis of the National Model flows show that an expressway is the appropriate standard.

Figure 4.56 shows the route alignment.

Figure 4.56	Constanta-Tulcea-Braila Expressway (including Braila Bridge) Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

The route connects to the nodes of Constanta, Tulcea and Braila on the TEN-T network.

Brief Description of Problem(s) Addressed:

There are poor average travel times; typical average speeds for this route are around 71 kph. The bulk of the infrastructure provision in the corridor is poor – 60% of the route is only single carriageway standard.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€1,369M

Outcome(s):

This has project a BCR of 1.07, and carries around 16,000 vehicles (AADT). Almost all of the existing traffic in the immediate corridor switches to the new route; the flow changes are illustrated in Figure 4.57 overleaf.

Figure 4.57 Flow Changes Due to Constanta-Tulcea-Braila Expressway (including Braila Bridge)

Source: AECOM analysis of Romanian National Transport Model

Average speeds increase to 90 kph as a consequence of the scheme. Peak time journey times are reduced by 28%.

The primary economic indicators for the project are as follows:

- NPV	= €66M;

- BCR	= 1.07; and

- EIRR	= 5.4%.

Level 2 Schemes – OR12 – Nadaselu - Suplacu de BarcauExpressway

Brief Description of Proposal

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds on significant regional connectivity corridors such as Cluj to the Border near Oradea. In conjunction with the Reference Case projects and OR2 (Bucharest – Sibiu), the project provides linkage to Hungary and the rest of Europe from the central and south of Romania.  Analysis of the National Model flows show expressway is the appropriate standard.

Figure 4.58 shows the expressway alignment and its relation to the Reference Case scheme.

Figure 4.58 Nadaselu - Suplacu de Barcau Expressway Indicative Alignment

Reference Case

Expressway

Reference Case

Expressway

Source: AECOM analysis of Romanian National Transport Model

Brief Description of Problem(s) Addressed:

There are poor average travel times; typical average speeds for this route are around 63 kph

The bulk of the infrastructure provision in the corridor is poor – 100% of the route is only single carriageway standard.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€550M

Outcome(s):

This project returns a BCR of 3.18, and carries around 20,000 – 25,000 vehicles (AADT). Around half of the existing traffic in the immediate corridor switches to the new route. The flow changes are illustrated in Figure 4.59 overleaf.

Figure 4.59	 Flow Changes Due to Nadaselu - Suplacu de Barcau Expressway

Source: AECOM analysis of Romanian National Transport Model

Average speeds increase to 103 kph as a consequence of the scheme. Peak time journey times are reduced by 30%.

The primary economic indicators for the project are as follows:

- NPV	= €1,458M;

- BCR	= 3.18; and

- EIRR	= 13.2%.

Level 2 Schemes – OR14 – Brasov-Pitesti Expressway

Brief Description of Proposal

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds between Brasov and Pitesti, thus improving regional connectivity. The proposal connects the industrial centres of Craiova and Pitesti with the central and north-eastern regions of Romania, in conjunction with OR 3 (Brasov – Bacau) and OR 7A (Bacau – Suceava). Analysis of the National Model flows shows that an expressway is the appropriate standard.

The indicative alignment is shown in Figure 4.60.

Figure 4.60 	Brasov-Pitesti Expressway Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

The route connects to the node of Brasov on the TEN-T network.

Brief Description of Problem(s) Addressed:

There are poor average travel times; typical average speeds for this route are around 63 kph.

The bulk of the infrastructure provision in the corridor is poor – 85% of the route is only single carriageway standard.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€1,694M

Outcome(s):

This project returns a BCR of 1.97, and carries around 20,000 vehicles (AADT). Almost half of the existing traffic using the immediate corridor switches to the new route. The flow changes are illustrated in Figure 4.61 overleaf.

Figure 4.61 Flow Changes Due to Brasov-Pitesti Expressway

Source: AECOM analysis of Romanian National Transport Model

Average speeds increase to 93 kph as a consequence of the scheme implementation. Meanwhile, peak time journey times are reduced by 37%.

The primary economic indicators for the project are as follows:

- NPV	= €1,222M;

- BCR	= 1.97 and

- EIRR	= 9.1%.

Level 2 Schemes – OR15 – Sibiu-Pitesti Expressway

Brief Description of Proposal

The high level objective for the project is to increase the economic efficiency of the transport network in Romania. The particular operational objective is to improve travel speeds between Sibiu and Pitesti, thus improving regional connectivity. In conjunction with the Reference Case projects, and the A1 between Bucharest and Pitesti, it forms a high quality route from the industrial and logistical centre of Pitesti with the west of Bucharest and Hungary / Rest of Europe. Following an analysis of the National Model flows, and the role which the route performs, it has been decided that a dual 2-lane motorway is the appropriate standard to adopt.

The indicative alignment is shown in Figure 4.62.

Figure 4.62 Sibiu-Pitesti Expressway Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

The route connects to the node of Sibiu on the TEN-T network.

Brief Description of Problem(s) Addressed

Analysis of the base year model shows poor average travel times; where typical average speeds for this route are around 72 kph.

The bulk of the infrastructure provision in the corridor is poor – 94% of the route is only single carriageway standard.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€2,471M

Outcome(s):

This project returns excellent value for money (BCR of 2.01), and carries around 60,000 vehicles (AADT). The project adds approximately 15-20,000 vehicles to the A1 between Bucharest and Pitesti, which will necessitate widening the A1 to 3 lanes in each direction.The flow changes are illustrated in Figure 4.63 overleaf.

Figure 4.63 Flow Changes Due to Sibiu-Pitesti Expressway

Source: AECOM analysis of Romanian National Transport Model

Average speeds increase to 89 kph as a consequence of the scheme. Peak time journey times are reduced by 27%.

- NPV	= €3,301M;

- BCR	= 2.96; and

- EIRR	= 12.1%.

Supplementary Level 2 Schemes – OR13C – Buzau – Focsani Expressway

Brief Description of Proposal:

This is a relatively small scheme which provides an expressway between the key centre of Buzau and Focsani to the North.

The follows a section of the Core TEN-T network and completes a gap in connectivity provided by the Level 2 network.

Brief Description of Problem(s) Addressed

The scheme alleviates some remaining lost time issues on the Bucharest – Buzau – Bacau corridor.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€282M

Outcome(s):

This project returns excellent value for money with the following economic indicators:

- NPV	= €1,350M;

- BCR	= 2.65; and

- EIRR	= 11.0%.

Supplementary Level 2 Schemes – OR19 – Bucharest – Alexandria Expressway

Brief Description of Proposal:

This provides an expressway connection between the capital and Alexandria along this TEN-T Core Network corridor.

The indicative alignment is shown in Figure 4.64.

Figure 4.64 Bucharest – Alexandria Expressway Indicative Alignment

Source: AECOM analysis of Romanian National Transport Model

Brief Description of Problem(s) Addressed

The scheme alleviates some remaining lost time issues on this corridor after the implementation of the Level 1 and other Level 2 schemes.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€370M

Outcome(s):

This project returns excellent value for money (BCR of 2.35), and carries around 20-25,000 vehicles (AADT).The flow changes are illustrated in Figure 4.65 overleaf.

- Figure 4.65 Flow Changes Due to Bucharest – Alexandria Expressway

Source: AECOM analysis of Romanian National Transport Model

The economic indicators for the scheme are as follows:

- NPV	= €486M;

- BCR	= 2.35; and

- EIRR	= 11.8%.

Supplementary Level 2 Schemes – OR19A – Henri Coanda Airport connection to A3

Brief Description of Proposal:

This project provides a useful regional link between road 1 and the A3.  It also provides a link to Henri Coanda Airport from the A3 and forms part of a potential Bucharest Outer Ring Road.

Brief Description of Problem(s) Addressed

The scheme provides an important link between two major corridors and also links to a major international gateway.

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€126M

Outcome(s):

This project returns good value for money (BCR of 1.99), carries around 4,000 vehicles (AADT) and also provides some relief to the Bucharest Ring Road. The flow changes are illustrated in Figure 4.66 overleaf.

Figure 4.66 Flow Changes Due To Henri Coanda to A3 Expressway

Source: AECOM analysis of Romanian National Transport Model

The economic indicators for the scheme are as follows:

- NPV	= €86M;

- BCR	= 1.99; and

- EIRR	= 9.3%.

Supplementary Level 2 Schemes – OR21 Widening of A1 Bucharest to Pitesti

Brief Description of Proposal:

This project involves widening the A1 between Bucharest and Pitesti as referred to in OR15.

Brief Description of Problem(s) Addressed

The scheme addresses the need for additional capacity following improvements on Sibiu – Pitesti which brings more traffic into this important corridor. .

Undiscounted costs (Million €, 2014 prices):

- CAPEX 	€442M

Outcome(s):

This project returns the following economic indicators:

- NPV	= €181M;

- BCR	= 1.65; and

- EIRR	= 7.6%.

Level 3 Schemes

Brief Description of Proposal:

To support the Level 1 and Level 2 schemes, there are further infrastructure improvements at a local level. The locations of the bypass schemes were determined by the volume and type of through traffic, together with the settlement size. These have been determined taking account of linkages with the higher tier (Level 1 and Level 2) schemes.

Economic Indicators:

Economic indicators for each scheme have been determined, and are presented in the following table.

Table 4.21 Summary of Bypass Testing Results (CBA)

| Ref. | Project | Investment cost (undiscounted) €M | NPV €M | BCR | EIRR % |
| --- | --- | --- | --- | --- | --- |
| BP1 | Oradea | 108 | 20 | 1.28 | 6.5% |
| BP2 | Dej | 30 | 107 | 5.91 | 19.7% |
| BP3 | Sighisoara | 48 | 92 | 3.84 | 15.8% |
| BP4 | Roman | 62 | 129 | 4.11 | 15.9% |
| BP5 | Focsani | 76 | 130 | 3.44 | 13.1% |
| BP6 | Ramnicu Sarat | 37 | 8 | 1.31 | 6.8% |
| BP7 | Buzau | 105 | 37 | 1.51 | 7.5% |
| BP8 | Alexandria | 55 | -18 | 0.53 | 1.5% |
| BP9 | Ramnicu Valcea | 195 | 31 | 1.23 | 6.2% |
| BP10 | Targoviste | 78 | 358 | 7.83 | 22.7% |
| BP11 | Filiasi | 28 | 68 | 4.40 | 15.6% |
| BP12 | Adjud | 46 | 170 | 5.96 | 19.0% |
| BP13 | Mizil | 36 | -11 | 0.57 | 1.9% |
| BP14 | Ludus | 102 | 113 | 2.53 | 11.2% |
| BP15 | Falticeni | 41 | 70 | 3.34 | 13.2% |
| BP16 | Caransebes | 81 | 7 | 1.13 | 5.7% |
| BP17 | Beclean | 42 | 24 | 1.81 | 8.4% |
| BP18 | Bistriţa | 157 | 32 | 1.31 | 6.5% |
| BP19 | Miercurea Ciuc | 111 | -3 | 0.95 | 4.8% |
| BP20 | Sfântu Gheorghe | 34 | 41 | 2.84 | 11.4% |
| BP21 | Giurgiu | 72 | 24 | 1.50 | 7.4% |
| BP22 | Vaslui | 73 | 31 | 1.64 | 8.6% |
| BP23 | Bârlad | 51 | 54 | 2.57 | 11.1% |
| BP24 | Slobozia | 23 | 16 | 2.06 | 10.6% |
| BP25 | Vatra Dornei | 18 | 57 | 5.65 | 17.9% |
| BP26 | Câmpulung Moldovenesc | 99 | -23 | 0.65 | 2.5% |
| BP27 | Mangalia | 44 | 14 | 1.49 | 7.5% |
| BP28 | Timisoara South | 127 | 104 | 2.21 | 9.7% |

- Source: AECOM analysis

MCA Analysis

The overall approach to project selection and the way in which projects are combined to form the final scenarios was set out in Section 2.6.

The previous sections in this chapter have dealt with the derivation of the preferred Level 1, Level 2 and Bypass schemes and the MCA now brings these together into a coherent strategy taking on board the funding allocations per sector.

The funding allocations are discussed fully in Section 11.3 but are summarised here for the road network in Table 4.22.

Table 4.22 Funding Allocation for Roads (€M)

| Type | Period |  |  |
| --- | --- | --- | --- |
|  | 2014-2020 | 2021-2030 | 2014-2030 |
| Motorways and expressways | 5,281 | 11,556 | 16,836 |
| BPs | 232 | 834 | 1,066 |

Source: AECOM analysis

Whilst the final grouping of schemes is informed by the model and financial analysis, it is not a purely mechanistic process. At each stage, the masterplan team undertook reality and sense checks to ensure the process did not result in a poorly connected network or one that brought schemes forward for implementation in an illogical manner.

There was also regular dialogue with the Ministry, and its advisors, JASPERS, in order to ensure the process was subject to independent scrutiny. Some particular issues of note are covered in the following.

Bacau - Iasi Motorway was excluded from the list for the following reasons:

Land configuration was identified as being very difficult and there would be considerable uncertainty over the investment costs as a result;

Economic performance indicators and MCA have very low values for a Level 1 project; and

Connections between Iasi-West of Romania and Iasi-Bucuresti could be covered by other means.

An express road linking Buzau and Focsani was added and tested with a cost-benefit analysis in order to rectify a connectivity issue in that corridor.

With regard to the Campia Turzii - Targa Mures – Iasi scheme, it was agreed that the Iasi-Ungheni connection shall be included (which was initially included in the Bacau-Iasi motorway). Also, following discussions with CNADNR representatives on 21.08.2014, the elimination of Campia – Turzii  - Targu Mures section was decided, as this is to be implemented as a motorway, under the current programming period.

The final ES and EES scenarios are shown in Table 4.23 and 4.24 which set out the designation (in TEN-T terms) of the section, the MCA score, the EIRR, the cost (together with a cumulative cost) and the implementation period. The networks are also illustrated in Figures 4.65 to 4.68.

Recognising the importance of the TEN-T network definition, it was agreed to also undertake a further comparator test based on only those identified schemes which were on the Core TEN-T.

Table 4.25 shows a similar breakdown in terms of TEN-T schemes and how they could be accommodated within the financial constraints discussed earlier.

With the TEN-T comparator, fewer individual schemes are achievable within the period up to 2020.

In terms of economic efficiency, or value for money, the ES and EES scenarios perform very similarly with BCRs of the order of 2.1. The Core TEN-T comparator fares considerably less well at around 1.7.

Table 4.24 ES Scenario – Prioritisation of schemes

|  | Code |  | Project Description | TEN-T | Score | EIRR | Cost (2014 prices) | Cumulated Cost |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Level 1 | H7 | Sibiu-Brasov Motorway | Comprehensive TEN-T link | 74.3 | 17.3% | 817.3 | 817.3 |
| 2 |  | H8 | Ploiesti-Comarnic Motorway | Comprehensive TEN-T link | 48.7 | 12.5% | 310.4 | 1,127.7 |
| 3 |  | H6 | Craiova-Pitesti Motorway | Comprehensive TEN-T link | 47.3 | 12.2% | 870.3 | 1,998.0 |
| 4 |  | H1 | Comarnic-Brasov Motorway | Comprehensive TEN-T link | 29.3 | 8.8% | 1,117.0 | 3,115.0 |
| 5 |  | H12 | Brasov-Bacau Motorway | Comprehensive TEN-T link | 20.1 | 7.1% | 2,067.6 | 5,182.6 |
| 1 | Level 2 | OR18B | Bucharest Southern Ring Road Upgrade | Core TEN-T link | 100.0 | 14.5% | 175.7 | 5,358.3 |
| 2 |  | OR7A | Bacau-Suceava Expressway | Core TEN-T link | 86.3 | 12.6% | 645.4 | 6,003.7 |
| 3 |  | OR15 | Sibiu-Pitesti Motorway | Core TEN-T link | 82.3 | 12.1% | 2,471.2 | 8,474.9 |
| 4 |  | OR19 | Bucharest-Alexandria Expressway | Core TEN-T link | 79.8 | 11.8% | 369.6 | 8,844.5 |
| 5 |  | OR7B | Suceava-Siret Expressway | Core TEN-T link | 79.1 | 11.7% | 186.1 | 9,030.6 |
| 6 |  | OR13C | Buzau-Focsani Expressway | Core TEN-T link | 74.5 | 11.0% | 282.0 | 9,312.6 |
| 7 |  | OR13D | Targu Neamt-Iasi-Ungheni Motorway | Core TEN-T link | 72.7 | 10.8% | 700.0 | 10,012.6 |
| 8 |  | OR12 | Nadaselu - Suplacu de Barcau Motorway | Comprehensive TEN-T link | 69.3 | 13.2% | 550.0 | 10,562.6 |
| 9 |  | OR10 | Lugoj- Craiova Expressway | Core TEN-T link | 66.6 | 10.0% | 1,810.9 | 12,373.5 |
| 10 |  | OR9B | Turda-Halmeu Expressway | Other links | 63.0 | 13.5% | 975.4 | 13,348.9 |
| 11 |  | OR19D | Henri Coanda Airport connection to A3 | Core TEN-T link | 61.9 | 9.3% | 125.6 | 13,474.5 |
| 12 |  | OR17 | Gaesti-Ploiesti-Buzau-Braila Expressway | Comprehensive TEN-T link | 60.0 | 11.9% | 1,279.6 | 14,754.1 |
| 13 |  | OR13 | Targu Mures-Targu Neamt Motorway | Core TEN-T link | 55.0 | 8.4% | 3,400.0 | 18,154.1 |
| 14 |  | OR6B | Bacau-Focsani-Braila-Galati Expressway | Comprehensive TEN-T link | 54.4 | 11.2% | 1,024.2 | 19,178.3 |
| 15 |  | OR21 | A1 Widening Bucharest-Pitesti | Core TEN-T link | 49.1 | 7.6% | 442.0 | 19,620.3 |
| 16 |  | OR8 | Bacau-Piatra Neamt Expressway | Other links | 41.2 | 10.6% | 335.1 | 19,955.4 |
| 17 |  | OR14 | Brasov-Pitesti Expressway | Comprehensive TEN-T link | 39.0 | 9.1% | 1,842.6 | 21,798.0 |
| 18 |  | OR18A | Bucharest Ring Road Motorway | Core TEN-T link | 35.2 | 5.7% | 1,683.8 | 23,481.8 |
| 19 |  | OR7C | Suceava-Botosani Expressway | Other links | 28.4 | 8.9% | 345.8 | 23,827.6 |
| 20 |  | OR11 | Constanta-Tulcea-Braila Expressway (including Braila Bridge) | Comprehensive TEN-T link | 11.9 | 5.4% | 1,369.3 | 25,196.9 |
| 1 | Bypasses | BP12 | Adjud | Core TEN-T link | 85.4 | 19.0% | 46.2 | 46.2 |
| 2 |  | BP10 | Targoviste | Comprehensive TEN-T link | 79.0 | 22.7% | 78.0 | 124.2 |
| 3 |  | BP4 | Roman | Core TEN-T link | 73.2 | 15.9% | 62.0 | 186.1 |
| 4 |  | BP11 | Filiasi | Core TEN-T link | 71.8 | 15.6% | 27.7 | 213.8 |
| 5 |  | BP15 | Falticeni | Core TEN-T link | 62.4 | 13.2% | 41.3 | 255.1 |
| 6 |  | BP5 | Focsani | Core TEN-T link | 62.1 | 13.1% | 76.1 | 331.2 |
| 7 |  | BP25 | Vatra Dornei | Comprehensive TEN-T link | 60.2 | 17.9% | 18.3 | 349.4 |
| 8 |  | BP3 | Sighisoara | Comprehensive TEN-T link | 51.8 | 15.8% | 47.7 | 397.1 |
| 9 |  | BP28 | Timisoara South | Core TEN-T link | 48.5 | 9.7% | 127.4 | 524.5 |
| 10 |  | BP7 | Buzau | Core TEN-T link | 39.7 | 7.5% | 104.8 | 629.3 |
| 11 |  | BP21 | Giurgiu | Core TEN-T link | 39.3 | 7.4% | 72.0 | 701.4 |
| 12 |  | BP6 | Ramnicu Sarat | Core TEN-T link | 37.1 | 6.8% | 37.0 | 738.4 |
| 13 |  | BP9 | Ramnicu Valcea | Core TEN-T link | 34.8 | 6.2% | 195.4 | 933.8 |
| 14 |  | BP20 | Sfântu Gheorghe | Comprehensive TEN-T link | 34.3 | 11.4% | 34.0 | 967.8 |
| 15 |  | BP14 | Ludus | Comprehensive TEN-T link | 33.5 | 11.2% | 102.5 | 1,070.3 |
| 16 |  | BP23 | Bârlad | Comprehensive TEN-T link | 33.3 | 11.1% | 51.1 | 1,121.4 |
| 17 |  | BP16 | Caransebes | Core TEN-T link | 32.6 | 5.7% | 80.8 | 1,202.2 |
| 18 |  | BP24 | Slobozia | Comprehensive TEN-T link | 31.1 | 10.6% | 23.3 | 1,225.6 |
| 19 |  | BP13 | Mizil | Core TEN-T link | 30.0 | 1.9% | 36.0 | 1,261.5 |
| 20 |  | BP22 | Vaslui | Comprehensive TEN-T link | 23.4 | 8.6% | 72.9 | 1,334.5 |
| 21 |  | BP17 | Beclean | Comprehensive TEN-T link | 22.6 | 8.4% | 42.2 | 1,376.7 |
| 22 |  | BP27 | Mangalia | Comprehensive TEN-T link | 18.9 | 7.5% | 44.1 | 1,420.8 |
| 23 |  | BP18 | Bistriţa | Comprehensive TEN-T link | 15.1 | 6.5% | 157.0 | 1,577.8 |
| 24 |  | BP19 | Miercurea Ciuc | Comprehensive TEN-T link | 9.0 | 4.8% | 110.5 | 1,688.3 |
| 25 |  | BP26 | Câmpulung Moldovenesc | Comprehensive TEN-T link | 9.0 | 2.5% | 99.3 | 1,787.6 |

Source: AECOM analysis

Table 4.25	 EES Scenario – Prioritisation of schemes

|  | Code |  | Project Description | TEN-T | Score | EIRR | Cost (2014 prices) | Cumulated Cost |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Level 1 | H7 | Sibiu-Brasov Motorway | Comprehensive TEN-T link | 74.3 | 17.3% | 817.3 | 817.3 |
| 2 |  | H8 | Ploiesti-Comarnic Motorway | Comprehensive TEN-T link | 48.7 | 12.5% | 310.4 | 1,127.7 |
| 3 |  | H6 | Craiova-Pitesti Motorway | Comprehensive TEN-T link | 47.3 | 12.2% | 870.3 | 1,998.0 |
| 4 |  | H1 | Comarnic-Brasov Motorway | Comprehensive TEN-T link | 29.3 | 8.8% | 1,117.0 | 3,115.0 |
| 5 |  | H12 | Brasov-Bacau Motorway | Comprehensive TEN-T link | 20.1 | 7.1% | 2,067.6 | 5,182.6 |
| 1 | Level 2 | OR18B | Bucharest Southern Ring Road Upgrade | Core TEN-T link | 71.0 | 14.5% | 175.7 | 5,358.3 |
| 2 |  | OR7A | Bacau-Suceava Expressway | Core TEN-T link | 70.2 | 12.6% | 645.4 | 6,003.7 |
| 3 |  | OR12 | Nadaselu - Suplacu de Barcau Motorway | Comprehensive TEN-T link | 57.0 | 13.2% | 550.0 | 6,553.7 |
| 4 |  | OR19 | Bucharest-Alexandria Expressway | Core TEN-T link | 56.6 | 11.8% | 369.6 | 6,923.3 |
| 5 |  | OR13C | Buzau-Focsani Expressway | Core TEN-T link | 55.8 | 11.0% | 282.0 | 7,205.4 |
| 6 |  | OR7B | Suceava-Siret Expressway | Core TEN-T link | 52.1 | 11.7% | 186.1 | 7,391.5 |
| 7 |  | OR13D | Targu Neamt-Iasi-Ungheni Motorway | Core TEN-T link | 51.5 | 10.8% | 700.0 | 8,091.5 |
| 8 |  | OR15 | Sibiu-Pitesti Motorway | Core TEN-T link | 51.3 | 12.1% | 2,471.2 | 10,562.6 |
| 9 |  | OR6B | Bacau-Focsani-Braila-Galati Expressway | Comprehensive TEN-T link | 46.5 | 11.2% | 1,024.2 | 11,586.8 |
| 10 |  | OR9B | Turda-Halmeu Expressway | Other links | 44.0 | 13.5% | 975.4 | 12,562.2 |
| 11 |  | OR19D | Henri Coanda Airport connection to A3 | Core TEN-T link | 43.8 | 9.3% | 125.6 | 12,687.8 |
| 12 |  | OR17 | Gaesti-Ploiesti-Buzau-Braila Expressway | Comprehensive TEN-T link | 43.4 | 11.9% | 1,279.6 | 13,967.4 |
| 13 |  | OR10 | Lugoj- Craiova Expressway | Core TEN-T link | 40.1 | 10.0% | 1,810.9 | 15,778.3 |
| 14 |  | OR21 | A1 Widening Bucharest-Pitesti | Core TEN-T link | 34.7 | 7.6% | 442.0 | 16,220.3 |
| 15 |  | OR7C | Suceava-Botosani Expressway | Secondary connectivity with TEN-T | 32.3 | 8.9% | 345.8 | 16,566.1 |
| 16 |  | OR13 | Targu Mures-Targu Neamt Motorway | Core TEN-T link | 31.9 | 8.4% | 3,400.0 | 19,966.1 |
| 17 |  | OR14 | Brasov-Pitesti Expressway | Comprehensive TEN-T link | 25.4 | 9.1% | 1,842.6 | 21,808.7 |
| 18 |  | OR18A | Bucharest Ring Road Motorway | Core TEN-T link | 24.7 | 5.7% | 1,683.8 | 23,492.5 |
| 19 |  | OR8 | Bacau-Piatra Neamt Expressway | Other links | 19.4 | 10.6% | 335.1 | 23,827.6 |
| 20 |  | OR11 | Constanta-Tulcea-Braila Expressway (including Braila Bridge) | Comprehensive TEN-T link | 17.1 | 5.4% | 1,369.3 | 25,196.9 |
| 1 | Bypasses | BP10 | Targoviste | Comprehensive TEN-T link | 57.0 | 22.7% | 78.0 | 78.0 |
| 2 |  | BP12 | Adjud | Core TEN-T link | 56.6 | 19.0% | 46.2 | 124.2 |
| 3 |  | BP4 | Roman | Core TEN-T link | 47.9 | 15.9% | 62.0 | 186.1 |
| 4 |  | BP11 | Filiasi | Core TEN-T link | 46.9 | 15.6% | 27.7 | 213.8 |
| 5 |  | BP25 | Vatra Dornei | Comprehensive TEN-T link | 43.6 | 17.9% | 18.3 | 232.1 |
| 6 |  | BP15 | Falticeni | Core TEN-T link | 40.1 | 13.2% | 41.3 | 273.3 |
| 7 |  | BP5 | Focsani | Core TEN-T link | 39.9 | 13.1% | 76.1 | 349.4 |
| 8 |  | BP3 | Sighisoara | Comprehensive TEN-T link | 37.6 | 15.8% | 47.7 | 397.1 |
| 9 |  | BP28 | Timisoara South | Core TEN-T link | 33.2 | 9.7% | 127.4 | 524.5 |
| 10 |  | BP20 | Sfântu Gheorghe | Comprehensive TEN-T link | 25.1 | 11.4% | 34.0 | 558.5 |
| 11 |  | BP14 | Ludus | Comprehensive TEN-T link | 24.5 | 11.2% | 102.5 | 661.0 |
| 12 |  | BP23 | Bârlad | Comprehensive TEN-T link | 24.3 | 11.1% | 51.1 | 712.1 |
| 13 |  | BP7 | Buzau | Core TEN-T link | 23.9 | 7.5% | 104.8 | 817.0 |
| 14 |  | BP21 | Giurgiu | Core TEN-T link | 23.7 | 7.4% | 72.0 | 889.0 |
| 15 |  | BP24 | Slobozia | Comprehensive TEN-T link | 22.8 | 10.6% | 23.3 | 912.3 |
| 16 |  | BP6 | Ramnicu Sarat | Core TEN-T link | 22.0 | 6.8% | 37.0 | 949.4 |
| 17 |  | BP9 | Ramnicu Valcea | Core TEN-T link | 20.5 | 6.2% | 195.4 | 1,144.8 |
| 18 |  | BP16 | Caransebes | Core TEN-T link | 18.9 | 5.7% | 80.8 | 1,225.6 |
| 19 |  | BP22 | Vaslui | Comprehensive TEN-T link | 17.3 | 8.6% | 72.9 | 1,298.5 |
| 20 |  | BP13 | Mizil | Core TEN-T link | 17.0 | 1.9% | 36.0 | 1,334.5 |
| 21 |  | BP17 | Beclean | Comprehensive TEN-T link | 16.7 | 8.4% | 42.2 | 1,376.7 |
| 22 |  | BP27 | Mangalia | Comprehensive TEN-T link | 14.1 | 7.5% | 44.1 | 1,420.8 |
| 23 |  | BP18 | Bistriţa | Comprehensive TEN-T link | 11.4 | 6.5% | 157.0 | 1,577.8 |
| 24 |  | BP19 | Miercurea Ciuc | Comprehensive TEN-T link | 7.0 | 4.8% | 110.5 | 1,688.3 |
| 25 |  | BP26 | Câmpulung Moldovenesc | Comprehensive TEN-T link | 7.0 | 2.5% | 99.3 | 1,787.6 |

Source: AECOM analysis

Table 4.26 	TEN-T Comparator Scenario – Prioritisation of schemes

|  | Code |  | Project Description | TEN-T | Score | EIRR | Cost (2014 prices) | Cumulated Cost |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Completion of Core TEN-T | OR18B | Bucharest Southern Ring Road Upgrade | Core TEN-T link | 100.0 | 14.5% | 175.7 | 175.7 |
| 2 |  | H2 | Sibiu-Pitesti Motorway | Core TEN-T link | 74.7 | 12.1% | 2,471.2 | 2,646.9 |
| 2a |  | H11a | Ploiesti-Bacau Motorway | Core TEN-T link | 63.6 | 11.0% | 1,700.0 | 4,346.9 |
| 3 |  | H11 | Bacau-Suceava-Siret Motorway | Core TEN-T link | 63.6 | 11.0% | 1,781.3 | 6,128.1 |
| 4 |  | H13a | Campia Turzii - Targu Mures Motorway | Core TEN-T link | 47.4 | 9.5% | 450.0 | 6,578.1 |
| 5 |  | H15 | Targu Mures-Iasi-Ungheni Motorway | Core TEN-T link | 39.4 | 8.7% | 5,056.3 | 11,634.4 |
| 6 |  | H28 | Lugoj - Craiova Motorway | Core TEN-T link | 28.5 | 7.7% | 2,399.2 | 14,033.6 |
| 7 |  | H9 | Bucharest-Alexandria-Craiova Motorway | Core TEN-T link | 23.0 | 7.2% | 1,189.8 | 15,223.4 |
| 8 |  | H10 | Craiova-Calafat Motorway | Core TEN-T link | 2.3 | 5.2% | 419.2 | 15,642.6 |
| 9 |  | H29 | Drobeta - Calafat Motorway (including H28) | Core TEN-T link | 0.0 | 4.5% | 482.0 | 16,124.6 |
| 10 |  | H27 | Timisoara - Moravita Motorway | Core TEN-T link | 0.0 | 3.7% | 470.4 | 16,595.1 |

Source: AECOM analysis

Figure 4.65 EES Final Strategy (2020) Highway Network

Source: AECOM analysis of Romanian National Transport Model

Figure 4.66 EES Final Strategy (2030) Highway Network

Source: AECOM analysis of Romanian National Transport Model

Figure 4.67 ES Final Strategy (2020) Highway Network

Source: AECOM analysis of Romanian National Transport Model

Figure 4.68 ES Final Strategy (2030) Highway Network

Source: AECOM analysis of Romanian National Transport Model

Summary of Interventions

In addition to the infrastructure measures which have been discussed in this Chapter, there are a number of other proposed measures which seek to address the overall objectives of the Masterplan.

Maintenance Related

As set out in Table 4.4 earlier in this chapter, only just over 50% of the national network is classified as good with a further 30% average and 20% poor. The expectation should be that the national network should be at the top end of the standard for any country. Figure 4.4 also provided information in respect of the proportion of the national road network for which the design life has expired.

Furthermore, 48% of all routes have some element classified poor. Of these the average proportion, by length, is 24%. There is no distinction by topography, thereby suggesting a network wide issue.

The World Economic Forum, in their report on “Global Competitiveness (2011–2012)”, places Romania 137th out of 142 countries considered with regard to the quality of the road infrastructure.

It is evident that at least part of the problem revolves around maintenance activities, the way they are undertaken and managed as well as the way in which they are prioritised. It is proposed that the following measures are implemented to help improve the maintenance regime in Romania and meet the goal of a safe and operational road transport network:

Adopt ‘Best Practice’ Pavement Management System (PMS);

Undertake an Asset Management Review;

Detailed Analysis of Operation and Management Funding Requirements;

Operation and Management Prioritisation and Allocation;

Define Quality of Service Parameters; and

Longer Term Contractual Arrangements.

A PMS will:

Evaluate the road condition;

Assess priorities; and

Optimise interventions.

The outcome will be a more transparent, evidence-based approach to maintenance activities. Whilst there will always be limits on the amount of funding available, a best practice PMS will optimise the use of what funds are available and ensure best value is obtained by applying whole life costing approaches.

The PMS will form the basis for planning and prioritising all future road maintenance activities in Romania and will support the principles set out in Clause 8 of EU Regulation No. 1315/2013 supporting the rehabilitation and upgrading of existing infrastructure.

The Asset Management Review proposal will involve carrying out a detailed study review of the existing road assets data and systems in order to establish accuracy, completeness and appropriateness of the currently available inventory, inspection and maintenance records data and functionality of the associated asset management systems such as PMS, BMS etc. together with appropriate improvements measures.

A detailed analysis of operation and management funding requirements (both for backlog removal and on-going routine & planned maintenance activities for all current and planned assets) and historic allocations will identify appropriate annual operation and management funding needs and their earmarking.

This should be ensured in long-term by government commitments, supported by appropriate additional revenue sources e.g. increased vignette levels.

Operation and Management Prioritisation and Allocation involves introducing an efficient and effective system for prioritisation and allocation of operation and management expenditures on a multi-year basis. Furthermore, developing dedicated backlog removal plans for all major assets (roads and bridges). This will lead to a structured approach to operation and maintenance, thus ensuring schemes are prioritised effectively.

In order to define Quality of Service Parameters, a review of the existing relevant operation and management standards and specifications, in terms of their appropriateness and affordability, will be required. The result will lead to a standardised level of service provision.

A move towards longer term contracts for maintenance operations will improve the service provided with better consistency at a regional level. These will become more effective given that the contracts will be performance related. It is proposed that these contracts are regionally based.

The proposal will reform the current maintenance practice of around 50 contracts, lasting a maximum of two years, by proceeding systematically with longer term (5-7 years) performance based contracts at regional scale, more in line with best practice elsewhere in the EU.

Project Management Reform

A high level objective for the project is to improve the management and operation of the infrastructure in Romania. Improved institutional & organisation stability and capacity will ensure efficient and effective management of activities in the road sector during all project phases.

We recommend a full audit and review of the existing situation followed by identification of relevant measures and recommendations in terms of the organisation of the departments to be in charge of all stages of a project cycle. This would include a Training Needs Assessment for all key staff.

Furthermore we would recommend that the Ministry appoint and retain management staff, based on longer term performance-based contracts. These reforms in project management will make the delivery of schemes more effective.

Goods Vehicle Related

There are a number of proposals associated with goods vehicles:

Monitoring of Axle Loading;

Network Charging;

Parking Areas for Good Vehicles; and

ITS (Border Delays).

Reviewing and reforming legislation around the monitoring of axle loading, and the subsequent enforcement of regulations, will reduce the instances of asset damage, therefore reducing maintenance requirements.

The high level objective for the project is to improve the management and operation of the infrastructure in Romania. In particular, the operational objective is to limit damage to roads caused by excessive overloading of trucks. The proposal is to review the appropriateness of axle load legislation and ensure its effective enforcement via regular spot-checks and fines for non-compliance.

There will also be a requirement to ensure sufficient monitoring and checking procedures / facilities in place.

Currently, there is a lack of a co-ordinated charging regime for goods vehicles with county and municipal authorities charging additional fees over and above national vignette. This leads to high administrative costs on local and national hauliers from complex and uncoordinated charging regimes. These vignettes are not co-ordinated or easily payable by international or regional hauliers.

A national charging system for goods vehicles will standardise the current local arrangements and provide a mechanism to ease the administration for international hauliers.

There is currently no secure truck parking provision in Romania, even on new highway schemes. This presents a security and safety problem for hauliers and other road users and imposes unnecessary costs on the system.

EC Regulation 1315/2013 requires for TEN-T appropriate parking areas every 100 km. As the entire TEN-T network (core and comprehensive) comprises approximately 4,900 km, this implies 49 such facilities or 27 for the core only. Unit costs for a service area are approximately €20M so the total cost would be €540M for the core and €980M for the core and comprehensive.

Secure parking locations will adhere to EU requirements, and best practice, and reduce the cost of haulage operations through improved security during journeys.

Using ITS to provide alternative route information and improved management of border crossing arrangements will help to reduce delays. The European Standard, as set out in the Trade and Transport Facilitation in Southeast Europe Program (TTFSE), is that the maximum waiting time for any vehicle is 40 minutes at the border crossing points.

Rail Transport

Rail Transport

Existing Situation and Historic Trends

Passenger usage

The first part of this chapter reviews the recent trends in passenger numbers, as well as assessing the pattern of rail trips between the main population centres using 2011 data. The Figure 5.1 illustrates the number of journeys declined between 2004 and 2012 from about 100m per annum to 58m. The 28% decline in patronage experienced in Romania between 2004 and 2009 is the largest decrease across all the EU Member states.

Figure 5.1: Number of Rail Passenger Journeys per annum in Romania (2004-2012)

Source: AECOM analysis of CFR Calatori data

Rail demand in Romania, measured by kilometres travelled per person, is two to three times lower compared with other countries. The EU27 average is 650km per passenger per year while the equivalent figure for Romania is 66% lower. The relationship between passenger kilometres per person has also been benchmarked for Romania against other EU countries. The ratio for Romania (239 passenger kilometres / person) is lower than other EU countries except Estonia (172), Greece (118) and Lithuania (34). The results for Romania are lower than Bulgaria (270 passenger kilometres / person) or Hungary (731), as shown in Figure 5.2. Compared with countries with a similar level of rail network provision per head of population, the rate of railway use in Romania is low.

Figure 5.2: Comparison of Total Rail Passenger Kilometres – Romania and Other EU Countries

Source: http://ec.europa.eu/transport/facts-fundings/statistics/pocketbook-2013_en.htm

Figure 5.3 shows the magnitude of the network reductions experienced between 1970 and 2011 in most of the EU counties, with an average reduction of 13% across EU, including Germany (-23%), Belgium (-23%), France (-18%), UK (-17%), Poland (-26%), and Portugal (-22%). There were also substantial reductions before 1970, in countries such as Ireland (-60%), UK (-36%), Sweden (-28%) and significant also in France or Germany (around -10%). Overall it can be concluded that reductions in the range of 30% or even more of the network are quite common in Europe during this 30 year period, including in some of the richest Member States like Germany, France and UK. Compared to these, the reductions put in place in Romania by 2011 (-5% since 1990 or -2% since 1970) are insignificant.

Figure 5.3: Reduction in Network Size (1970-2011)

Source: Eurostats

Overall Mode Share Statistics

Table 5.1 illustrates the change in the proportion of trips by rail, road and other modes between 2004 and 2012. Data supplied by CFR Calatori indicates rail mode share has dropped from almost one-third in 2004 to about one-sixth by 2012. The percentage of trips by road has increased by over 10% over the same period, with the domestic air market accounting for the remaining trips.

Table 5.1: Mode Share Comparison for Rail and Road – 2004 to 2012

|  | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 | 2010 | 2011 | 2012 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rail | 31.3% | 27.8% | 29.1% | 27.4% | 20.7% | 20.6% | 20.1% | 19.7% | 17.8% |
| Road | 68.2% | 71.6% | 70.2% | 71.6% | 78.4% | 76.7% | 76.7% | 76.7% | 78.7% |
| Other | 0.5% | 0.5% | 0.7% | 1.1% | 1.0% | 2.8% | 3.2% | 3.6% | 3.4% |

Source: CFR Calatori

These statistics indicate rail is capturing a good percentage of the total trips, albeit recognising the actual trip rate per person is significantly lower versus other EU partners. The comparison of passenger kilometres per person for Romania against other EU countries indicates there is significant scope to grow the market if service improvements were delivered.

Travel data for 2011 has been examined to identify the most prominent travel patterns. The following sectors cover a much larger geographic area compared with the individual zones in the National Model. Table 5.2 illustrates the total number of trips between the sectors, plus the intra-sector demand

| - Bucharest | - Constanta / Bacau / Iasi / Suceava | - Galati / Focsani / Buzau |
| --- | --- | --- |
| - Bacau / Iasi / Suceava | - Brasov / Sibiu / Targu Mures | - Craiova |
| - Timisoara / Arad | - Oradea / Cluj Napoca / Satu Mare / Baia Mare |  |

Table 5.2: Summary of Daily Rail Trips – Sector to Sector Demand

| Sector | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 Bucharest | 15,562 | 1,618 | 2,757 | 2,015 | 1,908 | 3,370 | 1,178 | 1,223 | 29,631 |
| 2 Constanta | 1,755 | 3,900 | 394 | 577 | 302 | 360 | 597 | 241 | 8,126 |
| 3 Galati / Focsani / Buzau | 2,873 | 376 | 10,890 | 850 | 787 | 357 | 242 | 305 | 16,680 |
| 4 Bacau / Iasi / Suceava | 2,103 | 570 | 871 | 14,673 | 552 | 590 | 1,040 | 1,358 | 21,757 |
| 5 Brasov / Sibiu / Targu Mures | 1,944 | 292 | 596 | 525 | 17,927 | 623 | 1,123 | 1,349 | 24,379 |
| 6 Craiova | 3,446 | 364 | 360 | 590 | 652 | 9,805 | 1,165 | 357 | 16,739 |
| 7 Timisoara / Arad | 1,256 | 670 | 257 | 1,228 | 1,158 | 1,229 | 17,873 | 1,026 | 24,697 |
| 8 Oradea / Cluj / S Mare / B Mare | 1,252 | 254 | 307 | 1,492 | 1,330 | 357 | 983 | 15,787 | 21,762 |

Source: AECOM analysis of National Transport Model

The main conclusions are:

Bucharest generates the highest number of trips, with almost 30,000 journeys per day. About 36% of all rail journeys start or finish in this sector, which covers a wide area including Ploiesti. There is some evidence of longer distance travel from Bucharest, with about 3,400 trips to Craiova, 2,750 to Galati / Focsani / Buzau and 2,000 to Bacau / Iasi / Suceava and Brasov / Sibiu / Targu Mures;

There are two other sectors which generate over 24,000 daily trips. In contrast to Bucharest, the Brasov / Sibiu / Targu Mures and Timisoara / Arad sectors have a much higher percentage of trips that start / finish within the same sector (about 70% in both cases). These results indicate the relatively self-contained nature of these sectors, and the extent of the local rail networks around Brasov and Timisoara. In the case of the Timisoara / Arad sector, the long distance to Bucharest is a contributory factor;

Although Galati / Focsani / Buzau, Craiova, Oradea / Cluj Napoca / Satu Mare / Baia Mare generate a smaller number of daily trips compared with the other sectors described above (typically 16,500 to 22,000), there is also a relatively high percentage of internal journeys. Again, this illustrates the self-contained nature of the rail trips generated from the population catchments within these sectors. The Constanta sector generates the smallest total number of trips, with less than 8,200 per day. Less than 50% of trips have an origin and destination within this sector. The relative proximity to Bucharest and the lack of other major catchments adjacent to Constanta has contributed to this outcome.

The high percentage of trips that start / finish within the same sector indicate rail is only used for longer distance journeys by a relatively small percentage of passengers. This reflects historic journey patterns which are a product of relatively slow connections between the major cities, but it also indicates that an important market for rail are inter-city journeys of between 2-4 hours duration.

The rail market share for some of the largest travel flows in Romania is shown below. The corridors with the highest rail flows attract rail mode shares of up to 25%, particularly if the road based alternatives are less convenient, for example, Craiova to Bucharest. Other short distance flows affected by strong competition from road based modes attract just 2% of journeys by rail, for example, Timisoara to Arad.

Bucuresti to Craiova: 19% mode share;

Bucuresti to Arad: 25% mode share;

Bucuresti to Cluj Napoca: 10% mode share;

Bucuresti to Brasov: 5% mode share;

Bucuresti to Iasi: 9% mode share;

Bucuresti to Galati: 9% mode share;

Bucuresti to Constanta: 4% mode share;

Craiova to Timisoara: 1% rail mode share

Timisoara to Arad: 2% mode share;

The journey patterns summarised have also been presented graphically in Figure 5.4. There is some overlap with the busiest network sections in terms of the service frequencies also attracting the highest passenger numbers. The principal flows are summarised below:

The approach to Bucuresti Nord attracts up to 70,000 passengers per day;

Bucuresti Nord to Focsani, plus the sections to Brasov and Craiova attract over 9,000 trips per day with up to 27,500 using the busiest sections;

The number of passengers using the rail network towards Timisoara, Arad, Oradea, Cluj Napoca, Iasi and Suceava exceeds 3,500 per day, although there are generally fewer trains compared with the frequencies towards Bucuresti Nord;

There are numerous branch lines which attract fewer than 1,000 passengers per day, although this is generally consistent with the lower service frequencies (typically less than 8 trains per day).

Figure 5.4: Summary of Daily Rail Passenger Trip Patterns (2011)

Source: AECOM analysis

Station Usage

An analysis of demand shows that approximately 23% of stations are generating 90% of railway demand, which includes 1% of stations that generate 42% of trips.  This suggests that there are a large number of stations that only generate a negligible demand. Nevertheless, these stations continue to be served, albeit with relatively low frequencies. For example, nearly 1,000 stations in Romania generate fewer than 50 trips per day, with some stations getting no service. This has implications on the cost required to operate services, given the limited revenue from these stations;

Table 5.3: Station Boarding Patterns

| Number of Daily Boardings | Number of stations |
| --- | --- |
| >1,000 boardings | 42 |
| 500-1,000 boardings | 42 |
| 251-500 boardings | 71 |
| 101-250 boardings | 188 |
| 51-100 boardings | 175 |
| 11-50 boardings | 435 |
| <10 boardings | 535 |

Source: AECOM analysis of CFR Calatori data

A review of the detailed station loading data from other countries indicates the following trends:Romania: 1% of stations generate 42% of demand, with 23% of stations generating 90%;

England: 1% of stations generate 35% of demand, with 33% of stations generating 90%;

Scotland: 1% of stations generate 44% of demand, with 37% of stations generating 90%;

Wales: 1% of stations generate 35% of demand, with 31% of stations generating 90%.

This comparison highlights there is a higher percentage of stations which only generate 10% of demand versus England, Scotland or Wales. These results indicate there are a much larger percentage of stations in Romania that generate very limited demand compared with the UK examples. Figure 5.5 illustrates the cumulative distribution.

Figure 5.5: Cumulative Distribution of Passenger Numbers using Stations

Source: AECOM analysis of data from the Romanian National Transport Model

Ticketless Travel

There is a range of evidence which illustrates the level of ticketless travel. For example, CFR Calatori has previously estimated that the percentage of passengers travelling without tickets is about 5% which would be comparable to some other European railways. However, there is alternative evidence which suggests that ticketless travel is significantly higher, with almost one-third of passengers travelling without a ticket. Anecdotal evidence suggests that the proportion travelling without a ticket may be closer to the higher estimate. In any event, measures which seek to minimise the proportion of ticketless travel should form the basis of future proposals since the incremental revenue will help to strengthen the financial case for investment. The estimated revenue loss is about €50m per annum. Lines operated by CFR Calatori generally have the highest levels of fare evasion which reflects the most onerous revenue protection measures now enforced by private operators.

Ticket Prices

A benchmarking exercise indicates that Romanian rail fares are relatively high compared with other European countries when they are normalised to take account of comparative spending power. This constraint is particularly relevant for advance purchase tickets. The high fares restrict the opportunities to attract new passengers and reduces the net economic benefit of the railway system, as shown in Figure 5.6.

Figure 5.6: Comparison of Romanian Fares with Selected EU Examples (Cost € per 100km trip)

Source: AECOM analysis of a selection of fares per 100 km of journey. Fares are adjusted to take account of differences in GDP

Freight Usage

Romania is relatively large and hence is suited to rail. Although it is not universally the case, goods that have to be transported large distances can be moved more economically by rail due to the better economies of scale that rail offers over road. However, Romania’s rail freight sector is in a long term decline which is due in part to established industries such as steel-making being affected by lower level of output. Rail still achieved an 18% market share in 2012 which represented a 1% fall from 2011. However rail freight is expected to continue losing market share unless it becomes involved in newer, more dynamic industrial sectors. Rail freight’s three largest commodity flows in 2011 were solid mineral fuels, petroleum products and metal products – which are all linked to more traditional industriesas shown in Figure 5.7. Furthermore, road has established itself as a strong competitor to rail, offering cheaper prices alongside faster and more reliable journey times.

Figure 5.7: Freight Market Share in Romania (2012)

Source:  http://www.insse.ro/cms/files/publicatii/Romania%20in%20cifre%202013_ro.pdf

Rail’s current top three commodity groups are Solid Mineral Fuels, Petroleum Products and Metal Products (see Figure 8.19).

Figure 5.8: Daily Commodity Flows (Tonnes) by Type and Mode Share

Source:  http://www.insse.ro/cms/files/publicatii/Romania%20in%20cifre%202013_ro.pdf

The movement of traditional bulk commodities is not as time sensitive as consumer goods, food and automotive products. Consequently, having low average speeds on the railway may not have been an issue to customers. Aside from ensuring traditional bulk customers are well looked after it is important to learn the lessons from countries where rail freight is growing, such as Germany, Italy and the UK and selectively apply these to Romania. This includes the shipment of manufactured, consumer and retail goods, often transported in intermodal units such as containers, and these commodities require trains to arrive on time and offer competitive journey times and costs with road freight. Between these two extremes are a range of products where improvements in the rail freight offer could increase the percentage of traffic moved by rail in Romania. These have been identified through comparison with other rail freight markets. In sectors such as Metal Products and Fertilisers, there is scope for rail to exploit its current good position to further enhance its modal share through targeted investment leading to improved journey time reliability and reduced costs for the key corridors where these commodities are moved.

With about 28% of goods moved by rail, Romania is ranked 6th compared with other countries. This proportion was about 10% higher compared with the EU27 average of 18.4% (2011 data was the most recent year for a full countrywide comparison available on Eurostat at the time of writing). This proportion could reduce if roads are improved, hauliers become more efficient or if the rail industry is not modernised. Data for Romania in 2012 showed a significant downward trend in market share of tonne kilometres from 28% to 21% by 2011. The decline of the traditional heavy industries has also contributed to the contraction of the rail freight market including a decline from 72m tonnes in 2008 to 56m tonnes in 2012. This reduction in core business reduces the revenue to the railway network, and has not been replaced by demand from newer emerging markets. Figure 5.9 demonstrates that the volume of goods transported by rail has decreased since 2006, with the market share held by CFR Marfa declining from 75% to 60%. This trend has continued and the state operator had just 50% of the market in 2012.

Figure 5.9: Change in Rail Freight Tonnes (2006-2011)

Source: Eurostat

Service Level

The passenger rail network in Romania is designated as either “interoperable” or non-interoperable”. Trains on interoperable lines are operated either by the state-owned operator CFR Calatori or private operators and maintained by the state-owned infrastructure company CFR SA. To provide context, Figure 5.10 illustrates the extent of the inter-operable and non inter-operable lines. The inter-operable lines account for 85% of the network. In contrast, the non inter-operable routes have been transferred to the private sector, following competitive tendering.

Most single lines are closed for maintenance during the morning (typically between 08.00 and 13.00), or only open 16 hours a day. Closures also apply to some freight terminals and customs facilities. Trains may need to be stabled to await the section being re-opened or be timetabled to avoid such closures. This constraint also applies to passenger services, with gaps of up to seven hours between some trains that call at local stations between Bucharest and Brasov or Craiova. As a consequence these routes fail to offer timetables that are aligned with customer requirements.

Figure 5.10: Inter-operable and Non interoperable Lines

Source: AECOM analysis of National Transport Model data

Network Performance

Network performance has been summarised in terms of the suitability of the timetable and the pattern of services. Figure 5.10 illustrates the median service frequency for passenger services using each corridor, but also highlights significant gaps in the timetable affecting selected routes. Departure times along the busiest sections of the railway network, such as services between Bucharest and Craiova, Constanta, Brasov and Focsani/Suceava, have irregular headways, which results in a complex timetable pattern for passengers. This reduces the convenience of the rail travel. Furthermore, the frequency from smaller local stations is very limited, with a low number of departures at irregular times. For example, there are gaps of 7 hours between Ploiesti to Brasov, Arad to Oradea. The sections between Brasov to Alba Iulia via Sibiu, Caracal to Sibiu via Ramnicu Valcea, plus Suceava to Dej are predominantly single track and this track layout restricts the number of trains operating expecially during the daytime given the maintenance work. Numerous overnight services also compensate for the low number of daytime trains.

Figure 5.11: Summary of the Median Rail Frequencies

Source: AECOM analysis of the CFR Calatori timetable

Rail Speeds

Figure 5.12 compares operating and design speeds on the network, highlighting the parts of the network impacted by a particularly low speed. Much of the network approaching Bucharest has a ratio of between 0.4-0.6, highlighting the relatively slow speeds compared with the theoretical maximum. Some parts of the network have a ratio of above 0.8 but the distribution of these links does not follow a specific pattern, for example, links closest to Bucharest benefiting from the highest speeds. The removal of many intercity services from the timetable and their subsequent replacement with slower InterRegio trains, which typically feature additional intermediate stops, is likely to have contributed to this outcome.

Figure 5.12: Comparison of Average and Design Speeds

Source: AECOM analysis of CFR Calatori timetables and data from CFR Infrastructure

Selected rail journey speeds in Romania have been compared with a sample of European services as shown in Figure 5.12. For example, the average speeds in Romania are about 65-70km/h, yet a comparison with the UK and Germany indicates rail journey speeds in Romania are about twice as slow as the examples. Furthermore, timings in Romania are 40% slower compared with Hungary and Poland. The topography in Romania is a partial factor for some of these slower speeds, for example, the hilly terrain between Bucharest and Cluj, but there are other contributory factors including the numerous intermediate stops, lengthy dwell times and various technical reasons described below.

Figure 5.13: Comparison of Rail Journey Times – Romanian versus European Examples (km/h)

Source: AECOM analysis of CFR Calatori timetables and DB website

There are several factors that contribute to the slow journey speeds described above. These include extended journey times at stations including:

changes from electric to diesel traction;

the requirement to reverse at some stations which is exacerbated by the operation of loco-hauled sets rather than deploying electric or diesel multiple units or push/pull units for longer distance services,

delays when awaiting train paths at single track sections; and

the requirement for periodic inspections of old rolling stock.

The current border control procedures also contribute to lengthy delays. These delays are not conducive to encouraging cross-border trade. This includes border control delays at fellow EU Member States. At the Hungarian border freight train delays are typically up to 30 hours on the Hungarian side and 4–5 hours on the Romanian side. This makes cross-border rail freight very uncompetitive given the much shorter delays affecting road transport. Extended delays at border controls also impact on passenger services, although their duration is significantly shorter than freight.

CFR has recently obtained funding for the installation of overheated axle boxes and brakes detectors, although the introduction of newer rolling stock with better braking systems would also address this issue. Passenger services with numerous intermediate stops also incur significant boarding delays at stations, although the scope to address this issue is limited unless the stopping pattern is revised or stations closed. Figure 5.13 illustrates the total dwell time as a proportion of the overall travel time for selected services and Figure 5.14 presents the average dwell time for all stations. The latter diagram illustrates the locations where the highest average delays occur and confirms the border crossings incur the longest delays.

Figure 5.14: Overall Dwell Time as a Percentage of the Total Travel Time

Source: AECOM analysis of CFR Calatori timetables

Figure 5.15: Average Dwell Time at Railway Stations / Service

Source: AECOM analysis of CFR Calatori data

Only 37% of the current network is electrified which means a number of services that require traction changes and results in extended journey times. One of the main gaps is Cluj Napoca to Oradea, whilst other gaps include Caracal to Craiova, Iasi to Barlad, plus the line serving Botosani, Piatra Neamt, Baia Mare and Satu Mare. There are other gaps in the electrified network which affect the TEN-T network including Craiova to Calafat, Giurgiu to Bucharest and Suceava to Ukraine.

Rail Journey Times

Figure 5.15 illustrates the journey time contours to Bucharest for generalised rail journey time. This dataset represents travel time spent on the train, as well as the impact of the irregular frequencies which results in extended wait times as discussed earlier. Craiova, Pitesti, Buzau and Predeal have a GJT less than 400 minutes to Bucharest. The GJT from Cluj to Bucharest is over 1,000 minutes, whilst the time period from Satu Mare, Baia Mare, Oradea and Arad is over 1,200 minutes. This reflects the slow timings spent on the train and the lengthy wait time for services.

Figure 5.16: Generalised Rail Journey Times to Bucharest

Source: AECOM analysis of Romanian National Transport Model

Passenger Rolling Stock

- There are two key issues affecting rolling stock. Firstly, the deployment of units is generally inefficient, for example the Bucharest - Constanta trains layover for up to five hours between services. This reduces the utilisation of rolling stock and means a larger fleet size is required. Large gaps between passenger services result in the relatively inefficient deployment of rolling stock. There are substantial scope to improve the operating efficiencies of both train sets and the crew, thus creating opportunities to generate additional revenue. Figure 5.16 compares the turnaround times for the Bucharest to Constanta route with other European examples.

Figure 5.17: Comparison of Turnaround Times (minutes)

Source: AECOM analysis of CFR Calatori data

The second issue is the age of the rolling stock fleet. The average age of the units is about 35 years, although some units are over 60 years old, with just 50% of the latter in use. Several other types of unit including loco-diesel electric, loco-diesel, and EMUs have an average age greater than 30 years, yet have an utilisation rate less than 60%. Table 5.4 summarises the information.

Table 5.4: Summary of Passenger Rolling Stock Availability (2013)

| Type | Inventory | Necessary | % in use | ...of which new / refurbished | Average age | Speed range (km/h) |
| --- | --- | --- | --- | --- | --- | --- |
| Loco-electric | 363 | 282 | 78% |  |  |  |
| Loco-diesel electric | 244 | 133 | 55% | 59 | 36 | 100-120 |
| Loco-diesel | 185 | 103 | 56% | 18 | 36 | 100 |
| DMU (Desiro) | 120 | 89 | 74% | 120 | 8 | 120 |
| EMU | 19 | 19 | 100% | 19 | 42 | 120 |
| DMU | 143 | 74 | 52% | 30 | 64 | 70-120 |
| Total | 1066 | 700 | 66% | 295 | 35.4 |  |

Source: Information from CFR Calatori (2003)

The relative age of the rolling stock contributes to the low utilisation. Older fleets are generally less reliable, and require higher maintenance to deliver a timetable ilar service compared with an operation using a more modern fleet. This means a surplus is retained. Old rolling stock is also less attractive to passengers. It is understood that a lack of spare parts and low levels of investment in maintenance have also contributed to this outcome. The utilisation of passenger rolling stock is about 65%, compared with the 85-90% which is normally achieved in other European countries. In the case of the newer, eight year old, Desiro trains less than 80% of the fleet is required to operate the currently scheduled services. It is understood that a programme of wheel modifications is currently underway, and this has reduced the number of units available. Once these revisions have been completed, fleet productivity should increase.

It is notable that the railway network in Romania has experienced a reduction in the total train kilometres of 10% since 2008. Over the same period, there has been a 10% rise in the number of units required to operate the timetable. This is mainly due to the increased rolling stock average age leading to declining maintenance and the additional speed restrictions which have been imposed. Both factors constrain the productivity of the fleet. The high average age of units and low utilisation, explains the requirement for an increased fleet size. The low utilisation rates mean almost 240 extra units are required to operate the current timetable compared with fleet from a West European country.

The use of loco-hauled passenger trains composed of just 3-4 carriages is very cost-inefficient when compared with DMU or EMU operation. Loco-hauled services also lead to inefficiencies at terminal stations with incoming locos are then ‘trapped’. This contributes to the lengthy turnaround times at the head of incoming trains.

Freight Rolling Stock

A high proportion of CFR Marfă’s stock is in poor condition although it is understood from interviews with operators that relatively few units held by private rail operators are obsolete. The average age of a CFR Marfă locomotive was around 33 years and 30 years for wagons. Only 370 out of the 907 locomotives were active in 2012, with just less than 60% of wagons used. Much of the operational fleet is not suited to the emerging markets, including the expanding intermodal sector. There is a large monetary value associated with both the scrap metal of abandoned rolling stock and the track it’s stored on. The rolling stock fleet is shown in Table 5.5.

Table 5.5: Summary of Freight Rolling Stock Availability (2013)

| Rolling stock for CFR Marfa |  | 2007 | 2008 | 2009 | 2010 | 2011 | 2012 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rolling stock CFR Marfă | Locomotives | 991 | 956 | 907 | 907 | 907 | 907 |
|  | Wagons | 50,151 | 42,925 | 41,754 | 39,741 | 39,149 | 38,498 |
| Active rolling stock CFR Marfă % | Locomotives | 44.9% | 40.1% | 26.8% | 24.8% | 42.2% | 40.8% |
|  | Wagons | * | * | * | * | * | 59.9% |

Source: Rail Directorate - MT

Older freight rolling stock will invariably require additional maintenance, be less reliable with more unscheduled maintenance required. Furthermore, the availability of spare parts has been reduced. There is also a shortage of open hopper wagons. A further constraint is the axle weight limit which is just 20.5 tonnes and less than many other EU countries (22.5 tonnes). As a result, payload per wagon is reduced and means a higher number of wagons will be required to move the equivalent quantity of goods. This is problematic for trains crossing borders or using other routes in Romania, which have to use the lowest weight limit in order to remain legal throughout their journey. Although some rehabilitated lines have been upgraded to 22.7 tonnes, a train in Romania that uses non-rehabilitated routes or makes cross border journeys will travel for much of its journey using the lower weight limits and hence becomes more inefficient.

Maintenance and Renewals Regime

The spending required to keep the railway network in a good condition are categorised under two headings: maintenance and renewals. These terms are defined in this section, together with an assessment of the current and forecast spending under these two categories. Maintenance is ongoing expenditure to ensure the safe use of an infrastructure item.  The ‘steady state’ level of maintenance ensures that the infrastructure item continues to be used to the standard it was built, for example ensuring trains are able to travel safely at the design speed of the network.  If maintenance falls below the steady state level, its condition will deteriorate leading to less efficient use and operation, commonly the imposition of speed restrictions, either at specific points or over longer sections of track, or in extreme cases, line closures.

Recent investment in railway infrastructure has not yielded journey time improvements along the main routes. For example, journey times between Bucharest and Constanta remained relatively static between 2001 and 2012 despite significant investment to rehabilitate the line. This calls into question the value of infrastructure investment, and highlights that there are other institutional and organisational barriers that exist which do not allow rail to fulfil its potential. For example in 2001 freight trains were allowed to run at a maximum speed of 120km/hour if the track was suitable, but a safety-related blanket restriction has subsequently been imposed at 80km/hour on all freight on all routes including rehabilitated lines. This is currently being reviewed for the 2013/14 timetable and may, under certain operating circumstances, facilitate increased line speeds. Figure 5.17 illustrates the ‘before’ and ‘after’ situation for a corridor which has benefited from rehabilitation route.

Figure 5.18: Example of Pre and Post Rehabilitated Networks

Recent information from CFR Infrastructure indicates maintenance expenditure was €342.72m (2010), €320.90m (2011), €314.99m (2012) and €324.32m (2013). It is uncertain whether this level of funding will be continued. For example, CFR SA previously produced data which implied that the maintenance budget between 2013 and 2020 would be €958m, an average of €137m per year. Between 2020 and 2030, a maintenance budget of €1,368m is assumed, equating to €137m per year.

All infrastructure items have a defined life expectancy (for example: a bridge may be 60 years) and will then require replacement.  A long-term programme of ongoing renewals of infrastructure items each year is also required in order to ensure that the extent of infrastructure that is life-expired is reduced and eventually removed.  This, combined with the steady state maintenance and repair programme, will ensure that speed restrictions do not start to materialise again and trains operate at the maximum possible speeds. Recent information from CFR Infrastructure indicated the expenditure was €17.55m in 2010, with €9.97m (2011), €8.34m (2012) and €12.36m (2013).

The condition of the infrastructure has deteriorated with the combination of a lengthening backlog of maintenance and renewal, resulting in life expired assets and speed restrictions. Average speeds over the network would decrease. By 2012, about 65% of the track, 80% of the turnouts, 85% of the overhead catenary, 66% of the embankments and 40% of the bridges appear to be life expired and require renewal (“capital repairs”). This reinforces the conclusion that recent renewals expenditure has been completely inadequate. Locally controlled signalling systems are in place across much of the network. This results in high staff and maintenance costs to operate services, regardless of low service frequencies.

The lack of current funding for maintenance has resulted in 1,800 temporary speed restrictions being imposed, which in turn result in slower overall journey times. There were 3.18 million delay minutes in 2012, with speed restrictions accounting for over one-third of this total. These restrictions particularly impact on passenger services. Figure 5.18 presents the annual delay minutes affecting passenger services and the organisation responsible between 2005 and 2010.

Figure 5.19: Summary of Total Annual Delay Minutes and the Contributory Factors – Passenger Services

Source: AECOM analysis of CFR Infrastructure and Calatori data

The delay minutes shown in Figure 5.16 are sub-divided into three categories, infrastructure related, CFR Calatori and other factors. There was some evidence of a reduction in delay minutes during 2011. Whilst there was a 45% drop in delays, there is evidence that the introduction of revised timetables with extended journey times has contributed to this outcome. At present, single track sections are closed during the morning from 08.00 to about 13.00 for maintenance, whilst there is a requirement for single track working on double track routes for similar reasons. This either results in lengthy gaps in services that use single line sections, or longer journey times on other parts of the network. A review of operating practice in Hungary and Poland using several case studies demonstrates that routes do not close for an extended period during the morning for maintenance. Furthermore, these lines are generally served by a higher frequency than five trains per day.

Selected parts of the network are affected by regular and ongoing theft of cables and other equipment, particularly on the Constanta corridor. This has had a significant adverse impact on service reliability. Furthermore, the response from the operator has been to reduce the operating speed and therefore include additional time in the timetable to take account of the repairs to the signalling systems for all services.

Despite this, Figure 5.19 demonstrates that CFR Marfa experiences fewer delays than private operators. Furthermore, the most significant cause of delays associated with the state operator is the action of others. Private operators experience over twice as many delays as the state operator, with the most significant primary cause being operator incurred delays. This appears consistent with the changing market share noted in Figure 5.9 between the state and private operators. The figures also show that delays associated with CFR Marfa are in decline, whilst those associated with private operators have risen between 2010 and 2012.

Figure 5.20: Summary of Total Annual Freight Delay Minutes and the Contributory Factors

Source: CFR Marfa

Figure 5.20 demonstrates the average delays at Constanta compared with other locations in Romania. This indicates delays are much more significant in the area around the port than other parts of the country.

Figure 5.21: Average Minutes Delays per 100 train / kilometres

Source: CFR Marfa

Labour Efficiency

In the last decade there has been a considerable improvement in labour efficiency. However the ratio the ratio of operating costs per employee has risen by around 20% during the last 5 years, whereas the ratio of passenger kilometres and freight tonne kilometres is significantly lower compared with examples in Western Europe.

Staff numbers employed by CFR Calatori have reduced by 20% between 2005 and 2012. The ratio of passenger trips to staff numbers has dropped by 10% between 2009 and 2013, the ratio of passenger kilometres to total staff has dropped by 24%. There is considerable scope to further improve efficiencies if staff numbers were rationalised and alternative methods of train control delivered. A progression towards centralised signalling for the main lines, with radio controlled operations on branch lines, would enable staff to control services over a larger geographic area.

On an average day, there were about 223,000 train kilometres (2012). This comprises 80% of services operated by CFR Calatori, with the remainder operated by private operators. Of the latter, Regiotrans is the largest private operator which provides a wide range of services, mainly on branch lines, though some services are also operated on the other routes. Based on a network covering 10,820 kilometres, and 70.7m train kilometres per annum, this equates to about 20.9 trains per route kilometre each day. Whilst this total is broadly comparable to the statistics for the Hungarian network, it is much lower than the totals for the Netherlands (134 trains/route-km), UK (95 trains/route-km) and France (47 trains/route-km).

In the last decade there has been a significant improvement in labour efficiency affecting CFR SA as shown in Table 5.6. The number of CFR SA employees has almost halved between 2002 and 2011, whereas the size of the network is virtually unchanged. This implies that the ratio of employees per route kilometre which could be used as a measure of labour productivity, demonstrates a reduction from 4.12 employees per route kilometre in 2002 to 2.21 in 2011. This equates to a 46% improvement in labour productivity. There may be further scope to achieve efficiency savings if other changes are introduced, for example, centralised signalling control.

Table 5.6: CFR SA Employees per Route Km

| Year | Number of Employees | Route km | Employees / Route km |
| --- | --- | --- | --- |
| 2002 | 45,337 | 11,002 | 4.12 |
| 2003 | 35,896 | 11,077 | 3.24 |
| 2004 | 33,003 | 11,053 | 2.98 |
| 2005 | 28,006 | 10,948 | 2.56 |
| 2006 | 28,189 | 10,789 | 2.61 |
| 2007 | 27,951 | 10,777 | 2.59 |
| 2008 | 27,610 | 10,785 | 2.56 |
| 2009 | 26,830 | 10,784 | 2.49 |
| 2010 | 24,993 | 10,785 | 2.32 |
| 2011 | 23,839 | 10,777 | 2.21 |

Source: Eurostat, CFR Consolidated Financial Accounts 2002-11

Public Service Contracts

Public Service Contracts are awarded to CFR Calatori, whilst other organisations run rail services which CFR Calatori has recently declined to operate. A number of private bidders are invited to submit proposals to operate trains based on a minimum service specification. These contracts tend to be described in a relatively prescriptive manner. The private sector operators receive a payment based on the train kilometres operated. If services are cancelled, the operator is not paid for running these trains. The Ministry of Transport has recently announced changes to the payments to private sector operators, with 20% of the compensation linked to the total passenger kilometres, and the remaining 80% based on train kilometres. This regime means the operator has relatively limited incentive to introduce more commercially focussed initiatives. Furthermore, the payment rates for passenger kilometres have been significantly reduced, although this impact has been partially offset by a rise in payments for train kilometres. This approach applies to services both on secondary branch lines, and trains using the main lines. Private sector operators pay track access charges to CFR SA if their trains run on the interoperable lines and are compensated for delays but receive no compensation for delays caused by infrastructure or other operator’s defects. On non-interoperable routes, these private companies maintain the track themselves (via a sister company).

The payment of public subsidy is dependent on a specified number of trains being operated. With the closure of some routes during the morning for maintenance requirements, this necessitates a requirement to run some trains overnight to ensure the train mileages are met. This means many services attract very few passengers.

Availability of Redundant Land

The high number of train services previously operated in Romania and ancillary required significant amounts of land for stabling rolling stock and conducting maintenance activities. The reduction in train services and other efficiency savings mean the land requirements are significantly reduced and hence there is considerable scope to re-use this now redundant land for other purposes, for example, office development or retail. This opportunity is particularly relevant in the larger population centres if the redundant parcels of land are located adjacent to the city centre or the main road network since the value of these sites would be greater. This proximity to the major office developments in the city centre or the principal road network for retail activity would improve the attractiveness of the site to be sold for such uses. Similarly, it may also be possible to sell an existing stabling area adjacent to the city centre for commercial activities if an alternative site elsewhere could be identified. It is understood that the ownership of railway land is relatively complex, involving the State, CFR-SA and private owners. Land has been gifted to the State by CFR-SA to help re-pay former debts, but this arrangement means the infrastructure operator has little or no commercial incentive to identify potential schemes.

Safety

The number of fatalities involving passengers or railway employees is relatively high when expressed in terms of numbers per billion passenger kilometres when compared with other EU countries. There was a relatively high number of deaths due to suicides and other incidents at level crossings, with 79 rail-related fatalities (all deaths occurring on railway property) per billion train kilometres between 2007 and 2011.  This is almost four times higher than the EU average, and the 4th highest rate overall, as demonstrated in Figure 5.21.

Figure 5.22: Comparative Number of Fatalities (per billion train kilometres)

Source: European Union information, printed in the Guardian 13 May 2013

Road Competition

The rail journey times have been compared with road to assess the relative competitiveness of each mode to Bucharest. The ratio of results is presented in terms of a contour, with shading illustrating areas where rail is up to 50% slower than car, up to 100% and more than 100% slower. There are very few areas of Romania where the difference in journey time between rail and car is less than 50% as shown in Figure 5.22. Furthermore, large areas of Romania have a rail GJT to Bucharest which is at least double compared with the equivalent by car. The corridor to Craiova from Bucharest, plus parts of Corridor IX towards Suceava, has rail journey times which are only 50% slower than car. The relatively slow parallel road corridors have influenced this outcome, rather than these rail services being notably faster. This analysis also highlights network gaps which necessitate a longer journey by rail, for example, the route from Bucharest to Ramnicu Valcea and Sibiu is currently via Brașov, when the actual crow-fly distance is much shorter. In summary, rail journey times to a number of cities from Bucharest to a number of cities, such as Sibiu, Cluj-Napoca, Oradea, Baia Mare, Satu Mare and Timisoara are significantly higher than road which is the main competing mode. A combination of factors contributes to this outcome. These are:

the numerous intermediate stops which result from rail trying to fulfil multiple functions. These include trying to link the largest towns and cities in Romania with the capital, as well as trains serving smaller catchments which connect to the nearest regional centre;

the extended dwell times at some intermediate stations; and

the inadequate maintenance regime also contribute to the slower rail journey times.

The overall impact is that, for those travellers with a choice, rail is less convenient for accessing job opportunities, visiting friends and relatives or leisure trips including holiday travel.

Figure 5.23: Comparison of Generalised Journey Times by Rail and Road to Bucharest

Source: AECOM analysis of Romanian National Transport Model

Figure 5.23 presents the rail mode share, the total number of daily non-rail trips, the ratio of rail journey time versus car, the average rail speed and the average rail headway for the main corridors. Link flows are also illustrated.

Figure 5.24: Summary of Baseline Multi-modal Data for Selected Corridors

Source: AECOM analysis of Romanian National Transport Model

Strategic objectives

This section describes the overarching themes to link the problems and issues from the baseline to possible solutions, depending on the overall value for money case.

Figure 5.25: Summarises the overarching objectives for the TEN – T Core and Comprehensive networks

Ensuring a sustainable, economically efficient, flexible, environmental friendly, safe and balanced rail network that integrates with other modes and is compatible with TEN–T Core and Comprehensive networks plus other EU strategies and regulations is an overarching objective. The General Objectives will be achieved through the following ways:

Common Objectives setting for all modes;

Specific Objectives setting for each mode.

Five more detailed objectives have also been defined comprising:

Economic Efficiency: a transport system that generates benefits that are greater than its costs;

Equity: the costs and benefits of the transport system should be distributed fairly among citizens, industries and geographic areas;

Safety: the transport infrastructure and services should be provided in a manner that protects people from death and injury;

Integration: the transport system should enable people to travel conveniently and reliably using a combination of different modes of transport, and to minimise the costs of transporting goods;

Environment: the transport system should protect the environment and by so doing should support social and economic development for the benefit of today’s and future generations.

In addition to the strategic objectives identified, a number of operational objectives were also collated. These objectives were informed by the problems identified by the baseline analysis. These operational objectives (OR) were grouped into themes:

OR1: Reform radically the system as to establish a sustainable framework for development;

OR 2: Focus the scarce maintenance and development resources on a reduced sustainable network;

OR 3: Establish a competitive and commercial framework for passengers operations;

OR 4: Establish a sound performance accountability system;

OR 5: Improve the management efficiency of the rail companies and thus the financial sustainability of the system.

As well as the generic operational objectives, the following corridors have also been identified to improve the competitiveness of passenger rail services:

OR 6 Bucharest to Arad/Cluj via Brasov and Teius;

OR 7: Bucharest to Constanta;

OR 8: Bucharest to Arad via Craiova and Timisoara;

OR 9: Bucharest to Iasi via Bacau, and Buzau to Galati;

OR 10: Bucharest to Sibiu via Pitesti and Ramnicu Valcea;

OR 11: Cluj-Napoca to Iasi;

OR 12: Cluj-Napoca to Oradea: including electrification and line doubling;

OR 13: Oradea to Timisoara;

OR 14: Oradea to Baia Mare, and Satu Mare to Cluj-Napoca via Baia Mare.

The selection of the corridors described above reflects current traffic levels using individual corridors in the baseline scenario as well as taking account of the requirements of Regulation no 1315/2013 of the European Parliament on the development of the trans-European transport network. The components of the projects takes account of the mandatory elements of the legislation which include infrastructure enhancements, signalling improvements, electrification, and the operation of high quality rolling stock. If the economic assessment demonstrates that some components of each package are deemed to be uneconomic, exceptions will need to be sought from the Commission.

To address the uncompetitive rail services, the corridors were identified and analysed in terms of rail mode share, total travel market, understanding the components of overall generalised cost including journey time and headway.

Table 5.7 summarises the main problems using supporting evidence, with the operational objectives described above ‘mapped’ to the issues. The proposed interventions and their suitability for testing using the National Transport Model are also shown.

Table 5.7: High Level Objectives and Supporting Evidence for Passenger Rail and Freight

| High level | Problem |  |  |  | Summary of supporting information | Operational objectives |  | Interventions(s) | Testing plan |  |  | Project |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | No. | Theme |  | Description |  | Description | Ref | Description / Alternatives | Suitable for testing | Type of project to be tested | Level of aggregation to be tested |  |  |
| Sustainability | General |  |  | The railway sector in Romania is in crisis. Since 1990 there has been a severe and continuous decline in all the key performance indicators. Unless the system is radically reformed the investments in infrastructure upgrade have no actual benefits. There is a complex of problems behind the crisis, which are grouped under the following sub-themes. | See Existing Conditions Report and Problem Definition Report. | Reform radically the system as to establish a sustainable framework for development. | OR 01 | A comprehensive reform package to be implemented as soon as possible (its key components being presented below). To this end a Rail Reform Agency should be established by June 2015 under the authority of MoT to implement the radical reforms required and then evolve in a permanent rail public transport authority. The Rail Reform Agency should seek top international expertise support via TA and also possibly by an IFI advisory task force. | No | No | N/A | DS12A |  |
| Sustainability | (1) |  | Infrastructure sustainability | The current network size faces a substantial and growing disparity against both the demand and the financial resources available to maintain and operate it. | - 90% of the traffic is carried by some 54% of the routes (63% of track-km). - The resources actually spent annually for maintenance and renewals are less than 40% compared to what would be needed under a normal regime for the size of the network in operation and less than 20% as compared to what is actually required considering also the renewal backlog. - Whilst traffic volumes reduced by 93% since 1990 the network size reduced has only by 5% over the same period. - The process to reduce the network in Romania is actually ineffective as even after a line is cut from the “interoperable” network and classified as “non-interoperable”, it does not stop it being financed from public funds. | Focus the scarce maintenance and development resources on a reduced sustainable network. | OR 02 | 1. The classification into “interoperable” and “non-interoperable” lines should be replaced (by modifying the relevant legislation) with a clearer structure based on: - A primary network defined as being of national public interest based on clear routes including both infrastructure and passengers operations. If a route is defined as being part of the public service it shall be subsidised/compensated both for infrastructure and (where relevant for passengers services) for operations. - A secondary network which is not of national public interest (which could however be of local interest and thus taken over partially or entirely and subsidised e.g. by county or municipal authorities). Once excluded from the national public service no state funding will be possible and in cases where the line cannot be transferred either to local authorities or to private operators it will be automatically closed and could not return to CFR Infrastructure ownership. 2. The primary network should be limited to approx. 60% of the current track-km size (as per indicative map annexed in the Rail Note) since this would capture about 95% of current passenger and freight demand. However this should be fine-tuned within a detailed closure programme (to be supported by a dedicated traffic analysis including more detailed market and financial sustainability analysis). 3. Increased budget spending from approximately 350 M€/year to some 500 M€/year for the regular maintenance and renewals regime of the primary network. On this basis a rolling renewal programme covering 200-250 track-km every year shall be initiated. This should be ensured through a long-term Government commitment. | Yes | Proposed network following identification of optimal routes | Revised network | DS13A |
| Sustainability | (2) |  | Passengers Operations | The passenger services rail business is uncompetitive and largely uncommercial. The services are generally not profitable, therefore services exist only through subsidies from the Ministry of Transport under Public Service Contracts (PSC). The PSCs are not competitively tendered but directly awarded to the operators and CFR Calatori is awarded approx. 85% of the services (2011). | The lack of competitive environment combined with a lack of business/market orientation, which is not encouraged by the terms of the PSC, and the lack of performance accountability, has led to service levels even lower than the condition of the infrastructure allows. This is evidenced by: - Poor network availability & irregular timetables with up to seven hour gaps between services during daytime. - Significantly longer travel times generated by high delay times at stations which can account for up to 50% of the overall travel time. - Poor quality of rolling stock: 87% of the locomotives are older than 20 years and 82% of the coaches are older than 25 years. The in-house maintenance approach (employing some 7,000 staff) appears to be very inefficient; for example 40% of the new Desiro DMUs fleet went out of service in less than 10 years. - Poor utilisation of available rolling stock with turnaround times of up to 5 hours. - High train operating costs generated by (i) energy inefficient old locomotives and (ii) the poor productivity of the rolling stock. - Relatively high fares - as compared with prices in France, Germany, Italy or Spain especially for advance tickets and substantially higher than of the competing local bus services. - Poor services in stations – even the large stations rehabilitated/modernised under various financing programmes are passengers unfriendly, commercially unattractive and poorly maintained. | Establish a competitive and commercial framework for passengers operations | OR 03 | 1. Define clear level of service requirements - limited to the primary network above (including measures to ensure an attractive frequency, performance indicators, etc.) and tender the lines and services openly on this basis the Public Service Contracts (for example by lines, service groups or regions, possibly including the operation of the stations on the respective lines). Review the scope for in-fill electrification for selected routes. There is potential for the proposed new enhanced service specification to be operated using fewer rolling stock units compared with the current timetable which assumes improved rolling stock utilisation. The target for tendering the PSC is 2016, in view of starting the new services in 2017. The process should be managed by the Rail Reform Agency. 2. Procure a set of high quality rolling stock (e.g. mostly EMUs, including possibly tilting sets) to be made available to the operators as part of the tender for the public service contracts. Some new diesel units may be required depending on the extension to the electrified network. Procurement to be possibly structured to include a long-term maintenance contract to avoid the experience following the limited availability of the Desiro DMUs fleet due to the reliability issues. 3. In order to ensure a level playing field for all bidders, applying for the new services a rolling stock leasing company (ROSCO) should be established under the ministry (or alternatively the Rail Reform Authority could undertake this role directly. This would procure the new rolling stock and take over the old assets after the current contract with CFR Calatori has expired. For example, in 2016 from CFR Calatori). This would ensure an open tender system was introduced for subsequent opportunities. | Partially | Regular interval timetable, introduction of new rolling stock tested using the model. Impact of a revised ticket structure will not be testing using the model | Rolling stock / timetable interventions will be tested for individual corridors | DS14A |
| Sustainability | (3) |  | System regulation & organisation | The regulatory, organisation and management systems currently in place fail to ensure any of the following fundamentals: - Accountability for performance in the system. - Efficient allocation of the scarce public resources. - Competitive environment for the passengers services, - Efficient and commercially oriented management of the state-owned companies. | See Existing Conditions Report and Problem Definition Report. | Establish a sound performance accountability system | OR 04 | The relations between the public transport authority, CFR Infrastructure and the operators shall be restructured by: - Defining robust performance indicators for the public service parameters which would be automatically linked with the payment terms such as penalties from compensation for delays compared to the travel times defined in the public service contract. - Extending the same system to the regime between operators and infrastructure. This would automatically link the payment terms including penalties from track access charges for delays generated by infrastructure compared to the travel times defined in the Public Service Contract. - Force majeure circumstances to be clearly defined and limited, and regulatory authority to effectively supervise the application of the system. | No | N/A | N/A | DS15A |
|  | (4) |  | Management efficiency | The management of the public rail companies is inefficient. | See Ministry of Transport Strategic Plan prepared with World Bank support, 2013 | Improve the management efficiency of the rail companies and thus the financial sustainability of the system | OR 05 | Re-launch private management techniques with a proper selection process of an efficient business management profile, ideally with experience in reforming similar rail companies elsewhere. 2. Audit of assets, operations and costs of CFR Infrastructure and CFR Calatori. 3. Cost-cutting programme to be developed in particular which would include the sale of: - Non-core business and assets - Excess of sidings - High number of stations (e.g. 533 stations) with less than 10 passengers boarding per day | No | N/A | N/A | DS16A |
| Economic efficiency | (5) |  | Poor infrastructure and train services | Uncompetitive passenger services between the main cities of Romania because of low commercial speeds and poor frequencies which have resulted in a low rail market share below its potential | Average commercial speeds between 50-60 km/h (which is typically between 40-60% of the initial design speed and has resulted from a lack of appropriate maintenance & renewals). Average headway of 3-4 hours on many important city to city routes including flows from Bucharest. Generalised costs are 50-100% higher as compared to the competing road transport routes. The rail passenger market share is 2% on the Bucharest to Sibiu and compares with 37% on the better serviced lines (Bucharest-Craiova). | Improve the competitiveness of passenger rail services on the route between Bucharest and Hungary via Brasov, Teius / Cluj | OR 06 | Enhance service frequency with a regular interval timetable between Cluj and Bucharest (0.5tph) plus 0.5tph from Bucharest to Deva and 1tph Bucharest to Brasov. Raise the speed of the line to the design parameters (Fiches 038 and 057) Examine the scope to improve the efficiency of border crossings to reduce delays | Yes | Combination of timetable, rolling stock & infrastructure enhancements | Individual corridor | DS01A, DS01B |
|  |  |  |  |  |  | Improve the competitiveness of passengers rail services on the route between Bucharest and Constanta | OR 07 | Enhance service frequency with 2tph | Yes | Combination of timetable, rolling stock & infrastructure enhancements | Individual corridor | DS02A |
|  |  |  |  |  |  | Improve the competitiveness of passenger rail services on the route between Bucharest and Arad via Craiova and Timisoara | OR08 | Enhance service frequency with a regular interval timetable operating every 2 hours to Arad via Timisoara from Bucharest, 0.5tph from Bucharest to Craiova (1tph) and 0.5tph to Simeria via Craiova and Targu Jiu. Rehabilitation to enhance design speed (Fiche 045) | Yes | Combination of timetable, rolling stock & infrastructure enhancements | Individual corridor | DS03A, DS03B |
|  |  |  |  |  |  | Improve the competitiveness of passenger rail services on the route between Bucharest and Iasi / Suceava / Galati | OR 09 | Enhance service frequency with a regular interval timetable operating every 2 hours to Marasesti, Suceava, Galati, and Iasi via Bacau Rehabilitation to design speed Examine the scope to improve the efficiency of border crossings to reduce delays | Yes | Combination of timetable, rolling stock & infrastructure enhancements | Individual corridor | DS04A, |
|  |  |  |  |  |  | Improve the competitiveness of passenger rail services on the route between Bucharest and Sibiu via Pitesti and Ramnicu Valcea | OR10 | Enhance service frequency with a regular interval timetable operating every hour from Bucharest to Pitesti with alternate trains extended to Sibiu via a new link to Ramnicu Valcea | Yes | Combination of timetable, rolling stock & infrastructure enhancements including electrification | Individual corridor | DS05A |
|  |  |  |  |  |  | Improve the competitiveness of passenger rail services on the route between Cluj Napoca and Iasi | OR11 | Enhance service frequency with a regular interval timetable operating every 2 hours from Cluj to Iasi Rehabilitation to design speed (Fiches 056 and 057) | Yes | Combination of timetable, rolling stock & infrastructure enhancements | Individual corridor | DS06A, DS06B |
|  |  |  |  |  |  | Improve the competitiveness of passenger rail services on the route between Cluj Napoca and Oradea | OR12 | Enhance service frequency with a regular interval timetable operating every hour from Cluj to Oradea Rehabilitation to design speed (Fiche 067) | Yes | Combination of timetable, rolling stock & infrastructure enhancements including electrification | Individual corridor | DS07A, DS07B, DS07C |
|  |  |  |  |  |  | Improve the competitiveness of passenger rail services on the route between Oradea and Timisoara | OR13 | Enhance service frequency with a regular interval timetable operating every 2 hours from Timisoara to Oradea | Yes | Combination of timetable, rolling stock & infrastructure enhancements including electrification | Individual corridor | DS08A |
|  |  |  |  |  |  | Improve the competitiveness of passenger rail services on the route between Oradea and Cluj Napoca via Baia Mare and Satu Mare | OR14 | Enhanced service frequency with a regular interval timetable operating every 2 hours between Baia Mare and Oradea, and 2 hourly from Satu Mare to Cluj Napoca | Yes | Combination of timetable, rolling stock & infrastructure enhancements including electrification | Individual corridor | DS09A |
|  |  |  |  |  |  | Improve the competitiveness of passenger rail services on the route between Bucharest and Giurgiu | OR15 | Enhanced service frequency with a regular interval timetable operating every 2 hours between Bucharest and Giurgiu | Yes | Combination of timetable& infrastructure enhancements including electrification | Individual corridor | DS10A, DS10B |
|  |  |  |  |  |  | Improve the competitiveness of passenger rail services on the route between Craiova and Calafat | OR16 | Enhanced service frequency with a regular interval timetable operating every 2 hours between Craiova and Calafat | Yes | Combination of timetable& infrastructure enhancements including electrification | Individual corridor | DS11A |
|  |  |  |  | Low Average Commercial Speed of Freight Trains | A safety-related blanket restriction has been imposed at 80km/hour on all freight on all routes including rehabilitated lines. The current average commercial speed is just 21/22kph. Journey times are about 28 hours to go 400kms. All passenger trains (including extremely slow local services) are currently given priority over even high-value freight services | Increase rail speeds to a level consistent with target design speeds | OR1 | CFR Infrastructure to allow a higher top speed for certain freight trains on rehabilitated lines - 120kph instead of 80kph such as intermodal block trains. | No | N/A | N/A | R31 |
|  |  |  |  |  |  | Enhance the asset performance of the railway especially opportunities to achieve operational efficiencies | OR13 | CFR Infrastructure to enable better train control by utilising a 2 Tier system to allow properly equipped and dedicated intermodal trains to be prioritised over other types of bulk freight and potentially local, stopping passenger services. This would reduce journey times over the core network to enable freight to compete more effectively with road. | No | N/A | N/A | R33 |
|  |  |  |  |  |  |  |  | Establish a contract between infrastructure provider and train operators which requires compensation for delays. | No | N/A | N/A | R36 |
| Economic efficiency | (6) |  | Management and Operations | Old systems and paperwork based ways of working coupled with a lack of training are hindering the development of a modern railway | Train sheets are filled in manually with every wagon number recorded in handwriting and then where each wagon needed to be moved to. There are some modern systems (e.g. ARGOS) but only limited numbers of trained operators. | Enhance the asset performance of the railway especially opportunities to achieve operational efficiencies | OR13 | Ensure companies adopt modern best practice and technologies such as the use of GPS for rolling stock tracking. Modern training courses in rail operations, systems and technology should be phased in for both existing and new staff. It is important for the rail sector to have a good blend of experience with technology. | No | N/A | N/A | R27 |
| Economic efficiency | (7) |  | Infrastructure Deficit | Old freight terminals are inefficient and/or poorly located particularly intermodal sites | Reducing handling times per tonne and other improved efficiencies could be achieved through containerisation and encourage modal shift. | Develop a network of “open user” freight terminals to serve Romania’s highest demand potential regions, cities and major EU markets in Bulgaria and Hungary. See Supporting Data for full description of terminal requirements and location determinants. | OR16 | Close Bucarestii Noi freight terminal and create a new intermodal terminal with extra capacity in Bucharest, with possibilities for tri-modal capabilities in the future. | Yes | Intermodal Terminals | Bucharest Only | R42 |
|  |  |  |  |  | Bucharest’s terminals are small and do not have space to accommodate planned growth |  |  |  |  |  |  |  |
|  |  |  |  |  | Numerous terminals have been closed despite proximity to large numbers of potential customers and industries (e.g. Craoiva), or are poorly sited for current and predicted freight flows (e.g. Iasi). |  | OR13 | Rehabilitate the intermodal terminal serving Iasi at Socola serving this city that currently does not have its own facility and also to provide transhipment potential with Russian gauge railways. | Yes | Intermodal Terminals | Iasi Only | R38 |
|  |  |  |  |  |  |  |  | Safeguard the mothballed Semenic Terminal at Timisoara serving this city that currently does not have its own facility | Yes | Intermodal Terminals | Timisoara Only | R41 |
|  |  |  |  |  |  |  |  | Build a new intermodal terminal at Craiova, serving this city which offers an important industrial location currently without open user facilities. | Yes | Intermodal Terminals | Craiova Only | R43 |
|  |  |  |  |  |  |  |  | Rehabilitate existing terminals to provide a comprehensive and strategic network for international and domestic intermodal freight at: Cluj Napoca Est, Turdu, Suceava & Bacau. | Yes | Intermodal Terminals | Network-wide | R44 |
| Economic efficiency | (8) |  | Infrastructure Deficit | The majority of the Romanian rail network has a low maximum axle weight of 20.5 tonnes | A 20.5 tonne axle limit is currently in force, but this is less than the EU rail network standard of 22.5t. For a 30 wagon train this equates to an additional 240 tonnes of commodity (a 15% productivity enhancement). | Enhance the asset performance of the railway especially opportunities to achieve operational efficiencies | OR13 | To encourage international and transit traffic allow 22.5 tonne axle weights on rehabilitated routes. Clearly the whole route needs to be cleared for operators to use this productively. This feature should be integrated into all future route rehabilitation. | No | N/A | N/A | R35 |
|  |  |  |  | Old network infrastructure not taking advantage of modern benefits and opportunities, such as in energy efficiency | A substantial amount of the rail infrastructure in Romania is life-expired or does not enable modern technology to operate fully. There is currently no section of track, for example, where power recovery is possible. |  |  | Undertake a feasibility study with regard to introducing power systems with the capability to return energy generated from regenerative braking to the power network during all future rehabilitation work. Regenerative braking can save 5% on power use on freight but up to 17% on commuter passenger trains | No | N/A | N/A | R34 |
| Economic efficiency | (9) |  | Management and Operations | CFR Marfă’s delayed privatisation | The continued uncertainty and delays regarding CFR Marfă’s privatisation mean that rail freight is locked into a cycle of under-investment and short-term contracts. | Increase revenue capture and efficiency by ensuring contracts are more commercially focussed | OR7 | Remove uncertainty over the future of CFR Marfa and privatise as soon as possible. This would give the whole rail industry the opportunity to compete more effectively in the freight sector | No | N/A | N/A | R32 |

Proposed Improvements

Using the information collated from the Problems and Issues report, the interventions to resolve these issues have been grouped into five main themes, as shown in Table 5.8.

Table 5.8: Summary of the Potential Improvements

| Theme | Proposed improvements |
| --- | --- |
| Theme 1: General | - Creation of a Rail Reform Agency to oversee delivery of improvements |
| Theme 2: Infrastructure sustainability | - Increase maintenance and renewals budget - Steady state maintenance of the core network - Repairs and renewals programmes - Line rehabilitation to design speed - Line rehabilitation to enhanced speed - Define a primary network to be retained which comprises nationally important routes and would serve the highest percentage of traffic given the likely funding available for maintenance and renewals - Identify the secondary routes which could be managed by local authorities - Undertake overnight maintenance and repairs rather than completing these works during normal working hours |
| Theme 3: Management and Operations | - Identify rolling stock strategy including the requirement for new units - Establish a rolling stock leasing company to manage the deployment of units - Yield management - Fare evasion strategy |
| Theme 4: System regulation and organisation | - Introduce a new transparent performance regime for compensation between infrastructure and service providers |
| Theme 5: Management efficiency | - Identify a programme of station closures which reflects the low footfall - Introduce new signalling systems to modernise train control enabling staff numbers to be further rationalised - Sell redundant railway assets and other facilities to generate revenue |
| Theme 6: Poor infrastructure and unattractive passenger services | - Enhance frequencies on the core rail corridors to improve the competitiveness of rail - Expand the network of electrified lines - New track infrastructure – doubling or new lines |
| Theme 7: Poor infrastructure leading to unattractive freight | - Introduce a higher permitted top speed to enhance the competitiveness of rail freight versus other modes |
| Theme 8: Inadequate freight management and operations | - Apply new technology and systems to the rail freight sector to improve efficiency |
| Theme 9: Infrastructure deficit affecting freight services | - Implement new modern inter-modal freight terminals at strategic locations (described in Chapter 8) - Increase maximum axle weight to 22.5 tonnes - Procure new freight locomotives with functionality for regenerative braking |

Description of the Proposals

Theme 1: Creation of a new Rail Reform Agency

Problem: Fundamental revisions are needed to the governance regime. Consequently, the payments to operators for the Public Service Contracts (PSC) are mainly linked to the number of trains run, rather than the timing of these trains or the actual patronage attracted. The PSC mechanism to compensate train operators is also fairly simplistic, with 80% of compensation paid to CFR Calatori depending on the number of train kilometres operated, with a further 20% based on passenger revenues. As a result, this mechanism offers little or no financial incentive to maximise efficiency, reduce delays or improve performance;

Proposed Solutions: A fundamental revision to the PSC is therefore required and is dependent on the delivery of other interventions described above. This includes delivery of a more intensive operating timetable with 85-90% of units deployed rather than the current 65%. This will deliver substantial savings in future rolling stock requirements for the current timetable and would affect the number of replacement units required. In addition to the rolling stock efficiencies, there is also scope to reduce staff costs by allocating train crew in a more efficient way. Other measures include the implementation of an alternative maintenance regime which enables services to operate throughout the morning, rather than leaving the current gap of up to seven hours. The revised approach  will help to attract new passengers and boost revenues. Various commercial initiatives including more effective yield management, introduction of more competitive rail services plus strategies to target fare evasion.

The introduction of a revenue share framework may also be advantageous to provide further incentives for the operator to achieve future growth. These changes will need to be introduced gradually to avoid the operator being affected by financial problems if the timetable revisions are not introduced quickly enough or passengers do not respond in a timely manner. Once the system has been introduced, useful background evidence will illustrate the scope for revenue growth and / or cost savings which will enable the future commercial targets to be tailored accordingly.

To ensure the transition between the current compensation regime and a future framework which includes a greater commercial focus is managed effectively, the feasibility of operating passenger rail services as a concession rather than a franchise for an initial period must be examined. Instead of a new operator taking the revenue risk for services that will initially undergo substantial change, the feasibility of the Romanian Government taking revenue risk in the short term during the transitional period is recommended. This would enable the financial impacts of taking revenue risk during this period to be minimised. As part of the concession, the operator should still be incentivised to deliver wider changes and contribute to achieving revenue growth. Once the transitional period is completed, an alternative contractual regime could then be introduced based on a franchise with the operator taking revenue risk.

The proposed Rail Reform Agency would be established to implement many of these changes described above. Its role would vary over time, with a remit to tackle specific tasks in a prioritised order to ensure the wider programme of works is completed in a timely manner. This would help to improve the overall financial position of the operator and the infrastructure provider.

The RRA would be responsible for:

recruiting staff and defining roles;

managing the transition between the current system and the implementation of different initiatives;

managing the process to identify the primary network and then implement the programme of network closures;

defining the geographic areas for concessions initially and then the transition towards franchises in the longer term;

helping CFR Infrastructure draw up a Plan according to EU 34/2012 and obtain agreement with the Ministry of Transport for finance and preparing market responsive contracts between the operator and infrastructure manager to ensure efficient services.

Theme 2: Infrastructure Sustainability

Rail infrastructure in Romania has experienced a steady deterioration over the past ten years with an increasing maintenance and renewals backlog. Furthermore, a greater number of assets including track, signals and overhead electrification have become life expired. This has resulted in speed restrictions being enforced, leading to a substantial increase in passenger travel times. Although significant funds have been invested to upgrade the rail network on selected corridors including Bucharest to Constanta and Brasov, these improvements have had limited impact on the journey times thus far. Travel times after rehabilitation on the Bucharest to Constanta line are currently 9 minutes longer than before rehabilitation or about 40% slower than the target time of 2 hours. On other routes, journey times have deteriorated at even faster rate since 2000. Consequently, there is a clear need to identify realistic options to deliver a financially sustainable network that can properly serve the core rail markets to enable this to secure a long-term competitive advantage.

Theme 2: Increase Maintenance and Renewals Budget

There are two key issues to highlight. Firstly, maintaining the current size of the rail network to the required standard is unaffordable. Secondly, significant parts of the current network carry very low volumes of passenger or freight traffic which absorb resources that could potentially be better deployed elsewhere. With low passenger and freight usage per track-km using selected parts of the network, as well as the limited availability of funds, an approach which focuses investment towards key corridors that serve a national public interest and carries most of the traffic is therefore required. This would be achieved by focussing on the definition of a primary network which covered a reduced size in terms of the track kilometres. The existing designation of interoperable and non-interoperable lines would instead be replaced by primary routes that fulfil a national interest. Secondary routes which serve a local function could be retained and managed by a local authority albeit with no state funding. If a secondary line was not transferred from CFR Infrastructure’s ownership, it would be automatically closed. The size of the primary network should be dependent on passenger demand and financial resources available.

Theme 2: Repairs and Renewals Programme

With a substantial shortfall in expenditure for renewals and repairs compared with the required levels, and low traffic densities using an under-utilised network, there are a number of possible measures which would produce a more sustainable situation, and cut the funding requirement.  These include:

rationalising the network so the funding requirement for maintenance and renewals is smaller;

introduction of various commercial initiatives to increase revenue; and

Increased funding.

Other initiatives would include the introduction of more competitive passenger services, greater accountability and an improvement programme to deliver better efficiencies.  An exercise to prioritise those links which generated the highest revenue (both from passenger farebox and freight track access charges) compared with the costs (train operating costs, and maintenance and renewals costs) was completed to generate cost to revenue ratios (CRR). The operable and non-interoperable network was treated in the same manner to ensure a sustainable network was identified regardless of ownership or the alternative funding mechanisms which are currently adopted. The main trends include:

The routes generating the highest CRR include the Bucharest to Timisoara via Craiova and Caransebes, plus the branch line used by coal trains north west of Strehaia. Many of these routes have a CRR above 0.6;

Bucharest to Constanta has a range of 0.4 which reflects the mixture of freight and passenger services using this corridor;

Whilst Bucharest to Suceava via Ploiesti and Ploiesti to Brasov lines have CRR of about 0.4, there are numerous branch lines with a CRR less than 0.1. The majority of these lines are single track and not electrified. Although this contributes to lower maintenance requirements, the lack of passengers and freight trains is the main factor.

Theme 2: Defining a Primary and Secondary Network

Large parts of the rail network in Romania generates relatively little revenue, either through fare-box income or freight track access costs, relative to their operating and maintenance costs. Different funding scenarios were examined to understand the trade-offs between the percentage of passenger and freight traffic that would be retained by the alternative proposed networks and the funding implications. This assessment highlighted the retention of about 55% of the 18,973km network would enable about 99% of the traffic to continue. The estimated 18,973 track kilometres includes redundant station sidings and running lines, so the impact on the actual operational network would be smaller. Using unit rates per track kilometre for maintenance and renewals which are comparable to other parts of the EU, the budget required would equate to €287m and €245m for renewals per annum respectively, plus a further €518m each year for a 15 year period to address the current backlog. The costs assumed for line rehabilitation are based on average unit rates to represent single or double track, electric or diesel traction, but there is no account for the specific geographic characteristics of individual lines for example, routes located in challenging topographical areas.

The introduction of some commercial initiatives including more effective revenue protection strategies might boost revenues by a further €45m per annum. Figure 5.25 presents the coverage of the network that would be retained and includes a small number of links that would also be retained for strategic connectivity. These add-ons include sections near Timisoara and border crossing north of Suceava towards Ukraine. Some other links may offer a strategic importance in terms of carrying coal or oil to a power station or raw materials to a steelworks. It should be acknowledged that the use of average costs and revenues may bias the results against privately-operated lines, so it is recommended that some marginal links which have initially been identified for closure will need to be re-examined before decisions are finalised.

Figure 5.26: Definition of the Primary Network

Source: AECOM proposal

Theme 2: Alternative Maintenance Regime

In addition to the funding required to maintain the network, the timing of these works also needs to be revised. The completion of works in the morning is inconvenient for passengers travelling at that time, since trains either do not operate, or are affected by slower journey times. Instead of completing maintenance works between 08.00 and 13.00 it is proposed these works are rescheduled to be completed overnight. Although this change could affect overnight freight services with a requirement to identify necessary mitigation measures, it would facilitate the introduction of a more intensive passenger service in the morning by removing the lengthy timetable gaps which currently reduces the attractiveness of rail. Whilst this revised strategy would result in higher labour costs and a requirement for additional plant equipment to facilitate overnight works, CFR Infrastructure could recoup these costs from higher track access charges levied to the operator. In turn, CFR Calatori would then be able to attract a higher number of passengers and the increased revenues would help to offset the increased costs.

Table 5.9 summarises the potential impacts if the maintenance backlog was removed. This can be measured in terms of the current average speed and the results if the design speed was increased. These results are presented for the main rail corridors. The average increase in speed between the current versus the potential scenario is about 33%, although the speeds between Arad and Timisoara would more than double if the line speed was increased.

Table 5.9: Summary of the current and average design speed for selected lines

| Corridor(s) | Movement | Average Speed (km/h) | Average speed with design speed - fast service (km/h)(1) | % improvement |
| --- | --- | --- | --- | --- |
| IV-S/900 | Bucharest - Timisoara | 61 | 70 | 15 |
| IV-S/900 | Bucharest - Craiova | 69 | 80 | 16 |
| IV-S | Craiova - Timisoara | 55 | 65 | 18 |
| IV-S | Timisoara - Arad | 45(2) | 92 | 104 |
| IV-N | Bucharest - Constanta | 77 | 113 | 47 |
| IV-N | Bucharest - Brasov | 60 | 87 | 45 |
| IV-N | Bucharest - Arad | 59 | 79 | 34 |
| IV-N / 300 | Bucharest – Cluj-Napoca | 57 | 73 | 28 |
| IV-N / 300 | Bucharest - Oradea | 56 | 69 | 23 |
| IX | Bucharest - Bacau | 63 | 83 | 32 |
| IX | Bucharest - Iasi | 59 | 82 | 39 |
| IX / 700 | Bucharest - Galati | 55 | 79 | 44 |
| IX | Bucharest - Ploiesti | 77 | 98 | 27 |
| Other TEN-T / IV-N | Cluj-Napoca - Timisoara | 55 | 68 | 24 |
| Other TEN-T / IX | Cluj-Napoca - Iasi | 55 | 62 | 13 |
| Other TEN-T | Cluj-Napoca - Oradea | 49 | 56 | 14 |
| Other TEN-T | Bucharest - Sibiu | 54 | 72 | 33 |
| 1 Interregio and Intercity services 2 Includes Regio Services |  |  |  |  |

To support the increased line speeds, a programme of upgrades to the existing level crossings to full barrier operation as part of the line rehabilitation. This initiative, along with other safety related proposals, will help to address the relatively poor safety record in Romania which occurs leading to a high fatality rate per billion passenger kilometres compared with other European countries.

Theme 3: Passenger Operations

Rolling Stock Fleet

Problem: Only 65% of the current fleet is required to operate the service pattern in Romania. This proportion is lower than other European countries (typically 85-90%). The high rolling stock age contributes to poor reliability and necessitates a larger allowance for spare units to cover breakdowns. A combination of inefficient rolling stock scheduling and a requirement for a higher number of fleet to reflect the poor reliability of old units have contributed to this outcome;

Proposed solution: A restructured, more intensive timetable that features modern rolling stock will deliver a significantly improved service ‘offer’ for passengers and will help to strengthen the case for other investment. During the lifetime of the Master plan, all rolling stock will need to be replaced with the exception of the Desiro units. This provides an opportunity to replace some of the inefficient loco-hauled sets with electric or diesel multiple units. Instead of replacing loco-hauled trains on a like-for-like basis, the EMUs or DMUs have lower operating costs and would enable the capacities to be more closely aligned with forecast demand. A more detailed description of the rolling stock strategy is set out below.

Rolling Stock Strategy

Using the ratio of train kilometres per day, the average speed of trains and the actual number of units deployed by Northern Rail in the UK, the revised number of trains required to operate the current timetable in Romania has been calculated. Even if an allowance is included to reflect less efficient diagramming of services compared with Northern Rail, an estimated 680 units would be required to operate the current service, with about 580 in daily service.

In addition, the number of units required to operate the regular InterRegio interval timetable has been calculated. Table 5.10 summarises the fleet requirements by service proposal, and indicates the type of rolling stock that could be introduced to best meet the journey patterns of passengers. A total of 104 units (or nearly 550 carriages) would be required to operate this timetable.

Table 5.10: Requirement for Rolling Stock Requirement

| Scheme | Scheme | Rolling stock requirement | Traction | No. of carriages |
| --- | --- | --- | --- | --- |
| Bucharest to Arad / Cluj via Brasov | DS01 | 22 | Electric | 5 |
| Bucharest to Constanta | DS02 | 4 | Electric | 4 |
| Bucharest to Arad via Craiova | DS03 | 8-7-4 | Electric | 3-4-9 |
| Bucharest to Galati / Iasi / Suceava | DS04 | 2-23 | Electric | 3-5 |
| Bucharest to Sibiu via Ramnicu Valcea | DS05 | 7 | Electric or diesel | 4 |
| Cluj Napoca to Iasi | DS06 | 7 | Electric | 4 |
| Cluj to Oradea | DS07 | 4 | Electric or diesel | 2/3 |
| Timisoara to Oradea | DS08 | 5 | Electric or diesel | 4 |
| Oradea to Cluj via Baia Mare | DS09 | 6 | Diesel | 3 |
| Bucharest to Giurgiu | DS10 | 2 | Electric | 5 |
| Craiova to Calafat | DS11 | 3 | Electric | 3 |
| Combined scenario | DS99 | 104 | Electric and diesel | 2 to 9 |

Source: AECOM calculation

The initial demand forecasts and the proposed service patterns described above were used to inform the selection of rolling stock, in terms of the the mixture of electric or diesel multiple units. With the main service improvements focussed on InterRegio trains, this reinforces that the new rolling stock will need to be tailored to the long distance travel market. A phased rolling stock programme to progressively replace older rolling stock units by modern EMUs and DMUs will return significant benefits to rail passengers whilst enabling higher flexibility and lower delays and operating costs per train-km.

Furthermore, there may be potential to introduce rolling stock with the capability to tilt. This type of unit may enable some journey time savings to be achieved, albeit with higher purchase costs. A separate feasibility study is needed to determine whether there is a business case for higher specification rolling stock once the possible line speed improvements have been evaluated. A generic cost per coach for new rolling stock has been used in the scheme appraisal.  References to ‘electric’ or ‘diesel’ in Table 5.14 also take account of the potential opportunities to introduce alternative traction, depending on the outcome of the cost-benefit analysis for electrification. The number of carriages is dependent on the estimated demand, although these assumptions will require further review in advance of procuring the new sets.

Yield Management

Problem: The existing fare structure for rail travel in Romania is relatively inflexible with limited financial incentives to encourage passengers to book in advance to secure cheaper tickets. Most fares are distance based, although some tickets can be purchased in advance using the CFR Calatori website. However, there is considerable scope for further improvements. With advance tickets in Romania between 75% and 100% more expensive than other European countries for similar distances when adjusted to take account of wage differences, this merits further review;

Proposed solutions: This could be addressed by the introduction of measures to maximise yields on the most popular trains. In parallel, greater discounts for passengers able to travel at less popular times could be introduced. Season tickets and discounts for frequent travellers could also be introduced and these measures would attract more passengers to rail. The opportunities to revise the timetable highlighted earlier further reinforce the importance of introducing more effective yield management. These timetable changes would deliver additional capacity on selected routes, particularly for journeys lasting 2-3 hours to / from Bucharest.

Fare Evasion

Problem: Discussions with CFR Calatori indicate that 25-30% of passengers are either travelling with the incorrect ticket or do not have a ticket at all. The estimated revenue loss is about €50m per annum. Lines operated by CFR Calatori generally have the highest levels of fare evasion which reflects the more comprehensive revenue protection measures that private operators have introduced;

Proposed solutions: To address the current levels of ticketless travel, measures which incentivise operators to collect a greater proportion of the total revenue are urgently required and should be linked to the Governance section described above. In addition, campaigns which highlight the social and financial implications of fare evasion are required, with more stringent fines for passengers found without a ticket, or the correct ticket for their journey. Many of the private operators who have successfully introduced such measures have boosted revenue by 6-7 times compared with the CFR Calatori services. This indicates the initiatives to reduce fare evasion should be self-funding.

Theme 4: System Regulation and Organisation

Problem: the existing governance arrangements mean there is relatively incentive for operators to implement commercial initiatives, or act in a commercially focussed way. There are limited financial incentives for the infrastructure provider or the operator to implement initiatives which will help to attract new passengers, or introduce efficiencies;

Proposed solution: A revised scenario with the range of commercial and other initiatives could be delivered. The implementation of these measures will enable both costs and revenues to be boosted prior to changes affecting the current PSC. Revisions to the existing contract will then need to be examined and should include transferring a much greater commercial risk onto the operator. At present, the commercial risk being taken by the operator is relatively limited and this adversely affects their willingness to introduce alternative initiatives. A contractual model that takes account of the differences between the revenues and costs once the above initiatives have been implemented is recommended. Some of these improvements intended to deliver cost savings and patronage growth is required since it is recognised some improvements may need to be delivered incrementally over a longer time period. The structure of the financial support needs to be tailored to achieve these outcomes.

Theme 5: Management Efficiency

Rationalising Stations

Problem: Romania has about 1,100 passenger stations, although many of these are local halts which attract low passenger demand. The number of local services calling at them is also very limited with lengthy gaps between trains. This reduces the attractiveness of rail. Station usage statistics from 2011 indicate that over 65% of the stations are used by fewer than 100 passengers per day, with only 8% of stations used by at least 500 passengers per day;

Proposed solution: The scope to reduce the number of stations through a programme of closures must be examined, particularly if the overall travel market using a branch line is small and therefore offers limited scope for growth. Reducing the number of stations could also create opportunities to cut journey times between the larger stations. It would also reduce the costs associated with maintaining a large number of under-utilised stations.

Introduce new signalling systems

Problems: Some possible solutions to improve signalling and address the current limitations are described below. There are significant inefficiencies affecting CFR Calatori and CFR Infrastructure. Slow speeds and the current pattern of timetables means CFR Calatori requires 15-20% more drivers than necessary to operate services. Meanwhile, CFR SA still operates many signalling systems manually which means staff numbers are higher than necessary;

Proposed solutions: For example, stations with electric signals require just one third of the total staff compared with manual controls. A progression towards greater automation would enable staff numbers to be reduced. Furthermore, the introduction of ERTMS pilot would help to achieve these objectives, whilst the feasibility of better communications with centralised control would also contribute.

Availability of Redundant Land

Problem: The reduction in train services and other efficiency savings mean the requirement for land to stable trains and for maintenance have been significantly reduced. Consequently, there is considerable redundant land available in certain locations which could be reused for other purposes. This includes redundant sidings, buildings at stations, or part of depots or marshalling yards;

Proposed solutions: Some of this land could be redeveloped for office development, retail or as an intermodal facility. These opportunities are particularly relevant in the larger population centres if the plots are adjacent to the city centre or the main road network. Bucuresti Triaj has been suggested as a possible scheme, although other sites in the capital should also be identified and then prioritised if the potential is stronger financial case versus other examples. Best practice from elsewhere including Krakow in Poland may also be informative to identify possible schemes.

Theme 6 Poor Infrastructure and Unattractive Passenger Services

Enhanced Frequencies on the Core Rail Corridors

The introduction of a revised maintenance strategy would enable trains to be scheduled throughout the morning to address the lengthy gaps which currently affect many routes. This would allow timetable gaps to be filled, and ensure trains operate at times passengers wish to travel rather than imposing a service-led timetable. Details of the service changes for individual corridors are set out below. These timetable revisions have been examined in ‘packages’ to maximise the benefits. For example, introducing new rolling stock will not achieve competitive speeds unless the rail network is upgraded and these units will not be efficiently deployed unless more intense timetables are operated.

The main objective of the proposed timetables is to provide a fast and frequent service between key stations on each corridor, maximising the rail market share with minimal incremental change in operating costs. Existing InterCity (IC) and InterRegio (IR) services on each corridor are replaced by a regular interval service pattern timetable.

Track and Timetables

Problem: Productivity is low compared to other European countries with Romanian railways only achieving 40% of the average. The timetables are constrained by a number of factors including slow line speeds, the requirement to complete brake safety checks while units are in service. The lengthy dwell times at stations account for up to 10% of total travel time on some services. Turnaround times at some terminus stations are also longer than necessary, with trains sitting for up to five hours. Furthermore, the lack of a regular interval timetable with a consistent timing of departures and trains to the same destinations is confusing for passengers.

Proposed solution: Introducing measures to improve the productivity ratio must be a priority and would be tackled by introducing a more intensive timetable, as described in the timetable interventions below. This would help to attract a higher number of passengers to the rail network and is especially relevant for demand between Bucharest and cities located 2-3 hours travel time from the capital. In particular, there is significant scope to grow the rail market share between Bucharest and Constanta, Craiova, Brasov, Ploiesti, and Buzau, especially those corridors which are less well served by road since this will strengthen the competitive advantage of rail. For the designated primary network, timetable changes are proposed which feature higher frequencies on the busiest corridors with a simplified, regular pattern. Scheduling is also more efficient with shorter turnarounds at the terminus station. The proposals are discussed below for individual lines;

Rail interchanges are enhanced to maximise rail connectivity, reduce delays and increase the opportunity to complete long / medium distance journeys. IR feeder services from major branch lines and other parts of the network will be connected to other stations at interchange points along the corridor. IC trains are generally substituted by shorter and more frequent IR trains, with the exception of International services which are assumed to be unaffected. There would be a journey time saving for trains using a rehabilitated line. Figure 5.26 summarises the proposed timetable structure, with each line representing a train every 2 hours. Services would depart at regular intervals with a standardised calling pattern to ensure the timetable becomes more memorable and simplified for passengers. Bucharest Gara du Nord would become an even more important interchange for passengers making journeys beyond the capital since no trains would operate as cross-capital. It is assumed the proposed trains would adopt a calling pattern that stops at a limited number of intermediate halts. The service groups comprise:

DS01 Bucharest to Arad and Cluj Napoca via Brasov: 2tph between Bucharest and Brasov, with 0.5tph extended to both Arad and Cluj Napoca

DS02 Bucharest to Constanta: 1tph between these stations;

DS03 Bucharest to Deva and Arad via Craiova: 2tph between Bucharest and Craiova with 0.5tph extended to both Deva and Arad via Timisoara;

DS04 Bucharest to Galati, Focsani, Iasi and Suceava: 0.5tph to each station;

DS05 Bucharest to Pitesti, Sibiu via a new link west of Ramnicu Valcea: 1tph between Bucharest and Pitesti, with alternate trains extended to Sibiu, plus a 2 hourly service from Brasov to Deva;

DS06 Iasi to Cluj Napoca via Suceava and Dej: 2 hourly service, plus rehabilitation of track to design speeds;

DS07 Cluj Napoca to Oradea: 2 hourly service, plus rehabilitation of track to design speeds, line doubling and electrification;

DS08 Stamora Moravita to Oradea via Timisoara: 2 hourly service, plus rehabilitation of track to design speeds and electrification;

DS09 Oradea to to Cluj Napoca via Satu Mare and Baia Mare: 2 hourly service, plus rehabilitation of track to design speeds;

DS10 Bucharest to Giurgiu: 2 hourly service, plus electrification;

DS11 Craiova to Calafat: 2 hourly service, plus electrification.

In addition to the proposed service pattern above, there are a number of key interchanges including Cluj Napoca, Timisoara, Brasov and Oradea where several services converge, albeit with relatively low frequencies. The timing of these connecting trains needs to be carefully scheduled to maximise the opportunities for interchange between these limited stop services, the complementary Regio trains and wider bus network.

In addition to the proposed network which could be served by a regular interval timetable, several other corridors were identified for improvements. These corridors include Fetesti to Faurei which would support the development of the agricultural economy in the area, plus links to Moldova from Faurei via Tecuci, Barlad, Vaslui and Iasi. The freight market would be the main beneficiary of the proposed improvements. However, making the economic case for these service proposals will be challenging, especially given the proposed improvements for passengers services to / from Iasi via Pascani.

Furthermore, stakeholders have identified a number of lines which could fulfil a tourism function, but do not feature on the proposed primary network. For example, Oraviţa-Anina, Caransebeş-Haţeg, Sibiu-Agnita, Turda-Abrud, Luduş-Magheruş and Tg. Mureş-Sovata Băi could be retained as secondary routes by the relevant local authorities, subject to more detailed assessments which take account of their tourism potential.

The introduction of the new passenger services, along with the rehabilitation of the lines to help restore former line speeds, will also contribute to improving connectivity with the major ports. For example, rail links to the ports at Constanta, Galati, Braila, Cernavoda, Giurgiu, Calafat and Drobeta Turnu Severin would also be improved as a result of delivering these proposals. They would contribute to improving port / rail modal integration.

Station Facilities

Problem: The facilities available at most railway stations in Romania are relatively poor and do not offer a high quality passenger environment when waiting for a service. For example, there are no stations monitored by CCTV, whilst adequate lighting, seating areas, toilets and covered waiting shelters are generally not available;

Proposed Solutions: Real time information and communication systems also help to generate higher passenger demand by reducing the uncertainty about potential delays. In addition to improving the facilities described above, better links with other transport modes and pedestrian networks are required. The opportunity to improve facilities within each station will be determined by the current and potential usage, as well as the likely construction costs. The scope of improvements need to cover the feasibility of improved inter-modal facilities to ensure connections to onward modes become more convenient. In addition, facilities for the mobility impaired, installation of real time information, lifts, escalators, waiting facilities and refreshments will be required.

It is essential the stations identified for rehabilitation must be linked to those routes which are also expected to be modernised since there is little benefit of addressing the former without the impact of improving the latter. Possible stations that could benefit from upgrades include Baia Mare, Satu Mare, Timisoara Nord, Miercurea Ciuc and Rosiori de Vede.

Figure 5.27: Proposed regular interval timetables

Source: AECOM proposal

Rail Network Electrification

The potential impact of electrification schemes has been evaluated. Data presented in a World Bank report indicated about 6,000km of the current network has not been electrified, so the business case for investment must be evaluated. The business case for electrification is usually determined by the potential reduction in operating costs that could be realised, although these efficiencies can be supplemented by the quantifying the economic value of reducing emissions if electric trains are introduced to replace diesel examples. The following assumptions have been applied to calculate the benefits and costs associated with electrification:

DMU cost for Inter-Regio trains: 52.6 lei / train kilometre;

EMU cost for Inter-Regio trains: 39.5 lei / train kilometre;

Value of air pollution – diesel train kilometre: 0.6239 EUR (costs were converted to lei to provide consistency with other inputs);

Value of air pollution – electric train kilometre: 0 EUR (costs were converted to lei to provide consistency with other inputs);

Value of greenhouse gas – diesel train kilometre: 0.1792 EUR (costs were converted to lei to provide consistency with other inputs);

Value of greenhouse gas – electric train kilometre: 0.1557 EUR (costs were converted to lei to provide consistency with other inputs);

Capital costs per kilometre are estimated as 3.09m lei per single track kilometre. This estimate reflects the typical electrification costs per track kilometre for other European schemes, adjusted to reflect the lower wage rates in Romania;

Other inputs include specifying the minimum number of train kilometres per day and hence the annual total, plus the estimated load factor, required to demonstrate the scheme offers a robust economic case (for example, the benefit cost ratio exceeds 1.0).

The values for the environmental parameters are based on the ‘rural’ rather than the ‘metropolitan’ area. The percentage of the railway that operates in the latter area is relatively small, so it is assumed that 100% of the route is located within a former area.

For a notional 100km electrification scheme, the high level results indicate an average of 1 train per hour in each direction for a 17 hour period would need to operate using electric traction to produce a positive business case (benefits exceed costs). All trains operating on this 100km section are assumed to be converted from diesel to electric traction to generate the necessary benefits. A conversion factor of 350 has been assumed to convert the daily totals to annuals. This minimum frequency threshold to make a positive business would need to be increased if there are sections of double track on the individual route sections, for example, Cluj Napoca to Oradea. Capital costs would be higher if a double track section of the network was electrified rather than a single track section would require a higher service frequency to produce a robust economic case. Assuming about 50% of the proposed route is double, the minimum frequency would need to increase by a similar proportion to generate sufficient benefits.

The business case for electrifying individual route sections has been examined to determine whether minimum frequencies are reached. This assessment has been conducted based on the current timetable, as well as the future service pattern featuring a regular interval pattern. This analysis indicated the section between Oradea and Cluj Napoca should be electrified regardless of the potential future timetable changes. There may be a case for electrifying the line from Galati / Tecuci to Barlad and Iasi, although some parts of this line do not have sufficient trains to make an economic case.

There are a small number of other schemes that may generate a positive case for electrification assuming an improved timetable is introduced. This includes Bucharest to Pitesti and Brasov to Alba Iulia via Sibiu. In addition, there may be a case for electrifying between Santana to Oradea and Baia Mare to Dej, although this depends on the likely changes to the Regio services once an improved InterRegio timetable is introduced. The current train frequencies operating on non-electrified lines are included in Table 5.13.

The electrification of the Core TEN-T network will include Craiova to Calafat, Giurgiu to Bucharest and Suceava to Ukraine. However, using the criteria described above, the business case to electrify these parts of the network would be relatively weak given the current number of trains.

Table 5.11: Summary of Current and Proposed Daily Frequencies (one-way)

| Route Section | Current | Future | Route Section | Current | Future |
| --- | --- | --- | --- | --- | --- |
| Galati – Barlad – Crasna – Iasi – Husi | 10 | 10 | Brasov – Sibiu – Alba Iulia | 14 | 17 |
| Veresti – Botosani – Dangeni – Iasi – Cristesti Jijia | 4-9 | 4-9 | Blaj – Tarnaveni | 11 | 11 |
| Darmanesti – Dornesti | 10 | 10 | Tarnaveni – Praid | 4 | 4 |
| Dornesti – Nisipitu | 4 | 4 | Razboleni – Targu Mares – Deda | 13 | 13 |
| Bicaz – Bacau | 9 | 9 | Razboleni – Sarmasu – Sieu Magherus | 7 | 7 |
| Roman – Buhaiesti | 6 | 6 | Deva – Arad via Santana | 6 | 6 |
| Ploiesti – Urziceni – Giurgeni | 5 | 5 | Santana – Oradea | 12 | 19 |
| Bucharest – Urziceni – Faurei | 10 | 10 | Oradea – Cluj Napoca | 23 | 27 |
| Faurei – Tecuci | 2 | 2 | Oradea – Satu Mare | 10 | 10 |
| Ploiesti – Slanic | 3 | 3 | Satu Mare – Baia Mare | 9 | 14 |
| Ploiesti – Maneciu | 2 | 2 | Baia Mare – Dej | 11 | 19 |
| Bucharest – Pitesti | 16 | 22 | Jibou – Saculeni | 6 | 6 |
| Pitesti – Curtea de Arges | 4 | 4 | Satu Mare – Bixad | 6 | 6 |
| Pitesti – Argesel | 5 | 5 | Jibou – Carei | 5 | 5 |
| Pitesti – Rosiori de Vede | 10 | 10 | Timisoara – Resita | 11 | 11 |
| Pitesti – Craiova | 10 | 10 | Timsisoara – Jimbola | 3 | 3 |
| Rosiori de Vede – Zimnices | 9 | 9 | Timisoara – Sannicolau Mare | 5 | 5 |
| Rosiori de Vede – Turnu Magurele | 4 | 4 | Salva – Valea Viseului | 6 | 6 |
| Corabia – Caracal | 6 | 6 | Medgidic – Tulcea | 4 | 4 |
| Caracal – Sibiu | 10 | 13 | Medgidic – Negru Voda | 2 | 2 |
| Craiova – Calafat | 4 | 4 | Eforie – Mangalia | 7 | 7 |
| Bucharest – Oltenita | 2 | 2 | Videle – Giurgiu | 7 | 7 |
| Buzau – Neholasu | 9 | 9 | Caransebes – Subcetate | 5 | 5 |
| Sibiu – Copsa Mica | 13 | 13 | Lugoj – Illa | 9 | 9 |
| Tecuci – Barlad | 15 | 15 | Bucharest – Giurgiu via Baneasa | 2 | 2 |
| Timisoara – Stamora Moravita | 10 | 10 |  |  |  |

Source: AECOM proposal

New Track Infrastructure

The electrification of the Core TEN-T network will include Craiova to Calafat, Giurgiu to Bucharest and Suceava to Ukraine. However, using the criteria described above, the business case to electrify these parts of the network would be relatively weak given the current number of trains.

Theme 7 Poor Infrastructure and Unattractive Freight Services

Problem: The existing permitted top speed for rail freight on several corridors is too slow to compete effectively with road based alternatives. This intervention will address the current low average speed of rail freight in Romania (21km/h), which makes rail freight expensive, slow and uncompetitive compared to road transport;

Proposed Solution: AECOM proposes that CFR Infrastructure allows a higher top speed for certain freight trains on rehabilitated lines - 120kph for intermodal block trains whilst the 80kph design speed should be achievable for other bulk and mixed traffic. This would increase rail speeds to the target design speeds. Furthermore, the creation of a two tier system to allow properly equipped and dedicated intermodal trains to be prioritised over other types of bulk freight should be examined. This would potentially give freight services priority versus local stopping passenger services. This intervention will encourage the uptake of rail freight for intermodal transport, significantly reducing carbon emissions resulting from in switch from road.

Theme 8 Inadequate freight management and operations

Problem: Current working practices are not consistent with modern best practice and need to be revamped to promote greater efficiency. The current uptake of more efficient new technologies is low, with old systems and paperwork coupled with lack of training hindering the development of a modern railway. For example, train sheets are filled in manually with every wagon number recorded in handwriting and then where each wagon needed to be moved to.

Proposed Solution: Companies should adopt modern practice and technologies such as the use of GPS for rolling stock tracking. This needs to be accompanied by the creation of modern training courses in rail operation systems and technology for existing and new staff. This approach would enhance the asset performance of the railway by encouraging operational efficiencies. This scheme would be implemented by CFR SA, CFR Marfa, private operators and training bodies between 2015 and 2020.

Theme 9 Infrastructure deficit affecting freight services

Problem: A 20.5 tonne axle limit is currently in force, but this is less than the EU rail network standard of 22.5t. Many trains and rolling stock are thus used in a sub-optimal way. Furthermore, there is currently no section of track, for example, where power recovery from overhead electrification is possible which could significantly reduce costs and improve the sustainability of the network.

Proposed Solution: To encourage international and transit traffic interventions that allow 22.5 tonne axle weights on rehabilitated routes should be supported. Clearly the whole route needs to be cleared for operators to use this effectively. This feature should be integrated into all future route rehabilitation. It would enhance the asset performance of the railway especially the opportunities to achieve operational efficiencies. Upgrading to the higher axle weight will enable more efficient loading of wagons. For a 30 wagon train, this equates to an additional 240 tonnes of product (a 15% increase). It will also improve through-running of international trains.

Undertake a feasibility study to assess the scope for introducing power systems with the capability to return energy generated from regenerative braking to the power network during all future rehabilitation work. Regenerative braking can save 5% on power use on freight and up to 17% on commuter passenger trains. This scheme would be implemented by CFR SA and would be undertaken immediately to enable any regenerative technology to installed on key corridors as part of the rehabilitation process.

Other Themes

There are aspirations for several tourist lines to be retained as part of the Masterplan. However, these routes do not form part of the primary network and serve a fundamentally different travel market. The retention of these lines will be dependent on applying a fundamental different travel market compared with the approach used to assess the other primary and secondary routes. On this basis, an alternative approach which takes account of tourist visits to the area should be used.

Testing Rail Service Options

It has been assumed that the base line for the rail service option assessment is steady state maintenance and repairs of the core network, ‘as now’ maintenance of the rest of the network and that the 2011 timetable will be retained in 2020, 2030 and 2040 (except for passenger services on line 902 which assumes the Arges river bridge is reinstated). Table 5.10 includes a brief description of the improvements modelled for each alternative.

Table 5.12: Description of Service Proposals

| Description |  | Code | Test description |  |
| --- | --- | --- | --- | --- |
| Bucharest to Arad / Cluj via Brasov |  | DS01A | Restore core TEN-T corridor IV-N and link to Cluj-Napoca to design speed. Package of measures includes track rehabilitation to design speed, improvements to the power supply and signalling equipment (ERTMS-2), new rolling stock and improved station facilities at major nodes (for example, Bucharest Nord, Ploiesti, Brasov, Sighisoara, Medias, Blaj, Teius, Alba Iulia, Deva, Arad, Razboieni, Campia Turzil and Cluj Napoca). Network to be rehabilitated: 795km Service frequency: 2 tph to Brasov, then 0.5tph extended to Arad and 0.5tph to Cluj Napoca Number of units required: 22 EMUs Further electrification: No Double tracking: No |  |
|  |  | DS01B | Core TEN-T corridor IV-N and link to Cluj-Napoca to enhanced speed. Other interventions as per the proposals above |  |
| Bucharest to Constanta |  | DS02A | Line from Bucharest to Constanta comprises new rolling stock and improved station facilities at major nodes (Fundulea, Lehliu-Gara, Ciuinita, Fetesti, Cernavoda Pod, Medgidia, Constanta). Interventions will make better use of the previously rehabilitated network. There may be some scope for further improvements when bridges and stations which are currently being rehabilitated are completed Network to be rehabilitated: 0km (except bridges and stations) Service frequency: 1 tph to Constanta Number of units required: 4 EMUs Further electrification: No Double tracking: No |  |
| Bucharest to Arad via Craiova |  | DS03A | Restore line from Bucharest to Arad via Craiova to design speed. Improve power supply and introduce regenerative braking, enhanced signalling equipment (ERTMS-2), new rolling stock and improved station facilities at major nodes (for example, Bucharest Nord, Videle, Rosiori de Vede, Caracal, Craiova, Filiasi, Strehaia, Drobeta, Baile Herculane, Caransebes, Lugoj, Timisoara and Arad) Network to be rehabilitated: 875km Service frequency: 2 tph to Craiova, then 0.5tph extended to Arad via Timisoara and 0.5tph to Deva Number of units required: 19 EMUs Further electrification: No Double tracking: No |  |
|  |  | DS03B | Increase line speed from Bucharest to Arad via Craiova. Other interventions as per the proposals above |  |
|  |  | DS03C | Restore line from Bucharest to Arad via Craiova and Filiasi to Deva to design speed. Other interventions as per the proposals above |  |
| Bucharest to Galati / Iasi / Suceava |  | DS04A | Restore core TEN-T corridor IX and links Buzau to Galati and Pascani to Suceava to design speed. Improve power supply and introduce regenerative braking, enhanced signalling equipment (ERTMS-2), new rolling stock and improved station facilities at major nodes (for example, Bucharest Nord, Ploiesti, Buzau, Ramnicu, Focsani, Marasesti, Adjud, Bucau, Roman, Pascani, Dollnasca, Veresti, Suceava, Faurei, Braila, Galati, Pascani, Tagu Frumos and Iasi). Network to be rehabilitated: 1,260km Service frequency: 0.5tph to Suceava, 0.5tph to Iasi, 0.5tph to Galati, 0.5tph to Focsani Number of units required: 25 EMUs Further electrification: No Double tracking: No |  |
| Bucharest to Sibiu via Ramnicu Valcea |  | DS05A | Restore line from Bucharest to Pitesti to design speed and new link Videle - Ramnicu Valcea. Package of other improvements to include enhanced signalling equipment, new rolling stock and improved station facilities at major nodes (for example, Bucharest Nord, Titu, Gaesti, Pitesti). Network to be rehabilitated: 320km Service frequency: 1tph to Pitesti, then 0.5tph extended to Craiova and 0.5tph to Sibiu Number of units required: 7 DMUs Further electrification: No Double tracking: No |  |
|  |  | DS05B | Restore line from Bucharest – Pitesti to design speed, new link Videle - Ramnicu Valcea and line electrification. Other interventions as per the proposals above, plus improved signalling and regenerative braking Network to be rehabilitated: 320km Number of new units: 7 EMUs Further electrification: Yes |  |
|  |  | DS05C | Restore line from Bucharest – Pitesti and Pitesti – Craiova to design speed with the construction of a new link between Vialcele and Ramnicu Valcea, plus electrification of the line. Other interventions as per the proposals above, plus improved signalling and introduction of regenerative braking Network to be rehabilitated: 470km Number of new units: 11 EMUs Further electrification: Yes |  |
| Cluj Napoca to Iasi |  | DS06A | Restore line from Cluj-Napoca to Iasi to design speed. Other interventions include improve power supply and introduce regenerative braking, enhanced signalling equipment, new rolling stock and improved station facilities at major nodes (for example, Cluj Napoca, Gherla, Dej Calatori, Beclean pe Sommes, Salva, Ilva Mica, Vatra Dornei, Compulang Mold, Suceava, Veresti, Dollnasca, Pascani, Targu Frumos, Iasi) Network to be rehabilitated: 740km Service frequency: 0.5tph from Cluj Napoca to Iasi Number of units required: 7 EMUs Further electrification: No Double tracking: No |  |
|  |  | DS06B | Line Cluj-Napoca to Iasi to enhanced speed. Other interventions as per the proposals above. |  |
| Cluj to Oradea |  | DS07A | Restore line from Cluj-Napoca to Oradea to design speed. Other interventions include improve power supply and introduce regenerative braking, enhanced signalling equipment, new rolling stock and improved station facilities at major nodes (for example, Cluj Napoca, Huedin and Oradea). Network to be rehabilitated: 270km Service frequency: 1tph to Cluj Napoca to Oradea Number of units required: 4 DMUs Further electrification: No Double tracking: No |  |
|  |  | DS07B | Restore line Cluj-Napoca to Oradea to design speed plus electrification and line doubling Other interventions as per the proposals above. Number of units required: 4 EMUs Further electrification: Yes Double tracking: Yes |  |
|  |  | DS07C | Electrify Cluj-Napoca to Oradea line Further electrification: Yes |  |
| Stamora Moravita to Oradea via Timisoara |  | DS08A | Restore line from Timisoara – Oradea to design speed. Other interventions include improve power supply and introduce regenerative braking, enhanced signalling equipment, new rolling stock and improved station facilities at major nodes (for example, Timisoara Nord, Arad and Oradea). Network to be rehabilitated: 200km Service frequency: 0.5tph to Stamora Moravita to Oradea via Timisoara Number of units required: 5 DMUs Further electrification: Partial Double tracking: No |  |
|  |  | DS08B | As per the proposal above including the section Timisoara - Stamora Moravita |  |
| Oradea to Cluj via Baia Mare and Satu Mare |  | DS09A | Restore line from Oradea to Satu Mare and Satu Mare to Cluj-Napoca to design speed. Package of other improvements to include enhanced signalling equipment, new rolling stock and improved station facilities at major nodes (for example, Oradea, Satu Mare, Baia Mare, Dej and Cluj Napoca) Network to be rehabilitated: 475km Service frequency: 0.5tph between Baia Mare and Oradea via Satu Mare, 0.5tph to Satu Mare via Cluj Napoca via Baia Mare Number of units required: 6 DMUs Further electrification: No Double tracking: No |  |
| Bucharest to Giurgiu |  | DS10A | Restore line from Bucharest to Giurgiu via Gradistea to design speed. Package of other improvements to include enhanced signalling equipment, new rolling stock and improved station facilities at major nodes (for example, Bucharest Nord, Videle and Giurgiu). Network to be rehabilitated: 95km Service frequency: 0.5tph to Giurgiu from Bucharest Number of units required: 2 DMUs Further electrification: No Double tracking: No |  |
|  |  | DS10B | Restore line from Bucharest to Giurgiu via Gradistea to design speed plus line electrification. Package of other improvements to include enhanced signalling equipment, new rolling stock and improved station facilities at major nodes (for example, Bucharest Nord, Videle and Giurgiu). Number of units required: 2 EMUs Further electrification: Yes |  |
| Craiova to Calafat |  | DS11A | Restore core TEN-T corridor IV-N section from Craiova to Calafat to design speed. Package of other improvements to include enhanced signalling equipment, new rolling stock and improved station facilities at major nodes (for example, Craiova and Calafat). Network to be rehabilitated: 115km Service frequency: 0.5tph to Craivoa from Calafat Number of units required: 3 DMUs Further electrification: No Double tracking: No |
|  |  | DS11B | Restore core TEN-T corridor IV-N section from Craiova to Calafat to design speed and electrification. Package of other improvements to include enhanced signalling equipment, new rolling stock and improved station facilities at major nodes (for example, Craiova and Calafat). Network to be rehabilitated: 115km Service frequency: 0.5tph to Craivoa from Calafat Number of units required: 3 EMUs Further electrification: Yes Double tracking: No |
| Combined scenario |  | DS99A | Combined scenario with the upgrades considered for the best performing alternative for each corridor. Package of other improvements to include enhanced signalling equipment, new rolling stock and improved station facilities at major nodes |

Source: AECOM proposals

There are a limited number of further stations which benefit from station improvements but are not served by the proposed regular interval timetable, for example, Targoviste and Miercurea Ciuc. The case for improving these stations would need to be evaluated separately, with the scope of improvements informed by the likely demand.

The above package of proposals has been compared against the Reference case assumption which is based on a similar level of service to the 2011 timetable to assess their impact. Whilst there may have been modest service improvements delivered since 2011, the package of measures described above will offer substantial improvements for those corridors which offer the strongest opportunities to increase the existing rail travel market.

In addition to the described model runs, an alternative set of tests using the naming convetion (DSXXR) have been defined to assess the case for removing low costs speed restrictions and implementing a regular interval timetable on each major corridor. The speed restrictions impose significant delays to passenger and freight trains and affect service reliablility. The implementation of this package would represent an opportunity to deliver some “quick wins”. The proposed regular interval timetables are comparable to those specified to for the rehabilitation tests in terms of daily frequencies, but have been adjusted to the forecasted travel times after the discussed restrictions are addressed. This outcome has some implications for rolling stock requirements and hence the total scheme costs.

High Level Results for the Service Options

Figure 5.27 summarises the overall benefit cost ratio for each test. Although the combined scenario produces a benefit cost ratio of 1.0, individual schemes that comprise service proposals radiating from Bucharest offer strong value for money (a BCR over 1), whilst the results for routes linking other cities generally produce a business case which offers poor value for money (BCR under 1). Scenario DS10 is the best performing scheme with a BCR of about 4.2. This high figure is due to the characteristics of the corridor with no rail services operating in 2011 on line 902 from Giurgiu beyond Gradistea due to the collapse in 2005 of the Arges river bridge. Reconstructing this bridge, bringing line speeds back to the design parameters, and implementing a regular interval timetable on the line generates significant benefits at a modest cost.

Similarly, test DS02A could deliver substantial benefits from the deployment of a more intensive timetable operated by modern rolling stock on the Constanta line. Although this line has been rehabilitated in the last decade to a high standard with design speeds of up to 200 km/h, services have not achieved the anticipated travel times after rehabilitation. Evidence indicates that the potential rail market share on this section has not been achieved, with the upgraded infrastructure currently underutilised.

Scenarios involving rehabilitation of long corridors radiating from Bucharest such as DS01, DS03 and DS04, yield BCR’s marginally above 1. This indicates that the rehabilitation programme is worthwhile. With the forecasts showing demand using sections of the rail network closer to Bucharest increases, the analysis is also presented for individual sections to illustrate these impacts.

Figure 5.28: BCR of the best performing test for each scenario involving infrastructure upgrades

Source: AECOM calculations using the National Transport Model

Results Analysis – Key Performance Indicators

Table 5.11 summarises outputs for the rail model runs described above using the National Transport Model. The results illustrate the performance of each modelled test versus the reference scenario. This demonstrates the potential to achieve rail growth if an appropriate package of investment is delivered. It should be highlighted that steady state maintenance and repairs of the core network will translate into increased maintenance and renewals costs. The outputs include change in passenger kilometres, change in tonne kilometres, the Net Present Value of the scheme and the benefit cost ratio. Upgrading a corridor has several impacts:

Overall increase in rail demand (generation of trips);

Rerouting of traffic towards the upgraded line (abstraction from other corridors).

The main lines to / from Bucharest attract more demand and produce a higher benefit cost ratio compared with other remote corridors that attract lower passenger numbers and / or freight traffic. This implies the main lines around Bucharest should be prioritised if limited funds for investment are available. In general terms, the forecasts demonstrate the impacts on the passenger sector are generally greater than the freight sector. This is partially due to the assumption that freight trains only receive a smaller time saving after corridor rehabilitation. This assumption can also be validated by observing freight travel times on the Constanta line before and after the line rehabilitation. The very low rail mode share and significant proportion of the population without a car available contribute to the relatively high increase in passenger demand.

Similarly, Table 5.11 highlights that scenario “A” variants tend to perform better than the alternative scenario “B”. This conclusion is particularly evident if the only difference is the assumed maximum design speed. This implies that the extra benefits generated from the higher maximum speeds does not justify the additional costs. There are sections where upgrades beyond the current design speed would help to smooth speed profiles, and these proposals should be examined in more detail. For example, Test DS05B comprises the electrification of the line between Bucharest and Sibiu via Pitesti and Ramnicu Valcea (Corridor 200). The electrification removes air pollution dis-benefits that would otherwise occur in DS05A from the diesel freight trains. The only enhancement in test DS05B versus DS05A is electrification, whilst DS07B includes line doubling and electrification.

The results for scenario DS07 suggest that either variant ‘A’ or ‘B’ could be included in the combined scenario alternative (DS99A). However, DS07A has been selected since it generates the highest NPV. Test DS02A results in a minor reduction in freight traffic following the removal of speed restrictions between Bucharest and Constanta. This corridor did not achieve a reduction in freight journey time savings in the early 2000s suggesting that the current condition of the line does not constrain travel times. In this context, the passenger trips transferring from road to rail slightly improves the traffic flow using the parallel motorway which in turn improves the attractiveness of road freight.

Table 5.11 also presents the results for the package of low cost improvements which involve the removal of low cost speed restrictions and regular interval timetables. This package of measures offers good value for money versus the reference case, with increased number of passengers and rail freight tonnes transported. In the ‘R’ variant scenario for DS07, there is a decrease in the number of passengers compared with the reference case. This is a result of the additional time imposed on passengers travelling from Oradea to the centre and southern parts of Romania given the requirement to change trains. In this case therefore timetables on the line Oradea to Cluj-Napoca could be adjusted to match the arrival and departure times of trains running on corridor IV-N Bucharest and minimise this impact.

The introduction of these low cost options should play a core role in shaping the future strategy. The opportunity to secure some quick-wins which are predicated on the implementation of some lower cost infrastructure measures will be benefical in helping to reverse the decline in passenger numbers using rail which has occurred during the last 10-20 years. The introduction of the timetable changes, new rolling stock and infrastructure measures which remove the speed restrictions will help to reverse the downward spiral in rail usage. However, this package of measures should only form a starting point for further investment to upgrade the infrastructure and deliver extra journey time reductions. The latter schemes are expected to offer a lower benefit cost ratio, but this should not prevent this investment from proceeding.

As in the line rehabilitation tests, freight traffic experiences only a modest growth as a result of the forecasted journey time savings. Test DS12R is mostly a freight test, does not include regular interval timetable, which considers the impact of removing speed restrictions from line 702 between Fetesti and Faurei.

Table 5.13: Headline Results from Selected Model Runs (2030)

| Model run | Description | Change - Pass-km (000’s) | Change in Pass-km share | Change in Tonne-km (000’s) | Change in Tonne-km share | NPV € Mill (2014 Prices) | BCR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DS01R | Bucharest to Arad / Cluj via Brasov | +1,781 (+8%) | +0.5% | +19(0%) | +0.0% | 277 | 8.32 |
| DS01A |  | +5,814 (+27%) | +1.7% | +4,175 (+9%) | +1.1% | 183 | 1.09 |
| DS01B |  | +6,374 (+30%) | +1.9% | +4,189 (+9%) | +1.1% | -36 | 0.99 |
| DS02A | Bucharest to Constanta | +1,422 (+7%) | +0.4% | -14 (+0%) | 0% | 255 | 13.62 |
| DS03R | Bucharest to Arad via Craoiva | +2,403 (+11%) | +0.7% | +566 (+1%) | +0.2% | 422 | 2.55 |
| DS03A |  | +4,946 (+23%) | +1.5% | +4,101 (+6%) | +1.2% | 106 | 1.06 |
| DS03B |  | +5,721 (+27%) | +1.7% | +4,083 (+6%) | +1.2% | 5 | 1.00 |
| DS04R | Bucharest to Galati / Iasi / Suceava | +1,370 (+6%) | +0.4% | 30 (0%) | +0.0 | 298 | 3.64 |
| DS04A |  | +6,783 (+32%) | +2.0% | +2,159 (+3%) | +0.6% | 529 | 1.23 |
| DS05R | Bucharest to Sibiu via Ramnicu Valcea | +73(+0%) | +0% | +26 (0%) | 0% | -87 | - |
| DS05A |  | +1,253 (+6%) | +0.4% | +1,435 (+2%) | +0.5% | -605 | 0.01 |
| DS05B |  | +1,238 (+6%) | +0.4% | +1,358 (+2%) | +0.4% | 659 | 1.61 |
| DS05C |  | +1,580 (+7%) | +0.5% | +2,855 (+4%) | +0.8% | 291 | 1.23 |
| DS06R | Cluj Napoca to Iasi | +272 (+1%) | +0.1% | +24 (+0%) | 0% | 75 | - |
| DS06A |  | +1,390 (+7%) | +0.4% | +1,220 (+2%) | +0.3% | -1,183 | 0.39 |
| DS06B |  | +2,156 (+10%) | +0.6% | +1,221 (+2%) | +0.3% | -1,740 | 0.35 |
| DS07R | Cluj Napoca to Oradea | -234 (-1%) | -0.1% | +11 (0%) | 0% | -8 | 0.73 |
| DS07A |  | +179 (+1%) | +0.1% | +456 (+1%) | +0.1% | -316 | 0.30 |
| DS07B |  | +389 (+2%) | +0.1% | +456 (+1%) | +0.1% | -740 | 0.39 |
| DS07C |  | +58 (0%) | +0% | -19 (0%) | 0% | -5 | 0.98 |
| DS08R | Stamora Moravita to Oradea via Timisoara | +421 (2%) | +0.1% | -12 (0%) | 0% | -108 | 0.35 |
| DS08A |  | +1,212 (+6%) | +0,4% | +281 (+1%) | +0.1% | -98 | 0.63 |
| DS08B |  | 1,251 (+6%) | 0.4% | +281 (0%) | 0.1% | -98 | 0.63 |
| DS09R | Oradea to Cluj via Baia Mare and Satu Mare | +444 (+2%) | +0.1% | -10 (0%) | +0.0% | -103 | 0.12 |
| DS09A |  | +1,085 (+5%) | +0.3% | +414 (+1%) | +0.1% | -633 | 0.25 |
| DS10R | Bucharest to Giurgiu | +456 (+2%) | +0.1% | +54 (0%) | +0% | -395 | 11.18 |
| DS10A |  | +545 (+3%) | +0.2% | +123 (+0%) | 0% | 335 | 4.20 |
| DS10B |  | +545 (+3%) | +0.2% | +123 (+0%) | 0% | 347 | 2.68 |
| DS11R | Craoiva to Calafat | +151 (+1%) | +0.0% | -1 (0%) | 0.0% | 33 | 3.65 |
| DS11A |  | +363 (+2%) | +0.1% | +92 (+0%) | 0% | -12 | 0.91 |
| DS11B |  | +363 (+2%) | +0.1% | +92% (+0%) | 0% | 90 | 0.64 |
| DS12RR | Freight test | +20 (+0%) | +0% | +18 (0%) | 0% | -15 | - |
| DS99A | Combined test | +24,289 (115%) (+115%) | +7.1% | +8,525 (13%) (1(+19%) | +2.5% | 248 | 1.03 |

Source: AECOM forecasts

Detailed Results Analysis

An approximate BCR has been calculated by route section by sub-dividing the costs and benefits, as shown in Table 5.12. Although this exercise produces aggregated results, it does provide a helpful indicator to rank projects at a more detailed level and then help inform the selection of priorities. A more detailed description of the test results is included in Appendix C. Table 5.12 demonstrate how sections of the network closer to Bucharest generate a better value for money. The two exceptions include Timisoara to Arad and Baia Mare to Satu Mare which link medium to large size cities with a poor rail service at present. Higher frequency regular interval timetables have been proposed between these cities which are expected to attract significant patronage given the competition with local bus services. The Reference Case also assumes some sections of corridor Core Ten-T IV-N will be rehabilitated by 2020. In accordance with the model outputs, the network changes governance and service improvements have the potential to reverse the trend of falling passenger and freight demand and deteriorating network conditions. There is significant scope to increase passenger and freight traffic on core routes which would improve the rail network and provide better services at lower cost.

Table 5.14: Detailed Results for selected tests (2014 prices)

| Test | Section | From | To | PVC (Mill €) | PVB (Mill €) | NPV (Mill ) | BCR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DS01A | All | Bucharest | Arad / Cluj | 2080 | 2263 | 183 | 1.09 |
|  | 1 | Bucharest | Campina | 94 | 417 | 323 | 4.44 |
|  | 2 | Campina | Predeal | 2 | 191 | 189 | - |
|  | 3 | Predeal | Brasov | 142 | 104 | -37 | 0.74 |
|  | 4 | Brasov | Sighisoara | 630 | 751 | 121 | 1.19 |
|  | 5 | Sighisoara | Coslariu | 8 | 129 | 121 | - |
|  | 6 | Coslariu | Simeria | 4 | 136 | 132 | - |
|  | 7 | Simeria | km 614 | 681 | 356 | -325 | 0.52 |
|  | 8 | km 614 | Hungary | 0 | 16 | 16 | - |
|  | 9 | Coslariu | Cluj-Napoca | 518 | 162 | -356 | 0.31 |
| DS01B | All | Bucharest | Arad / Cluj | 2477 | 2441 | -36 | 0.99 |
|  | 1 | Bucharest | Campina | 95 | 449 | 355 | 4.75 |
|  | 2 | Campina | Predeal | 2 | 207 | 205 | - |
|  | 3 | Predeal | Brasov | 246 | 115 | -131 | 0.47 |
|  | 4 | Brasov | Sighisoara | 636 | 817 | 181 | 1.28 |
|  | 5 | Sighisoara | Coslariu | 8 | 143 | 134 | - |
|  | 6 | Coslariu | Simeria | 4 | 146 | 142 | - |
|  | 7 | Simeria | km 614 | 687 | 373 | -314 | 0.54 |
|  | 8 | km 614 | Hungary | 0 | 18 | 18 | - |
|  | 9 | Coslariu | Cluj-Napoca | 799 | 174 | -625 | 0.22 |
| DS03A | All | Bucharest | Arad | 1658 | 1764 | 106 | 1.06 |
|  | 1 | Bucharest | Craiova | 594 | 786 | 192 | 1.32 |
|  | 2 | Craiova | Filiasi | 171 | 45 | -126 | 0.26 |
|  | 3 | Filiasi | Caransebes | 593 | 582 | -11 | 0.98 |
|  | 4 | Caransebes | Timisoara | 187 | 210 | 23 | 1.12 |
|  | 5 | Timisoara | Arad | 107 | 149 | 42 | 1.39 |
|  | 6 | Filiasi | Deva | 6 | -8 | -15 | - |
| DS03B | All | Bucharest | Arad | 2015 | 2020 | 5 | 1.00 |
|  | 1 | Bucharest | Craiova | 606 | 816 | 210 | 1.35 |
|  | 2 | Craiova | Filiasi | 173 | 47 | -126 | 0.27 |
|  | 3 | Filiasi | Caransebes | 926 | 800 | -126 | 0.86 |
|  | 4 | Caransebes | Timisoara | 195 | 222 | 27 | 1.14 |
|  | 5 | Timisoara | Arad | 108 | 142 | 34 | 1.31 |
|  | 6 | Filiasi | Deva | 6 | -6 | -13 | - |
| DS03C | All | Bucharest | Arad | 2223 | 1889 | -334 | 0.85 |
|  | 1 | Bucharest | Craiova | 608 | 827 | 220 | 1.36 |
|  | 2 | Craiova | Filiasi | 175 | 48 | -128 | 0.27 |
|  | 3 | Filiasi | Caransebes | 608 | 517 | -90 | 0.85 |
|  | 4 | Caransebes | Timisoara | 192 | 192 | 0 | 1.00 |
|  | 5 | Timisoara | Arad | 109 | 143 | 34 | 1.31 |
|  | 6 | Filiasi | Deva | 531 | 161 | -369 | 0.30 |
| DS04A | All | Bucharest | Galati / Iasi / Suceava | 2276 | 2805 | 529 | 1.23 |
|  | 1 | Bucharest | Ploiesti | 57 | 371 | 314 | 6.52 |
|  | 2 | Ploiesti | Buzau | 204 | 533 | 329 | 2.61 |
|  | 3 | Buzau | Focsani | 197 | 383 | 186 | 1.95 |
|  | 4 | Focsani | Bacau | 296 | 519 | 223 | 1.75 |
|  | 5 | Bacau | Pascani | 313 | 173 | -140 | 0.55 |
|  | 6 | Pascani | Moldova | 426 | 157 | -269 | 0.37 |
|  | 7 | Pascani | Ukraine | 407 | 81 | -326 | 0.20 |
|  | 8 | Buzau | Galati | 376 | 589 | 213 | 1.57 |
| DS05C | All | Bucharest | Craiova / Sibiu | 1275 | 1566 | 291 | 1.23 |
|  | 1 | Bucharest | Sibiu | 1010 | 1530 | 520 | 1.51 |
|  | 2 | Pitesti | Craiova | 265 | 36 | -229 | 0.14 |
| DS06A | All | Cluj Napoca | Iasi | 1945 | 762 | -1183 | 0.39 |
|  | 1 | Cluj-Napoca | Iasi | 317 | 223 | -94 | 0.70 |
|  | 2 | Cluj-Napoca | Dej | 112 | 23 | -89 | 0.21 |
|  | 3 | Dej | Beclean pe Somes | 385 | 172 | -213 | 0.45 |
|  | 4 | Beclean pe Somes | Vartra Dornei | 427 | 148 | -279 | 0.35 |
|  | 5 | Vartra Dornei | Suceava | 399 | 108 | -291 | 0.27 |
|  | 6 | Suceava | Pascani | 306 | 89 | -217 | 0.29 |
| DS06B | All | Cluj Napoca | Iasi | 2687 | 947 | -1740 | 0.35 |
|  | 1 | Cluj-Napoca | Iasi | 324 | 183 | -141 | 0.56 |
|  | 2 | Cluj-Napoca | Dej | 114 | 21 | -93 | 0.18 |
|  | 3 | Dej | Beclean pe Somes | 778 | 301 | -477 | 0.39 |
|  | 4 | Beclean pe Somes | Vartra Dornei | 745 | 277 | -467 | 0.37 |
|  | 5 | Vartra Dornei | Suceava | 411 | 98 | -313 | 0.24 |
|  | 6 | Suceava | Pascani | 314 | 67 | -248 | 0.21 |
| DS08B | All | Oradea | Serbia | 393 | 180 | -213 | 0.46 |
|  | 1 | Oradea | Timisoara | 321 | 176 | -145 | 0.55 |
|  | 2 | Timisoara | Serbia | 72 | 4 | -68 | 0.05 |
| DS09A | All | Cluj-Napoca | Oradea | 845 | 212 | -633 | 0.25 |
|  | 1 | Cluj-Napoca | Dej | 307 | 68 | -239 | 0.22 |
|  | 2 | Dej | Baia Mare | 308 | 20 | -289 | 0.06 |
|  | 3 | Baia Mare | Satu Mare | 68 | 115 | 47 | 1.69 |
|  | 4 | Satu Mare | Oradea | 162 | 10 | -152 | 0.06 |

Source: AECOM forecasting model

In addition to the results from the core option presented in Table 5.12, the impact of alternative assumptions for infrastructure repairs has been collated. For example, addressing just the temporary speed restrictions caused by badly maintained track such as turnouts forms the basis for the initial package of works, albeit with an assumption that a more extensive set of infrastructure works would be implemented downstream. The proposals for the revised timetable and new rolling stock as described in the core timetable option are retained for these sensitivity tests. Summary results for the core option (comprising rehabilitation, new rolling stock and a revised timetable – Scenario A), a more optimistic option (featuring higher line speeds – Scenario B), plus the pessimistic option with minimal infrastructure works (Scenario R). The BCR results for the pessimistic scenario are higher compared with the core or the optimistic timetable options and this reflects the less extensive infrastructure works which incur a lower capital cost. These results indicate that the timetable and rolling stock generate the majority of improvements, since the incremental change in benefits is greater than the revised costs. It should be noted that whilst the BCRs are higher in the pessimistic tests compared with the core option, there should still be a requirement to undertake the more extensive infrastructure works even though the incremental BCR is lower.

The initial elements to deliver a wider scheme will usually generate the highest BCR. However, this outcome should not affect the opportunity for more extensive infrastructure works in the future assuming the BCR for the overall scheme still exceeds 1.0. For some tests, there is a significant change in the BCR result for a sensitivity test versus the core option. This can be attributed to the relationship between the total benefits versus the costs. In particular, the relationship between the operating costs and the capital costs may differ. If the capital costs account for a high percentage of the total cost, the postponement of the majority of infrastructure costs until a latter phase of works will lead to a larger change in the total discounted costs. As a result, the BCR would be higher. Table 5.13 presents the results for the alternative tests including the lower cost options. There are two key messages to highlight. Firstly, the benefit cost ratio for the lower cost options is generally higher compared with the core option or the proposals which feature a higher line speed and reflects the smaller capital costs which are assumed. Secondly, there is a significant difference between the relative scale of the benefits and costs. Using Test DS01 as an example, the benefits and costs for variants A and B range from £2.1bn to £2.3bn over the 60 year period appraisal, yet the costs for variant R are less than £40m with benefits lower than £310m. The ability of the variant A and B for each test in generating freight benefits and costs represents one of the most important contributory factors for these differences. With faster journey times in scenarios A and B, this encourages additional freight to transfer from road to rail. This leads to higher costs and benefits. This pattern of results occurs for many of the tests, although the scale of the benefits and costs generated by the freight movements using rail differs depending on the change in journey times between the scenarios.

Table 5.15: Summary of the Economic Appraisal Results – Sensitivity Tests (2014 prices)

| Test and Description |  |  |  | PVC Total1 | PVB Total1 | Cost to infrast. Manager | Cost to operator (Pass.) | Cost to operator (Freight.) | Benefits (Pass.) | Benefits (Freight.) | BCR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DS01 |  | Bucharest to Arad / Cluj via Brasov | R | 38 | 315 | 4 | 30 | 4 | 306 | 9 | 8.32 |
|  |  |  | A | 2080 | 2263 | 2035 | 22 | 642 | 1703 | 1179 | 1.09 |
|  |  |  | B | 2477 | 2441 | 2433 | 30 | 642 | 1885 | 1184 | 0.99 |
| DS02 |  | Bucharest to Constanta | A | 20 | 275 | 0 | 104 | 0 | 349 | 10 | 13.62 |
| DS03 |  | Bucharest to Arad via Craoiva | R | 273 | 696 | 18 | 220 | 36 | 439 | 256 | 2.46 |
|  |  |  | A | 1658 | 1764 | 1623 | 220 | 604 | 1398 | 1154 | 1.06 |
|  |  |  | B | 2015 | 2020 | 1979 | 220 | 601 | 1655 | 1151 | 1.00 |
|  |  |  | C | 3079 | 2745 | 2187 | 220 | 672 | 1480 | 1265 | 0.89 |
| DS04 |  | Bucharest to Galati / Iasi / Suceava | R | 113 | 410 | 1 | 104 | 8 | 320 | 90 | 3.64 |
|  |  |  | A | 2276 | 2805 | 2213 | 110 | 297 | 2189 | 960 | 1.23 |
| DS05 |  | Bucharest to Sibiu via Ramnicu Valcea | R | 84 | -3 | 12 | 67 | 5 | -12 | 9 | - |
|  |  |  | A | 613 | 8 | 578 | 161 | -94 | 586 | -547 | 0.01 |
|  |  |  | B | 1088 | 1746 | 1052 | 144 | -88 | 646 | 1121 | 1.61 |
|  |  |  | C | 1275 | 1566 | 1240 | 169 | 211 | 711 | 1200 | 1.23 |
| DS06 |  | Cluj Napoca to Iasi | R | -30 | 46 | 2 | -35 | 4 | 37 | 9 | - |
|  |  |  | A | 1945 | 762 | 1910 | -35 | 137 | 482 | 346 | 0.39 |
|  |  |  | B | 2687 | 947 | 2634 | -18 | 139 | 668 | 347 | 0.35 |
| DS07 |  | Cluj Napoca to Oradea | R | 28 | 21 | 1 | 24 | 4 | 12 | 9 | 0.73 |
|  |  |  | A | 454 | 137 | 444 | 24 | 63 | 121 | 93 | 0.30 |
|  |  |  | B | 1215 | 475 | 1200 | 9 | 63 | 198 | 334 | 0.39 |
|  |  |  | C | 229 | 224 | 229 | -14 | 0 | 37 | 174 | 0.98 |
| DS08 |  | Stamora Moravita to Oradea via Timisoara | R | 166 | 58 | 0 | 149 | 17 | 54 | 4 | 0.35 |
|  |  |  | A | 260 | 163 | 235 | 163 | 49 | 313 | 37 | 0.63 |
|  |  |  | B | 393 | 180 | 367 | 163 | 62 | 297 | 83 | 0.46 |
| DS09 |  | Oradea to Cluj via Baia Mare and Satu Mare | R | 117 | 15 | 4 | 110 | 3 | 13 | 1 | 0.12 |
|  |  |  | A | 845 | 212 | 822 | 110 | 83 | 266 | 116 | 0.25 |
| DS10 |  | Bucharest to Giurgiu | R | 39 | 434 | 0 | 25 | 14 | 399 | 35 | 11.18 |
|  |  |  | A | 105 | 440 | 105 | 25 | 25 | 437 | 52 | 4.20 |
|  |  |  | B | 207 | 553 | 207 | 26 | 25 | 445 | 160 | 2.68 |
| DS11 |  | Craoiva to Calafat | R | 13 | 46 | 1 | 11 | 1 | 37 | 9 | 3.65 |
|  |  |  | A | 123 | 111 | 123 | 11 | 14 | 115 | 22 | 0.91 |
|  |  |  | B | 252 | 161 | 252 | 11 | 14 | 124 | 63 | 0.64 |
| DS12 |  | Freight test | R | 2 | -12 | 0 | 0 | 2 | -17 | 5 | - |
| DS99 |  | Combined test | A | 9594 | 9842 | 9303 | 1271 | 881 | 7959 | 3744 | 1.03 |
|  | 1 Note that for the PVC, PVB and BCR calculations some costs are computed as negative benefits |  |  |  |  |  |  |  |  |  |  |

Source: AECOM forecasting model

The table above splits the costs and benefits of each test into three categories, infrastructure, passenger and freight. Infrastructure costs relate to the track condition, and therefore include any upgrades or renewals of the studied line. Similarly, costs and benefits of freight and passenger trains account for the costs of operating the services, procurement of new rolling stock units, and the externalities and benefit to users of passenger and freight rail. The variant R generate lower costs and benefits than the variants A and B which involve more extensive rehabilitation works, with the latter returning a better value for money.

Tests DS06 show a reduction in passenger operator costs. The estimated operating cost saving can be attributed to the assumption that  newer EMU trains over the appraisal period has been forecasted to exceed the costs of purchasing new units. Similarly, an operating cost saving has been also forecasted in test DS07C, where some diesel trains are replaced by electric units.  In Tests DS05A and DS05B, freight trains benefit from a shorter route between Bucharest and the Northwest via the new link between Valcele and Ramnicu Valcea. The model also forecasts a small dis-benefit to rail passengers in test DS05R. This is due to the importance of coordinating arrivals and departures at interchange stations. Freight traffic in test DS05A generate significant environmental dis-benefits by running diesel trains in a densely urbanised area, this negative impact is addressed in tests DS05B and DS05C with the electrification of the line.

The example presented in Figure 5.28 illustrates the forecast traffic changes for the combined scenario, which comprises the impact of including all the best performing options for each corridor. Passenger traffic is forecast to increase across the majority of the network, with the estimated rail patronage more than doubling compared with the reference scenario. The number of daily passengers between Bucharest and Ploiesti increases of least 25,000 in both directions, whilst there are increases of at least 10,000 between Timisoara and Arad, Bacau and Ploiesti, Brasov and Ploiesti and Craiova to Bucharest. The only parts of the network affected by a reduction in passengers include Iasi to Tecuci and Bistrita to Miercurea Ciuc, although these changes are caused by service improvements on parallel corridors.

Figure 5.29: Change in Passenger Demand versus the Reference Case- 2030 Combined Scenario

Source: AECOM forecasting model

The increase in passenger traffic is even more evident when the foreasted flow for the 2030 combined scenario is compared against the observed 2011 flows, as Figure 5.29 demonstrates. Core corridors experience a substantial passenger traffic increase, with flows to and from Bucharest facing the largest increases in absolute terms.

Figure 5.30: Change in Passenger Demand versus the base 2011 Case- 2030 Combined Scenario

Source: AECOM forecasting model

Freight traffic has been also forecast to rise about 20% in relative terms as shown in Figure 5.30.This increase would be achieved by restoring the rail network to design conditions, improving freight travel times considerably. Further demand growth could be delivered if new intermodal terminals were constructed and this is examined in Chapter 10. The corridors attracting most of the additional traffic are the lines from Bucharest to Arad via Craiova, Core Corridor Ten-T IX and the line Bucharest to Sibiu via the proposed new link between Valcele and Ramnicu Valcea.

Figure 5.31: Change in Freight Demand - Combined Scenario

Source: AECOM forecasting model

Detailed Results Analysis: Core TEN-T corridor IV-N and link to Cluj-Napoca to design speed (Test DS01A)

Proposal description: Upgrade programme for Core TEN-T corridor IV-N between Bucharest Nord and Arad /Hungarian border; and line 300 between Cluj-Napoca and Core TEN-T corridor IV-N. The sections Bucharest to Predeal, Sighisoara to Deva and Hungarian border to km 614 (Northwest of Lipova) are either already rehabilitated or under upgrade. This project includes: - Rehabilitation to design speed of corridors: 300 Bucharest Nord to Cluj-Napoca, 200A, 200 Hungarian border to Alba Iulia (some sections have been already rehabilitated). - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communications systems, ERTMS will be implemented on Core TEN-T corridor IV-N. - Improved station facilities at Bucharest, Ploiesti, Campina, Predeal, Brasov, Sigishoara, Medias, Blaj, Aiud, Campia Turzii, Cluj-Napoca, Alba Iulia, Simeria, Deva, Ilia, Radna and Arad. - Interregio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Bucharest to Cluj-Napoca (0.5 tph), Bucharest to Arad (0.5 tph), Bucharest to Brasov (1 tph), Deva to Cluj-Napoca (0.5 tph).

Figure 5.32: Description of Service Proposals – DS01A

Source: AECOM forecast

### Problems addressed: This intervention addresses the following issues: - Deteriorating track condition due to underspend in maintenance and renewals. Slow running speeds of passenger and freight trains between Bucharest and Cluj-Napoca, Bucharest and Arad and towards Hungary; - Delays due to poor infrastructure condition, with commercial speeds below design parameters between Predeal and Arad and Teius and Cluj-Napoca. Maximum speeds are as low as 45% of the design parameters (section Augustin – Racos); - Low rail market share on the Bucharest – Ploiesti – Brasov section, with rail not competitive versus road on the corridor; - Poor rolling stock and station facilities condition which is country wide issue; - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns which affects all services; - Poor reliability record and inefficient signalling systems which affect the wider network; - Limited capacity and obsolete signalling systems. Infrastructure between Bucharest and Brasov may not be able to cope with the forecast rail traffic after rehabilitation. A pilot ERTMS test planned on this line would increase capacity and improve safety.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network and infrastructure rehabilitation of corridors either previously restored or under upgrade.

Table 5.16: Summary of Infrastructure Costs and Operating Costs (DS01A)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 3,030 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signaling equipment New rolling stock Improved station facilities |
| OPEX | 1,935 | Additional train operating costs |
| Total | 4,965 | CAPEX + OPEX |

Source: AECOM analysis of data from CFR Marfa and CFR Calatori

### Outcomes

This intervention returns good value for money (BCR = 1.09), and a significant overall increase in rail passenger and freight traffic with an increase of 27% and 9% respectively, as shown in Table 5.17.

Table 5.17: Summary of Results

| Test code | DS01A |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +5,814 (+27%) |
| Overall inc. in Pass-km share (2030) | +1.7% |
| Overall inc. in Tonne-km (1000,s 2030) | +4,175 (+9%) |
| Overall inc. Tonne-km share (2030) | +1.2% |
| NPV € Mill (2014 Prices) | 183 |
| BCR | 1.09 |
| EIRR | 5.47% |
| Upgraded main line infrastructure (track-km) | 795 |
| Required rolling stock units | 25 |

Source: AECOM analysis of outputs from the National Transport Model

Figures 5.32 and 5.33 illustrate the forecast changes in passenger and freight traffic for 2030 assuming that the project operates from 2020.

Figure 5.33: Change in Passenger Demand – DS01A

Source: AECOM forecasts

As Figure 5.33 demonstrates, this project generates an increase in passenger demand on the Bucharest – Ploiesti – Brasov – Arad line. Patronage between Ploiesti and Brasov is expected to grow by +115%, and by up to +155% between Deva and Arad. Much of the new demand will use the line from Bucharest to Arad via Craiova and Timisoara. Demand between Craiova and Drobeta - Turnu Severin is forecast to drop by -40% as a result of longer distance passengers abstracted onto the corridor via Brasov.

Figure 5.34: Change in Freight Demand – DS01A

Source: AECOM forecasts

Freight traffic is also expected to grow overall, with a 60% increase in the number of tonnes transported between Brasov and Sighisoara. Similar to the forecast trend in the passenger market, some freight traffic is expected to transfer from the line via Craiova to the upgraded corridor via Brasov. The number of transported tonnes has been estimated to fall by -50%.

### Implementing organisation:

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years:

- Section Bucharest – Predeal completed.

- Sighisoara to Deva and Hungarian border to km 614 between 2014 and 2020.

- Other corridors on this test estimated 2020

Core TEN-T corridor IV-N and link to Cluj-Napoca to enhanced speed (Test DS01B)

### Proposal description Upgrade programme for Core TEN-T corridor IV-N between Bucharest Nord and Arad /Hungarian border; and line 300 between Cluj-Napoca and Core TEN-T corridor IV-N. Lines with design speed below 100 km/h upgraded to enhanced speed. The sections Bucharest to Predeal, Sighisoara to Deva and Hungarian border to km 614 (Northwest of Lipova) are either already rehabilitated or under upgrade. This project includes: - Rehabilitation to design speed of corridors: 300 Bucharest Nord to Cluj-Napoca, 200A, 200 Hungarian border to Alba Iulia (some sections have been already rehabilitated). - Rehabilitation to enhanced speed of sections with design speed below 100 km/h on the following corridors: 300 Bucharest Nord to Cluj-Napoca, 200A, 200 Hungarian border to Alba Iulia (Some sections have been already rehabilitated). - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communications systems, ERTMS will be implemented on Core TEN-T corridor IV-N. - Improved station facilities at Bucharest, Ploiesti, Campina, Predeal, Brasov, Sigishoara, Medias, Blaj, Aiud, Campia Turzii, Cluj-Napoca, Alba Iulia, Simeria, Deva, Ilia, Radna and Arad. - InterRegio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Bucharest to Cluj-Napoca (0.5 tph), Bucharest to Arad (0.5 tph), Bucharest to Brasov (1 tph), Deva to Cluj-Napoca (0.5 tph).

Figure 5.35: Description of Service Proposals – DS01B

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition due to underspend in maintenance and renewals. Slow running speeds of passenger and freight trains between Bucharest and Cluj-Napoca, Bucharest and Arad and towards Hungary. - Delays due to poor infrastructure condition, with commercial speeds below design parameters between Predeal and Arad and Teius and Cluj-Napoca. Maximum speeds are as low as 45% the design parameters (section Augustin – Racos). - Low rail market share on the Bucharest – Ploiesti – Brasov section since rail is not competitive versus road on the corridor. - Low design speeds of the sections Predeal to Brasov and Razboieni to Apahida. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems. Infrastructure between Bucharest and Brasov may not be able to cope with the forecast rail traffic after rehabilitation; a pilot ERTMS test planned on this line would increase capacity and improve safety.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network and infrastructure rehabilitation of corridors either previously restored or under upgrade.

Table 5.18: Summary of Infrastructure Costs and Operating Costs (DS01B)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 3,578 | Rehabilitation of track to provide current or enhanced design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment New rolling stock Improved station facilities |
| OPEX | 1,962 | Additional train operating costs |
| Total | 5,540 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns low value for money (BCR = 0.99), in spite of a significant overall increase in rail passenger and freight traffic of 30% and 9% respectively, as shown in Table 5.19.

Table 5.19: Summary of Results (DS01B)

| Test code | DS01B |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +6,374 (+30%) |
| Overall inc. in Pass-km share (2030) | 1.9% |
| Overall inc. in Tonne-km (1000,s 2030) | +4,189 (+9%) |
| Overall inc. Tonne-km share (2030) | 1.1% |
| NPV € Mill (2014 Prices) | -36 |
| BCR | 0.99 |
| EIRR | 4.92% |
| Upgraded main line infrastructure (track-km) | 795 |
| Required rolling stock units | 25 |

Source: AECOM analysis of outputs from the National Transport Model

Figures 5.36 and 5.37 illustrate the forecast changes in passenger and freight traffic for 2030 assuming that the project operates from 2020.

Figure 5.36: Change in Passenger Demand (DS01B)

Source: AECOM forecasts

As the image above demonstrates this project leads to an increase in passenger demand on the Bucharest – Ploiesti – Brasov – Arad line, slightly higher than for Test DS01A. Patronage between Ploiesti and Brasov is expected to grow by +130%, and by up to +155% between Deva and Arad. There is a significant element of demand attraction from the line Bucharest – Craiova – Timisoara – Arad as a result of the improved passenger services. Demand between Craiova and Drobeta – Turnu Severin has been forecast to plumb by -45%.

Figure 5.37: Change in Freight Demand (DS01B)

Source: AECOM forecasts

Freight traffic is also expected to grow overall, with a 60% increase in the number of tonnes transported between Brasov and Sighisoara. Similarly to the forecast trend in the passenger market, some freight traffic is expected to transfer from the line via Craiova to the upgraded corridor via Brasov. The number of transported tonnes has been estimated to fall by -50%.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

- Section Bucharest – Predeal completed.

- Sighisoara to Deva and Hungarian border to km 614 between 2014 and 2020.

- Other corridors on this test estimated 2020

Regular interval timetable on the line Bucharest to Constanta (Test DS02A)

### Proposal description Regular interval timetable on the line Bucharest to Constanta operated by new rolling stock. Although this line has been upgraded in the last decade it has been acknowledged that minor improvement works at stations may be required. This project includes: - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communications systems. - Improved station facilities at Bucharest, Lehliu, Ciulnita, Fetesti, Cernavoda, Medgidia and Constanta, some improvement works already carried out on this corridor. - InterRegio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Bucharest to Constanta (1 tph), which comprises a ‘fast’ and a ‘stopping’ service every two hours.

Figure 5.38: Description of Service Proposals (DS02A)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Low rail market share, rail is not competitive versus road on the corridor. In principle the existing infrastructure enables faster rail than car journeys between Bucharest and Constanta. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Operational gaps for maintenance tasks, this work should be carried out at night. - Poor reliability record and inefficient signalling systems, network wide issues. - Low frequency between Bucharest and Constanta.

### Undiscounted costs

The project will incur in the following costs, excluding steady state maintenance.

Table 5.20: Summary of Infrastructure Costs and Operating Costs (DS02A)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 31 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment New rolling stock Improved station facilities |
| OPEX | 255 | Additional train operating costs |
| Total | 286 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns very good value for money (BCR = 13.62). There is a 7% increase in rail passenger traffic for the network as shown in Table 5.21.

Table 5.21: Summary of Results (DS02A)

| Test code | DS02A |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +1,422 (+7%) |
| Overall inc. in Pass-km share (2030) | 0.4% |
| Overall inc. in Tonne-km (1000,s 2030) | -14 (+0%) |
| Overall inc. Tonne-km share (2030) | 0.0% |
| NPV € Mill (2014 Prices) | 255 |
| BCR | 13.62 |
| EIRR | 48.92% |
| Required rolling stock units | 5 |

Source: AECOM analysis of outputs from the National Transport Model

Figure 5.39 illustrates the forecast changes in passenger and freight traffic for 2030 assuming that the project is operational from 2020.

Figure 5.39: Change in Passenger Demand (DS02A)

Source: AECOM analysis of outputs from the National Transport Model

Passenger demand on the corridor grows significantly compared to the reference case, the forecast traffic increase on the Lehilu to Ciulnita section is +115%, this figure decreases to +80% on the Fetesti to Cernavoda section.It has been assumed that no significant freight travel time savings will be achieved.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented by 2020 if funding is secured.

Bucharest to Arad via Craiova to design speed (Test DS03A)

### Proposal description Upgrade programme for Core TEN-T corridor IV-S between Arad and Craiova and line 900 between Craiova and Bucharest. This test also contemplates a regular interval connection to Core TEN-T corridor IV-N via line 202 and includes: - Rehabilitation to design speed of corridor 900 between Bucharest and Timisoara and the section Timisoara to Arad. - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - Improved station facilities at Bucharest, Videle, Rosiori de Vede, Caracal, Craiova, Filiasi, Drobeta – Turnu Severin, Caransebes, Lugoj, Timisoara and Arad. - Interregio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Bucharest to Arad via Timisoara (0.5 tph), Bucharest to Craiova (1 tph), Bucharest to Deva via Craiova (0.5 tph). Services to Deva could be extended to Cluj-Napoca if rail patronage increases.

Figure 5.40: Description of Service Proposals (DS03A)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition due to underspend in maintenance and renewals. Slow running speeds of passenger and freight trains on Core TEN-T corridor IV-S. - Delays due to poor infrastructure condition and speed restrictions. Commercial speeds are below design parameters on many sections of the line (Bucharest to Videle, Olteni to Rosiori de Vede Nord ...) - Low rail market share on the Bucharest – Craiova section. There is suppressed demand between these cities due to the poor passenger service. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems in many stations along the corridor.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.22: Summary of Infrastructure Costs and Operating Costs (DS03A)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 2,440 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment New rolling stock Improved station facilities |
| OPEX | 2,416 | Additional train operating costs |
| Total | 4,856 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns good value for money (BCR = 1.06), and a significant increase in rail passenger and freight traffic, of +23% and +9% respectively over the whole system, see table below:

Table 5.23: Summary of Results

| Test code | DS03A |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +4,946 (+23%) |
| Overall inc. in Pass-km share (2030) | 1.5% |
| Overall inc. in Tonne-km (1000,s 2030) | +4,101 (+6%) |
| Overall inc. Tonne-km share (2030) | 1.2% |
| NPV € Mill (2014 Prices) | 106 |
| BCR | 1.06 |
| EIRR | 5.32% |
| Upgraded main line infrastructure (track-km) | 875 |
| Required rolling stock units | 21 |

Source: AECOM analysis of outputs from the National Transport Model

Figures 5.41 and 5.42 illustrate the forecast changes in passenger and freight traffic for 2030 assuming that the project is in operation from 2020.

Figure 5.41: Change in Passenger Demand (DS03A)

Source: AECOM analysis of outputs from the National Transport Model

As the image above demonstrates this project leads to an increase in passenger demand on the Bucharest – Craiova – Timisoara – Arad line. Patronage between Rosiori de Vede and Caracal is expected to grow by +125%, and by up to +180% between Craiova and Drobeta - Turnu Severin. There is a significant element of demand attraction from the line Bucharest – Brasov - Arad as a result of the improved passenger services. Demand between Brasov and Sighisoara has been forecast to fall by -30%.

Figure 5.42: Change in Passenger Demand (DS03A)

Source: AECOM analysis of outputs from the National Transport Model

Freight traffic is also expected to grow overall, with an 80% increase in the number of tonnes transported between Craiova and Filiasi. Similarly to the forecast trend for the passenger market, some freight traffic will transfer from the line Bucharest – Brasov – Arad to the upgraded corridor, freight traffic on the former corridor is estimated to fall by -40%.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented between 2020 and 2030 if funding is secured.

Bucharest to Arad via Craiova to enhanced speed (Test DS03B)

### Proposal description Upgrade programme for Core TEN-T corridor IV-S between Arad and Craiova and line 900 between Craiova and Bucharest; this test also contemplates a regular interval connection to Core TEN-T corridor IV-N via line 202. This project includes: - Rehabilitation to design speed of corridor 900 between Bucharest and Timisoara and the section Timisoara to Arad. - Rehabilitation to enhanced speed of sections with design speed below 100 km/h on the Bucharest to Arad via Timisoara corridor. - Steady state maintenance of the core network (baseline assumption for Do Something tests). - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - Improved station facilities at Bucharest, Videle, Rosiori de Vede, Caracal, Craiova, Filiasi, Drobeta – Turnu Severin, Caransebes, Lugoj, Timisoara and Arad. - Interregio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Bucharest to Arad via Timisoara (0.5 tph), Bucharest to Craiova (1 tph), Bucharest to Deva via Craiova (0.5 tph). The service to Deva could be extended to Cluj-Napoca if rail patronage increases.

Figure 5.43: Description of Service Proposals (DS03B)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition due to underspend in maintenance and renewals. Slow running speeds of passenger and freight trains on Core TEN-T corridor IV-S. - Low design speeds of some sections on the stretch Drobeta-Turnu Severin to Caransebes. - Delays due to poor infrastructure condition and speed restrictions. Commercial speeds are below design parameters on many sections of the line (Bucharest to Videle, Olteni to Rosiori de Vede Nord ...) - Low rail market share on the Bucharest – Craiova section. There is a considerable degree of suppressed demand between these cities due to the poor passenger service. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems in many stations along the corridor.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.24: Summary of Infrastructure Costs and Operating Costs (DS03B)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 2,931 | Rehabilitation of track to provide current or enhanced design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment New rolling stock Improved station facilities |
| OPEX | 2,409 | Additional train operating costs |
| Total | 5,340 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns lower value for money than alternative DS03A (BCR =1.00), albeit with a greater increase in rail passenger and freight traffic than DS03A (27% and 9% respectively) across the network as shown in Table 5.25.

Table 5.25: Change in Passenger Demand

| Test code | DS03B |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +5,721 (+27%) |
| Overall inc. in Pass-km share (2030) | 1.7% |
| Overall inc. in Tonne-km (1000,s 2030) | +4,083 (+6%) |
| Overall inc. Tonne-km share (2030) | 1.2% |
| NPV € Mill (2014 Prices) | 5 |
| BCR | 1.00 |
| EIRR | 5.01% |
| Upgraded main line infrastructure (track-km) | 875 |
| Required rolling stock units | 21 |

Source: AECOM analysis of outputs from the National Transport Model

Figure 5.44 illustrates the forecast change in passenger traffic for 2030 assuming that the project is in operation from 2020.

Figure 5.44: Change in Passenger Demand (DS03B)

Source: AECOM analysis of outputs from the National Transport Model

As Figure 5.44 demonstrates, this project leads to an increase in passenger demand on the Bucharest – Craiova – Timisoara – Arad line. Patronage between Rosiori de Vede and Caracal is expected to grow by +125%, and by up to +180% between Craiova and Drobeta - Turnu Severin. There is a significant element of long distance trips between Bucharest and Arad via Brasov as a result of the improved passenger services. Demand between Brasov and Sighisoara is forecast to fall by -35%. Changes in freight flows on this test are similar to DS03A.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented between 2020 and 2030 if funding is secured.

Bucharest to Arad via Craiova and Filiasi to Simeria to design speed (Test DS03C)

### Proposal description Upgrade programme for Core TEN-T corridor IV-S between Arad and Craiova, line 900 between Craiova and Bucharest and the line Filiasi - Simeria; this test also contemplates a regular interval connection to Core TEN-T corridor IV-N via line 202. This project includes: - Rehabilitation to design speed of corridor 900 between Bucharest and Timisoara, the section Timisoara to Arad and the line Filiasi to Simeria. - Rehabilitation to design speed of the lines. - Steady state maintenance of the core network (baseline assumption for Do Something tests). - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - Improved station facilities at Bucharest, Videle, Rosiori de Vede, Caracal, Craiova, Filiasi, Drobeta – Turnu Severin, Caransebes, Lugoj, Timisoara, Arad, Targu Jiu, Petrosani and Simeria - Interregio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Bucharest to Arad via Timisoara (0.5 tph), Bucharest to Craiova (1 tph), Bucharest to Deva via Craiova (0.5 tph). The service to Deva could be extended to Cluj-Napoca if rail patronage increases.

Figure 5.45: Description of Service Proposals (DS03C)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition due to underspend in maintenance and renewals. Slow running speeds of passenger and freight trains on Core TEN-T corridor IV-S. - Delays due to poor infrastructure condition and speed restrictions. Commercial speeds are below design parameters on many sections of the line (Bucharest to Videle, Olteni to Rosiori de Vede Nord ...) - Low rail market share on the Bucharest – Craiova section. There is a considerable degree of suppressed demand between these cities due to the poor passenger service. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems in many stations along the corridor.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.26: Summary of Infrastructure Costs and Operating Costs

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 3,217 | Rehabilitation of track to provide current or enhanced design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment New rolling stock Improved station facilities |
| OPEX | 2,662 | Additional train operating costs |
| Total | 5,879 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns lower value for money than the alternative schemes (DS03A and DS03B), since the BCR is 0.89. The results summary is presented in Table 5.27.

Table 5.27: Summary of Results (DS03C)

| Test code | DS03C |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +5,526 (+25%) |
| Overall inc. in Pass-km share (2030) | 1.5% |
| Overall inc. in Tonne-km (1000,s 2030) | +4,625 (+7%) |
| Overall inc. Tonne-km share (2030) | 1.2% |
| NPV € Mill (2014 Prices) | -334 |
| BCR | 0.89 |
| EIRR | 4.16% |
| Upgraded main line infrastructure (track-km) | 875 |
| Required rolling stock units | 21 |

Source: AECOM analysis of outputs from the National Transport Model

Changes in freight and passenger flows on this test are similar to DS03A.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented between 2020 and 2030 if funding is secured.

Core TEN-T corridor IX and links Buzau to Galati and Pascani to Suceava to design speed (Test DS04A)

### Proposal description Upgrade programme for Core TEN-T corridor IX between Ploiesti and Iasi, Pascani to Suceava of line 500 and Buzau to Galati of lines 702 and 700. This project includes: - Rehabilitation to design speed of: Core TEN-T corridor IX between Ploiesti and Iasi, line 500 between Pascani and Suceava and line 700 between Buzau and Galati. - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communication systems to increase safety and efficiency. - Improved station facilities at Bucharest, Ploiesti, Buzau, Ramnicu Sarat, Focsani, Marasesti, Adjud, Bacau, Roman, Pascani, Dolhasca, Veresti, Suceava, Faurei, Braila, Galati, Targu Frumos and Iasi. - Interregio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Bucharest to Galati (0.5 tph), Bucharest to Iasi via Bacau (0.5 tph), Bucharest to Suceava (0.5 tph), Bucharest to Focsani (0.5 tph), Suceava to Iasi (0.5 tph)

Figure 5.46: Description of Service Proposals (DS04A)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition due to underspend in maintenance and renewals. Slow running speeds of passenger and freight trains between Bucharest and the East / Northeast Romania. - Delays due to poor infrastructure condition, commercial speeds below design parameters along the considered corridors. For example, the maximum speed between Ploiesti and Suceava is limited to 80 km/h, 40 km/h below design parameters. - Low rail market between Bucharest and major cities on corridor 500, rail is not competitive versus road on the corridor. - Passenger trains travelling between Bucharest and Iasi need to either change traction before joining corridor 600, or run as diesel units between Bucharest and Tecuci. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.28: Summary of Infrastructure Costs and Operating Costs (DS04A)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 3,366 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment New rolling stock Improved station facilities |
| OPEX | 839 | Additional train operating costs |
| Total | 4,204 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns good value for money with a BCR of 1.23. The scheme also generates a significant increase in rail passenger and freight traffic of 32% and 5% respectively, as shown in Table 5.29.

Table 5.29: Summary of Results (DS04A)

| Test code | DS04A |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +6,783 (+32%) |
| Overall inc. in Pass-km share (2030) | 2.0% |
| Overall inc. in Tonne-km (1000,s 2030) | +2,159 (+5%) |
| Overall inc. Tonne-km share (2030) | 0.6% |
| NPV € Mill (2014 Prices) | 529 |
| BCR | 1.23 |
| EIRR | 6.15% |
| Upgraded main line infrastructure (track-km) | 1,260 |
| Required rolling stock units | 28 |

Source: AECOM analysis of outputs from the National Transport Model

Figures 5.47 and 5.48 illustrate the forecast changes in passenger and freight traffic for 2030 assuming that the project is in operation from 2020.

Figure 5.47: Change in Passenger Demand (DS04A)

Source: AECOM analysis of outputs from the National Transport Model

One of the main outputs from this test is the significant increase in the overall passenger demand, with the upgraded corridor experiencing a 115% growth in total demand between Buzau and Focsani, 180% between Faurei and Galati, 130% between Pascani and Iasi and 85% from Pascani to Suceava. Since the upgraded corridor becomes the faster route to the North East (mainly to Iasi), line 600 loses about 45% of its passenger traffic compared with the reference scenario.

Figure 5.48: Change in Freight Demand (DS04A)

Source: AECOM analysis of outputs from the National Transport Model

Core Corridor Ten-T IX also attracts significant freight demand, with a 175% increase between Buzau and Focsani and 165% from Bacau to Pascani. Similar to the change in passenger flow, the number of tonnes transported between Barlad and Vaslui is expected to decline by 50%.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented by 2030 if funding is secured.

Line Bucharest to Pitesti to design speed and new link Videle - Ramnicu Valcea (Test DS05A)

### Proposal description Upgrade programme for the line Bucharest to Pitesti to design speed and new rail link between Videle and Ramnicu Valcea, this project includes 4 local stations on the new link. Passenger services on this test are extended to Ramincu Valcea and Sibiu. This project includes: - Rehabilitation to design speed of corridor 901 between Bucharest and Pitesti. - New rail link between Valcele and Ramnicu Valcea, some infrastructure elements already exist on the line. - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - Improved station facilities at Bucharest Nord, Titu, Gaesti, Pitesti, Valcele, Ramnicu Valcea Nord, Talmaciu and Sibiu. - InterRegio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Bucharest to Ramnicu Valcea (0.5 tph), Bucharest to Sibiu via Pitesti (0.5 tph).

Figure 5.49: Description of Service Proposals (DS05A)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition due to underspend in maintenance and renewals. Slow running speeds between Bucharest and Pitesti. - Poor rail connectivity of passenger trains between Bucharest and Ramnicu Valcea / Sibiu. - No direct route available for freight trains travelling from Constanta towards the Northwest and central Europe. - Delays due to poor infrastructure condition, commercial speeds below design parameters between Chitila and Ghergani. - Low rail market share on the Bucharest to Pitesti section, rail is not competitive versus road on the corridor, there is a motorway parallel to the rail line. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.30: Summary of Infrastructure Costs and Operating Costs (DS05A)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 854 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment New rail link New rolling stock Improved station facilities |
| OPEX | 116 | Additional train operating costs |
| Total | 970 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns very low value for money (BCR = 0.01), and a sensible increase in rail passenger and freight traffic, of +6% and +3% respectively, see table below. It is to notice that the significant local pollution generated by diesel freight trains drives the poor results of this test.

Table 5.31: Summary of Results (DS05A)

| Test code | DS05A |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +1,253 (+6%) |
| Overall inc. in Pass-km share (2030) | 0.4% |
| Overall inc. in Tonne-km (1000,s 2030) | +1,435 (+3%) |
| Overall inc. Tonne-km share (2030) | 0.5% |
| NPV € Mill (2014 Prices) | - 605 |
| BCR | 0.01 |
| EIRR | - 11.32% |
| Upgraded main line infrastructure (track-km) | 320 |
| Required rolling stock units | 8 |

Source: AECOM analysis of outputs from the National Transport Model

Figures 5.50 and 5.51 illustrate the forecast changes in passenger and freight traffic for 2030 assuming that the project is in operation from 2020.

Figure 5.50: Change in Passenger Demand (DS05A)

Source: AECOM analysis of outputs from the National Transport Model

This project leads to an increase in passenger traffic travelling between Bucharest and the Pitesti / Ramnicu Valcea / Sibiu area. Patronage between Bucharest and Pitesti grows by 130%, whilst traffic west of Pitesti rises by up to 10-15 times compared with the Reference scenario. Traffic on routes parallel to the upgraded corridor is expected to fall slightly as demand is abstracted from other parallel corridors.

Figure 5.51: Change in Freight Demand (DS05A)

The new link constructed attracts significant freight traffic since it becomes the more direct route between Bucharest and central Romania. Freight demand on the section Bucharest to Pitesti grows sharply by about +320% at the expense of the Brasov corridor.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented by 2020 if funding is secured.

Line Bucharest – Pitesti to design speed, new link Videle - Ramnicu Valcea and line electrification (Test DS05B)

### Proposal description Upgrade programme for the line Bucharest to Pitesti to design speed and new rail link between Videle and Ramnicu Valcea, this project includes 4 local stations on the new link. Passenger services on this test are extended to Ramnicu Valcea and Sibiu. This project includes: - Rehabilitation to design speed of corridor 901 between Bucharest and Pitesti. - New rail link between Valcele and Ramnicu Valcea, some infrastructure elements already exist on the line. - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - Electrification of the lines: Bucharest to Pitesti, Pitesti to Ramnicu Valcea (via the new link), Ramnicu Valcea to Sibiu, Sibiu to Core Ten-T corridor IV-N (Southwest of Alba Iulia) and corridor 208. - Improved station facilities at Bucharest Nord, Titu, Gaesti, Pitesti, Valcele, Ramnicu Valcea Nord, Talmaciu and Sibiu. - InterRegio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Bucharest to Ramnicu Valcea (0.5 tph), Bucharest to Sibiu via Pitesti (0.5 tph).

Figure 5.52: Description of Service Proposals (DS05B)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition due to underspend in maintenance and renewals. Slow running speeds between Bucharest and Pitesti. - Poor rail connectivity of passenger trains between Bucharest and Ramnicu Valcea / Sibiu - No direct route available for freight trains travelling from Constanta towards the Northwest and central Europe. - Delays due to poor infrastructure condition, commercial speeds below design parameters between Chitila and Ghergani. - Low rail market share on the Bucharest to Pitesti section, rail is not competitive versus road on the corridor, there is a motorway parallel to the rail line. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems. - Significant local emissions forecast in test DS05A.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.32: Summary of Infrastructure Costs and Operating Costs (DS05B)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 1,262 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment New rail link Line electrification New rolling stock Improved station facilities |
| OPEX | 622 | Additional train operating costs |
| Total | 1,884 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns a good value for money with a BCR of 1.61. Rail passenger and freight traffic increases by 6% and 2% respectively over the whole system. The change in passenger kilometres is comparable to the results shown for Test DS05A.

Table 5.33: Summary of Results

| Test code | DS05B |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +1,238 (+6%) |
| Overall inc. in Pass-km share (2030) | 0.4% |
| Overall inc. in Tonne-km (1000,s 2030) | +1,358 (+2%) |
| Overall inc. Tonne-km share (2030) | 0.4% |
| NPV € Mill (2014 Prices) | 659 |
| BCR | 1.61 |
| EIRR | 8.13% |
| Upgraded main line infrastructure (track-km) | 320 (line upgrade) 520 (electrification) |
| Required rolling stock units | 8 |

Source: AECOM analysis of outputs from the National Transport Model

Figures 5.53 and 5.54 illustrate the forecast changes in passenger and freight traffic for 2030 assuming that the project is in operation from 2020. This project leads to a marked increase in passenger traffic travelling between Bucharest and the Pitesti / Ramnicu Valcea / Sibiu area. Patronage between Bucharest and Pitesti grows by 130%, whilst traffic west of Pitesti is comparable to the results in test DS05A. Flows via Brasov are expected to reduce in response to the more convenient travel opportunities via Pitesti.

Figure 5.53: Change in Passenger Demand (DS05B)

Source: AECOM analysis of outputs from the National Transport Model

Figure 5.54: Change in Freight Demand (DS05B)

The new link constructed on this test attracts significant freight traffic to the new corridor. This new link will then become the more direct link between Bucharest and central Romaia, and this is reflected by the 320% increase in demand between Bucharest and Pitesti.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented by 2020 if funding is secured.

Line Bucharest – Pitesti and Pitesti to Craiova to design speed, new link Videle - Ramnicu Valcea and line electrification (Test DS05C)

### Proposal description Upgrade programme for tthe line Bucharest to Pitesti to design speed and new rail link between Videle and Ramnicu Valcea, this project includes four local stations on the new link. Passenger services on this test will be extended to Ramnicu Valcea and Sibiu. This project includes: - Rehabilitation to design speed of corridor 901. - New rail link between Valcele and Ramnicu Valcea, although some infrastructure elements already exist on the line. - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - Electrification of the lines between Bucharest and Pitesti, plus Pitesti to Ramnicu Valcea (via the new link), Ramnicu Valcea to Sibiu, Sibiu to Core Ten-T corridor IV-N (Southwest of Alba Iulia) and corridor 208. - Improved station facilities at Bucharest Nord, Titu, Gaesti, Pitesti, Valcele, Ramnicu Valcea Nord, Talmaciu and Sibiu. - InterRegio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Bucharest to Ramnicu Valcea (0.5 tph), Bucharest to Sibiu via Pitesti (0.5 tph), Pitesti to Craiova (0.5tph).

Figure 5.55: Description of Service Proposals (DS05C)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition due to underspend in maintenance and renewals. Slow running speeds between Bucharest and Pitesti. - Poor rail connectivity of passenger trains between Bucharest and Ramnicu Valcea / Sibiu. - No direct route available for freight trains travelling from Constanta towards the Northwest and central Europe. - Delays due to poor infrastructure condition, commercial speeds below design parameters between Chitila and Ghergani. - Low rail market share on the Bucharest to Pitesti section, rail is not competitive versus road on the corridor, there is a motorway parallel to the rail line. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems. - Significant local emissions forecast in test DS05A.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.34: Summary of Infrastructure Costs and Operating Costs (DS05C)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 1,542 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment New rail link Line electrification New rolling stock Improved station facilities |
| OPEX | 1,710 | Additional train operating costs |
| Total | 3,252 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns a good value for money with a BCR of 1.23, with a 7% and 4% increase in rail passenger and freight traffic respectively, as shown in Table 5.35.

Table 5.35: Summary of Results (DS05C)

| Test code | DS05C |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +1,580 (+7%) |
| Overall inc. in Pass-km share (2030) | +0.4% |
| Overall inc. in Tonne-km (1000,s 2030) | +1,358 (+4%) |
| Overall inc. Tonne-km share (2030) | +0.4% |
| NPV € Mill (2014 Prices) | 291 |
| BCR | 1.23 |
| EIRR | 6.29% |
| Upgraded main line infrastructure (track-km) | 469 (line upgrade) 520 (electrification) |
| Required rolling stock units | 8 |

Source: AECOM analysis of outputs from the National Transport Model

Changes in freight and passenger flows on this test are similar to DS05B.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented by 2020 if funding is secured.

Line Cluj-Napoca to Iasi to design speed (Test DS06A)

### Proposal description Upgrade programme for the rail line between Cluj-Napoca and Iasi, via Beclean and Suceava. This project includes: - Rehabilitation to design speed of: line 300 between Cluj-Napoca and Apahida, line 401 between Apahida and Dej, line 400 between Dej and Beclean, line 401 between Beclean and Ilva Mica, line 502 between Ilva Mica and Suceava, line 500 between Suceava and Pascani and line 606 between Pascani and Iasi. - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - Improved station facilities at Cluj-Napoca, Gherla, Dej Calatori, Beclean pe Somes, Salva, Ilva Mica, Vatra Dornei, Campulung Moldovenesc, Suceava, Veresti, Dolhasca, Pascani, Targu Frumos and Iasi. - Interregio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Cluj-Napoca to Iasi (0.5 tph).

Figure 5.56: Summary of Service Proposals (DS06A)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition due to underspend in maintenance and renewals. Slow running speeds between Cluj-Napoca and Iasi. - Delays due to poor infrastructure condition and existing speed restrictions, commercial speeds below design parameters on several corridors, for example between Dej Calatori and Beclean. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.36: Summary of Infrastructure Costs and Operating Costs (DS06A)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 2,808 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment New rolling stock Improved station facilities |
| OPEX | 70 | Additional train operating costs |
| Total | 2,878 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns poor value for money with a BCR of just 0.39. There is a 7% and 5% increase in rail passenger and freight traffic respectively as shown in Table 5.37.

Table 5.37: Summary of Results (DS06A)

| Test code | DS06A |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +1,390 (+7%) |
| Overall inc. in Pass-km share (2030) | 0.4% |
| Overall inc. in Tonne-km (1000,s 2030) | +1,220 (+3%) |
| Overall inc. Tonne-km share (2030) | 0.3% |
| NPV € Mill (2014 Prices) | -1,183 |
| BCR | 0.39 |
| EIRR | 0.29% |
| Upgraded main line infrastructure (track-km) | 740 |
| Required rolling stock units | 8 |

Source: AECOM analysis of outputs from the National Transport Model

Figures 5.57 and 5.58 illustrate the forecast changes in passenger and freight traffic for 2030 assuming that the project is in operation from 2020.

Figure 5.57:

Passenger traffic on the upgraded corridor is forecast to grow significantly, with an average demand increase between Apahida and Dej of 60%, 70% between Ilva Mica and Suceava and 50% between Suceava and Pascani. Corridors 400 and 501 will see a slight reduction in passenger traffic with some longer distance passengers abstracted to the more attractive route.

Figure 5.58: Change in Passenger Demand (DS06A)

Source: AECOM analysis of outputs from the National Transport Model

The most noticeable increase in freight demand is between Apahida and Beclean. Traffic via Gherla rises by almost 5,000 tonnes per day (+115%). Freight traffic also increases sharply on the rest of the upgraded corridor with a 45% rise between Floreni and Vatra Dornei, and a 15% between Veresti and Dolhasca.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented by 2030 if funding is secured.

Line Cluj-Napoca to Iasi to enhanced speed (Test DS06B)

### Proposal description Upgrade programme for the rail line between Cluj-Napoca and Iasi, via Beclean and Suceava. This project includes: - Rehabilitation to design speed of: line 300 between Cluj-Napoca and Apahida, line 401 between Apahida and Dej, line 400 between Dej and Beclean, line 401 between Beclean and Ilva Mica, line 502 between Ilva Mica and Suceava, line 500 between Suceava and Pascani and line 606 between Pascani and Iasi. - Rehabilitation to enhanced speed of sections with design speed below 100 km/h on the Cluj-Napoca to Iasi corridor. - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - Improved station facilities at Cluj-Napoca, Gherla, Dej Calatori, Beclean pe Somes, Salva, Ilva Mica, Vatra Dornei, Campulung Moldovenesc, Suceava, Veresti, Dolhasca, Pascani, Targu Frumos and Iasi. - Interregio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Cluj-Napoca to Iasi (0.5 tph).

Figure 5.59: Summary of Service Proposals (DS06B)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition due to underspend in maintenance and renewals. Slow running speeds between Cluj-Napoca and Iasi. - Delays due to poor infrastructure condition and existing speed restrictions, commercial speeds below design parameters on several corridors, for example between Dej Calatori and Beclean. - Low design speeds of some sections on the Cluj-Napoca to Iasi corridor. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.38: Summary of Infrastructure Costs and Operating Costs (DS06B)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 3,832 | Rehabilitation of track to provide current or enhanced design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment New rolling stock Improved station facilities |
| OPEX | 78 | Additional train operating costs |
| Total | 3,910 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns poor value for money with a BCR of 0.36. The proposals would generate a 10% and 3% increase in rail passenger and freight traffic respectively as shown in Table 5.39.

Table 5.39: Summary of Results (DS06B)

| Test code | DS06B |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +2,156 (+10%) |
| Overall inc. in Pass-km share (2030) | 0.6% |
| Overall inc. in Tonne-km (1000,s 2030) | +1,220 (+3%) |
| Overall inc. Tonne-km share (2030) | 0.3% |
| NPV € Mill (2014 Prices) | -1,740 |
| BCR | 0.35 |
| EIRR | -0.32% |
| Upgraded main line infrastructure (track-km) | 740 |
| Required rolling stock units | 8 |

Source: AECOM analysis of outputs from the National Transport Model

Figures 5.61 and 5.62 illustrate the forecast changes in passenger and freight traffic for 2030 assuming that the project is in operation from 2020.

Figure 5.60: Change in Passenger Demand (DS06B)

Source: AECOM analysis of outputs from the National Transport Model

Passenger traffic on the upgraded corridor is forecast to grow significantly, with an increase between Apahida and Dej of 85%, and 135% between Ilva Mica and Suceava. There is also a 70% increase between Suceava and Pascani. In response to the improved services between Cluj Napoca and Iasi, there is abstraction from corridors 400 and 501 with passengers diverting to a more convenient route.

Figure 5.61: Change in Freight Demand (DS06B)

Source: AECOM analysis of outputs from the National Transport Model

The most noticeable increase in freight demand in this scenario is expected between Apahida and Beclean whilst there is a rise of almost 5,000 tonnes per day via Gherla. Freight traffic also increases by 45% between Floreni and Vatra Dornei and 15% between Veresti and Dolhasca.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented by 2030 if funding is secured.

Line Cluj-Napoca to Oradea to design speed (Test DS07A)

### Proposal description Upgrade programme for corridor 300 between Cluj-Napoca and Oradea. This project includes: - Rehabilitation to design speed of corridor 300 between Cluj-Napoca and Oradea. - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - Improved station facilities at Oradea, Alesd, Huedin and Cluj-Napoca. - InterRegio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Cluj-Napoca to Oradea (1 tph).

Figure 5.62: Description of Service Proposals (DS07A)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition due to underspend in maintenance and renewals. Slow running speeds of passenger and freight trains between Cluj-Napoca and Oradea. - Delays due to poor infrastructure condition and speed restrictions. Commercial speeds below design parameters between Vadu Crisului and Oradea. - Low rail market share on the Cluj-Napoca and Oradea section, potential to increase rail patronage on the corridor significantly. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems, single track sections between Cluj-Napoca and Oradea limit the network’s capacity.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.40: Summary of Infrastructure Costs and Operating Costs (DS07A)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 658 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signaling equipment New rolling stock Improved station facilities |
| OPEX | 220 | Additional train operating costs |
| Total | 878 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns poor value for money (BCR = 0.30), and a minimal increase in rail passenger and freight traffic, of +1% and +1% respectively over the whole system, see table below:

Table 5.41: Summary of Results (DS07A)

| Test code | DS07A |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +180 (+1%) |
| Overall inc. in Pass-km share (2030) | 0.1% |
| Overall inc. in Tonne-km (1000,s 2030) | +456 (+1%) |
| Overall inc. Tonne-km share (2030) | 0.1% |
| NPV € Mill (2014 Prices) | -316 |
| BCR | 0.30 |
| EIRR | -0.21% |
| Upgraded main line infrastructure (track-km) | 270 |
| Required rolling stock units | 5 |

Source: AECOM analysis of outputs from the National Transport Model

The drawings below illustrate the forecast changes in passenger and freight traffic for 2030 assuming that the project is in operation from 2020.

Figure 5.63: Change in Passenger Demand (DS07A)

Source: AECOM analysis of outputs from the National Transport Model

Passenger traffic on the Oradea to Cluj-Napoca corridor increases in this scenario by about 60%, with some longer distance passengers from the Arad / Timisoara area towards Cluj switching to the upgraded corridor rather than the route via Deva.

Figure 5.64: Change in Freight Demand (DS07A)

Source: AECOM analysis of outputs from the National Transport Model

Freight traffic between Cluj-Napoca and Oradea is forecast to increase by about 50%, although this is offset by the abstraction from lines 200 and 413. The number of freight tonnes via Orastie falls by -10%, whilst traffic on corridor 413 falls by -5%.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented by 2030 if funding is secured.

Line Cluj-Napoca to Oradea to design speed plus electrification and line doubling (Test DS07B)

### Proposal description Upgrade programme for corridor 300 between Cluj-Napoca and Oradea. This project includes: - Rehabilitation to design speed of corridor 300 between Cluj-Napoca and Oradea. - Rehabilitation to enhanced speed of sections with design speed below 100 km/h on line 300 between Cluj-Napoca and Oradea. - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - Line doubling of single track sections. - Corridor electrification. - Improved station facilities at Oradea, Alesd, Huedin and Cluj-Napoca. - Interregio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Cluj-Napoca to Oradea (1 tph).

Figure 5.65: Description of Service Proposals (DS07B)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition due to underspend in maintenance and renewals. Slow running speeds of passenger and freight trains between Cluj-Napoca and Oradea. - Delays due to poor infrastructure condition and speed restrictions. Commercial speeds below design parameters between Vadu Crisului and Oradea. - Low design speeds of some sections of the Cluj-Napoca to Oradea line. - Low rail market share on the Cluj-Napoca and Oradea section, potential to increase rail patronage on the corridor significantly. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems, single track sections between Cluj-Napoca and Oradea limit to be upgraded to double track. - Line electrification to increase interoperability and reduce delays caused by traction changes.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.42: Summary of Infrastructure Costs and Operating Costs (DS07B)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 1,603 | Rehabilitation of track to provide current or enhanced design speeds Line doubling and electrification Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment New rolling stock Improved station facilities |
| OPEX | 394 | Additional train operating costs |
| Total | 1,997 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns poor value for money with a BCR of 0.39, and a minimal increase in rail passenger and freight traffic of just 2% and 1% respectively as shown in Table 5.43 below.

Table 5.43: Summary of Results (DS07B)

| Test code | DS07B |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +389 (+2%) |
| Overall inc. in Pass-km share (2030) | 0.1% |
| Overall inc. in Tonne-km (1000,s 2030) | +456 (+1%) |
| Overall inc. Tonne-km share (2030) | 0.1% |
| NPV € Mill (2014 Prices) | -740 |
| BCR | 0.39 |
| EIRR | -0.12% |
| Upgraded main line infrastructure (track-km) | 3301 |
| Required rolling stock units | 5 |
| 1 Including line doubling and electrification |  |

Source: AECOM analysis of outputs from the National Transport Model

Figures 5.66 and 5.67 illustrate the forecast changes in passenger and freight traffic for 2030 assuming that the project is in operation from 2020.

Figure 5.66: Change in Passenger Demand (DS07B)

Source: AECOM analysis of outputs from the National Transport Model

Passenger traffic travelling on the Oradea to Cluj-Napoca corridor increases in this scenario by about +90%, passengers from the Arad / Timisoara area switch to the upgraded corridor to travel to Cluj-Napoca and the Northeast.

Figure 5.67: Change in Freight Demand (DS07B)

Source: AECOM analysis of outputs from the National Transport Model

Freight traffic between Cluj-Napoca and Oradea is forecast to increase by about 50%, although this increase can be partially offset by the abstraction from lines 200 and 413. The number of freight tonnes transported through Orastie falls by -10%, whilst traffic on corridor 413 falls by 5%.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented by 2030 if funding is secured.

Line Timisoara – Oradea to design speed (Test DS08A)

### Proposal description Upgrade programme for Core TEN-T corridor IV-S between Timisoara and Arad and line 310 from Arad to Timisoara. This project includes: - Rehabilitation to design speed of corridors: 310 between Oradea and Arad and the section Arad to Timisoara of Core TEN-T corridor IV-S. - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - Improved station facilities at Timisoara, Arad, Chisineu-Cris, Salonta and Oradea. - InterRegio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Timisoara to Oradea (0.5 tph).

Figure 5.68: Description of Service Proposals (DS08A)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition and speed restrictions due to underspend in maintenance and renewals. Slow running speeds of passenger and freight trains between Timisoara and Oradea. - Delays due to poor infrastructure condition. Commercial speeds below design parameters over the whole length of the corridor (between Timisoara and Arad and Oradea and Arad). Significant time savings could be achieved. - Low rail market share of rail trips between Arad and Timisoara, potential to increase rail patronage on this section significantly. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.44: Summary of Infrastructure Costs and Operating Costs (DS08A)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 390 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment New rolling stock Improved station facilities |
| OPEX | 543 | Additional train operating costs |
| Total | 933 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns poor value for money with a BCR of 0.70. There is a small increase in rail passenger and freight traffic of 6% and 1% respectively, as shown in Table 5.45.

Table 5.45: Summary of Results

| Test code | DS08A |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +1,212 (+6%) |
| Overall inc. in Pass-km share (2030) | +0,4% |
| Overall inc. in Tonne-km (1000,s 2030) | +281 (+1%) |
| Overall inc. Tonne-km share (2030) | 0.1% |
| NPV € Mill (2014 Prices) | -98 |
| BCR | 0.63 |
| EIRR | 2.82% |
| Upgraded main line infrastructure (track-km) | 200 |
| Required rolling stock units | 6 |

Source: AECOM analysis of outputs from the National Transport Model

Figures 5.69 and 5.70 illustrate the forecast changes in passenger and freight traffic for 2030 assuming that the project is in operation from 2020.

Figure 5.69: Change in Passenger Demand (DS08A)

Source: AECOM analysis of outputs from the National Transport Model

This project is expected to generate significant passenger demand on the upgraded infrastructure, with demand more than doubling. The increased traffic forecast includes a 350% between Timisoara and Arad, Arad to Salonta (195%) with a 110% rise between Salonta and Oradea. Other routes also benefit Traffic between Caransebes and Lugoj is expected to increase by 30%. The model outputs indicate there will be some abstraction of longer distance trips between Arad – Deva – Cluj-Napoca to the enhanced route in response to the faster journey times.

Figure 5.70: Change in Freight Demand

Source: AECOM analysis of outputs from the National Transport Model

Freight demand has been forecast to increase. In particular, it has been estimated that freight traffic will grow by about 40% on the section between Arad and Timisoara and by 100% between Arad and Cluj-Napoca.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented by 2030 if funding is secured

Line Timisoara – Oradea to design speed (Test DS08B)

### Proposal description Upgrade programme for Core TEN-T corridor IV-S between Timisoara and Arad, line 310 from Arad to Timisoara, and line 922. This project includes: - Rehabilitation to design speed of corridors: 310 between Oradea and Arad, the section Arad to Timisoara of Core TEN-T corridor IV-S and line 922.. - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - Improved station facilities at Timisoara, Arad, Chisineu-Cris, Salonta and Oradea. - InterRegio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Timisoara to Oradea and Timisoara to Serbian border (0.5 tph).

Figure 5.71: Description of Service Proposals (DS08B)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition and speed restrictions due to underspend in maintenance and renewals. Slow running speeds of passenger and freight trains between Timisoara and Oradea. - Delays due to poor infrastructure condition. Commercial speeds below design parameters over the whole length of the corridor (between Timisoara and Arad and Oradea and Arad). Significant time savings could be achieved. - Low rail market share of rail trips between Arad and Timisoara, potential to increase rail patronage on this section significantly. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems. - Poor conectivity rail with Serbia.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.46: Summary of Infrastructure Costs and Operating Costs

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 531 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment New rolling stock Improved station facilities |
| OPEX | 687 | Additional train operating costs |
| Total | 1,217 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns poor value for money with a BCR of 0.46. There is a 6% increase in rail passenger traffic as shown in Table 5.47.

Table 5.47: Summary of Results (DS08B)

| Test code | DS08B |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +1,251 (+6%) |
| Overall inc. in Pass-km share (2030) | +0,4% |
| Overall inc. in Tonne-km (1000,s 2030) | +382 (+1%) |
| Overall inc. Tonne-km share (2030) | 0.1% |
| NPV € Mill (2014 Prices) | -213 |
| BCR | 0.46 |
| EIRR | 1.03% |
| Upgraded main line infrastructure (track-km) | 258 |
| Required rolling stock units | 6 |

Source: AECOM analysis of outputs from the National Transport Model

Changes in freight and passenger flows on this test are similar to DS08A.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented by 2030 if funding is secured.

Line Oradea to Satu Mare and Satu Mare to Cluj-Napoca to design speed (Test DS09A)

### Proposal description Upgrade programme for the rail line between Oradea and Baia Mare via Satu Mare, and Baia Mare to Cluj-Napoca via Dej. This project includes: - Rehabilitation to design speed of corridors: 300 between Oradea and Episcopia Bihor, 402 between Episcopia Bihor and Satu Mare, 400 from Satu Mare to Dej, 401 from Dej to Apahida and Corridor 300 between Apahida and Cluj-Napoca. - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - Improved station facilities at Oradea, Sacuieni, Valea lui Mihai, Carei, Satu Mare, Baia Mare, Jibou, Dej and Cluj-Napoca. - InterRegio regular interval timetable operated by new rolling stock. The proposed regular interval service pattern is: Baia Mare to Oradea via Satu Mare (0.5 tph) and Satu Mare to Cluj-Napoca via Baia Mare (0.5 tph).

Figure 5.72: Description of Service Proposals (DS09A)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition and speed restrictions due to underspend in maintenance and renewals. Slow running speeds of passenger and freight trains between Satu Mare and Cluj-Napoca. - Delays due to poor infrastructure condition and speed restrictions. Commercial speeds below design parameters between Apa and Baia Mare and Apahida to Dej Calatori. - Low rail market share of rail trips between Satu Mare and Baia Mare, significant potential to increase rail patronage on this section. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.48: Summary of Infrastructure Costs and Operating Costs (DS09A)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 1,208 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment New rolling stock |
| OPEX | 501 | Additional train operating costs |
| Total | 1,710 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns poor value for money (BCR = 0.25), and a small increase in rail passenger and freight traffic, of +6% and +1% respectively over the whole system, see table below

Table 5.49: Summary of Results (DS09A)

| Test code | DS09A |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +1,085 (+5%) |
| Overall inc. in Pass-km share (2030) | 0.3% |
| Overall inc. in Tonne-km (1000,s 2030) | +414 (+1%) |
| Overall inc. Tonne-km share (2030) | 0.1% |
| NPV € Mill (2014 Prices) | -633 |
| BCR | 0.25 |
| EIRR | -1.37% |
| Upgraded main line infrastructure (track-km) | 475 |
| Required rolling stock units | 7 |

Source: AECOM analysis of outputs from the National Transport Model

Figures 5.73 and 5.74 illustrate the forecast changes in passenger and freight traffic for 2030 assuming that the project is in operation from 2020.

Figure 5.73: Change in Passenger Demand (DS09A)

Source: AECOM analysis of outputs from the National Transport Model

Patronage between Baia Mare and Satu Mare is expected to increase substantially in this proposal with some flows expected to treble. Other notable changes include Satu Mare to Oradea (150%), Baia Mare to Dej (130%), and Dej to Cluj-Napoca (45%).

Figure 5.74: Change in Freight Demand (DS09A)

Source: AECOM analysis of outputs from the National Transport Model

Freight traffic mainly increases on the Alba Iulia to Dej section, particularly between Apahida and Dej which is expected to rise by 130%. An overall growth in rail freight traffic has been forecast as a result of the proposed network improvement. Some of the benefits affect a relatively wide geographic area.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented by 2030 if funding is secured.

Line Bucharest to Giurgiu via Gradistea to design speed (Test DS10A)

### Proposal description Upgrade programme for the line between Bucharest Nord and Giurgiu via Gradistea. This project includes: - Rehabilitation to design speed of corridor 902 between Bucharest and Giurgiu. - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - It has been considered that the Arges River bridge reconstruction is part of the Reference case scenario, a Do Minimum project. - Regio regular interval timetable operated by existing rolling stock. The proposed regular interval service pattern is: Bucharest to Giurgiu (0.5 tph).

Figure 5.75: Description of Service Proposals (DS10A)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition due to underspend in maintenance and renewals. Passenger services currently diverted via Videle. - Halts on corridor 902 have currently no direct rail connection to Bucharest. Travel times from Giurgiu to Bucharest are markedly higher on the route via Videle. - Low rail market share between Giurgiu and Bucharest, potential to increase rail patronage on the corridor significantly. - Poor rolling stock condition and frequency on the services between Gradistea and Giurgiu, country wide issue. - Limited capacity and obsolete signalling systems.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.50: Summary of Infrastructure Costs and Operating Costs (DS10A)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 144 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment |
| OPEX | 168 | Additional train operating costs |
| Total | 311 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns very good value for money with a BCR of 4.20. However, the changes in rail passenger and freight traffic are 3% and 1% respectively, as shown in Table 5.51.

Table 5.51: Change in Passenger Demand (DS10A)

| Test code | DS10A |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +545 (+3%) |
| Overall inc. in Pass-km share (2030) | 0.2% |
| Overall inc. in Tonne-km (1000,s 2030) | +123 (+0%) |
| Overall inc. Tonne-km share (2030) | 0.0% |
| NPV € Mill (2014 Prices) | 335 |
| BCR | 4.20 |
| EIRR | 14.67% |
| Upgraded main line infrastructure (track-km) | 95 |

Source: AECOM analysis of outputs from the National Transport Model

Figures 5.76 and 5.77 illustrate the forecast changes in passenger and freight traffic for 2030 assuming that the project is in operation from 2020.

Figure 5.76: Change in Passenger Demand (DS10A)

Source: AECOM analysis of outputs from the National Transport Model

The results of this test highlight the significant level of suppressed demand on the Bucharest to Giurgiu line. A frequent regular interval timetable will generate much higher patronage (185%) compared with the reference case.

Upgrading corridor 902 also leads to a small increase in freight tonnes with a 5-10% increase.

Figure 5.77: Change in Freight Demand (DS10A)

Source: AECOM analysis of outputs from the National Transport Model

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented by 2020 if funding is secured.

Line Bucharest to Giurgiu via Gradistea to design speed and line electrification (Test DS10B)

### Proposal description Upgrade programme for the line between Bucharest Nord and Giurgiu via Gradistea. This project includes: - Rehabilitation to design speed of corridor 902 between Bucharest and Giurgiu. - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Electrifaction of the corridor. - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - It has been considered that the Arges River bridge reconstruction is part of the Reference case scenario, a Do Minimum project. - Regio regular interval timetable operated by existing rolling stock. The proposed regular interval service pattern is: Bucharest to Giurgiu (0.5 tph).

Figure 5.78: Description of Service Proposals (DS10B)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition due to underspend in maintenance and renewals. Passenger services currently diverted via Videle. - Halts on corridor 902 have currently no direct rail connection to Bucharest. Travel times from Giurgiu to Bucharest are markedly higher on the route via Videle. - Low rail market share between Giurgiu and Bucharest, potential to increase rail patronage on the corridor significantly. - Poor rolling stock condition and frequency on the services between Gradistea and Giurgiu, country wide issue. - Limited capacity and obsolete signalling systems.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.52: Summary of Infrastructure Costs and Operating Costs (DS10B)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 226 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment |
| OPEX | 301 | Additional train operating costs |
| Total | 526 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns very good value for money with a BCR of 2.68 in spite of the minimal increase in rail passenger and freight traffic comprising 3% and 1% respectively as shown in Table 5.53.

Table 5.53: Summary of Results (DS10B)

| Test code | DS10B |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +545 (+3%) |
| Overall inc. in Pass-km share (2030) | 0.2% |
| Overall inc. in Tonne-km (1000,s 2030) | +123 (+0%) |
| Overall inc. Tonne-km share (2030) | 0.0% |
| NPV € Mill (2014 Prices) | 347 |
| BCR | 2.68 |
| EIRR | 11.96% |
| Upgraded main line infrastructure (track-km) | 95 |

Source: AECOM analysis of outputs from the National Transport Model

Changes in freight and passenger flows on this test are similar to DS10A.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented by 2020 if funding is secured.

Core TEN-T corridor IV-N section Craiova to Calafat to design speed (Test DS11A)

### Proposal description Upgrade programme for the line between Craiova and Calafat. This project includes: - Rehabilitation to design speed of corridor 912 between Craiova and Calafat. - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - Regio regular interval timetable operated by existing rolling stock. The proposed regular interval service pattern is: Craiova to Calafat (0.5 tph).

Figure 5.79: Description of Service Proposals (DS11)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition and speed restrictions due to underspend in maintenance and renewals. Travel times by rail markedly higher than by car. - Delays due to poor infrastructure condition. Commercial speeds below design parameters between Segarcea and Calafat. Although the design speed of that section is 80 km/h, passenger services between Segrarcea and Calafat are limited to 40 km/h. - Very low market share of rail trips between Craiova and Calafat, significant potential to increase rail patronage on this section. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems. - Slow speeds on the access to the new rail bridge over the Danube East of Calafat.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.54: Summary of Infrastructure Costs and Operating Costs (DS11A)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 183 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment Improved station facilities |
| OPEX | 55 | Additional train operating costs |
| Total | 239 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention offers poor value for money with a BCR of 0.91. The minimal increase in rail passenger traffic of 2% has contributed to this result, as shown in Table 5.55

Table 5.55: Summary of Results (DS11A)

| Test code | DS11A |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +363 (+2%) |
| Overall inc. in Pass-km share (2030) | 0.1% |
| Overall inc. in Tonne-km (1000,s 2030) | +92 (+0%) |
| Overall inc. Tonne-km share (2030) | 0.0% |
| NPV € Mill (2014 Prices) | -12 |
| BCR | 0.91 |
| EIRR | 4.47% |
| Upgraded main line infrastructure (track-km) | 115 |

Source: AECOM analysis of outputs from the National Transport Model

Figures 5.80 and 5.81 illustrate the forecast changes in passenger and freight traffic for 2030 assuming that the project is in operation from 2020.

Figure 5.80: Change in Passenger Demand (DS11A)

Source: AECOM analysis of outputs from the National Transport Model

The proposed upgrades will lead to a very high increase in passenger numbers since demand in the reference scenario is low as a result of the numerous speeds restrictions affecting the line. Demand could grow by 10-12 times on line 912.

Figure 5.81: Change in Freight Demand (DS11A)

Source: AECOM analysis of outputs from the National Transport Model

A modest increase in freight growth has been forecast after the line is upgraded. It has been acknowledged however freight demand may increase beyond the forecast levels if further enhancements are implemented.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented by 2020 if funding is secured.

Core TEN-T corridor IV-N section Craiova to Calafat to design speed and electrification (Test DS11B)

### Proposal description Upgrade programme for the line between Craiova and Calafat. This project includes: - Rehabilitation to design speed of corridor 912 between Craiova and Calafat. - Steady state maintenance of the core network (baseline assumption for all Do Something tests). - Improved signalling and communication systems to enhance running speeds and increase the corridor’s capacity. - Line electrification - Regio regular interval timetable operated by existing rolling stock. The proposed regular interval service pattern is: Craiova to Calafat (0.5 tph).

Figure 5.82: Description of the Service Proposals (DS11B)

Source: AECOM analysis of outputs from the National Transport Model

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition and speed restrictions due to underspend in maintenance and renewals. Travel times by rail markedly higher than by car. - Delays due to poor infrastructure condition. Commercial speeds below design parameters between Segarcea and Calafat. Although the design speed of that section is 80 km/h, passenger services between Segrarcea and Calafat are limited to 40 km/h. - Very low market share of rail trips between Craiova and Calafat, significant potential to increase rail patronage on this section. - Poor rolling stock and station facilities condition, country wide issue. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems, network wide issues. - Limited capacity and obsolete signalling systems. - Slow speeds on the access to the new rail bridge over the Danube East of Calafat.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network.

Table 5.56: Summary of Infrastructure Costs and Operating Costs

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 271 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment Improved station facilities |
| OPEX | 252 | Additional train operating costs |
| Total | 524 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns poor value for money with a BCR of 0.64. An increase in passenger numbers of 2% over the whole system has contributed to this outcome, as shown in Table 5.57.

Table 5.57: Summary of Infrastructure Costs and Operating Costs

| Test code | DS11B |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +364 (+2%) |
| Overall inc. in Pass-km share (2030) | 0.1% |
| Overall inc. in Tonne-km (1000,s 2030) | +92 (+0%) |
| Overall inc. Tonne-km share (2030) | 0.0% |
| NPV € Mill (2014 Prices) | -90 |
| BCR | 0.64 |
| EIRR | 2.01% |
| Upgraded main line infrastructure (track-km) | 115 |

Source: AECOM analysis of outputs from the National Transport Model

### Changes in freight and passenger flows on this test are similar to DS11A.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

It has been estimated that the project could be implemented by 2020 if funding is secured.

Combined strategy (Test DS99A)

### Proposal description This test combines the best performing solution for each corridor to assess synergies and incompatibilities. A combined scenario with more frequent regular interval timetables running on more corridors will enhance the role of rail interchanges, providing a more robust and reliable transport alternative to passengers. This project includes the upgrades contemplated for tests DS01A, DS02A, DS03A, DS04A, DS05B, DS06A, DS07A, DS08A, DS09A, DS10A and DS11A. DS07A has been selected as the preferred alternative over DS07B for its higher NPV, the modest traffic increase of this scenario does not justify the higher construction costs of test DS07B.

### Problems addressed This intervention addresses the following issues: - Deteriorated track condition due to underspend in maintenance and renewals. Slow running speeds of passenger and freight trains on several corridors in the network. - Delays due to poor infrastructure condition and speed restriction, commercial speeds are often below design parameters. - Low rail market share, marked road competition. - Poor rolling stock condition. - Inefficient timetable leading to low staff efficiency and poor rolling stock usage. Long turnaround times and irregular stopping patterns, network wide issues. - Poor reliability record and inefficient signalling systems. - Limited capacity and obsolete signalling systems.

### Undiscounted costs

It has been estimated that the project will incur in the following costs, excluding steady state maintenance of the core network and infrastructure rehabilitation of corridors either previously restored or under upgrade.

Table 5.58: Summary of Infrastructure Costs and Operating Costs (DS99A)

| Item | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| CAPEX | 13,679 | Rehabilitation of track to provide current design speeds Rehabilitation of power supply, including regenerative braking Rehabilitation of signalling equipment New rolling stock Improved station facilities |
| OPEX | 5,633 | Additional train operating costs |
| Total | 19,312 | CAPEX + OPEX |

Source: AECOM analysis of CFR Marfa and CFR Calatori data

### Outcomes

This intervention returns medium value for money with a BCR of 1.03. There is a high increase in rail passenger and freight traffic of 115% and 19% respectively as shown in Table 5.59.

Table 5.59: Summary of Results (DS99A)

| Test code | DS99A |
| --- | --- |
| Overall inc. in Pass-km (1000's 2030) | +24,289 (+115%) |
| Overall inc. in Pass-km share (2030) | 7.1% |
| Overall inc. in Tonne-km (1000,s 2030) | +8,525 (+19%) |
| Overall inc. Tonne-km share (2030) | 2.5% |
| NPV € Mill (2014 Prices) | 284 |
| BCR | 1.03 |
| EIRR | 5.14% |
| Upgraded main line infrastructure (track-km) | 46201 + 5202 |
| Required rolling stock units | 109 |
| 1Line upgrade 2Line electrification |  |

Source: AECOM analysis of outputs from the National Transport Model

Figures 5.84 and 5.85 illustrate the forecast changes in passenger and freight traffic for 2030 assuming that the project is in operation from 2020.

Figure 5.83:

Figure 5.84 illustrates the impact of the network upgrades and changes in passenger usage for the combined scenario. Patronage grows over the whole network, particularly between the capital and Ploiesti, Craiova, Constanta which have a travel time of 2-3 hours. Demand between Arad and Timisoara is also expected to rise with an increase in daily traffic of over 11,000 passengers and reflects the large potential travel market and the weak rail service at present.

Figure 5.84: Change in Freight Demand (DS99A)

The combined scenario will also see a significant rise in freight traffic, with more tonnes transported on most of the network compared with the reference scenario. The new link between Valcele and Ramnicu Valcea also provides a shorter connection for freight traffic from the Constanta and Galati areas to the Western Romania and other parts of Europe which attracts a significant number of freight trains.

### Implementing organisation

This scheme would be implemented by CFR SA and the Rail Operating Companies

### Implementation years

The implementation of this project is greatly constrained by the available budget.

MCA Analysis

As noted in Chapter 2.8, project prioritisation forms a critical step in producing the Master Plan, since the costs identified are greater than the available funding. This implies the necessity of ranking projects against a set of predefined evaluation criteria, which will apply a fair and consistent process to prioritise projects. Adding the funding restrictions to the list of ranked projects means the implementation calendar can be obtained. The following schemes have been prioritised using the multi-criteria assessment, as shown in Table 5.14. The overall approach to project selection and the way in which projects are combined to form the final scenarios was set out in Section 2.6. The funding allocations are discussed fully in Section 11.3 but are summarised here for the rail network in Table 5.14.

Table 5.60: Funding Allocation for Rail Investments (€M)

| 2014-2020 | 2021-2030 | 2014-2030 |
| --- | --- | --- |
| 4,316 | 11,077 | 15,393 |

Whilst the final grouping of schemes is informed by the model and financial analysis, it is not a purely mechanistic process. At each stage, the study team completed reality and sense checks to ensure the process did not lead to a poorly connected network or one that brought schemes forward for implementation in an illogical manner. In addition, the prioritisation criteria also took account of the project’s readiness and its maturity. This implies promoting schemes as priority interventions which have been developed to a more advanced stage of preparation, whilst taking account of funding availability in 2020 or 2030. To take account of these considerations, the following changes were completed to the prioritisation:

DS01A Bucharest to Hungary via Brasov + Teius to Cluj. Rehabilitation to design speed was promoted as a priority in 2020. This implies the completion of Corridor IV North rehabilitation, which is currently a high priority for Romania since it also meets EC objectives;

There are about 2.4 bn EUR available in 2020 which includes Phase 1 of DN04A between Bucharest and Iasi via Bacau plus Buzau to Galati and Pascani to Ukraine. Rehabilitation to design speed is recommended. This implies the rehabilitation to design speed of section between Bucharest and Sabaoani.

Summary of Interventions

Table 5.61: Results of the Multi-Criteria Analysis – Rail Schemes

|  | Code |  | Project Description | TEN-T | Score | EIRR | Cost (2014 prices) |  | Cumulated Cost |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Rail | DS10R | Emergency interventions for section Bucharest to Giurgiu via Gradistea | Core TEN-T link | 80.0 | 49.0% | 0.0 |  | 0.0 |
| 2 |  | DS11R | Emergency interventions for section Craiova to Calafat | Core TEN-T link | 50.5 | 23.0% | 1.5 |  | 1.5 |
| 3 |  | DS03R | Emergency interventions for section Bucharest to Arad via Craiova and Timisoara | Core TEN-T link | 49.2 | 21.9% | 73.2 |  | 74.7 |
| 4 |  | DS04R | Emergency interventions for section Bucharest to Iasi via Bacau + Buzau to Galati + Pascani to Ukraine | Core TEN-T link | 45.4 | 18.6% | 90.7 |  | 165.4 |
| 5 |  | DS06R | Emergency interventions for section Cluj-Napoca to Iasi | Core TEN-T link | 45.2 | 18.4% | 52.5 |  | 217.9 |
| 6 |  | DS10A | Bucharest to Giurgiu via Gradistea. Rehabilitation to design speed. | Core TEN-T link | 34.1 | 13.9% | 144.5 |  | 362.3 |
| 7 |  | DS03S | Freight test Filiasi - Tg Jiu | Comprehensive TEN-T link | 33.3 | 16.7% | 6.1 |  | 368.4 |
| 8 |  | DS01R | Emergency interventions for section Bucharest to Hungary via Brasov + Teius to Cluj | Core TEN-T link | 30.0 | 29.5% | 67.7 |  | 436.1 |
| 9 |  | DS04A | Bucharest to Iasi via Bacau + Buzau to Galati + Pascani to Ukraine. Rehabilitation to design speed. | Core TEN-T link | 27.3 | 7.9% | 2,360.2 |  | 2,796.3 |
| 10 |  | DS02A | Bucharest to Constanta. New rolling stock and re-timetabling | Core TEN-T link | 25.6 | 6.4% | 2,375.1 |  | 5,171.4 |
| 11 |  | DS01A | Bucharest to Hungary via Brasov + Teius to Cluj. Rehabilitation to design speed. | Core TEN-T link | 25.6 | 6.4% | 2,375.1 |  | 7,546.4 |
| 12 |  | DS03A | Bucharest to Arad via Craiova and Timisoara. Rehabilitation to design speed. | Core TEN-T link | 25.0 | 5.9% | 2,038.7 |  | 9,585.2 |
| 13 |  | DS11A | Craiova to Calafat. Rehabilitation to design speed. | Core TEN-T link | 24.0 | 4.7% | 160.9 |  | 9,746.0 |
| 14 |  | DS06A | Cluj-Napoca to Iasi. Rehabilitation to design speed. | Core TEN-T link | 24.0 | 5.0% | 1,087.0 |  | 10,833.0 |
| 15 |  | DS05B | Bucharest to Sibiu via Pitesti and Ramnicu Valcea. New link, rehabilitation to design speed and electrification. | Comprehensive TEN-T link | 17.7 | 8.3% | 1,130.4 |  | 11,963.4 |
| 16 |  | DS07B | Cluj-Napoca to Oradea. Rehabilitation to enhanced speed, line doubling and electrification. | Comprehensive TEN-T link | 15.1 | 6.0% | 456.0 |  | 12,419.4 |
| 17 |  | DS08A | Oradea to Timisoara. Rehabilitation to design speed. | Comprehensive TEN-T link | 14.0 | 5.0% | 234.0 |  | 12,653.4 |
| 18 |  | DS09A | Oradea to Cluj-Napoca via Satu Mare and Baia Mare. Rehabilitation to design speed. | Comprehensive TEN-T link | 14.0 | 1.9% | 549.0 |  | 13,202.4 |

Ports and Waterways

Ports and Waterways

- This chapter considers Romania’s ports and waterways as part of the National Transport Master Plan. This chapter is ordered in the following sections:

Existing Conditions

Strategic Objectives

Operational Objectives

Testing Interventions

Other Interventions

Summary

- Each section is furthered ordered by listing the ports, starting with Constanta and then running up the Danube from the mouth of the river to where the Danube is no longer in Romania. Consideration is then given to the waterways including the Danube itself as well as the Danube-Black Sea Canal.

- AECOM has compiled this chapter using information from a number of sources including the Ministry of Transport as well as using outputs from consultation with industry stakeholders including port operators, administrators and shipping companies.

Existing Conditions

Over 318 million tonnes of freight was lifted in 2011. Figure 6.1 shows that of this a large amount of water based freight occurs both maritime (12%) and on the River Danube and canal systems (9%). This shows that water freight has an important role in the transportation of mostly bulk freight in Romania, mainly associated with traditional industries.

Containerisation of freight in Romania is currently just 4%. This is at a low level and it could be reasonably expected for this rate to increase over the next 20 years to a Western European average of around 12%. Containerisation will be prompted by rising labour costs and the need to reduce the cost of handling products in the supply chain. It remains to be seen whether water freight can offer competitive services for domestic flows of containers in the future. This subject is discussed in more detail in the intermodal section.

Source: INSE and AECOM

Figure 6.1 – Modal Market Share of Freight Movements in Romania 2011

Source: APDF Giurgiu, APDM Galati, ACN

Figure 6.2 – Tonnes Handled by Romania’s Ports in 2011

Figure 6.2 shows the major ports in Romania (listed in the following section) and demonstrates the volume of freight that they handled in 2011. Figure 6.2 also shows the major commodity handled by the particular ports in 2011. This shows that some ports such as Galati and Braila are diversified as the major commodity handled by the port (in this case iron ore and cereals respectively) does not dominate the total tonnes handled. However at other ports such as Tulcea and Basarabi the major commodity dominates the overall total.

- Ports

Constanta

As Romania’s major seaport Constanta is the most developed and offers the widest range of facilities. The port handled 55 million tonnes in 2013 and had 14,066 vessel movements of which 34% were maritime-related and 66% on to the river network (9,233). 64% of the port volume consists of dry bulk and cereals have recently been the most significant individual commodity at the port. Constanta also handles around 10 million tonnes of liquid bulk and around 4 million tonnes of general cargo. In addition, Constanta is the largest container port in the Black Sea mainly due to DP World’s Constanta South Container Terminal which opened in 2004 and in 2012 handled 97% of container movements at the port. Although Constanta is a major regional container port, it is not one the top 20 container ports (by volume) in Europe. It handled 661,000 TEU (Twenty foot equivalent units) in 2013, although the peak year for containers was 2007 when the port handled 1.41 million TEU. The ongoing economic conditions, particularly the low rates being paid for container movements, have affected shipping lines to the extent that fewer shipping lines serve the port and shared services arrive with containers for several shipping lines.

Constanta has a large hinterland and acts as a transhipment point for many goods. Several other European ports already provide good links to Central Europe but there is scope for Constanta to increase its throughput provided that road, rail and water links are improved. Being further east and geographically closer to China by several days sailing time than northern European ports means Constanta could attract market share from other ports if the economies of scale and shipping rates are attractive. Constanta is strategically situated to potentially feed freight into the heartland of Central and Eastern Europe but due to the poor intermodal infrastructure it is not possible to fully capitalise on this opportunity.

The port has completed several infrastructure projects in recent years such as Rehabilitation of the Breakwaters of Constanta’s Port (€70 million); New Container Terminal on Pier II South Constanta Port ($90 million); Barge Terminal (€24 million), Constanta Port Environment and Infrastructure Project (€22 million); these were financed mainly by IFIs (EIB, Phare Programme, JBIC, EBRD) and co-financed by the state budget and the company’s own resources.

Ongoing developments include four main projects financed by the Structural Funds of European Union and co-financed by the State Budget through the Sectoral Operational Programme in Transports 2007- 2013. These are the extension of the North Breakwater by 1050 m, (€136 million); the road bridge over the Black Sea Danube Canal and related road access infrastructure (€30 million); the development of the railway capacity in the river-maritime sector (€17 million) and the Southward Extension of the Lighter Berth in the Port (€5million).

Constanta has berths dedicated to a range of commodities. Many of these berths are well utilised but the number of containers the port can handle is not becoming of its status as a major container port.

Regaining market share and thereby container volumes by reassigning port facilities to container movements and regenerating older infrastructure should be considered a priority for Constanta. The port has commissioned its own Masterplan in 2013 aimed at identifying major investment plans, for example a €300m development at the south side, a plan for the development of a LPG/oil terminal (depending on private investor input) and other developments such as an ongoing dredging programme, and highway improvements including widening the main internal roads from two to four lanes. This will address the problems of Port Infrastructure and services in greater detail.

Problem: Constanta has some inadequate and old infrastructure that is unsuitable for handling new flows, including containers. It also could improve its connectivity.

Proposed Solution: Build a new container terminal at Constanta (III & IVS)

The south of Constanta Port offers development potential for a container terminal, with the main advantage conferred by larger depths of water for berths. Ship size is growing rapidly and at the time of writing 19,000 TEU ships are now in use and they are only able to serve ports with the deepest water. A range of full and empty container handling should be developed in accordance with traffic projections to ensure Constanta competes with other major container ports. Whilst current capacity at Constanta is sufficient it is considered inadequate to accommodate growth in the longer term. Any such solution should be phased according to market conditions. The plan is to invest around €18 million in an extra rail terminal plus other berth improvements. The volume of containers handled in 2007 was 1.41 million TEUs and this is known to be near current maximum capacity, approximately 1.5 million. So the port reached 94% of its theoretical capacity in 2007.  Although capacity is sufficient for meeting the immediate demand of 700,000 TEUs, for the following seven reasons it is expected that the volume will grow and additional capacity will be needed by around 2020 or soon after:

Containerisation in Romania is currently just 4% and at a low level although it is expected that this will grow over the next 20 years to a Western European average of around 12%. This growth will be prompted by rising labour costs and the need to reduce the cost of handling products in the supply chain. The cost of handling containers is cheaper and quicker than traditional methods.

Romania did suffer with the economic recession and this was particularly manifested in much lower imports. With the economy growing again and with more economic buying power it is expected that volumes of imports and hence the number of containers will grow again.

Road and rail improvements made to Romania’s transport network over the next decade will improve journey times to the hinterland making transport costs lower and more competitive. The faster journey times will encourage more shippers to choose routes to and through Romania. Currently transit freight to many landlocked countries such as Hungary and Austria is being routed through Mediterranean and Adriatic ports (e.g. NAPA ports) in preference to Black Sea ports. The most surprising fact is that many shippers use non-Romanian ports to import goods for destinations in Western Romania such as Arad. During consultation we learned of one shipper that was asked if it could guarantee a journey time by rail of 25 hours between Constanta and Budapest. As it could not guarantee this, the trade was completely lost to Romania. This situation will change as new roads, railways and systems are introduced.

Improvements to the internal road and rail facilities and terminal arrangements at the Port of Constanta will increase efficiency, thereby lowering costs and as a result encourage freight to return to the port. There have been bottlenecks caused by poor infrastructure or antiquated administrative procedures which delay goods handling unnecessarily.

Problems with political instability in Ukraine and Russia are likely to affect shipper’s decisions in that rather than using Black Sea ports in these countries, it would instead be prudent to use Romanian ports instead.

Competition in the rail market, and privatisation of CFR Marfa will stimulate the rail freight sector to be more competitive and offer a better and quicker intermodal service.

The Intermodal Terminal Strategy aimed to rehabilitate or completely rebuild inland terminals will provide freight forwarders with a sustainable, efficient option for moving freight inland. Having a network of over ten inland terminals is likely to stimulate a network of domestic and international rail freight services.

Sulina

Sulina is a comprehensive TEN-T port and is located on the mouth of the Sulina Channel at the Black Sea. The port provides an essential service to the residents for both passenger and freight services in the Danube delta region. Sulina port has a quay length of 5,940m and has four berths. However, it is mostly focused on the movement of passengers, handling very little cargo traffic. Sulina town can only be accessed by the port so the port handles a number of passenger ferry services.

There are ample berths available at Sulina and the low volume of freight means that there is not a capacity issue at this port.

No problems have been identified at the port

Tulcea

This is a major port with 41 berths (5 berths servicing freight traffic) and is designated as a comprehensive TEN-T port. It has eight gantry cranes (max 16t) and a total area 82,762m2 (open storage of 70,000m2). The port serves as a gateway to the Danube delta region and has passenger vessels and also serves the local industry. The port is dominated by the handling of raw mineral products (quarrying and gravel, gypsum, slag, salt) and is mainly involved in supplying the construction sector. As such the facilities it requires are based around quarry extraction and loading. Slightly further along the river is the Industrial Port of Tulcea. It was built in 1974 in order to provide raw materials for factories processing metal in Tulcea. The main activities are loading and unloading of various raw materials such as manganese, bauxite, iron ore, limestone, ferroalloys from maritime and river vessels.

Tulcea has the following utilisation:

- Table 6.1: Current and Forecast Utilisation at Tulcea

|  |  |  | Reference Case |  |
| --- | --- | --- | --- | --- |
| Terminal | 2011 |  | 2020 | 2030 |
| General | 7.97% |  | 11.00% | 11.36% |
| Bulk | 100.60% |  | 117.47% | 113.69% |
| Containers |  |  |  |  |
| Oil |  |  |  |  |
| Grain | 1.53% |  | 1.79% | 1.73% |
| Fertiliser |  |  |  |  |
| Cement |  |  |  |  |
| Coal |  |  |  |  |
| Steel |  |  |  |  |

The table shows that Tulcea’s bulk handling terminal was fully utilised in 2011. AECOM’s Model forecasts that future demand for this terminal will exceed capacity.

Tulcea has berths available for bulk and general cargo but it does not have terminals that specialise in handling specific commodities such as grain.

Tulcea is surrounded by arable farmland producing grain.

Problem: The operating potential of cereals available in large quantities in the Dobrogea area is not exploited to the full partly due to a lack of specialised grain handling facilities at the port. Such grain flows are currently transported by trucks on  journeys to Constanta.

Proposed Solution: Development of a general cargo terminal through modernising the infrastructure and creating berths which can also handle grain would enable all flows produced in the area to be handled by the port and transported by river to the export markets. This would allow grain produced in the region to be handled by the port. Making the terminal capable of handling general cargo increases its flexibility and increases potential profitability of Tulcea.

Tulcea is situated in the Danube Delta and there are some communities located between it and Sulina. Tulcea is an important river-sea port for passenger traffic. The port does have a passenger terminal able to serve international traffic as well as the river traffic.

Problem: People living in the Sulina area itself are not well connected to the national transport network due to Sulina’s geography. Excessive crowding of the mooring means the port is operating at capacity and the lack of related services for ships and passengers means that the port has a poor service offer.

Proposed Solution: Improve the existing passenger ferry service running from Tulcea to Sulina and other minor ports as well as the mooring facilities and the services offered. Additional passenger ferry services would improve connectivity at Tulcea. The effect of this would be to reduce living costs for those people in the Danube Delta and increase the usage of Tulcea port. The connection with Braila and Galati will satisfy the Regional Development objective and offer some agglomeration benefits to Tulcea and Sulina. Additionally, the Danube Delta is a tourist attraction thanks to its unique biodiversity, and the ferry will be an environmentally-friendly way of transporting the increasing numbers of tourists to the area. By developing the overall port in conjunction with the construction of 3 berths for the direct loading of ships with bulk produce such as grain, the passenger berths would also benefit. This would also require the construction of suitable operating/loading facilities and some dredging to provide adequate depth. Connections to and from the port will also need to be upgraded.

Tulcea has been awarded €30 million from the Territorial Integrated Tool for rehabilitating and modernising the port which can be used to fund these projects.

Galati

Galati is the second largest port in Romania handling over 5 million tonnes in 2011, and is designated a core TEN-T port. The region also produces a lot of material which is suitable for transit. Galati handles grain, aggregate, steel, iron ore, coal and scrap iron. However, the current lack of intermodal facilities represents a major obstacle in the integration of port logistics in international flows. Additionally the port’s infrastructure and railway marshalling yard are old and inadequate for modern logistics needs and the connections to the national road and rail transport networks are slow and ineffective. These factors limit the tonnage handled by Galati, leading to the port’s current under-utilisation.

Galati port is:

The biggest river-maritime harbour in Romania

Is the second maritime gate of access from the Black Sea making a railway link from the Danube to landlocked countries of Central Europe

Provides transshipment services for all transport modes and temporary storage, customs services, free zone regime, provision and repair for ships, waste retrieval from ships

It is located on the eastern border of the EU next to Republic of Moldavia and is the only border crossing point transiting Republic of Moldavia to the south region of Ukraine

Benefits of road and rail connections with the hinterland (Rhine-Danube corridor and IX EU-PAN corridor including rail links from Republic of Moldavia and Ukraine, being the only port which can operate in direct transhipment from the maritime and fluvial ships on rail both with standard rail gauge (1435mm) and wide rail gauge (1530mm)

The mooring front has a length of 7065 mm organized in 56 river-maritime berths. The port enclosure surface is 864.131m2 out of which outdoor storage is 38.320 m2 and indoor storage of 7.200m2.

Port facilities include floating cranes and mobile equipment. Galati Port operates goods on external routes (import, export and transit) as well as internal routes (cabotage). Its location is extremely favorable for freights transshipped on long distances, being a multimodal hub serving the trade exchanges between the Black Sea countries and the Baltic Sea and Central Europe countries. Currently, the bulk freight (ore for Mittal Steel production, grains, scrap iron, building materials, etc.) is the majority of traffic share. General and standardized goods are sporadic, the main obstacles having a physical, technical and functional nature.Galati is potentially well located to handle increased flows and serve north east Romania and Moldova but has old infrastructure which in parts is over 50 years old and no longer complies with modern port requirements. The port suffers from poor connectivity and both of these factors are hindering its development. The ageing infrastructure was designed around the needs of the local steel industry which has declined over the last decade. With lower volumes of coal and iron ore the port needs to diversify to meet the requirements of modern industry. Analysis of the intermodal sector, particularly in the Black Sea basin indicates a potential to increase the demand for intermodal services at the port.

Galati has the following utilisation:

- Table 6.2: Current and Forecast Utilisation at Galati

|  |  |  | Reference Case |  |
| --- | --- | --- | --- | --- |
| Terminal | 2011 |  | 2020 | 2030 |
| General | 41.83% |  | 45.39% | 49.49% |
| Bulk | 100.79% |  | 109.37% | 119.26% |
| Containers |  |  |  |  |
| Oil |  |  |  |  |
| Grain | 30.21% |  | 32.78% | 35.74% |
| Fertiliser |  |  |  |  |
| Cement |  |  |  |  |
| Coal | 7.22% |  | 7.84% | 8.54% |
| Steel | 0.00% |  | 0.00% | 0.00% |
| Other | 0.96% |  | 1.90% | 2.62% |

The table shows that the bulk handling terminal is operating at capacity. AECOM’s model forecasts the terminal will be over utilised in the future. By contrast the large coal and steel handling terminals are handling very low volumes and this is forecast to continue. In theory if every berth could be used for any product then there were not be a problem but the reality is that many berths are specific to one traditional product and if this declines then the berth cannot handle other commodities without major refurbishment.

Galati handles a number of commodities including grain, coal and steel. Galati has a theoretical capacity of 7.8 million tonnes per year. However 6.8 million tonnes are for handling coal and steel associated with the declining steel industry in the area. As such Galati needs to refit and modernise the existing terminals and berths to increase capacity for new traffic.

Problem: Galati lacks intermodal facilities which is limiting the tonnage handled by the port. This means the port must currently overcome physical, technical and functional barriers in order to handle containers.

Proposed Solution: Build a new trimodal terminal

The current lack of intermodal facilities presents a major obstacle in the integration of port into international flows, port infrastructure and railway marshalling yard are old and inadequate. The Port’s connectivity with the hinterland, although assuring the interoperability of all modes of transport, do not meet the quality standards established by TEN-T Regulation no 1315/2013 for the central network infrastructure, for which Galati Port is included.The connections to the national road and rail transport networks are slow and ineffective with the road connections being provided by a single carriageway road and the marshalling yard and rail system serving the port require modernization (including electrification) and adaptation to operate 750m length trains. The physical barriers are the tight quays that prevent the direct mooring of ships.

A new trimodal terminal will facilitate direct transhipment of containers between vessels, trains and trucks. The port already has both rail systems (1435mm and 1520mm) which ensure interoperability of rail freight transport. The terminal will also be equipped with RO-RO and facilities for trucks and will offer some logistics services. The project will link with the port’s Free Trade Zone and take advantage of Galati’s proximity with Moldova and Ukraine. Galati has proposed a project called "Multimodal Platform Galati - removing major bottlenecks by upgrading existing infrastructure and providing missing links to the core network Rhine-Danube / Alps" which is targetting CEF funding and AECOM’s suggestion is that the tri-modal terminal acts as phase 1 of this project.

Problem: Galati’s infrastructure is old, set up to handle flows that witnessed significant decline from what was intended. This also means that the technology and processes used are outdated which leads to inefficiencies. The port also suffers from poor connectivity such as single carriageway roads that intersect with the port’s rail connections which also causes bottlenecks within the port. All of this is hindering the port’s development.

Proposed Solution: Modernisation of existing bulk terminals

Modernising the port and the existing bulk terminals so that a new terminal for palletised goods can be created to allow the port to improve its capacity to handle modern freight flows. This will also ensure that ships can moor directly which allows for smooth and efficient handling of goods from ship to shore (and vice versa). Modernising the port processes and systems and integrating them across each of the modes of transport will realise further efficiency.

Braila

This is a major port and designated as a comprehensive TEN-T port. It has twelve gantry cranes (max 16t), eight mobile cranes (max 25t), two floating cranes (max 30t), five reach stackers and conveyor belts and pneumatic equipment suitable for grain handling. The total port area is 398,630m2 (open storage of 250,350m2 and covered storage of 10,804m2) of which 22,750m3 is of grain and 6,000m3 animal fodder silos. The port is rail connected but this infrastructure is old and the roads leading to the port, particularly from its northern entrance are not ideal with poor signage and security. Braila handles raw mineral products, grain, wood products and fertiliser.

Braila has some berths that need upgrading and the road and rail network inside the port experiences congestion. The access road to the north of the port is of particularly poor quality with a lack of signage and good security. Upgrading the berths and port infrastructure will help to improve the port’s capacity and which in turn will help to relieve its current high utilisation. Braila’s berths are ageing and the port suffers from poor connectivity. The port has been highly utilised between 2007-2011. Braila has the following capacity:

Braila’s has a theoretical maximum capacity of 3.1 million tonnes per year. In 2008 and 2011 Braila handled 1.2 million. The port currently has ample capacity.

- Problem: Braila’s berths are ageing and the port suffers from poor connectivity

- Problem: The port is running out of capacity

- Proposed Solution: Both of these problems can be resolved by increasing capacity through berth improvement schemes. Existing, aged berths can be renovated and this will improve efficiency by allowing the berths to handle the goods that need to be handled. Note that the port is already undertaking schemes to address these issues. This Reference Case involves a berth improvement scheme as well as upgrading and improving the existing rail and road bottlenecks in the port to ensure best use is made of the increased capacity. Improving the berths will ensure that the berths are able to handle increased freight from the growth sectors. If the proposed interventions in the Reference Case prove, over time, to be insufficient for the growing needs of Braila port then additional capacity may be required.

Cernavoda

Cernavoda is a designated TEN-T core port and has three gantry cranes (max 16t), an open storage area of 20,000m2 and 2,000m2 of covered storage. APDF purchased a ship which is based at the port to handle ship waste, facilitate treatment and store water.

Cernavoda has the following utiilisation:

- Table 6.3: Current and Forecast Utilisation at Cernavoda

|  |  |  | Reference Case |  |
| --- | --- | --- | --- | --- |
| Terminal | 2011 |  | 2020 | 2030 |
| General | 109.86% |  | 64.63% | 68.13% |
| Bulk |  |  |  |  |
| Containers |  |  |  |  |
| Oil |  |  |  |  |
| Grain |  |  |  |  |
| Fertiliser |  |  |  |  |
| Cement |  |  |  |  |
| Coal |  |  |  |  |
| Steel |  |  |  |  |

The table shows that the port is currently over utilised and whilst in the future the port is forecast to have lower utilisation levels it is still well utilised and would benefit from further capacity.

Cernavoda can serve the grain and oil industries as well as handle bulk and general goods.

Cernavoda handles the unloading of raw minerals and wood (unloading 101,065 tonnes of the former and 24,396 tonnes of the latter in 2011, as opposed to loading 5,322 tonnes and 1,050 tonnes respectively) and as such does not require heavy investment in other commodity types, although the port should aim to be flexible in the range of commodities it handles.

The port is poorly equipped to handle other commodity types, which restricts its flexibility and ability to adapt to changes in circumstances. This is reflected in freight volumes: the port handled over 130,000 tonnes in 2011, but the berths were only 5% utilised.

Problem: Cernavoda is not able to handle further volumes of bulk freight

Solution: Develop the infrastructure

Infrastructure improvements to Cernavoda would consist of dredging across the mouth of the basin as well as modernisation and refurbishment of cargo berths within the basin, including the provision of utility networks.

Medgidia

Medgidia Port offers both industrial (17) and commercial (5) berths in addition to its passenger services. It also has 3 gantry cranes and 2 floating cranes and its main industries are agricultural operations and cement handling. However, there is room for improvement to the area for ships waiting for access, where the concrete has become damaged. Medgidia can handle both general and bulk cargo. Whilst the port has been busy in recent years this has been due to the construction of local infrastructure, which is temporary in its nature. Medgidia has a theoretical capacity of 5 million tonnes per year.

Port of Basarabi/Murfatlar

Basarabi/Murfatlar port is located on the Danube-Black Sea Canal, and offers 11 berths within its basin for industrial use, along with its single passenger berth. It also has a couple of gantry cranes and 3 mobile cranes for cargo handling.

Basarabi can handle both general and bulk cargo and these are transhipment services. Whilst the port has been busy in recent years this has been due to the construction of local infrastructure, which is temporary in its nature. Basarabi can handle up to 700,000 tonnes per year.

Problem: The infrastructure at Basarabi is poorly set up to handle modern freight requirements, has poor connectivity to the road network and is forecast low volumes for the future

Proposed Solution: Modernise and rehabilitate the infrastructure at the port as well as improve connections

Modernising and rehabilitating the infrastructure at the port would mean that the port could handle more and diversified freight in order to secure future loads.

Port of Calarasi

Calarasi port is accessed by a short navigation channel and is designated as a comprehensive TEN-T port. The operating area is split between 81,505m2 for Calarasi Commercial, 62,500m2 for Calarasi industrial and 5,091m2 for Calarasi Chiciu. The commercial port (on the Danube) handles mostly agricultural goods currently, whilst the industrial port (on the navigation channel) was designed to be a key shipment point for steel from the vast Combinatul Siderurgic Călărași – Siderca steel works. Now mostly abandoned, apart from a limited production area, this has not provided the tonnage for which the port was developed (although it has an estimated capacity of 470,000 tonnes per year), and extensive looting has resulted in significant damage to links with the under-utilised steel mill. APDF purchased a ship which is based at the port to handle ship waste, facilitate treatment and store water.

The effect of reduced output from the steel works has been to reduce the freight volumes passing through the port. As such the port is not achieving volumes for which the port was developed. This prevents the port’s full use for freight.

The berths originally created to handle steel are no longer handling any significant volumes of steel. This means that these berths are dormant and will not be able to handle other commodities and traffic.

Problem: The infrastructure at Calarasi is ageing and does not offer much in the way of added value services. Furthermore, much of the infrastructure was set up to handle steel from the nearby steel works. That steel works has now reduced much of its production and reduced its size. This means Calarasi is providing the wrong type of capacity, capacity which is no longer required.

Proposed Solution: Modernise and rehabilitate the infrastructure at the port

Modernising and rehabilitating the infrastructure at the port would mean that the port could handle more freight that is actually being moved as opposed to the defunct flows.

Port of Oltenita

This port is designated as a comprehensive TEN-T port and has three quay cranes and can accommodate barges of up to 2,000t and handles around 520,000t per year. Oltenita is currently undergoing quay works to build 200m of stepped frontage and has invested €25 million for processing passengers and bulk goods. A project called "Port infrastructure rehabilitation and modernization in Oltenita Port", which saw the port also acquire a ramp for very heavy items which serves the industrial area of Bucharest. Despite its close location to Bucharest, and unlike Giurgiu, Oltenita handles little to no container traffic, instead mostly handling raw mineral products and grain. The port has also been highly utilised for the period 2007-2011.

Oltenita has both general and bulk handling facilities. The port has a theoretical maximum capacity of 590,000 tonnes per year.

Problem: Oltenita is highly utilised and could run out of available capacity

Proposed Solution: Develop the bulk handling infrastructure

Oltenita needs additional handling infrastructure in order to handle additional freight. The berths need to be improved at Oltenita which will make them suitable to handle mixed traffic.

Port of Giurgiu

Giurgiu is designated as a core TEN-T port. It is located on the intersection between the River Danube and Corridor IX, which is the north-south route between the Baltics and Bulgaria, Greece and Turkey. The bridge between Giurgiu and Ruse in Bulgaria has been a key link for rail and road services over the years. The border crossing has seen significant volumes of goods traffic often foreign lorries and the route is in the top four Romanian rail freight border crossings. Giurgiu is also one of the closest Danube ports to Bucharest which makes it geographically important. The port has eight berths, two gantry cranes (max 16t), one mobile crane (max 50t), and one reach stacker. The port has a 17,000m2 freeport zone with a customs bonded warehouse and 7,200m2 of covered storage. It also has 10,000m2 container storage yard with container stuffing and stripping. APDF recently purchased a ship which is based at the port to handle ship waste, facilitate treatment and store water.

Giurgiu Port operates from 4 locations which offer dedicated port facilities:

Commercial Port "Ramadan": passenger harbour, plus berths handling agricultural products from vertical grain elevators, ballast (gravel) products, coal, general cargo.

"Canalul Plantelor / Sf. Gheorghe" Port: grain elevator of 10.000 tons, also handles aggregate and general cargo.

Cioroiu Port: oil terminal.

Giurgiu Free Zone Port: operating general goods and containers and a privately operated oil terminal.

Giurgiu has the following utilisation:

- Table 6.4: Current and Forecast Utilisation at Giurgiu

|  |  |  | Reference Case |  |
| --- | --- | --- | --- | --- |
| Terminal | 2011 |  | 2020 | 2030 |
| General | 58.33% |  | 84.58% | 126.48% |
| Bulk |  |  |  |  |
| Containers |  |  |  |  |
| Oil |  |  |  |  |
| Grain | 34.81% |  | 55.47% | 86.31% |
| Fertiliser |  |  |  |  |
| Cement |  |  |  |  |
| Coal |  |  |  |  |
| Steel |  |  |  |  |

The port has a well utilised general cargo handling terminal which is forecast to be over utilised by 2030.

Giurgiu has both bulk and general handling facilities and also has the ability to handle grain. There are no dedicated container facilities at Giurgiu. Giurgiu has a theoretical maximum capacity of 2.5 million tonnes per year.

Given its location near the land-based border crossing to Bulgaria, and its location near Bucharest (along with its size) this port is an important node for future freight flows on the Danube. Giurgiu has had mixed fortune since 2008. This is likely to be due to the economic crisis. The volume handled in 2011 was just 45% of the tonnage handled in 2007. Containers have a growing role at the port and grain and cereal products are key commodities too.

Problem: The general cargo handling facilities at Giurgiu are approaching capacity, despite the port being less than 7% utilised in 2011. As such there is underdeveloped general cargo handling infrastructure at Giurgiu.

Potential Solution: Infrastructure and modernisation works

Infrastructure and modernisation works set out in "D.A.N.U.B.E- Access network to the Danube” project at the port will see the port rearrange its infrastructure to accommodate modern logistics practices.

Proposed Solution: Build a new tri-modal terminal in the port of Giurgiu and connections with the hinterland

A new trimodal terminal at Giurgiu will increase the port’s ability to handle intermodal containers. This development will include railway sidings of sufficient capacity, good road connections and secure access to the port itself.

Port of Corabia

Corabia currently handles little freight traffic and is in a relatively severe state of dilapidation, with the exception of some grain and cereal traffic and storage facilities. This is reflected in the low volume of freight that the port handled in 2011, which is down almost 50% since 2007. However, it is well located as it is the only port of considerable potential between Giurgiu and Calafat, and has therefore a large hinterland which could be accessed from the port as it has retained a functioning rail connection. It is also over 227,000m2 offering plenty of space for further development and expansion.

Corabia has the following utilisation:

- Table 6.5: Current and Forecast Utilisation at Corabia

|  |  |  | Reference Case |  |
| --- | --- | --- | --- | --- |
| Terminal | 2011 |  | 2020 | 2030 |
| General | 18.84% |  | 58.21% | 107.16% |
| Bulk |  |  |  |  |
| Containers |  |  |  |  |
| Oil |  |  |  |  |
| Grain |  |  |  |  |
| Fertiliser |  |  |  |  |
| Cement |  |  |  |  |
| Coal |  |  |  |  |
| Steel |  |  |  |  |

Although the port is not currently well utilised the forecast shows that it will get busier in the future and will be over utilised by 2030.

The current state of Corabia means that it only has one berth which is set up to handle grain.

As such the scheme explored the potential for improving the quality of the berthing facilities at the port:

Problem: Corabia has antiquated infrastructure that is preventing effective and efficient flows of freight.

Proposed Solution: Modernisation and rehabilitation of infrastructure to improve efficiency which in turn will increase capacity

The scheme would involve works to improve the berths which are currently in poor condition. Some of the higher piers require some remedial structural work to ensure their continued use.

Port of Bechet

The port of Bechet currently has little regular freight traffic, although floating cranes are available if notified in advance for the loading/unloading of freight. The main user of Bechet is the RO-RO ferry across the Danube to Bulgaria. However, the port is located near to the industrial hub of Craiova, and as such may be a potentially useful terminal for the handling of abnormal loads.

Port of Calafat

Calafat is designated a core TEN-T port which has three berths, two land-based cranes, assorted floating cranes and can accommodate barges of up to 2,000t and handled 139,000t in 2011 which is up 228% from 2007.. It has 11,000m2 of storage and RO-RO facilities. APDF has a depollution ship that is based at the port and plans to acquire a vessel to handle ship waste, facilitate treatment and store water. Calafat is on the recently designated TEN-T Corridor IV South which makes use of the newly opened road and rail bridge linking Vidin in Bulgaria to Romania. This corridor has a significant potential as it could become a core route for freight flows from Germany and Central Europe to Turkey and could offer a real alternative to existing routes through Serbia, and indeed has seen significant recent growth in tonnage handled.

Calafat has the following utilisation:

- Table 6.6: Current and Forecast Utilisation at Calafat

|  |  |  | Reference Case |  |
| --- | --- | --- | --- | --- |
| Terminal | 2011 |  | 2020 | 2030 |
| General | 79.62% |  | 90.98% | 113.51% |
| Bulk |  |  |  |  |
| Containers |  |  |  |  |
| Oil |  |  |  |  |
| Grain |  |  |  |  |
| Fertiliser |  |  |  |  |
| Cement |  |  |  |  |
| Coal |  |  |  |  |
| Steel |  |  |  |  |

The table shows that Calafat is already well utilised and this utilisation is forecast to increase in the future. By 2030 the port is forecast to be over utilised.

Calafat is set up to handle bulk and general cargo although this is only in small volumes. Calafat port has a theoretical maximum capacity of 518,000 tonnes per year.  As such Calafat is not currently able to process large volumes of freight.

Due to existing growth, along with potential new flows and the new border crossing it is important to improve links with the port by other modes so that full realisation of these benefits can be achieved.

Problem: Calafat has underdeveloped infrastructure that is restricting its usable capacity

Proposed Solution: Develop bulk handling infrastructure which will increase the port’s capacity

Improve and modernise the handling infrastructure in order to be able to handle increasing freight volumes now that the port has new opportunities with the opening of the new bridge.

Port of Drobeta Turnu Severin

Drobeta Turnu Severin is designated a core TEN-T port and is the first port classified by AECOM as being of national importance when approaching Romania from Central Europe and is located on the TEN-T Corridor South. The port has seven berths, three gantry cranes (max 16t), one luffing-slewing crane (max 60t) and 13,725m2 of open storage. APDF purchased a ship which is based at the port to handle ship waste, facilitate waste treatment and store water.

Drobeta Turnu Severin is strategically located to act as a transhipment point on the Danube for traffic headed to northwest Romania and to cities such as Craiova. Furthermore, tonnage has proven resilient at the port despite the economic downturn and is now growing strongly once more.

Drobeta Turnu Severin needs to be able to handle this increase in freight tonnes by developing its infrastructure otherwise it will not be able to take competitive advantage. Whilst the port is only estimated to be 40% utilised in 2011, it has witnessed large growth in petroleum products and iron ore and needs to be able to handle these increased commodity volumes.

Drobeta Turnu Severin has the following utilisation:

- Table 6.7: Current and Forecast Utilisation at Drobeta Turnu Severin

|  |  |  | Reference Case |  |
| --- | --- | --- | --- | --- |
| Terminal | 2011 |  | 2020 | 2030 |
| General | 8.70% |  | 10.50% | 10.91% |
| Bulk | 88.59% |  | 106.93% | 111.08% |
| Containers |  |  |  |  |
| Oil |  |  |  |  |
| Grain |  |  |  |  |
| Fertiliser |  |  |  |  |
| Cement |  |  |  |  |
| Coal |  |  |  |  |
| Steel |  |  |  |  |
| Other | 56.13% |  | 66.28% | 69.11% |

The bulk handling terminal at Drobeta Turnu Severin is already well utilised and is forecast to be over utilised in the future.

Drobeta Turnu Severin is able to handle both general and bulk cargo. The port has a theoretical maximum capacity of 500,000 tonnes per year. However Drobeta Turnu Severin does not currently have infrastructure and berths dedicated to the handling of containers.

Problem: Drobeta Turnu Severin port does not have a dedicated container handling infrastructure which is causing inefficiency when handling containers.

Potential Solution: Build a new trimodal terminal

This would see the existing intermodal facilities developed to take advantage of its position in Romania and the increasing flows of intermodal containers

Problem: Drobeta Turnu Severin has old warehousing and storage facilities which are not suited to modern logistics practices. This affects the port’s usable capacity.

Potential Solution: Develop the infrastructure which will in turn increase efficiency and capacity

This scheme sees the improvement of facilities available at berths including handling equipment and provision of services. Improvement works would allow for the easier handling of freight. This would reduce handling costs and improve handling times thus increasing the attractiveness of the port.

Port of Orsova

This port has four gantry cranes (max 16t), 16,000m2 of open storage and 6,650m3 of grain silos. However the infrastructure is aged and in need of some modernisation and this is therefore considered under the following scheme. Building materials and mineral products are important components of freight trade at Orsova however no one industry is dominant nor is tonnage particularly consistent.

Orsova has the following utilisation:

- Table 6.8: Current and Forecast Utilisation at Orsova

|  |  |  | Reference Case |  |
| --- | --- | --- | --- | --- |
| Terminal | 2011 |  | 2020 | 2030 |
| General | 6.20% |  | 9.93% | 15.47% |
| Bulk | 44.46% |  | 71.20% | 110.86% |
| Containers |  |  |  |  |
| Oil |  |  |  |  |
| Grain |  |  |  |  |
| Fertiliser |  |  |  |  |
| Cement |  |  |  |  |
| Coal |  |  |  |  |
| Steel |  |  |  |  |
| Other | 0.00% |  | 1.83% | 6.48% |

The bulk handling terminal at Orsova is forecast to be over utilised by 2030.

Orsova port has a theoretical maximum capacity of 806,000 tonnes per year. However the port infrastructure is ageing and it is unable to achieve this in reality.

Problem: Orsova’s antiquated infrastructure is preventing effective and efficient flows of freight. This is a restriction on capacity.

Potential Solution: Modernisation of the port

This scheme would see the modernisation of the infrastructure through repairs and introduction of new facilities.

Port of Moldova Veche

Moldova Veche has been included for consideration with a possible improvement scheme as it is the first port reached in Romania when travelling downstream. The port handles a variety of freight, the type and volume of which can vary considerably. It has 3 berths, luffing-slewing cranes (max 36t) and 30,000m2 of open storage and 2,000m2 of covered storage. The port can provide container maintenance and repair, storage and container stuffing and stripping.

The port’s infrastructure, in particular the berths and water depth is not adequate for efficient and effective freight operations. This is reflective in the number of tonnes that the port has been handling and its low utilisation. Moldova Veche is a small port that handles a mix of freight that varies in volume from year to year.

Moldova Veche has the following utilisation:

- Table 6.9: Current and Forecast Utilisation at Moldova Veche

|  |  |  | Reference Case |  |
| --- | --- | --- | --- | --- |
| Terminal | 2011 |  | 2020 | 2030 |
| General |  |  |  |  |
| Bulk | 4.22% |  | 5.00% | 4.38% |
| Containers |  |  |  |  |
| Oil |  |  |  |  |
| Grain |  |  |  |  |
| Fertiliser |  |  |  |  |
| Cement |  |  |  |  |
| Coal |  |  |  |  |
| Steel |  |  |  |  |
| Other | 0.00% |  | 0.00% | 0.00% |

There are no current or future capacity issues identified at this port.

Moldova Veche is able to handle bulk and general cargo. The port has a theoretical maximum capacity of 374,000 tonnes however some of these berths are old and have antiquated equipment meaning achieving the maximum capacity is not possible.

Problem: The infrastructure at Moldova Veche is inadequate and inefficient. This is reflected by the low volumes handled

Potential Solution: Development of the infrastructure

This scheme would see the development of the infrastructure at Moldova Veche so that the port would be able to handle a wider variety of freight which will reduce the port’s reliability on volatile flow volumes.

- Local Ports

Port of Drencova

The port of Drencova is a small port, handling fewer than five vessels a year at its single berth. As such it is not seen as currently requiring further development or study for the masterplan.

Port of Gruia

Gruia mostly handles bulk ballast and gravel and is a small port (1,000m2) which does not require further assessment at this time due to its limited infrastructure and future development potential for alternative uses. Gruia handled no registered freight in 2011.

Port of Cetate

Similarly, Cetate is a port of limited current size (1,000m2) which handles bulk ballast and gravel from the dredging of the Danube and is currently not seen to require further development at this time. Cetate handled no registered freight in 2011.

Port of Turnu Magurele

Turnu Magurele is associated mostly with the chemical/fertiliser plant which is located adjacent to the port, handling both the chemical inputs and the fertiliser output of the plant. Its specialisation towards this industry and limited use for other purposes would suggest that further development should be pursued in collaboration with the fertiliser plant rather than solely through public funds.

Port of Zimnicea

The steel sector is one of the port’s key users, along with some ballast shipment. Due to its close working relationship with the steel industry, it is recommended that any further development should be pursued in collaboration with this sector rather than solely through public funds as its alternative uses are currently limited.

Port of Harsova

The port of Harsova has a single basin with 500m of sloped quay; which aside from passenger services is currently only used for shipment of sand extracted from the river bed. It is not felt that further development of this port is needed at the current time.

Port of Turcoaia

The port of Turcoaia is focused on the processing of construction stone from the Dantana Lui Manole quarry. As such, there is currently little scope for alternative development and so it is not considered further within the masterplan.

Port of Macin

Macin handles stone for the domestic construction industry (including road infrastructure works) and is focused entirely on the shipment of this; as such it requires no further consideration at the current time as it is not suitable for more general use.

Port of Gura Arman

Gura Arman is focused on handling stone from the Iacob-Deal quarry. As such, there is currently little scope for alternative development and so it is not considered further within the masterplan.

Port of Isaccea

Isaccea operates exclusively with the construction industry in terms of transporting and handling wood, stone and sand, in particular for road infrastructure construction. Whilst this is important within this sector it is not currently foreseen that it will need development for more general goods traffic and further assessment was therefore not undertaken.

Port of Mahmudia

Mahmudia is focused on the domestic shipment of chalk for use at Mittal Steel at Galati; with no need for alternative use and so its development is therefore not considered further.

Port of Ovidiu

Ovidiu is a port located on the Poarta Alba-Midia Branch of the Danube-Black Sea Canal. The port has two berths and handled 529,000 tonnes in 2011. No problems have been found at Ovidiu.

Port of Chilia Veche

Chilia Veche currently handles little or no commercial freight traffic, rather it is focussed on passenger traffic. There are limited facilities for the operation and temporary storage of grain. As such is not considered further in this masterplan.

Port of Fetesti

Fetesti port is on the Bratul Borcea part of the River Danube. The port is not a significant freight operation. No problems have been identified at Fetesti.

Port of Tisovita

Tisovita port is not a significant freight operation and did not handle any freight in 2010 and 2011. No problems have been identified at Tisovita.

Port of Rast

Rast port is on the River Danube in the shared Romanian-Bulgarian section. The port is a very small operation.

Port of Bazias

Bazias port is on the River Danube close to the border with Serbia. The port is a very small operation.

Port of Luminita

Luminita port is part of the Danube-Black Sea Canal authority’s (ACN) network of ports. It is close to Midia port (Constanta’s satellite port). Whilst Luminita handles some freight, this is mainly mineral products. Luminita’s proximity to Midia and Constanta means most freight generated in the area would be handled at these ports.

- Waterways

River Danube

The Danube is a Class VII river under UN categorisation. It flows for approximately 1,075km within Romania and hence is a significant natural transport corridor forming much of Romania’s southern border with Bulgaria. The Danube-Black Sea Canal directly links Romania’s major seaport Constanta to the Danube. The Danube carries 9% of all freight in Romania (source: INSSE, year 2011), in terms of tonnes of transported commodities. There are some major settlements and industries located along the Danube corridor but many major settlements are not located on the waterway network (including Bucharest) and much of the country is poorly connected to the river.

Danube – Black Sea Canal

The Danube-Black Sea Canal was created to provide a shorter link to the Black Sea from the Danube, thereby avoiding having to navigate the difficult Danube delta. The canal bifurcates with the main canal going south towards the port of Constanta at Agigea. The north canal reaches the Black Sea at Midia port.

The Danube-Black Sea Canal was designed to facilitate the transit of convoys comprising as much as 6 towed barges, up to 3,000 tonnes each (therefore up to 18,000 tonnes per convoy). Ships of up to 5,000 tonnes (as well as respecting the maximum dimensions) can pass through the canal.

The Danube-Black Sea Canal has handled the following domestic and international tonnages (Figure 6.3)

Figure 6.3 – Annual Water Freight Tonnes Handled by Danube-Black Sea Canal

A high proportion of the 2013 tonnes are international in nature and this reflects the fact that the Danube-Black Sea Canal (and the River Danube it connects) is an international waterway allowing landlocked countries access to the rest of the world. Tonnages have increased between 2009 and 2013 and this will be due to recovery from the economic crisis. The Danube-Black Sea Canal’s peak year was in 2005 when it handled over 15m tonnes. 2013 was its second busiest year on record (by tonnage handled).

Sulina Channel

The Sulina Channel (also referred to as the Sulina Branch) is located in the Danube Delta. The town of Sulina sits at the mouth of the Sulina Channel, on the Black Sea. Sulina is the shortest of the three main branches (the others being Chilia branch and Sfantul Gheorghe branch). Its short length means it is the preferred choice for navigation (navigation for major vessels is not recommended on the other channels) and Sulina has received dredging to ensure navigability.

Depths in the Sulina Channel range from 10m to 14m and repeated dredging allows vessels of maximum draft 7.32m to berth. Varying draught restrictions can be enforced and so vessel captains must check before using the channel. Ice may form during February and March and the port at Sulina is kept open by icebreakers.

Vessels are charged a transit fee (based on vessel weight) to use the Sulina Channel.

Future Trends in Water Freight Transport

Constanta

Figure 6.4 shows the future trends for Constanta port in terms of river freight

Figure 6.4 – Annual Water Freight Tonnes Handled by Constanta

Figure 6.4 shows that despite a slight drop in the number of tonnes handled by Constanta between 2011 and 2020, between 2020 and 2030 the number of water freight tonnes handled by the port is forecast to increase.

Danube Ports

Figure 6.5 shows that the future trends for the Danube ports that are part of the Primary Economic Network (see section 6.1.2 for further information on the Primary Economic Network).

Figure 6.5 – Annual Water Freight Tonnes Handled by Danube Ports that are part of the Primary Economic Network

Figure 6.5 shows that despite a slight drop in the number of tonnes handled by the Danube Ports between 2011 and 2020, between 2020 and 2030 the number of water freight tonnes handled by the ports is forecast to increase.

Existing Conditions of the Selected Ports and Waterways

The selected ports come from AECOM’s Primary Economic Network. AECOM defined the Primary Economic Network as a mechanism for where we believe investment should be focused. This network was devised using a number of criteria:

The existing designated TEN-T Ports

Existing and predicted future volume of tonnage

Connectivity with the rest of Romania

Connectivity with key border crossing points (for other modes)

Economically significant corridors, defined as those which carry high volumes of goods traffic

The selected Ports form a coherent network that provides convenient mooring and servicing facilities for river traffic. The ports are shown on Figure 6.6:

Figure 6.6: Romanian Water Freight Primary Economic Network

Note that the choice of Primary Economic Network ports does not mean that the remaining ports should be neglected. Many are tactical for one or two commodities and even those that that see little freight should be safeguarded for the future.

Tri-Modal Terminals

Currently there are a limited number of containers moving inland by water freight. There are only a few ports handling this type of cargo on a regular basis. Containerisation has been growing on a global basis and  growth is expected to continue in Romania. It is important for sustainability and environmental reasons that any growth is shared around the most appropriate mode of transport - road, rail and barge depending on customer requirements and suitability. To facilitate this it is important to have a network of river ports strategically located to act as tri-modal terminals. These terminals should be able to accommodate any combination of modal transfer between water, road and rail.

It is recommended that as well as having port terminals at Constanta, which is the main generator/attractor of maritime containers there should be intermodal terminals serving the east, central and western parts of Romania. This provides customers with an access point to the hinterland. These are explored fully below.

- East Romania

It is considered that the most likely point for a tri-modal terminal at the eastern end of the Danube is at the Port of Galati. The reasons are that this is already the biggest port on the River Danube in Romania and has the infrastructure necessary for a tri-modal terminal, which however requires modernisation and improvements. Galati Port has land available for refurbishment and expansion including the Free Zone and the Industrial Park for supporting logistics activities. 2 million people live within 100km of the port and it is the best located port to serve the North East which is the least prosperous part of Romania. Galati is the only port in Romania with both Russian and European gauge internal rail track and is well connected to serve Moldova and Ukraine. A terminal here could attract significant amounts of tonnage. In addition, due to the reduction in volume of commodities, traditionally handled at the port, the facility now needs restructuring to handle emerging flows.

AECOM recommend that the eastern tri-modal terminal is situated at Galati.

- Central Southern Romania

There are two ports well located to serve the southern central section of the Danube; Giurgiu and Oltenita. The Port of Giurgiu is on TEN-T Corridor IX which runs from the Baltics to Greece and Turkey. It is adjacent to the border crossing to Ruse and Bulgaria. Various routes within Bulgaria linking Ruse to their coastal ports and Sofia are being upgraded. Giurgiu already handles some containers but does not have modern, purpose built facilities for this, hence the potential scheme. Giurgiu is within an hour’s drive by truck to Bucharest on National road DN5 and as such has over 3 million people within a catchment area of 100km. Giurgiu is well located for freight and raw materials coming from the west along the Danube.

The Port of Oltenita is also within an hour’s drive by truck to Bucharest, on National road DN4 and similar to Giurgiu has over 3 million people within a catchment area of 100km. As the port is nearer to the larger feeder ports of Constanta and Galati than Giurgiu it has attracted some tonnage destined for the Bucharest area. The distance by barge between Oltenita and Giurgiu is around 100km or about 8 hours sailing time upstream and hence is likely to attract cheaper shipping rates from the east than Giurgiu. However the opposite is also true from the west. More time sensitive containers are likely to travel by road or rail in the future so barge traffic needs to cater for less urgent traffic.

AECOM recommend that the central tri-modal terminal is situated at Giurgiu.

- Western Romania

There are a number of ports which would provide a suitable geographic location for a western intermodal terminal; these are assessed briefly below, with a summary of useful details in Table 6.10.

- Table 6.10: Summary of Distances to Western Hubs from Selected Danube Ports

| Port Name | Timisoara |  | Craiova |  | Rail Connection |
| --- | --- | --- | --- | --- | --- |
|  | Distance | HGV Driving Time | Distance | HGV Driving Time |  |
| Orsova | 197km | 3h 00m | 144km | 2h 15m |  |
| Drobeta Turnu Severin | 223km | 3h 20m | 114km | 2h 00m |  |
| Calafat | 323km | 5h 00m | 90km | 1h 40m |  |

Orsova has both a large natural harbour and a rail connection, and whilst it handles a large range of different bulk materials, it currently handles little container tonnage. Whilst it is one of the better located ports for an intermodal terminal given the onward journey times, this location also means it is located above the Iron Gate II hydroelectric power plant, dam and border crossing, making it less attractive for traffic coming from the east and Constanta than ports below the Iron Gates, assuming that the vessel is not continuing upstream beyond Romania.

The port of Drobeta Turnu Severin is located downstream of the Iron Gate II dam and is located on Corridor IV (S) electrified rail route, as well as being on the E70 road corridor. The port is capable of being developed to handle a growing amount and range of tonnage. Although the port is surrounded by the city of Turnu Severin, there is room for rationalisation and re-arrangement of port facilities to create a suitable space for an intermodal terminal. The port is able to serve both Craiova and Timisoara.

Calafat, although currently a small port focusing on bulk transport has potential for growth following the construction of the Calafat-Vidin crossing of the Danube. This will be accompanied with an upgrade and rehabilitation of the currently, unelectrified rail link to Corridor IV (S) making this a potentially suitable location. However, it is envisaged that the port will continue to specialise in bulk goods, especially given that the port of Vidin across the Danube is developing a container terminal. This combined with its longer journey time to Timisoara, means that it is not recommended as a location for a western intermodal terminal.

AECOM propose that the western tri-modal terminal is situated at Drobeta Turnu Severin.

This would result in the following tri-modal terminals (Figure 6.7):

- Figure 6.7: Map of Tri-Modal Terminals in Romania (Current and Proposed)

There are some terminal owners or operators associated with ports and the following table gives the location of these terminals. These cater for a range of commodities and many specialise in particular goods such as oil and grain.

Table 6.11 Location of ports in Romania with private terminals

| Terminal | Location |
| --- | --- |
| Port of Constanta | Constanta |
| Port of Galati | Galati |
| Port of Tulcea | Tulcea |
| Port of Braila | Braila |
| Port of Giurgiu | Giurgiu |
| Port of Calarasi | Calarasi |
| Port of Oltenita | Oltenita |
| Port of Corabia | Corabia |
| Port of Drobeta Turnu Severin | Drobeta Turnu Severin |
| Port of Calafat | Calafat |
| Port of Orsova | Orsova |

River Danube

The River Danube has few standard physical pinchpoints such as locks. The only lock on the Romanian section of the Danube is near the Iron Gates and is located 1,890km from the mouth of the river. Between the Iron Gates to Braila the Danube is more than 2km wide with numerous islands. The navigation channel moves several times per year due to the currents and remediation work is often needed to maintain the minimum 2.5m depth. There can also be problems with the width of the navigation channel which sometimes necessitates a single file direction flow operation (as opposed to two way flow).

The Danube is not always navigable 365 days a year. Weather related factors such as drought or flood or ice do affect the river’s navigation and the amount of traffic it can process. Ice is a problem during the winter months and in 2012 three months were lost as the Danube froze over which prohibited navigation. There is no dedicated ice breaker on the Danube. Water level is also an issue with the depth varying throughout the year. One operator estimated that full and efficient operation is only possible on about 250 days a year which means for the remaining 100 days operators need to make contingencies to get their loads delivered, either through sub-optimally loaded barges or in worst case scenarios having to use other modes of transport to deliver the goods.

The UN defines the Danube as an international waterway and the draught required for such status is 2.5m, although 2.8m is preferable. Seven sections of the Danube regularly fall below 2.5m draught and navigation was impossible on the lower river for more than 38 days in September and October 2011 because of insufficient water levels. A barge requires a draught of 0.5m when empty and up to 3m when fully laden. This variance allows an operator to vary how much load their barge can carry depending on the water levels. The only constant is the pushers that require between 1.8-2m of water. Shallow waters can make it impossible to push 2 or 3 barge-width convoys so some operators will split the convoy into two and double back. When this happens in the Zimnicea sector, the 150km stretch that should normally be traversed in 6 hours can take between 1 and 2 days. There is also the issue of barges having enough navigable ‘fairway’ in order to be able to pass each other.

Operators want a guarantee of water depth of 2.5 meters because otherwise they are not able to operate. Work is being done to try and obtain this through intensive dredging (section Călărași-Brăila) and then this will be maintained which will cost less on an annual basis. Romania is currently spending €4m annually on dredging the fairway whereas Bulgaria only spends €100,000.

Some sections of the Danube are particularly vulnerable to shifting sands, especially in the Zimnicea sector. Annual maintenance through dredging does help but this is very expensive and cannot always maintain the desired 2.5m depth. More effective management of water on the secondary arms is thought to be needed to effectively combat this.

Figure 6.8 – Critical Navigation Points on the River Danube

The Danube has pinchpoints along its length where the fairway depth or width is reduced to below optimal standards (Figure 6.8 shows a snapshot of these). Low temperatures can cause the Danube to freeze and there is no dedicated ice breaking vessel for the Romanian section.

The maintenance budget Romania provides for the Danube is low compared to other countries which may be a contributory issue to the navigation problems on the Romania section.

As a consequence of the reduced navigability, vessels have had to make unscheduled stops and loads have sometimes been pilfered from them.

Danube – Black Sea Canal

The Danube – Black Sea Canal runs between Cernavoda on the River Danube to Constanta on the Black Sea. The Danube-Black Sea Canal was created to provide a shorter link to the Black Sea from the Danube, thereby avoiding having to navigate the difficult Danube delta. The canal bifurcates with the main canal going south towards the port of Constanta at Agigea. The north canal reaches the Black Sea at Midia port.

The Danube-Black Sea Canal was designed to facilitate the transit of convoys comprising as much as 6 towed barges, up to 3,000 tonnes each (therefore up to 18,000 tonnes per convoy). Ships of up to 5,000 tonnes (as well as respecting the maximum dimensions) can pass through the canal.

There are locks on the Canal at Cernavoda, Agigea, Navodari and Ovidiu. The Canal’s administrator ACN charges for transit of the Canal

Sulina Channel

The Sulina Channel forms one of the three main branches of the Danube as it passes through the Danube Delta into the Black Sea. Due to the natural flow of the Danube waters, along the course there is a continuous evolution of the morphology of the river. The branches, in cluding the Sulina Channel, are particularly susceptible to these changes.

The morphological changes can be caused by a combination of changes in land use, changes in water level, changes in water flow velocity, bow waves from passing ships and ice can accelerate its morphological evolution of the riverbed in some sectors. These phenomena have influenced the Sulina Channel especially after floods in 1970 and 1975 that destroyed almost all of the old bank protection.

Problem: Erosion of the banks of the Sulina Channel threatens the use of the channel and adjacent facilities. The material eroded from banks is deposited at the mouth of the sea, which led to the development of Sulina bar. The erosion also endangers large areas of the Danube Delta Biosphere Reserve.

Potential Solution: Introducing measures to restore and maintain their shores which are:

- - Stopping the erosion in some areas by protecting the banks of Sulina channel,

- - Stabilising and securing the navigability of the canal

- - Reducing the risk of flooding of coastal settlements and economic facilities along the canal

- - Improving environmental protection in the sector.

Adopting these measures will lead to greater safety of navigation by providing general stability of the canal. This is the second phase of an ongoing project on the Sulina Channel.

Summary

Table 6.12 summarises the identified problems for each of the ports.

- Table 6.12 – Summary of the Identified Problems at the Ports

| Port | Identified Problems |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Old or Undeveloped Infrastructure | Poor Connectivity | Unable to Handle New Flows | Lacking Intermodal Functionality | Lacking Capacity |
| Constanta |  |  |  |  |  |
| Tulcea |  |  |  |  |  |
| Galati |  |  |  |  |  |
| Braila |  |  |  |  |  |
| Cernavoda |  |  |  |  |  |
| Calarasi |  |  |  |  |  |
| Oltenita |  |  |  |  |  |
| Giurgiu |  |  |  |  |  |
| Corabia |  |  |  |  |  |
| Calafat |  |  |  |  |  |
| Drobeta Turnu Severin |  |  |  |  |  |
| Orsova |  |  |  |  |  |
| Moldova Veche |  |  |  |  |  |

Problems identified on the River Danube include a lack of a consistent navigation, a low maintenance budget and poor safety and security.

Strategic Objectives

Strategic Vision

The strategic vision for Romania for water freight can be defined between the ports and its waterways. The strategic vision will satisfy the following strategic objectives:

Economic Development

Economic Efficiency

Safety

Sustainability

Environmental Impact

For Romania’s ports the strategic vision is a core network of ports (called the Primary Economic Network – Figure 6.9) that strategically serves Romania with modern and efficient equipment and logistics practices. These ports have been chosen based on specified criteria such as their status as TEN-T ports, scale, current and potential for freight operations and location.

Figure 6.9 – the Primary Economic Network

For Romania’s port the strategic objectives that will be satisfied are:

Economic Development

Economic Efficiency

Sustainability

Environmental Impact

For Romania’s waterways the strategic vision is a network that provides 24/7 access to all waterways users. This will ensure a common service level that is of a high standard that gives operators and passengers and freight confidence to use the waterways. This will be achieved by improving and maintaining the fairway to the 2.5m depth required as well as investing in features that will reduce or eliminate any impact of avoidable instances of reduced navigation such as dedicated ice breaking vessels.

Operational Objectives

The water based operational objectives are:

OW1 – Improve availability of the Danube

OW2 – Improve Danube connectivity to reduce cost and time

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW5 – Improve coordination between levels of government and enable investment

OW6 – Review existing assets to focus intervention to increase capacity

OW7 – Reduce procedural delays to water freight

OW8 – Reduce number and cost of accidents on Danube

OW9 – Reduce emissions of CO2 and Sulphur Dioxide

OW10 – Increase the amount of freight moved by sustainable modes

OW11 – Make efficient use of port land and facilities

OW12 – Increase use of intermodal transport

Each intervention has been rated against the operational objectives to determine validity. This is demonstrated in Section 6.4.

Testing Interventions

This section considers the results from the National Transport Model on the interventions. Note that certain interventions were not able to be been tested by the model. It should be noted that whilst the certain interventions have been tested as part of the National Transport Master Plan, more detailed feasibility studies should be conducted for the interventions listed and tested against prevailing market conditions.

Where ports have multiple projects there may be scope for synergies such as cost. However such synergies will be planned through detailed feasibility studies. For the purposes of the Masterplan project costs have been considered in isolation.

Constanta:

Build a new container terminal at Constanta (III & IVS)

Proposal Description:

The south of Constanta Port offers development potential for a container terminal, with the main advantage conferred by large depths for berths. There is some intermodal infrastructure already in place with three container handling facilities at Constanta, however only one has relatively modern equipment. The port can currently process 400 containers a day but increasing containerisation is likely to require the port to process larger numbers in the future. A range of full and empty container handling should be developed in accordance with traffic projections to ensure Constanta competes with other major container ports. Whilst current capacity at Constanta is sufficient it is considered inadequate for long term future growth. Any such solution should be phased according to market conditions.

Specifically the project will incorporate:

Safe handling and storage facilities

Mobile or STS cranes for vessel loading and unloading

Figure 6.10 – Plans for a New Container Terminal at Constanta

Problems Addressed:

This intervention addresses the following issues at Constanta:

A lack of modern infrastructure at the port

A forecast lack of future capacity at Constanta

The increasing competition from other ports by ensuring Constanta remains an attractive choice to international freight movers

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW10 – Increase the amount of freight moved by sustainable modes

OW12 – Increase use of intermodal transport

Potential Market

This proposal would be for Constanta’s large container operations.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.13 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Costs | 351.1 | New container terminal Handling and storage area Mobile or STS cranes |

Outcomes:

The intervention gives a good return with a benefit-cost ratio (BCR) of 2.02. The terminal is anticipated to generate just short of a further 600,000 tonnes of containerised freight through Constanta in 2020. Based on these favourable results this project is recommended for inclusion in the Masterplan.

- Table 6.14 – Project modelled outputs

| Test code | P-CO-S (W25) |
| --- | --- |
| Annual Water Tonnes Change (2020) | 599,554 |
| NPV € Mill (2014 Prices) | 228 |
| BCR | 2.02 |
| EIRR | 8.9% |
| Masterplan Recommendation |  |

Figure 6.11 – Water Freight Tonnes Handled at Constanta 2007-2011 and Forecast in 2020

Implementing organisation:

This scheme would be implemented by Constanta Port Authority in conjunction with private terminal operators.

Potential Ownership Models

There are three potential models suggested by as options in terms of implementation and ownership.

- Port Authority develops, builds and operates the terminal as a going concern: This model sees the Port Authority take all of the risk due to investing all of the money, but benefit from all of any gain

- Port Authority develops and builds the terminal but the terminal is operated by a third party: This model sees the Port Authority take on some of the capital risk, in that they might invest their money in creating a terminal, but none of the operational risk as a third party would take on responsibility for managing the terminal on a day to day basis. The third party would pay the Port Authority in order to manage the terminal. This option could be considered medium risk with medium gain.

- Port Authority leases the land upon which a third party builds and operates the terminal: This model sees the Port Authority lease the land to a third party. The third party then builds the terminal to their specifications and operates the terminal with minimal interference from the Port Authority. This option is low risk to the Port Authority however it is also low gain as the third party would only be renting the land upon which the terminal stands and not the terminal itself like in the option above.

Any of these ownership models (or variant thereof) could be chosen by any of the ports in this National Transport Master Plan. It is up to the Port Authority to consider which option is best for them.

Potential Ownership Models

There are three potential models suggested by as options in terms of implementation and ownership.

- Port Authority develops, builds and operates the terminal as a going concern: This model sees the Port Authority take all of the risk due to investing all of the money, but benefit from all of any gain

- Port Authority develops and builds the terminal but the terminal is operated by a third party: This model sees the Port Authority take on some of the capital risk, in that they might invest their money in creating a terminal, but none of the operational risk as a third party would take on responsibility for managing the terminal on a day to day basis. The third party would pay the Port Authority in order to manage the terminal. This option could be considered medium risk with medium gain.

- Port Authority leases the land upon which a third party builds and operates the terminal: This model sees the Port Authority lease the land to a third party. The third party then builds the terminal to their specifications and operates the terminal with minimal interference from the Port Authority. This option is low risk to the Port Authority however it is also low gain as the third party would only be renting the land upon which the terminal stands and not the terminal itself like in the option above.

Any of these ownership models (or variant thereof) could be chosen by any of the ports in this National Transport Master Plan. It is up to the Port Authority to consider which option is best for them.

Implementation years: 2021-2030

The port is still below its peak handling figure for containers (which was in 2007), however it is forecast that the port will exceed 2007’s peak by the end of the decade. The consequences of this would mean that Constanta would exceed its current available capacity to handle containers. Therefore Constanta should begin to build this terminal by the end of the decade. The implementation schedule should be subject to market conditions at the port and adjusted accordingly.

Page intentionally left blank

Tulcea:

Development of a general cargo terminal

Proposal Description:

This would see the construction of a new terminal which will handle general cargo but also have the ability to handle grain for which the port does not currently have. Specifically the project will incorporate:

Modernisation of 3 berths for the direct loading of ships with grain.

Construction of suitable operating/loading facilities

Dredging to provide adequate depth to the berths

Problems Addressed:

Currently the grain from Dobrogea area is transported by road where ultimately much of it ends up at Constanta to be exported around the world. This intervention would address the following issues:

A lack of grain handling infrastructure at the port

Reduction in road freight miles

Maintain Tulcea’s viability and competitiveness as a port

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW10 – Increase the amount of freight moved by sustainable modes

Potential Market

This proposal would be for grain operations.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.15 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Costs | 16 | New grain terminal Handling and storage area Mobile or STS cranes |

Outcomes:

The intervention gives a limited return with a benefit-cost ratio (BCR) of 0.59. The terminal is anticipated to generate over 328,000 tonnes of additional freight in 2020. As the EIRR is less than 3% this project is not recommended for inclusion in the Masterplan. However due to the scheme being of regional importance the decision below is reversed in the final table.

- Table 6.16 – Project modelled outputs

| Test code | P-TL-S (W41) |
| --- | --- |
| Annual Water Tonnes Change (2020) | 328,527 |
| NPV € Mill (2014 Prices) | -4 |
| BCR | 0.59 |
| EIRR | -1.1% |
| Masterplan Recommendation |  |

Figure 6.12 –Tonnes Handled at Tulcea in 2008 and 2011 and Forecast in 2020

Tulcea does not currently have facilities that specialise in handling grain. Providing infrastructure that can handle grain will have the effcet of increasing the tonnage handled by over 328,000 tonnes.

- Table 6.17 – Forecast Utilisation at Tulcea

|  | Reference Case |  |  |  | Do Something |  |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal | 2020 | 2030 |  |  | 2020 | 2030 |
| General |  |  |  | General | 1.32% | 1.00% |
| Bulk | 117.47% | 113.69% |  | Bulk | 83.63% | 63.20% |
| Containers |  |  |  | Containers |  |  |
| Oil |  |  |  | Oil |  |  |
| Grain | 1.79% | 1.73% |  | Grain | 1.27% | 0.96% |
| Fertiliser |  |  |  | Fertiliser |  |  |
| Cement |  |  |  | Cement |  |  |
| Coal |  |  |  | Coal |  |  |
| Steel |  |  |  | Steel |  |  |
| Other | 11.00% | 11.36% |  | Other | 3.04% | 4.70% |

By creating a terminal that can handle general cargo as well as having dedicated grain handling equipment, Tulcea’s forecast capacity issues are averted.

Page intentionally left blank

Improve the existing passenger ferry service running from Tulcea to Sulina

Proposal Description:

This proposal involves an improved passenger ferry service running between Tulcea and Sulina and intermediate communities. Specifically the project will incorporate:

The purchase of three new ferries

The upgrade of 15 terminals

Problems Addressed:

For several communities in the Danube Delta the most suitable method of transport is on water. This intervention would address the following issues:

Poor and dilapidated state of some of the terminals

Improved connectivity for those who rely on the Danube Delta as either their home or business

Improved potential for tourism

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

Potential Market

This proposal would be for passenger operations and possibly some light freight (such as mail).

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.18 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 19.2 | 15 upgraded passenger terminals 3 new ferries |

Outcomes:

The intervention gives a good return with a benefit-cost ratio (BCR) of 1.47. The terminal is anticipated to generate over 176,000 passengers in 2020. This project is recommended for inclusion in the Masterplan due to its green credentials and the increased connectivity it would have to the region.

- Table 6.19 – Project modelled outputs

| Test code | W101 |
| --- | --- |
| Number of Passengers per annum (2020) | 176,713 |
| NPV € Mill (2014 Prices) | 9 |
| BCR | 1.47 |
| EIRR | 4.4% |
| Masterplan Recommendation |  |

Implementing organisation:

This scheme would be implemented by the private passenger terminal operators through a franchising system.

Implementation years: 2014-2020

The implementation could happen as soon as project sign off is completed. It is not dependent on any other conditions.

Galati:

Build a new trimodal terminal (multimodal platform)

Proposal Description:

As part of a strategic network of intermodal terminals throughout Romania, Galati has been identified as a key location for international freight movements. The proposal is to build a new trimodal terminal at the port so that intermodal containers can take advantage of Galati’s road, rail and water connections. This would provide Galati with dedicated intermodal facilities and berths, which it currently does not have and for its connections to be improved as required by Regulation 1315/2013. Specifically the proposal will incorporate:

A new intermodal terminal 7 hectares in size, including handling equipment, with modern infrastructure including quays that will allow the direct mooring of ships.

Links to both European and Russian gauge railway lines

RoRo facilities for trucks

Logistics facilities

Connectivity to the port’s Free Trade Zone and modernisation of the road and rail links to a standard that complies with Regulation 1315/2013 which includes a dual carriageway that does not intersect with the railway network, reconfiguration of therail marshalling yard to allow the operation of 750m long trains, electrification and signalling works.

Galati has proposed a project called "Multimodal Platform Galati - removing major bottlenecks by upgrading existing infrastructure and providing missing links to the core network Rhine-Danube / Alps" which is targetting CEF funding. AECOM’s proposal is suggested to act as Phase 1 of this project. If AECOM’s proposal is successful then further developments should occur as and when market conditions make it suitable to do so.

Problems Addressed:

The port does not currently have a dedicated intermodal terminal which is limiting its potential. This intervention would address the following issues:

Lack of a dedicated intermodal terminal

Lack of an intermodal terminal network in Romania

Bottlenecks in the local transport network

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW10 – Increase the amount of freight moved by sustainable modes

OW12 – Increase use of intermodal transport

Potential Market

This proposal would be for container operations.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.20 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 23 | Trimodal terminal including handling equipment Links to European and Russian gauge railways Logistics facilities |

Outcomes:

The intervention gives a limited return with a benefit-cost ratio (BCR) of 0.94. The terminal is anticipated to generate over 186,000 of containerised freight in 2020. Whilst the modelled outputs are not entirely favourable this project is recommended for inclusion in the Masterplan as it is considered an important node in Romania’s intermodal network and a port of Galati’s size should be able to handle containers using modern methods.

- Table 6.21 – Project modelled outputs

| Test code | I-GL-S (W18) |
| --- | --- |
| Annual Water Tonnes Change (2020) | 186,743 |
| NPV € Mill (2014 Prices) | -1 |
| BCR | 0.94 |
| EIRR | 4.7% |
| Masterplan Recommendation |  |

Implementing organisation:

This scheme would be implemented by a partnership consisting of the port administrator, APDM Galati with a private port operator and other relevant stakeholders.

Implementation years: 2021-2030

The implementation could happen as soon as project sign off is completed however it is recommended that development of the intermodal terminal is phased to suit prevailing market conditions.

Modernisation of existing bulk terminals

Proposal Description:

There are bulk terminal facilities at Galati that are old and inefficient. This inefficiency is leading to an underutilisation of the port due to limiting the amount of bulk freight that can be handled by port in comparison to the available demand. Modernising these facilities will improve options and handling capacity at the port. Specifically the project will include:

Quay modernization to allow ships to undertake direct mooring as well as providing services to ships

Upgrading the loading and unloading operations

Rehabilitation of the storage areas and the utilities at the port

Problems Addressed:

Current facilities are old and not suitable for modern logistics practices. Whilst the port has spare capacity on paper, this is not realised in reality due to the condition of some of the facilities which prevents their use by freight traffic. This proposal would address the following problems:

Give a new lease of life for old infrastructure and terminals by making it suitable for modern logistics requirements

Turn over underutilised capacity to industries and commodities that need additional capacity

Realign the basin to allow modern vessels to easily use the port

Prevent further decay of the port and secure the port’s long term future and viability

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW10 – Increase the amount of freight moved by sustainable modes

Potential Market

This proposal would be for Galati’s bulk operations such as grain, aggregate and steel.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.22 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 17.6 | Modernised bulk terminal Dredging and basin realignment Reinforced structures |

Outcomes:

The intervention gives a good return with a benefit-cost ratio (BCR) of 32.89. The terminal is anticipated to generate over 618,000 of freight in 2020. This project is recommended for inclusion in the Masterplan.

- Table 6.23 – Project modelled outputs

| Test code | P-GL-S (W34) |
| --- | --- |
| Annual Water Tonnes Change (2020) | 618,501 |
| NPV € Mill (2014 Prices) | 348 |
| BCR | 32.89 |
| EIRR | 39.9% |
| Masterplan Recommendation |  |

Figure 6.13 –Tonnes Handled at Galati 2007-2011 and Forecast in 2020

Galati has a theoretical capacity of 7.8 million tonnes per year however 6.8 million tonnes are for handling coal and steel associated with the declining steel industry in the area. The steel production is just 25% of the originally designed capacity. So the operating capacity can be calculated as 2 million tonnes per year (1 million tonnes associated with steel production and 1 million tonnes of other goods). As such Galati needs to refit and modernise the existing terminals and berths for new traffic from other commodities.

- Table 6.24 – Forecast Utilisation at Galati

|  | Reference Case |  |  |  | Do Something |  |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal | 2020 | 2030 |  | Terminal | 2020 | 2030 |
| General | 45.39% | 49.49% |  | General | 15.17% | 21.33% |
| Bulk | 109.37% | 119.26% |  | Bulk | 63.97% | 89.93% |
| Containers |  |  |  | Containers | 7.80% | 12.00% |
| Oil |  |  |  | Oil |  |  |
| Grain | 32.78% | 35.74% |  | Grain | 21.91% | 30.80% |
| Fertiliser |  |  |  | Fertiliser |  |  |
| Cement |  |  |  | Cement |  |  |
| Coal | 7.84% | 8.54% |  | Coal | 5.24% | 7.36% |
| Steel | 0.00% | 0.00% |  | Steel | 0.00% | 0.00% |
| Other | 1.90% | 2.62% |  | Other | 0.65% | 0.91% |

Modernising the bulk handling terminals so that they more efficient means that Galati has increased its capacity so that it is able to handle the forecast demand. The creation of a trimodal terminal also allows for the handling of containers.

Implementing organisation:

This scheme would be implemented by APDM Galati, possibly in conjunction with a private terminal operator.

Implementation years: 2014-2020

The implementation could happen as soon as project sign off is completed however it is recommended that any terminal upgrades are phased to suit prevailing market conditions.

Page intentionally left blank

Cernavoda:

Develop the infrastructure

Proposal Description:

Cernavoda could do with its infrastructure being developed. The volumes that the port has handled over the period 2007-2011 are low compared to overall potential capacity and volatile in volume. A sensible conclusion of this is that Cernavoda is too reliant on a small range of commodities. This proposal will include:

Modernisation and refurbishment of the berths so that it can handle multiple commodities

Dredging of its basin

Improved navigation signals

Problems Addressed:

Undertaking this proposal would address the following problem:

- A reliability on a small range of commodity types which has lead to high volatility in freight volumes handled in the past

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW10 – Increase the amount of freight moved by sustainable modes

Potential Market

This proposal would be for Cernavoda’s raw minerals as well as potential new commodities like aggregates. New markets would be determined by a future feasibility study.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.25 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 6.9 | Modernised and refurbished berths Basin dredging Navigation signals |

Outcomes:

The intervention gives a good return with a benefit-cost ratio (BCR) of 6.98. The terminal is anticipated to generate over 365,000 of freight in 2020. This project is recommended for inclusion in the Masterplan.

- Table 6.26 – Project modelled outputs

| Test code | P-CV-S (W28) |
| --- | --- |
| Annual Water Tonnes Change (2020) | 365,350 |
| NPV € Mill (2014 Prices) | 26 |
| BCR | 6.98 |
| EIRR | 22.4% |
| Masterplan Recommendation |  |

Figure 6.27 –Tonnes Handled at Cernavoda 2007-2011 and Forecast in 2020

- Table 6.27 – Forecast Utilisation at Cernavoda

|  | Reference Case |  |  |  | Do Something |  |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal | 2020 | 2030 |  | Terminal | 2020 | 2030 |
| General | 64.63% | 68.13% |  | General | 46.89% | 53.24% |
| Bulk |  |  |  | Bulk |  |  |
| Containers |  |  |  | Containers |  |  |
| Oil |  |  |  | Oil |  |  |

The table shows that by developing the infrastructure at Cernavoda the port can increase its handling capacity and better handle forecasted future flows.

Implementing organisation:

This scheme would be implemented by APDM Galati, possibly in conjunction with a private terminal operator.

Implementation years: 2014-2020

The implementation could happen as soon as project sign off is completed.

Basarabi:

Modernise the infrastructure

Proposal Description:

This scheme would see the some of the port infrastructure being modernised. Specifically the proposal includes:

Modernisation of some port infrastructure

Rehabilitation of 1,000m of road to improve connectivity to the port

Improved security and provision of port services

Problems Addressed:

Basarabi has recently handled large volumes of freight related to local road building. Now that this is complete the volumes to be handled by the port are forecast to drop. Undertaking this proposal will address the following problems:

A limitation in the current port infrastructure to handle modern freight

Poor connectivity to the road network

Forecast low volumes for the future

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW10 – Increase the amount of freight moved by sustainable modes

Potential Market

This proposal would be for the port’s existing operations.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.28 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 6 | Modernise some of the port infrastructure Rehabilitate main connecting road Improve security and port services |

Outcomes:

The intervention gives a limited return with a benefit-cost ratio (BCR) of 0.46. The terminal is anticipated to generate just over 5,000 tonnes of freight in 2020. Because of the poor modelled results this project is not recommended for inclusion in the Masterplan.

- Table 6.29 – Project modelled outputs

| Test code | P-BS-S (W26) |
| --- | --- |
| Annual Water Tonnes Change (2020) | 5,145 |
| NPV € Mill (2014 Prices) | -2 |
| BCR | 0.46 |
| EIRR | 1.2% |
| Masterplan Recommendation |  |

Figure 6.15 –Tonnes Handled at Basarabi 2007-2011 and Forecast in 2020

Whilst Basarabi has a theoretical capacity of 700,000 tonnes per year, the port is not predicted to handle anywhere near that volume in the years to come. Recent traffic has been due to construction which is now complete.

Oltenita:

Develop the bulk handling infrastructure

Proposal Description:

Oltenita is close to Giurgiu. As Oltenita does not handle any containerised freight AECOM has designated Oltenita as the port to handle the region’s bulk freight, particularly that heading towards Bucharest. Oltenita can achieve this by developing its bulk handling infrastructure, for which more berths are currently orientated to handling. Specifically the proposal consists of:

Improvements to six berths which would allow them to handle mixed freight

Dredging of the port basin

Improved port services

Problems Addressed:

Designating Oltenita as the regional bulk handling port means that it does not inadvertently compete with Giurgiu. Undertaking this proposal will address the following problems:

- Ensure that both Oltenita and Giurgiu have a role to play in Romania’s economy

- Gives Oltenita a clear purpose

- Enables the port to handle modern freight in an efficient manner

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW10 – Increase the amount of freight moved by sustainable modes

Potential Market

This proposal would be for Oltenita’s bulk handling operations.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.30 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 5.6 | Modernise some of the port infrastructure Rehabilitate main connecting road Improve security and port services |

Outcomes:

The intervention gives a good return with a benefit-cost ratio (BCR) of 29.35. The terminal is anticipated to generate over 35,000 tonnes of freight in 2020. This project is recommended for inclusion in the Masterplan.

- Table 6.31 – Project modelled outputs

| Test code | P-OT-S (W30) |
| --- | --- |
| Annual Water Tonnes Change (2020) | 35,908 |
| NPV € Mill (2014 Prices) | 99 |
| BCR | 29.35 |
| EIRR | 30.1% |
| Masterplan Recommendation |  |

Figure 6.16 –Tonnes Handled at Oltenita 2007-2011 and Forecast in 2020

- Table 6.32 – Forecast Utilisation at Oltenita

|  | Reference Case |  |  |  | Do Something |  |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal | 2020 | 2030 |  | Terminal | 2020 | 2030 |
| General |  |  |  | General |  |  |
| Bulk | 196.01% | 269.73% |  | Bulk | 44.98% | 75.85% |
| Grain | 70.69% | 97.01% |  | Grain | 40.99% | 69.11% |

The table shows that by developing the bulk handling terminal capacity at Oltenita the forecast over utilisation is no longer a problem.

Implementing organisation:

This scheme would be implemented by APDF Giurgiu, possibly in conjunction with a private terminal operator.

Implementation years: 2014-2020

The implementation could happen as soon as project sign off is completed, however to achieve the maximum potential benefits it is recommended that project completion is scheduled after the completion of proposed navigation improvements to the River Danube.

Giurgiu:

Build a new tri-modal terminal in the port of Giurgiu and connections with the hinterland

Proposal Description:

As part of a strategic network of intermodal terminals throughout Romania, Giurgiu has been identified as a key location. The proposal is to build a new trimodal terminal at the port so that intermodal containers can take advantage of Giurgiu’s road, rail and water connections. By specialising Giurgiu to handle containers this ensures that the port does not inadvertently compete with the geographically close by port of Oltenita. Giurgiu’s proximity to Bucharest makes it an ideal choice for inclusion on Romania’s intermodal network. Specifically the proposal will incorporate:

A new intermodal terminal 6-8 hectares in size, including handling equipment

Logistics facilities

Connectivity to the port’s Free Trade Zone

Problems Addressed:

The port currently has the ability to handle containers but would benefit from a modern and dedicated intermodal terminal operation to achieve its potential. This intervention would address the following issues:

Lack of a large and modern dedicated intermodal terminal

Lack of an intermodal terminal network in Romania

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW10 – Increase the amount of freight moved by sustainable modes

OW12 – Increase use of intermodal transport

Potential Market

This proposal would be for container operations.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.33 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 10 | New intermodal terminal 6-8 hectares in size Logistics facilities Improved connectivity |

Outcomes:

The intervention gives a limited return with a benefit-cost ratio (BCR) of 0.56. The terminal is anticipated to generate over 69,000 tonnes of intermodal freight in 2020. Even though the modelled outputs suggest a limited return, this project is of strategic importance for Romani and so is recommended for inclusion in the Masterplan.

- Table 6.34 – Project modelled outputs

| Test code | I-GR-S (W20) |
| --- | --- |
| Annual Water Tonnes Change (2020) | 69,803 |
| NPV € Mill (2014 Prices) | -5 |
| BCR | 0.56 |
| EIRR | 1.4% |
| Masterplan Recommendation |  |

Implementing organisation:

This scheme would be implemented by APDF Giurgiu, possibly in conjunction with a private terminal operator.

Implementation years:

The implementation could happen as soon as project sign off is completed, however to achieve the maximum potential benefits it is recommended that project completion is scheduled after the completion of proposed navigation improvements to the River Danube (see later). Given that the port currently handles containers, the project should be phased in scale to suit prevailing market conditions.

Infrastructure and modernisation works

Proposal Description:

Giurgiu is a large and strategically important port that counts Bucharest as part of its hinterland. Some of its infrastructure is no longer suitable to accommodate modern logistics requirements. This proposal will incorporate:

Improvements to the berths as mentioned in the project "D.A.N.U.B.E- Access network to the Danube”

Figure 6.17 – Infrastructure and Modernisation Plans at Giurgiu

Problems Addressed:

The port intends to increase its ability to handle containers but must ensure that its infrastructure can cope. This intervention would address the following issues:

Old infrastructure that is ill equipped for modern logistics needs

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW10 – Increase the amount of freight moved by sustainable modes

Potential Market

This proposal would be for all operations at Giurgiu including grains and cereals.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.35 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 4.3 | Rehabilitated berths |

Outcomes:

The intervention gives a good return with a benefit-cost ratio (BCR) of 17.53. The proposal is anticipated to generate over 54,000 tonnes of freight in 2020. This project is recommended for inclusion in the Masterplan.

- Table 6.36 – Project modelled outputs

| Test code | P-GR-S (W37) |
| --- | --- |
| Annual Water Tonnes Change (2020) | 54,372 |
| NPV € Mill (2014 Prices) | 46 |
| BCR | 17.53 |
| EIRR | 24.3% |
| Masterplan Recommendation |  |

Figure 6.18 –Tonnes Handled at Giurgiu 2007-2011 and Forecast in 2020

Based upon the capacity figures Giurgiu has a theoretical maximum capacity of 2.5 million tonnes per year. The port is not achieveing these levels because the current infrastructure is not suitable. Refurbishing the terminal and replacing the equipment will help to boost volumes at the port.

- Table 6.37 – Forecast Utilisation at Giurgiu

|  | Reference Case |  |  |  | Do Something |  |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal | 2020 | 2030 |  | Terminal | 2020 | 2030 |
| General | 84.58% | 126.48% |  | General | 33.90% | 57.29% |
| Containers |  |  |  | Containers | 4.52% | 5.69% |
| Grain | 55.47% | 86.31% |  | Grain | 33.05% | 55.87% |

The table shows that refurbishing the terminal and replacing the equipment will help to improve capacity at the port so that the forecast over utilisation is no longer an issue.

Implementing organisation:

This scheme would be implemented by APDF Giurgiu, possibly in conjunction with a private terminal operator.

Implementation years: 2014-2020

The implementation could happen as soon as project sign off is completed, however to achieve the maximum potential benefits it is recommended that project completion is scheduled after the completion of proposed navigation improvements to the River Danube (see later).

Corabia:

Modernisation and rehabilitation of infrastructure

Proposal Description:

Corabia is well located but currently handles little freight traffic and is in a relatively severe state of dilapidation, with the exception of some grain and cereal traffic and storage facilities. This proposal will see the rehabilitation of its dilapidated infrastructure and modernised to suit current logistics practices. Specifically the proposal incorporates:

Rehabilitation of the berths

Remediation work to some of the infrastructure

Modernisation to accommodate modern logistics requirements

Problems Addressed:

The problems addressed by this proposal are:

Modernisation will attract other freight flows increasing the tonnage handled by the port

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW10 – Increase the amount of freight moved by sustainable modes

Potential Market

This proposal would benefit Corabia’s current operations in cereals and grain but also any other potential commodities identified in a feasibility study.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.38 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 4.5 | Rehabilitated berths Remediated and modernised infrastructure |

Outcomes:

The intervention gives a good return with a benefit-cost ratio (BCR) of 11.04. The proposal is anticipated to generate over 73,000 tonnes of freight in 2020. This project is recommended for inclusion in the Masterplan.

- Table 6.39 – Project modelled outputs

| Test code | P-CB-S (W32) |
| --- | --- |
| Annual Water Tonnes Change (2020) | 73,848 |
| NPV € Mill (2014 Prices) | 29 |
| BCR | 11.04 |
| EIRR | 25.1% |
| Masterplan Recommendation |  |

Figure 6.19 –Tonnes Handled at Corabia 2007-2011 and Forecast in 2020

- Table 6.40 – Forecast Utilisation at Corabia

|  | Reference Case |  |  |  | Do Something |  |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal | 2020 | 2030 |  | Terminal | 2020 | 2030 |
| General | 58.21% | 107.16% |  | General | 12.16% | 20.76% |
| Grain |  |  |  | Grain | 0.54% | 1.03% |

The table shows that by modernisation and rehabilitation of Corabia’s infrastructure will improve effiency thereby improving capacity so that the forecast over utilisation is no longer a problem

Implementing organisation:

This scheme would be implemented by APDF Giurgiu, possibly in conjunction with a private terminal operator.

Implementation years: 2014-2020

The implementation could happen as soon as project sign off is completed, however to achieve the maximum potential benefits it is recommended that project completion is scheduled after the completion of proposed navigation improvements to the River Danube (see later).

Calafat:

Develop bulk handling infrastructure

Proposal Description:

Calafat is linked to Vidin, Bulgaria and the port is located near to the newly opened bridge. To maximise the benefit of this new link the port needs to develop its bulk handling infrastructure. The proposal would incorporate:

Restoration and extension of the berths

New berths to accommodate new freight

Figure 6.20 – Plans to Develop Bulk Handling Infrastructure at Calafat

Problems Addressed:

The problems addressed by this proposal are:

Current growth trends suggest that the port will run out of capacity

Some of the berths are not set up for modern logistics needs

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW10 – Increase the amount of freight moved by sustainable modes

Potential Market

This proposal would be for Calafat’s current operations as well as new flows that will arrive due to the construction of the bridge.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.41 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 20 | Rehabilitated berths New berths |

Outcomes:

The intervention gives a limited return with a benefit-cost ratio (BCR) of 0.6. The proposal is anticipated to generate over 11,000 tonnes of freight in 2020. Since the EIRR is less than 3% this project is not recommended for inclusion in the Masterplan.

- Table 6.42 – Project modelled outputs

| Test code | P-CF-S (W27) |
| --- | --- |
| Annual Water Tonnes Change (2020) | 11,871 |
| NPV € Mill (2014 Prices) | -5 |
| BCR | 0.60 |
| EIRR | 2.4% |
| Masterplan Recommendation |  |

Figure 6.20 –Tonnes Handled at Calafat 2007-2011 and Forecast in 2020

Based upon the capacity figures Calafat port has a theoretical maximum capacity of 518,000 tonnes per year.

- Table 6.43 – Forecast Utilisation at Calafat

|  | Reference Case |  |  |  | Do Something |  |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal | 2020 | 2030 |  | Terminal | 2020 | 2030 |
| Bulk | 90.98% | 113.51% |  | Bulk | 32.63% | 43.55% |

The table above shows that by developing the bulk handling infrastructure at Calafat the port will have sufficient capacity to handle forecast demand.

Drobeta Turnu Severin:

Build a new trimodal terminal

Proposal Description:

Drobeta Turnu Severin is strategically place to benefit from a trimodal intermodal terminal as part of Romania’s intermodal network. Developing intermodal facilities will boost the port and provide its hinterland a useful asset. Specifically the proposal will incorporate:

An intermodal 6-8 hectares in size with accompanying handling equipment

2 gauge cleared rail lines of 750m in length

Improved road connections

Figure 6.21 – Trimodal Terminal Plans at Drobeta Turnu Severin

Problems Addressed:

Drobeta Turnu Severin port is not currently well set up to handle containers. The problems addressed by this proposal are:

A modern container terminal that can handle containers efficiently and effectively

Establish Drobeta as the regional container port

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW10 – Increase the amount of freight moved by sustainable modes

OW12 – Increase use of intermodal transport

Potential Market

This proposal would be for container operations.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.44 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 8 | Intermodal terminal 6-8 hectares Handling equipment 2 gauge cleared railway lines Improved road connections |

Outcomes:

The intervention gives a limited return with a benefit-cost ratio (BCR) of 0.6. The proposal is anticipated to generate over 11,000 tonnes of freight in 2020. Although the model outputs are limited this project is recommended for inclusion in the Masterplan.

- Table 6.45 – Project modelled outputs

| Test code | I-DB-S (W19) |
| --- | --- |
| Annual Water Tonnes Change (2020) | 100,5149 |
| NPV € Mill (2014 Prices) | 1 |
| BCR | 1.11 |
| EIRR | 5.6% |
| Masterplan Recommendation |  |

Implementing organisation:

This scheme would be implemented by APDF Giurgiu, possibly in conjunction with a private terminal operator.

Implementation years: 2021-2030

The implementation could happen as soon as project sign off is completed, however to achieve the maximum potential benefits it is recommended that project completion is scheduled after the completion of proposed navigation improvements to the River Danube (see later).

Develop the infrastructure

Proposal Description:

Drobeta Turnu Severin’s port infrastructure needs improving to be able to handle freight. Improved freight handling will increase the port’s handling capacity. The current general freight handling ability is just 30 tonnes per hour which is a limiting factor. This proposal will incorporate:

Improvements to existing handling equipment and facilities

New berths

Additional freight handling areas through rearrangement of port facilities

Figure 6.22 – Infrastructure Development Plans at Drobeta Turnu Severin

Problems Addressed:

The problems addressed by this proposal are:

The current port layout is not maximising efficient use of port facilities and land

Current handling equipment is not suitable for modern freight requirements

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW10 – Increase the amount of freight moved by sustainable modes

Potential Market

This proposal would be for all of Drobeta Turnu Severin’s operations including its petroleum and iron ore trade.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.46 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 17.3 | Improvement to handling equipment and facilities New berths Rearranged port layout |

Outcomes:

The intervention gives a limited return with a benefit-cost ratio (BCR) of 0.6. The proposal is anticipated to generate over 11,000 tonnes of freight in 2020. Although the model outputs are limited this project is recommended for inclusion in the Masterplan.

- Table 6.47 – Project modelled outputs

| Test code | P-DB-S (W23) |
| --- | --- |
| Annual Water Tonnes Change (2020) | 108,401 |
| NPV € Mill (2014 Prices) | 117 |
| BCR | 11.68 |
| EIRR | 22.8% |
| Masterplan Recommendation |  |

Figure 6.23 –Tonnes Handled at Drobeta Turnu Severin 2007-2011 and Forecast in 2020

Based upon the capacity figures the port has a theoretical maximum capacity of 500,000 tonnes per year. By 2020 it is predicted that the port will have exceeded this theoretical maximum capacity and so the recommended projects will help the port to achieve its potential.

- Table 6.48 – Forecast Utilisation at Drobeta Turnu Severin

|  | Reference Case |  |  |  | Do Something |  |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal | 2020 | 2030 |  | Terminal | 2020 | 2030 |
| General | 10.50% | 10.91% |  | General | 2.92% | 4.74% |
| Bulk | 106.93% | 111.08% |  | Bulk | 13.84% | 22.48% |
| Containers |  |  |  | Containers | 0.35% | 0.57% |
| Other | 66.28% | 69.11% |  | Other | 20.32% | 33.01% |

The table shows that by developing the infrastructure at the port and including a trimodal terminal has the effect of providing sufficient capacity to address the forecast utilisation issues.

Implementing organisation:

This scheme would be implemented by APDF Giurgiu, possibly in conjunction with a private terminal operator.

Implementation years: 2014-2020

The implementation could happen as soon as project sign off is completed, however to achieve the maximum potential benefits it is recommended that project completion is scheduled after the completion of proposed navigation improvements to the River Danube (see later).

Orsova:

Modernisation of the port

Proposal Description:

The infrastructure at Orsova is old and needs modernising. Specifically this proposal incorporates:

Improvements to the berth area

Improvements to signalling

Creation of new berths to handle modern freight

Figure 6.24 – Modernisation Plans at Orsova

Problems Addressed:

The problems addressed by this proposal are:

Tonnes handled from one year to next varies and so a general modernisation programme at the port will provide stability

Old infrastructure is not suitable to modern freight requirements

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW10 – Increase the amount of freight moved by sustainable modes

Potential Market

This proposal would benefit all operations at the port such as building materials and raw minerals and may attract new flows.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.49 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 7.8 | Improvement to berth area New berths Improved signalling |

Outcomes:

The intervention gives a good return with a benefit-cost ratio (BCR) of 12.7. The proposal is anticipated to generate over 66,000 tonnes of freight in 2020. This project is recommended for inclusion in the Masterplan.

- Table 6.50 – Project modelled outputs

| Test code | P-OV-S (W33) |
| --- | --- |
| Annual Water Tonnes Change (2020) | 66,045 |
| NPV € Mill (2014 Prices) | 58 |
| BCR | 12.70 |
| EIRR | 24.8% |
| Masterplan Recommendation |  |

Figure 6.25 –Tonnes Handled at Orsova 2007-2011 and Forecast in 2020

The capacity figures at Orsova give the port a theoretical maximum capacity of 806,000 tonnes per year. However the port infrastructure is ageing and it is unlikely to be able to achieve this in reality. Modernisation will help the port to realise its potential.

- Table 6.51 – Forecast Utilisation at Orsova

|  | Reference Case |  |  |  | Do Something |  |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal | 2020 | 2030 |  | Terminal | 2020 | 2030 |
| General | 9.93% | 15.47% |  | General | 10.39% | 17.28% |
| Bulk | 71.20% | 110.86% |  | Bulk | 39.85% | 66.29% |
| Other | 1.83% | 6.48% |  | Other | 39.23% | 66.93% |

Modernising the port has the effect of more efficient operations which increases capacity at the port. This increased capacity addresses the forecast utilisation issues.

Implementing organisation:

This scheme would be implemented by APDF Giurgiu, possibly in conjunction with a private terminal operator.

Implementation years: 2014-2020

The implementation could happen as soon as project sign off is completed, however to achieve the maximum potential benefits it is recommended that project completion is scheduled after the completion of proposed navigation improvements to the River Danube (see later).

Moldova Veche:

Development of the infrastructure

Proposal Description:

The volume of freight that passes through Moldova Veche varies considerably and so the development of the infrastructure at the port is intended to give greater stability in flow volumes. Greater stability will provide a strong platform with which to increase volumes at the port. Specifically the proposal incorporates:

Improving 2 existing berths and 3 new berths

Dredging of port basin

Improved port services

Problems Addressed:

The problems addressed by this proposal are:

Tonnes handled from one year to next varies and so a general development programme at the port will provide stability

Old infrastructure is not suitable to modern freight requirements

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW10 – Increase the amount of freight moved by sustainable modes

Potential Market

This proposal would be to attract new flows to the port which would be identified in a detailed feasibility study.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.52 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 3.7 | Improvement to berth area New berths Dredging of port basin Improved port services |

Outcomes:

The intervention gives a good return with a benefit-cost ratio (BCR) of 1.51. For this reason the project is recommended for inclusion in the Masterplan.

- Table 6.53 – Project modelled outputs

| Test code | P-MV-S (W29) |
| --- | --- |
| Annual Water Tonnes Change (2020) | 2,961 |
| NPV € Mill (2014 Prices) | 1 |
| BCR | 1.51 |
| EIRR | 7.2% |
| Masterplan Recommendation |  |

Figure 6.26 –Tonnes Handled at Moldova Veche 2007-2011 and Forecast in 2020

Based on the capacity figures the port has a theoretical maximum capacity of 374,000 tonnes however some of these berths are old and have old equipment meaning achieving the maximum capacity is not possible. Developing the infrastructure can help achieve this.

- Table 6.54 – Forecast Utilisation at Moldova Veche

|  | Reference Case |  |  |  | Do Something |  |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal | 2020 | 2030 |  | Terminal | 2020 | 2030 |
| General |  |  |  | General | 29.18% | 46.17% |
| Bulk | 5.00% | 4.38% |  | Bulk | 5.64% | 8.93% |

The table shows that developing the infrastructure at the port allows it to handle more volume of goods.

Implementing organisation:

This scheme would be implemented by APDF Giurgiu, possibly in conjunction with a private terminal operator.

Implementation years: 2021-2030

The implementation could happen as soon as project sign off is completed, however to achieve the maximum potential benefits it is recommended that project completion is scheduled after the completion of proposed navigation improvements to the River Danube (see later).

Waterways:

Improvements to the Navigation of the Danube

Proposal Description:

The Danube can suffer from poor navigability which can be caused by a number factors such as silting, low or high volumes of water or freezing. This proposal would incorporate:

Dredging of the fluvial Danube to maintain an adequate fairway

Purchase of a dedicated ice breaker

Provide an adequate maintenance budget

Problems Addressed:

The problems addressed by this proposal are:

Silting of the Danube will be addressed by one off significant dredging and planned maintenance dredging

Ice will be broken by a dedicated ice breaker

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW1 – Improve availability of the Danube

OW2 – Improve Danube connectivity to reduce cost and time

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW10 – Increase the amount of freight moved by sustainable modes

Potential Market

This proposal would benefit all operations on the Danube.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.55 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 103 | Dredging Danube fairway |

Outcomes:

The intervention gives a good return with a benefit-cost ratio (BCR) of 4.70. The proposal is anticipated to generate over 307,000 tonnes of freight in 2020 this project is being recommended for inclusion in the Masterplan.

- Table 6.56 – Project modelled outputs

| Test code | W1 |
| --- | --- |
| Annual Water Tonnes Change (2020) | 307,672 |
| NPV € Mill (2014 Prices) | 612 |
| BCR | 4.70 |
| EIRR | 19.6% |
| Masterplan Recommendation |  |

Implementing organisation:

This scheme would be implemented by AFDJ Galati.

Implementation years: 2014-2020

This scheme should be implemented as soon as possible as it will affect all of the ports in this Masterplan.

Bucharest-Danube Channel

Proposal Description:

This would involve the creation of a direct link between Bucharest and the River Danube. A link was started in the 1980s but never completed. Specifically the proposal incorporates:

Creation of 104km waterway including making the Arges river navigable

Creation of a water freight terminal at Bucharest

Problems Addressed:

Bucharest has no direct link to the Danube and so freight and passengers must tranship, usually at Giurgiu

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW1 – Improve availability of the Danube

OW2 – Improve Danube connectivity to reduce cost and time

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW9 – Reduce emissions of CO2 and Sulphur Dioxide

OW10 – Increase the amount of freight moved by sustainable modes

Potential Market

This proposal would benefit all operations in particular building materials and solid mineral fuels.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.57 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 1,500 | Completion of Bucharest-Danube Channel Creation of water freight terminal at Bucharest |

Outcomes:

The intervention gives a limited return with a benefit-cost ratio (BCR) of 0.95. The proposal is anticipated to generate over 2.7m tonnes of freight in 2020. This project is being recommended for inclusion in the Masterplan although it should be revisited closer to 2030 by a detailed feasibility study.

- Table 6.58 – Project modelled outputs

| Test code | W36 |
| --- | --- |
| Annual Water Tonnes Change (2020) | 2,760,878 |
| NPV € Mill (2014 Prices) | -48 |
| BCR | 0.95 |
| EIRR | 4.7% |
| Masterplan Recommendation |  |

Figure 6.27 - Water Freight Tonnes on the Bucharest – Danube Channel (per annum)

Implementing organisation:

A new channel will need a new authority in order to manage and maintain the channel, similar to how ACN manages the Danube-Black Sea Canal. In the interim it is suggested that the MT are the owner organisation.

Implementation years: After 2030

This is a long term project and not recognised as a priority for the Masterplan and so the timeline for implementation is suggested as being post 2030. This is due to:

1 – the expense of the project

2 – Bucharest is already well connected by other modes and these modes will improve over time (in line with the forecasts made by this Master Plan)

3 – Funding this project would undermine the benefits of investments made at both Oltenita and Giurgiu

4 – this project is entirely reliant on the availability of the Danube

Improvements to the Sulina Channel

Proposal Description:

This would involve introducing measures to restore and maintain their shores which are:

- - Stopping the erosion in some areas by protecting the banks of Sulina channel,

- - Stabilising and securing the navigability of the canal

- - Reducing the risk of flooding of coastal settlements and economic facilities along the canal

- - Improving environmental protection in the sector.

This is the second phase of an ongoing project on the Sulina Channel.

Problems Addressed:

Adopting these measures will lead to greater safety of navigation by providing general stability of the canal.

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW1 – Improve availability of the Danube

OW2 – Improve Danube connectivity to reduce cost and time

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW9 – Reduce emissions of CO2 and Sulphur Dioxide

OW10 – Increase the amount of freight moved by sustainable modes

Potential Market

This proposal would benefit all operations in particular building materials and solid mineral fuels.

Undiscounted Costs:

It has been estimated that the project will incur the following costs:

- Table 6.57 – Undiscounted project costs

|  | Undiscounted costs (Million €, 2014 prices) | Description of improvements included |
| --- | --- | --- |
| Cost | 20 | Strengthening hydraulic structures and shore defences Rehabilitation of the Sulina Channel |

Outcomes:

The intervention gives a return with a benefit-cost ratio (BCR) of 1.55. This project is being recommended for inclusion in the Masterplan.

- Table 6.59 – Project modelled outputs

| Test code | SULINA |
| --- | --- |
| Annual Water Tonnes Change (2020) |  |
| NPV € Mill (2014 Prices) | 13 |
| BCR | 1.55 |
| EIRR | 9.51% |
| Masterplan Recommendation |  |

The EIRR is 9.51% which is over the 3% threshold for inclusion in the Masterplan.

Implementing organisation:

This should be managed by AFDJ Galati as they are managing phase 1.

Implementation years: 2014-2020

River Danube Maintenance

Proposal Description:

The Danube maintenance can broadly be split into three: routine maintenance, non-routine and emergency maintenance. By maintenance AECOM means maintaining the viability of the Danube’s navigation.

Routine maintenance: This is maintenance that was planned and is regularly required on the Danube. This will typically include scheduled routine maintenance to the Danube’s fairway and river banks. This is essentially day to day maintenance of the Danube. Planned preventative maintenance is included within routine maintenance. By spending money on maintaining the Danube and keeping it in a good working condition, the Danube is less likely to require more expensive emergency maintenance in the future as prevention is almost always cheaper than the cure.

Non-routine maintenance: This is maintenance that can be foreseen however it occurs on an unscheduled basis. This can include ice breaking during winter months when it is possible to break the ice to allow safe and complete navigation.

Emergency maintenance: This type of maintenance arises from situations that cannot be regularly anticipated such as a critical failure of the Danube’s infrastructure such as through flooding or drought. Any repairs as a result of emergency maintenance should take place as soon as possible to minimise the disruption experienced by the Danube’s users.

Problems Addressed:

Once the one-off dredging of the Danube has been completed it will be necessary to ensure the situation does not go back to how it currently is by undertaking regular and planned maintenance. The maintenance budget Romania provides for the Danube is currently low compared to other countries which may be a contributory issue to the current navigation problems experienced on the Romania section. It is understood that there is an even smaller budget in Bulgaria.

Operational Objectives Satisfied:

This intervention would satisfy the following operational objectives:

OW1 – Improve availability of the Danube

OW2 – Improve Danube connectivity to reduce cost and time

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW9 – Reduce emissions of CO2 and Sulphur Dioxide

OW10 – Increase the amount of freight moved by sustainable modes

Implementing organisation:

AFDJ is responsible for the administration of the whole of the Romanian section of the Danube. AFDJ Galati’s main objective is to facilitate transport on the Danube with the aim of maintaining a minimum depth of 2.5m on the “fluvial” section and 8.5m on the “maritime” section. Other key responsibilities include monitoring depths, dredging, maintenance of navigation aids and information provision.

Implementation years:

Once the one-off dredging described in the previous section has been completed

Institutional Reform

The following are institutional reforms recommended by AECOM that, when enacted, should more water a more attractive mode of transport:

Management and Operations:

Authorities to be Flexible in Opening Times

Proposal Description:

Port authorities are to be flexible in opening times/days so that facilities such as customs, booking offices and terminals are available if demand exists.

Problems Addressed:

Port functions are sometimes unavailable when water freight operators require them, causing unnecessary cost and delays.

Operational Objectives Satisfied:

- This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW6 – Review existing assets to focus intervention to increase capacity

OW7 – Reduce procedural delays to water freight

Implementing organisation:

All port authorities

Implementation years:

Immediate

Transfer Ownership of Rail Infrastructure to Port Authorities

Proposal Description:

Rail infrastructure ownership should be transferred to port authorities where the port authority has shown a commitment to improving and upgrading the rail infrastructure.

Problems Addressed:

Rail infrastructure is currently owned and managed by CFR. However this infrastructure is not considered a priority for maintaining or upgrading. As such the current state of such infrastructure is poor in a number of ports such as Galati.

Operational Objectives Satisfied:

- This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW6 – Review existing assets to focus intervention to increase capacity

OW10 – Increase the amount of freight moved by sustainable modes

Implementing organisation:

All port authorities

Implementation years:

Immediate

Adopt a Commercial Attitude

Proposal Description

Adopt a commercial attitude and review port charges to operators to ensure water freight is competitive

Problems Addressed:

Outdated charging methods make water freight less competitive against other modes. Adopting a commercial attitude will address this problem.

Operational Objectives Satisfied:

- This intervention would satisfy the following operational objectives:

OW4 – Reduce costs through operational efficiency

OW7 – Reduce procedural delays to water freight

OW10 – Increase the amount of freight moved by sustainable modes

Implementing organisation:

All ports and waterways authorities

Implementation years:

Immediate

International Cooperation

Proposal Description:

Romania needs to work with other Danube countries to agree operational plans that will help to address the identified issues.

Problems Addressed:

The problems identified are:

- Differing maintenance budgets

- Differing rules on environmental standards

- Different systems are being used on the Danube

Operational Objectives Satisfied:

- This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW5 – Improve coordination between levels of government and enable investment

OW7 – Reduce procedural delays to water freight

OW10 – Increase the amount of freight moved by sustainable modes

Implementing organisation:

The Danube countries, in particular their respective Transport Ministries.

Implementation years:

Immediate

Training

Proposal Description:

Establish suitable training for the needs of the naval sector.

Problems Addressed:

There are currently between 50 and 100 accidents each year on the Romanian section of the Danube involving personal injury. The cost of these accidents in 2011 has been estimated to be €2.8m. Accidents that occur within the ports have significant impact due to a lack of protection measures and proper instruction to port employees. Proper training within a properly set up regulatory framework will help to reduce the frequency and severity of these accidents.

Operational Objectives Satisfied:

- This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW8 – Reduce number and cost of accidents on the Danube

Implementing organisation:

ANR, the Romanian Naval Authority, is responsible for the safety of civil navigation, the port administrators are responsible for issuing the licences for the port employees and both should be responsible for this, along with certified training providers.

Implementation years:

Immediate; although consideration should be given to the quality of the current training provided, along with its content, to assess its suitability.

Use Modern Systems

Proposal Description:

The industry should use modern and integrated systems for customs, navigation, regulation and administration. For example the DoRIS river information system is now available by APP for mobiles in Austria.

Problems Addressed:

Different countries are using different systems and devices. Romania should follow the consensus to avoid issues further down the line.

Operational Objectives Satisfied:

- This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW7 – Reduce procedural delays to water freight

OW8 – Reduce number and cost of accidents on the Danube

Implementing organisation:

The MT in its capacity as the overarching body for other organisation such as AFDJ.

Implementation years:

Consultation and cooperation with other Danube countries should begin immediately with a decision made on the systems to adopt made within five years.

Security on the Danube

Proposal Description:

Enhance security to reduce thefts from vessels and port facilities.

Problems Addressed:

Theft costs all parties involved time and money and seriously affects the reputation of water freight as a mode of transport. Enhancing security to prevent further losses will address this.

Operational Objectives Satisfied:

- This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

Implementing organisation:

Port authorities and water freight operators should all take responsibility to enhance security.

Implementation years:

Action should be taken immediately.

Safety and Regulation:

Review port and customs administration procedures

Proposal Description:

Review of the port procedures, the checking procedureson the state border crossing points, customs administration procedures in clearing goods and other procedures to be fulfilled on the import/export/transit of freight and to rationalise and improve communication.

Problems Addressed:

Bureaucracy is limiting the efficiency of water freight when compared against other modes. Reducing bureaucracy will address the negative impacts.

Operational Objectives Satisfied:

- This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW5 – Improve coordination between levels of government and enable investment

OW7 – Reduce procedural delays to water freight

Implementing organisation:

All port and customs authorities.

Implementation years:

Action to address this should be taken immediately.

Safety Training

Proposal Description:

Develop a safety plan including training to reduce accidents.

Problems Addressed:

Improved safety training will address the number of accidents that occur on the Danube.

Operational Objectives Satisfied:

- This intervention would satisfy the following operational objectives:

OW3 – Unblock barriers to operational efficiency

OW4 – Reduce costs through operational efficiency

OW8 – Reduce number and cost of accidents on the Danube

Implementing organisation:

ANR, the Romanian Naval Authority, is responsible for the safety of civil navigation and should be responsible for this, along with certified training providers.

Implementation years:

Immediate; although some consideration should be given to the quality of the current training provided, along with its content, to assess its suitability.

Summary of Interventions

This section summarises the interventions that were tested using the National Transport Model.

Table 6.60 – Summary of Tested Interventions

| Intervention | Reference | Location | Undiscounted costs (€, 2014 prices) | Benefits | Benefit to Cost Ratio | Economic Internal Rate of Return | Masterplan Recommendation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Build a new container terminal at Constanta (III & IVS) | 6.4.3 | Constanta | €351.1m | €452m | 2.02 | 8.9% |  |
| Development of a cargo terminal at the port | 6.4.12 | Tulcea | €10.15m | €6m | 0.59 | -1.1% |  |
| Improve the existing passenger ferry service running from Tulcea to Sulina | 6.4.20 | Tulcea | €19.2m | €28.2m | 1.47 | 4.4% |  |
| Build a new trimodal terminal | 6.4.28 | Galati | €14.4m | €17m | 0.94 | 4.7% |  |
| Modernisation of existing bulk terminals | 6.4.37 | Galati | €17.6m | €359m | 32.89 | 39.9% |  |
| Develop infrastructure | 6.4.47 | Cernavoda | €6.9m | €30m | 6.98 | 22.4% |  |
| Modernise the infrastructure | 6.4.54 | Basarabi | €3.51m | €1.63m | 0.46 | 1.2% |  |
| Develop bulk handling infrastructure | 6.4.61 | Oltenita | €5.6m | €103m | 29.35 | 30.1% |  |
| Build a new tri-modal terminal | 6.4.70 | Giurgiu | €6.8m | €5.8m | 0.56 | 1.4% |  |
| Infrastructure and modernisation works | 6.4.78 | Giurgiu | €4.3m | €48m | 17.53 | 24.3% |  |
| Modernisation and rehabilitation of infrastructure | 6.4.87 | Corabia | €4.5m | €31m | 11.04 | 25.1% |  |
| Build a new trimodal terminal | 6.4.104 | Drobeta Turnu Severin | €5.2m | €9.7m | 1.11 | 5.6% |  |
| Develop infrastructure | 6.4.112 | Drobeta Turnu Severin | €17.3m | €128m | 11.68 | 22.8% |  |
| Modernisation of the port | 6.4.122 | Orsova | €7.8m | €63m | 12.70 | 24.8% |  |
| Development of Infrastructure | 6.4.132 | Moldova Veche | €3.7m | €3m | 1.51 | 7.2% |  |
| Improvements to the navigation of the Danube | 6.4.142 | Danube | €136m | €777.7m | 4.70 | 19.6% |  |
| Create Bucharest - Danube Canal Connection | 6.4.150 | Danube-Bucharest | €939m | €891.1m | 0.95 | 4.7% |  |
| Improvements to the Sulina Channel | 6.4.167 | Sulina Channel | €20.5m | €84m | 1.55 | 9.51% |  |
| Recommended project that does not meet the threshold |  |  |  |  |  |  |  |
| Develop bulk handling infrastructure | 6.4.96 | Calafat | €20m | €7m | 0.60 | 2.4% |  |

    - Much of Romania’s port infrastructure was originally designed to handle heavy industry such as steel production. Romania’s economy is changing and these heavy industries are now in a managed decline and the consequence of this for the ports is that the ports now offer, to some degree, the wrong type of capacity. More products than ever before are now suitable for containerisation and freight forwarders prefer to use containers as it is an internationally accepted standard. As a consequence, the interventions suggested by this National Transport Master Plan for the ports focus on reconfiguring capacity at the ports to either accept containers (such as a trimodal terminal) or developing capacity for terminals that are close to maximum utilisation.

Table 6.61 – Summary of Tonnages for Tested Interventions (Ports only excluding intermodal and passenger traffic)

| Intervention | Reference | Location | 2011 Tonnes (Actual) | 2020 Tonnes (Reference Case) | 2020 Tonnes (Do Something) |
| --- | --- | --- | --- | --- | --- |
| Build a new container terminal at Constanta (III & IVS) | 6.4.3 | Constanta | 8,747,432 | 12,308,789 | 12,908,344 |
| Development of a cargo terminal at the port | 6.4.12 | Tulcea | 1,650,320 | 1,147,443 | 1,475,970 |
| Modernisation of existing bulk terminals | 6.4.37 | Galati | 5,099,714 | 5,823,027 | 6,441,528 |
| Develop infrastructure | 6.4.47 | Cernavoda | 131,833 | 249,360 | 614,711 |
| Modernise the infrastructure | 6.4.54 | Basarabi | 1,719,000 | 40,397 | 46,955 |
| Develop bulk handling infrastructure | 6.4.61 | Oltenita | 508,407 | 672,085 | 707,994 |
| Infrastructure and modernisation works | 6.4.78 | Giurgiu | 256,288 | 459,373 | 513,745 |
| Modernisation and rehabilitation of infrastructure | 6.4.87 | Corabia | 22,613 | 70,123 | 143,971 |
| Develop bulk handling infrastructure | 6.4.96 | Calafat | 139,105 | 156,016 | 167,887 |
| Develop infrastructure | 6.4.104 | Drobeta Turnu Severin | 490,112 | 594,367 | 702,769 |
| Modernisation of the port | 6.4.122 | Orsova | 187,752 | 291,081. | 357,127. |
| Development of Infrastructure | 6.4.1232 | Moldova Veche | 21,505 | 25,726 | 28,688 |

This section summarises additional interventions, which by their nature cannot be modelled using the national Transport Model, but which AECOM is recommending be included as part of the Masterplan.

Table 6.62 – Summary of other Recommended Interventions

| Reference | Intervention | Location |
| --- | --- | --- |
| W3 | Invest in ice breaking facilities including replacing Perseus | Danube |
| W4 | Increase maintenance budgets to match consistently across borders and implementing Danube maintenance | Danube |
| W5 | Authorities to be flexible in opening times/days so that facilities such as customs, booking offices and terminals are available if demand exists. | All ports |
| W6 | Adopt a commercial attitude and review port charges to operators to ensure water freight is competitive | All infrastructure |
| W7 | Romania to work with Danube countries to agree operational plans | NA |
| W8 | Review port and customs administration procedures and rationalise and improve communication | All ports |
| W9 | Establish suitable training for the needs of the naval sector | NA |
| W10 | Industry to use modern and integrated systems for customs, navigation, regulation and administration | NA |
| W11 | Develop a safety plan including training to reduce accidents | NA |
| W12 | Enhance security to reduce thefts from vessels and port facilities at Ports on the Primary Economic Network | All infrastructure and vessels |
| W13 | Reduce emissions from naval activity by adopting best practice: enforce EU regulations regarding emissions by non-EU vessels | NA |
| W14 | Safeguard land and facilities at minor and underutilised ports | All ports |
| W15 | Improve immediate local and internal road/rail connections | Galati, Calafat, Giurgiu, Drobeta, Braila |
| W16 | Transfer rail infrastructure ownership to ports | All ports |

Aviation

Aviation

Introduction

This section gives an overview of the existing conditions of the aviation sector within Romania. It summarises the air traffic trends of the past, describe the roles and classification of airports and sets out the future air passenger forecasts.  These forecasts are then used to assess the infrastructure future demand and capabilities of each airport.

Finally, the section sets out the strategic objectives for the aviation sector, the operational objectives and concludes with the interventions for each of the operational objectives.

Existing Conditions

Romania’s air transport infrastructure aims to provide regional centres with a means of fast transport to Bucharest, the capital, along with other regional centres. Air transport is also relied upon to provide international connectivity. As the country’s highway network remains in development, and even the fastest section of the rail network (currently between Bucharest and Constanta) are subject to speed restrictions, air travel is a preferred choice for longer distance domestic transport where fast journey times, rather than financial cost, are a critical factor.

A total of 21 airfields are listed in Romania’s Aeronautical Information Publication (AIP).  Fifteen of these airports currently have scheduled flight operations. In some cases these scheduled services may not be year-round but seasonal. Four airports are operated by the Romanian government (Henri Coanda – the main airport for Bucharest, Baneasa – Bucharest, Timisoara and Constanta), with all the other airports being operated by the local counties. Of the 21 aerodromes published in the AIP Romania, three are private property (Tuzla, Sibiu Magura and Vatra Dornei) and of these three only Tuzla has fares for commercial transport published in the AIP Romania.

The Romanian Civil Aeronautical Authority (RCAA) was established in 1993 as a self-financed and legal entity responsible for the safety oversight of the civil aviation industry in Romania and as an executive arm of the Directorate for Civil Aviation.  The functions of the RCAA are to ensure the application of the national regulations and to provide procedures and instructions for the application of these regulations.  The RCAA also act as the National Security Agency’s Competent Authority.

The Civil Aviation Safety Investigation and Analysis Centre (CIAS) is the specialised technical body to perform civil aviation safety investigations.  Romania ranks 4th in Europe and 10th in the world in the field of civil aviation safety.

Air Traffic Trends

Passenger numbers have grown from 7.8 mppa in 2007 to 10.8 mppa in 2011.

Figure 7.1: Historic Air Traffic Trends

Table 7.1 sets out the 2011 domestic and international air passenger numbers.

Table 7.1: 2011 Domestic and International Passenger Figures for Romanian Airports

| Airport | Domestic Passengers | International Passengers | TOTAL Passengers |
| --- | --- | --- | --- |
| Bucharest Henri Coanda | 649,682 | 6,670,884 | 7,320,566 |
| Timisoara | 336,152 | 1,019,867 | 1,356,019 |
| Cluj-Napoca | 189,139 | 815,682 | 1,004,821 |
| Bacau | 21,106 | 306,308 | 327,414 |
| Targu Mures | 10,477 | 216,361 | 226,838 |
| Iasi | 139,185 | 45,298 | 184,483 |
| Sibiu | 26,482 | 150,424 | 176,906 |
| Constanta | 11,647 | 64,817 | 76,464 |
| Oradea | 58,887 | 1,659 | 60,546 |
| Craiova | 19,397 | 11,872 | 31,269 |
| Suceava | 26,224 | 984 | 27,208 |
| Satu Mare | 19,534 | 3,207 | 22,741 |
| Baia Mare | 18,017 | 551 | 18,568 |
| Arad | 0 | 0 | 0 |
| Brasov | 0 | 0 | 0 |
| Tulcea | 0 | 0 | 0 |
| Bucharest Baneasa | 0 | 0 | 0 |
| Tuzla | 0 | 0 | 0 |
| TOTAL | 1,525,929 | 9,307,914 | 10,833,843 |

- Source: Actual flight departures by airport that were provided by the airport authority and used as baseline figures for the Air Forecasting Model

In 2011, a total of 10.8 million passengers travelled through all Romanian airports. Of this total, over 50% of passengers travelled through Bucharest’s Henri Coanda International Airport – showing the strong capital-centric nature of the country’s air transport system.

Comparisons could be drawn with Poland since the assession of Romania to the EU. The migrations of Poles to the United Kingdom and Ireland undoubtedly exerted a large influence on the development of low-cost travel in Poland. Incoming traffic via low-cost operators increased considerably, especially with the opening of routes to Central and Eastern Europe, especially Poland. This coincided with the growth of Eastern and Central European citizens taking up employment in the United Kingdom after the EU expansion.

The low-cost airlines operated routes to countries which were the main directions of the economic migration of Poles. Connections to the United Kingdom and Ireland formed almost 60% of Polish low-cost trips. In 2000 there were only five scheduled flights between the United Kingdom and Poland. Six years later, according to figures from the aviation regulator – the Civil Aviation Authority, 27 different connections between 12 Polish and British cities were identified.

Development of regional airports in Poland also began after accession to the EU. In 2004 there airports served 31.1% passengers. Their share in the number of passengers gradually increased year on year and in 2008 it amounted to 54.32%. This trend is already been seen in Romania with WizzAir’s new hub at Craiova Airport.

Role and Classification of Airports

In line with the Primary Economic Networks defined for road and rail modes, we believe that it is important to develop a hierarchy of airports within Romania, based on their roles and future potential to attract both international and domestic traffic.

In Romania the current airports fall into four categories, airports are allocated to category based on 2011 service patterns:

International Major Hub Airport (Bucharest Henri Coanda)

International Hub Airports (Timisoara and Cluj-Napoca)

Regional Airports (Bacau, Iasi, Sibiu and Targu Mures)

Smaller Regional Airports (Baia Mare, Constanta, Craiova, Oradea, Satu Mare, Suceava and Tulcea, Bucharest Baneasa, Tuzla)

Figure 7.1 illustrates the locations and 2011 classifications of the airports.

Source: AECOM Analysis

Figure 7.1: Location and 2011 Classification of Airports in Romania

The number of passenger trips through each of these airport categories is quite different and as such a change in role of an airport, with the implied step-change in flights offered at that airport, would have a major impact on the predicted air passenger demand. Using this principle and taking into account the variables above, it has been possible to construct a direct demand model for future airport patronage prediction.

The input data collated for each airport is as follows:

City and wider catchment population

Total number of flights split by domestic and international (2011 data)

Total number of passengers split by domestic and international (2011 data)

Car ownership rate in catchment area as an indicator of relative income levels

The future airport classification is based on the size of the catchment area and the service patterns (eg domestic/international and number of flights) of the existing classification.  Therefore if an airport changes it’s role and growths  into a higher classification airport, it has the potential of similar service patterns, and passenger trip rates, of an airport currently in that category.

Table 7.2 sets out the catchment population and the future classification. With a catchment population above 5 million population the airport is classified as a International Major Hub Airport, between 1 million and 5 million as an International Hub Airport, between 500,000 and 1 million as a Regional airport and below 500,000 as a Smaller Regional Airport.

Table 7.2: Airport catchment area and proposed future classification

| Airport | Catchment Population | Future Classification |
| --- | --- | --- |
| Bucharest | 6,170,897 | International Major Hub Airport |
| Craiova | 2,146,082 | International Hub Airport |
| Timisoara | 1,466,773 | International Hub Airport |
| Sibiu | 1,302,866 | International Hub Airport |
| Cluj | 1,031,213 | International Hub Airport |
| Bacau | 1,702,721 | International Hub Airport |
| Iasi | 1,040,752 | International Hub Airport |
| Targu Mures | 710,953 | Regional Airport |
| Oradea | 653,170 | Regional Airport |
| Brasov | 595,543 | Regional Airport |
| Constanta | 664,680 | Regional Airport |
| Suceava | 972,480 | Regional Airport |
| Tulcea | 911,819 | Regional Airport |
| Baia Mare | 640,587 | Regional Airport |
| Arad | 371,077 | Smaller Regional Airport |
| Satu Mare | 368,169 | Smaller Regional Airport |

Figure 7.2 gives an illustration of the catchment areas for the various airports as per the aviation forecasting model. For the purpose of estimating total demand,  one passenger cannot be in two catchment areas as that would cause double counting of demand, leading to potential over-provision of facilities.  In practice of course the boundaries are not rigid.

Tulcea

Tulcea

Figure 7.2: Current and Future Airport Locations and Catchment Areas

The proposed airports’ classifications are set out in Figure 7.3.  The structure and location of these airports gives good coverage of airports which should act as International Hubs, in different regions of the country, supported by a denser network of Regional Airports.

Source: AECOM Analysis

Figure 7.3: Location and Future Classification of Airports in Romania

Air Forecasts

As we indicated in paragraph 7.2.19, a bespoke aviation model has been developed to provide a basis forecasting future levels of air travel demand in Romania. Air passenger demand by airport is a function of:

The size of the catchment area of the airport in terms of population, and also attractions in terms of tourism;

The average income of the catchment area as higher income areas generate more air travel per capita;

The nature of the flights that are available from the airport in terms of the type, domestic/international, and the number of flights;

Future air demand has been shown to be strongly linked to GDP growth; and

The capacity of the airport in terms of flights that can be handled and the terminal capacity for passenger throughput.  The approach taken has been to estimate potential demand first, based on the future proposed role of the airport and the services that implies, and then assess the capacity shortfall.

The derivation of a model to predict future air demands therefore needs to take each of the above into consideration in developing the modal parameters and the forecasting processes. One of the main drivers of major changes in demand through an airport is due to a change in category of airport as a result of investment in facilities and success in attracting new airline operators to run new services from the airport.  It should be emphasized that the growth of an airport is not only dependent on the increase in capacity and facilities but depends on services committed to an airport.  No investment should be made unless there are appropriate assurances in place regarding future services, since these are the major determinants of demand.

A description of the model is provided in a separate Technical Note.

Tables 7.3 and 7.4 set out the 2020 and 2025 forecasts per airport, their TEN-T status and the proposed airport classification.

Table 7.3: Romanian Airports’ 2020 Forecasts, TEN-T Classification and Future AECOM Classification

| Airport | TEN-T Classification | 2020 Forecasts |  |  | AECOM |
| --- | --- | --- | --- | --- | --- |
|  | Core/ Comprehensive | Domestic | International | TOTAL | Classification |
| Bucharest | Core | 861,945 | 8,850,387 | 9,712,332 | International Major Hub Airport |
| Craiova | Comprehensive | 437,186 | 1,574,902 | 2,012,088 | International Hub Airport |
| Timisoara | Core | 444,687 | 1,349,157 | 1,793,844 | International Hub Airport |
| Sibiu | Comprehensive | 329,746 | 1,187,865 | 1,517,612 | International Hub Airport |
| Cluj | Comprehensive | 253,842 | 1,094,722 | 1,348,564 | International Hub Airport |
| Bacau | Comprehensive | 258,620 | 931,641 | 1,190,261 | International Hub Airport |
| Iasi | Comprehensive | 155,122 | 558,807 | 713,929 | International Hub Airport |
| Targu Mures | N/A | 149,948 | 540,166 | 690,114 | Regional Airport |
| Oradea | Comprehensive | 52,305 | 185,671 | 237,976 | Regional Airport |
| Brasov | N/A | 52,199 | 185,297 | 237,496 | Regional Airport |
| Constanta | Comprehensive | 51,479 | 182,740 | 234,219 | Regional Airport |
| Suceava | Comprehensive | 51,390 | 182,425 | 233,816 | Regional Airport |
| Tulcea | Comprehensive | 42,081 | 149,378 | 191,459 | Regional Airport |
| Baia Mare | Comprehensive | 39,405 | 139,882 | 179,287 | Regional Airport |
| Arad | N/A | 24,147 | 12,475 | 36,621 | Smaller Regional Airport |
| Satu Mare | N/A | 26,241 | 4,308 | 30,550 | Smaller Regional Airport |
|  |  | 3,094,350 | 16,877,817 | 19,972,167 |  |

Source AECOM Analysis

Table 7.4: Romanian Airports’ 2025 Forecasts, TEN-T Classification and Future AECOM Classification

| Airport | TEN-T Classification | 2025 Forecasts |  |  | AECOM |
| --- | --- | --- | --- | --- | --- |
|  | Core/ Comprehensive | Domestic | International | TOTAL | Classification |
| Bucharest | Core | 1,067,966 | 10,965,787 | 12,033,752 | International Major Hub Airport |
| Craiova | Comprehensive | 555,340 | 2,000,536 | 2,555,877 | International Hub Airport |
| Timisoara | Core | 549,561 | 1,667,339 | 2,216,900 | International Hub Airport |
| Sibiu | Comprehensive | 409,844 | 1,476,408 | 1,886,252 | International Hub Airport |
| Cluj | Comprehensive | 317,455 | 1,369,059 | 1,686,514 | International Hub Airport |
| Bacau | Comprehensive | 320,053 | 1,152,948 | 1,473,002 | International Hub Airport |
| Iasi | Comprehensive | 194,879 | 702,024 | 896,903 | International Hub Airport |
| Targu Mures | N/A | 186,371 | 671,377 | 857,748 | Regional Airport |
| Oradea | Comprehensive | 65,574 | 232,774 | 298,348 | Regional Airport |
| Brasov | N/A | 64,510 | 228,997 | 293,507 | Regional Airport |
| Constanta | Comprehensive | 64,220 | 227,967 | 292,186 | Regional Airport |
| Suceava | Comprehensive | 63,848 | 226,648 | 290,496 | Regional Airport |
| Tulcea | Comprehensive | 52,005 | 184,608 | 236,613 | Regional Airport |
| Baia Mare | Comprehensive | 49,402 | 175,368 | 224,771 | Regional Airport |
| Arad | N/A | 29,841 | 15,417 | 45,258 | Smaller Regional Airport |
| Satu Mare | N/A | 32,861 | 5,395 | 38,256 | Smaller Regional Airport |
|  |  | 3,854,703 | 20,989,429 | 24,844,132 |  |

Source AECOM Analysis

Figure 7.4 provides an illustration of the full growth compared to the factored past trends growth.

- Source: Actual flight departures by airport that were provided by the airport authority and used as baseline figures for the Air Forecasting Model

Figure 7.4: Potential Air Forecasts Compared with Trend Analysis

Airport Infrastructure, Future Demand and Capacities

Airside constraints – aprons, taxiways and runways

Table 7.5 sets out the assumptions in calculating the demand for apron stands.

Table 7.5: Assumptions in Determining Apron Space

| Airport Classification | Pax Per Code D Aircraft (70% load) | Code C Percentage Of Flights | Pax Per Code D Aircraft (70% load) | Code C Percentage Of Flights | Pax Per Code D Aircraft (90% load) | ATR Percentage Of Flights |
| --- | --- | --- | --- | --- | --- | --- |
| International Major Hub Airport | 210 | 70% | 126 | 20% | 45 | 10% |
| International Hub Airports | 210 | 50% | 126 | 40% | 45 | 10% |
| Regional Airports | 210 | 40% | 126 | 40% | 45 | 20% |
| Smaller Regional Airport | 210 | 0% | 126 | 10% | 45 | 90% |

The assumptions above were used to then determine the number of the various aircraft types within the peak hour. These peak hour requirements where then compare to the current number of aircraft stands to determine the shortfall in aircraft stands.

Table 7.6 sets out the current number of apron stands as well as the demand for 2020.  Again the peak hour passengers were used to determine the number of aircraft that will be required within the peak hour.  A 20% factor has been assumed (40% for Bucharest) and added to include aircraft from before or after the peak hour that will still occupy apron stands at the airport.

Table 7.6: Airports Peak Hour Apron Space Requirement (2020)

| Airport Name | Current Number of Apron Stands | Peak Hour Passengers | Code D Apron Stands | Code C Apron Stands | ATR Apron Stands | Total Number Of Stands | Shortfall Number Of Stands |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bucharest | 62 | 4,310 | 15 | 7 | 10 | 45 | 0¹ |
| Craiova | 12 | 1,786 | 5 | 6 | 4 | 18 | 6 |
| Timisoara | 26 | 1,592 | 4 | 6 | 4 | 17 | 0 |
| Sibiu | 16 | 1,347 | 4 | 5 | 3 | 15 | 0 |
| Cluj | 14 | 1,197 | 3 | 4 | 3 | 12 | 2 |
| Bacau | 4 | 1,056 | 3 | 4 | 3 | 12 | 8 |
| Iasi | 12 | 634 | 2 | 3 | 2 | 9 | 0 |
| Targu Mures | 9 | 817 | 2 | 3 | 4 | 11 | 2 |
| Oradea | 10 | 282 | 1 | 1 | 2 | 5 | 0 |
| Brasov | 0 | 281 | 1 | 1 | 2 | 5 | 0 |
| Constanta | 7 | 277 | 1 | 1 | 2 | 5 | 0 |
| Suceava | 6 | 277 | 1 | 1 | 2 | 5 | 0 |
| Tulcea | 4 | 227 | 1 | 1 | 2 | 5 | 1 |
| Baia Mare | 5 | 212 | 1 | 1 | 1 | 4 | 0 |
| Arad | 9 | 54 | 0 | 1 | 2 | 4 | 0 |
| Satu Mare | 2 | 45 | 0 | 1 | 1 | 3 | 1 |

- ¹ These figure does not include cargo apron stands

Table 7.7 sets out the apron stands demand for 2025.

Table 7.7: Airports Peak Hour Apron Space Requirement (2025)

| Airport Name | Current Number of Apron Stands | Peak Hour Passengers | Code D Apron Stands | Code C Apron Stands | ATR Apron Stands | Total Number Of Stands | Shortfall Number Of Stands |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bucharest | 62 | 5,340 | 18 | 9 | 12 | 55 | 0¹ |
| Craiova | 12 | 2,269 | 6 | 8 | 6 | 24 | 12 |
| Timisoara | 26 | 1,968 | 5 | 7 | 5 | 21 | 0 |
| Sibiu | 16 | 1,674 | 4 | 6 | 4 | 17 | 1 |
| Cluj | 14 | 1,497 | 4 | 5 | 4 | 16 | 2 |
| Bacau | 4 | 1,307 | 4 | 5 | 3 | 15 | 11 |
| Iasi | 12 | 796 | 2 | 3 | 2 | 9 | 0 |
| Targu Mures | 9 | 1,015 | 2 | 4 | 5 | 14 | 5 |
| Oradea | 10 | 353 | 1 | 2 | 2 | 6 | 0 |
| Brasov | 0 | 347 | 1 | 2 | 2 | 6 | 0 |
| Constanta | 7 | 346 | 1 | 2 | 2 | 6 | 0 |
| Suceava | 6 | 344 | 1 | 2 | 2 | 6 | 0 |
| Tulcea | 4 | 280 | 1 | 1 | 2 | 5 | 1 |
| Baia Mare | 5 | 266 | 1 | 1 | 2 | 5 | 0 |
| Arad | 9 | 67 | 0 | 1 | 2 | 4 | 0 |
| Satu Mare | 2 | 57 | 0 | 1 | 2 | 4 | 2 |

¹ This figure does not include cargo apron stands

From Table 7.7 it is clear that the following airports will run out of apron stand capacity by 2025:  Craiova, Cluj-Napoca, Bacau, Oradea, Tulcea and Satu Mare.

Table 7.8 sets out the Navigational Aids, Approach Lighting and Aircraft Rescue and Firefighting (AFFR) at the various airports.

Table 7.8 – Technical Characteristics of Romanian Airfields

| Airport Name | Highest PCN | Navigational Aids | Approach Lighting | ARFF Category |
| --- | --- | --- | --- | --- |
| Bucharest Henri Coanda | 78 R/D/W/T | NDB, DME, ILS Cat. III | Cat. I, ALSF-II | 9 |
| Timisoara | 42 R/B/W/T | NDB, DME, ILS Cat. III | Cat. I, ALSF-II | 8 |
| Cluj | 114 R/B/W/T | DVOR/DME, ILS Cat. II | Cat. I | 8 |
| Iasi | 99 F/D/W/T | NDB, DME, ILS Cat. II | Cat. III | 6 |
| Craiova | 29 R/A/W/T | DVOR/DME, ILS Cat. I | Cat. I | 7 |
| Bacau | 19 R/C/W/T | DVOR/DME, NDB | Cat. I | 6 |
| Constanta | 62 R/D/W/T | VOR/DME, NDB, ILS Cat. II | ALS II | 7 (8) |
| Targu Mures | 45 R/D/W/T | NDB, DME, ILS Cat. II | Cat. III | 7 |
| Sibiu | 56 R/D/W/T | DVOR/DME, NDB, ILS Cat. II | ALS II | 7 (8) |
| Oradea | 55 R/D/W/T | NDB, DME, ILS Cat. II | Cat. I | 6 |
| Brasov | Not open yet |  |  |  |
| Suceava | 65 R/D/W/T | DVOR/DME | Cat. I | 5 |
| Arad | 41 R/C/W/T | VOR, NDB | Cat. II | 7 |
| Satu Mare | 61 R/C/W/T | DVOR/DME, ILS Cat. II | Cat. I | 5 (7) |
| Baia Mare | 22 R/D/W/T | NDB, DME, ILS Cat. II | Cat. II | 5 (6) |
| Tulcea | 36 R/C/W/T | NDB(LO) | ALS-II, SALS | 5 (6) |

DVOR/DME – Doppler VHF Omni-directional Range     	ILS – Instrument Landing System

NDB – Non-directional Beacon				ALSF – Approach Lighting System with Sequenced Flashing Lights

ALS – Approach Lighting System

Figure 7.5 summarises the current situation regarding runway constraints. The restriction is primary due to the relatively low PCN of the airfields and the fact that 8 out of 14 are rated the lowest possible, in terms of the strength of the material underlying the pavement.  This does mean that it may be possible for an A310 (Code 4D aircraft) to be operated at greater number of airfield if restrictions were imposed (for example, if the payload or fuel weight was reduced).

Figure 7.5: Airports with Runway Constraints

Table 7.10 sets out the detailed dimensions of the longest runway at each airport (width and length), and the ICAO (International Civil Aviation Organization) reference code.  This code is calculated by examining the airport’s runway length (forming the number), as well as the maximum wing span and outer main gear wheel span of aircraft that the airfield can safely accommodate (forming the letter).  Table 7.11 details the precise technical characteristics taken into account in this calculation.

Table 7.10 - Romanian Airports Detailed Runway Characteristics and ICAO Reference Codes

| Airport Name | Longest Runway (Length x Width) in metre | ICAO Reference Code |
| --- | --- | --- |
| Bucharest Henri Coanda | 3,500 x 45 | 4E |
| Timisoara | 3,500 x 45 | 4D |
| Cluj | 2,100 x 45 | 4C |
| Iasi | 2,400 x 45 | 4D |
| Craiova | 2,500 x 45 | 4C |
| Bacau | 2,500 x 80 | 4D |
| Constanta | 3,500 x 45 | 4D |
| Targu Mures | 2,000 x 45 | 4C |
| Sibiu | 2,630 x 45 | 4D |
| Oradea | 2,100 x 30 | 4D |
| Brasov |  |  |
| Suceava | 2,460 x 45 | 4C |
| Arad | 2,000 x 45 | 4C |
| Satu Mare | 2,500 x 45 | 4D |
| Baia Mare | 1,790 x 30 | 3C |
| Tulcea | 2,000 x 30 | 4C |

Table 7.11 - Calculating an Airfield’s ICAO Reference Code

| Code Number | Reference Field Length (m) | Code Letter | Wing Span (m) | Outer Main Gear Wheel Span (m) |
| --- | --- | --- | --- | --- |
| 1 | < 800 | A | < 15 | < 4.5 |
| 2 | 800 < 1,200 | B | 15 < 24 | 4.5 < 6 |
| 3 | 1,200 < 1,800 | C | 24 < 36 | 6 < 9 |
| 4 | > 1,799 | D | 36 < 52 | 9 < 14 |
|  |  | E | 52 < 65 | 9 < 14 |
|  |  | F | 65 < 80 | 14 < 16 |

Table 7.12 - Airfield Requirements of Popular Commercial Aircraft

(Italics indicate a type currently operated by TAROM)

| Aircraft | ARFF Category | Aircraft Classification Number (ACN) |  |  |  | ICAO Ref. Code |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D |  |
| A310-300 | 8 | 40-47 | 48-56 | 57-66 | 65-75 | 4D |
| A318 | 6 | 31 | 34 | 36 | 38 | 3C |
| ATR42-500 | 4 | 10 | 11 | 12 | 12 | 3C |
| ATR72-500 | 5 | 13 | 14 | 14 | 15 | 3C |
| 737-300 | 6 | 40 | 42 | 44 | 46 | 4C |
| 737-700 | 6 | 43 | 46 | 48 | 50 | 4C |
| 737-800 | 7 | 51 | 53 | 55 | 57 | 4C |
| A320 | 6 | 19-46 | 23-49 | 27-51 | 31-53 | 4C |
| A321 | 7 | 47-56 | 50-59 | 52-62 | 54-64 | 4C |
| 757-200 | 7 | 32 | 38 | 45 | 52 | 4D |
| A330-200 | 8 | 48-53 | 56-61 | 66-73 | 78-85 | 4E |
| A330-300 | 9 | 46-54 | 54-62 | 64-74 | 75-86 | 4E |
| 767-300ER | 8 | 40 | 47 | 57 | 66 | 4D |
| 777-200ER | 9 | 50 | 63 | 82 | 101 | 4E |
| 747-400 | 9 | 53 | 62 | 74 | 85 | 4E |
| 747-8 | 10 | 64 | 71 | 89 | 112 | 4F |
| A380-800 | 10 | 55 | 64-67 | 76-88 | 88-110 | 4F |

While the ICAO reference code is able to offer a straightforward insight as to whether a particular aircraft would be able to operate at a particular airfield (by comparing the airfield’s reference code with the aircraft’s reference code), there are also further operational aspects which must be considered. Table 7.12 details the airfield requirements of a number of popular commercial aircraft.

The majority of low-cost carriers’ fleet include aircraft such as the A320 and B737-800.  Taken that into consideration Tulcea Airport’s runway needs to be upgraded, not only because of the length of the runway but the width of 30 metres put an additional restriction to the airport’s operations. The upgrade of Oradea Airport’s runway is part of the reference case and therefore not part of this master plan.

Although Cluj Airport has recently built a new runway, the intention is to have direct flight to the USA and therefore needs to extend the runway to 3,420m.

Passenger terminal capacity constraints.

In determining the passenger terminal capacity demand, we have analysed airport flight schedules to establish the number of passenger flights within the peak hour to calculate the peak hour capacity.  During our consultation process we sent out questionnaires to the various airports to get the current peak hour capacities of the airports. We then compared the existing peak hour capacities of the various airports with the current peak hour capacity demand to determine which airports have passenger terminal constraints.

Table 7.13 sets out the 2011 peak hour passenger terminal capacity as well as the peak hour capacity demand.

Table 7.13 – Airports Peak Hour Capacity (2011)

| Airport Name | Peak Hour Capacity (Passengers)¹ | Daily Passengers | Percentage of Flights within the Peak Hour | Peak Hour Capacity Demand |
| --- | --- | --- | --- | --- |
| Bucharest Henri Coanda | 4,500 | 20,056 | 15% | 3,000 |
| Timisoara | 1,200 | 3,715 | 36% | 1,337 |
| Cluj | 1,500 | 2,753 | 16% | 440 |
| Iasi | 300 | 505 | 22% | 111 |
| Craiova | 600 | 86 | 29% | 25 |
| Bacau | 200 | 897 | 30% | 269 |
| Constanta | 400 | 209 | 100% | 209 |
| Targu Mures | 580 | 980 | 40% | 392 |
| Sibiu | 300 | 485 | 13% | 63 |
| Oradea | 600 | 166 | 100% | 166 |
| Brasov | Not open yet |  |  |  |
| Suceava | 150 | 75 | 100% | 75 |
| Arad | 200 | 41 | 100% | 41 |
| Satu Mare | 200 | 62 | 100% | 62 |
| Baia Mare | 100 | 51 | 100% | 51 |
| Tulcea | Not available | 0 | 100% | 0 |

¹ Completed Questionnaires

To determine the peak hour passenger demand for the 2020 do nothing scenario we have reviewed the current number of flights within the peak hour at each type of airport (see Table 7.13). The peak hour demand is also dependant on the runway capacity of an airport. For the smaller regional airports the peak hour percentage seems high although some of these airports only have one or two flights per day. The following percentage of flights within the peak hour for each airport category is therefore assumed:

International Major Hub Airport – 15%

International Hub Airports – 30%

Regional Airports – 40%

Smaller Regional Airports – 50%

It is important to note that the busier an airport gets in terms of passenger and air traffic movements, the more spreadout the flights are during the day and hence a lower percentage of flights within the peak hours.

Table 7.14 sets out the Airport Passenger Terminal Demand for 2020.

Table 7.14 – Airports Peak Hour Passenger Terminal Capacity Requirements (2020)

| Airport Name | Peak Hour Capacity (Passengers)¹ | Daily Passengers¹ | Percentage of Flights within the Peak Hour | Peak Hour Passengers (Passenger Demand) |
| --- | --- | --- | --- | --- |
| Bucharest | 4,500 | 28,735 | 15% | 4,310 |
| Craiova | 600 | 5,953 | 30% | 1,786 |
| Timisoara | 1,200 | 5,307 | 30% | 1,592 |
| Sibiu | 300 | 4,490 | 30% | 1,347 |
| Cluj | 1,500 | 3,990 | 30% | 1,197 |
| Bacau | 200 | 3,521 | 40% | 1,056 |
| Iasi | 1,422 | 2,112 | 40% | 634 |
| Targu Mures | 580 | 2,042 | 40% | 817 |
| Oradea | 600 | 704 | 40% | 282 |
| Brasov | n/a | 703 | 40% | 281 |
| Constanta² | 400 | 693 | 40% | 277 |
| Suceava | 150 | 692 | 50% | 277 |
| Tulcea | 150 | 566 | 50% | 227 |
| Baia Mare | 100 | 530 | 50% | 212 |
| Arad | 200 | 108 | 50% | 54 |
| Satu Mare | 200 | 90 | 50% | 45 |

¹ Completed Questionnaires

² Constanta Airport Level of Service is below LOS C

From Table 7.14 it is clear that the following airports will run out of or reach the existing passenger terminal capacity by 2020:  Craiova, Timisoara, Sibiu, Bacau, Iasi, Targu Mures, Suceava, Tulcea, Baia Mare. Although Constanta Airport seems to be capable of handling the growth in passengers, the passenger building currently operates at a very low Level of Service and hence, for a Level of Service C would require a new terminal building. Table 7.15 sets out the Airport Passenger Terminal Demand for 2025.

Table 7.15 – Airports Peak Hour Passenger Terminal Capacity Requirements (2025)

| Airport Name | Peak Hour Capacity (Passengers)¹ | Daily Passengers | Percentage of Flights within the Peak Hour | Peak Hour Passengers (Passenger Terminal Demand) |
| --- | --- | --- | --- | --- |
| Bucharest | 4,500 | 35,603 | 15% | 5,340 |
| Craiova | 600 | 7,562 | 30% | 2,269 |
| Timisoara | 1,200 | 6,559 | 30% | 1,968 |
| Sibiu | 300 | 5,581 | 30% | 1,674 |
| Cluj | 1,500 | 4,990 | 30% | 1,497 |
| Bacau | 200 | 4,358 | 40% | 1,307 |
| Iasi | 1,422 | 2,654 | 40% | 796 |
| Targu Mures | 580 | 2,538 | 40% | 1,015 |
| Oradea | 600 | 883 | 40% | 353 |
| Brasov | n/a | 868 | 40% | 347 |
| Constanta | 400 | 864 | 40% | 346 |
| Suceava | 150 | 859 | 50% | 344 |
| Tulcea | 150 | 700 | 50% | 280 |
| Baia Mare | 100 | 665 | 50% | 266 |
| Arad | 200 | 134 | 50% | 67 |
| Satu Mare | 200 | 113 | 50% | 57 |

¹ Completed Questionnaires

From Table 7.15 it is clear that the following airports will run out of or reach the existing passenger terminal capacity by 2025:  Bucharest Henri Coanda, Craiova, Timisoara, Sibiu, Bacau, Iasi, Targu Mures, Suceava, Tulcea and Baia Mare.

For airports that will run out of passenger terminal capacity and grow from a Regional Airport into an International Hub Airport we propose the development of a new passenger terminal building instead of extending the existing building.  The existing passenger terminals could then be used for domestic passengers while the new passenger terminals could be designated for international passengers.

Availability of airports during extreme weather conditions.

According to TAROM, flights to Sibiu, Targo Mures and Suceava were discontinued from November 2013 due to the lack of infrastructure during winter months at these airports.  During the winter of 2012 only 40% of flights scheduled to these airports could land as a result of the combination of the lack of infrastructure and severe weather conditions.

Poor integration between local authorities and airport managers regarding the future planning for future expansion plans.

According to the Romanian Civil Aviation Authority, there is poor integration between local authorities and airport managers regarding the planning for future expansion of airports. Future developments around airports could potentially limit airport extensions.  Local plans do not always take into consideration future expansion plans of airport and hence developments permitted around airports jeopardise the future expansion of airports in terms of infrastructure development, safety zones and flight patterns. Air transport to, from and within Romania creates an economic benefit. By limiting airport expansion economic growth within the wider region will be compromised.

Surface access to and from airports.

There is currently a lack of good public transport links connecting urban areas and Bucharest Henri Coanda Airport with no direct rail, light rail or express bus connections to facilitate ease of movement. This coupled with the relatively long journey times between conurbations and the airport can create a significant access problem and could cause major delays.

The Master Plan supports the need for a dedicated fixed link connection between Henri Coanda and Bucharest City Centre, but a more detailed  study is required as part of Sustainable Urban Mobility Plan to determine the nature of  this link.

More detailed studies are also required for all International Hub Airports to determine the requirement for dedicated fixed links to these airports as part of the Sustainable Urban Mobility Plan. In the case of airports that are located close to a railway line (eg. Timisoara and Cluj), the possibility of linking the airport with the existing rail linkages should be investigated.

Lack of cargo terminal facilities/intermodal freight facilities

Few airports have any recognised cargo handling facility, with the exception of Henri Coanda International Airport, with most cargo hubs based in other European countries. This situation arises despite Romania’s central location between the Middle East, North Africa, Western Europe and Russia. Coupled with the poor transport connections, it means Romania becomes an end destination for air freight, rather than a hub.

A lack of cargo facilities could deter investments from companies that specialise in freight and logistics or manufacturing companies requiring complex or rapidly moving supply chains such as electronics or pharmaceuticals. It also limits the ability of airlines to spread their risk through the offer of air freight services both in terms of dedicated freighters and belly hold cargo.

Airport security

The European Commission has made the Security Industry one of the essential parts of the EU 2020 flagship intitiative ‘An Integrated Industrial Policy for the Globalisation Era Putting Competitiveness and Sustainability at Centre State’. Therein the Commission announced the launch of a dedicated initiative on a Security Industry Policy which includes the establishment of an EU wide harmonised certification system for airport screening (detection) equipment.

Interventions on aviation security therefore refer to security screening (detection) equipment for the hold and hand baggage to detecting prohibited items. Current equipment therefore need to be replaced to comply with this new European standards in aviation security.

Safety improvements for small, private and state aerodromes

To provide a harmonized territorial coverage that will allow the connection of all the regions for mail activities, business transport, medical emergency services and activities that are time constrained, the following safety improvements need to be implemented based on the criteria established by the Ministry of Transport by a state incentive scheme:

- Safety improvement on local interest aerodromes, certified, dedicated to small aviation (for MTOW aircraft < 5,700kg, VFR flights)

- Development/modernization of the local interest aerodromes certified, dedicated to small aviation (for MTOW planes < 5,700kg, VFR flights, concreate/asphalt runways by 900m placed in an unincorporated area and at a certain distance from any airport)

Strategic Objectives

The aviation sector in a country is directly linked the GDP, and hence an important part of a country’s economy.  One of the key objectives of the aviation industry is to provide a safe and secure mode of transport for passengers.  The aviation sector in Romania should provide a network of domestic routes within the country but also provide linkage between other European countries and the rest of the world.

Operational Objectives

The operational objectives for the aviation sector in Romania are set out below:

Ensuring that Bucharest Henri Coanda Airprot provides the infrastructure capacity and operational facilities required for the 2020 and 2025 demand.

Ensuring that Craiova airport provides the infrastructure capacity and operational facilities required for the 2020 and 2025 demand.

Ensuring that Timisoara Airport provides the infrastructure capacity and operational facilities required for the 2020 and 2025 demand.

Ensuring that Sibiu Airport provides the infrastructure capacity and operational facilities required for the 2020 and 2025 demand.

Ensuring that Cluj-Napoca Airport provides the infrastructure capacity and operational facilities required for the 2020 and 2025 demand.

Ensuring that Bacau Airport provides the infrastructure capacity and operational facilities required for the 2020 and 2025 demand.

Ensuring that Iasi Airport provides the infrastructure capacity and operational facilities required for the 2020 and 2025 demand.

Ensuring that Targu Mures Airport provides the infrastructure capacity and operational facilities required for the 2020 and 2015 demand.

Ensuring that Constanta Airport provides the infrastructure capacity and operational facilities required for the 2020 and 2025 demand.

Ensuring that Suceava Airport provides the infrastructure capacity and operational facilities required for the 2020 and 2025 demand.

Ensuring that Tulcea Airport provides the infrastructure capacity and operational facilities required for the 2020 and 2025 demand.

Ensuring that Baie Mare Airport provides the infrastructure capacity and operational facilities required for the 2020 and 2025 demand.

We support in principle the concept of providing modern air cargo facilities at the International Hub Airports based on the catchment area of the airports and the international services that should be attracted to such airports, but we cannot independently verify the economic justification for individual projects. This is because the factors determining the throughput are local in nature, and depend on the type of local industry and services, it’s distribution patterns (the balance of demostic Romanian and International trade), and the type of destinations served by air services in the future. Therefore, we have included air cargo terminals in the overall budget for improvements to these airports, where this need has been identified by the airport authorities (such as Iasi), or supported detailed feasibility studies where these are proposed, such as at Cluj. Details are given in the following sections.

It is important to note that the air sector differs from other modes in that demand is heavily dependent on both landside and airside facilities and services.  An airport without appropriate flights does not generate air traffic, but in turn an airline will not start services unless suitable ancillary facilities are available on the ground.  The relationship is symbiotic, and large numbers of passengers can be “generated” when the relationship is successful.

Interventions

To determine the costs for the passenger terminal extensions the peak hour demand were calculated from the demand forecasts. Various airports with a Level of Service C were reviewed to determine the space requirement of a terminal building per peak hour passenger. The total space requirements for each airport were then multiplied with a building cost of €2,500 per m².

To reach the operational objectives for the various airports, the following interventions are proposed:

Upgrade programme for Bucharest Henri Coanda Airport

Proposal Description

In line with the Primary Economic Networks defined for road and rail modes, it is important to develop a hierarchy for Romanian airports, based on their role and potential to attract both international and domestic traffic in the future. The airport classifications are:

International Major Hub Airport

International Hub Airport

Regional Airport

Small Regional Airport

According to the airport classification set out above, Bucharest is classified as the only International Major Hub Airport in Romania.

The upgrade project for Bucharest Henri Coanda aims to ensure that infrastructure capacity and operational facilities are able to deal with demand increase for 2020 and 2025. The project includes:

The development or extension of a new passenger terminal and infrastructure by 2025 in order to maintain a Class C Level of Service.

Extension of the taxiway system and development of 56 new apron stands by 2020.

Rehabilitation of apron, taxiway and runways.

New ground transportation connection

Analyse the opportunity to develop a cargo terminal.

Figure 7.6 sets out the location of Bucharest Henri Coanda Airport in relations to the other Romanian airports.

Figure 7.6: Location of Bucharest Henri Coanda Airport

Problems Addressed

This intervention addresses the following issues:

To facilitate growth in passenger numbers in order to maintain a level of service C.

The airport will run out of apron capacity by 2020.

International connectivity will generate increased freight flows.

Inadequate dedicated bus and rail links.

Undiscounted Costs

It has been estimated that the project will incur in the following costs

| Undiscounted costs (Million €, 2014 prices) |  | Description |
| --- | --- | --- |
| CAPEX | 247.3 | Includes: - Development of a new passenger terminal or extension of the current terminal - Construction of new apron stands and the rehabilitation and extension of existing apron stands - Extension of taxiway - New ground transport systems - Feasibility of developing a cargo terminal |
| OPEX | 12.4 | Additional operating costs |
| Total | 259.6 | (2020-25 projects combined) |

Outcomes

| PVC (€ (2014 Prices)) | 273,472,534 |
| --- | --- |
| PVB (€ (2014 Prices)) | 1,017,104,744 |
| NPV (€ (2014 Prices)) | 743,632,210 |
| BCR | 3.72 |
| EIRR | 12% |

In 2011 the airport handled 7,320,566 (649,682 domestic and 6,670,884 international) passengers per annum. If the upgrade programme is undertaken fully, passenger numbers could grow to 9,712,332 by 2020 (861,945 domestic and 8,850,387 international) and 12,033,753 by 2025 (1,067,966 domestic and 10,965,787 international).

Implementing Organisation

This scheme would be implemented by National Company Bucharest Airports.

Implementation Years

| Project | Implementation Year |
| --- | --- |
| Develop passenger terminal | 2025 |
| Extension of apron space | 2020 |
| Extension taxiway and apron space | 2025 |
| Rehabilitation of aprons, taxiways and runways | 2020 |
| New ground transport system | 2020 |

Upgrade programme for Craiova Airport

Proposal Description

In line with the Primary Economic Networks defined for road and rail modes, it is important to develop a hierarchy for Romanian airports, based on their role and potential to attract both international and domestic traffic in the future. The airport classifications are:

International Major Hub Airport

International Hub Airport

Regional Airport

Small Regional Airport

According to the current airports classification system, Craiova Airport is classified as a Small Regional Airport but due to its location and catchment area might have the opportunity to grow into and International Hub Airport in future. The upgrade project for Craiova Airport aims to ensure that infrastructure capacity and operational facilities are able to deal with demand increase for 2020 and 2025. The project includes:

Extension of passenger terminal by 2020

Extension of passenger terminal by 2025

The extension of apron stands by 2020

The extension of apron stands by 2025

Analyse the opportunity to develop a cargo terminal

Figure 7.7 sets out the location of Craiova Airport in relations to the other Romanian airports.

Figure 7.7: Location of Craiova Airport

Problems Addressed

This intervention addresses the following issues:

The airport will run out of passenger terminal capacity by 2020

The airport will run out of apron capacity by 2020

International connectivity will generate increased freight flows.

Undiscounted Costs

It has been estimated that the project will incur in the following costs

| Undiscounted costs (Million €, 2014 prices) |  | Description |
| --- | --- | --- |
| CAPEX | 17.3 | Includes: - Development of a new passenger terminal - Extension of existing apron stands - Feasibility of developing a cargo terminal |
| OPEX | 0.9 | Additional operating costs |
| Total | 18.2 | (2020-2025 projects combined) |

Outcomes

| PVC (€ (2014 Prices)) | 21,908,810 |
| --- | --- |
| PVB (€ (2014 Prices)) | 248,711,856 |
| NPV (€ (2014 Prices)) | 226,803,046 |
| BCR | 11.35 |
| EIRR | 40.3% |

In 2011 the airport handled 31,269 (19,397 domestic and 11,872 international) passengers per annum. If the upgrade programme is undertaken fully, passenger numbers could grow to 2,012,088 by 2020 (437,186 domestic and 1,574,902 international) and 2,555,876 by 2025 (555,340 domestic and 2,000,536 international).

Note: When the BCR is set to 3, the total annual passenger demand is 340,624 (74,011 domestic and 266,613 international) by 2020.

Implementing Organisations

This project would be implemented by Craiova Airport / Craiova Local Authority.

Implementation Years

| Project | Implementation Year |
| --- | --- |
| Extension of passenger terminal | 2020 |
| Extension of passenger terminal | 2025 |
| Extension of apron | 2020 |
| Extension of apron | 2025 |
| Analyse opportunity to develop a cargo terminal | 2020 |

Upgrade programme for Timisoara Airport

Proposal Description

In line with the Primary Economic Networks defined for road and rail modes, it is important to develop a hierarchy for Romanian airports, based on their role and potential to attract both international and domestic traffic in the future. The airport classifications are:

International Major Hub Airport

International Hub Airport

Regional Airport

Small Regional Airport

Timisoara Airport is classified as a International Hub Airport. The upgrade project for Timisoara Airport aims to ensure that infrastructure capacity and operational facilities are able to deal with demand increase for 2020 and 2025. The project includes:

Extension of passenger terminal by 2020

Extension of passenger terminal by 2025

Analyse the opportunity to develop a cargo terminal

Figure 7.8 sets out the location of Timisoara Airport in relations to the other Romanian airports.

Figure 7.8: Location of Timisoara Airport

Problems Addressed

This intervention addresses the following issues:

The airport will run out of terminal capacity by 2020

International connectivity will generate increased freight flows.

Undiscounted Costs

It has been estimated that the project will incur in the following costs

| Undiscounted costs (Million €, 2014 prices) |  | Description |
| --- | --- | --- |
| CAPEX | 76.5 | Includes: - Development of a new passenger terminal - Feasibility of developing a cargo terminal |
| OPEX | 4 | Additional operating costs |
| Total | 80.4 | (2020-2025 projects combined) |

Outcomes

| PVC (€ (2014 Prices)) | 98,719,402 |
| --- | --- |
| PVB (€ (2014 Prices)) | 165,568,470 |
| NPV (€ (2014 Prices)) | 66,849,068 |
| BCR | 1.68 |
| EIRR | 5.8% |

In 2011 the airport handled 1,356,019 (336,152 domestic and 1,019,867 international) passengers per annum. If the upgrade programme is undertaken fully, passenger numbers could grow to 1,793,844 by 2020 (444,687 domestic and 1,349,157 international) and 2,216,900 by 2025 (549,561 domestic and 1,667,339 international).

Implementing Organisations

This project would be implemented by Timisoara Airport / Ministry of Transport.

Implementation Years

| Project | Implementation Year |
| --- | --- |
| Extension of the passenger terminal | 2020 |
| Extension of the passenger terminal | 2025 |
| Analyse the opportunity of developing a cargo terminal | 2020 |

Upgrade programme for Sibiu Airport

Proposal Description

In line with the Primary Economic Networks defined for road and rail modes, it is important to develop a hierarchy for Romanian airports, based on their role and potential to attract both international and domestic traffic in the future. The airport classifications are:

International Major Hub Airport

International Hub Airport

Regional Airport

Small Regional Airport

Sibiu Airport is currently classified as a Regional Airport but is forecasted to develop into an International Hub Airport. The upgrade project for Sibiu Airport aims to ensure that infrastructure capacity and operational facilities are able to deal with demand increase for 2020 and 2025. The project includes:

Extension of passenger terminal by 2020

Extension of passenger terminal by 2025

Extension of taxiways by 2020

Analyse the opportunity to develop a cargo terminal by 2020

Upgrade lighting system from CAT I to CAT II by 2020

Acquire de-icing equipment by 2020

Install a beaconing system at the November Taxiway

Figure 7.9 sets out the location of Sibiu Airport in relations to the other Romanian airports.

Figure 7.9: Location of Sibiu Airport

Problems Addressed

This intervention addresses the following issues:

The airport will run out of terminal capacity by 2020

International connectivity will generate increased freight flows

In previous winters a significant proportion of flights scheduled to Sibiu have been unable to land due to a lack of infrastructure and severe weather conditions

Undiscounted Costs

It has been estimated that the project will incur in the following costs

| Undiscounted costs (Million €, 2014 prices) |  | Description |
| --- | --- | --- |
| CAPEX | 49.8 | Includes: - Development of a new passenger terminal - Extension of taxiway - Feasibility of developing a cargo terminal - Upgrade lighting system, install de-icing equipment and install a beaconing system |
| OPEX | 2.5 | Additional operating costs |
| Total | 52.3 | (2020-2025 projects combined) |

Outcomes

| PVC (€ (2014 Prices)) | 63,803,895 |
| --- | --- |
| PVB (€ (2014 Prices)) | 651,827,026 |
| NPV (€ (2014 Prices)) | 588,023,131 |
| BCR | 10.22 |
| EIRR | 34.9% |

In 2011 the airport handled 176,906 (26,482 domestic and 150,424 international) passengers per annum. If the upgrade programme is undertaken fully, passenger numbers could grow to 1,517,611 by 2020 (329,756 domestic and 1,187,865 international) and 1,886,252 by 2025 (409,844 domestic and 1,476,408 international).

Implementing Organisations

This project would be implemented by Sibiu Airport.

Implementation Years

| Project | Implementation Year |
| --- | --- |
| Extension of Passenger Terminal | 2020 |
| Extension of Passenger Terminal | 2025 |
| Analyse the opportunity of developing a cargo terminal | 2020 |
| Upgrade from CAT I to CAT II lighting system | By 2020 |
| Acquire de-icing equipment | By 2020 |
| Installing of beaconing system at the November Taxiway | 2020 |

Upgrade programme for Cluj-Napoca Airport

Proposal Description

In line with the Primary Economic Networks defined for road and rail modes, it is important to develop a hierarchy for Romanian airports, based on their role and potential to attract both international and domestic traffic in the future. The airport classifications are:

International Major Hub Airport

International Hub Airport

Regional Airport

Small Regional Airport

Cluj-Napoca Airport is classified as a International Hub Airport. The upgrade project for Cluj-Napoca Airport aims to ensure that infrastructure capacity and operational facilities are able to deal with demand increase for 2020 and 2025. The project includes:

Expansion of the runway and two turning bays

Expansion/construction of new taxiways and apron space

Analyse the opportunity to develop a cargo terminal

Figure 7.10 sets out the location of Cluj-Napoca Airport in relations to the other Romanian airports.

Figure 7.10: Location of Cluj-Napoca Airport

Problems Addressed

This intervention addresses the following issues:

The airport will run out of apron capacity by 2020

International connectivity will generate increased freight flows

Although not needed at the moment, direct flights to USA are proposed but the current runway capacity is not sufficient to support this.

Undiscounted Costs

It has been estimated that the project will incur in the following costs

| Undiscounted costs (Million €, 2014 prices) |  | Description |
| --- | --- | --- |
| CAPEX | 82.8 | Includes: - Construction of new aprons and taxiways - Analyse opportunity to develop a cargo terminal - Extenstion of runway with two turning bays |
| OPEX | 4.1 | Additional operating costs |
| Total | 87 | (2020-2025 projects combined) |

Outcomes

| PVC (€ (2014 Prices)) | 108,418,495 |
| --- | --- |
| PVB (€ (2014 Prices)) | 136,137,415 |
| NPV (€ (2014 Prices)) | 27,718,920 |
| BCR | 1.26 |
| EIRR | 4.6% |

In 2011 the airport handled 1,004,821 (189,139 domestic and 815,682 international) passengers per annum. If the upgrade programme is undertaken fully, passenger numbers could grow to 1,348,564 by 2020 (253,842 domestic and 1,094,722 international) and 1,686,515 by 2025 (317,455 domestic and 1,369,059 international).

Implementing Organisations

This project would be implemented by Cluj-Napoca Airport.

Implementation Years

| Project | Implementation Year |
| --- | --- |
| New taxiway and expansion of apron space | 2020 |
| Expansion of apron space | 2025 |
| Analyse the opportunity of developing a cargo terminal | 2020 |

Upgrade programme for Bacau Airport

Proposal Description

In line with the Primary Economic Networks defined for road and rail modes, it is important to develop a hierarchy for Romanian airports, based on their role and potential to attract both international and domestic traffic in the future. The airport classifications are:

International Major Hub Airport

International Hub Airport

Regional Airport

Small Regional Airport

Bacau Airport is currently classified as a Regional Airport but could be developed into an International Hub Airport due to its catchment area. The upgrade project for Bacau Airport aims to ensure that infrastructure capacity and operational facilities are able to deal with demand increase for 2020 and 2025. The project includes:

Extension of passenger terminal

Expansion of apron space

Increase runway bearing capacity

Analyse the opportunity to develop a cargo terminal

Figure 7.11 sets out the location of Bacau Airport in relations to the other Romanian airports.

Figure 7.11: Location of Bacau Airport

Problems Addressed

This intervention addresses the following issues:

The airport will run out of terminal capacity by 2020

The airport will run out of apron space by 2020

The runway has a very low PCN value and needs to be upgraded to handle more flights

International connectivity will generate increased freight flows

Undiscounted Costs

It has been estimated that the project will incur in the following costs

| Undiscounted costs (Million €, 2014 prices) |  | Description |
| --- | --- | --- |
| CAPEX | 85 | Includes: - Development of a new passenger terminal - Expansions of apron space - Increase runway bearing capacity - Analyse opportunity to develop a cargo terminal |
| OPEX | 4.3 | Additional operating costs |
| Total | 89.3 | (2020-2025 projects combined) |

Outcomes

| PVC (€ (2014 Prices)) | 110,017,027 |
| --- | --- |
| PVB (€ (2014 Prices)) | 367,635,291 |
| NPV (€ (2014 Prices)) | 257,618,264 |
| BCR | 3.34 |
| EIRR | 11.0% |

In 2011 the airport handled 327,414 (21,106 domestic and 306,308 international) passengers per annum. If the upgrade programme is undertaken fully, passenger numbers could grow to 1,190,261 by 2020 (258,620 domestic and 931,641 international) and 1,473,001 by 2025 (320,053 domestic and 1,152,948 international).

Implementing Organisations

This project would be implemented by Bacau Airport.

Implementation Years

| Project | Implementation Year |
| --- | --- |
| Extension of Passenger Terminal | 2020 |
| Extension of Passenger Terminal | 2025 |
| Expansion of Apron space | 2020 |
| Expansion of Apron space | 2025 |
| Increase the bearing capacity of the runway | 2020 |
| Analyse the opportunity of developing a cargo terminal | 2020 |

Upgrade programme for Iasi Airport

Proposal Description

In line with the Primary Economic Networks defined for road and rail modes, it is important to develop a hierarchy for Romanian airports, based on their role and potential to attract both international and domestic traffic in the future. The airport classifications are:

International Major Hub Airport

International Hub Airport

Regional Airport

Small Regional Airport

Iasi Airport is currently classified as a Regional Airport but has the potential to be developed into an International Hub Airport. TAROM Airlines is already using Iasi Airport as their second hub. The upgrade project for Iasi Airport aims to ensure that infrastructure capacity and operational facilities are able to deal with demand increase for 2020 and 2025. The project includes:

Analyse the opportunity to develop a cargo terminal

Figure 7.12 sets out the location of Iasi Airport in relations to the other Romanian airports.

Figure 7.12: Location of Iasi Airport

Problems Addressed

This intervention addresses the following issues:

International connectivity will generate increased freight flows

Undiscounted Costs

It has been estimated that the project will incur in the following costs

| Undiscounted costs (Million €, 2014 prices) |  | Description |
| --- | --- | --- |
| CAPEX | 5.1 | Includes: - Analyse opportunity to develop a cargo terminal |
| OPEX | 0.3 | Additional operating costs |
| Total | 5.4 |  |

Outcomes

| PVC (€ (2014 Prices)) | 6,674,212 |
| --- | --- |
| PVB (€ (2014 Prices)) | 78,948,864 |
| NPV (€ (2014 Prices)) | 64,765,707 |
| BCR | 5.57 |
| EIRR | 17.4% |

In 2011 the airport handled 184,483 (139,185 domestic and 45,298 international) per annum. If the upgrade programme is undertaken fully, passenger numbers could grow to 713,729 by 2020 (155,122 domestic and 558,807 international) and 896,903 by 2025 (194,879 domestic and 702,024 international).

Implementing Organisations

This project would be implemented by Iasi Airport.

Implementation Years

| Project | Implementation Year |
| --- | --- |
| Analyse the opportunity of developing a cargo terminal | 2020 |

Upgrade programme for Targu Mures Airport

Proposal Description

In line with the Primary Economic Networks defined for road and rail modes, it is important to develop a hierarchy for Romanian airports, based on their role and potential to attract both international and domestic traffic in the future. The airport classifications are:

International Major Hub Airport

International Hub Airport

Regional Airport

Small Regional Airport

Targu Mures Airport is classified as a Regional Airport. The upgrade project for Targu Mures Airport aims to ensure that infrastructure capacity and operational facilities are able to deal with demand increase for 2020 and 2025. The project includes:

Extension of apron

Upgrade lighting system

Acquire de-icing equipment

Figure 7.13 sets out the location of Targu Mures Airport in relations to the other Romanian airports.

Figure 7.13: Location of Targu Mures Airport

Problems Addressed

This intervention addresses the following issues:

The airport will run out of apron capacity by 2020

In previous winters a significant proportion of scheduled flights have been unable to land due to inadequate infrastructure and severe weather conditions

Undiscounted Costs

It has been estimated that the project will incur in the following costs

| Undiscounted costs (Million €, 2014 prices) |  | Description |
| --- | --- | --- |
| CAPEX | 15.6 | Includes: - Upgrade lighting system and acquire de-icing equipment - Development of a new terminal building |
| OPEX | 0.8 | Additional operating costs |
| Total | 16.4 | (2020-2025 projects combined) |

Outcomes

| PVC (€ (2014 Prices)) | 21,183,421 |
| --- | --- |
| PVB (€ (2014 Prices)) | 179,020,630 |
| NPV (€ (2014 Prices)) | 157,837,209 |
| BCR | 8.45 |
| EIRR | 53.5% |

In 2011 the airport handled 226,838 (10,477 domestic and 216,361 international) passengers per annum. If the upgrade programme is undertaken fully, passenger numbers could grow to 302,113 by 2020 (13,954 domestic and 288,159 international) and 375,498 by 2025 (17,343 domestic and 358,155 international).

Implementing Organisations

This project would be implemented by Bacau Airport.

Implementation Years

| Project | Implementation Year |
| --- | --- |
| Extension of apron at Targu Mures | 2020 |
| Extension of apron at Targu Mures | 2025 |
| Extension of passenger terminal | 2020 |
| Extension of passenger terminal | 2025 |
| Upgrade of lighting system and navigational aids | 2020 |
| Acquire de-icing equipment | 2020 |

Upgrade programme for Constanta Airport

Proposal Description

In line with the Primary Economic Networks defined for road and rail modes, it is important to develop a hierarchy for Romanian airports, based on their role and potential to attract both international and domestic traffic in the future. The airport classifications are:

International Major Hub Airport

International Hub Airport

Regional Airport

Small Regional Airport

Constanta Airport is currently classified as a Small Regional Airport but has the potential to be developed into a Regional Airport. The upgrade project for Constanta Airport aims to ensure that infrastructure capacity and operational facilities are able to deal with demand increase for 2020 and 2025. The project includes turning facilities at Threshold 18.

Figure 7.14 sets out the location of Constanta Airport in relations to the other Romanian airports.

Figure 7.14: Location of Constanta Airport

Problems Addressed

This intervention addresses the following issues:

Due to the fact that the airport is also used for military operations the turning facility will increase the number of traffic movements.

Undiscounted Costs

It has been estimated that the project will incur in the following costs

| Undiscounted costs (Million €, 2014 prices) |  | Description |
| --- | --- | --- |
| CAPEX | 9.3 | Includes: - Turning facilities at Threshold 18 - New Passenger Terminal Building |
| OPEX | 0.5 | Additional operating costs |
| Total | 9.8 |  |

Outcomes

| PVC (€ (2014 Prices)) | 12,682,073 |
| --- | --- |
| PVB (€ (2014 Prices)) | 9,635,658 |
| NPV (€ (2014 Prices)) | -3,046,415 |
| BCR | 0.76 |
| EIRR | 2.1% |

In 2011 the airport handled 76,464 (11,647 domestic and 64,817 international) passengers per annum. If the upgrade programme is undertaken fully, passenger numbers could grow to 234,219 (51,479 domestic and 182,740 international) and 292,187 by 2025 (64,220 domestic and 227,967 international).

Implementing Organisations

This project would be implemented by Constanta Airport.

Implementation Years

| Project | Implementation Year |
| --- | --- |
| Construction of a turning facility at Threshold 18 | 2020 |
| Extension of passenger terminal | 2020 |
| Extension of passenger terminal | 2025 |

Upgrade programme for Suceava Airport

Proposal Description

In line with the Primary Economic Networks defined for road and rail modes, it is important to develop a hierarchy for Romanian airports, based on their role and potential to attract both international and domestic traffic in the future. The airport classifications are:

International Major Hub Airport

International Hub Airport

Regional Airport

Small Regional Airport

Suceava Airport is currently classified as a Small Regional Airport but has the potential to be developed into a Regional Airport. The upgrade project for Suceava Airport aims to ensure that infrastructure capacity and operational facilities are able to deal with demand increase for 2020 and 2025. The project includes:

Extension of passenger terminal

Acquire de-icing equipment

Figure 7.15 sets out the location of Suceava Airport in relations to the other Romanian airports.

Figure 7.15: Location of Suceava Airport

Problems Addressed

This intervention addresses the following issues:

The airport will run out of terminal capacity by 2020

In previous winters a significant proportion of scheduled flights have been unable to land due to inadequate infrastructure and severe weather conditions

Undiscounted Costs

It has been estimated that the project will incur in the following costs

| Undiscounted costs (Million €, 2014 prices) |  | Description |
| --- | --- | --- |
| CAPEX | 4 | Includes: - Expansion of terminal - Acquire de-icing equipment - Acquire and install safety equipment |
| OPEX | 0.2 | Additional operating costs |
| Total | 4.2 |  |

Outcomes

| PVC (€ (2014 Prices)) | 4,888,167 |
| --- | --- |
| PVB (€ (2014 Prices)) | 38,824,803 |
| NPV (€ (2014 Prices)) | 33,936,637 |
| BCR | 7.94 |
| EIRR | 30.6% |

In 2011 the airport handled 27,228 (26,244 domestic and 984 international) passengers per annum. If the upgrade programme is undertaken fully, passenger numbers could grow to 233,815 by 2020 (51,390 domestic and 182,425 international) and 290,496 by 2025 (63,848 domestic and 226,648 international).

Implementing Organisations

This project would be implemented by Suceava Airport.

Implementation Years

| Project | Implementation Year |
| --- | --- |
| Extension of Passenger Terminal | 2020 |
| Extension of Passenger Terminal | 2025 |
| Acquire de-icing equipment | 2020 |
| Acquire and install safety equipment | 2020 |

Upgrade programme for Tulcea Airport

Proposal Description

In line with the Primary Economic Networks defined for road and rail modes, it is important to develop a hierarchy for Romanian airports, based on their role and potential to attract both international and domestic traffic in the future. The airport classifications are:

International Major Hub Airport

International Hub Airport

Regional Airport

Small Regional Airport

Tulcea Airport is currently classified as a Small Regional Airport but has the potential to be developed into a Regional Airport. The upgrade project for Tulcea Airport aims to ensure that infrastructure capacity and operational facilities are able to deal with demand increase for 2020 and 2025. The project includes:

Extension of passenger terminal

Extension of apron

Upgrading of the runway and taxiways

Figure 7.16 sets out the location of Tulcea Airport in relations to the other Romanian airports.

Figure 7.16: Location of Tulcea Airport

Problems Addressed

This intervention addresses the following issues:

The airport will run out of terminal capacity by 2020

The airport will run out of apron capacity by 2020

Undiscounted Costs

It has been estimated that the project will incur in the following costs

| Undiscounted costs (Million €, 2014 prices) |  | Description |
| --- | --- | --- |
| CAPEX | 15.3 | Includes: - Extension of passenger terminal - Extension of apron space - Upgrading of runway |
| OPEX | 0.8 | Additional operating costs |
| Total | 15.4 |  |

Outcomes

| PVC (€ (2014 Prices)) | 20,100,174 |
| --- | --- |
| PVB (€ (2014 Prices)) | 30,690,216 |
| NPV (€ (2014 Prices)) | 10,590,042 |
| BCR | 1.53 |
| EIRR | 5% |

In 2011 the airport had no scheduled flight, during recent years airlines have started to fly from Tulcea. If the upgrade programme is undertaken fully, passenger numbers could grow to 191,459 by 2020 (42,081 domestic and 149,378 international) and 236,613 by 2025 (52,005 domestic and 184,608 international).

Implementing Organisations

This project would be implemented by Tulcea Airport.

Implementation Years

| Project | Implementation Year |
| --- | --- |
| Extension of passenger terminal | 2020 |
| Extension of passenger terminal | 2025 |
| Expansion of apron space | 2020 |
| Upgrading of runway and taxiways | 2020 |

Upgrade programme for Baia Mare Airport

Proposal Description

In line with the Primary Economic Networks defined for road and rail modes, it is important to develop a hierarchy for Romanian airports, based on their role and potential to attract both international and domestic traffic in the future. The airport classifications are:

International Major Hub Airport

International Hub Airport

Regional Airport

Small Regional Airport

Baia Mare Airport is currently classified as a Small Regional Airport but have the potential to be developed into a Regional Airport. The upgrade project for Baia Mare Airport aims to ensure that infrastructure capacity and operational facilities are able to deal with demand increase for 2020 and 2025. The project includes:

Extension of passenger terminal

Figure 7.17 sets out the location of Baia Mare Airport in relations to the other Romanian airports.

Figure 7.17: Location of Baia Mare Airport

Problems Addressed

This intervention addresses the following issues:

The airport will run out of terminal capacity by 2020

Undiscounted Costs

It has been estimated that the project will incur in the following costs

| Undiscounted costs (Million €, 2014 prices) |  | Description |
| --- | --- | --- |
| CAPEX | 2.3 | Includes: - Extension of terminal |
| OPEX | 0.1 | Additional operating costs |
| Total | 2.4 |  |

Outcomes

| PVC (€ (2014 Prices)) | 2,769,760 |
| --- | --- |
| PVB (€ (2014 Prices)) | 7,698,694 |
| NPV (€ (2014 Prices)) | 4,928,934 |
| BCR | 2.78 |
| EIRR | 10.4% |

In 2011 the airport handled 18,568 (18,017 domestic and 551 international) passengers per annum. If the upgrade programme is undertaken fully, passenger numbers could grow to 179,287 by 2020 (39,405 domestic and 139,882 international) and 224,770 by 2025 (49,402 domestic and 175,368 international).

Implementing Organisations

This project would be implemented by Tulcea Airport.

Implementation Years

| Project | Implementation Year |
| --- | --- |
| Extension of Passenger Terminal | 2020 |
| Extension of Passenger Terminal | 2025 |

Results of Testing

Table 7.16 and 7.17 sets out the project testing results

Table 7.16: 2020 Project Testing Results

| Airport | Type of Projects | Year to be Implemented | Cost 2014 prices (Euros) (CAPEX) | OPEX 2014 prices (Euros) | BCR |
| --- | --- | --- | --- | --- | --- |
| Bucharest Henri Coanda | Rehabilitation of aprons, taxiways, runway andextension of apron | 2020 | 93,804,760 | 4,690,238 | 3.72 |
| Craiova | Extension of apron and passenger terminal, intermodal cargo terminal | 2020 | 14,771,881 | 738,594 | 11.35 |
| Timisoara | Passenger Terminal, Intermodal Cargo Terminal | 2020 | 71,326,200 | 3,566,310 | 1.68 |
| Sibiu | Passenger Terminal, Lighting, De-Icing Equipment and Taxiway extensions | 2020 | 45,084,727 | 2,254,236 | 10.22 |
| Cluj-Napoca | Runway extension, Apron and Taxiway extensions; Movement Surfaces , cargo terminal | 2020 | #### 81,000,000 | #### 4,099,000 | 1.26 |
| Bacau | Passenger terminal, runway bearing capacity, apron, cargo terminal | 2020 | 80,190,916 | 4,009,546 | 3.34 |
| Iasi | Intermodal cargo terminal | 2020 | 10,800,000 | 540,000 | 5.57 |
| Targu Mures | Apron extensions, passenger terminal, Lighting and De-icing Equipment | 2020/2025 | 8,323,221 | 416,161 | 8.45 |
| Constanta | Runway turning facilities, passenger terminal | 2020 | 5,086,484 | 254,324 | 0.76 |
| Suceava | De-icing and safety equipment | 2020 | 2,990,985 | 149,549 | 7.94 |
| Tulcea | Extension of Passenger Terminal and Apron | 2020 | 15,177,064 | 768,000 | 1.53 |
| Baia Mare | Extension of Passenger Terminal | 2020 | 1,553,800 | 77,690 | 3.13 |

Table 7.17: 2025 Project Testing Results

| Airport | Type of Projects | Year to be Implemented | Cost 2014 prices (Euros) (CAPEX) | OPEX 2014 prices (Euros) | BCR |
| --- | --- | --- | --- | --- | --- |
| Bucharest Henri Coanda | Rehabilitation of aprons, taxiways, runway; New Passenger Terminal, Intermodal cargo platform | 2025 | 153,459,400 | 7,672,970 | 3.72 |
| Craiova | Passenger Terminal, Extension of apron | 2025 | 2,574,068 | 128,703 | 11.35 |
| Timisoara | Passenger Terminal | 2025 | 5,215,822 | 260,791 | 1.68 |
| Sibiu | Passenger Terminal | 2025 | 4,729,940 | 236,497 | 10.22 |
| Cluj-Napoca | Apron extensions | 2025 | 829,761 | 41,488 | 2.74 |
| Bacau | Passenger terminal, apron extension | 2025 | 4,872,424 | 243,621 | 3.34 |
| Targu Mures | Apron extensions, passenger terminal | 2025 | 7,271,322 | 363,566 | 8.45 |
| Constanta | Passenger terminal | 2025 | 4,256,882 | 212,844 | 0.76 |
| Suceava | Expansion of Passenger Terminal | 2025 | 982,613 | 49,131 | 7.94 |
| Tulcea | Extension of Passenger Terminal and Apron | 2025 | 185,468 | 9,273 | 10.78 |
| Baia Mare | Extension of Passenger Terminal | 2025 | 745,592 | 37,280 | 3.13 |

Due to the high volatility in strong dependence of the operating decisions of the airlines, a cautious approach would be to limit the time horizon of the master plan interventions to be taken up for preparation/implementation to the year 2020, and that any further interventions beyond 2020 be confirmed on the basis of the actual market developments.

Intermodal Transport

Intermodal Transport

Existing Conditions

Before the recession the global logistics industry was estimated at roughly EUR 5.4 trillion, or 13.8% of the global Gross Domestic Product (GDP).

On average, logistics costs account for 10-15% of the final cost of the finished product. Estimates made in 2007 put the share of the logistics industry in Romania at 9.7% of GDP, which is around 50% lower than the average in Europe (close to 14% of GDP). (Source: “Freight Transport Logistics Action Plan”, COM (2007) final, Commission of the European Communities.)

The conclusion is that either the cost base in Romania is much lower than in other EU countries or the logistics industry is not well developed and hence is not serving the economy as well as it could. Our understanding is that both factors are true. Based on the same principles of cost, time and risk reduction, logistics supply chains seek to route goods along “the path of the least resistance”. The countries that minimise the barriers to trade adopt simple and efficient customs procedures and have good transport infrastructure and efficient terminals that attract freight movements.

Containerisation has grown rapidly over the last fifty years and has replaced traditional forms of shipping for many types of commodity but especially manufactured and consumer goods. Trade patterns have also changed during this period and now 70% of all containerised cargo coming to Europe originates mainly from Asia. Romania is in strong competition with many countries in South and Eastern Europe to become a preferred location for feeding goods into the European hinterland, due to its proximity to the Suez Canal in sailing time. Romania is also well placed to attract inward investment especially for the manufacturing assembly and related logistics activities due to relatively low labour rates and being located on the east-west trade route.

Aside from containerised traffic the position of Romania, and the Port of Constanta, on the TRACECA corridor (“Silk route”) puts the country and the port in a good position to operate Ro-Ro services across the Black Sea (e.g. Turkey and Georgia), especially given the time and environmental savings that can be made by these connections. This development could strengthen the competitive position of the Port of Constanţa as a hub serving the Black Sea region. The increasing economic relationships between Europe and the Caucasus region, means that Romania could have an increasingly key role to play as a transport hub on this important trade route. The important point is whether Romania is seen by shippers and freight forwarders as being on a path of least resistance or in other words a cost effective and time sensitive international transport corridor.

From an overall logistics assessment, the Port of Constanţa’s hinterland includes Bulgaria, Serbia, Croatia, Hungary, Austria, Slovak Republic, South Poland, Republic of Moldova, and the Black Sea bordering countries (mainly Ukraine and parts of Russia).

The total market for the Central & Eastern Europe “CEE package” is estimated at over 18 million TEU (Twenty Foot Equivalent Units) in 2020. Expert reports estimate that the Port of Constanţa could attract over 4 million TEU out of this total, on the condition that a “path of least resistance” in and through Romania exists. The business case worked out in the “European Gateway Project” forecasts the volume of transport associated to the new assembly activities to be performed inside Romania at 2.4 million TEU in 2020 (referred to as “New Assembly package”).

Currently, Romania has significantly low levels of freight in containers which is transported by rail (less than 5%) compared to other European countries (see Table 8.1), including lower levels than Bulgaria. If Romania is to meet European recommendations for moving freight to sustainable modes by 2030 then improvements need to be made to encourage this method of transport. Currently, future growth, unless accompanied by interventions on the rail network, will come mainly through road-based transport, particularly given planned improvements to the road network, coupled with the issues affecting the rail network such as speed restrictions.

Table 8.1: Annual railway transport of goods (containers and swap bodies) in intermodal transport units and the % of total rail freight containers represent

| Country | Total volume of containers and swap bodies ('000 tonnes) |  | % in containers of total goods transported by rail |  |
| --- | --- | --- | --- | --- |
|  | 2011 | 2012 | 2011 | 2012 |
| Austria | 16,312 | 15,806 | 18.2% | 19.0% |
| Bulgaria | 789 | 664 | 5.8% | 5.6% |
| Czech Republic | 7,321 | 7,852 | 8.4% | 9.5% |
| Germany | 64,301 | 66,230 | 17.2% | 18.1% |
| Italy | 34,275 | 33,985 | 43.4% | 45.1% |
| Romania | 2,611 | 2,372 | 4.6% | 4.7% |
| Turkey | 7,601 | 8,264 | 30.7% | 33.2% |
| United Kingdom | 11,098 | 11,742 | 11.1% | 10.2% |

- Source: Eurostat

Intermodal forecasts for Romania indicate a volume in and through Romania of 1.2 million TEU. In the DIOMIS forecast it is expected that the Port of Constanţa will not have a significant competitive position and thus will not attract the traffic destined to the other CEE countries in significant volumes.

The estimates in the paragraphs above are other consultants’ forecasts, and depend on variables such as economic forecasts, the competitive position of Constanta Port vis-a-vis other Black Sea, Mediterranean, and Northern European ports, investment in Constanta Port, pricing, level of service offered by rail and road operators inside Romania, and the commercial agreements between shippers and final customers.  Whether such volumes can be realised given the current situation (in 2011 Constanta Port handled 662,000 TEU), is open to debate: nevertheless it seems clear that there is at least potential for greater container throughput at Constanta, and for intermodal transport within, and through, Romania.

Existing Intermodal Operations

The Black Sea Port of Constanta is home to the largest container port in the Black Sea and is strategically situated at the mouth of the Danube Black Sea Canal which feeds freight into the heartland of Central and Eastern Europe. Constanta port handled 1.41 million TEU in 2007 before the economic crisis, but volumes more than halved to just 557,000 TEU in 2010. This downward trend has reversed slightly with volumes growing to 663,000 in 2011, which is still only 47% of the 2007 volumes.

Constanta is also experiencing competition from the port of Illichivsk in Ukraine. Illichivsk’s inland routes to Russia, the Baltic States and Northern Europe are potentially shorter and quicker than through Romania that has a poorer infrastructure. In 2008, the Port of Illichivsk reached the half-million mark in transhipments of containers, thereby becoming Ukraine's main container port.

The Port of Constanta is reacting to competition and is seeking more partnerships. In the last two years the port has signed a protocol of cooperation in the field of maritime transport and port infrastructure for example with the port of Jebel Ali in Dubai. The cooperation agreement was initiated by DP World, which operates at both ports. It is hoped that the deal will lead to an increase in cargo flows between the two ports, with the facilities agreeing to support initiatives of any operator to establish lines between the two.

Constanta South Container Terminal (CSCT)

DP World is operating around 50 terminals across 30 countries. It decided to invest in Romania as a gateway to Eastern Europe. Since beginning operations in 2004, CSCT has established itself as the principal container hub for the Black Sea. This terminal is by far the biggest container terminal at Constanta but not the only one. The terminal grew to handle the vast majority of the 1,4million TEU handled at Constanta in 2007 and 2008. Its current terminal capacity is 1.5 million TEU with sufficient land to expand to 4.5 million TEU.

Figure 8.1 DP World’s Offices at Constanta South Container Terminal (CSCT)

The terminal handles a range of container ships from deep sea vessels to coastal feeders and barges destined to river ports on the Danube. It is understood the biggest vessel that can currently be handled is an 8,000 TEU deep-sea container ship. There are feeder services to all Black Sea container ports. Several regular barge services operate including domestic transit boxes handled at Giurgiu Free Zone and this vessel also serves Svistou (Bulgaria) as required. There are international transit services from Constanta to Belgrade with options for Hungary, Slovakia and Austria.

The main berth is 634 metres long with a feeder berth of 411 metres. The water draught is a minimum of 14.5m allowing large capacity vessels to operate. There are plans for two additional berths (129, 130) giving an extra 510m of mooring.

The container terminal has 8 ship-to-shore cranes, 13 rubber tyred and two rail mounted gantry cranes and these are supplemented by 4 reach stackers and 4 empty handlers. The port also has 55 ITVs (Internal transfer vehicles) for moving boxes within the terminal. The port operated 364 days per year, 24 hours per day in two 12 hour shifts but like most ports does have busier times of the day/week.

DP World has its own rail terminal with 3 rail lines, each 600m long capable of handling 3 complete 30 wagon trains at once. There are several holding sidings with container trains waiting to be handled. There are about 6 to 7 container trains a day according to DP World. From a visit by the project team an observation was that the rail wagons looked old and there were large lengths of wasted space on the trains that could have better productivity. Two rail mounted gantry cranes service their tracks. It was noted in the DP World Handbook that parts of the mainline track network are being upgraded over the next 3 years to accommodate higher speed freight trains. Improvements to the port rail network will have a significant effect on rail transit times. Additional tracks will link the north to the south port.

Road freight is the dominant mode and there are 3 goods inwards bays and 1 goods outwards in regular use but they have extra gates that could be brought into use if additional capacity is needed. They try to turn a truck round in 15 minutes but it can take an hour. The terminal handles about 850 container trucks over a typical day. The period of busy activity is from 2pm until 10pm with the peak hour being 5-6pm which saw 54 container movements. Note that this is based on an observation made over a 24 hour period. They are considering introducing a Vehicle Booking system like that used at Southampton in the UK.

- Figure 8.2 Draw-bar container truck in DP World’s terminal moving two 20’ containers for Mediterranean Shipping Company

According to the terminal operator it handles a mixture of box sizes with about 55% being 20’, with the balance being mostly 40’ but there are some 30’ and 45’. The terminal services include 624 reefer plugs and monitoring for temperature controlled containers, container stuffing and unloading, EDI messaging and office facilities. DP World has a sophisticated IT system to EDIFACT standard and the terminal operating system is provided by Navis.

Maritime

- Table 8.2 Constanta port has handled the following tonnages since 2005

| Years | 2005 | 2006 | 2007 | 2008 | 2009 | 2010 | 2011 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Containers (number) | 493,214 | 672,443 | 912,509 | 894,876 | 375,293 | 353,711 | 414,096 |
| Containers (TEUs) | 768,099 | 1,037,077 | 1,411,414 | 1,380,935 | 594,299 | 556,694 | 662,796 |

Source: Constanta Handbook

Road

AECOM undertook a traffic count at Constanta’s Gates 10 and 14 whereby every HGV entering and leaving the port was classified, to understand what it was carrying. The traffic count took place over 24 hours. 57% of freight vehicles are involved in carrying containers. It is currently possible to drive to Bucharest from Constanta and return in a single day. It is thought that many of the containers that are being transported by road will end up at Bucharest as it is a major population centre.

The A2 has been extended with a new junction for the port. It is estimated that this will save 30 minutes from a journey time and potentially make road even more attractive as a mode. This could enable road to gain a larger market share.

River

The Danube and other manmade navigations are an important part of Romania’s and Europe’s infrastructure. Currently there are also long distance domestic flows, as an example there is an established significant iron ore flow by barge from south west Romania to Galati which is a distance of 800kms. There could be potential for increasing domestic intermodal flows providing the terminal infrastructure is built and services are reliable and cost efficient. At the moment the number of containers moving on the Danube is relatively small at around 2% of the number of containers handled at Constanta, but there is potential for growth. Containers can be handled at most river ports using a traditional crane but this is not very fast or efficient. The port of Giurgiu handles some containers that then can go north by road or rail.

The flow of TEUs on the Romanian section of the Danube that start or end at Constanta is listed below. The average tonnage per TEU in 2010 was 10.8 tonnes. These are handled by Romanian carriers.

Table 8.3 – TEUs on Danube River

|  | 2008 | 2009 | 2010 |
| --- | --- | --- | --- |
| TEUs | 10,753 | 8,550 | 10,057 |
| Tonnes | 106,919 | 80,344 | 108,783 |
| Ave. Tonnes per TEU | 9.9 | 9.4 | 10.8 |

Source: Constanta Handbook

It can be seen that the number of TEUs and the number of tonnes transported dropped in 2009 coincident with the economic crisis in Europe. The 2010 tonnage recovered to a similar level observed in 2008. The largest carrier of TEUs on the Danube in Romania is Transcanal and in 2010 it handled 77% of TEUs and 75% of the associated tonnes.

Although the Port of Constanta is by far the biggest generator of intermodal freight in Romania the Port of Galati is trying to attract Black Sea trade in preference to Constanta and is offering cheaper prices. Their aim is to set up bulk and container trains to international destinations, for example Duisberg in Germany. Galati also acts as a transhipment port from barge to coastal ship for onward delivery to Black Sea ports serving Ukraine and Russia. It offers a direct freight train service to Kazakhstan as it is the only Romanian port offering both 1435 (European gauge) and 1520mm (Russian gauge) capability.

Rail

CFR Marfă has provided information for their 14 active intermodal terminals. (12 are inactive). Tables 8.4 and 8.5 give details of these terminals.

Table 8.4 Location of CFR Marfă Intermodal Freight Terminals

| No. | Terminal | Crane Equipment | Current Status |
| --- | --- | --- | --- |
| 1 | Bucurestii Noi | 4 cranes (1 functioning) | Active |
| 2 | Bucuresti Sud ( Titan ) | 3 cranes (1 functioning) | Active |
| 3 | Bucuresti Progresu | 1 crane in conservation | Closed |
| 4 | Ploiesti Crang | 3 cranes (1 active) | Not currently active |
| 5 | Bradu de Sus | 2 cranes functioning | Not currently active |
| 6 | Bujoreni Valcea | 2 cranes (1 functioning and 1 not functioning) | Not currently active |
| 7 | Craiova | 3 cranes not functioning (1 in conservation) | Not currently active |
| 8 | Semenic | 3 cranes (2 functioning) | Not currently active |
| 9 | Glogovat | 2 cranes not functioning | Closed |
| 10 | Cluj Napoca Est | 1 functioning | Active |
| 11 | Oradea Est | 2 cranes (1 in conservation and 1 approved for disposal) | Not currently active |
| 12 | Bistrita Nord | 1 crane functioning | Active |
| 13 | Baia Mare | 2 cranes (1 in conservation and 1 approved for disposal) | Closed |
| 14 | Turda | 1 crane functioning | Active |
| 15 | Zalau Nord | 1 crane functioning | Active |
| 16 | Brasov Triaj | 2 crane functioning | Active |
| 17 | Medias | 1 crane functioning | Not currently active |
| 18 | Sibiu | 1 crane in conservation | Closed |
| 19 | Targu Mures Sud | 1 crane in conservation | Closed |
| 20 | Socola Marfuri (Iasi) | 1 crane functioning | Not currently active |
| 21 | Suceava | 1 crane functioning | Active |
| 22 | Bacau | 1 crane functioning | Active |
| 23 | Botosani | No crane | Closed |
| 24 | Galati Marfuri | 3 cranes in conservation | Not currently active |
| 25 | Buzau Sud | 2 cranes (one functioning and one in conservation | Not currently active |
| 26 | Constanta Marfuri | No crane, 3 cranes were moved | Closed |

Source: CFR Marfă, March 2013

The only existing dedicated intermodal port terminals are located at the publicly owned Port of Constanta where DP World, SOCEP and UMEX are the private operators of these terminals. The DP World Constanta South Container Terminal currently handles 97% of the current intermodal volume at the port.

However some other ports on the Danube can currently handle containers, albeit not with dedicated intermodal facilities. These are located at Galati, Tulcea, Braila, Giurgiu, Calarasi, Oltenita, Corabia, Drobeta Turnu Sevrin, Calafat and Orsova. These are shown in Figure 8.5

There are some privately operated rail-enabled intermodal terminals, these are shown in Table 8.5.

Table 8.5 Location of Private Intermodal Freight Terminals

| No. | Terminal | Location |
| --- | --- | --- |
| 1 | Europolis Park/Tibbett Logistics | Bucharest |
| 2 | Comat Electro | Bucharest |
| 3 | Parcul Industrial Faur | Bucharest |
| 4 | Railport Arad | Arad |
| 5 | Trade Trans Terminal | Arad |
| 6 | DB Schenker Romtrans SA | Arad |
| 7 | DB Schenker Romtrans SA | Iasi |
| 8 | DB Schenker Romtrans SA | Oradea |
| 9 | Allianso Group | Ploiesti |

Source: AECOM Analysis

Some of these terminals are linked to specific industrial sites or production chains and so are not suitable for further consideration in this report. Others offer the opportunity for open access intermodal use, and where this is he case (such as at Arad, Bucharest and Ploiesti) we have factored the capacity of these terminals into the development of the masterplan.

\

Figure 8.5 – Location of Publicly and Privately Operated Ports and Terminals in Romania

Source: AECOM Analysis

Figure 8.6 A view of many rail sidings at the Port of Constanta

The table below shows the number of containers handled by CFR Marfă in terms of Unitati de Transport Intermodal (UTI) at its terminals in 2011.

Table 8.6 Containers handled (UTI) in terminals owned by CFR Marfă, 2011

| No | Name of container terminal | containers handled(UTI) |
| --- | --- | --- |
| 1 | Bucurestii Noi | 1,804 |
| 2 | Bucurestii Titan | 360 |
| 3 | Ploiesti Crang | 4,192 |
| 4 | Bradu de Sus | 1,308 |
| 5 | Bujoreni Valcea | 564 |
| 6 | Semenic | 324 |
| 7 | Cluj Napoca Est | 1,412 |
| 8 | Oradea Est | 72 |
| 9 | Turda | 1,780 |
| 10 | Zalau | 1,720 |
| 11 | Brasov Triaj | 3,900 |
| 12 | Medias | 72 |
| 13 | Socola Marfuri | 60 |
| 14 | Suceava | 2,732 |
| 15 | Bacau | 1,604 |
| Total |  | 21,904 |

Source: CFR Marfă Data

The figures above show that 21,904 containers were handled at CFR Marfa depots, of which 4,192 were at Ploiesti Crang which is now reported to be inactive. Although the information did not give origins and destinations of the containers nor whether they were 20’ or 40’ boxes the above volumes only represent about 4% of the volume handled at Constanta. Therefore we have used the raw data of commodities on trains provided by CFR Marfă to establish the volume and flows of containers both empty and full on their trains in 2011. Our estimate is that the majority of containers moved by CFR Marfă are to or from industrial customers at their own private terminals. This represents around 19% of the volume handled at Constanta.

Table 8.7 Top 20 Rail Container Movements in Romania by CFR Marfă by tonnage

| Rank | Region | Origin | Destination | Total |
| --- | --- | --- | --- | --- |
| 1 | 5 | VINTU DE JOS | CONSTANTA | 362,384 |
| 2 | 6 | DORNESTI | CONSTANTA | 231,047 |
| 3 | 2 | GOVORA | CONSTANTA | 83,022 |
| 4 | 5 | VINTU DE JOS | CURTICI | 75,981 |
| 5 | 5 | BRASOV TRIAJ | CONSTANTA | 62,499 |
| 6 | 1 | PLOIESTI CRING | CONSTANTA | 61,701 |
| 7 | 2 | CIUMESTI | CONSTANTA | 71,648 |
| 8 | 6 | DORNESTI | CURTICI | 52,436 |
| 9 | 4 | POIENI | DILGA | 43,528 |
| 10 | 8 | CONSTANTA | BUCURESTI | 57,078 |
| 11 | 1 | BUCURESTI | STAMORA MORAV. | 64,714 |
| 12 | 6 | BACAU | CONSTANTA | 43,058 |
| 13 | 3 | CURTICI | CURTICI | 35,337 |
| 14 | 7 | SUCEAVA | CONSTANTA | 34,688 |
| 15 | 8 | CONSTANTA | TIRGOVISTE | 32,762 |
| 16 | 4 | TURDA | CONSTANTA | 25,634 |
| 17 | 2 | BUJORENI VILCEA | CONSTANTA | 25,317 |
| 18 | 4 | CLUJ NAPOCA | CONSTANTA | 26,061 |
| 19 | 8 | CONSTANTA | ZALAU | 20,165 |
| 20 | 8 | CONSTANTA | CONSTANTA | 18,925 |
| Total |  |  |  | 1,427,985 |

Source: AECOM Analysis of CFR Marfă Data

The top 20 laden container routes operated by CFR Marfă represent 87% of all container tonnes moved by CFR Marfă showing that they are consolidated on to certain key corridors. The rail network is divided up into 8 regions with Bucharest being region 1 and Constanta region 8. The above table shows that the top two container flows by tonnage are from regions 5 and 6 which are the central and eastern production areas with goods to Constanta for export. Unsurprisingly the top two flows of empty boxes are from the port to the same regions.

Table 8.8 Top 20 Empty Rail Container Movements in Romania by CFR Marfă by tonnage

| Rank | Region | Origin | Destination | Total |
| --- | --- | --- | --- | --- |
| 1 | 8 | CONSTANTA | VINTU DE JOS | 51,588 |
| 2 | 8 | CONSTANTA | DORNESTI | 34,136 |
| 3 | 8 | CONSTANTA | CIUMESTI | 16,443 |
| 4 | 3 | CURTICI | VINTU DE JOS | 15,041 |
| 5 | 6 | DORNESTI/VICSAN | CIUMESTI | 13,380 |
| 6 | 3 | CURTICI | DORNESTI | 11,044 |
| 7 | 8 | CONSTANTA | BRASOV TRIAJ | 8,343 |
| 8 | 8 | CONSTANTA | GOVORA | 7,551 |
| 9 | 8 | CONSTANTA | PLOIESTI CRING | 6,729 |
| 10 | 8 | CONSTANTA | BUCURESTI | 9,478 |
| 11 | 8 | CONSTANTA | SUCEAVA | 5,158 |
| 12 | 3 | CURTICI | CURTICI | 4,091 |
| 13 | 8 | CONSTANTA | CLUJ NAPOCA | 3,343 |
| 14 | 8 | CONSTANTA | BACAU | 3,696 |
| 15 | 8 | CONSTANTA | TIRGU MURES | 2,914 |
| 16 | 8 | CONSTANTA | BUJORENI VILCEA | 2,700 |
| 17 | 1 | TIRGOVISTE | CONSTANTA | 2,580 |
| 18 | 4 | ZALAU | CONSTANTA | 2,043 |
| 19 | 8 | CONSTANTA | LUNCA DE MIJLOC | 1,744 |
| 20 | 8 | CONSTANTA | CONSTANTA | 1,742 |
| Total |  |  |  | 203,744 |

Source: AECOM Analysis of CFR Marfă Data

Empty Container Movements by Tonnage

There is a clear imbalance in the flow of loaded containers by rail with more loaded with finished products going for export than for import. This necessitates the movement of empty boxes inland to key factory sites.

The highest container flow on trains operated by CFR Marfă is to and from Vintu De Jos. The area is one of the main sources of timber products and Kronospan have a major wood processing plant there employing over 500 staff. It produces a wide range of forest products including fibreboard and furniture and much of it is for export through the Port of Constanta in containers. Vintu De Jos hosts the reception sidings for the various separate parts of the plant. Products arriving at the plant include lumber, chemicals, mineral (nitrogen), and empty containers and generally full containers leave with furniture, fibreboard, and sawn wood.

The other high volume container flows reflect the location of important industrial sites in Romania. Rail lines serve industrial zones such as those near Bacau, Bujoreni, Cluj, Turda and Timisoara close to Stamora. Buhusi, located 28km from Bacau has the biggest textile factory in Eastern Europe. Ploiesti is also one of the key textile manufacturing locations in Romania. Additionally, Comat SA, owns a production facility of various products like electrical appliances, PVC, etc outside of the city. Kronospan, has another factory with direct access for rail near Brasov. One of the biggest chemical factories in Romania is located in Bujoreni while in Cluj, Romanoff Industries have established a site of significant size. Chemical fertilisers company Azomures SA is served with direct access to rail tracks in Targu Mures. Finally, in Targoviste Cromsteel Industries (steel production) and Mechel SA (manufacturing and mining) have production facilities in the city.  Thus, the large-volume container flows are aligned with major industries.

The top 20 routes with empty containers operated by CFR Marfă is 90% of all empty container tonnage moved by CFR Marfă. It can be seen that in the top 20 movements for both laden and empty containers, Constanta features highly. As Romania’s main deep sea container port this is to be expected.  As noted earlier, more loaded containers arrive at Constanta than depart from there. In other words, at present rail-borne containers are used for industrial exports rather than imports of consumer goods. This implies that the rail based logistics do not (yet) offer competitive services for higher value consumer products or FMCGs, and that there is an opportunity to load containers both ways (to and from Constanta) for increased efficiency

Existing and Potential Demand – Future Intermodal Network

Intermodal transport is one of the easiest ways of reducing transport-related emissions through transferring either part of or the entirety of a journey from road transport to more sustainable modes such as rail and water. There are also large efficiency and cost benefits through increased handling speeds, lower handling costs and improved security.

Table 8.8 shows the number of TEUs (twenty foot equivalent units) that moved in 2011 in Romania and what they carried. Rail moved around 45.5% of all recorded containers between inland destinations. The three largest commodity groups were Manufactured Goods, Fertilisers and Metal Products. Manufactured goods include a wide range of products including furniture, chipboard and consumer goods.

Table 8.9: Base year container traffic (TEUs per day)

| Code | Commodity | Water | Rail | Road |
| --- | --- | --- | --- | --- |
| 0 | Agricultural Products | 0 | 0 | 0 |
| 1 | Foodstuffs | 0 | 12 | 62 |
| 2 | Solid Mineral Fuels | 0 | 0 | 0 |
| 3 | Crude Oil | 0 | 0 | 0 |
| 4 | Ores, Metal Waste | 0 | 0 | 0 |
| 5 | Metal Products | 0 | 69 | 159 |
| 6 | Building Minerals & Materials | 0 | 0 | 0 |
| 7 | Fertilisers | 13 | 220 | 16 |
| 8 | Chemicals | 0 | 26 | 60 |
| 9 | Machinery & Heavy Manufacturing | 0 | 1 | 47 |
| 10 | Petroleum Products | 0 | 21 | 11 |
| 11 | Mail & Parcels | 0 | 0 | 0 |
| 12 | Manufactured Goods | 15 | 228 | 161 |
| 13 | Domestic & Industrial Waste | 1 | 0 | 15 |
| 14 | Forestry Products | 2 | 6 | 137 |
| 15 | Livestock | 0 | 0 | 0 |
|  | Total | 31 | 584 | 669 |
|  | Proportion | 2.4% | 45.5% | 52.1% |

Although the 45.5% modal share figure is high, it must be noted that the intermodal industry in Romania is currently under-developed: 1,300 TEUs per day is a very low volume. As previously discussed  Romania compares poorly in terms of containerised rail freight with neighbouring countries and many western European  countries. Rail freight’s share of the market is dramatically affected by the length of the journey. Figure 8.7 shows that after 200km, the choice of rail for container movements increases significantly. This indicates a distance/price trade off in the transport buyer’s decision making process, although if rail’s offer remains unchanged whilst road freight is improved, the distances involved can be expected to become significantly greater.

Figure 8.7 – Proportion of Base Year Container Traffic on Rail by distance

However, a lack of development of the container market needs to be accounted for. From the forecast of potential for containerisation by commodity undertaken in the model, Figure 8.8 shows the container traffic transported by rail as a proportion of the forecasted container market. It shows that maintaining its current level of activity, rail would only account for 18% of the market, essentially losing out on two thirds of potential growth. On the other hand, if rail could maintain its current share of traffic travelling 300kms, (58%), the analysis implies a trebling of container traffic by rail, which could place severe pressure on current infrastructure.  Clearly if there was a network of efficient intermodal terminals across Romania then this would ease pressure and better enable the sector to capitalise on any growth. This intervention is discussed in greater detail later in this note.

Figure 8.8 – Proportion of Potential Container Traffic on Rail by distance

From an origin and destination perspective, the following is a review of the actual and potential market for containerised freight transport using data from the National Model. All values are average daily tonnes.

Figure 8.9: Existing container rail freight flow 2011, tonnes

Figure 8.9 shows the flow of containerised tonnes on Romania’s railways in 2011. The largest cumulative flow is between Bucharest and Constanta. In addition a significant number of flows join at Fetesti through to Constanta. This is understandable as Constanta is Romania’s major port and handles 80% of its containers. A lighter flow of containers running between the centre of the country and Constanta on Corridor IV. Another flow of containers heads northwards from Constanta linking up with Corridor IX.

Contrasting this, Figure 8.10, however shows the maximum potential container flow on Romania’s railways. The analysis uses an assumption on the types of goods that are suited to containerisation (see Table 8.10). The plot has two-way flows, and is colour coded showing the existing rail market share compared with the potential.

Table 8.10: Industries suited to containerisation and distances at which rail freight becomes competitive

| Code | Commodity | Distance | Code | Commodity | Distance |
| --- | --- | --- | --- | --- | --- |
| 1 | Foodstuffs | 60km | 9 | Machinery and Manufacturing | 100km |
| 5 | Metal Products | 100km | 12 | Manufactured Goods | 130km |
| 7 | Fertilisers | 100km | 13 | Domestic & Industrial Waste | 100km |
| 8 | Chemicals | 100km | 14 | Forestry Products | 100km |

Figure 8.10: Potential container traffic and current mode share by rail flow

This can be further assessed through a quick analysis of each of the potential commodities containerised flows.

Foodstuffs (1): Flows of foodstuffs by rail are relatively small, (approximately 1,133 tons per day) of which approximately a third offers the potential for containerisation. This is mostly due to the time requirements of food shipments. However, of that which is suitable, there is potential for improved rail modal share on corridors IV south and IX especially. In the case of the former, modal share (of potential total containerised foodstuffs) is approximately 20 – 30% between Caracal and Carahsebes, with the modal share between there and the border (past Timisoara and Arad) below 10% northbound and 20% southbound. Improved intermodal facilities at Timisoara may help to increase this modal share through reducing handling time and costs, as would improvements to border crossing procedures, line speed and journey time reliability. The use of temperature controlled containers in some countries is enhancing the volume of foodstuffs being moved by rail and is worth further consideration for Romania, however this would require the installation of specialised facilities at terminals such as plug-in points for temperature controlled containers.

Metals (5): Between Bucharest and Constanta rail has already established a modal share dominance, which continues on Corridor IV and Corridor IX. Corridor IV South is affected by the proximity of the rail corridor to the Danube, and improvements made to intermodal terminals here will draw more traffic away from river because of its time sensitivity. However, there is room for growth between Galati, Bucharest, Pitesti and Craiova, where rail’s potential modal share (of potential containerised metal products) is low. Metals are a key component of the automotive industry and flows for car production are important. Improvements are also possible at the western end of Corridor IV South between Arad and Caransebes (via Timisoara). The proposed intermodal terminal at Timisoara, along with improvements to line speed, reliability and reduced handling costs will increase the potential for shift to rail from road. The potential flows are shown in Figure 8.11 below:

Figure 8.11: Potential Containerised Metal Flows and Rail Modal Share

Fertiliser (7): There are nine fertiliser production plants in Romania and the country both imports and exports the commodity. The sector has been expanding in line with positive growth in cereal production. A key flow with regard to (potentially containerised) fertiliser is that from the large plant near Targu Mures through to the port of Constanta along Corridor IV. The plant currently produces around 900 tonnes per day and future investment is projected to increase this flow to over 1,400 tonnes per day by 2015. Rail has a large share of this market, although it has a poorer share of the return flow from Bucharest, past Targu Mures to Cluj-Napoca. Improving rail’s share of the industry on this return leg would enable better utilisation to be made of the rolling stock within this industry. Modal share is also high on the much smaller flows operating on Corridor IX (southbound) and on Corridor IV South (both directions), although this latter corridor has smaller potential than the others due to competition from water freight on the Danube. This is demonstrated in Figure 8.12 below:

Figure 8.12: Potential Containerised Fertiliser Flows and Rail Modal Share

Petrochemicals (8 & 10): On Corridor IV rail has a large modal share of potential eastbound flows, from the border at Curtici all the way to Constanta. Flows on Corridor IV south are affected by competition from water freight on the Danube, although improved links between modes at the Danube ports will offer opportunities to increase tonnage handled. On Corridor IV the amounts handled (under the potential containerised scenario) are low, although rail has a relatively high modal share on southbound movements, although northbound the modal share is significantly lower (under 10%). Liquid or granular chemicals can travel in tanktainers.

Machinery, Manufactured Goods and Heavy Manufactures (9 & 12): Corridor IV South is noticeable when looking at this commodity type due to water freight taking a large modal share of this important flow. However, there are also significant volumes travelling on corridors IV and IX, of which rail has a large share of the potential containerised exports. However, there is significant room for improvement on domestic movements, particularly with regard to the large flows moving between Suceava (on Corridor IX) and Dej in the north. There is also a significant flow of this commodity type over the border with Ukraine, most of which currently travels by alternative modes.

Forestry Products (14): It is estimated that around 20% of all forestry products are suitable for containerisation, and that rail has a good potential modal share (above 80% of this hypothetical traffic) on all three of the relevant corridors (IV, IV South and IX). Already one of the largest flows in containers is from Kronospan’s production plants to the Port of Constanta. However, at both the northern and southern extremities of Corridor IX (within Romania) rail’s modal share is projected at below 10%. Improved border crossings, axle weight loadings and journey time reliability will help rail to compete more effectively against road in these locations.

Conclusions

Regardless of commodity type, all of the rail network in Romania would be more attractive to intermodal freight as a result of improved maintenance, efficiency and lower costs. As such, any intervention that improves the speed of freight trains and lowers the cost (in time and money) of modal transfer are to be welcomed, whether these are maintenance and rehabilitation works, improved line speeds, new locomotives and rolling stock or increased axle weights. In particular, there is a need to improve the pathing of intermodal block trains in order to reduce layover and stabling times in order to make journey times more competitive. However, certain corridors will benefit from more targeted interventions, and these are outlined below.

Corridor IV South: There is significant potential for improvements to Corridor IV South that result in a notable increase in the amount of container traffic moving on the railway. In particular, improvements to this route would facilitate the growth of transit traffic along this corridor. Furthermore, this is not taking into account the extra benefits which would come from improving links with certain key ports along the Danube itself, including Drobeta-Turnu-Severin (recommended for a new intermodal terminal elsewhere in this strategy) and the new railway bridge at Calafat-Vidin (and the access it provides to the proposed Vidin tri-modal terminal). Supporting better connections between the Danube and the rail network will enable increases in the amount of intermodal freight that can be moved and its penetration (on sustainable modes) into areas further away from the Danube and Corridor IV South. This latter goal will be particularly assisted by the creation of an intermodal terminal at Craiova, Timisoara and a new terminal in Bucharest. Due to this connectivity, and the ability of the line to link with Bucharest, Constanta and the western border whilst avoiding much of Romania’s mountainous interior, Corridor IV South is perhaps best suited to benefit from; improved line speeds, larger axle weights and more modern infrastructure, rolling stock and locomotives that will increase the amount of international freight handled by rail, as well as domestic traffic which currently goes by road.

Corridor IV (North): In Romania this runs from the Hungarian border crossing near Arad (Curtici) in the west through Bucharest to the port at Constanta in the southeast. As such it passes near several important towns and cities such as Ploiesti and Brasov, in addition to providing the key link between Bucharest and the Black Sea. Whilst this latter trade is well developed (with rail having a relatively high modal share) the links further inland are less so. Interventions which improve journey times, lower costs and improve efficiency (particularly at border crossings) will assist, alongside targeted interventions such as the new terminal in Bucharest and the potential rehabilitation of terminals in central and western Romania. It is the rehabilitated routes such as Corridor IV between Constanta and Bucharest that would benefit most from the instigation of a two-tier speed limit for intermodal freight trains (and an improvement in their relative priority versus some passenger trains), although this will benefit all of the corridors to some extent. Similarly, this applies to other interventions such as allowing modern trains to be more energy efficient through the use of regenerative braking for example.

Corridor IX: The main north-south corridor in the east of the country, this corridor links onto corridor IV near Ploiesti to the north of Bucharest. It also links with the important ports of Galati, Braila and Giurgiu, and has border connections with Moldova, Ukraine and Bulgaria. It is mostly a freight line, and there is potential to build on the current flow of containers from Suceava (generally travelling towards Bucharest and Constanta) and develop international intermodal flows with the other countries on the corridor. As already established, the border crossing with Ukraine at Dornesti is CFR Marfa’s second busiest, and Corridor IX has great potential for future growth of transit traffic as a result of Project Viking. This ambitious project plans to move up to 1,000 trains a year on the corridor from the Baltic States to Bulgaria, Turkey and possibly Greece via Ukraine and Romania. However, for this to happen and be an ongoing success, significant interventions to improve axle weight loadings, line speed and journey time reliability would be needed. Furthermore, Corridor IX also links with Galati which is well-located as a transhipment point from water or Russian gauge railways for onwards connections. As on Corridor IV, there is generally a poor modal share of inbound containers, which could be aided through the rehabilitation of intermodal terminals in the northeast, such as at Suceava and Iasi. Again, the line would benefit from improved maintenance and the other identified interventions which apply to all corridors.

Building on these foundations then, a clear programme of interventions with regard to rehabilitating the route to facilitate higher axle weights and faster intermodal trains is clear. The proposed rail network enhancements are discussed in Section 5.

Intermodal Terminals - Factors for Success

From AECOM’s experience the key success factors for intermodal services are:

Availability of frequent, scheduled, reliable and punctual services;

Schedules geared to the movement of business and consumer goods;

Flexible door-to-door solutions, including trucking alternatives;

Control and management of a secure door-to-door supply chain;

Fast handling at terminals to ensure efficient round-trip schedules for trucking companies (road collection and delivery), including efficient road access infrastructure;

Consistency of delivery;

Seamless international intermodal services (interoperability, synchronization of processes between railways and operators, data interchange);

Infrastructure access charges ensuring a level playing field between intermodal solutions and road transport;

Presence of a significant number of customers close as possible to the intermodal terminals;

Availability of efficient terminals;

Availability of empty container depots at strategic hubs;

Supportive activities by authorities;

Transparency and accountability

Market awareness of the possibilities of intermodal transport;

Experience of the intermodal market; and

Competitive end to end pricing.

In order to achieve this, each terminal will need to have some (if not all) of the following:

Typical Requirements for an Intermodal Rail Freight Terminal The following are typical requirements that need considering when setting up a new terminal. Security - Security office - Security Fencing - Good lighting - Electronic gates - CCTV Systems - Central Control Room with communication to operatives and handling equipment - Modern IT systems with internet based train management - Automated electronic document control - Modern Train control system - Modern Communication systems - Integrated Train Signalling - Latest customs checking systems that can verify goods even whilst moving Handling Equipment - Electric gantry cranes for container handling with minimum 4 track reach and lorry loading capability - Reach stackers - Possibly cross dock for cargo wagon (box car) loading/unloading with sufficient space for fork-truck turning circle and through roads - Handling equipment recharging/refuelling facilities Road Facilities - Lorry Parking area with electronic calling mechanism to instruct driver when to approach designated loading bay - Lorry road movement is one-way for safety and smooth operational reasons - Lorry driver rest area, cafe etc. - Lorry garage/maintenance facilities - Road fuel service stations Staff Facilities - Train driver rest area, cafe etc. - Staff rest area, cafe etc. Train Operations - Locomotive refuelling facilities recognising that many trains are likely to be electric powered - Cripple sidings for defective trains - Train maintenance area - Reception sidings for train arrivals - Sidings for spare trains waiting for next duty - The loading/unloading sidings should accommodate a maximum length train (minimum of 80 wagons long), consider future proofing this length - Ideally all tracks will be capable of handling container trains - Some sidings may be equipped for box cars/cargowagons for loose, bagged and palletised goods - Some sidings may be equipped for bulk products such as grain Customs (may be appropriate for some inland terminals) - Customs office - Customs checking warehouse - Customs reception sidings - Customs impounded goods area Storage Area - Small secure storage area - Temperature Controlled container plug in points - Inbound train unloading storage stacks - Outgoing train preparation area for stacking containers - Assumes that most of the containers handled will continue their journey by road.

Operational Objectives

The proposed interventions will support the following operational objectives as indicated in Figure 8.13.

Figure 8.13: Links between Problems, Interventions and Operational Objectives

Interventions

A comprehensive, national network of Intermodal Terminals is essential for Romania’s rail freight and logistics industry to modernize and be competitive.

The location of the proposed intermodal terminal network is a function of the following five factors:

Connectivity with other modes

Geographic distribution of ports and population centres throughout Romania

Current road/rail Infrastructure

Current and Potential Containerised Freight Flows

Population within a short drive by Heavy Goods Vehicle (One hour for inland rail terminals, two hours for ports)

The most crucial of all the judged criteria was the ease with which a terminal location enabled interchange between road and other modes.

To meet the needs of industry, a successful intermodal terminal is likely to realize the following capital requirements.

Secure, well lit perimeter (i.e. fencing, flood lighting, CCTV coverage etc)

Good access to road network as well as internal roads system

Two rail sidings with a platform length of (ideally) 750m (EU standard) with a run-round loop

Reach Stacker (electric gantry cranes for busier sites)

Hard-standing with a minimum 750x20m to allow reach stacker to manoeuvre, cleared to 22.5 tonne axle weights

Additional space for container storage

Administrative/Control Office

Welfare Facilities for staff and drivers

Operational requirements such as labour have not been factored in. However, they may include:

Site Maintenance

IT

Security Personnel

Drivers

Terminal Manager

Administration

Mechanics

Fuel

Support Vehicles

Based on the following assumptions, building such a facility will provide a capacity of approximately 129,000 TEUs per siding, per annum or a total of 358,000 TEU overall, with an additional reach stacker.

Table 8.11: Assumptions for terminal capacity estimates

| Attribute | Meters | Comments |
| --- | --- | --- |
| Wagon Length | 19.7 | Astra Rail SGnss 60' bogie wagon |
| Loco Length | 21.4 | Typical electric/diesel locomotive |
| TEU per wagon | 3 |  |
| Siding Length | 750 |  |
| Working Days | 300 |  |
| Trains per day | 4 | Based on 6 hours load/unload per train |
| Wagons per Train | 36 |  |

Bigger sites are likely to have an electric gantry crane in addition to a reachstacker, e.g. Bucharest. Space should be safeguarded to extend both rail and road facilities as the terminal grows in the future. Bucharest, for example, will probably require three tracks to enable two trains to be handled simultaneously, boosting capacity to feasibly the aforementioned 358,000 TEU annually. This would also provide capacity for immediate onward transhipment from international trains to domestic trains destined for Constanta.

Although the movement of containers by river is likely to remain a relatively small percentage of total water freight, currently around 1%, it is expected to grow in volume in proportion with the market. Only around 2% of the containers being handled at Constanta move inland by barge and this compares to ports such as Rotterdam and Antwerp where significant volumes, around 35%, go by barge. Indeed the GTMP model shows that tonnage will grow by 70% by 2020 when it is entirely possible that daily sailings of container barges leave Constanta either destined for Galati or for a central and western feeder service connecting two or three Romanian ports with other ports in Hungary and Serbia.

An added incentive for the development of water freight is that goods are not counted as being imported until they are landed; thus a transhipment to barge at Constanta and then landing at Drobeta could provide a day or two’s steaming where VAT on transport costs would not be applicable. This could reduce costs and encourage increased use of water freight quite substantially, as such a reduction (c. 20%) in costs will induce increased traffic from less sustainable modes.

Proposed Terminals

As the Danube only serves the extreme south of the country, for a truly comprehensive network it will also be necessary for several inland rail/road intermodal terminals to be built. The location of the terminals was led by a consideration of the nearby population and industry (hence the current and potential containerised flows) and on the basis of existing infrastructure which was suitable for rehabilitation as an intermodal terminal. It was also important that these terminals provided suitable geographic coverage of Romania to encourage the use of rail freight for long-distance trunking movements within the country. The main population centres in Romania were assessed with regard to their population, economy and existing facilities.

Potential demand for these sites was based on the potential maximum containerised flow (2011) from the National Transport Model, which took into account potential movements given a higher rail modal share for each commodity group that was suitable for containerisation. These movements were calculated on the existing rail network regarding speeds and terminals. Although the flows thus produced are a current “best case” scenario given the state of the rail network in 2011 (the base year), they are a fair representation of what could be expected from flows in these commodities in other countries with better developed inter-modal infrastructure.

There is significant room for growth in containerised freight, particularly as it plays a key role in boosting efficiency and lowering costs for transport. Romania’s population is set to decline by approximately 1.5m people by 2035, whilst current trends point towards de-urbanisation occurring across the country. Containerisation will provide an important way to maintain and improve standards of living through lowered costs of this distribution network which will need to serve more places more cheaply. However, the current facilities do not offer scope for efficient and orderly increases in container traffic, and would indeed struggle to meet demand if more industry utilised rail given their current operating conditions and locations.

This is demonstrated by the inclusion of the figures for “potential rail freight”, which estimates the amount of traffic which would utilise each terminal assuming that significant containerisation of freight had taken place across the country, in response to the improved network and infrastructure for its distribution. It should be noted that the potential containerised rail freight is explicitly not a forecast, but an indication of the significant role which containerised rail freight could play in distributing goods and manufactures around Romania, given better conditions than current and a more competitive offer compared with road haulage.

The current capacity of existing terminals was developed based on the following assumptions, that the terminal would operate for 300 days a year, with twenty metre wagons being used carrying upto 3 TEUs each. Limitations apart from the rail sidings and handling equipment are not considered, for example, the state of the hard-standing. The full length of each siding can be used twice a day for either loading or unloading by any operational cranes at any given site. The crane operates on the current CFR Marfa average of seven minutes per TEU movement for the existing 12 hours of operational time per day.

By contrast the “best practice” capacity, developed as part of the potential rail demand scenario, proposed that each siding could be used up to six times per day, with all extant cranes restored to full working order moving a box in three minutes, in line with international best practice. This was to assess whether or not the facilities would be able to cope with a significant increase in utilisation even with improved operational conditions.

For situations where there was a significant shortfall of capacity over potential demand under these varied conditions then a rehabilitation scheme was considered. Furthermore, major centres which had a terminal mothballed were modelled to see if the capacity of these facilities is adequate and if so they are recommended for rehabilitation and re-opening as the intermodal market expands. In these instances it is suggested that the sites, after rehabilitation, are operated by private logistics operators to ensure open access on equal terms for all users. Various operating approaches have been utilised across Europe with success, and a study of which approach is most suitable for Romania should be undertaken as part of the rehabilitation programme.

These interventions create the following network of rehabilitated and/or modern terminals:

Figure 8.14: Map of Proposed Interventions

When this is combined with the existing terminals which have been assessed as not requiring significant rehabilitation under the aegis of the masterplan, the network of inland terminals which results is as follows:

Figure 8.15: Map of Inland Terminal Network

Having arrived at these sites as a result of analysis and potential demand assessment, it is worthy to compare the proposal with those of other stakeholders who have assessed the problem. The proposed network is shown in Table 8.12, and we have considered the suggestions of other parties are shown in Table 8.13. As can be seen, the proposed network matches up well with those suggested elsewhere by either other consultants or train operators themselves.

- Table 8.12 Proposed Network of Intermodal Terminals

| Inland Terminals |  |  |
| --- | --- | --- |
| Extant | Refurbished or New Build | New Build |
| Arad Zalau Bistrita Brasov Ploiesti Pitesti | Timisoara Cluj-Napoca Turda Suceava Iasi Bacau Oradea | Craoiva Bucharest |

A comparison of Tables 8.12 and 8.13 demonstrates that the proposed network has widespread support from a variety of key stakeholders, in particular there is clear support for additional terminals in the Arad/Timisoara/Semenic area, Bucharest, Cluj and Craoiva. All terminal locations which received more than three recommendations were assessed in the course of developing the proposed network, in addition to the other factors discussed above.

The cost of these schemes varies to reflect the different condition of the existing yards available for use. Therefore in some yards a lighter rehabilitation programme is required, whereas other terminals require the construction of new, modern and bespoke facilities.

This intermodal terminals network provides for large retail demand in population centres and also potential industrial containerised flows in important industrial sectors. Furthermore, it offers an opportunity for a coherent, comprehensive sustainable transport network for containers across the nation which should promote modal shift from road to more sustainable transport modes.

According to the national transport model, the recommended set of intermodal terminals, when combined with the full set of rail improvements (alongside water improvements and the proposed tri-modal terminals) do indeed promote modal shift, with approximately 900,000 fewer road tonne kilometres, over 100,000 additional rail tonne kilometres, over 1.1 million additional water freight tonne kilometres and approximately 400,000 more freight tonne kilometres than the reference case by 2030.

However, these terminals will need to be accompanied by rehabilitated rail corridors to enable heavier trains (22.5 tonne axle weight) and also effective pathing of container trains to enable higher speeds and journey time reliability and punctuality. These improvements will also benefit and cater for predicted growth in non-containerised freight, where rail remains more competitive against road.

Table 8.13 Proposed Networks of Terminals from previous studies and Consultation

| Reference /Consultee | Year | No. of Terminals |  | Most Appropriate Locations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Strategic | Small | Bucharest | Arad /Timisoara/Seminic | Ploiesti | Brasov; | Sibiu; | Iasi; | Cluj; | Vidin | Constanta; | Galati; | Giurgiu | Oradea Est | Zalau | Baia Mare | Turda | Medias | Craiova | Buzau | Suceava | Botasani | Bacau | Bujoreni | Bradu de sus | Ciumesti | Vinto de Jos | Slatina | Pitesti |
| Syancu | 2004 | 7 |  | x | x |  | x |  |  |  |  | x |  |  | x |  |  |  |  | x |  |  |  | x |  |  |  |  |  |  |
| Halcrow | 2006 | 4 | 6 | X | X | x |  | x | x | x |  | X | X |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  | x |  |
| Diomis | 2010 | 4 |  | x | x |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |
| Kombi Consult | 2013 | 7 |  | x | x |  |  |  | x | x |  |  |  | x |  |  |  | x |  | x |  |  |  | x |  |  |  |  |  | x |
| IM Strategy – Romania 2020 | 2010 | 8 |  | x | x | x | x |  | x | x |  | x |  | x |  |  |  | x |  | x | x |  |  |  |  |  | x |  |  | x |
| Train Operator 1 | 2013 | 4 | 17 | X | X | x | x | x | X | x |  | X | x |  | x | x | x | x | x | x | x | x | x | x | x | x |  |  |  |  |
| Train Operator 2 | 2013 | 5 |  | x |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  | x |  | x |  |  |
| Train Operator 3 | 2013 | 3 | * | Not Specified |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Train Operator 4 | 2013 | 5 |  | x | x |  | x |  |  | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Train Operator 5 | 2014 | 9 |  | x | x | x | x |  |  | x |  |  |  |  |  |  |  |  |  | x |  | x |  | x |  |  | x |  |  |  |
| Private Company | 2012 | 12 |  | x | x | x | x | x | x | x | x | x | x | x |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |  |

* This train operator felt that whilst there should be additional, smaller terminals for domestic traffic spread throughout the country, the specific locations were not necessarily pre-determined but should be demand-led.

The resultant proposed network of new and rehabilitated terminals provides both strategic, international terminals and smaller domestic terminals where potential and expected demand supports development. Additionally, the set of terminals proposed will enable direct transshipment from Russian to European gauge at Iasi. This will result in a more sustainable and cost-efficient distribution network for the country as a whole

Testing interventions

For ease of understanding, this section is split into two sections. The first section will assess the new and refurbished Intermodal Terminals, whilst the second section will include other interventions.

New and Refurbished Intermodal Terminals

These proposals fall under the High-Level Objective of Improving Economic Efficiency. First the new terminals are considered followed by the refurbished existing terminals.

New terminals are those terminals which do not currently exist

Refurbished terminals are those terminals that currently exist and have undergone refurbishment which brings them into line with modern logistics practices, in particular with regard to handling containers.

AECOM has also considered the outcome of undertaking a new build at selected existing intermodal terminals. A comparison of model outputs is made between the option of refurbishment or a new build.

In all cases, it is envisaged that local Highway Authorities will ensure road access and building permits are available to private operators who wish to operate the given terminal. CFR SA will be responsible for ensuring the terminal’s connection to the main rail network and the private sector will design, build and operate the terminals (see below).

Institutional Issues – Operation of Intermodal Terminals

Intermodal terminals generally work more efficiently when operated by the private sector as they are able to be entrepreneurial in operations, staffing, costing and investing in modern technology and handling equipment. Some terminals offer added value services such as stuffing and destuffing containers, box repairs and local deliveries as ways of maximising revenue. It is recommended that Romanian terminals are operated on a commercial basis that gives operators and customers the flexibility they need to be successful. The use of private operators needn’t preclude the use of EU funding to provide some investment towards the terminals, provided that it is utilised only as a bridge to cross the funding gap and ensure that construction of the terminals is undertaken and not as an ongoing subsidy of any kind.

With regard to the recommendations that are therefore made later in this chapter, it is important therefore to clarify that it has been assumed that there will not be any legal barriers to the creation of these privately operated public terminals. However, it is noted that, especially where the rehabilitation or re-opening of an existing terminal is suggested, there may be legal constraints to the handover of property from CFR Marfa or local authorities to the private sector. As these conditions vary from terminal to terminal, the development of any individual one must take this potential cause of delay into account. In extreme circumstances, there may be a need to develop a new terminal location rather than re-develop an existing terminal.

An increasingly common approach to the construction of these intermodal terminals is for the government to provide the land and drive for the terminal’s construction, with construction and operation often being tendered to the private sector in order to minimise risks for the government and maximise innovation and value for money through private sector expertise. This model should also encourage low costs for handling containers, which will help rail freight to compete with road transport.

Bucharest (New Terminal)

AECOM recommend the closure of the existing Bucarestii Noi facility and the establishment of a new, larger, high-capacity terminal located near the ring road and motorway network. Ideally this should have the potential for conversion to a tri-modal terminal if built alongside the proposed route of the Danube-Bucharest Canal, should this be built at a future date.

Operational Objective

OR16, Develop a network of “open user” freight terminals to serve Romania’s highest demand potential regions, cities and major EU markets in Bulgaria and Hungary.

Problem Addressed

Current intermodal freight volumes in Romania are low, and this is partly due to the poor state of the facilities designed to receive them. When combined with expected growth in the intermodal sector, the current facilities at Bucharest are too small for anticipated demand with capacity rapidly being outstripped by supply (see table below under Outcomes) and so a new terminal is recommended.

Outcomes

As demonstrated in the table and chart below, the intervention (in concert with the other intermodal terminals) provides a significant boost to the amount of intermodal freight travelling by rail.

Table 8.14 Current and Potential Demand (TEU) at Selected Milestone Dates

|  |  | 2011 | 2020 | 2030 | 2040 |
| --- | --- | --- | --- | --- | --- |
| Reference Case | Daily Demand | 35 | 82 | 131 | 196 |
|  | Percentage of Capacity (Current) | 34% | 80% | 128% | 192% |
| With Terminal | Daily Demand | 35 | 179 | 270 | 380 |
|  | Percentage of Capacity (Current) | 34% | 175% | 265% | 373% |
|  | Percentage of Capacity (Best Practice) | 8% | 39% | 59% | 83% |

Figure 8.16: Chart of Difference between All Terminals and Reference Scenarios

The new terminal has significant economic benefits:

- Table 8.15: Economic Impact of Terminal Construction

| Present Value of Costs | 18.0 |
| --- | --- |
| EIRR | 13.8% |
| Benefit/Cost Ratio | 3.91 |

Implementation:

This scheme would be implemented as suggested in paragraph 8.4.6. The development of the intermodal terminals is viewed as being very important for the development of rail freight in Romania as they are integral to allowing rail to compete in a modern and growing sector. The terminal at Bucharest is scheduled for implementation in the period 2015 – 2020.

Craiova (New Terminal)

AECOM recommend the disposal of the existing, mothballed terminal at Craiova and instead the creation of a new terminal with better links to the road network in order to cater for expected growth in the intermodal sector.

Operational Objective

OR16, Develop a network of “open user” freight terminals to serve Romania’s highest demand potential regions, cities and major EU markets in Bulgaria and Hungary.

Problem Addressed

Current intermodal freight volumes in Romania are low, and this is partly due to the poor state of the facilities designed to receive them. When combined with expected growth in the intermodal sector and the industrial composition of the area around Craiova, there is potential for demand to outstrip capacity at the existing, dilapidated location. The terminal could help serve the local automotive cluster as well as consumer demand from the sixth largest city in the country. Craiova also is well positioned on the TEN-T corridor from Hungary to Bulgaria and will benefit when this route is rehabilitated.

Outcomes

As demonstrated in the table and chart below, the intervention (in concert with the other intermodal terminals) provides a significant boost to the amount of intermodal freight travelling by rail.

Table 8.17 Current and Potential Demand (TEU) at Selected Milestone Dates

|  |  | 2011 | 2020 | 2030 | 2040 |
| --- | --- | --- | --- | --- | --- |
| Reference Case | Daily Demand | 0 | 0 | 0 | 0 |
|  | Percentage of Capacity (Current) | 0% | 0% | 0% | 0% |
| With Terminal | Daily Demand | 0 | 41 | 60 | 83 |
|  | Percentage of Capacity (Current) | 0% | 40% | 59% | 81% |
|  | Percentage of Capacity (Best Practice) | 0% | 9% | 13% | 18% |

Figure 8.19: Chart of Difference between All Terminals and Reference Scenarios

The new terminal has significant economic benefits:

- Table 8.18: Economic Impact of Terminal Construction

| Present Value of Costs | 18.0 |
| --- | --- |
| EIRR | 6.6% |
| Benefit/Cost Ratio | 1.39 |

Implementation:

This scheme would be implemented as suggested in paragraph 8.4.6.  The development of the intermodal terminals is viewed as being very important for the development of rail freight in Romania as they are integral to allowing rail to compete in a modern and growing sector. The terminal at Craiova is scheduled for implementation in the period 2015 – 2020

Iasi (Refurbished or New Build Terminal)

AECOM propose that the Terminal is re-opened to take advantage of the potential from transhipping containers from European to Russian gauges and also the ability to handle domestic traffic. If reopening is not possible then AECOM proposes that a new terminal is created at Iasi.

Operational Objective

OR16, Develop a network of “open user” freight terminals to serve Romania’s highest demand potential regions, cities and major EU markets in Bulgaria and Hungary.

Problem Addressed

Current intermodal freight volumes in Romania are low, and this is partly due to the poor state of the facilities designed to receive them. When combined with expected growth in the intermodal sector and the opportunity to tranship to/from Russian gauge, there is a need for the currently closed terminal to be rehabilitated and reopened.

Outcomes

As demonstrated in the table and chart below, the intervention (in conjunction with the other intermodal terminals) provides a significant boost to the amount of intermodal freight travelling by rail.

Table 8.19 Current and Potential Demand (TEU) at Selected Milestone Dates

|  |  | 2011 | 2020 | 2030 | 2040 |
| --- | --- | --- | --- | --- | --- |
| Reference Case | Daily Demand | 1 | 2 | 3 | 5 |
|  | Percentage of Capacity (Current) | 2% | 4% | 6% | 9% |
| With Terminal | Daily Demand | 1 | 25 | 34 | 47 |
|  | Percentage of Capacity (Current) | 2% | 46% | 63% | 87% |
|  | Percentage of Capacity (Best Practice) | 0% | 12% | 16% | 22% |

Figure 8.20: Chart of Difference between All Terminals and Reference Scenarios

- The new terminal has some economic benefits and they indicate that a refurbishment is the most appropriate decision for the intermodal terminal at Iasi. The BCR for a new build terminal is forecast as less than 1.

- Table 8.20: Economic Impact of Terminal Construction

|  | Refurbished | New Build |
| --- | --- | --- |
| Present Value of Costs | 4.0 | 14.4 |
| EIRR | 12.0% | 3.9% |
| Benefit/Cost Ratio | 3.01 | 0.81 |

Implementation:

This scheme would be implemented as suggested in paragraph 8.4.6.  The development of the intermodal terminals is viewed as being very important for the development of rail freight in Romania as they are integral to allowing rail to compete in a modern and growing sector. The refurbishing or new build of the terminal at Iasi is scheduled for implementation in the period 2015 – 2020.

Timisoara (Refurbished or New Build Terminal)

AECOM propose that there are two options for an intermodal terminal at Timisoara to provide Romania’s second city with its own intermodal facility as well as relieve pressure on private facilities at Arad to cope with growth in the intermodal sector. Option 1 is to consider reopening the Seminic terminal or Option 2 is to build a new terminal. The latter proposed idea to create an intermodal facility near the airport is worthy of consideration as although generally air freight and rail freight are not particularly compatible for products needing transhipment from one of these modes to another, the idea of a new business park with cargo facilities offering shippers the choice of road, rail or air freight depending on the goods is potentially an attractive one.

Operational Objective

OR16, Develop a network of “open user” freight terminals to serve Romania’s highest demand potential regions, cities and major EU markets in Bulgaria and Hungary.

Problem Addressed

Current intermodal freight volumes in Romania are low, and this is partly due to the poor state of the facilities designed to receive them. This is a good opportunity to cater for expected growth in the intermodal sector and to expand facilities near the border in this important economic region in the west of the country.

Outcomes

As demonstrated in the table and chart below, the intervention (in concert with the other intermodal terminals) provides a significant boost to the amount of intermodal freight travelling by rail.

Table 8.21 Current and Potential Demand (TEU) at Selected Milestone Dates

|  |  | 2011 | 2020 | 2030 | 2040 |
| --- | --- | --- | --- | --- | --- |
| Reference Case | Daily Demand | 2 | 2 | 3 | 17 |
|  | Percentage of Capacity (Current) | 3% | 4% | 5% | 10% |
| With Terminal | Daily Demand | 2 | 40 | 57 | 84 |
|  | Percentage of Capacity (Current) | 3% | 56% | 79% | 117% |
|  | Percentage of Capacity (Best Practice) | 1% | 14% | 20% | 29% |

Figure 8.21: Chart of Difference between All Terminals and Reference Scenarios

The new terminal has significant economic benefits. Whilst a new build terminal would satisfy the Master Plan criterion and regional development plans for Timisoara, the lower costs of a refurbished terminal offer a better BCR: Clearly a Masterplan for the area that shows a new business park and new metro system linking to the city centre would benefit from a new intermodal freight terminal in order to help attract inward investment.

- Table 8.22: Economic Impact of Terminal Construction

|  | Refurbished | New Build |
| --- | --- | --- |
| Present Value of Costs | 1.5 | 14.4 |
| EIRR | 24.4% | 6.6% |
| Benefit/Cost Ratio | 8.45 | 1.33 |

Implementation:

This scheme would be implemented as suggested in paragraph 8.4.6. The development of the intermodal terminals is viewed as being very important for the development of rail freight in Romania as they are integral to allowing rail to compete in a modern and growing sector. The refurbishing or new build of the terminal at Timisoara is scheduled for implementation in the period 2015 – 2020.

Cluj Napoca (Refurbished or New Build Terminal)

AECOM propose that the existing terminal at Cluj Napoca is refurbished to increase efficiency, lower costs and transit times and to promote growth of the intermodal sector. If reopening is not possible then AECOM proposes that a new terminal is created.

Operational Objective

OR16, Develop a network of “open user” freight terminals to serve Romania’s highest demand potential regions, cities and major EU markets in Bulgaria and Hungary.

Problem Addressed

Current intermodal freight volumes in Romania are low, and this is partly due to the poor state of the facilities designed to receive them. Expected growth in the intermodal sector means that capacity at this terminal will be outgrown within a short time frame (see Outcomes) expected to be around 2020 and so modernisation is required.

Outcomes

As demonstrated in the table and chart below, the intervention (in conjunction with the other intermodal terminals) provides a significant boost to the amount of intermodal freight travelling by rail.

Table 8.23 Current and Potential Demand (TEU) at Selected Milestone Dates

|  |  | 2011 | 2020 | 2030 | 2040 |
| --- | --- | --- | --- | --- | --- |
| Reference Case | Daily Demand | 9 | 23 | 39 | 62 |
|  | Percentage of Capacity (Current) | 17% | 43% | 72% | 115% |
| With Terminal | Daily Demand | 9 | 55 | 77 | 103 |
|  | Percentage of Capacity (Current) | 17% | 102% | 143% | 191% |
|  | Percentage of Capacity (Best Practice) | 4% | 25% | 36% | 48% |

Figure 8.22: Chart of Difference between All Terminals and Reference Scenarios

- A new build terminal would satisfy the Master Plan criterion at Cluj Napoca however a refurbished terminal is an option. Regardless, the terminal has significant economic benefits:

- Table 8.24: Economic Impact of Terminal Construction

|  | Refurbished | New Build |
| --- | --- | --- |
| Present Value of Costs | 5.3 | 14.4 |
| EIRR | 14.5% | 6.3% |
| Benefit/Cost Ratio | 3.86 | 1.25 |

Implementation:

This scheme would be implemented as suggested in paragraph 8.4.6.  The development of the intermodal terminals is viewed as being very important for the development of rail freight in Romania as they are integral to allowing rail to compete in a modern and growing sector. The refurbishing or new build of the terminal at Cluj Napoca is scheduled for implementation in the period 2015 – 2020 so that capacity can be provided before it is outgrown.

Oradea (Refurbished or New Build Terminal)

AECOM propose that the terminal at Oradea is re-opened from its existing mothballed state to provide an important and useful interchange point between rail and road networks. This is particularly useful given the site’s location at the centre of the north-east, near the rail crossing with Hungary and the E60 core road. If reopening is not possible then AECOM proposes that a new terminal is created.

Operational Objective

OR16, Develop a network of “open user” freight terminals to serve Romania’s highest demand potential regions, cities and major EU markets in Bulgaria and Hungary.

Problem Addressed

Current intermodal freight volumes in Romania are low, and this is partly due to the poor state of the facilities designed to receive them. The opportunity to cater for expected growth in the intermodal sector and to expand facilities near the border in this important economic region of the country together means the rehabilitation of this terminal is recommended.

Outcomes

As demonstrated in the table and chart below, the intervention (in concert with the other intermodal terminals) provides a significant boost to the amount of intermodal freight travelling by rail.

Table 8.25 Current and Potential Demand (TEU) at Selected Milestone Dates

|  |  | 2011 | 2020 | 2030 | 2040 |
| --- | --- | --- | --- | --- | --- |
| Reference Case | Daily Demand | 1 | 2 | 3 | 5 |
|  | Percentage of Capacity (Current) | 2% | 4% | 6% | 9% |
| With Terminal | Daily Demand | 15 | 25 | 34 | 47 |
|  | Percentage of Capacity (Current) | 28% | 46% | 63% | 87% |
|  | Percentage of Capacity (Best Practice) | 7% | 12% | 16% | 22% |

Figure 8.23: Chart of Difference between All Terminals and Reference Scenarios

The new terminal has some economic benefits, with a refurbishment being the more appropriate decision for the intermodal terminal at Oradea as a new build terminal has a poor BCR:

- Table 8.26: Economic Impact of Terminal Construction

|  | Refurbished | New Build |
| --- | --- | --- |
| Present Value of Costs | 5.3 | 14.4 |
| EIRR | 10.6% | 3.8% |
| Benefit/Cost Ratio | 2.46 | 0.80 |

Implementation:

This scheme would be implemented as suggested in paragraph 8.4.6. The development of the intermodal terminals is viewed as being very important for the development of rail freight in Romania as they are integral to allowing rail to compete in a modern and growing sector. The refurbishing or new build of the terminal at Oradea is scheduled for implementation in the period 2021– 2030.

Turda (Refurbished or New Build Terminal)

AECOM propose that the existing terminal at Turda is either refurbished or rebuilt to increase efficiency, lower costs and transit times and to promote growth of the intermodal sector, complementing the terminal at Cluj Napoca.

Operational Objective

OR16, Develop a network of “open user” freight terminals to serve Romania’s highest demand potential regions, cities and major EU markets in Bulgaria and Hungary.

Problem Addressed

Current intermodal freight volumes in Romania are low, and this is partly due to the poor state of the facilities designed to receive them. Expected growth in the intermodal sector means that capacity at this terminal will be outgrown within a short time frame (see Outcomes) and so modernisation is required. This site has continued to be buoyant even in the recession, catering for specific local industries and hence why it justifies consideration for improvement.

Outcomes

As demonstrated in the table and chart below, the intervention (in concert with the other intermodal terminals) provides a significant boost to the amount of intermodal freight travelling by rail.

Table 8.27 Current and Potential Demand (TEU) at Selected Milestone Dates

|  |  | 2011 | 2020 | 2030 | 2040 |
| --- | --- | --- | --- | --- | --- |
| Reference Case | Daily Demand | 11 | 16 | 23 | 32 |
|  | Percentage of Capacity (Current) | 37% | 53% | 77% | 107% |
| With Terminal | Daily Demand | 15 | 28 | 37 | 47 |
|  | Percentage of Capacity (Current) | 50% | 93% | 123% | 157% |
|  | Percentage of Capacity (Best Practice) | 13% | 23% | 31% | 39% |

Figure 8.24: Chart of Difference between All Terminals and Reference Scenarios

- The economic results indicate that a refurbishment is the most appropriate decision for the intermodal terminal at Turdu as a new build terminal has both a poor EIRR and BCR.

- Table 8.28: Economic Impact of Terminal Construction

|  | Refurbishment | New Build |
| --- | --- | --- |
| Present Value of Costs | 5.3 | 14.4 |
| EIRR | 6.6% | 0.1% |
| Benefit/Cost Ratio | 1.26 | 0.41 |

Implementation:

This scheme would be implemented as suggested in paragraph 8.4.6. The development of the intermodal terminals is viewed as being very important for the development of rail freight in Romania as they are integral to allowing rail to compete in a modern and growing sector. The refurbishing of the terminal at Turda is scheduled for implementation in the period 2015 – 2020 so that capacity can be provided before it is outgrown.

Suceava (Refurbished or New BuildTerminal)

AECOM propose that the existing terminal at Suceava is refurbished to increase efficiency, lower costs and transit times and to promote growth of the intermodal sector. Given the strong growth in tonnage already experienced, Suceava offers an opportunity to further develop and encourage intermodal traffic on corridor IX. If rehabilitation is not possible then AECOM proposes that a new terminal is created.

Operational Objective

OR16, Develop a network of “open user” freight terminals to serve Romania’s highest demand potential regions, cities and major EU markets in Bulgaria and Hungary.

Problem Addressed

Current intermodal freight volumes in Romania are low, and this is partly due to the poor state of the facilities designed to receive them. Expected growth in the intermodal sector means that capacity at this terminal will be outgrown within a short time frame (see Outcomes) and so modernisation is required, particularly given the growth witnessed during the recession. Suceava stands to benefit from being on the route of the potential “Project Viking” services and as such should be equipped to serve this new set of north-south intermational intermodal services in an efficient and timely way.

Outcomes

As demonstrated in the table and chart below, the intervention (in concert with the other intermodal interventions) has the following impact:

Table 8.29 Current and Potential Demand (TEU) at Selected Milestone Dates

|  |  | 2011 | 2020 | 2030 | 2040 |
| --- | --- | --- | --- | --- | --- |
| Reference Case | Daily Demand | 11 | 19 | 34 | 55 |
|  | Percentage of Capacity (Current) | 26% | 45% | 81% | 131% |
| With Terminal | Daily Demand | 11 | 63 | 93 | 130 |
|  | Percentage of Capacity (Current) | 26% | 150% | 221% | 310% |
|  | Percentage of Capacity (Best Practice) | 7% | 38% | 55% | 77% |

Figure 8.25: Chart of Difference between All Terminals and Reference Scenarios

- A new build terminal would satisfy the Master Plan criterion at Suceava however the lower costs of a refurbished terminal means that the refurbished terminal has a better BCR when compared to a new build.

- Table 8.30: Economic Impact of Terminal Construction

| Economic Impact | Refurbished | New Build |
| --- | --- | --- |
| Present Value of Costs | 5.3 | 14.4 |
| EIRR | 14.3% | 6.4% |
| Benefit/Cost Ratio | 4.00 | 1.29 |

Implementation:

This scheme would be implemented as suggested in paragraph 8.4.6. The development of the intermodal terminals is viewed as being very important for the development of rail freight in Romania as they are integral to allowing rail to compete in a modern and growing sector. The refurbishing or new build of the terminal at Suceava is scheduled for implementation in the period 2015 – 2020 so that capacity can be provided before momentum from the terminal’s current increased use is lost. Also it is difficult to assess the full benefits of through transit traffic such as Project Viking but it can only be a positive thing.

Bacau (Refurbished or New Build Terminal)

AECOM propose that the existing terminal at Bacau is refurbished and expanded to the south east to increase efficiency, lower costs and transit times and to promote growth of the intermodal sector. If refurbishment is not possible then AECOM proposes that a new terminal is created.

Operational Objective

OR16, Develop a network of “open user” freight terminals to serve Romania’s highest demand potential regions, cities and major EU markets in Bulgaria and Hungary.

Problem Addressed

Current intermodal freight volumes in Romania are low, and this is partly due to the poor state of the facilities designed to receive them. Expected growth in the intermodal sector means that the current layout at Bacau will soon be overwhelmed (see Outcomes), and so improvements need to be made to increase throughput and enable greater container storage.

Outcomes

As demonstrated in the table and chart below, the intervention (in concert with the other intermodal terminals) provides a significant boost to the amount of intermodal freight travelling by rail.

Table 8.31 Current and Potential Demand (TEU) at Selected Milestone Dates

|  |  | 2011 | 2020 | 2030 | 2040 |
| --- | --- | --- | --- | --- | --- |
| Reference Case | Daily Demand | 15 | 24 | 41 | 62 |
|  | Percentage of Capacity (Current) | 36% | 57% | 98% | 148% |
| With Terminal | Daily Demand | 15 | 74 | 107 | 150 |
|  | Percentage of Capacity (Current) | 36% | 176% | 255% | 357% |
|  | Percentage of Capacity (Best Practice) | 9% | 44% | 64% | 89% |

Figure 8.26: Chart of Difference between All Terminals and Reference Scenarios

- A new build terminal would satisfy the Master Plan criterion at Bacau however the lower costs of a refurbished terminal means that the refurbished terminal has a better BCR than a new build.

- Table 8.32: Economic Impact of Terminal Construction

|  | Refurbished | New Build |
| --- | --- | --- |
| Present Value of Costs | 5.3 | 14.4 |
| EIRR | 14.4% | 6.4% |
| Benefit/Cost Ratio | 3.95 | 1.28 |

Implementation:

This scheme would be implemented as suggested in paragraph 8.4.6.  The development of the intermodal terminals is viewed as being very important for the development of rail freight in Romania as they are integral to allowing rail to compete in a modern and growing sector. The refurbishing of the terminal at Bacau is scheduled for implementation in the period 2015 – 2020 so that capacity can be provided before the existing terminal is over-capacity.

General Rail Freight Improvements that will also Benefit Intermodal

Freight will be able to benefit from the improvements being made for the passenger network to become more competitive with road, and as such much of its potential growth depends on the rehabilitation of the network and the improved reliability which is outlined in the main rail section. In particular the rehabilitation of key corridors to their full design speed and capacity will remove speed restrictions, reducing journey times whilst simultaneously improving reliability. This is particularly the case given the current operational speed limit for all freight trains being limited to 80km/h. We have seen no compelling evidence why the speed limit for certain freight trains could not be raised to 120 km/h on rehabilitated tracks, where line speeds, and gradients, allow.

Journey times will benefit from the creation of a sound performance accountability system to encourage effective and consistent timetabling and use through a compensatory regime for delays. Similarly, improvements to train handling procedures and administrative systems at borders will also benefit international freight flows.  The evidence we have seen suggests that private operators can deal with the border crossing procedures in less than an hour, whereas trains run by the state-owned operator CFR Marfa typically take more than two hours to obtain clearances.  Reducing times for administrative procedures improves track utilisation and reduces the need for additional train storage areas.

There are a number of interventions which will provide improvements to Romania’s rail freight potential and network. International interoperability of trains is an important European aim to facilitate long distance international rail freight and therefore, as part of the rehabilitation programme, it is important that axle weight limits on all rehabilitated corridors are improved to 22.5 tonnes, in line with European norms. Although a proportion of the existing legacy rolling stock will not be able to make use of higher axle weights, some wagons are already suitable and all new and refurbished wagons would have enhanced capability. This will not only enable better use of most rolling stock (as wagons will not need to be underloaded) but also smooth the flow of trains from other countries as they will not have to be loaded to meet Romania’s unique axle weight restrictions but instead be able to travel widely across the EU. On a 30 wagon train, this could improve productivity by approximately 15% due to the increase in tonnage that could potentially be carried (an extra 240 tonnes). Although this will not be the case on every single service operated, it is an important increase in capacity and efficiency. This is a key intervention and will carry on throughout the duration of the masterplan, resulting in a network of reliable, more productive freight corridors across the country.

The rehabilitation programme will enable better speeds for freight trains across the network, in particular this should be capitalised on through the creation of a two-tier system differentiating between heavier bulk and lighter intermodal freight trains. It is recommended that containers are run on dedicated block trains rather than the existing procedure of transporting a proportion of containers on the stopping pick-up services or mixed wagon trains. The intermodal block services should be capable of speeds of 120km/h and prioritised over slower freight services and even some slow passenger services in order to ensure lower journey times and improved journey time reliability. Intermodal transport is a key future market sector for rail freight, and offers the best potential to ensure sustainable modal shift from road to rail transport provided that rail’s offer is competitive. Intermodal and bulk rail freight transport will also benefit from a range of technological interventions. There is an opportunity to streamline and increase efficiency through improved training and use of technology. Whilst some modern systems are utilised on the network (ARGOS and the utilisation of GPS tracking by some private rail freight operators for example) this is currently limited. The roll-out of technology is further hampered by the traditional and somewhat out-moded training delivered to workers on the railway, therefore the introduction of new technology should be integrated with a revamped training system to make best use of the new facilities both in the short and long-term.

The privatisation of CFR Marfa will enable private operators to develop long-term relationships with key customers. Currently the uncertainty over CFR Marfa’s future means that the railway is locked into a cycle of short-term contracts, limited maintenance and an inability to make investment decisions, as there is little long-term thinking in an organisation that is still uncertain over its future.

Such long-term thinking and new technology could also be used on the network to make its use more sustainable. A proportion of current rolling stock is being sub-optimally used. The trains could be used more productively, by being loaded/unloaded quicker, have reduced journey times, carry more payload and generally increase annual revenue per train set per year. An example of a system not used in Romania but fitted to a number of locomotives used by the private sector in the country is regenerative braking, whereby energy used in slowing trains feeds power back into the grid. The life-expired infrastructure is currently unable to accept such inputs. A plan to introduce this technology could significantly lower the cost of providing rail freight services as power use can be reduced by 5% for freight trains and up to 17% on passenger services, and as such a feasibility study into how this could best be implemented is recommended. This should be undertaken immediately as it would be logical to combine this with the other rehabilitation work.

Together the interventions combined provide a blueprint for a successful, sustainable rail freight network in Romania. Not only will the rehabilitation of the rail corridors enable improvements to bulk freight reliability and delivery speeds, but in combination with the other operational interventions it will provide the basis for a strong intermodal offering in competition with road. In particular, this intermodal market has been identified as a key sector for future growth, and this will be encouraged by the proposed network of intermodal terminals which are strategically located to offer comprehensive coverage of Romania’s key economic hubs and population centres.

Summary of Interventions for Intermodal

In order to deliver a competitive intermodal offer, it is necessary for Romania’s intermodal infrastructure to offer a comprehensive and high-quality network of terminals which reflect and serve the forecast future of intermodal rail freight in the country. In addition to the proposed works at Constanta for the facilitation of the import and export trade (including additional terminal capacity in the next ten years as the market expands), the masterplan also recommends the creation or rehabilitation of several other intermodal terminals. There is no doubt that new build offers the best opportunity to construct efficient modern terminals. Analysis has shown that there is strong demand for intermodal services at Iasi, Oradea and Turda, but insufficient demand to make these viable in economic terms, as entirely new build. For these reasons we recommend as a first phase that the existing terminals are refurbished at a lower cost. As demand for intermodal transport increases then it may well be feasible to develop new sites at these locations. If, however, there is a private sector initiative for new builds at these locations, then the Ministry of Transport, through the Master Plan, would support such initiatives.

The plan is to establish a network of ‘open-user’ terminals operated in an efficient manner by the private sector. The network seeks to build on the existing provision of terminals, some of which are already operating actively in the private sector. In other locations it requires major refurbishment or replacement of selected existing terminals previously owned and operated by CFR Marfa. In future these should be operated by the private sector. Whilst all of the terminals will enable and encourage international traffic, the terminals at Timisoara and Iasi will act as gateways to the international network given their locations in the west and east respectively. Iasi also provides the opportunity for transhipment to Soviet gauge services for Ukraine, Russia and much of Eastern Europe. Other terminals are chosen for their location which allows them to serve large centres of the Romanian industrial production, economic activity and population; Craiova, Cluj Napoca, Turdu, Suceava and Bacau.

Along with the proposed tri-modal terminals on the Danube at Drobeta-Turnu-Severin, Giurgiu and Galati, Romania will be able to offer an extensive intermodal terminal network linked by rehabilitated rail lines promising reliable and fast freight services – a competitive offer when compared with road haulage.

This proposed intermodal terminal network for Romania is outlined in Figure 8.27:

Figure 8.27 Proposed Intermodal Terminal Network

As can be seen, comparing this network with a map of the largest population centres in Romania demonstrates that this is a ‘good fit’ in terms of providing access to population and industrial heartlands for potential containerised flows. Population centres are shown in Figure 8.28.

Figure 8.28: Twenty Largest Population Centres in Romania

Overall the intermodal strategy outlined above, when combined with the complete package of water and rail improvements (as well as the tri-modal terminals at Galati, Giurgiu and Drobeta-Turnu-Severin) has the potential to make a significant contribution to Romania’s sustainability and economic growth through the promotion of modal shift and improvements to freight transport. The implementation of the strategy will result in approximately 900,000 fewer road tonne kilometres, over 100,000 additional rail tonne kilometres, over 1.1 million additional water freight tonne kilometres and approximately 400,000 more freight tonne kilometres than the reference case by 2030.  The strategy is summarised in Tables 8.33 and 8.34.

Table 8.33 Summary of Recommended Interventions and Impact on Usage (Daily Figures)

| Intervention | Location | 2011 TEU (Actual) | 2020 TEU (Reference Case) | 2020 TEU (Do Something) |
| --- | --- | --- | --- | --- |
| Build a new terminal | Bucharest | 35 | 82 | 179 |
| Build a new terminal | Craiova | 0 | 0 | 41 |
| Build a new terminal or refurbish existing terminal | Timisoara | 2 | 2 | 40 |
| Build a new terminal or refurbish existing terminal | Cluj Napoca | 9 | 23 | 55 |
| Build a new terminal or refurbish existing terminal | Suceava | 11 | 19 | 63 |
| Build a new terminal or refurbish existing terminal | Bacau | 15 | 24 | 74 |
| Refurbish existing terminal | Iasi | 1 | 2 | 25 |
| Refurbish existing terminal | Oradea | 1 | 2 | 25 |
| Refurbish existing terminal | Turdu | 11 | 16 | 28 |

Table 8.34 Summary of Economic Return on Interventions

| Location | Intervention | Scheme Value (CAPEX) | Benefits | Benefit to Cost Ratio | Economic Internal Rate of Return |
| --- | --- | --- | --- | --- | --- |
| Bucharest | New Build Terminal | 12.8 | 50.1 | 3.91 | 13.8% |
| Craiova | New Build Terminal | 13.6 | 18.8 | 1.39 | 6.6% |
| Timisoara | New Build Terminal | 14.4 | 19.1 | 1.33 | 6.6% |
|  | Refurbished Terminal | 2.3 | 19.1 | 8.45 | 24.4% |
| Cluj Napoca | New Build Terminal | 14.4 | 18.0 | 1.25 | 6.3% |
|  | Refurbished Terminal | 4.7 | 18.0 | 3.86 | 14.5% |
| Suceava | New Build Terminal | 14.4 | 18.6 | 1.29 | 6.4% |
|  | Refurbished Terminal | 4.7 | 18.6 | 4.00 | 14.3% |
| Bacau | New Build Terminal | 14.4 | 18.4 | 1.28 | 6.4% |
|  | Refurbished Terminal | 4.7 | 18.4 | 3.95 | 14.4% |
| Iasi | Refurbished Terminal | 3.9 | 11.7 | 0.81 | 3.9% |
| Oradea | Refurbished Terminal | 4.7 | 11.4 | 2.46 | 10.6% |
| Turdu | Refurbished Terminal | 4.7 | 5.9 | 1.26 | 6.6% |

Funding Analysis

Funding Analysis

Basic Assumptions

Context

A key step in the process of drafting the investment plan for 2020 and 2030 is to define the assumptions regarding the funding allocations available for new investments, on all modes of transport.

This will enable the production of a realistic investment plan, as the final result of the process of identifying problems, defining operational objectives and selecting the best interventions to address these objectives.

Although the projects’ deliverability (or buildability) is not considered a distinct criterion in the multicriteriate analysis (hence it is not part of the prioritisation), the funding restrictions do represent a key element of the Master Plan process, as the investment needs identified in the Master Plan are far greater than the available financial allocations.

Following the Master Plan process a long list of prioritised potential interventions have been identified; matching these with the financial constraints per mode of transport will lead to a realistic implementation calendar for the periods specified in the Terms of Reference, respectively 2020, 2030 and after 2030.

Approach

A spreadsheet model was produced (Appendix A) to quantify the available funds for Master Plan projects to be implemented by 2020 and 2030. Its key assumptions and the considered methodology are described below.

The European Commission has specified that the Master Plan should be developed based on a hierarchical approach to allocation of funds to expenditure items, with only the remaining funds (after these commitments have been fully funded) being available for new investment and maintenance:

The elaboration of the Master Plan shall be based on a clearly endorsed political assumption on the budgetary effort allocated to the transport sector over the next 20 years, including all direct and indirect costs afferent to the construction and maintenance of infrastructures, while taking into account the potential sources from the application of access fees in particular for the railway sector or the tolling revenues if relevant. A letter by the Minister of Finance with these budgetary assumptions has been requested.

This assumption will determine the available budget for new investments. From the total available budget shall be deducted the necessary required routine operation and maintenance costs, calculated through general ratios (EUR/km), based on accepted standards, and obligations stemming from recent and on-going projects. Some potential savings can be generated on the maintenance costs, mostly by planning future reduction of the railway network, including stations.

The mandated expenditures to be allocated as priorities are, as per instructions of DG Regio:

Maintenance costs for restructured network, based on international standards;

Liabilities afferent to current investment projects (loans and availability payment instalments);

Rehabilitation works for the remaining, economically viable railway network, in order to bring it to the expected standard speed; and

National public support to State owned companies, consistent with State Aid rules

A decision of the Romanian Government was issued in December 2013 on the commitment of 2% of GDP for the transport sector, as an action to fulfilling the ex-ante conditionalities related to the Partnership Agreement and the Operational Programmes (among, the production of a Transport Master Plan is one key element). It is clearly specified in this commitment that the allocation of 2% of GDP shall be allocated/used only for investments and maintenance works for the transport infrastructure, on all modes of transport.

The Ministry of Transport confirmed that the debt service, availability payments, national subsidies for the public rail service and national public support to state companies are outside this 2% allocation and that the EU allocations are part of this budget. The inclusion of the EU allocations in the 2% of GDP might have a counter effect on the available remaining budget, as their increase will lead to higher national efforts to cover the necessary co-financing.

Following this hierarchical approach, the structure of the financial plan for 2014-2020 and 2021-2030 is described in Table 9.1 attached.

Table 9.1. Structure of GTMP Financial Plan 2014-2030

| 2% from GDP available for the transport sector | A |  | 2% of GDP over the 2014-2030 period, in real terms |
| --- | --- | --- | --- |
| Maintenance and renewal costs | B1 |  | Considers a gradual increase in maintenance for the road, rail and waterways networks |
| Rehabilitation works for the viable road network | B2 |  | Includes the treatment of backlog road rehabilitation works |
| Available Public Funds for new investments, from which | C | =A-B1-B2 | Represents the available funds after the deduction of the mandatory financial allocations related to maintenance, renewals and rehabilitation |
| EU Contribution | C1 |  | Estimated total funds are: CF: 3.404 bn EUR, ERDF: 1.728 bn EUR, CEF: 1.200 bn EU (source: MT, according to provisions in 2014-2020 POIM) |
| National Co-financing | C2 |  | 25% from total eligible costs for projects funded by CF and ERDF 15% from total eligible costs for projects funded by CEF |
| Sustainability indicator/Net National Funds | D | =C-C1-C2 | Represents one of the key figures in the financial plan. It shows the available net national funds after the deduction from C of the EU funds (C1) and the corresponding national co-financing (C2). If D > 0 then the financial plan is sustainable – this means that the EU funded projects can be supported from the National Budget (the 2% of GDP) considering in advance the mandated expenditures included in B1 and B2. |
| Phased projects and Bucharest Metro | E |  | Includes the investment backlog for projects started in the 2007-2013 operational programme that will continue after 2014 (total estimated budget is 1.185 bn EUR) in addition to the allocations for metro projects (estimated budget of 0.727 bn EUR). Source of the data is MT. |
| Available for GTMP projects (total incl. EU + national) | F | =C-E | Represents the available funds for transport investments. Taking in consideration the potential financial corrections and the national overcommitment estimated at 30% the total available funds are equal to G. |
| + 30% Overcommitment | G | =F*1.3 | Includes the financial corrections and potential projects contracted over the initial budget allocations |
| Rail rehabilitation backlog | H |  | Not included, as treatment of rail rehabilitation backlog is one of the key requirements identified as part of the Master Plan process. Rail rehabilitation costs are being prioritised and determined as part of the Master Plan and will therefore being funded out of the remaining available budget |
| Available for GTMP projects excluding rail rehabilitation backlog | I | =G-H | Represents the available estimated budget for Master Plan projects following the treatment of rail rehabilitation backlog |

Source: AECOM, MT

Historical Context

As part of the process of identifying the investment needs for the transport sector, MT produced in June 2013 an analysis on the past expenditures for transport. This analysis included the real historical costs for each transport mode, during 2007-2012 and estimations for year 2013.

Table 9.2. Total expenditures for transport and percentage from GDP, 2007-2013 (’000 lei)

| Year | Total for transport | GDP | % of GDP |
| --- | --- | --- | --- |
| 2007 | 1.613.269 | 416.006.800 | 0,39 |
| 2008 | 3.670.717 | 514.700.000 | 0,71 |
| 2009 | 6.428.839 | 501.139.400 | 1,28 |
| 2010 | 6.502.971 | 523.693.300 | 1,24 |
| 2011 | 7.129.614 | 556.708.400 | 1,28 |
| 2012 | 8.036.044 | 587.466.400 | 1,37 |
| 2013 | 10.562.387 | 599.215.728 | 1,76 |

Source: MT

Based on these historical figures and on the assessment of future investment needs,

    - 2015		2.15%

    - 2016		2.35%

    - 2017		2.00%

    - 2018		2.00%

    - 2019		2.00%

    - 2020		2.00%

GDP Forecast Scenario

The assumptions for the GDP forecast scenario were:

For the period 2014-2017, the source is the latest available GDP forecasts published by the National Committee of Prognosis. It considers  the following real growth rates:

    - 2015		2.5%

    - 2016		3.0%

    - 2017		3.3%

After year 2017, the National Transport Model central forecasts were used (a real increase of 3.5% p.a.)

The estimated GDP for year 2014, according to CNP, is 664.4 bn Lei, representing a real increase of 2.3% compared to 2013.

Mandated Expenditure for Maintenance and Renewals

As stated in section 11.1, the available funds for new investments will be determined after the deduction of the necessary expenditures for maintenance and renewals.

(B1)	Rail maintenance was determined as per AECOM analysis of spend required to cover maintenance and repairs on rail network which retains 55% of entire current network including lines within stations and sidings and 99% of passenger/freight tonne kms Rail network; it comprises a gradual increase from the current expenditure of 324 mill EUR to the desired level of 532 mill EUR, over a period of 6 years (see Section 6.5).

(B1)	Road maintenance considers an estimate of Maintenance and Renewal Costs based on World Bank Cost Recovery Model; it comprises a gradual increase from the current expenditure of 191 mill EUR to the desired level of 679 mill EUR, over a period of 6 years (see Section 5.6);

(B1)	For the Danube fairway maintenance works, a significant increase was considered (up to 25 million EUR per year, according to the needs assessment, based on the analysis of the existing conditions). See Section 8 for additional details.

(B2)	Road rehabilitation backlog was calculated based on AECOM estimates of Rehabilitation Costs (Lifecycle) using World Bank Cost Recovery Financial Model, with current status of pavement based on information provided by CNANDR. It was assumed that rehabilitation/removal of backlog will be completed in 2020, considering a gradual increase over the first 6 years period, with road lifecycle costs post 2020 coming under standard renewals (see Section 5.6). The total estimated corresponding budget is 4.578 bn EUR over the 2014-2020 period, equivalent to 654 mill EUR per annum.

Rail rehabilitation backlog will be treated part of the Master Plan process of identifying and prioritising interventions. See Section 6.5 for further details.

Available Expenditure for Investment

Based on the methodology described in section 11.1 and on the committed expenditures estimated in section 11.2, the total available budget for investments in the transport sector is estimated, for the timelines 2020 and 2030.

Following consultations with MT, an assumed distribution on modes was established, considering the provisions of the 2014-2020 Operational Programme for Transport. These were extrapolated further to the 2021-2030 period. The assumed distribution on modes and specific investments for year 2020 is:

51% - road transport

  - 90% -  construction of new motorways and expressways

  - 2% - safety interventions

  - 3% - construction of bypasses

  - 5% - modernisation of national roads

44% - rail transport

5% - investments in ports and waterways, aviation and intermodal, out of which:

- 66% - ports and waterways

- 21% - aviation

- 13% - intermodal transport

The headline figures of the financial plan are presented in Table 9.3 and illustrated in Figure 9.1.

Table 9.3 Distribution of expenditures in the transport sector for 2014-2020 and 2021-2030 (mill EUR, 2014 fixed prices)

|  |  |  |  | 2014-2020 |  |  | 2021-2030 |  |  | 2014-2030 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2% from GDP available for the transport sector | A |  |  | 22,599 |  |  | 43,216 |  |  | 65,815 |  |
| Maintenance and lifecycle costs | B1 |  |  | 7,260 | 32.1% |  | 20,650 | 47.8% |  | 27,910 | 42.4% |
| Rehabilitation works for the viable road network | B2 |  |  | 4,578 | 20.3% |  | 0 | 0.0% |  | 4,578 | 7.0% |
| Available Public Funds for investments, including EU and National Contribution, from which | C | =A-B1-B2 |  | 10,761 | 47.6% |  | 22,566 | 52.2% |  | 33,327 | 50.6% |
| EU Contribution | C1 |  |  | 6,332 |  |  | 9,046 |  |  | 15,379 |  |
| National Co-financing | C2 |  |  | 1,923 |  |  | 2,747 |  |  | 4,669 |  |
| Sustainability indicator/Net National Funds | D | =C-C1-C2 |  | 2,506 |  |  | 10,773 |  |  | 13,279 |  |
| Phased projects and Bucharest Metro | E |  |  | 1,911 |  |  | 3,200 |  |  | 5,111 |  |
| Available for GTMP projects (total inc. EU + national) | F | =C-E |  | 8,850 | 39.2% |  | 19,366 | 44.8% |  | 28,216 | 42.9% |
| + 30% Overcommitment | G | =F*1.3 |  | 11,505 |  |  | 25,176 |  |  | 36,680 |  |
| Roads |  |  |  | 5,867 | 51% |  | 12,840 | 51% |  | 18,707 | 51% |
| Construction of Motorways and Expressways |  |  |  | 5,281 | 90% |  | 11,556 | 90% |  | 16,836 | 90% |
| Safety Interventions |  |  |  | 100 | 2% |  | 50 | 0% |  | 150 | 1% |
| Construction of Bypasses |  |  |  | 232 | 4% |  | 834 | 6% |  | 1,066 | 6% |
| Modernisation of National Roads |  |  |  | 255 | 4% |  | 400 | 3% |  | 655 | 4% |
| Rail |  |  |  | 5,062 | 44% |  | 11,077 | 44% |  | 16,139 | 44% |
| IWT, Aviation, Intermodals |  |  |  | 575 | 5% |  | 1,259 | 5% |  | 1,834 | 5% |
| Ports, IWT |  |  |  | 324 | 56% |  | 732 | 66% |  | 1,056 | 58% |
| Aviation |  |  |  | 188 | 33% |  | 357 | 28% |  | 545 | 30% |
| Intermodal |  |  |  | 63 | 11% |  | 170 | 14% |  | 233 | 13% |

Source: AECOM GTMP Financial Plan

Figure 9.1. Distribution of funds on expenditure categories and modes (billion EUR, 2014 fixed prices)

Source: AECOM GTMP Financial Plan

Some key conclusions are included below:

Between 2014-2030, the committed expenditures for maintenance and treatment of rehabilitation backlog amount to more than 50% of the total available budget. This would require a firm commitment from the Romanian Government to assure the needed financial resources to cover these allocations. This is a critical factor in the development of a sustainable transport sector.

2% of GDP represents a significant increase from historic average of 1.15% for 2007-2013 (Table 9.2).

There is a significant increase in the available average budget per annum, from 1.264 bn EUR (2014-2020) to 1.937 bn EUR (2021-2030). This is due to the assumed real increase in GDP.

After 2020, treatment of road rehabilitation backlog is included in the maintenance costs.

The funding allocations by sector are then used in the development of the prioritised Investment Plan, discussed in Section 12.

Overall Strategy, 2020 and 2030

Overall Strategy, 2020 and 2030

Introduction

The General Transport Master Plan (GTMP) represents a unique opportunity for Romania.  For the first time Romania will have a soundly-based, comprehensive plan for all the major modes of transport, for the period up to 2030.  It provides a staged programme of interventions which encompass not only proposals to improve the transport infrastructure, but also dealing with maintenance, management and operations, and safety.

The Master Plan also provides the justification for projects to be included in the Sectoral Operational Programme for Transport (SOPT) for the period 2014 – 2020 in detail, and for projects promoted under the Connecting Europe Facility (CEF).  It will also provide the basis for projects to be financed in the period beyond 2020, although the precise nature of the successor programmes to the SOPT have yet to be decided.

The Master Plan will only succeed in its objectives if there is continuous, whole-hearted support for its programme over the medium and long term.  Major transport investments take 6-10 tears to plan, two – four years to build, and have an economic life of 30-50 years.  This applies not only to the organisations who are responsible for implementation of the projects and policies, but also to the successive Governments and key Ministries such as Transport, European Funds, and Finance.

Providing good-quality transport is not an end in itself.  Efficient transport is a critical component of economic development, globally and nationally.  Transport availability affects global development patterns and can be a boost or a barrier to economic growth within individual nations. Transportation investments link factors of production together in a web of relationships between producers and consumers to create a more efficient division of production, leverage geographical comparative advantage, and provide the means to expand economies of scale and scope.

The overall process for creating the Master Plan is shown on Figure 10.1.  The process has three main elements

a Policy input which determines the overall objectives, the funds available to implement the Master Plan; the Strategic Environmental Assessment (SEA), and feedback from Public Consultation;

the Appraisal Process, which determines how projects are selected, tested, and the criteria for inclusion in the Master Plan and Implementation Strategy; and

the Analytical Tools for testing and quantifying the impact of projects.  The most important are the National Transport Model (NTM) and the Cost-Benefit Analysis (CBA) Tool.

At various stages in the Project key Reports were produced which describe the important processes in more detail.  These are shown on Figure 10.1.

- Figure 10.1  Master Plan Processes

Existing Situation

The Plan involved a thorough investigation of the existing conditions, problems and their underlying causes.  Across all modes, four common themes emerged:

Romania has a serious infrastructure deficit, in terms of its quality: coverage of the transport networks is generally good.

Inadequate Maintenance and Renewals, particularly of the rail networks has been inadequate leading to a decline in level of service and reliability, and contributing to a decline in passenger volumes especially, and to a lesser extent, freight volumes.

This situation is largely a result of inadequate finance over a long period of time, but it is exacerbated by Management and Operating practices, which affect rail, road and water transport, but again rail is particularly adversely affected.

Safety is a concern, particularly on the road network where Romania has the worst safety record in Europe.

In the following paragraphs we give an overview of the current status of each mode of transport, the most serious issues to be addressed, and a summary of the Master Plan approach in addressing these problems.  There is a discussion in detail in each of the modal chapters in this Report (Chapters 4-8), and a detailed analysis of the problems of each mode in the Problem Definition Report (available on the Ministry of Transport’s website).  The connection between Problems, Objectives and Interventions is described in a series of Technical Notes.

The availability of funds constrains investment in transport in Romania, as it does in all European countries.  Therefore, the Master Plan cannot contain solutions for all the identified problems and still remain a realistic plan.  Therefore, the final list of interventions is a list of priorities for public sector investment for the foreseeable future.

Rail Transport

In our opinion, it is no exaggeration to say that Romanian Railways are in a crisis situation.  Several steps are required.  These include:

serious reforms to the structure of the railway and the PSC,

substantially increased spending on maintenance and renewals to prevent further deterioration,

increased investment in rehabilitation to current design speeds, and

the introduction of regular interval timetable, convenient for passenger needs.

Unless these steps are taken, it is our view that within 10 years the railway will cease to play a national role in Romania.

Since 1990 passenger kilometres have fallen by 90%, and freight kilometres by 70%, although the position with rail freight has stabilised.  Average speeds for passenger trains have fallen to 45kph in 2012 from 60kph in 1990, and the average speed of freight trains is a mere 23kph.  Between 60-80% of the track-related assets are life expired, there were 1,800 temporary speed restrictions in 2012, and we estimate that current speeds are 20-30% below the design speed of the track.  The track and other fixed assets are under-utilised: about 90% of traffic (both passengers and freight) is transported on 54% of the routes (63% of track-km), whilst about 20% of the routes (14% of track-km) carry only 1% of the traffic.  1,000 stations generate less than 50 trips per day and 533 stations have less than 10 passengers per day.

A trend-based forecast suggests that passenger kilometres would decline by a further 75% by 2030, while the NTM analysis suggests a decline of 22% by 2020 and 40% by 2030.  Whichever forecast is assumed, the future is bleak without drastic action.

These reductions are primarily due to three factors:

inadequate maintenance which leads to longer, and therefore uncompetitive, journey times;

a timetable which is not commercially driven (the PSC is mainly dependant on train kms rather than timetables which meet customer needs); and

demographic changes such as the dispersion of population and the increase in car ownership.

Tests using the National Model suggest that inadequate maintenance is responsible for the lare majority of the forecast 22% decline in passenger kilometres.

The Master Plan proposals concentrate largely on the first two elements,that of increasing maintenance spend to European norms, and comprehensive rehabilitation of the mainline network, combined with introducing a regular interval timetable for Inter-Regio services operated by modern rolling stock.  At current (low) levels of car ownership, there is little that Government can do to limit increasein ownership, but there are many interventions which can be introduced to make rail more attractive and increase rail useage.

There are also institutional changes that can be made which will help to make the rail industry in Romania more dynamic.  These are included in the interventions.

Road Transport

Road maintenance is also inadequate, although the situation is not as serious as that of rail.  Only 50% of the National Road network in good condition, and approximately 65% of the national network is beyond its service life.. The estimate using the method developed for the World Bank suggests that regular maintenance and renewals expenditure should increase by €560m and rehabilitation and modernisation by €650m per annum to clear the backlog up to 2020.

The Master Plan therefore contains a larger financial allocation for road maintenance and renewals that will, over time, keep the national network in a mostly good condition.  The Plan also makes a recommendation regarding the organisation of the maintenance contracts which will improve the quality and efficiency of maintenance.

Slow journey times lead to inefficient use of both working and non-working time and have an adverse impact on national and regional economies, and reduce employment, commercial, shopping and leisure activities.  Romania is a large country with many regional centres, and the economic competitiveness of these centres is adversely affected by the poor level of service offered by the road network.

The level of service provided by the road network is generally poor, in terms of average speeds.  These are 66 kph on the National Road network, 44% below the target of 100kph for the EU Core and Comprehensive Networks.  Romania has the lowest provision of motorway-standard road in the EU per head of population.

The Master Plan therefore contains proposals to develop a network of motorways which will link the main economic regions of Romania, and it’s main trading partners, with a motorway standard network.  This will be supplemented by a modern expressway-standard (ie 2*2 lanes with grade-separated junctions) network which will complete national network of high quality roads.

Romanian has the worst road safety record in the EU.  In terms of the following Key Performance Indicators:

Fatalities per million inhabitants: 94 against an EU average of 60, rank 24th out of 28

Fatalities per 10 billion passenger kilometres: 259 versus an EU average of 61, rank 28th out of 28; and

Fatalities per million passenger cars: 466 versus an EU average of 126, rank 28th out of 28.

While motorways and expressways have a much lower accident rate than single-carriageway roads, it is not feasible, or economic, to replace all single carriageway roads by these safer roads.  Therefore, the Master Plan also contains proposals for low cost safety improvements at 138 worst locations (“blackspots”) for accidents across the country.  These projects give a very good return in economic terms, and will provide safer environments for both pedestrians and drivers before the much more expensive road building programme is completed.

Ports and Waterways

Romania has three main maritime ports, Constanta, Galati and Braila.  The latter two are located on the maritime section of the River Danube, while Constanta is a deep water port on the Black Sea.  It is connected with the River Danube via a man-made channel, and has good road and rail connections with Bucharest, and thence the south and west of the country and Hungary.

Connections of these three ports with central, and Northern Romania are less good and the road and rail projects adrees this issue.

There are also smaller maritime ports of Mangalia and Midia on the Black Sea, and a series of ports on the River Danube, the largest of which are Giurgiu, Oltenita and Drobeta Turnu-Severin.

Constanta is by far the largest Port in Romania.  Table 10.1 shows data of tonnes handled in 2011 for the top 10 ports in Romania.  Constanta handled 83% of the total freight in these ports, the next highest being Galati with 9%.  These data demonstrate the importance of Constanta as the main maritime gateway for Romania’s imports and exports.

- Table 10.1  Freight Handled at Romanian Ports

| Rank | Port Name | Port Type | Tonnes Handled (per annum, 2011) | % of top 10 ports |
| --- | --- | --- | --- | --- |
| 1 | Constanta | Maritime (Deep Sea) | 46,000,000 | 82.6% |
| 2 | Galati | Maritime | 5,100,000 | 9.2% |
| 3 | Tulcea | Maritime | 1,650,000 | 3.0% |
| 4 | Braila | Maritime | 1,203,000 | 2.2% |
| 5 | Oltenita | Fluvial | 508,000 | 0.9% |
| 6 | Drobeta-Turnu-Severin | Fluvial | 490,000 | 0.9% |
| 7 | Giurgiu | Fluvial | 256,000 | 0.5% |
| 8 | Orsova | Fluvial | 188,000 | 0.3% |
| 9 | Calafat | Fluvial | 139,000 | 0.2% |
| 10 | Cernavoda | Fluvial | 132,000 | 0.2% |
| Total |  |  | 55,666,000 | 100.0% |

- Source: MT Naval Directorate

Nonetheless, the Master Plan recognises that investment in selected Ports, as well as at Constanta, is required to exploit the opportunities that the River Danube offers for particular commodities where water transport is competitive.  There are two main issues to be addressed:

First, the River Danube is a natural waterway and as such experiences continual problems of variability in the depth and width of the navigable channel (the so-called “fairway”).  This leads to delays and unpredictable journey times which do not meet the requirements of today’s logistics industry; and

Secondly, the maritime Ports in particular often have sufficient theoretical capacity but the real problem at many of Romania’s Ports is not a shortage of capacity per se, but antiquated and inefficient infrastructure that is not suited to the modern logistics industry.  An efficient and competitive Port requires sufficient berthing, modern crainage and handling equipment for its existing and future markets, modern storage facilities for specific commodities (eg cereals), and good land connections.

The Master Plan proposals therefore concentrate on investment in making the Danube navigable all the year round (except during extreme weather events), and investment in modern facilities at Ports which have a long term future.

Similar to the roads and rail sectors, inadequate maintenance is also an issue for waterways.  Romania spends 11,300 EUR per km maintaining the section of the Danube for which it is responsible, compared with 250,000 EUR per km, which is spent by Austria.  Romania’s Danube users rely on maintenance by Bulgaria on its section, and the latest information we have is that Bulgaria currently spends only 2,100 EUR per km.

Aviation

The aviation sector in Romania is well developed with a number of major airlines serving destinations across Europe, particularly the major national and international airport at Bucharest, but also from the west of Romania.  The sector is well developed with a number of major airlines serving destinations across Europe, particularly the major national and international airport at Bucharest, but also from the west of Romania.

Airports play an important role in the economic development of a region, as well as the nation as a whole.  Airports facilitate the rapid movement of people and high value, time-sensitive goods and therefore foster trade and commerce.  Tourism is relatively undeveloped in Romania.  Airports, together with services by low-cost operators in particular, offer increased accessibility, which in turn fuels the tourism sector.  Increasing the number of visitors and airport users means more money flows into the local economy.  Airports play an important role in the economic development of a region, as well as the nation as a whole.  Airports facilitate the rapid movement of people and high value, time-sensitive goods and therefore foster trade and commerce.  Tourism is relatively undeveloped in Romania.  Airports, together with services by low-cost operators in particular, offer increased accessibility, which in turn fuels the tourism sector.  Increasing the number of visitors and airport users means more money flows into the local economy.

The numbers of passengers using Romanian airports is set out in Table 10.2 below.  Bucharest (Henri Coanda) is the national air gateway for international travelers, and it accounts for approximately 67% of all passenger traffic.  Together with Timisoara and Cluj, these three airports accounts for 89% of passenger traffic.

- Table 10.2  Passengers at Romanian Airports, 2012

| Airport | Domestic | International | TOTAL |
| --- | --- | --- | --- |
| Bucharest (Henri Coanda) | 649,682 | 6,670,884 | 7,320,566 |
| Timisoara | 336,152 | 1,019,867 | 1,356,019 |
| Cluj-Napoca | 189,139 | 815,682 | 1,004,821 |
| Bacau | 21,106 | 306,308 | 327,414 |
| Targu Mures | 10,477 | 216,361 | 226,838 |
| Iasi | 139,185 | 45,298 | 184,483 |
| Sibiu | 26,482 | 150,424 | 176,906 |
| Constanta | 11,647 | 64,817 | 76,464 |
| Oradea | 58,887 | 1,659 | 60,546 |
| Craiova | 19,397 | 11,872 | 31,269 |
| Suceava | 26,224 | 984 | 27,208 |
| Satu Mare | 19,534 | 3,207 | 22,741 |
| Baia Mare | 18,017 | 551 | 18,568 |
| Arad | 0 | 0 | 0 |
| Brasov | 0 | 0 | 0 |
| TOTAL | 1,525,929 | 9,307,914 | 10,833,843 |

- Source: Civil Aviation Authority

Nevertheless, in a country with the size, and topography of Romania there is potential for air to play an increasing role.  The Master Plan identifies a hierarchy of airports to give regional access to international air travel, as well as domestic connectivity to more localised catchments.  The designations are as follows:

Major International Airport - airport with international routes serviced by national carriers, low cost carriers, chartered flights (above 5 million passengers per annum)

International Hub Airports - airports that serves as hubs with European and regional routes served by national carriers, low cost carriers and chartered flights (between 1 million and 5 million passengers per annum)

Regional Airports - airports mostly served by low cost carriers and chartered flights within Romania and adjacent countries (between 30,000 and 1 million passengers per annum)

Smaller Regional Airports - airports that is mainly served by domestic and chartered flights (below 30,000 passengers per annum).

Further details are given in Chapter 7.

That said there are a number of issues to be addressed in order to ensure continued growth and competitiveness of the sector.  These are:

Upgrading of navigational equipment, and aircraft de-icing equipment, to enable all the year round operations except during extreme weather events;

Lengthening and strengthening runways to enable airports to handle the types of aircraft typically used by European operators, and to perform their designated roles;

Increasing taxiway and apron capacity where there is sufficient demand;

More detailed assessments of the demand for air cargo terminals, especially at regional airports;

Increasing passenger terminal capacity to meet forecast demand; and

Improvements to surface access (fixed public transport links such as metro and heavy rail) where demand is sufficient to justify these projects.

The air sector differs from other modes in that demand is heavily dependent on both terminal facilities and services.  An airport without appropriate flights does not generate air traffic, but in turn an airline will not start services unless suitable facilities are available on the ground.  This reality has two practical consequences for the Master Plan:

Our forecasts for air passenger traffic are asessments of potential demand if the airport can attract services, particularly international flights, commensurate with its designation; and

Our recommendations for investment in increased capacity are conditional upon airports having agreements with operators for additional flights in place, if the airport improves its facilities.  We regognise the practical problems with such a policy but the master Plan cannot support speculative investments.

Intermodal Transport

Intermodal transport, by which we mean road/rail and water/rail where the trunk haul is made by rail, represents the future for modern rail freight transport. Since the 1960s there has been a global trend of increased containerisation of goods. Containers are unitised in a standard format which allows global shipping companies (as well as other transport companies) to effectively and efficiently plan their loads. It also means that handling equipment is the same, and handling costs are far below those of traditional methods.  Containerisation reduces the chance of theft and spillage as containers are sealed at source.  It means that turnround time of ships and loaded trains has more than halved and transport costs have reduced sharply. Containers can be used transport most types of commodities including re-frigerated goods and liquids.

However, the degree of containerisation in Romania transports is far smaller than in many other European countries (although greater than in neighbouring Bulgaria). The data in table 10.4 shows that containerisation is particularly advanced in Italy, Turkey, Austria and Germany but Bulgaria and Romania are lagging behind. The volume and percentage in countries like Austria are high due to the Alpine effect where on certain routes it is compulsory to use rail services and there is a considerable volume of transit traffic.  But the sheer size of Romania, and the potential for transit traffic, means that intermodal transport should have a bright future.

The low level of containerisation in Romania is due to a number of factors.  These include historic working practices, relatively cheap labour, lack of investment in inland terminals including cranage and secure storage, lack of modern supply chains, lack of modern rail wagons designed for efficient container operation, and very slow, unreliable journey times by rail.

The Master Plan prposals address these issues in several ways: We have identified locations where intermodal terminals will be successful in atttracting viable throughput of more than 7,500 TEU per annum by 2030.  Once established we are cofident that our forecasts are at the lower end, since in Romania intermodal transport is essentially a „new Mode”.  In chapter 8 we also set out potential throughputs which are considerably in excess of our base forecasts.

These terminals will provide a network of intermodal terminals across Romania.  There are four major „tri-modal” terminals located at Constanta and the major river ports, six existing terminals which are operating successfully, and 10 new terminals which could either be re-furbishments of existing terminals, or, more likely, new builds with modern facilities.  Some of these could be integrated into commercial developments such as „freight villages” with other wharehousing and distribution facilities, haulage companies businesses and vehicle servicing, but we have assesed and costed only the intermodal elements.

The operation of intermodal terminals is closely integrated with the logistics industry, which is 100% privately run.  Having consulted existing and potential operators of intermodal facilities, our recommendation is that the intermodal terminals should be designed, built and operated by the private sector in order to fully exploit their potential.  However, as we point out, operating intermodal terminals is virtually a start-up industry in Romania and in many cases a small amount of public invstment may be required.

We recommend that, at a minimum, the public sector contributes planning permits, land purchase, and the connections to the national, or local road and rail networks.

The rail rehabilitation proposals described outlined above will also benefit rail freight transport, because the reabilitation will include re-signalling, regenerative braking .and increases in axle loads.  But for block container trains in particular we also recommend raising the speed limit to 120 kph and to change the protocols of train control so that these trains are given higher priority.

Overall Objectives

High-Level Objectives

A series of high-level objectives for the Master Plan were formulated which guided the formulation of interventions proposed for the Plan.  These objectives were derived from the following National and European documents:

The objectives in the Terms of Reference (ToR)

Mission: Minister’s Statement in the Forward to the Strategic Plan of the Ministry of Transport and Infrastructure

EU White Paper on Transport 2011

Romanian Government Statement on Transport Policy (Program de Guvernare 2012-2016)

Partnership Agreement 2014 – 2020 (see pages 176-177)

AECOM Existing Conditions Report

National Spatial Plan Section 1 Transport Networks

EU Core Networks for Road and Rail

From these documents the following highlevel objectives were derived::

- Economic Efficiency: the transport system should be economically efficient as far as transport operations and users themselves are concerned.  Specifically, the benefits of investments in transport should exceed the cost of that investment.

- Sustainability: the transport system must economically, financially and environmentally sustainable.  The so-called sustainable modes of transport – rail, bus and waterways - which are more energy efficient and have lower emissions should be developed as a priority.

- Safety: investment in transport should produce a safer transport system.  The economic cost of accidents is monetised in the economic evaluation, but since the goals of the Government, the EU and the ToR are clearly a reduction in transport-related accidents, safety must remain as a separate objective.

- Environmental Impact: Transport investment should minimise negative impact on the physical environment.

- Balanced Economic Development.  The transport system should be configured to enable economic development both nationally and regionally.  The investment should also favour equity as far as Romanian citizens are concerned.

Funding:  Availability of EC funding from the Structural Funds (CF and ERDF, Connecting Europe Facility (CEF)) and PPP will affect “buildability” and therefore the prioritisation of projects.  The overall programme will have to be within a realistic estimate of national and other funds over the plan period.

Below these objectives, operational objectives were established for each mode based on a detailed, location-specific analysis of the problems which each mode experiences.  Extensive consultations were carried with stakeholders in both the public and private sector as part of the problem analysis.

Operational Objectives

The operational objectives were derived from the in-depth analysis of the problems of each mode of transport.  Care was taken in the problem analysis to ensure that each defined problem was specific, identified the underlying cause, and was geographically precise wherever possible.  The operational objectives are therefore specific to each problem, but were grouped under the high-level objectives described above.

The operational objectives are set out in the modal chapters (chapters 4-8), and in the Problems/ Objectives/ Interventions Technical Notes.

Interventions

The definition of interventions follow the setting of operational objectives.  This procedure ensures there is a clear and identifiable connection between high-level objectives, the identified problems and the corresponding operational objectives, and the interventions themselves.  This approach ensures that the interventions address real, transport-related problems.  The use of the National Model, and associated data, ensures that there is a quantitative basis for the problem definition, objectives, and interventions.

We invited the Ministry of Transport and stakeholders to submit their proposals and aspirations for their own areas of responsibility.  The value of the interventions they proposed, which were predominantly for new or improved infrastructure, amounted to €74.8 bn.  The funds available for infrastructure projects are €9.5 bn to 2020, and €24.7 to 2030.  Thus the available funds represent less than half of the aspirations of the project sponsors.

The Master Plan cannot generate additional funding, but it can ensure that, in a situation where funding is limited, the available funds are allocated to areas where they provide the greatest benefit.  The AECOM team, in conjunction with the Ministry of Transport and JASPERS, developed a series of interventions which include, but are not limited to, infrastructure interventions.  In some cases these corresponded to projects proposed by stakeholders, but in many cases the projects were different, or scaled back in scope to meet the specific problem, or the forecast demand.

The analysis of the existing and future situations highlighted the serious shortcomings in maintenance and renewals on the railways and roads.  The overall strategy therefore has two main pillars:

The protection of existing assets by ring-fencing long term financing for improved maintenance and renewals.  This will benefit all Romanian citizens (and visitors) who use the countries’ transport systems; and

Selected infrastructure improvements, which gave the best value for money, and met the operational objectives.

Testing Projects

Interventions in the domains of rail, road, ports and waterways, and intermodal transport were tested with the National Model.  The model is fully described in the Model Development Report, which is summarised in Chapter of this Report, but there are some important features of the model which need to be fully understood when interpreting the results.

The model uses the well established four stage process which involves:

Stage 1: Trip production and attraction: estimating the total number of passenger journeys or tonnes of freight generated by, and attracted to, each zone.

Stage 2: The distribution of passenger journeys and tonnes of freight between zones;

Stage 3: The choice of for passenger journeys or freight movements between zones

Stage 4: The route chosen between zones for every zone to zone movement of pasengers and freight, for each mode of transport.

Mathematical relationships, calibrated by observed data, determine how the model carries out the functions in each of these four stages.

The model is highly complex – the full runs of the model for three years take approximately 30 hours - but it is important to understand how the interventions affect the model outputs.  Gnerally speaking, the driver of change in distribution of traffic, mode choice and route choice is change in travel cost, either in terms of travel time (since time is money), or the money cost of travel  such as fares, vehicle operating costs, tariffs, and handling charges for freight.  Various examples are given below.  In these example the word “traffic” is generic: it ecompasses all modes of travel, and pasengers and freight.

An improvement to either road infrastructure, or faster trains and more frequent services, will produce a change in the distribution of traffic, because travel between the cities and town served by the new road or better train service will be easier and these settlements will be accessible from a larger area.  In addition, a change in mode choice will occur because one mode, either road or rail in this example, will become relatively more attractive.  The final response will be a change in route choice, because the improved route will offer s quicker journey, and greater capacity.  Not only will traffic transfer to the improved route from the roads or rail services in the same corridor, but will transfer from further afield.  This means that the traffic on a new or significantly improved route will be much greater than the existing traffic.

An improved or new intermodal terminal (or network of terminals since there must be proper facilities at both ends of the joirney) will attract additional containerised traffic because of reduced handling costs.  If combined with faster container trains, the additional traffic will be reinforced.  There will be a switch of traffic from road to rail, through the mode choice mechanism, and some change to the distribution of freight traffic, although not as marked as with passenger travel.

These mechanisms apply in a similar way to all modes of transport.  It is important to note that the process is essentially neutral in its operation: the mechanisms for distribution, mode choice and route choice are mathematical and based on observed behaviour.  Secondly, the changes in volumes of traffic by road, rail, water and intermodal transport are outputs from the National Model, not inputs to it.  Thus there are no pre-conceived targets or outcomes in the process.

A large number of projects were tested with the National Model.  The panel below gives the approximate number.  Many of the projects were tested several times due to variations in the project specifrication, or in combination with other projects, so the numbers given below are conservative:

| Mode | Road | Rail | Ports and Waterways | Intermodal Transport |
| --- | --- | --- | --- | --- |
| Number of Projects | 92 | 34 | 12 | 13 |

The rail projects comprised three elements, infrastructure re-habilitation on a complete long distance route, revised timetables, and new rolling stock, so each rail project is in effect a whole route strategy.

Aviation projects were not tested with the National Model, but with a specially developed aviation model, which reflects the additional traffic which would arise if an airport attracts additional services as well as improves its own facilities.  The model uses a combination of catchment areas and trip rates for different levels of flights to forecast potential passenger traffic.  However, the aviation model does use data from the National Model in the CBA analysis.

As discussed in section 10.2, and in greater detail in Chapter 4 and the Problem Definition Report, road safety is a serious concern in Romania.

Project Appraisal – Role of CBA and MCA

As described in the National Assessment Guidelines, Volume 1, and summarised in Section 2.6 of this Report, projects were initially sorted according to their economic performance.

For road projects, this was a two stage process: first, the individual projects which met each operational objective were assessed using CBA, and secondly, the individual projects were assembled into two alternative “Level 1” strategies which made a coherent network; these were again subject to a CBA, with the best performing strategy and the projects within it being carried forward to an MCA.  Further residual problems were identified assuming the preferred Level 1 strategy was in place, and so-called “Level 2” projects were identified to address these problems.

Strictly speaking, the CBA pass/fail criteria were that projects should have an EIRR greater than 5% (the discount rate), and a Benefit-Cost Ratio (BCR) greater than 1.  However, these criteria were relaxed for rail projects, for a number of reasons:

As we have shown earlier, rail passenger transport in particular has been in decline for a number of years, with the result that existing rail traffic is not always sufficient to justify the large investments required in rail improvements; if CBA criteria are strictly applied it would reinforce this situation which would reinforce railway decline to the point where rail would cease to play a worthwhile role in national transport;

Because of under-investment in railways over a long period, the scale of investment required is larger than it would have been under a “normal” investment programme; this means investment costs are higher;

Rail infrastructure has to be built to meet exacting construction and safety standards; gradients cannot be severe, high speed running requires good horizontal alignment without severe curves, and there are EU standards for minimum line speeds on the Core TEN-T network.  This means that there is limited scope for adjusting standards and therefore costs to improve economic performance.

Following the assessment using CBA, projects were assembled into two scenarios using Multi-Criteriate Analysis (MCA) techniques.  The two scenarios are characterised as follows:

Economically Sustainable (“ES”) Scenario: This scenario emphasises projects which perform well in economic terms, with the economic efficiency criteria having the largest weighting (70%) having the highest weighting.  Projects on the Core TEN-T were given 30% weighting.

Economic and Environmentally Sustainable (“EES”) Scenario: This scenario gives additional weighting to the physical environmental impacts, especially the impact of Natura 2000 sites, with less on economic efficiency.  The weighting for economic efficiency was reduced to 50%, physical impact on the environment 20%, and whether or not the project was located on the Core Ten-T network, 20%.  An additional criterion, with 10% weighting, namely “balanced economic development”, was introduced.  This latter criterion gave some weighting to projects which would assist economic development in Romania’s regions.

The criteria used, which were agreed with MT, JASPERS and the EC, and their weightings, were as follows:

Table 10.3  Criteria and Weights for the ES and EES Scenarios

| Criteria | ES | EES |
| --- | --- | --- |
| Economic Efficiency | 70% | 50% |
| Trans-European Integration/TEN-T Policy | 30% | 20% |
| Environmental Impact | - | 20% |
| Sustainability | Not scored but dealt with the distribution of funds by mode |  |
| Balanced Economic Development | - | 10% |

- Source: AECOM, MT, JASPERS and EC

The measure of economic efficiency includes the benefits due to accident reduction, and changes in emissions and air pollution, so there is an implicit recognition of some environmental impacts in the cost-benefit analysis.

The scoring of each criterion is given in Tables 10.4-5 below, for the ES and EES scenarios.

Table 10.4 Scoring of Projects for the ES Scenario Criteria

| No. | Master Plan High-Level Objective | Criteria | Indicator/ Reference | Weight | Scoring criteria | Scoring (points) |
| --- | --- | --- | --- | --- | --- | --- |
| A | Economic Efficiency | Economic Performance | EIRR | 70% | 5% | 0 |
|  |  |  |  |  | Maximum EIRR | 100 |
|  |  |  |  |  | > 5% < highest EIRR% | Proportionally from 100 = highest EIRR |
| B | Trans-European Integration | Relation with TEN-T network | TEN-T Regulation | 30% | Core TEN-T link | 100 |
|  |  |  |  |  | Comprehensive TEN-T link | 30 |
|  |  |  |  |  | Other links | 0 |
| C | Sustainability | Contribution to cleaner transport modes policy | White Paper | Not scored under MCA but dealt with via pre-allocation of funding per sectors: this scenario assumes 51% roads, 44% for rail and 5% for Ports, IWT, Intermodal and Aviation |  |  |

Source: AECOM, MT, JASPERS and EC

Table 10.5  Scoring of Projects for the ES Scenario Criteria

| No. | Master Plan High-Level Objective | Criteria | Indicator/ Reference | Weight | Scoring criteria | Scoring (points) |
| --- | --- | --- | --- | --- | --- | --- |
| A | Economic Efficiency | Economic Performance | EIRR | 50% | 5% | 0 |
|  |  |  |  |  | Maximum EIRR | 100 |
|  |  |  |  |  | > 5% < highest EIRR% | Proportionally from 100 = highest EIRR |
| B | Trans-European Integration | Relation with TEN-T network | TEN-T Regulation | 20% | Core TEN-T link | 100 |
|  |  |  |  |  | Comprehensive TEN-T link | 50 |
|  |  |  |  |  | Secondary connectivity with TEN-T | 10 |
|  |  |  |  |  | Other links | 0 |
| C | Environmental Impact | Potential environmental impact (in particular on NATURA 2000 sites) | SEA | 20% | Very high | -100 |
|  |  |  |  |  | High | -80 |
|  |  |  |  |  | Moderate | -50 |
|  |  |  |  |  | Low | -30 |
|  |  |  |  |  | None | 0 |
| D | Sustainability | Contribution to cleaner transport modes policy | White Paper | Not scored under MCA but dealt with via pre-allocation of funding per sectors: this scenario assumes 51% roads, 44% for rail and 5% for Ports, IWT, Intermodal and Aviation |  |  |
| E | Balanced Economic Development | Improving the accessibility of less accesible regions | GTMP accessibility maps | 10% | Improving link to an area with low accessibility both to foreign and domestic markets | 100 |
|  |  |  |  |  | Improving link to an area with low accessibility to foreign markets | 70 |
|  |  |  |  |  | Improving link to an area with low accessibility to domestic markets | 50 |
|  |  |  |  |  | Links to areas with good accessibility | 0 |

- Source: AECOM, MT, JASPERS and EC

We should clarify the respective roles of the MCA and the Implementation Strategy.  The MCA procedure gives an “order of merit” to projects, essentially of their value to the economic and social development of Romania.  However, for practical reasons such as project maturity, which includes factors such as availability of Feasibility Studies, environmental investigations, national and local political support, as well as availability of funds, such as Cohesion Funds, which the EC has stipulated must be used first for projects on the currently-defined Core TEN-T the Implementation Strategy will have a different order to the projects to that produced by the MCA procedure.  In other words, the MCA procedure determines which projects should be in the Preferred Strategy, but the overall strategy will remain the same.

In section 10.7 below we give the results of the MCA process and the resulting composition of the ES and EES scenarios.

ES and EES Scenarios

Economically Sustainable (“ES”) Scenario

This section describes the projects included in the ES Scenario following the application of the MCA.  Table 10.6 sets out the ES Scenario road projects according to their scores from the MCA.

Table 10.6 List of road investments – ES Scenario

Source: AECOM GTMP MCA Scoring of Projects

Table 10.7 sets out the EES Scenario road projects according to their scores from the MCA

Table 10.7  EES Scenario Road Projects (latest)

Source: AECOM GTMP MCA Scoring of Projects

Some identified bypasses are part of larger Level 1 or Level 2 projects.  The decision to advance these proposals as separate projects will be determined within the Implementation Strategy which will examine the funding opportunities and the maturity of each project. If the more significant project appears as feasible early in the implementation plan, then a separate bypass will not be needed. A value for money analysis based ont he lifetime forecast for the bypass as an individual entity will determine the adequate solution..

Tables 10.8-9  set out the ES and EES Scenario rail projects according to their scores from the MCA

Table 10.8  ES Scenario Rail Projects

Source: AECOM GTMP MCA Scoring of Projects

Table 10.9  EES Scenario Rail Projects

Source: AECOM GTMP MCA Scoring of Projects

Table 10.10 sets out the ES and EES Scenario Ports and Waterways projects according to their scores from the MCA

Table 10.10  Ports and Waterways Projects, ES and EES Scenarios

Source: AECOM GTMP MCA Scoring of Projects

The Bucharest – Danube Channel could have commenced construction in the period 2020 – 2030.  However, we are reluctant to recommend this project at this time, for several reasons:

Bucharest is already well-connected to the east and west by rail and road, and one of the primary objectives of the Master Plan is to re-invigorate the railway.  Testing with the National Model showed that the channel would have a significant, adverse impact on the Constanta – Bucharest – Craiova – Hungary rail corridor.

The proposals for improvements to the Ports at Giurgiu and Oltenita gave very good economic value, with EIRR of 24% and 30% respectively, for modest investments of €4.3m and €5.6m respectively.  If the Bucharest – Danube Channel were to be implemented, it would jeopardise these investments.

The Channel is expensive and barely economically viable.  It has an estimated cost of €1.5bn, and an EIRR of only 4.7%.

For these reasons we recommend that the viability of the Channel is re-examined in the mid 2020s when the impact of the Port improvements, and rail improvements, would become clearer.

Tables 10.11-12 set out the ES and EES Scenario Aviation projects according to their scores from the MCA.

Table 10.11  Aviation Projects, ES Scenario

Source: AECOM GTMP MCA Scoring of Projects

Table 10.12  Aviation Projects, EES Scenario

The 2020 and 2030 proposals for intermodal transport are shown on Tables 10.13-14, for the ES, and EES scenarios respectively.

Table 10.13  Intermodal Projects, ES Scenarios

Source: AECOM GTMP MCA Scoring of Projects

Table 10.14  Intermodal Projects, EES Scenarios

Source: AECOM GTMP MCA Scoring of Projects

In each of these three scenarios there will be included the interventions on rehabilitation and modernisation of national roads.  These projects will be modernised using the budget set aside for renewals, and included in the funding analysis.  The rehabilitation of these roads does not result in an increase in capacity, or connectivity, but is rather a structure programme for ensuring timely heavy maintenance in order to ensure that serious deteriorisation does not take place in their condition.  The projects were prioritised using a combination of demand and surface condition.  The resulting priority list of projects is shown  as shown in Table 10.15.

Table 10.15  Rehabilitation and Modernisation of National Roads

Appraisal of the ES and EES Scenarios

The final stage in the apparaisal process is the comparison between the ES and EES scenarios.

Projects have been prioritized in two potential development scenarios, as follows:

An Economic Sustainability Scenario (ES);

An Economic and Environmental Sustainability Scenario (EES);

Prioritized projects for each mode of transport have been combined in these two scenarios.  The Scenarios were tested with the National Transport Model, in view of identifying the ones which generate the best economic efficiency results.

Following the economic evaluation, the two scenarios were assessed using an Appraisal Summary Table (AST).  The outcomes of economic, and wider apparaisal, for the two scenarios is given below.

Economic Indicators

Tables 10.16 shows the key economic performance indicators for the ES and EES scenarios.

Table 10.16  Key Economic Indicators, ES and EES Scenario

| Project Title | EES Final Strategy |  | ES Final Strategy |  | Difference (EES - ES) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Undiscounted Costs (million EUR in 2010 prices) | 31,550.65 |  | 32,441.85 |  | -891.19 |  |
| CAPEX (million EUR) | 31,290.57 |  | 32,181.76 |  | -891.19 |  |
| OPEX (million EUR) | 260.08 |  | 260.08 |  | 0.00 |  |
| Discount year | 2010 |  | 2010 |  | - |  |
|  | Incremental Cost or Benefit (Million EUR) counted | Share in Total Costs / Benefits | Incremental Costor Benefit(Million EUR) Discounted | Share in Total Costs / Benefits | Absolute Difference | % of Total Difference in Costs/ Benefits |
| Cost to Infrastructure Manager |  |  |  |  |  |  |
| CAPEX | 18,503.78 | 100% | 18,980.22 | 100% | -476.44 | 100% |
| OPEX | 40.55 | 0% | 40.55 | 0% | 0.00 | 0% |
| Cost to Operator |  |  |  |  |  |  |
| CAPEX | 0.00 | 0% | 0.00 | 0% | 0.00 | 0% |
| OPEX | 17.84 | 0% | 17.84 | 0% | 0.00 | 0% |
| Benefit To Users |  |  |  |  |  |  |
| Value of Time | 30,346.49 | 60.75% | 28,671.32 | 59.28% | 1,675.17 | 105.98% |
| Vehicle Operating Costs | 815.36 | 1.63% | 1,349.44 | 2.79% | -534.09 | -33.79% |
| External Impacts |  |  |  |  |  |  |
| Accidents (Safety) | 12,932.99 | 25.89% | 12,570.52 | 25.99% | 362.47 | 22.93% |
| Noise | 359.53 | 0.72% | 360.25 | 0.74% | -0.72 | -0.05% |
| Air Pollution | 6,425.50 | 12.86% | 6,160.33 | 12.74% | 265.18 | 16.78% |
| Climate Change | -912.19 | -1.83% | -724.82 | -1.50% | -187.37 | -11.85% |
| Present Value of Costs | 18,544.33 |  | 19,020.76 |  | -476.44 |  |
| Present Value of Benefits | 49,950.45 |  | 48,369.81 |  | 1,580.64 |  |
| Net Present Value (NPV) | 31,406.12 |  | 29,349.04 |  | 2,057.07 |  |
| EIRR | 10.86% |  | 10.50% |  | 0.36% |  |
| Benefit/Cost Ratio | 2.69 |  | 2.54 |  | 0.15 |  |

The underlying purpose of the Master Plan is to contribute to Romania’s economic growth.  The undiscounted benefits from the Master Plan total some €170bn, which equates to approximately 2% of Romania’s GDP in the period 2020 – 2050.

The two scenarios were also compared using more comprehensive criteria, which included Environment (physical impacts), Policy with regard to the TEN-T, and Accessibilty.

Table 10.17  ES and EES Scenarios: Appraisal Summary Table (AST)

The conclusion is that the Economic and Environmentally Sustainable scenario performed better, and that this strategy should be the long term plan for Romania.

Funding Analysis

Whilst the process of problems analysis, setting objectives and defining interventions, and the subsequent testing and appraisal of projects gives an order of merit to the projects, it does not generate the funds needed to implement these projects.  It is a Government responsibility to allocate funds to the various sectors for which it is responsible, and the responsibility of Ministries, including of course the Ministry of Transport, to spend that allocation wisely.  The role of the Master Plan is thus to ensure that the funds available are spent on projects that give the best return.

The funding analysis is based on a certain percentage of GDP (2% has been sanctioned by the Ministry of Finance) to be allocated to maintenance and capital investment.  It is recognised that there will, in reality, be other costs incurred by the Government and other parties involved in implementing the Master Plan, but the “rules of the game” are that these two elements are those which should provide the constraint in which the Master Plan projects will be implemented.  These two elements make up the majority of expenditure by the Government.

The so-called “soft measures” will require some financing to implement, but the costs involved are small compared with the maintenance and investment budgets, and in any case most of the soft measures should be sel-financing over time.

To prevent uncertainty regarding the long-term funding assurance for the pipeline of projects included in the Master Plan, a commitment from the Government of Romania is needed to allocate a certain percentage of GDP for the transport sector, seen as an ex-ante conditionality for the European Commission to approve the Master Plan and, further on, the Large Infrastructure Operational Programme (POIM) for the 2014-2020 period.

As an immediate action to this requirement, a decision of the Romanian Government was issued in December 2013 on the commitment of 2% of GDP for the transport sector. It is clearly specified in this commitment that the allocation of 2% of GDP shall be allocated only for investments and maintenance works for the transport infrastructure, on all modes of transport.

In addition to this, the European Commission has specified that the Master Plan should be developed based on a hierarchical approach to allocation of funds to expenditure items, with only the remaining funds (after these commitments have been fully funded) being available for new investment and maintenance. Following these requirements, a spreadsheet model was produced to quantify the available funds for Master Plan projects to be implemented by 2020 and 2030. Its key assumptions and the considered methodology are described below.

One of the particular aspects of the assumptions regarding the financial plan for the Master Plan was that the net EU funding (Cohesion Funds, European Regional Development Funds and Connecting Europe Facility funds) are included in the allocation of 2% of GDP. This means that there is a direct correlation between the required national contributions to sustain the EU funded projects and the available remaining budgets.

Based on the this approach and the estimations for the up-front committed expenditures (related mainly to maintenance, renewals and rehabilitations for road and rail network) the total available budgets for investments in the transport sector, by mode, are estimated for the timelines 2020 and 2030. The headline figures are included in Table 10.18.

Table 10.18  Distribution of expenditures in the transport sector for 2014-2020 and 2021-2030
(mill Eur, 2014 fixed prices)

|  |  |  |  | 2014-2020 |  |  | 2021-2030 |  |  | 2014-2030 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2% from GDP available for the transport sector | A |  |  | 22,599 |  |  | 43,216 |  |  | 65,815 |  |
| Maintenance and lifecycle costs | B1 |  |  | 7,260 | 32.1% |  | 20,650 | 47.8% |  | 27,910 | 42.4% |
| Rehabilitation works for the viable road network | B2 |  |  | 4,578 | 20.3% |  | 0 | 0.0% |  | 4,578 | 7.0% |
| Available Public Funds for investments, including EU and National Contribution, from which | C | =A-B1-B2 |  | 10,761 | 47.6% |  | 22,566 | 52.2% |  | 33,327 | 50.6% |
| EU Contribution | C1 |  |  | 6,332 |  |  | 9,046 |  |  | 15,379 |  |
| National Co-financing | C2 |  |  | 1,923 |  |  | 2,747 |  |  | 4,669 |  |
| Sustainability indicator/Net National Funds | D | =C-C1-C2 |  | 2,506 |  |  | 10,773 |  |  | 13,279 |  |
| Phased projects and Bucharest Metro | E |  |  | 1,911 |  |  | 3,200 |  |  | 5,111 |  |
| Available for GTMP projects (total inc. EU + national) | F | =C-E |  | 8,850 | 39.2% |  | 19,366 | 44.8% |  | 28,216 | 42.9% |
| + 30% Over commitment | G | =F*1.3 |  | 11,505 |  |  | 25,176 |  |  | 36,680 |  |
| Roads |  |  |  | 5,867 | 51% |  | 12,840 | 51% |  | 18,707 | 51% |
| Rail |  |  |  | 5,062 | 44% |  | 11,077 | 44% |  | 16,139 | 44% |
| Ports, Inland Waterways |  |  |  | 324 | 56% |  | 732 | 66% |  | 1,056 | 58% |
| Aviation |  |  |  | 188 | 33% |  | 357 | 28% |  | 545 | 30% |
| Intermodal |  |  |  | 63 | 11% |  | 170 | 14% |  | 233 | 13% |

Source: GTMP Funding Analysis

One of the key figures in the above financial plan is D. Sustainability Indicator. This shows the available net national funds after the deduction from the remaining funds (after the up-front allocations for maintenance, renewals and rehabilitations) of the EU funds and of the corresponding national contribution to sustain the EU funded projects. These represents the net national funds available to sustain projects which are a priority on short term (2020) but which:

- are not eligible for EU funding; or

- are eligible for EU funding but due to financial constraints or exceeded budgets cannot be promoted on short term only from EU budgets.

The total available budget to promote 2020 and 2030 Master Plan projects include two major funding sources:

- EU funds (CF, ERDF and CEF); and

- National Budget to support the co-financing and the projects supported from national funds only.

Total budgets by funding sources are presented in Table 10.19.

Table 10.19 Available budget for Master Plan projects by funding sources (mill EUR, 2014 fixed prices)

| Funding sources | 2014-2020 | 2021-2030 | 2014-2030 |
| --- | --- | --- | --- |
| - Total budget available, from which | 11,505 | 25,176 | 36,680 |
| - Net EU funds | 6,332 | 9,046 | 15,379 |
| Cohesion Funds - CF | 3,404 | 4,863 | 8,267 |
| European Regional Development Funds - ERDF | 1,200 | 1,714 | 2,914 |
| Connecting Europe Facility Funds - CEF | 1,728 | 2,469 | 4,197 |
| - National Co-financing | 2,111 | 3,015 | 5,126 |
| Cohesion Funds - CF | 1,135 | 1,621 | 2,756 |
| European Regional Development Funds - ERDF | 400 | 571 | 971 |
| Connecting Europe Facility Funds - CEF | 576 | 823 | 1,399 |
| - Total budget for EU funded projects (=1+2, EU + National contribution) | 8,443 | 12,062 | 20,505 |
| Cohesion Funds - CF | 4,539 | 6,484 | 11,023 |
| European Regional Development Funds - ERDF | 1,600 | 2,286 | 3,886 |
| Connecting Europe Facility Funds - CEF | 2,304 | 3,292 | 5,596 |
| - National Budget (=1-4) | 3,061 | 13,114 | 16,175 |
| - Total national funds (=3+5) | 5,172 | 16,129 | 21,302 |

Source: GTMP Funding Analysis

On average, for the first period of project implementation 2014-2020, 1.3 bn eur are available for projects respectively 1.9 bn EUR for the next 2021-2030 period. The sources of funding for the overall budget of 36.7 bn over the whole Master Plan period 2014-2030 are European Union funds (42%) and Romanian National Budget (58%, including the necessary co-financing for EU-funded projects).

To estimate the impact on the list of projects that can be implemented in the horizon of the Master Plan (2014-2030) several scenarios were developed regarding the available funds allocated from the National Budget (Table 10.20).

Table 10.20 Available budget for Master Plan projects as function of % of GDP (mill EUR)

| Percentage of GDP | 2014-2020 | 2021-2030 | 2014-2030 |
| --- | --- | --- | --- |
| 2.00% | 11,505 | 25,176 | 36,681 |
| 2.25% | 15,177 | 32,198 | 47,375 |
| 2.50% | 18,849 | 39,221 | 58,070 |

Source: GTMP Funding Analysis

Total investment cost for projects identified in the Master Plan process, across all modes, is 45.3 bn EUR in fixed 2014 prices. A sensitivity analysis shows that an allocation of 2.2% of GDP is enough to support the full list of identified interventions.

Public Consultation

Reference has already been made to the wide-ranging consultations which took place while the Master Plan was being formulated, particularly with respect to the specification of interventions.

Following the publication of the Master Plan at the end of August, a series of Public Consultations on the Master Plan took place at the Ministry of Transport in October and November, where presentations were given by the Ministry followed by question and answer sessions from the audience.  Each consultatioin was on a particular mode of transport, namely Roads, Railways, Ports, Aviation and Intermodal Transport.  A wide range of interests were represented at the consultations, including industry representatives, trade bodies, councillors, special interest groups, company representatives, journalists, and members of the public.

A number of written submissions were received, approximately 80 in total.  These were analysed and, and if the points made were relevant these were taken into account in the final plan.

Cross-Sectoral Issues

Although all of the analysis and modelling for the Master Plan has been undertaken on a modal basis, much of the analysis in this Report has been described on a modal basis.  In this section we describe the main issues at a cross-sectoral level.

EES Operational Performance

The NTM produces a large volume of data which demonstrate how the transport network is performing.  In this section, the performance of the EES is compared with the Reference Scenario, which represents the future situation that would exist without the Master Plan projects, that is with financially committed projects only.  This is the most valid comparison because it shows on a like for like basis what difference the Master Plan is forecast to make.

Comparisons with the Base situation (2011) are also shown but these contain changes that would occur in transport demand between the different years.

Figure 10.1 shows the forecast changes in passenger transport in 2020 and 2030.

Figure 10.1 Evolution of Passenger Transport in 2020 and 2030 EES vs. Ref Case

Source: AECOM, National Transport Model

The results show that, with the investments in improved maintenance, and in line speeds and service improvements, substantial increases in rail passenger traffic could be achieved.  The forecast increase is a 42% increase in rail passengers, and 73% increase in passenger kms by 2020, rising to 88% and 110% by 2030 when the full strategy should be in place.

By contrast, the forecast increases in road traffic are modest, with a small decrease in car passenger traffic and a modest rise in passenger kms.  The rail investments lead to a decrease in bus passenger kms of 11% and 17% in 2020 and 2030 respectively.

One feature of the forecast impacts is the increase in average journey lengths – passenger kms grow to a larger extent than passenger numbers, and this is to be expected given the higher speeds, shorter journey times, and greater connectivity, that the Master Plan interventions will provide.

Figure 10.2 provides similar information for freight traffic.

Figure 10.2 Evolution of Freight Transport in 2020 and 2030 EES vs. Ref Case

Source: AECOM, National Transport Model and CBA Tool

The same pattern for freight traffic is evident, although the impact is less, for two reasons.  First, the relative improvement in freight train speeds is less than for passenger services, and secondly, the transit time is only one element of the cost of moving freight.

Freight tonne kms grow at roughly twice the rate of tonnes lifted.Here the largest increases in tonnes carried and tonne kms are again in rail freight, where tonne kms are forecast to increase by 6% in both 2020 and 2030.  Waterborne freight is also forecast to increase in both 2020 and 2030, as a result of investment in increasing the navigability of the Danube, and port improvements.

Figure 10.3 shows the change in mode choice.

Figure 10.3 Changes in the Overall Mode Choice

Source: AECOM, National Transport Model and CBA Tool

Here, the performance of rail should be assessed against a historical backdrop of decreasing mode share. The forecasts from the National Model show that rail mode share can be increased with investment, improved maintenance and better services, even in an expanding travel market and with increasing car ownership, and with significant improvements to the road network.

The difference between the Base Year and 2020 and 2030 EES scenario are shown in Figure 10.4, for passenger and freight transport.

Figure 10.4  Change in Passenger and Freight Demand, 2011 – 2020, EES Strategy

These results show, on the pasenger transport side, increases in the number of passengers and passenger kms for all modes of transport.  Comparing these results with the Reference Case comparisons, we can see that the increases in road passengers is not due to the EES strategy itself (which leads to a reduction in road passengers), but is due to underlying factors such as increasing car ownership and the completion of motorways in the Reference Case.

What is impressive is the turnaround in rail pasenger transport.  After more than 20 years of continuous decline, the investment in improved maintenance, rehabilitation and additional services is forecast to reverse this decline and produce an increase of 27% in rail passenger kms.

The situation is similar in 2030, with increases in all modes of transport for both pasenger and freight.  The reason for the increase in road passenger (and freight) transport is the same as that in 2020: the increases are due to an underlying trend, not the EES projects.

Again, the forecast increase in both rail passenger and freight is noteworthy, demonstrating once more that the historical trends in rail transport can be reversed with suitable investment.

Figure 10.5  Change in Passenger and Freight Demand, 2011 – 2030, EES Strategy

Allocation of resources per mode: € per pass km, € per tonne km

One measure of the contribution of each mode to the national transport picture is passeneger and tonne kms.  We have analysed the investments in the road, rail and waterways sector in relationship to the contribution each make in terms of these measures.  The results are presented below:

Table 10.21  EES Scenario: Spending per Passenger km and tonne km

|  | Expenditure in the Master Plan (m. euro) | Passenger kms/day | Tonne kms/day | €/passenger km day | €/tonne km/day |
| --- | --- | --- | --- | --- | --- |
| Roads | 18,707 | 156,721,866 | 194,257,197 | 119 | 96 |
| Rail + Intermodal | 16,139 | 19,096,378 | 43,614,918 | 845 | 370 |
| Waterways | 1,056 |  | 41,717,021 |  | 25 |

Source: Funding Analysis and national Model

The table must be interpreted with care.  The two measures are not an indication of what each mode deserves to receive.  The projects in each mode are justified in a logical, structured manner, from problem identification through to intervention and appraisal, and each investment stands on its own merit.  The road and rail networks are starting from different points with regard to their development.  Water transport is largely provided by the River Danube, which is resource provided by nature, and is not entirely man-made like roads or railways.

Nevertheless, the data do illustrate a conscious intention in the Master Plan to address the decline in the railway network and attempt to re-vitalise the railway so that it again plays a significant role in national transport, and to safeguard its future.

Heavy Goods Vehicle Road User Charge

The introduction of a HGV Road User Charge (RUC) will result in a change in revenue to CNADNR.  This note outlines the estimated cash flows (costs and revenues) to CNADNR after 2020 if HGV RUC is introduced.

The Annual costs expected to be incurred by CNADNR have been taken from the cost recovery work done by for the World Bank in 2013.  It has been assumed that when a HGV RUC is introduced the existing vignette scheme will stop, therefore vignette revenues to CNADNR will also cease.

A RUC rate of 0.66 Lei (€0.15) per HGV km has been used; this has been calculated to reimburse the average annual CNADNR maintenance deficit between 2014-2020 and the environmental impact of HGV traffic in 2020.  It also includes a 20% uplift to cover the administration and operating costs of the RUC scheme.  It has been assumed that the 20% administration cost will not be available to CNADNR for activities other than administration of the RUC.

For comparison, the rates in other European countries in June 2014 were as follows:

| Country | Rate/km |
| --- | --- |
| Austria | 0.16 - 0.44 |
| Poland | 0.20 - 0.40 |
| Germany | 0.14 – 0.29 |

Sources: http://roadpricing.gw-world.com/en/RoadPricingAustria.aspx; http://www.gddkia.gov.pl/en/1126/motorway-tolls

http://www.ages.de/en/hgv-toll-germany-tariffs.html

The rates in these countries varies by size and weight of vehicle, and also by emission category, to encourage cleaner vehicles.  If an HGV charging scheme were introduced in Romania a similar tariff should operate.

The CNADNR share of RUC revenue has been calculated from the number of HGV km on the CNADNR network. It is assumed that RUC generated on non- CNADNR network will be allocated elsewhere.

The calculation of RUC revenues and fuel duty revenues take into account that once RUC is implemented HGV vehicle km will reduce over the ‘without RUC’ case.  In 2020 total HGV vehicle km on the CNADNR network are predicted to reduce by ~4% relative to a situation without RUC.

The table below shows the annual costs and revenues to CNADNR between 2020-2030 in m euro.

Table 10.18  Potential Revenues from an HGV Charging Scheme

AECOM Analysis using World Bank Study Cost Recovery Study and the National Model

In our view, the most realistic assumption regarding the potential surplus to CNADNR is the lower figure of €1.9bn over the period 2020 – 2030.  This assumes that the revenue from fuel duty will not be available to CNADNR or MT to spend on transport projects.

However, at this stage HGV RUC is not a recommendation of the Master Plan.  We present the results here to show what could be achieved by this initiative, but we recognise that it is a controversial policy which would be unpopular with industry as it increases their costs.  France has recently abandoned its scheme (the ecotaxe) because of opposition from the haulage industry, despite all-party support.

One particular issue concerns the use of the surplus revenue.  RUC schemes are never popular, but do enjoy more support if the surplus revenue is allocated to transport investment (and especially road transport investment).

However, HGV RUC is an EU policy.  Therefore, we recommend that a Technical Assistance project is launched to investigate the feasibility of such a scheme in Romania, including which roads should be included, the precise charge, the techno;logy to be used, how suplus the revenue should be spent, experience in other countries, and consultation with industry.

Accessibility

A additional analysis was done on the relative accessibility of the main urban areas, considering the number of employees.

The formula for the effectity density is given below:

U = ji≠jAjdijα+Ajdi

where:

U =	Effective Density in a specific zone i

Aj =	Measure of economic activity in other zones – we have used number of employees for the analysis

dij =	Generalised cost of travel between zone i and other zones (composite GC matrix)

Ai =	Measure of economic activity in the specific zone i – we have used number of employees

di  =	typical generalised cost of travel for an internal trip (assumed 30 mins for all zones within Romania)

α   =	1.0

The effective density numbers have units of employees per minute.  The absolute effective density number for a zone does not have an own meaning; what is important is the % difference in values, either between different regions in the same year or between the same regions in different scenarios or years.

Effective density for a zone is calculated by looking at each of the other zones in the model and calculating the number of employees in the zones divided by the generalised cost (in  minutes) of getting from the original zone to the other zones.  This gives a value for each of the other zones.  The values for all other zones are added together and this is the effective density for the original zone.  This process is then repeated for each zone in the model. we then take weighted average across zones within each county and across all zones in Romania.  The effective density calculations are done with a series of matrix calculations within the model as it requires cost of travel between all origin-destination pairs.

The effective density of a zone is therefore bigger for a zone where you can travel to a large number of employees in other zones in a short time; and low if you can only travel to a smaller numbers of employees in longer times.

Because of the way it is calculated the effective density has units of employees per minute; however, it is not a measure of the number of employees that can be accessed in 1 minute, rather it is an indicator of how easy it is to access other employees from a zone relative to other zones.  This is why % changes in effective density are used to assess impacts rather than at absolute values.

The Base Year, and 2030 Accessibility plots, for domestic travel are shown on Figures 10.6-10.7 below.

Figure 10.6 Base Year Accessibility, Domestic Travel.

Figure 10.7  2030 Accessibility, Domestic Travel.

Source: AECOM EES Startegy and National Model

A comparison between the two plots shows that accessibility to the peripheral parts of Romania, such as the north-eastern areas of the country in the Buzau – Iasi corridor has increased, and also to the north-west in the Cluj/Targu Mures area, and to the west around Timisoara.

Figures 10.8 and 10.9 show the accessibility of Romania to neighbouring countries, for the Base Year and the 2030 EES scenario.

Figure 10.8  Base Year Accessibility, International Travel.

Figure 10.9  2030 Accessibility, International Travel.

Source: AECOM EES Startegy and National Model

The contrast between the base year and 2030 is marked.  Most of the Western and Central areas of Romania will experience a significant increase in accessibility to external employment and markets, demonstrating the success that the Master Plan will have in helping Romania to improve its competitiveness in the wider, global market place.

Appendices

Appendices

Appendix A. GTMP Financial Plan

Appendix B. TEN-T Maps for Romania

Figure B.1 Core and comprehensive TEN-T road, ports, rail terminals and airports networks

Source: http://ec.europa.eu/transport/themes/infrastructure/revision-t_en.htm

Figure B.2 Core TEN-T passengers rail lines, airports and comprehensive railways and airports network

Source: http://ec.europa.eu/transport/themes/infrastructure/revision-t_en.htm

Appendix D. Potential Electrification Schemes

Summary of Current and Proposed Daily Frequencies (one-way)

| Route Section | Current | Future | Route Section | Current | Future |
| --- | --- | --- | --- | --- | --- |
| Galati – Barlad – Crasna – Iasi - Husi | 10 | 10 | Brasov – Sibiu – Alba Iulia | 14 | 17 |
| Veresti – Botosani – Dangeni - Iasi | 4-9 | 4-9 | Blaj – Tarnaveni | 11 | 11 |
| Darmanesti – Dornesti | 10 | 10 | Tarnaveni – Praid | 4 | 4 |
| Dornesti – Nisipitu | 4 | 4 | Razboleni – Targu Mares – Deda | 13 | 13 |
| Bicaz – Bacau | 9 | 9 | Razboleni – Sarmasu – Sieu Magherus | 7 | 7 |
| Roman – Buhaiesti | 6 | 6 | Deva – Arad via Santana | 6 | 6 |
| Ploiesti – Urziceni – Giurgeni | 5 | 5 | Santana – Oradea | 12 | 19 |
| Bucharest – Urziceni – Faurei | 10 | 10 | Oradea – Cluj Napoca | 23 | 27 |
| Faurei – Tecuci | 2 | 2 | Oradea – Satu Mare | 10 | 10 |
| Ploiesti – Slanic | 3 | 3 | Satu Mare – Baia Mare | 9 | 14 |
| Ploiesti – Maneciu | 2 | 2 | Baia Mare – Dej | 11 | 19 |
| Bucharest – Pitesti | 16 | 22 | Jibou – Saculeni | 6 | 6 |
| Pitesti – Curtea de Arges | 4 | 4 | Satu Mare – Bixad | 6 | 6 |
| Pitesti – Argesel | 5 | 5 | Jibou – Carei | 5 | 5 |
| Pitesti – Rosiori de Vede | 10 | 10 | Timisoara – Resita | 11 | 11 |
| Pitesti – Craiova | 10 | 10 | Timsisoara – Jimbola | 3 | 3 |
| Rosiori de Vede – Zimnices | 9 | 9 | Timisoara – Sannicolau Mare | 5 | 5 |
| Rosiori de Vede – Turnu Magurele | 4 | 4 | Salva – Valea Viseului | 6 | 6 |
| Corabia – Caracal | 6 | 6 | Medgidic – Tulcea | 4 | 4 |
| Caracal – Sibiu | 10 | 13 | Medgidic – Negru Voda | 2 | 2 |
| Craiova – Calafat | 4 | 4 | Eforie – Mangalia | 7 | 7 |
| Bucharest – Oltenita | 2 | 2 | Videle – Giurgiu | 7 | 7 |
| Buzau – Neholasu | 9 | 9 | Caransebes – Subcetate | 5 | 5 |
| Sibiu – Copsa Mica | 13 | 13 | Lugoj – Illa | 9 | 9 |

Appendix E. List of Reference Case Projects

| Project Title | Undiscounted Cost (mill EURO, 2012 prices) |
| --- | --- |
| Modernization of movement areas and beakoning system, control tower and ILS navigation system in Suceava Airport | 31.897 |
| Rehabilitation of the apron and parkings in Constanta (M. Kogalniceanu) Airport | 31.151 |
| Stationary platform at the entrance of the Danube - Black Sea Channel | 14.111 |
| Extension of the breakwater in the Constanta Port | 133.287 |
| Extension to the south of the gauge berth in the Port of Constanta | 4.664 |
| Rehabilitation and modernization of the infrastructure in Oltenita Port | 4.596 |
| Infrastructure works in the Port of Braila - berths 23 and 25 | 7.812 |
| Building an administrative complex in Port of Giurgiu | 2.596 |
| Newada Duo Project | 0.238 |
| CO-WANDA: Convention for waste management for inland navigation on the Danube | 6.3 |
| Master Plan for the Constanta Port | 2.033 |
| DAHAR - Strategic Development Plan for the Danube interior Ports | 0.145 |
| GIFT- Green Intermodal Freight Transport Corridors in South East Europe | 0.242 |
| Modernizing port infrastructure by providing increasing depths of channels and basins and safety of navigation in Constanta Port | 37.535 |
| Developping of the rail infrastructure system in the Constanta Port (the fluvial-maritime sector) | 16.858 |
| Rehabilitation of the bridges, culverts and railways tunnels - Sucursala Craiova | 18.239 |
| Rehabilitation of the bridges, culverts and railways tunnels - Sucursala Cluj | 26.579 |
| Rehabilitation of the bridges, culverts and railways tunnels - Sucursala Brasov | 11.155 |
| Rehabilitation of the bridges, culverts and railways tunnels - Sucursala Bucuresti | 17.657 |
| Rehabilitation of the bridges, culverts and railways tunnels - Sucursala Iasi | 9.566 |
| Rehabilitation of the bridges, culverts and railways tunnels - Sucursala Timisoara | 8.91 |
| Rehabilitation of the bridges, culverts and railways tunnels - Sucursala Constanta | 2.371 |
| Rehabilitation works for the Danube bridges at km 152+149 and at km 165+817, railway line Bucharest - Constanta - Regional Branch Constanta | 40.592 |
| Modernisation of the railway level crossings - 112 crossings - Stage I | 20.077 |
| Current repairs for the public railway infrastructure for 2013-2020 | 558 |
| RK for the 2013-2016 period | 73.615 |
| Rehabilitation of the Bucharest-Constanta line | 904.389 |
| Modernisation of the Coșlariu - Sighișoara line at maximum speed of 160 km/h for passenger trains | 944.826 |
| Modernisation of the Simeria - Coșlariu line at maximum speed of 160 km/h for passenger trains | 663.06 |
| Modernisation of the Border-Curtici-Arad-Simeria line: Section Border-Arad-km 614 | 282.713 |
| Electrification of the Doaga-Tecuci-Barbosi line | 57.227 |
| Developping the strategic noise maps and action plans for the major railways having more than 30,000 train movements per year - Stage I | 61.826 |
| Modernisation of the railway stations in Romania: Slatina, Râmnicu Vâlcea, Reşiţa Sud | 18.525 |
| Modernisation of the railway stations in Romania: Bistrita, Zalau | 28.4 |
| Modernisation of the railway stations in Romania: Giurgiu Oraş, Slobozia Veche, Călăraşi Sud | 18.102 |
| Modernisation of the railway stations in Romania: Sfântu Gheorghe, Târgu Mureş | 22.322 |
| Modernisation of the railway stations in Romania: Pitesti | 15.584 |
| Modernisation of the railway stations in Romania: Piatra Neamţ, Botoşani, Vaslui, Brăila | 29.398 |
| Modernisation of the Focsani station | 2.517 |
| Detection system for overheated axles and closed brakes (21 locations) | 12.694 |
| Operational pilot project for a ECTS/ERTMS Level 2 aplication on the line section Buciumeni Junction - Brazi | 45.125 |
| Upgrading the electromechanical interlocking systems for 11 stations | 35.985 |
| Upgrading the electromechanical interlocking systems for 16 stations | 68 |
| Modernization of Videle interlocking station | 9.581 |
| Bridge on DN2E km 57+400, at Clit | 0.298 |
| Bridge over Arges river, DN61 km 74+015 at Ionesti | 7.035 |
| Bridge over Sai at DN54 km 67+774 and new road alligment for DN54, km 65 +950 - km 70 +600 at Turnu Magurele | 5.753 |
| Link road DN66A km 47+600 - km 66+204, Campu lui Neag-Cerna | 54.221 |
| Rail overpass on the Arad Bypass (DN7 km 540 +248) | 10.575 |
| Rail overpass on the Arad Bypass (Brad-Curtici rail section and DJ709B) | 10.663 |
| Modernization DN 72 Gaiesti - Ploiesti km 0 +000 - km 76 +180 | 56.451 |
| Modernization of Bucharest Ring Road between A1 - DN7 and DN2 - A2 (widening to 4 lanes) | 121.873 |
| Bridge over the Danube-Black Sea Channel at km 0+540 and the access infrastructure in the Constanta Port; Phase 1 - construction of the bridge | 30.887 |
| Bridge over the Danube-Black Sea Channel at km 0+540 and the access infrastructure in the Constanta Port; Phase 2 - construction of the road connections | 5.961 |
| Alesd South and Alesd North Bypass | 76.65 |
| Alexandria Bypass | 41.619 |
| Bacau Motorway Bypass | 231.539 |
| Brasov Bypass | 85 |
| Caracal Bypass | 12.5 |
| Carei Bypass | 19.543 |
| Cluj-Napoca East Bypass | 132.228 |
| Craiova South Bypass | 31.833 |
| Deva - Orastie Motorway | 296.852 |
| Iasi South Bypass: Phase 4 | 24.316 |
| Lugoj - Deva Motorway | 1136.3 |
| Mihailesti Bypass | 8.708 |
| Nadlac-Arad Motorway | 307.673 |
| Oradea Bypass | 52.934 |
| Orastie - Sibiu Motorway | 600.18 |
| Sacuieni Bypass | 12.083 |
| Satu Mare Bypass | 139.646 |
| Sebes - Turda Motorway | 811.122 |
| Stei Bypass | 113.592 |
| Suceava Bypass | 68.689 |
| Targu Jiu Bypass | 88.284 |
| Targu Mures Bypass | 72.566 |
| Tecuci Bypass | 18.883 |
| Access Road to Agigea Lock and sea-port Agigea, CDMN, left bank, between km 61 +800 and 63 +500, L = 1.700 m | 1.668 |
| Consolidation works DN29D, km 18 +500 - km 20+816 | 7.669 |
| Ease of traffic on DN1 km. 8+100 - km. 17+100 - Phase 7, the bypass ring section DN7-DN1A | 44.66 |
| Rehabilitation DN66 Filiasi - Petrosani, km 0+000 - km 131+000 | 127.412 |
| Rehabilitation DN76 Deva-Oradea, km 0+000 - km 184+390 | 223.488 |
| Modernisation DN2N km 52+860 - km 60+000 Jitia - Biscoa and new bridge over Ramnicu Sarat river la km 53+300 | 5.218 |
| Modernisation DN5, section Bucuresti - Adunatii Copaceni | 22.417 |
| Rehabilitation DN2D Focsani - Ojdula km 0+000 - km 118+893 | 179.071 |
| Building the A3 motorway section in Bucharest Municipality | 89 |
| Rehabilitation DN24 (county limit Galati/Vaslui - Crasna)and DN24B | 107.424 |
| Rehabilitation DN6 Alexandria-Craiova | 171.338 |
| Rehabilitation DN66 Filiasi - Petrosani, km 0+000 - km 131+000 | 127.412 |
| Rehabilitation of DN56 Craiova - Calafat km 0 +000 - km 87+047 | 111.72 |
| Rehabilitation DN1H Rastoci -Zalau, km 75 +446 - km 128 +823 | 32.951 |
| Traffic calming measures on 4 lanes national roads | 29.032 |
| Safety measures on DN1 in linear villages and black spots | 37.984 |
| IRIS Europe 3 | 1.077 |
| Stationary platform for barges at the confluence of the Black Sea and Danube waterways Gate Alba Midia Navodari | 2.702 |
| The modernization of the water management system on the navigable channels by installing automatic monitoring stations | 3.562 |
| Upgrading the locks, equipments and installations for CDMN and CPAMN | 175.639 |
| System for gathering and processing of the ships waste and for pollution intervention on the Danube sector administrated by CN APDF SA Giurgiu | 8.634 |
| Monitoring the environmental impact of the works to improve navigation conditions on the Danube between Calarasi and Braila, km 375-km 175-Phase II | 1.4 |
| Protection for banks at Sulina Channel - Stage I | 76 |
| Improving the navigation conditions on the lower Danube (Calarasi-Braila) | 38.671 |
| Pier berth 31 in Port Docks Outdoor Galati |  |
| Creating a support system works on the Danube river in order to ensure minimum depth for navigation |  |
| Giurgiu bridge over the Danube at km 64+884 DN5 |  |
| Road Bridge over Olt in Slatina on DN6, km 48 +570 |  |
| Oituz road bridge over the Poiana Sarata on DN 11 km 90 +450 |  |
| Road Bridge over Jiu in Arginesti on DN6, km 268+371 |  |
| New bridge at Cosmesti , over Siret river on DN24 km 7+620 |  |

Locations of the Reference Case Schemes

See EC Letters dated  27/09/2013 and 11/12/2013

For more details see National Guide for Transport Project Evaluation, Volume 2, Appendix A: Guidance for Economic and Financial Cost Benefit Analysis and Risk Analysis.  AECOM.

Ibid Chapter 7 for details of the project and programme appraisal criteria.

This is an additional year which AECOM added to ensure a better forecast of economic benefits.

EUROPEAN COMMISSION, Directorate-General- Regional and Urban Policy, DRAFT -“Guidance on Ex ante Conditionality for the European Structural and Investment Funds”, PART II, “Criteria for fulfilment”

http://ec.europa.eu/transport/themes/infrastructure/revision-t_en.htm

http://ec.europa.eu/transport/themes/strategies/2011_white_paper_en.htm

http://www.drp.gov.ro/download.php?6b3a2e12faf92184a320aeeaa3f853cf

http://www.mt.ro/strategie/plan_strategic/planul%20strategic%20integrat%20revizuit%202009.pdf

http://www.fonduri-ue.ro/res/filepicker_users/cd25a597fd-62/2014-2020/acord-parteneriat/Acord_de_parteneriat_01.10.2013.pdf

http://eur-lex.europa.eu/LexUriServ/LexUriServ.do?uri=COM:2006:0006:FIN:EN:PDF

Programul Operațional Sectorial deTransport 2007 – 2013 (revizia 2 - aprobat la 26.07.2013)  Ministerul transporturilor și infrastructurii. http://www.ampost.ro/fisiere/pagini_fisiere/RO-POST_revizia_2.pdf

http://eur-lex.europa.eu/LexUriServ/LexUriServ.do?uri=COM:2010:2020:FIN:RO:PDF

http://ec.europa.eu/regional_policy/sources/docgener/panorama/pdf/mag37/mag37_ro.pdf

http://eur-lex.europa.eu/LexUriServ/LexUriServ.do?uri=COM:2013:0216:FIN:RO:PDF

http://europa.eu/legislation_summaries/maritime_affairs_and_fisheries/fisheries_resources_and_environment/l28164_ro.htm

http://www.mt.ro/strategie/strategii%20sectoriale_acte%20normative/strategie%20dezvoltare%20durabila%20noua%20ultima%20forma.pdf

http://www.mt.ro/nou/_img/documente/strategie_de_transport_intermodal_text.pdf

http://www.mmediu.ro/beta/wp-content/uploads/2012/10/2012-10-05-Strategia_NR-SC.pdf

http://biodiversitate.mmediu.ro/implementation/legislaie/politici/strategia-nationala-si-planul-de-actiune-pentru-conservarea-biodiversitatii/anexa-strategia-nationala-si-planul-de-actiune-pentru-conservarea/snpacb.pdf/download

http://www.mt.ro/transparenta/2012/ianuarie/1_17%20Anexa1.pdf

http://ec.europa.eu/clima/policies/adaptation/what/docs/swd_2013_137_en.pdf

www.icpdr.org

RGTMP National Assessment Guidelines Volume 1 Appraisal and Prioritisation of Projects for Inclusion in the Masterplan

Source: European Commission, EU Transport in Figures Statistical PocketBook, 2013

Maximum speed data is not available for some links, so these are shown as blank

Polish examples were Gdansk to Szczecin, Wolow to Glogow and Gorzow to Pila. The Hungarian examples were Debrechen to Puspokladany, Hodmezovasarhely to Bekesasaba and Pecs to Szekszard

IRG – Rail Marketing Monitoring Report February 2013

To be a phased development in line with demand and considered with the adjacent scheme to modernise existing bulk terminals at Galati

Tonnes change displayed here is for Romanian ports. Ports from other countries will benefit from a River Danube that has greater navigability however these ports are outside the scope of this study.

Considered to be of importance

To be a phased development in line with demand and considered with adjacent scheme at Galati

Romanian CAA, October 2013

Source: Intermodal Strategy Romania 2020

Source: DIOMIS – Evolution of intermodal rail/road traffic in Central and Eastern European Countries by 2020, UIC, March 2010

AECOM 24 count, March 2013

Source – Constantza port handbook

Extract from EC DG Regio letter E2/RI/ds (2013) 3331276 on 27/09/2014

Romanian Government letter no. 57338 submitted to European Commission on 09/12/2013

MT DG SMAE letter no. 8179 on 11.03.2014

AECOM was informed by MT that the National co-financing for the EU Funded projects (Cohesion Funds and European Regional Development Fund), part of the 2014-2020 Operational Programme – Transport, is to be considered at 25% of the total eligible costs; a national co-financing rate of 15% is to be considered for the CEF programme

MT letter no. 26515 submitted to Ministry of Finance on 11/06/2013

http://cnp.ro/user/repository/prognoza_macroeconomica_2014--2017.pdf

Romanian Government letter no. 57338 submitted to European Commission on 09/12/2013