NOTIFICATION TO AN AFFECTED PARTY OF A PROPOSED ACTIVITY

UNDER ARTICLE 3 OF THE CONVENTION FOR EIA REPORT IN THE TRANSBOUNDARY CONTEX

| 1. INFORMATION ON THE PROPOSED ACTIVITY: ”POD PESTE TISA ÎN ZONA TEPLIȚA DIN SIGHETU MARMAȚIEI” (Bridge over Tisa in Teplita area on Sighetu Marmatiei) |  |
| --- | --- |
| (i) Information on the nature of the proposed activity |  |
| Type of activity proposed: | The project aims to achieve a road connection between Maramures County and Ukraine by building a bridge over Tisa river and a connection road. The connection road from Ukraine will be carried out in base of other project. The analysed project involves only construction of 120 m from connection road, construction of one pier and one abutment on the Ukrainian territory, according to general layout from annex 1. The bridge will be executed on the Romanian border and will be launched over Tisa River, that so the impact on the Ukrainian environment will not be significant. |
| Is the proposed activity listed in Appendix I to the Convention? | The proposed activity it is not listed in Appendix I to the Convention. |
| Scope of proposed activity (e.g. main activity and any/all peripheral activities requiring assessment) | The project will create a modern communication way with implications for regional development of the area, traffic flow, increase traffic safety, decrease travel times, decrease pollution at all levels in areas transited and shortening the road connections with Russia, Baltic’s Countries, Poland, Hungary and Slovakia. For the achievement of the project will be carried out the following construction works: - bridge works: - bridge infrastructure will consist in two abutments and two piers; - the superstructure of each bridge will be made of a metal box with variable height, provided at the top with prestressed reinforced concrete flooring; - the path on each bridge will provide a carriageway of 8.00 m and 2.50 m wide sidewalk; - the bridge will be equipped with rainwater collection and disposal system; - road works: - achievement of a connection road of 1.200 ml, which include a bridge over Tisa river; - road platform: 19.533 m; - carriageway width: 4 x 3,75 m = 15,00; - roadway ditch - 2 x 0.90 m = 1.80 m; - sense separator – concrete cross-beam H2 type - 0.67 m; - H2 type concrete cross-beam working width – 2 x 1.03 m; - left – right sidewalk – 2 x 2.50 m; - safety barriers + left – right metallic parapet; - the transversal profile on the roadway will be roof type with e 2.50 % slope; - the transversal slope on the sidewalk will be 2.00%. - water drainage works: - ditches and culverts for collecting the rainwater; - oil separators for purifying rainwater; - culverts to maintain natural drainage system existing before construction of the connection road; - discharge bridges at km 0+530, 0+620 and 0+720; - customs checkpoint provided with parking area for cars and freight scales, parking for cars, office building. Access to the country and out of country will be made in five lanes for each movement direction. - works for traffic safety: traffic signs and horizontal marking. |
| Scale of proposed activity: (e.g. size, production capacity, etc.) | The connection road will have 1.200 m length, including the bridge over Tisa River (261.2 m). For crossing the Tisa river has been adopted the following technical solution: parallel bridges with joint deck concrete steel, continuous beam with variable height (a bridge for each movement direction). Each bridge related to a movement sense is ensured a carriageway of 8.00 m for two lanes and a sidewalk for pedestrians 2.05 m separated from the roadway. The access to the country and exit from the country within the custom checkpoint will be made on five lanes for each movement direction. The area permanent occupied by the construction works (including the bridge projection) it is 100.600 m2. The project does not involve the achievement of the production processes, but construction of a bridge over Tisa River and a connection road. During the operation period, the project will be destined to traffic and do not involve production processes. |
| Description of proposed activity: (e.g. technology used): | Will be adopted modern construction techniques and will be avoided the pollutant emissions in environment during achievement of the construction works. The works necessary for bridge infrastructure will be carried out using metal enclosures, in order to eliminate the risk of break in of the construction materials into the Tisa riverbed. Will be made the excavations for foundations, bearing block and reinforced concrete foundation. The bridge planks will be launched over Tisa River. The construction works will not be carried out in Tisa minor riverbed. Raw materials and waste will be stored at distance of Tisa riverbed, in special arranged areas, that so will not be any risk of their impairment in river course. During construction works period, will not be modified the water flow regime, water level, physic-chemical characteristics of the Tisa water. Also for the achievement of the connection road will be utilised modern technology and equipments, according to national and international standards. The connection road from Ukraine will be achieved in base of other project. |
| Description of purpose of proposed activity: | The project aims to achieve a road connection between Maramures County and Ukraine, near the most important locality in north of the county – Sighetu Marmaţiei, through its implementation contributing to the integrated development in the northwest border region of Romania (Maramures), south-western Ukraine region (Transcarpatia region). |
| Rationale for proposed activity: (e.g. socio-economic, physical geographic basis) | The project aims to create a modern communication way with implications for regional development of the area, flow of the traffic, increase traffic safety, decrease travel times, decrease pollution at all levels in areas currently and shortening transit road links with Russia, Baltics Countries, Poland, Hungary and Slovakia. To create a cross-border economy competitive at regional level, it is necessary to have access to, from and between various parts of the region. Due to the geographical location of the border area, its accessibility depends on the number and quality of roads. In the project area, the roads do not comply with European standards. For the Romanian side, in Maramures, secondary roads allow access to Ukraine by European road E81 and E85. In the study area, the transport infrastructure is underdeveloped and poorly maintained, limiting traffic speeds and increased travel times, so that it can ensure the availability and connectivity to international standards, leading to an isolation effect. The available technical conditions and the capacity available for custom clearance are inadequate to handle the actual and forecasted traffic volume. Queues and waiting periods are significant for tourism development in the project area, as well as cooperation between its inhabitants. |
| Additional information/comments: | Additional information are provided in the Documentation on environmental impact assessment in transboundary context and in technical report (attached). |
| (ii) Information on the spatial and temporal boundaries of the proposed activity |  |
| Location: | The project will be achieved in the administrative territory of the Sighetu Marmaţiei Municipality in Romania and on the administrative territory of the Bila Tserksva Village in Ukraine, according to general layout of the project from Annex 1. |
| Description of the location: (e.g. physical-geographic, socio-economic characteristics) | The analysed area is located on the terrace of Tisa river, in the Maramures Depression. Depression is bordered in the east and southeast of Maramures Mountains and Rodnei Mountains, in the south and west of the Gutai - Tibleş mountain chain and in north of the Tisa River that represent the state border of the country. On the Ukrainian territory the project will be carried out outside natural protected area. On the Romanian border, the project will be achieved within the overlapped territory of the site of community importance ROSCI0251 Tisa Superioară and special bird protection area ROSPA0143 Tisa Superioară. The lands where the project will be carried out are occupied by the low flooded meadow flora, Tisa river minor riverbed and arable lands cultivated annually (without a stable biota). Have not been identified protected species or habitats. |
| Rationale for location of proposed activity: (e.g. socio-economic, physical-geographic basis) | The project aims to create a modern road connection between Maramures County and Ukraine, through its implementation contributing to the integrated development at the northwest border region of Romania (Maramures) and south-western Ukraine (Transcarpatia region). The available technical conditions and the capacity available for custom clearance are inadequate to handle the actual and forecasted traffic volume. For the location of the project have been analysed many alternatives (presented in the documentation on environmental impact assessment in tranboundary context - attached) and have been selected the most feasible. |
| Time-frame for proposed activity: (e.g.: start and duration of construction and operation) | Implementation period of the construction works is 24 months. Start of the construction works it is forecasted in 2016. Normal operation period of a bridge is about 100 years, and of a road is 15 years, in conditions of achievement of maintenance and repairs works according to standard in force. |
| Maps and other pictorial documents connected with the information on the proposed activity: | The general layout and layout of the project are presented in Annex 1 and 2. |
| Additional information/comments: | Additional information are provided in the Documentation on environmental impact assessment in transboundary context and in technical report (attached). |
| (iii) Information on expected environmental impacts and proposed mitigation measures |  |
| Scope of assessment: (e.g. consideration of: cumulative impacts, evaluation of alternatives, sustainable development issues, impact of peripheral activities, etc.) | In the project location and in its vicinity does not exist other project with which the construction and operation of the bridge over Tisa in Tepliţa area in Sighetu Marmaţiei could have cumulative impact. Have been analyzed the zero alternative and several options for achievement of the project: - multiple location alternatives for the bridge and the connection road; - multiple construction technologies for achievement of the bridge and road. The studied alternatives and there impacts are described in documentation on environmental impact assessment in transboundary context (attached). The proposed alternative it is the most feasible in technical and economical terms and with the lower environmental impact. Achievement of the project will contribute to the sustainable development of the area and will have no significant impact on the environment. The probability of the impact it is low, occurs only in the construction works period. Due the specific and volume of the construction works, the impact on the Ukrainian environment will not be significant. |
| Expected environmental impacts of proposed activity: (e.g. types, locations, magnitudes) | Achievement of the objectives of this project (construction of the bridge over Tisa in Tepliţa area and of the connection road) will have no significant impact on environment. Will not be emissions in water or soil, and the air emissions will occur only in the project location and about 50 m of its limits. The impact will be insignificant, temporary and reversible, that so the environment will return to its initial state upon the completion of the construction works, except the areas permanent occupied by the new infrastructure. The connection road from Ukraine will be carried out in base of other project. The bridge will be executed on the Romanian border and will be launched over Tisa River. Due the specific and the volume of the construction works executed on the Ukrainian border, the impact on the Ukrainian environment will not be significant. |
| Inputs: (e.g. raw material, power sources, etc.) | The quantities of raw materials and energy necessary for the achievement of the project have been forecasted on base of the works volume. The utilities necessary for the achievement of the construction works are present in the project location and in its vicinity, on the Romanian border. The water necessary for the project will be up taken from a proper drilling and bottled drinking water will be purchased. For electricity supply the site organization will be connected to the existing electrical network from the vicinity of the site. The raw materials will be achieved from gravel pits / quarries / authorised centres from the vicinity of the project site. It is strictly forbidden up taking the natural resources from the project location. The asphalt and concrete mixtures necessary for achievement of the construction works will not be prepared on the project location, but will be brought from authorized centres in order to reduce the level of air pollutants and noise in the project location. The fuels necessary for the raw materials transport will be achieved from fuels station from the vicinity of the project location. |
| Outputs: (e.g. amounts and types of: discharges in air, discharges into the water system, solid waste) | During construction of the bridge over Tisa River will not be emissions in water or soil, and the air emissions will occur only in the project location and about 50 m of its limits. To the air pollution can contribute: activities in the site organization (storage and handling of building materials, machinery fuel supply), the activities in the working fronts (stripping / re – covering of the working areas, excavation / filling works, embankments execution, concrete / asphalt pouring), traffic on the site of the work and traffic on access roads to the site. The sources associated with the construction works of the bridge over Tisa in Sighetu Marmaţiei are open source (diffuse). These sources will act only in working fronts, without affecting the entire surface of the analysed area. The concentration of the air pollutants decrease with increase of the distance by the working site. Impact on air is temporary and reversible and occurs only in project location, without affecting the air quality. Upon completion of the construction works, the air quality will return to its initial state, will not be registered residual impact on air. The construction works will not be carried out in Tisa minor riverbed and the works from the vicinity of minor riverbed will be executed using metal enclosures. Raw materials and waste will be stored at distance of Tisa riverbed, in special arranged areas, that so will not be any risk of their impairment in the river course. The project implementation does not involve technological waste waters, but only sewage waste waters generated within the site organization. The waste waters will be treated by treatment plant located within the site organization and will be discharged in natural emissary only after their will comply with the provisions of the NTPA 001/2002, that so will not affect in any form the quality of Tisa River waters. The waste will be stored in special arranged area within the site organization location, on the Romanian border. The waste will be transported / eliminated by an authorised company, according to legislation in force. The custom checkpoint infrastructure are not object of the analysed project, but the management of this outputs will be carried out according to legislation in force, that so will not be registered significant impact on the environment. |
| Transboundary impacts: (e.g. types, locations, magnitudes) | Despite the fact that the project will be implemented in the border area, will have no significant impact on the Ukraine environment. The connection road from Ukraine will be carried out in base of other project. The analysed project involves only construction of 120 m from connection road, construction of one pier and one abutment on the Ukrainian territory. The main construction works will be carried out on the Romanian territory: the bridge will be executed on the Romanian border and will be launched over Tisa river. Due the specific and the volume of the construction works executed on the Ukrainian territory, the impact on the Ukrainian environment will not be significant. |
| Proposed mitigation measures: (e.g. if known, mitigation measures to prevent, mitigate, minimize, compensate for environmental effects) | In the documentation on environmental impact assessment in transboundary context (attached), have been proposed mitigation measures for each environmental factor, in order to prevent, mitigate and minimize any potential impact on environment. Has not been the case for compensatory measures, because the project will not have any significant impact on the environmental factors. |
| Additional information/comments: | - Have been obtained the permit of the curator of the two natural protected areas within project will be carried out in the Romanian border. The conditions and measures stipulated in the permit are compulsory for the constructor and beneficiary of the project. |
| (iv) Proponent/developer: |  |
| Name, address, telephone and fax numbers: | Holder: Romanian National Company of Motorways and National Roads Address: 38 Dinicu Golescu Blvd., sector 1, Bucharest Telephone: 004 021.319.61.24 Fax: 004 0213120984 E-mail: office@andnet.ro; Web page address: www.cnadnr.ro |
| (v) EIA documentation |  |
| Is the EIA documentation (e.g. EIA report or EIS) included in the notification? | The environmental impact assessment documentation elaborated under the provision of annex 2 of the Espoo Convention is attached to this notification. In the documentation are described the scope of the activity, the main objectives, the impact on environmental factors, including transboundary impact, the proposed mitigation measures. |
| If no/partially, description of additional documentation to be forwarded and (approximate) date(s) when documentation will be available | The EIA report will be elaborated only at the Maramures Environmental Protection Agency request, under the provision of the EIA legislation. |
| Additional information/comments: | Additional information are provided in the Documentation on environmental impact assessment in transboundary context and in technical report (attached) |
| 2. POINTS OF CONTACT |  |
| (i)Point of contact for the possible affected Part or Parties: |  |
| Authority responsible for coordinating activities relating to the EIA (refer to decision I/3,appendix): Name, address, tel. and fax numbers | Ministry of Ecology and Natural Resources of Ukraine 35 Vasilya Lipkivs’kogo Street, Kyiv, Ukraine, 03035 Telephone: +38 (044) 206-31-74 Fax: +38 (044) 206-31-07 e-mail: gr_priem@menr.gov.ua |
| List of affected parties to which notification is being sent: | Ukraine |
| (ii) Points of contact for the Party of origin |  |
| Authority responsible for coordinating activities relating to the EIA (refer to Decision I/3, appendix): Name, address, tel. and fax numbers | Ministry of Environment, Waters and Forests 12 Libertății Blvd., Sector 5, Bucharest, Romania , 040129 Telephone: 004 021 408 9615 Fax: 004 021 316 0421 e-mail: cabinet.ministru@mmediu.ro |
| Decision making authority if different than authority responsible for coordination activities relating to the EIA: Name, address, tel. and fax numbers | Local Environment Protection Agency Maramures 1A Iza Street, Baia Mare, Maramureș county, Romania 430073 Telephone: 004 0262 276 304 Fax: 004 0262 275 222 e-mail: office@apmmm.anpm.ro |
| 3. INFORMATION ON THE EIA PROCESS IN THE COUNTRY WHERE THE PROPOSED ACTIVITY IS LOCATED |  |
| (i) Information on the EIA process that will be applied to the proposed activity |  |
| Time schedule: | Duration: approximatively 8 months. |
| Opportunities for the affected party/parties to be involved in the EIA process: | The affected party can participate in the decision-making procedure as follows: -following the notification and transmission of documents attached thereto the affected party may decide to participate in the EIA procedure and may submit comments and remarks to be taken into account in the EIA documentation; -after receipt of the EIA documentation, the affected party is invited to submit comments to the documentation. If necessary, consultations will be made later to the authorities of the affected party, in ACC. with art. 5 of the Espoo Convention. |
| Opportunities for the affected party/parties to review and comment on the notification and the EIA documentation: | Comments on notification are expected, if Ukraine decides to participate in the EIA procedure. Comments on the EIA documentation are expected during the EIA procedure. |
| Nature and timing of the possible decision: | The possible decision is the issuing of environmental agreement and the construction authorization for this project. |
| Process for approval of the proposed activity: | The proposed activity will be approved by construction authorization after the environmental agreement (final EIA decision) is issued by the environmental competent authorities. |
| Additional information/comments: |  |
| 4. INFORMATION ON THE PUBLIC PARTICIPATION PROCESS IN THE COUNTRY OF ORIGIN |  |
| Public participation procedures: | In accordance with the provisions of Romanian legislation, the public participates in decision making during EIA procedure, as follows: -has a minimum of 30 days for submitting comments/observations to the EIA documentation in the procedural stages; - within the public debate organized after the submission of the EIA report; the public has access to EIA documentation and may formulate comments/observations to it both before and during the public debate. |
| Expected start and duration of public consultation: | In accordance with Romanian legislation, the public has a minimum of 30 days for submitting comments/observations to the EIA documentation in the procedural stages |
| Additional information/comments: | Contact persons from Ministry of Environment, Waters and Forests - General Directorate for Impact Assessment and Pollution Control Mihaela MĂCELARU, focal point on Espoo Convention e-mail: mihaela.macelaru@mmediu.ro Anca – Maria APREUTESEI , senior adviser e-mail: anca.apreutesei@mmediu.ro tel.: 004 021 408 9588 fax: 004 021 316 0421 |
| 5. DEADLINE FOR RESPONSE |  |
| Date: | Four weeks after receiving the notification. |