Comments made within the framework of the Espoo procedure in relation to the environmental impact study (EIS) of the planned opening of the Rosia Montana mine, compiled by the Hungarian Ministry of Environment and Water on the basis of government and non-governmental organizations

In August 2002 Hungarian Ministry of Environment and Water notified the Ministry of Environment and Water Management of Romania that Hungary can be potentially affected by the implementation of the planned mining investment in Rosia Montana. Hungary, as a potentially affected party, asked for the application of the UN ECE Convention on Environmental Impact Assessment in a Transboundary Context (Espoo Convention) after the permitting procedure of this mining activity had been launched by Romanian authorities. The international environmental impact assessment procedure according to Espoo Convention was started in January 2005, when answering the notification according to the Convention sent by the Romanian party, Hungary confirmed its intention to participate in the procedure. Romanian Ministry of Environment and Water Management sent the environmental impact assessment documentation compiled by the investor to Hungary on 13th June 2006.

Hungarian participation in the environmental impact assessment procedure of the Rosia Montana goldmining project has been run in complete accordance to the Espoo Convention so far, Hungary appreciates the willingness of cooperation manifested by the Romanian party.

The basis of this document consisted of the opinion of experts invited and/or employed by Hungarian authorities and incorporates remarks originating for non-governmental organizations and concerned citizens in relation to the environmental impact assessment documentation of the Project.

I.	Summarized and general comments

The EIS does not discuss the impacts of the planned investment in an appropriate professional manner and fails to provide an objective picture of the planned investment’s environmental effects.

The quantity of measurement data supporting and confirming certain statements of the EIS is insufficient, deficient, inaccurate, or not considered to be representative.

The complete material typically uses the terms “very low probability” or “very unlikely” in relation to risks which is not in conformity with any scientific approach, particularly if the essential final conclusions are drawn in such a manner. Such assertions must be supported with modeling and probability calculations.

The documentation does not provide a full and satisfactory answer to the questions raised in the scoping list submitted to the investor (it lacks, inter alia, the modeling of pollutants emitted into water systems and bodies in the event of an accident; the description of environmental conditions for the past 5 years; in relation to the analysis of processing technology, reference data on the 2 European plants of similar size and operating with similar technology is missing; the technical design parameters of certain facilities are omitted; analysis of ecological risks caused by hazardous materials emitted into the outlet water streams; application of environmental control systems; ensuring the accessibility of measurement data; technical inspection methods applicable in the course of construction).

Because of the considerable deficiencies and inaccuracies of the EIS provided by the Romanian party, - in spite of its large volume - it does not provide an adequate basis for objective evaluation within the framework of the Espoo procedure.

The material essentially declares general observations and subjective determinations as fact, frequently without any explanations or data of proving force.

Due to the excessively generalizing nature, false basic conclusions and major deficiencies of the EIS, we propose that the competent Romanian authorities not approve the impact study within the framework of the permission procedure.

The Hungarian party is ready to take part in consultations with the Romanian party as to the problems and comments formulated in this document.

II.	Baseline recording

- Various topics are mixed in the baseline report relating to waters, sediments, chemical and biological analyses. The titles of the sections are not in harmony with the content (e.g. the biological and bacteriological section deals with the description of the drainage areas). Parts of the material prepared by various experts are not correlated.

- Data relating to the quality of waters, particularly subsurface waters, is deficient; it is difficult to link data with the given period and origin. Frequent reference is made to long data series, but these have been omitted from the report. It is not clear as to which periods are incorporated in the aggregate tables. In general terms, most of the presented data is outdated (e.g. biological data from 1998), it does not reflect current conditions.

- The information content of certain determinations is inadequate, e.g. subsurface waters are of a “good condition” – what is the basis for such evaluation?

- In some instances the analyzed periods are short; data series of a few weeks or months are not appropriate for substantiating professionally correct conclusions. The range of analyzed components is not broad enough, some important components are missing (e.g. various forms of cyanide, organic micro-pollutants, etc.).

- The description of sampling locations is deficient; the number of wells included in the analysis is only a fraction of what is needed for an objective assessment of conditions.

- The analysis does not contain modeling events in relation to changes in subsurface waters (flow directions, pollution spread, etc.), notwithstanding the fact that there are procedures which can provide even a three dimensional picture on expected changes.

- A load test, material balance has not been completed; the degree and spread of subsurface water pollution cannot be accurately determined.

- The meteorology related baseline report bases its assertions on a measurement program conducted in 2004, although the majority of data series contained therein is linked to the period extending up to 1997-1998.

- Please support with facts the assertion that the mean temperature in the area in the period from December to March is 4 C because the Hungarian average values, for example, in this period are in the range – 2 to 4 C.

- The expected temperature conditions are not sufficiently taken into account (e.g. the significance of monthly average temperatures in the course of the cyanide dissociation process, risk of freezing surface pipelines).

- The geological baseline recordings are deficient; no recordings were made on the location of important facilities, such as temporary hazardous waste sites, temporary ore storage facilities, existing mining facilities.

- The impact study does not provide enough information on old mines, existing facilities located at the site of the investment, thus the possible changes cannot be followed through. The material should be supplemented with plans relating to the rehabilitation of the old mines.

- With regard to the water balance of the area, it is not analyzed as to how much water is to be removed prior to the commencement of mining and what will happen to the presumably polluted water.

- It is unacceptable that an accurate geological recording was not made in relation to the Corna valley tailings management facility (TMF) and the survey was conducted 150 meters lower. This may be important with regard to the accurate knowledge of the faults in the valley. The special characteristics of these must be available to decide whether foil insulation is needed underneath the storage facility.

- The material does not indicate the type of hydro-geological model used and basic data is unavailable, therefore results derived from the model cannot be verified.

III.	Technological processes

- The baseline descriptions, geological, water balance related risks, environmental impacts, etc., noted in the previous chapters are repeated in the study. Why are the seismological results, for example, found among technological specifications?

- Several references are made to studies which were not mentioned previously, e.g. in relation to hydrogeology.

- According to the technological specifications, the TMF was put 250 meters higher and the second dam 400 meters higher. Previously 150 m was specified. Which is the real value? Has the planned final location of the dam been realistically recorded?

- The list of chemical substances, fuels and explosives used in the technological process should be supplemented. The operational material balance has been omitted, thus we are unaware of the precise quantity and type of poisonous materials used in the technological process.

- The material does not analyze the impact mechanism of the pollutants (cyanides, heavy metals) applied by the technology, the occurrence and management of possible hazardous situations caused by these.

- The cyanide balance is questionable; the quantity dissociated into air, in the tailings and emitted in comparison to the received quantity.

- The risk of the spread of toxic aerosols and the occurrence of “cyanide rain” is not adequately taken into consideration.

- Similarly to the other documents, the chapter discussing waste basically repeats and recapitulates determinations contained in other volumes of the impact study document. Risks inherent to waste, however, are judged negligible compared to other risks related to the operation of the mine.

- The BAT reference in chapter 6 is not clear and the attached table is unintelligible as well.

IV.	Possible impacts

- The first volume discussing possible impacts on environmental elements (4.1 Water, Volume 11) also commences with repeated items (meteorological, climatic conditions, condition of surface, subsurface waters, etc.) which are discussed in detail in previous sections, or the same results are presented in another form, or other data is also presented.

- In the chapter discussing accidents, overflows and a dam burst is considered negligible (extremely unlikely), but the emission of water with cyanide content and a low pH value into living water is not ruled out in the event of extreme weather conditions occurring every 100 years.

- The model mentioned in chapter 7 cannot be identified; only the determinations were recorded in a table. In this form it cannot be verified.

- The chapter on waters does not mention or take into regard the Water Framework Directive (WFD), the objectives of which must be implemented by Romania, as well. Only positive impacts are assumed with regard to the implementation of the investment. Similarly, in the course of devising the monitoring systems, the WFD aspects are disregarded, biological parameters are not produced and the frequency of measurements is unsatisfactory, including the range of analyzed parameters.

- The long term waste management plan for acidic rock drainage (ARD) is not adequately substantiated, cost-profit analyses have been omitted.

V.	Analysis of possible versions

- The possible versions are examined in volume 16 of the impact study documentation. We do not agree with the assertion that the cancellation of the project would only imply adverse consequences, that is, the condition of the environment would further deteriorate due to the mining activity of the past, and the loss of new jobs would contribute to the growing underdevelopment and poverty of the region. To our knowledge and according to information provided by our Romanian partners, the program serving the closing of the Minvest tailings management facilities in Déva is underway, comprising part of the program serving the closing of 16 tailings management facilities, as defined in Annex 7 of the Romanian accession treaty. Approximately 80 million EUR is available for this purpose, that is, the environmental problems caused by the region’s environmentally damaging mining activity in the past may be resolved. In addition, the owner of Minvest, the Romanian state, is obliged to rehabilitate the environment of the region following the shutdown of mining in the course of this year. We understand that the Romanian Ministry of Environmental Protection and Water Management has allocated funds for the first phase of the closing.

- In relation to other development alternatives in the area (e.g. tourism, agriculture, light industry), the view is unacceptable that other types of development projects would produce environmental risks similar to those associated with the planned mining investment.

- The document fails to objectively examine benefits originating from other forms of development; there is no mention of the cost-profit analysis of possible versions. Other development projects should be compared to the costs, benefits and environmental risks related to the planned mining investment. For example, concepts elaborated for the development of tourism, regional development and agricultural funding possibilities preceding and following EU accession should have been taken into account.The study of such alternative economic development possibilities was also supported at the joint meeting of the Hungarian and Romanian governments.

- The study also failed to examine other development opportunities for the population of the area following the termination of mining activity (in roughly 17 years), e.g. the limitation of agricultural and tourism development due to mining, the future availability of other jobs, etc. On the basis of the foregoing, it is false and unacceptable to claim that only the implementation of the investment could contribute to improving the environment of the region and economic development.

- Technologies substituting cyanide were not analyzed in detail; generally there is one assertion stating that “other methods are potentially more dangerous or technologically/economically inappropriate”. This claim would also need to be evidenced with cost-profit analyses. Other non-pollutant or less pollutant substances, replacing sodium cyanide, should have been tested with semiplant experiments to determine the applicability of the technology.

- Various versions of waste water management should also be supplemented with cost-profit analyses.

- With regard to the closing of the mine, more detailed tests are required in connection with the backfilling of the open pits. In this case, too, its necessity should be evidenced with profitability calculations, and there is the need to prove that the backfilling does not pollute the subsurface waters due to the ARD potential and the presence of heavy metals.

VI.	Monitoring systems (Volume 17.)

- The description of the monitoring system monitoring the condition of water is not full in its scope; the measurement methods of important parameters (e.g. weak acid dissociable (WAD) cyanide) are not indicated, the creation of the sampling network is not representative and there are few monitoring stations. Moreover there is no information available on the surface and subsurface sampling locations, e.g. well data: depth filtering, etc. The material does not discuss issues related to quality assurance, data collection, validation and the disclosure of data.

- The chapter on monitoring is pasted together; the WFD requirements are omitted, references are incorrect, data is outdated in some places and is not regarded as characteristic of current conditions.

- The EIS lacks the necessity of devising a monitoring system which provides comprehensive and continuous data, immediately signaling to a central station in the event of a possible emergency or exceeded threshold values.

- The Emergency Preparedness and Spill Contingency Plan must contain a forecast-alarm system detecting the spread of pollutants along paths in surface waters.

- Additional tests are necessary in relation to possibly accumulating pollutant/toxic substances in deposits, and the range of ecological tests, complying with the requirements of the Water Framework Directive, should be expanded.

VII.	Risks (Volume 18.)

- The impact study material lacks the analysis of the environmental impacts of similar Romanian investments (TRANSGOLD in Nagybánya, Borsabánya), for these mines have on several occasions caused extraordinary pollution which also posed a risk to Hungarian surface waters.

- In the absence of appropriate water quality models, the environmental impact study does not offer a satisfactory answer regarding the potential environmental impacts on water, thus the evaluation of impacts on the water environment are not acceptable, either.

- A detailed (certified and evidenced) pollutant spread model test, relating to the total affected waterflow system, should be carried out, assuming the occurrence of an event of however small probability which may contribute to the emission of some of the substance contained in the tailings management facility into the surface waters under operating conditions. A detailed analysis of accident probability should furthermore be performed, closely linked to climatic change scenarios and the evaluation of the above in summarized tables.

- It is necessary to devise a detailed emergency plan primarily serving the protection of surface waters.

- The impact of cross-border pollution on protected natural values should be analyzed (primarily in the area of the Körös-Maros National Park).

- The chapter dealing with the occurrence of emergencies caused by natural catastrophes is inadequate and not supported with reliable evidence.

- When describing hazards, indicators expressing magnitudes should be indicated (in hydraulic engineering and in relation to earthquakes, this is revealed by the probability of occurrence and the proportionate magnitude).

- When estimating standard load, one must consider possible extreme load (precipitation, earthquakes) – occurring with lower probability – in the course of the roughly 20 year (30 years including termination) construction/operating period and the real risks this can create when combined with specific accident scenarios.

- Technological risks related to the management of chemical substances should be assessed differently if the hazards are not the result of natural phenomena but human error, negligence or defects in design/construction/material quality.

- The safety of cyanide transport on public roads/rail is not discussed (poor quality roads – particularly in the winter and spring season).

- Measures linked to the development of the transportation/delivery infrastructure lying outside of the plant premises should also be detailed, considering the lack of information related to cyanide transports partly made on public roads. We should expect the transport of 12 thousand tons of sodium cyanide annually partly on public roads. Considering a 20 ton freight per truck, this would correspond to at least two deliveries per working day (600 a year and a total of 8,000 deliveries). The specific logistical concepts are unknown, although the material makes a reference to the “maximum utilization of rail”, and no mention is made on the unloading station, the possible public road transport routes, transport related precautionary regulations and emergency and accident prevention plans. It is a cause for concern that an operation of 13-15 years – including cyanide deliveries deemed to be dangerous goods transports – is planned without the construction of a direct rail connection. In a period of such length, with transport performance of such size under the known road conditions, it is almost certain that multiple accidents will occur in which the transport vehicle directly falls into a water flow (see the Kumtor incident in Kirghistan), and no information may be available on the event for hours. The documentation does not contain data on the identification and prevention of the above risks (this deficiency must by all means be remedied from the point of view of the party suffering the impact).

- The risk analysis related to flood risks correctly recognizes the rise in the risk of flood accidents but fails to provide detailed information on the referenced forecasting models. Thus the assertion is unacceptable that planning provides sufficient protection against such events.

- In the chapter examining cross-border impacts, in several places certain events are falsely assessed as local impacts which potentially produce an extensive impact. The impact of cyanide, for example, possibly emitted into surface water with water seeping through the dam of the tailings management facility is evaluated as a local impact.

- In relation to quality risk analysis, it would be practical and necessary to provide more detailed quantity risk analyses on all accident series of events which

- have a frequency of over 1E-8/year and

- the consequences are greater than the moderate level and

- the risk index is ≥ 8

- It would be necessary to describe in more detail and identify the selected event series for the additional quantity analysis. In the current study it is not clear as to why only a few of the identified, possible serious accident event series were selected for the quantity risk analysis.

- It would be necessary to provide a more detailed description of the consequence and reliability analysis. The study only contains general modeling assumptions. It is also unclear as to how the frequency of possible serious accidents was calculated and determined in quantity (e.g. with the fault tree/event tree method).

- The detailed consequence-result of not a single possible serious accident event series was provided (e.g. impact zones of the event series, considering the time window of the specific event series as well).

- In relation to the stability of the TMF’s dam, we object to an “only” 60 m deep dam burst taken into consideration which is successfully contained by the lower dam. Why was no analysis made of a possibly greater accident? In knowledge of the consequences of the accidents in Nagybánya and Borsabánya, the claim is totally unsubstantiated that the escaping tailings reach a distance of 1.6 km.

- When analyzing the cross-border impact, the cyanide concentration is of relevance (WAD, total, free?) which may even reach the value of 1.3 ppm. In the summary, however, a concentration of “only” 0.03-0.5 ppm is mentioned. What are the real values produced by the model? The above assertion is only supported by a single, deficient table which basically only contains the final results of a few scenarios. The precise flow kilometers, for example, are missing in relation to the mentioned cities, as well as the rate of the water discharge on the examined river sections and substantive data to support the results.

- The data (in case of tailings concentration in TMF) only relates to an average of 4-5 ppm discharged cyanide concentration. Scenarios relating to a higher cyanide concentration are not discussed. (The cyanide pollution in the Nagybánya incident was considerably greater. If – in case of TMF - only the 10 ppm concentration set out in the mining waste directive of the EU could be ensured, greater pollution would occur, and a larger concentration is not indicated in the scenarios.) Moreover if cyanide concentration would be around 1.3 ppm in the outlet water streams, as described in the impact study, that would be 13 times higher than the threshold value!

- Several references are then made to models, yet there are no accurate references which can be verified.

VII/1.	Calculation of individual risk and social risk

- It would be necessary to review in detail the quantitative definition of risks and results thereof of each of the possible serious accident event series and prepare a report on these. It is important to understand the risk elements produced by the individual impacts.

- It is not clear why diagram 7.24 depicts both social and individual risks in the same diagram. Generally the individual risk curves are illustrated on the map as individual risk curves, and their value is determined by the coordinates and distances and impacts produced as consequences.

- It would be necessary to prepare some kind of risk classification (ranking), considering the total possible serious accident event series, in order that the impact of the total event series be incorporated in the full risk.

- With regard to Hungarian SEVESO regulation, the maximum and minimum risk criteria do not correspond to criteria considered in the study. In consideration of such criteria, the social risk curve in the study is between the Hungarian maximum and minimum risk criteria levels which means that risk reducing measures are required.

- It is also necessary to provide the applied (validated and verified) software codes for the reliability analyses and consequence and risk analyses.

- The process of modeling and accurately calculating the on-site health and environmental risks is not presented. It is necessary to describe the detailed method of the calculation process of environmental and health risks relating to all considered event series. It would be practical to understand why the quantitative values of tables 7-34 were produced.

- We do not agree with the risk analysis of the project’s alternatives; we disagree with the rejection of version “0”, that is, the non-implementation of the project. This solution does not increase the current risks; the management of such risks is ensured through the recultivation of the Minvest facilities in the region and we understand that the necessary funds are available for such purpose. The rehabilitation plans will manage the risks related to the abandoned dumps and the ARD related problem.

- We find the rejection of cyanide-free technologies unacceptable without more in-depth tests. Cost-profit analyses would also be necessary to enable the assessment of such alternatives.

VIII.	Description of difficulties, Volume 18

- The two (2!!!) page description does not list already known deficiencies, problems and uncertainties, therefore it is unacceptable!!! The Hungarian request, formulated in clause 8 of the scoping document, is not satisfied, namely, that “the analysis should examine in detail the deficiencies and uncertainties underlying knowledge”.

IX.	Cross-border impacts

- The most important chapter from a Hungarian point of view is rather limited in describing the analysis of cross-border impacts. Unfortunately, in place of a well presented, “clean” material, only references are made to the fact that detailed information is contained in various volumes. Accurate information could have been provided in this regard if we also consider the frequent repetitions of the other volumes.

- This section of the documentation is incomplete, it fails to meet the “Espooi requirements”. The listing of various international requirements is insufficient in itself, moreover it is in disregard, for example, of the very important conclusion that the positive impact of the mine opening on the environment is not proven and even less so that disadvantages would arise from the “zero solution” in the impacted area of Hungary if the investment was not implemented.

- It is unacceptable that a 21 page material starts discussing substantive issues from page 14 and only with table descriptions.

- We disagree with the assertion that the pollution of surface waters, ARD pollution could only happen on a local/regional level – the contrary has been proven by numerous pollution incidents in the past. The study, too, makes many references to instances of “historical” pollution which continuously imposes a load on waterflows, spreading to Hungarian territory. The same applies to accidents related to the tailings management facilities.

- On the basis of the above, the assertion that a possible dam burst would not cause any cross-border environmental impacts is unsubstantiated and unacceptable.

- In the course of analyzing cross-border impacts, conformity with the international regulations below should have been analyzed:

- OECD Guidelines for Multinational Enterprises (especially Chapter V. - Environment, p.-p. 3., 4.  and 5.),

- OECD Guiding Principles for Chemical Accident Prevention, Preparedness and Response (2003.) + OECD Guidance on Safety Performance Indicators (2003.),

- UN ECE Convention on the Transboundary Effects of Industrial Accidents (1992.),

- UN ECE Agreement concerning the International Carriage of Dangerous Goods by Road (ADR) – Geneva, 30 September 1957 (current text with all amendments in force),

- OTIF Regulations concerning the International Carriage of Dangerous Goods by Rail, 2005 (RID),

- UN ECE Protocol on Civil Liability and Compensation for Damage Caused by Effects of Industrial Accidents on Transboundary Waters (2003.),

- UN ECE Convention on Civil Liability for Damage Caused during Carriage of Dangerous Goods by Road, Rail and Inland Navigation (CRTD),

- Rotterdam Convention on the Prior Informed Consent Procedure for certain Hazardous Chemicals and Pesticides in International Trade (1998.),

- UNEP APELL (Awareness and Preparedness for Emergencies at Local Level) for Mining  (2003.),

- UNEP/ICOLD/ICME – Environmental Regulation for Accident Prevention in Mining: Tailings and Chemicals Management; Mining and Sustainable Development,

- UNEP/WB-IFC/MMSD – Finance, Mining and Sustainability.

- UN ECE Convention on Protection and Use of Transboundary Rivers and International Lakes.

- 76/464/EEC Discharge of Dangerous Substances Directive.

- Inaccuracies are contained in chapter 4.1 of the same volume (e.g. the length of the Maros in Hungary is given as 20 km).

- The claim that the planned investment does not cause a major improvement in the quality of the Abrud river seems contradictory for the improvement of the current condition is one of the chief motivating factors behind the opening of the mine. Obviously any measure could result in a cross-border impact.

- Assertions made earlier in several places – unsupported with evidence – that the pollution of the waters and the dam burst can only cause local problems are unacceptable. The same applies to risks underlying transports.

X.	Waste Management Plan (Volume 22.)

- The volume containing the waste management plan basically reiterates word for word the descriptions, conclusions already made in Volume 10.

- The chapter contains references to the rules of Directive 2006/21/EC on the management of waste from extractive industries, but does not contain any specific plans.

- Detailed plans should be elaborated on the basis of the above directive with regard to the following, for example:

- waste management plan (Article 5);

- procedural and material legal obligations relating to the prevention of serious accidents (Article 6, Annex I – accident prevention concept, safety control system, internal and external emergency plan);

- construction and control of waste management facilities (Article 11 – e.g. minimization of damage caused in the landscape, reporting obligations, costs);

- closing, post-treatment and costs thereof (Article 12);

- general preventive measures (Article 13);

- availability and amount of financial collateral (Article 14).

XI.	Water Management Plan

- The sections of the chapter (Volume 23) on water management, discussing the cleaning of waste water, contain references implying that the details will be described in a later version of this management program. This version does not contain information relating to the operation of the waste water cleaning facilities, the costs of their operation and data on incoming and outgoing waste water.

- In relation to surface mining, changes expected in the wells of the surrounding area have not been analyzed; the level of subsurface waters is likely to fall; this condition should be modeled.

XII.	Issues related to tailings management facilities (Volume 25)

- The compilation also uses materials already included, in a moderately revised form. It contains descriptions related to the design, construction and operation of TMFs but less verifiable data.

- An accurate geological – geotechnical cross-section is not available in relation to the area of the TMF.

- It must be substantively certified that the soil may be appropriately solidified up to reaching a water seal; justification is required as to why the implementation of technical protection is unnecessary in the area of the TMF. It is not sufficient to analyze seismic hazards for 17 years; the TMF will remain in place following the closing of the mine.

- No reserve storage capacity is planned in addition to the storage capacity planned in the Corna valley which would enable the diversion of continuously incoming tailings quantities in the event of a breakdown or emergency (e.g. precipitation of an intensity and/or duration significantly exceeding the average value which could fill up the tailings storage capacity). The omission of the above from planning should be justified and supplemented.

- No subsurface water emergency storage capacity, with adequate volume, is planned for the event of a dam suffering a fault (caused by overflow, earthquake or other stability related problem). The planning and construction of a subsurface water emergency storage capacity, with adequate volume, is extremely important with regard to establishing environmental safety, for this could prevent the spread of hazardous industrial waste water leaving the tailings storage facility in the event of a possible dam burst.

- Data relating to the specific density of the tailings is hypothetical; different values are listed in the table, in disregard of the recommendations and requirements of the Dangerous Substances Directive 76/464/EC.

- Only the pumping of tailings with 45% water content was examined; calculations related to alternatives with higher – safer – concentration are unavailable.

- The Corna valley is situated in a fault zone which has not been adequately researched with regard to the long term stability of the dam, the characteristic condition of the soil and the water sealing capacity of the TMF’s support.

- The construction of a multifunctional dam warrants that the clay core of the dam is made of water-proof material, it does not swell or slide and pressure does not cause any changes. Considering the extreme height of the dam and the quantity of stored material, stability must be evidenced with at least a model experiment and calculations. The study does not reflect recommendations defined by ICOLD (International Commission of Large Dams).

- It is unclear which model was used for modeling the stability of the dam; baseline data is unavailable.

- It is necessary to supplement the plan relating to the maintenance and monitoring of the dam.

- It is necessary to examine whether spills may nevertheless occur, possibly causing a fault in the dam, in the event of a critical precipitation load, possibly involving the breakthrough of “external waters” contained by the curtain drains (for the reason that the curtain drains are not sized to drain the maximum possible amount of precipitation).

- Is the volume of the second storage facility sufficient to withhold water possibly leaving the spillway? No subsurface water related tests were conducted in the surroundings of the TMF.

- The specification of the planned dam’s height is essentially important for the assessment of the risks of expected environmental impacts! From the point of view of risks, it is not irrelevant whether the dam is 185 or 200 m high (nearly 10% variation). Unfortunately the various volumes indicate different values.

XIII.	Cyanide management (Volume 26.)

- The cyanide management plan exclusively contains general descriptions regarding the transport and use of cyanide. (We analyzed above risks related to the management of cyanide.)

- According to the documentation, the decision is pending as to the transport route which we judge to be a major factor due to the risk estimates.

- In the course of applying the CIL technology, the manually controlled dosage of the cyanide solution is problematic; this procedure is not in conformity with BAT. We are of the view that in the INCO procedure the impact of winter cold weather on the applied HPD plastic pipes – running on the surface unprotected – was disregarded.

- There is no plan for introducing any environmental quality assurance system, related to the operation and monitoring of the system, which would significantly improve operating safety conditions.

- The BAT reference document on the management of mining tailings describes 3 European mines operating with cyanide technology. Of the above, 2 mines produce a WAD cyanide concentration of less than 2 ppm in the tailings management facility, this being the result of the best available technique (BAT) in Europe. The impact study targets “compliance with EU standards”, but plans a WAD cyanide concentration of less than 10 ppm in the tailing storage facility, notwithstanding the fact that this is not an existing but a new facility. Pursuant to Directive 96/61/EC (IPPC), however, as of October 2007, the given facility may only operate by the application of the best available technique; the value below 10 ppm requires clarification which is currently below 2 ppm in accordance with the prevailing BAT. If access to the BAT is indeed the purpose of the investor, it must follow the currently evidenced best technique.

XIV.	Preparation for emergencies and damage prevention plan

- The damage prevention plan primarily describes rules of procedure; measures to be applied in the event of an assumed incident, including the process of detection, notification and prevention.

- Standard Operational Procedures are prepared in certain cases; the referenced annexes provide information on the details thereof. Unfortunately, volume 28 does not have annexes. There is, however, reference to the fact that such SOPs will be later devised. Accordingly, the procedural methods described herein are not verifiable, no opinion may be issued on these in their current condition.

XV.	Mine closing, recultivation (Volume 29.)

- Additional information should be provided in relation to the RMGC (court of registration data, shareholders, capital composition, listing, banks providing the funds, information relating to operation and professional activity until now).

- The investor is responsible for presenting the appropriate financial guarantees for the costs of the mine’s closing. The argument is unacceptable that such costs shall be allocated in the course of mining activity. The creation of a liability insurance system is not discussed either.

- The presentation of similar financial guarantees is necessary in relation to monitoring in the period following the closing and the management of ARD waste water and escaped water (for a term of at least 30 years or indefinitely, as in many cases).

- The long term management of acidic subterranean water is not resolved; semi-passive management is not efficient enough in extreme (cold) weather conditions. The contrary to the above should be evidenced with reference results.

- The costs budgeted for the closing of the mine are unrealistically low (70 million USD – Volume 29, p. 130). No costs have been planned for monitoring and waste water cleaning  for the term of operation.

- The study allows us to conclude that the operator is unable to assume liability in accordance with Directive 2004/35/EC.

- No financial guarantees are available for the long term operation of the TMF.

- According to the study, the possibility of revitalization could be established through surface mining activity based on the best available technique (BAT), “but the implementation thereof depends on the decision of the authorities”. The message sent by the investor to the Romanian authorities seems to imply that the rehabilitation costs are not charged to the project budget.

- There is no adequate financial security for emergencies. The EIS does not make any proposals regarding the prevention of the possible consequences of the most severe accident, the depositing of a financial basis serving the restoration of the original condition.

- The study does not discuss the issue of the barrier layer related to the closing of the TMF. The question arises as to how this is resolved, the insulation used for protection against surface inwash, etc.

- The condition of the environment will only deteriorate following the termination of mining activity if environmental rehabilitation required following its closing is not implemented.

XVI.	Miscellaneous

- Biological diversity and the chapters on vegetation and the fauna contain few specifics; they do not reflect or falsely reflect the results of the basic studies. The increase in vehicle traffic, too, represents the greatest risk threatening natural values.

- According to chapter 9.5, there are no protected or endangered plant and animal species in the RMP area, whereas fully protected species do in fact exist in the area, according to the so-called Habitat Directive 92/43/EEC on the conservation of natural habitats and of wild fauna and flora.

- Species of flora are not listed in the chapter on biodiversity.

- In relation to the listed species, information is lacking on there area of habitat, number and distribution.

- It is necessary to analyze and evaluate the impact of pollutants emitted into the environment and hazardous to health.

- The main rules of Directive 1999/31/EC are infringed, e.g. there is no security for the management of environmental and health problems.

XVII.	Technical comments related to the EIS

- The structure of the documentation is complicated, containing frequent cross-references, numerous overlapping, repeated data. Some parts of the material are repeated through several volumes; the numbering of charts and tables is in many places incorrect. Moreover numerous important technical issues remain unanswered, and certain technical solutions are not appropriately substantiated. The documentation is partial, basically exclusively containing positive assessments, lacking any contradictions or points of uncertainty; the possible problems and negative consequences are not investigated in sufficient depth.

- The quality of the translation of the English language material provided to us on a CD is poor and inadequate; in many cases, the keys, references of the tables, maps and charts are in Romanian in the English version. Some tables and diagrams are not assigned dimensions, rendering these as unintelligible. The CDs did not contain documentation related to the recording of the baseline conditions (Baseline reports 1-6), this could only be downloaded from the websites.

International Commission on Large Dams,

International Council on Metals and the Environment

workshop held in cooperation with the Gov-t of Australia

World Bank – International Finance Corporation

Mining, Minerals and Sustainable Development Project